3GPP TSG-RAN WG4 Meeting # 110-bis                                                        R4-2406412

Changsha, China,  April 15 – 19, 2024

Title: 	FR2 HST RRM performance requirements

Agenda Item:	6.7.2

Source: 	Samsung

Document for:	Approval

Introduction

For Rel-18 work item on enhanced NR support for high speed train scenario in frequency range 2, there is one thread [212] organized for RRM discussion on RAN4#110-bis, and this document is used to capture the WF on topics in thread [212].

Topic #1: RRM Performance Requirements for Rel-18 FR2 HST

In WF [R4-2403319] agreed in RAN4#110-bis meeting, the following Test case list was updated and agreed for information.

Table 1 The planned RRM performance test cases list (for information)

| TC index | Corresponding Core Requirement | Necessity of New Test Case | Company for CR volunteer |
| --- | --- | --- | --- |
| TC1 | Requirements for Cell Re-selection | New test case needed. (New test case for A.7.1.1.X Cell reselection to FR2 inter-frequency NR case for Rel-18 FR2 HST intra-band CA) | Intel |
| TC2 | Requirements for SCell Activation Delay for Deactivated SCell | No new test case needed. |  |
| TC3-TC6 | Requirements for Inter-frequency measurement with measurement gaps | No new test case needed. |  |
|  |  | No new test case needed. |  |
|  |  | New test case needed. (New test case for A.7.6.2.X3 SA event triggered reporting tests for Rel-18 FR2 HST intra-band CA with SSB time index detection when DRX is not used (Pcell in FR2)) | Huawei |
|  |  | New test case needed. (New test case for A.7.6.2.X4 SA event triggered reporting tests for Rel-18 FR2 HST intra-band CA without SSB time index detection when DRX is not used (Pcell in FR2)) | Ericsson |
| TC7 | Requirements for Inter-frequency measurement without measurement gaps | No new test case needed. |  |
| TC8 | Requirements for One shot large UL timing adjustment for FR2 Power Class 6 UE | New test case(s) needed. | Nokia |
|  | Requirements for MAC-CE based TCI state switch delay in HST FR2 scenarios |  |  |
| TC9 | Requirements for SSB based radio link monitoring | No new test case needed. |  |
| TC10 | Requirements for SSB based beam failure detection | No new test case needed. |  |
| TC11 | Requirements for SSB based L1-RSRP | New test case needed. (New test case for A.7.6.3.X SSB based L1-RSRP measurement when DRX is used for FR2-1 PC6 UEs supporting SimultaneousReceptionFR2HST-r18) | Samsung |
| TC12 | Requirements for Intra-frequency measurements with/without measurement gaps | New test case needed. (New test case for A.7.6.1.X SA event triggered reporting test without gap under non-DRX for UE supporting [measurementEnhancementCAInterFreqFR2-r18]) | Nokia |

Sub-topic 1-1 Test case design on SSB based L1-RSRP in Rel-18 FR2 HST

Issue 1-1-1 AoA setup for multi-Rx chain DL reception in Rel-18 FR2 HST

- Agreement:

  - Necessity: It is necessary to define a new 2AoA setup for multi-Rx chain DL reception in Rel-18 FR2 HST.

    - The new 2AoA setup for FR2 HST RRM test cases is: 1 AoA in Rx beam peak direction, 1 in non Rx beam peak

    - The new 2AoA setup is applicable to support the SSB based L1-RSRP measurement test on FR2-1 PC6 UEs supporting SimultaneousReceptionFR2HST-r18

  - How to design: Define the new 2AoA setup in new sub-Clause (s) in A.3.15.4 based on the RF requirements defined in Section 7.3K.6 TS 38.101-2.

Issue 1-1-2 Test case SSB configuration

- Agreement:

  - RAN4 to use two SSBs in SSB based L1-RSRP measurement test on FR2-1 PC6 UEs supporting SimultaneousReceptionFR2HST-r18

Sub-topic 1-2 Test case design on UL timing adjustment/TCI state switching in Rel-18 FR2 HST

Issue 1-2-1 Whether to split the testing of the UL timing adjustment and TCI state switching delay

- Agreement from Wednesday RRM session:

Agreement: Introduce a new TC with 0 indication from MAC CE. Further discuss offline whether the corresponding Rel-17 legacy test can be skipped or kept.

  - Rel-17 HST UL timing TC (A.7.5.8.3) is applicable to PC6 devices supporting enhanced MAC CE

Issue 1-2-2 How to design the MAC-CE indication based TCI state switch delay TC

- Way Forward: Address directly at the CR stage.

Issue 1-2-3 Whether to define a new TC to verify the TCI state switching delay requirement in Multi-Rx scenario

- Agreement from Wednesday RRM session:

Agreement: For FR2 HST, it is RAN4 common understanding that the TCI state switching delay requirements defined in Rel-17 are applicable for the Rel-18 scenario with simultaneous reception. Not introduce a new Rel-18 test case on TCI state switching delay for the PC6 UE receiving two simultaneous DL signals associated with two different QCL TypeD RSs.

Sub-topic 1-3 Missing testing parameters

- Agreement:

  - RAN4 to define Minimum SSB_RP values in the Table B.2.3-2: Conditions for inter-frequency measurements in FR2 for PC6 UEs

    - The Minimum SSB_RP values shall be defined based on the identified values agreed in the RAN5 reply LS discussion

Sub-topic 1-4 Test cases for inter-frequency intra-band CA measurements

- Way forward:

  - Update draft CR to use two RF channels (two cells) for SA inter-frequency test cases for HST FR2 PC6 UEs in Rel-18.