3GPP TSG-WG4 Meeting #110bis	R4-2406418

# Changsha, China, April, 15 - 19, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR 38.133 Corrections to Case 1 and Case 2 core requirements for NR_MG_enh2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Corrections to Case 1 and Case 2 requirements for NR_MG_enh2 are needed as described in the summary of change. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In subclause 9.1.12.2, the term “[ConMGs with Pre-MG]” is used for support of FG 32-1. - In subclause 9.1.12.3, collision requirements for 2 Pre-MGs are clarified. - In subclause 9.1.12.4 requirements for the case of concurrent gap colliding with Pre-MG deactivation procedure for Pre-MG with lower priority are corrected. - In subclause 9.1.13.2, it is clarified that case 2 requirements are defined for the gap combination of concurrent gap and NCSG rather than the combination of measurement gap and NCSG. Table 9.1.13-1 just contains information on the gap combination configurations, whilst no NSCG pattern combination is specified therein. This is corrected. - In subclause 9.1.13.3, the list of gap combination configurations for collisions involving NCSGs is clarified to apply for both concurrent MG+NCSG and NCSG+NCSG configurations. Another gap combination configuration is added: one per-UE NCSG and one per-FR measurement gap. - Editorial errors are removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent requirements in 9.1.12 and 9.1.13 for Case 1 and Case 2 requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.12.2, 9.1.12.3, 9.1.12.4, 9.1.13.2, 9.1.13.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R4-2405879 with merging of R4-2405881, R4-2404773. |  |  |  |  |  |  |  |  |

--- Start of change 1 ---

#### 9.1.12.2	Requirements

If the UE requires measurement gaps and/or Pre-MGs to identify and measure intra-frequency cells and/or inter-frequency cells and/or inter-RAT E-UTRAN cells, and the UE supports [ConMGs with Pre-MG] but does not support independent measurement gap patterns for different frequency ranges as specified in Table 5.1-1 in [18, 19, 20], in order for the requirements in the following clauses to apply, the network can provide the UE with not more than two per-UE measurement gap patterns for monitoring all the frequency layers.

If the UE supports both [ConMGs with Pre-MG] and independent measurement gap patterns for different frequency ranges as specified in Table 5.1-1 in [18, 19, 20], in order for the requirements defined for concurrent measurement gaps with Pre-MG to apply, the network can provide the measurement gap patterns’ combinations specified in Table 9.1.12.1 for monitoring of all frequency layers.

Table 9.1.12-1: The number of Gap Combination Configurations by UE supporting both [ConMGs with Pre-MG] and independent measurement gap patterns

| Gap Combination Configuration Id | The number of simultaneous configured measurement gap patterns |  |  |
| --- | --- | --- | --- |
|  | Per-FR1 measurement gap | Per-FR2 measurement gap | Per-UE measurement gap |
| 0 | 2 | 1 | 0 |
| 1 | 1 | 2 | 0 |
| 2 | 0 | 0 | 2 |
| 3Note 1 | 1 | 0 | 1 |
| 4Note 1 | 0 | 1 | 1 |
| 5Note 1 | 1 | 1 | 1 |
| 6 | 2 | 0 | 0 |
| 7 | 0 | 2 | 0 |
| Note 1: Gap Combination Configuration Id #3, #4, #5 are only applicable when the per-UE measurement gap is associated to measure PRS for any RSTD, PRS-RSRP, UE Rx-Tx time difference measurement and PRS-RSRPP measurement defined in TS 38.215 [4]. [Note 2]: For UE capable of [Concurrent Pre-MG], up to 2 measurement gap patterns can be configured as Pre-MG in one FR, regardless of whether they are per-UE or per-FR configuration. Otherwise, the gaps can only be configured as Gap(s) configured via GapConfig without suffix or Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17. Note 3: In Gap Combination Configuration Id #0, #1, #6, #7, one per-FR measurement gap in an FR can be associated to measure PRS for any RSTD, PRS-RSRP, PRS-RSRPP, RSCP, RSCPD and UE Rx-Tx time difference measurement defined in TS 38.215 [4] provided that UE supports independentGapConfigPRS-r17. |  |  |  |

When UE supports [ConMGs with Pre-MG], the gap association for a frequency layer is configured by the network via [gapAssociation] in [ToBeMeasureConfig]. In this case the gap association rules in clause 9.1.8.2 shall also apply to either measurement gap or Pre-MG.

When autonomous mechanism [1] is used for activation/deactivation of Pre-MG pattern, the UE shall autonomously determine the Pre-MG status only based on the measurement objects associated with the concerned Pre-MG. The related Pre-MG autonomous activation/deactivation mechanism is specified in clause 9.1.7.3.1.

When network-controlled mechanism [1] is used for activation/deactivation, the requirements specified in clause 9.1.7.3.2 apply.

When UE supports [ConMGs with Pre-MG], where at least one of the concurrent gaps is Pre-MG, each measurement gap pattern supported by the UE is listed in Table 9.1.2-1 based on the applicability specified in table 9.1.2-3.

The requirements in clause 9.1.2 are applicable for the UE capable of [ConMGs with Pre-MG] and configured with multiple concurrent measurement gap patterns within each activated Pre-MG pattern.

#### 9.1.12.3	Collisions involving Pre-MG(s)

Collisions between a Pre-MG and a measurement gap may occur only when the Pre-MG is activated. No collisions can occur between a per-FR Pre-MG and a per-FR measurement gap when they are configured in different FRs.

Collisions between two Pre-MGs occur only when both Pre-MGs are activated and satisfy the collision rule defined in clause 9.1.8.3. No collisions can occur between per-FR Pre-MGs when they are configured in different FRs.

The requirements for [concurrent measurement gaps with Pre-MG] apply provided that the two measurement gaps (at least one of the gaps is activated Pre-MG) colliding with each other are configured with different priorities.

#### 9.1.12.4	Collision between Pre-MG activation/deactivation and measurement gap

A measurement gap occasion and a Pre-MG activation/deactivation procedure collide when the ending point of the Pre-MG activation/deactivation procedure occurs anywhere within a time period starting 4ms before the starting point of the gap occasion and ending 4ms after the ending point of the gap occasion. The ending point of the Pre-MG activation/deactivation procedure is defined in clause 8.19.5.3.

When a collision occurs between a measurement gap occasion and a Pre-MG activation procedure, and the Pre-MG is configured with higher priority, the UE shall perform measurements during the measurement gap occasion and the activation of the Pre-MG is delayed until 5ms after the ending point of the measurement gap occasion.

When a collision occurs between a measurement gap occasion and a Pre-MG deactivation procedure, and the Pre-MG is configured with higher priority, the measurement gap occasion shall be dropped. The measurement gap occasion shall remain to be dropped until the ending point of the Pre-MG deactivation procedure.

When the activated Pre-MG and measurement gap satisfy the collision rule defined in clause 9.1.8.3 and the Pre-MG is configured with lower priority, the UE shall perform measurements in the occasion of the measurement gap regardless of whether it collides with the Pre-MG activation procedure or it collides with the Pre-MG deactivation procedure.

The UE is expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI in the corresponding NR serving cells in the slots of the configured Pre-MG that are dropped according to the requirements in clause 9.1.8.4.

--- End of change 1 ---

--- Start of change 2 ---

#### 9.1.13.2	Requirements

If the UE requires concurrent measurement gaps and/or NCSGs to identify and measure intra-frequency cells and/or inter-frequency cells and/or inter-RAT E-UTRAN cells, and the UE supports [concurrentNCSGPerUE-OnlyMeasGapwithNCSG-r18] but does not support independent measurement gap patterns for different frequency ranges as specified in [14], in order for the requirements in the following clauses to apply, the network can provide one per-UE concurrent measurement gap and one per-UE NCSG or at most two per-UE NCSGs for monitoring of all frequency layers.

If the UE requires concurrent measurement gaps and/or NCSGs to identify and measure intra-frequency cells and/or inter-frequency cells and/or inter-RAT E-UTRAN cells, and the UE supports [concurrentNCSGPerUE-PerFRCombMeasGapwithNCSG-r18] as specified in [14], in order for the requirements defined for [concurrent measurement gaps with NCSG] to apply, the network can provide the concurrent measurement gap with NCSG  combination configurations specified in Table 9.1.13-1 for monitoring of all frequency layers.

Table 9.1.13-1: The number of Gap Combination Configurations by UE supporting both concurrent measurement gap with NCSG patterns, per-FR NCSG patterns and independent measurement gap patterns

| Gap Combination Configuration Id | The number of simultaneous configured measurement gap patterns |  |  |
| --- | --- | --- | --- |
|  | Per-FR1 [measurement gap] | Per-FR2 [measurement gap] | Per-UE [measurement gap] |
| 0 | 2 | 1 | 0 |
| 1 | 1 | 2 | 0 |
| 2 | 0 | 0 | 2 |
| 3Note 1 | 1 | 0 | 1 |
| 4Note 1 | 0 | 1 | 1 |
| 5Note 1 | 1 | 1 | 1 |
| 6 | 2 | 0 | 0 |
| 7 | 0 | 2 | 0 |
| Note 1: Gap Combination Configuration Id #3, #4, #5 will be only applied when the per-UE measurement gap with NCSG is concurrent MG (and cannot be NCSG) is associated to measure PRS for any RSTD, PRS-RSRP, UE Rx-Tx time difference and PRS-RSRPP measurement defined in TS 38.215 [4], and when the per-FR measurement gap with NCSG in an FR is NCSG. [Note 2: In Gap Combination Configuration Id #0, #1, #6, #7, one per-FR measurement gap in an FR (and cannot be NCSG) can be associated to measure PRS for any RSTD, PRS-RSRP, UE Rx-Tx time difference and PRS-RSRPP measurement defined in TS 38.215 [4] provided that UE supports independentGapConfigPRS-r17.] Note 3: In Gap Combination Configuration Id #0, #1, #2, #6, #7, one FR can be configured with up to 2 NCSGs, regardless they are per-UE or per-FR configured. Otherwise, the gaps can only be configured as Gap(s) configured via GapConfig without suffix or Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17. |  |  |  |

For UE configured in the SA operation mode, when monitoring of multiple inter-RAT E-UTRAN carrier frequency layers and inter-frequency NR carrier frequency layers as configured by PCell using gaps, each monitored carrier frequency layer, including following measurement types:

-	a measurement object with SSB based measurement,

-	a measurement object with CSI-RS based measurement,

-	E-UTRA inter-RAT measurement object,

can be associated to either one concurrent measurement gap pattern or one NCSG pattern, while the following measurement types:

-	E-UTRAN inter-RAT RSTD measurement,

-	NR PRS-based positioning measurement,

can be only associated to one concurrent measurement gap pattern. Requirements for [concurrent measurement gaps with NCSG] apply provided that each frequency layer is only associated with one concurrent measurement gap or one NCSG, and at least one of the gaps is NCSG. There can be one or more frequency layers associated with each concurrent measurement gap or each NCSG. Furthermore, if the UE is not capable of [concurrentMeasGapEUTRA-r17][2], all E-UTRAN measurement objects are expected to be associated with a single concurrent measurement gap or NCSG.

When UE supports [concurrent measurement gap with NCSG], where at least one of the concurrent gaps is NCSG, supported concurrent measurement gap patterns are listed in Table 9.1.2-1 based on the applicability specified in table 9.1.2-3, while supported NCSG patterns are listed in Table 9.1.9.3-1 based on the applicability specified in table 9.1.9.3-2.

The requirements in clause 9.1.2 are also applicable for the UE capable of and configured with multiple [concurrent measurement gap with NCSG] patterns within each concurrent measurement gap pattern. The requirements in clause 9.1.9 are also applicable for the UE capable of and configured with multiple [concurrent measurement gap with NCSG] patterns within each NCSG pattern.

#### 9.1.13.3	Collision involving NCSGs

Collisions between occasions of concurrent measurement gap and NCSG or of two NCSGs may occur as specified in this clause if the two occasions are

-	two per-UE NCSG, or

-	two per-FR NCSG in the same FR, or

-	one per-UE NCSG and one per-UE measurement gap, or

-	one per-UE measurement gap and one per-FR NCSG, or

-    one per-UE NCSG and one per-FR measurement gap, or

-	one per-FR NCSG and one per-FR measurement gap in the same FR.

and if the gap collision condition specified in clause 9.1.8.3 is met then the gap collision rule applies.

When the first occasion is NCSG, the ending point is the end of VIL2 and/or when the second occasion is NCSG, the starting point is the start of VIL1.The requirements with [concurrent measurement gaps with NCSG] apply provided that two gaps (at least one of the gaps is NCSG) colliding with each other are configured with different priorities.

--- End of change 2 ---