3GPP TSG-RAN WG4 Meeting # 110bis									R4-2406428

Changsha, China, 15th - 19th April, 2024

Title: 	WF on RRM requirements for NR ATG

Agenda Item:	4.4

Source: 	CMCC

Document for:	Approval

- Maintenance

Issue 1-1: Whether to change the reference point of the tolerance for deriveSSB-IndexFromCell and deriveSSB-IndexFromCellInter-r17

Agreement:

    - Do not change the existing tolerance requirement of deriveSSB-IndexFromCell/Inter-r17.

    - FFS whether need to clarify the applicable scenario for deriveSSB-IndexFromCell/Inter-r17

      - FFS: E.g., UE does not expect the deriveSSB-IndexFromCell/Inter-r17 to be set as true when RTD from UE between serving and neighbour cell is large than X

      - Other wording of applicability rules are not precluded.

Issue 2-1: Test method for UE with antenna array

Agreement:

    - Conducted test without scaling factor (Scaling factor = 1)

Issue 2-2: The sharing factor between L1 and L3 measurement in RLM and BFD test case

Agreement:

    - For ATG UE with the antenna array, if scaling factor is 1 in the test cases, no need to consider the sharing factor between L1 and L3 measurement.

Issue 2-3: UE location (GNSS) setting in the location-based CHO test case

Agreement:

    - keep the existing setting.