3GPP TSG-RAN WG4 Meeting #110bis		R4-2406430

Changsha, China, 15th – 19th April, 2024

Agenda item:			6.16.9

Source:	Moderator (Qualcomm Incorporated)

Title:	WF on RRM requirements for NR_NTN_enh

Document for:	Approval

# Introduction

The summary covers the contributions submitted under the following AI:

- 6.16.6	RRM core requirements	[NR_NTN_enh-Core]

  - 6.16.6.1	NR-NTN RRM requirements in above 10 GHz bands	[NR_NTN_enh-Core]

  - 6.16.6.2	Network verified UE location	[NR_NTN_enh-Core]

  - 6.16.6.3	NTN-TN and NTN-NTN mobility and service continuity enhancements	[NR_NTN_enh-Core]

- 6.16.7	RRM performance requirements	[NR_NTN_enh-Perf]

  - 6.16.7.1	NR-NTN RRM performance requirements in above 10 GHz bands	[NR_NTN_enh-Perf]

  - 6.16.7.2	Network verified UE location	[NR_NTN_enh-Perf]

  - 6.16.7.3	NTN-TN and NTN-NTN mobility and service continuity enhancements	[NR_NTN_enh-Perf]

# Topic #1: UL timing requirements in bands above 10 GHz

### Issue 1-6A: Te_NTN for 60kHz and 120kHz in Case2

Agreement: (ad-hoc)

- Further check and come back in next meeting

### Issue 1-6B: Te_NTN for 60kHz and 120kHz in Case3

Agreement: (ad-hoc)

- In Case-3, Te_NTN [Ts] for 120kHz UL SCS is:

  - [10]

<Note: No update was made compared RAN4#110 agreement>

# Topic #2: RRM requirements in bands above 10 GHz

No Agreement

# Topic #3: Network verified UE location

### Issue 3-2: Measurement period and accuracy requirements on RTD

Agreement (online):

- Remove factors related to multiple positioning frequency layers, multiple RX TEGs and Rx beam sweeping from the Rx-Tx requirements.

- Remove the below applicability rules from UE RX-TX time difference measurements:

  - FFS: When a serving cell change occurs during the measurement period, the UE shall continue and complete the UE Rx-Tx time difference measurement provided that the serving cell change does not impact SRS configuration for the UE Rx-Tx time difference measurement.

  - If UE uplink transmission timing changes due to the UE autonomous timing adjustment defined in clause 7.1C.2 during the UE Rx-Tx measurement period, then:

    - UE Rx-Tx measurement period requirements in this clause shall not apply for a cell, which is not the downlink reference cell (defined in section 7.1C.1) for SRS transmission. The UE Rx-Tx time difference measurement period may be restarted in such case.

- Supporting Nsample = 1 is a component of NW verified location (FG 44-2), and it does not require UE to support reduced sample number for TN positioning measurement (FG 27-3-1).

  - HW to draft the LS to RAN1/2 about the information

### Issue 3-4: Measurement accuracy requirements on UL timing drift

FFS (ad-hoc):

- No new applicability condition for UE Rx-Tx measurement requirements related to amount of variation in the applied TA during measurement period.

  - Including DL timing drift reporting as condition

### Issue 3-5: Other impact on RRM

Agreement (online):

- When UE switches to a new satellite switch with same PCI (through HO, CHO or satellite switch with re-sync for both hard and soft satellite switch), select one option from:

  - Option 1a: UE stops the PRS measurement after t_service and restart the UE Rx-Tx time difference measurement after switch is complete.

    - The PRS configuration of the two satellites with the same PCI follows RAN1/2 specficiaton, and no further restriction for gNB/LMF to be defined in RAN4.

    - The UE measurement accuracy requiremet does not apply if the PRS transmission from two satelliates overlap in time/frequency domain. Further clarify the definition of “overlapping” offline.

  - Option 1b: UE stops the PRS measurement after t_servicestart and restart the UE Rx-Tx time difference measurement after switch is complete.

    - The PRS configuration of the two satellites with the same PCI follows RAN1/2 specficiaton, and no further restriction for gNB/LMF to be defined in RAN4.

  - Option 2: UE stops the PRS measurement after t_service and until LMF triggers new measurement.

FFS (online):

- When UE switches to a new satellite switch with different PCI (through HO or CHO),

  - UE stops the PRS measurement for the source cell and re-start PRS measurement for the target cell after SRS (re)configuration on the target cell is complete.

# Topic #4: Idle/Inactive mode mobility enhancements

No Agreement

# Topic #5: Connected mode mobility enhancements

### Issue 5-2-S: Soft’ Satellite switch (5-2-S1 and -S2 from RAN4#110 are merged)

Agreement (online):

- Scheduling restrictions over [t-ServiceStart ~ t-Service] for UE incapable of parallelMeasurementWithoutRestriction-r17 and/or [differentSCS between SSB and data]

  - Define scheduling restriction during soft satellite switch from UE perspective, i.e. scheduling restriction are allowed only during SSB occasions of the target satellite (same as 9.2C.5.3)

    - For the scheduling restriction: For RSRP measurement, 1 additional symbol before and after SSB block. For RSRQ measurement, 1 additional symbol before and after RSSI symbols.

Agreement (online):

- Optimization on measurements

  - UE is allowed to skip measurements other cells and satellites than the target satellite and source satellite from T-serviceStart to the satellite switch completion.

Agreement (online):

- Impact on inter-satellite neighbour cell measurements

  - No optimization for the scenario of ‘(both hard and soft) satellite switch with re-sync’ on inter-satellite neighbor cell measurements

# Topic #6: Performance requirements

### Issue 6-2: Above 10 GHz, Test case list

Agreement: (ad-hoc)

- Define test cases for the below cases for both Type 1 and Type 2 UEs. Detailed configurations are to be discussed separately.

  - UL timing accuracy

  - Mobility in RRC Idle/Inactive mode

  - Mobility in RRC Connected mode

    - Intra-satellite

    - Inter-satellite (blind mobility)

  - RLM

  - L1-RSRP

  - L3 measurements

  - Measurement accuracy

    - If measurement accuracy requirements specified in Rel-18

### Issue 6-2-2: Above 10 GHz, Rx beam gain

Agreement: (ad-hoc)

- FFS whether the RF margin for different RX beams in existing TN FR2-1 intra-frequency relative accuracy requirements can be removed or  not

- Existing absolute measurement accuracy requirement of TN FR2 (including intra-frequency and inter-frequency) can be applied for NTN UE above 10GHz.

- Further discuss the minimum SSB_RP condition on accuracy requirements

### Issue 6-2-3: Above 10 GHz, UL timing accuracy

Agreement: (ad-hoc)

- Define UL timing test cases at least for following configuration:

  - UL SCS 120kHz with DL SSB SCS: 120kHz

- FFS whether any other configuration need to be considered or not

- For UL SCS is 120kHz,

  - Transmit Timing Test covering

    - Case 1,2,3

  - Timing advance adjustment accuracy covering

    - Case 1,2,3

- Note:

  - Case-1: Stationary UE for GSO

  - Case-2: Stationary UE for LEO

  - Case-3: Mobile UE for GSO

### Issue 6-3-1: Below 10 GHz, Test set-up and applicability rule

Agreement: (ad-hoc)

- NTN-TN inter-frequency cell reselection (Intra-RAT to NR TN and Inter-RAT to LTE TN)

  - For earth-moving cell, time-/location-based measurement initiation for cell reselection:

    - No test case

  - For earth fixed cell:

    - Define test cases

- NTN to NTN time-based measurement initiation for cell reselection in earth-moving cell

  - Not define test case

- NTN to NTN location-based measurement initiation for cell reselection in earth-moving cell

  - Not define test case

Agreement: (online)

- Define two TCs:

  - RACH-less for soft satellite switch

  - RACH-based for hard satellite switch

FFS:

- NTN to NTN RACH-less (C)HO

- NTN to NTN time-based trigger CHO enhancements

- NTN to NTN location- based trigger CHO enhancements

- Network verified UE location

# CR split

Agreement: (online)

The test configurations in [] are the baseline. Other configurations can be considered if consensus can be reached in the next meeting, proponents of other configurations to bring the corresponding CR in the next meeting.

| Band | Category | Test case | Configuration | Company |
| --- | --- | --- | --- | --- |
| Above 10 GHz | UL transmission timing | 1. Transmit Timing 2. Timing advance adjustment accuracy | UL SCS: 120kHz DL SSB SCS: 120kHz Case 1,2,3 | Samsung |
| Above 10 GHz | Connected mode mobility | 1. Intra-satellite handover 2. Inter-satellite (blind mobility) | Type 1 and Type 2 [Intra-frequency] | Qualcomm |
| Above 10 GHz | Signaling | RLM [Out-of-sync, In-sync] | Type 1 and Type 2 [Non-DRX, SSB] | vivo |
| Above 10 GHz | Measurement procedure | L3-RSRP [Intra-satellite] | Type 1 and Type 2 [Without gap, Intra-frequency, SSB, non-DRX, SSB index reading] | Xiaomi |
| Above 10 GHz | Measurement procedure | L1-RSRP | Type 1 and Type 2 [SSB, non-DRX] | Apple |
| Above 10 GHz | Measurement accuracy | L3-RSRP | Type 1 and Type 2 [Without gap, Intra-frequency, SSB, non-DRX, SSB index reading] | Nokia |
| Above 10 GHz | Measurement accuracy | L1-RSRP | Type 1 and Type 2 [SSB, non-DRX] | Nokia |
| Below 10 GHz | Idle mode mobility | 1. NTN-TN inter-frequency cell reselection (Intra-RAT to NR TN) 2. NTN-TN inter-frequency cell reselection (Inter-RAT to LTE TN) | Earth fixed cell | ZTE |
| Below 10 GHz | Connected mode mobility | 1. RACH-less for soft satellite switch 2. RACH-based for hard satellite switch | Earth fixed cell | CMCC |
| Below 10 GHz | Connected mode mobility | NTN to NTN RACH-less HO | Earth fixed cell [Inter-satellite, without MG] | MTK |
| Below 10 GHz | Connected mode mobility | NTN to NTN time-based trigger CHO enhancements Note: not test earth-moving cell |  | LGE |
| Below 10 GHz | Connected mode mobility | [NTN to NTN location-based trigger CHO enhancements] Note: not test earth-moving cell |  | N/A |
| Below 10 GHz | Measurement procedure and accuracy | [Network verified UE location] |  | N/A |
| Note1 - Type 1: fully electronically-steered beam UEs - Type 2: fully mechanically-steered beam UEs Note2 - Case-1: Stationary UE for GSO - Case-2: Stationary UE for LEO - Case-3: Mobile UE for GSO |  |  |  |  |

# References

[1] R4-2404832, “Topic summary for [110bis][221] NR_NTN_enh,” 3GPP TSG-RAN WG4 Meeting #110b

[2] R4-2406292, “Ad-hoc minutes on RRM requirements for NR_NTN_enh,” 3GPP TSG-RAN WG4 Meeting #110b