3GPP TSG-RAN WG4 Meeting #110bis	R4-2406468

# Changsha, China, April 15 - April 20, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on test case for LTM based PSCell switch from FR2 to FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Perf |  |  |  |  |  | Date: |  |  | 2024-04-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce the test case for LTM PSCell cell switch: RACH based PScell cell switch from FR2 to FR2 intra-frequency cell switch |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the test case for RACH based PSCell switch from FR2 to FR2 intra-frequency cell switch |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case for R18 NR_Mob_enh2 RRM requirements in TS38.133 is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7.3.x.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

A.7.3.x.1	Intra-frequency RACH-based LTM PSCell switch from FR2 to FR2

A.7.3.x.1.1	Test Purpose and Environment

This test is to verify RACH-based LTM PSCell Switch requirements for the NR FR2-NR FR2 intra frequency cell switch specified in clause 8.20 for both with and without early TCI state activation..

A.7.3.x.1.2	Test Parameters

Three cells are deployed in the test, which are FR1 PCell (Cell 1), source FR2 PSCell (Cell 2) and target FR2 PScell (Cell 3) on the same frequency as the PSCell. Test configurations are given in table A.7.3.x.1.2-1. PSCell Cell switch delay is tested by using the parameters in table A.7.3.x.1.2-2, A.7.3.x.1.2-3 and A.7.3.x.1.2-4.

The test consists of 4 tests, and UE is required to pass one among Test 1A, Test 1B, Test 2A and Test 2B.

-	Test 1: for a UE supporting ltm-MAC-CE-JointTCI-r18 and/or ltm-MAC-CE-SeparateTCI-r18

-	Test 1A: for a UE supporting ltm-MAC-CE-JointTCI-r18.

-	Test 1B: for a UE supporting ltm-MAC-CE-SeparateTCI-r18 and does not support ltm-MAC-CE-JointTCI-r18.

-	Test 2: for a UE not supporting ltm-MAC-CE-JointTCI-r18 and ltm-MAC-CE-SeparateTCI-r18

-	Test 2A: for a UE supporting ltm-BeamIndicationJointTCI-r18.

-	Test 2B: for a UE supporting ltm-BeamIndicationSeparateTCI-r18 and does not support ltm-BeamIndicationJointTCI-r18.

The test consists of four successive time periods, with time durations of T1, T2, T3 and T4, respectively. No gap patterns are configured in the test case.

During T1, for Test 1A, 1B,2A and 2B:

-	Cell 1 on radio channel 1 is powered on., Cell 2 and Cell 3 on radio channel 2 are powered on.

-	UE establishes connections with the Cell 1 and Cell 2.

-	A measurement object is configured for the frequency of the Cell 3, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

-	T1 ends with UE reporting an L3 measurement result of Cell 3 to Cell 1.

At the start of T2, for Test 1A, 1B, 2A and 2B:

-	At the start of T2, UE is provided with LTM-Candidate-r18 for Cell 3

-	Joint TCI state configuration as defined in Table A.7.3.x.1.2-2 for Test 1A and Test 2A are provided.

-	Separate TCI state configuration as defined in Table A.7.3.x.1.2-2 for Test 1B and Test 2B are provided.

-	UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports on candidate cell (Cell 3) in PUCCH format 2.

-	T2 ends with UE reporting a valid L1-RSRP result of Cell 3.

During T3, for Test 1A and 1B:

-	At the start of T3, UE receives candidate cell TCI state activation MAC CE for Cell 3.

-	In Test 1A, CandidateTCI-State#1 is activated.

-	In Test 1B, CandidateTCI-State#1 and CandidateTCI-UL-State#1 is activated.

-	T3 ends 50ms after the candidate cell TCI state activation MAC CE transmission.

-	In Test 2A and 2B, T3 is skipped.

During T4, for Test 1A, 1B and 2:

-	The start of T4 is the instant when the last TTI containing LTM cell switch command MAC CE is sent by Cell 2 to the UE. In the cell switch command, Cell 3 is the target PSCell. In test 1A, CandidateTCI-State#2 is indicated. In test 1B, CandidateTCI-State#2 and CandidateTCI-UL-State#2 are indicated. In test 2A, CandidateTCI-State#1 is indicated. In test 2B, CandidateTCI-State#1 and CandidateTCI-UL-State#1 are indicated.

-	The UE shall start to transmit the PRACH to Cell 3 less than Tcmd + TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU ms after the start of T2, where the values of Tcmd, TLTM-RRC-processing, TLTM-processing and Tfirst-RS + TRS-proc and TLTM-IU are as listed in A.7.3.x.1.4.

-	T4 ends upon the reception of PRACH at Cell 3.

Table A.7.3.Y.1.2-1: Intra-frequency PScell switch from FR2 to FR2 test configurations

| Config | Description |
| --- | --- |
| 1 | Pcell: FR1 NR 15KHz SSB SCS, 10MHz bandwidth, FDD duplex mode Source PScell: FR2 NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target PScell: FR2 NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

able A.7.3.x.1.2-2: General test parameters for Intra-frequency cell switch from FR2 to FR2

| Parameter |  | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1A | Test 1B | Test 2A | Test 2B |  |
| Initial conditions | Active cell |  | Cell 1, Cell 2 |  |  |  |  |
|  | Neighbouring cell |  | Cell 3 |  |  |  | Cell 3 is the candidate cell |
| Final condition | Active cell |  | Cell 1, Cell 3 |  |  |  |  |
| A3-Offset |  | dB | -30 |  |  |  |  |
| Hysteresis |  | dB | 0 |  |  |  |  |
| Time To Trigger |  | s | 0 |  |  |  |  |
| Filter coefficient |  |  | 0 |  |  |  | L3 filtering is not used |
| DRX |  |  | OFF |  |  |  | DRX is not used |
| Access Barring Information |  | - | Not Sent |  |  |  | No additional delays in random access procedure. |
| Time offset between cells |  |  | 0.3 s |  |  |  | RTD between cells is less than CP |
| deriveSSB-IndexFromCell |  |  | Enabled |  |  |  |  |
| LTM-CSI-ReportConfig | L1-RSRP reporting period | slot | 320 |  |  |  | Periodic L1-RSRP reporting configured |
|  | nrOfReportedCells |  | n1 |  |  |  | Report candidate cell’s (Cell 3) L1-RSRP measurement results. |
|  | nrOfReportedRS-PerCell |  | n1 |  |  |  |  |
|  | spCellInclusion |  | N/A |  |  |  |  |
| ltm-DL-OrJointTCI-StateToAddModList | CandidateTCI-State#1 |  | DLorJoint TCI.State.0 | DLorJoint TCI.State.2 | DLorJoint TCI.State.1 | DLorJoint TCI.State.3 | As specified in clause A.3.16B. In test 1A and 1B, CandidateTCI-State#1 and/or CandidateTCI-UL-State#1 are configured for early TCI state activation. CandidateTCI-State#2 and/or CandidateTCI-UL-State#2 are configured for TCI state indication in cell switch command. In test 2A and 2B, CandidateTCI-State#1 and/or CandidateTCI-UL-State#1 are configured for TCI state indication in cell switch command. |
|  | #2 CandidateTCI-State#2 |  | DLorJoint TCI.State.1 | DLorJoint TCI.State.3 | N/A | N/A |  |
| ltm-UL-TCI-StatesToAddModList | CandidateTCI-UL-State#1 |  | N/A | UL TCI.State.0 | N/A | UL TCI.State.1 |  |
|  | CandidateTCI-UL-State#2 |  | N/A | UL TCI.State.1 | N/A | N/A |  |
| ltm-ConfigComplete |  |  | True |  |  |  | Candidate cell’s configuration is complete configuration |
| T1 |  | s | <3 |  |  |  |  |
| T2 |  | s | 0.2 |  |  |  |  |
| T3 |  | s | 0.1 |  |  |  |  |
| T4 |  | s | 0.2 |  |  |  |  |

Table A.7.3.Y.1.2-3: Cell specific test parameters for Pcell (cell 1)

| Parameter |  |  | Unit | Cell 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1~T4 |  |
| NR RF Channel Number |  |  |  | 1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |
| DRx Cycle |  |  | ms | Not Applicable |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |
| OCNG Patterns |  |  |  | OP.1 |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |
| SSB Configuration |  | Config 1 |  | SSB.1 FR1 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 | kHz | 15 kHz |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1 | kHz | 15 kHz |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |
| BWP |  | Initial DL BWP |  | DLBWP.0.1 |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |
| Note2 | Config 1 |  | dBm/SCS | -98 |  |
|  |  |  | dB | 4 | 4 |
|  |  |  | dB | 4 | 4 |
| SSB_RP | Config 1 |  | dBm/SCS | -94 | -94 |
| IoNote3 | Config 1 |  | dBm/ 9.36MHz | -64.59 | -64.59 |
| Propagation condition |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.7.3.x.1.2-4: Cell specific test parameters for NR FR2-FR2 Intra frequency cell switch test case

| Parameter |  |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 ~ T4 | T1 ~ T4 |
| NR RF Channel Number |  |  |  | 2 | 2 |
| Assumption for UE beamsNote 6 |  |  |  | Rough | Rough |
| AoA setup |  |  |  | [Setup 1] as defined in A.3.15 |  |
| Duplex mode |  |  |  | TDD |  |
| TDD configuration |  |  |  | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 100: NRB,c = 66 |  |
| BWP BW |  |  | MHz | 100: NRB,c = 66 |  |
| PDSCH Reference |  |  |  | SR.1.1 FDD |  |
| CORESET Reference Channel |  |  |  | CR.1.1 FDD |  |
| CP length |  |  |  | Normal |  |
| TRS configuration |  |  |  | TRS.1.1 FDD |  |
| OCNG Patterns |  |  |  | OP.1 |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |
| SSB Configuration |  |  |  | SSB.1 FR2 |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | 15 kHz |  |
| PUCCH/PUSCH subcarrier spacing |  |  | kHz | 15 kHz |  |
| PRACH configuration |  |  |  | FR2 PRACH configuration 1 |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |
| Note2 |  |  | dBm/SCS | -98 |  |
|  |  |  |  | -95 |  |
|  |  |  | dB | -0.64 | -0.64 |
|  |  |  | dB | 8 | 3 |
| SSB_RP |  |  | dBm/SCS | -90 | -95 |
|  |  |  | dBm/SCS | -87 | -92 |
| IoNote3 |  |  | dBm/ 9.36MHz | -61.41 | -66.41 |
|  |  |  | dBm/ 38.16MHz | -55.31 | -60.31 |
| Propagation condition |  |  | - | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

A.7.3.x.1.3	Test Requirements

The UE shall start to transmit the PRACH to Cell 3 in no later than DLTM from the beginning of time period T4.

The rate of correct cell switch observed during repeated tests shall be at least 90%.

NOTE:	The cell switch delay can be expressed as DLTM (=Tcmd + TLTM-interrupt), where:

Tcmd = THARQ + 3ms and is specified in clause 6.3.1.2, TLTM-interrupt is defined in clause 8.20.3 as TLTM-interrupt  = TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU ms.

-	Tfirst-RS + TRS-proc= 0 ms for Test 1A and 1B, Tfirst-RS + TRS-proc= 22 ms for Test 2A and 2B

-	TLTM-IU_is the uncertainty on transmitting the first uplink transmission on Cell 3

-	TLTM-RRC-processing =10ms if UE does not support [Early processing of an LTM candidate cell RRC configuration], otherwise TLTM-RRC-processing =0ms

-	TLTM-processing =10ms if the UE supports [faster LTM processing] capability and UE reports 10ms for FR2-to-FR2 cell switch in the capability

-	TLTM-processing =15ms if the UE supports [faster LTM processing] capability and UE reports 15ms for FR2-to-FR2 cell switch in the capability

-	TLTM-processing =20ms if the UE does not support [faster LTM processing] capability.