3GPP TSG-RAN4 Meeting #110-bis	R4-2406483

# Changsha, China, 15 – 19 April, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on requirements for satellite switch with re-sync |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2024-03-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - RAN4 agreed that scheduling restriction would apply in soft satellite switch when UE performs sync towards the target cell while connecting to the source cell. The scheduling restriction requirements are missing. - RAN4 agreed that soft satellite switch with re-sync, UE is allowed not to meet measurement requirements for other cells than the target cell after t-Start. The requirements applicability is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add scheduling restriction requirements for soft satellite switch. - Add requirements applicability for measurement of other cells than the target cell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for soft satellite switch are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1C.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The draftCR is based on Big draftCR R4-2403572 from RAN4#110. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

6.1C.3.2.3	Satellite switch delay for soft satellite switch with re-sync

The Satellite switch delay is from t-serviceStart to the time instance for the first UL transmissionon of the new PRACH for RACH-based case if TA timer is not running and there is no PUCCH SR or configured grant based PUSCH, dynamic grant based PUSCH, SR on PUCCH for RACH-less based case, if the UE supports the feature for soft satellite switch and the softSatelliteSwitch-Resync-NTN-r18 is enabled.

When intra-frequency soft switch to NR SAN cell is commanded,

the satellite switch time shall be less than Tsoft_switch

Tsoft_switch = max(t-service-t-seviceStart, Tsearch + T∆ + Tmargin) + TIU + Tprocessing  ms

Where:

-	Tsearch is the time required to search the target NR SAN cell when the target cell is not already known when the handover command is received by the UE. [If the target cell is known, then Tsearch = 0 ms.] If the target cell is an unknown intra-frequency cell and the target cell Es/Iot ≥ -2 dB, then Tsearch = Tfirst_SSB ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

-	T∆ is same as the one defined in section 6.1C.2.2.2.1.

-	Tprocessing is same as the one defined in section 6.1C.2.2.2.1.

-	Tmargin is same as the one defined in section 6.1C.2.2.2.1.

-

-	TIU is the interruption uncertainty in acquiring the first UL transmission resource, which can be a configured grant based PUSCH, dynamic grant based PUSCH, SR on PUCCH, according to NW configuration and scheduling, or PRACH if TA timer is not running and there is no PUCCH SR

-	Tfirst_SSB is is the time to the end of the first complete SSB burst of target satellite indicated by

-	SSB of the source satellite serving cell + ssb-TimeOffset + propagation delay difference, where the propagation delay difference is the difference between the propagation delays of the serving satellite and the target satellite.

[In the interruption requirement a cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds before UE starts synchronizing with target satellite. Otherwise it is unknown. Relevant cell identification requirements are described in Clause 9.2C.5 for intra-frequency handover.]

During the time period from t-seviceStart to t-service, scheduling restriction as defined in clause 9.2C.5.3 is allowed, with the exception that the locations of SSB symbols of target satellite where scheduling restriction applies are determined by the SSB of the source satellite serving cell + ssb-TimeOffset + propagation difference.

UE is allowed to skip measurements for other cells and satellites than the target satellite and source satellite from t-seviceStart to the satellite switch completion.

<End of Change 1>