3GPP TSG-RAN WG4 Meeting # 110-bis					R4-2406561

China, Changsha, April 15 – April 19, 2024

Title:	TP to TR 38.899: Addition of PC2 and PC1.5 for CA_n40A-n77A

Source:	Nokia, nbn

Agenda item:	5.17.2

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.899 to add PC2 and PC1.5 for CA_n40A-n77A. CA_n40A-n77A PC3 is completed as a fallback and PC2 single UL is already enabled for n77A hence this TP modifies an existing entry in the TR. CA_n40B-n77A supports n77A PC2, so study of n77A PC2 has been completed, which means focus of this TP is to check impact of n40A PC2 + PC1.5 and n77A PC1.5.

************************************* Start of TP*****************************************

## 5.10	CA_n40-n77

### 5.10.1	Configurations

Table 5.10.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n40A-n77A | n40A8,9 n77A8,9 CA_n40A-n77A8 | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 904, 100 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.10.2	Maximum output power

Table 5.10.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n40-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 5.10.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.10.3.1	Power class 2 and 1.5 case a, b, c, d

Based on TS 38.101-1 Table 7.3A.4-1, there is no UL harmonic interference issue for this band combination.

Based on TS 38.101-1 Table 7.3A.4-4, there is harmonic mixing issue for n77 to n40 this is treated under the single band exceptions.

Based on TS 38.101-1 Table 7.3A.5-1, there is no IMD issues for this band combination.

Based on TS 38.101-1 Table 7.3A.6-1, there is cross band isolation issues for n77 to n40 this is treated under the single band exceptions.

#### 5.10.3.2	Power class 2 for single uplink n77

-		The 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n40.

-		 The 2nd, 3rd, 4th, and 5th order harmonic mixing do not fall into Rx frequencies of n40.

-		n40 DL suffers UL2/DL3 harmonic mixing from n77.

-		 Cross band isolation of n77 UL falls into n40 DL.

Therefore, MSD for cross band isolation and harmonic mixing should be defined.

Table 5.10.3.2-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC2 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n77 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2395 | 10 | 6.5 | >ACLR2 |
| n77 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2350 | 100 | 1.2 | >ACLR2 |

Table 5.10.3.2-2: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n40 | 20 | 30 | 50 (RBstart=0) | 10 | 13.2 | NOTE 3 | UL2/DL3 |
| n77 | n40 | 20 | 30 | 50 (RBstart=0) | 100 | 4.4 | NOTE 3 | UL2/DL3 |

#### 5.10.3.3	Power class 1.5 for single uplink n77

Similar as for PC2 MSD should be defined.

Table 5.10.3.3-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC1.5 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n77 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2395 | 10 | 9.0 | >ACLR2 |
| n77 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2350 | 100 | 2.2 | >ACLR2 |

Table 5.10.3.3-2: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC1.5 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n40 | 20 | 30 | 50 (RBstart=0) | 10 | 16.1 | NOTE 3 | UL2/DL3 |
| n77 | n40 | 20 | 30 | 50 (RBstart=0) | 100 | 6.7 | NOTE 3 | UL2/DL3 |

#### 5.10.3.4	Power class 2 and 1.5 for single uplink n40

According to previous co-existence studies there are no issues.

### 5.10.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

************************************* End of TP*****************************************