# 3GPP TSG-RAN WG4 Meeting #110-bis                                                          R4-2406563

# Changsha, China, 15 April – 19 April, 2024

Agenda Item:	5.17.2

Source: 	CMCC, ZTE Corporation, Huawei, HiSilicon, Murata Manufacturing Co Ltd.

Title: 	(HPUE_FR1_TDD_NR_CADC_SUL_R18) TP for TR 38.899 to introduce PC2 and PC1.5 CA_n39-n41

Document for:	Approval

- Introduction

The HPUE request of CA_n39-41 have been approved.

In this contribution, we will analyze the IMD studies for these band combinations.

- Reference

- . TR38.899, High power UE for FR1 NR inter-band CA/DC or NR SUL band combination with y (1<y<=6) bands DL and x (x=1, 2) bands UL and power class m (m<3) and high power on TDD band(s), 0.8.0

- TP to TR 38.899

## <<Start of Change for TR 38.899>>

5.x	CA_n39-n41

5.x.1	Configurations

Table 5.x.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier |  | NR Band |  | Channel bandwidth (MHz) |  |  | Bandwidth combination set |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n39A-n41A | n398 n418,9 CA_n39A-n41A8 |  | n39 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n41 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n39 |  | See n39 channel bandwidths in Table 5.3.5-1 |  |  | 4 and 5 |  |
|  |  |  | n41 |  | See n41 channel bandwidths in Table 5.3.5-1 |  |  |  |  |
| CA_n39A-n41C | n398 n418,9 CA_n41C8 CA_n39A-n41A8 CA_n39A-n41C8 | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | CA_n41C_BCS0 |  |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS4 and 5 |  |
| NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination |  |  |  |  |  |  |  |  |  |

5.x.2	Maximum output power

Table 5.x.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n39-n41 | CA power class | Carrier n39 power class | Carrier n41 power class |
| --- | --- | --- | --- | --- |
| CA_n39A-n41A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |
| CA_n39A-n41C | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

5.x.3	REFSENS requirements

PC2 UL single carrier

For PC3 CA_n39A-n41A, due to simultaneous Rx/Tx operation is supported, there are two types of MSD for this band combination, which are:

1: cross-band isolation MSD

2: Harmonic mixing MSD, where:

- 3rd harmonic of band n41 UL may fall into 4th harmonic of band n39 DL

- 4th harmonic of band n39 UL may fall into 3rd harmonic of band n41 DL.

For PC2 CA_n39A-n41A, by using the same co-existence study as PC3, and due to either PC2 single band n39 or PC2 single band n41 is supported, the above two types MSD should be defined.

PC2 UL CA_n41C

Due to PC2 UL CA_n41C are supported, the IMDs issue caused by intra-band UL contiguous CA_n41C can be found in table 5.x.3-1:

Table 5.x.3-1: Co-existence studies for Uplink Intra-Band Contiguous CA

| Configuration | Channel BW | Minimum channel separation | Maximum channel separation | Minimum frequency | Maximum frequency |  |
| --- | --- | --- | --- | --- | --- | --- |
| Data | 10 | 10 | 190 | 2496 | 2690 | - |
| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| Frequency | 2496 | 2516 | 2686 | 2690 | 2670 | 2500 |
| 2nd order IMD products | IfU1L-fU2LI | IfU1L-fU3LI | fU1L + fU2L | fU1H+fU2H |  |  |
| Interference ranges | 20 | 190 | 5012 | 5360 |  |  |
| 3rd order IMD products | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Interference ranges | 2306 | 2880 | 7508 | 8050 |  |  |
| 4th order IMD products | I2*fU1L-2*fU2LI | I2*fU1H-2*fU3H I | 3*fU1L-fU3L | 3*fU1H-fU3H | 3*fU1L+fU2L | 3*fU1H+fU2H |
| Interference ranges | 40 | 380 | 4802 | 5570 | 10004 | 10740 |
| 5th order IMD products | I3*fU1L-2*fU3LI | I3*fU1H-2*fU3H I | 4*fU1L-fU3L | 4*fU1H-fU3H | 4*fU1L+fU2L | 4*fU1H+fU2H |
| Interference ranges | 2116 | 3070 | 7298 | 8260 | 12500 | 13430 |
| 6th order IMD products | I3*fU1L-3*fU2LI | I3*fU1H-3*fU3H I | 4*fU1L-2*fU3L | 4*fU1H-2*fU3H | 5*fU1L-fU3L | 5*fU1H-fU3H |
| Interference ranges | 60 | 570 | 4612 | 5760 | 9794 | 10950 |
| 7th order IMD products | I4*fU1L-3*fU3LI | I4*fU1H-3*fU3HI | 5*fU1L-2*fU3L | 5*fU1H-2*fU3H | 6*fU1L-fU3L | 6*fU1H-fU3H |
| Interference ranges | 1926 | 3260 | 7108 | 8450 | 12290 | 13640 |

It can be found that there are no co-existence issues caused by CA_n41C impact band n39 DL.

PC2 UL CA_n39A-41A

For PC2 UL CA_n39A-n41A, except the cross band isolation and harmonic mixing MSD issue as mentioned as above, additional IMD MSD need to be checked. However, due to both n39 and n41 are TDD bands, there are no IMD issue due to the UL and DL will not operate at the same time.

PC2 UL CA_n39A-41C

For PC2 UL CA_n39A-n41A, similar with PC2 UL CA_n39A-n41A, the cross band isolation and harmonic mixing MSD issue have already been included in the PC2 UL single carrier.

For the impacts of PC2 UL CA_n41C, the analysis are included in the PC2 UL_CA_n41C as above.

For the triple beat IMD issue, due to both n39 and n41 are TDD bands, there are no IMD issue due to the UL and DL will not operate at the same time. Therefore, no additional triple beat IMD MSD are foreseen to be be defined to support PC2 UL CA_n39A-n41C.

To summarize, to support PC2 power class for the UL configuration of single carrier, CA_n41C, CA_n39A-n41A and CA_n39A-n41C, only the cross-band isolation MSD and Harmonic mixing MSD should be defined, which are given in the following sub-clauses. There are no IMD issue caused by UL CA_n39-n41 for the CA_n39-n41 band combination.

5.x.3.1	Power class 2 for case a

For PC2 case a, since the power configuration for each band is 23dBm, therefore comparing with PC3 CA_n39A-n41A, no additional cross-band isolation MSD or harmonic mixing MSD are expected for this PC2 CA_n39A-n41A with 1 uplink carrier.

5.x.3.2	Power class 2 for case b

For PC2 case b, due to the power class for n39 is still kept as PC3(i.e. 23dBm), comparing with PC3 CA_n39A-n41A, there are no additional cross-band isolation MSD or harmonic mixing MSD caused by PC3 UL n39 are expected.

However, due to the power class for n41 is increased to 26dBm, therefore comparing with PC3 CA_n39A-n41A,  cross-band isolation MSD or harmonic mixing MSD caused by UL PC2 n41 shall be defined.

the harmonic mixing MSD caused by UL PC2 n39 are proposed in table 5.x.3.2-1.

Table 5.x.3.2-1. Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n41 | n39 | 10 | 15 | 25 (RBstart=0) | 5 | 6.1 | NOTE X | UL3/DL4 |
| n41 | n39 | 10 | 15 | 25 (RBstart=0) | 40 | 1.5 | NOTE X | UL3/DL4 |
| NOTE X: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

Similarily, the cross band isolation caused by UL PC2 n41 is proposed in table 5.x.3.2-2.

Table 5.x.3.2-2. Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC2 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n41 | n39 | 2546 | 100 | 30 | 270 (RBstart=3) | 1917.5 | 5 | 2.7 | >ACLR2 |

NOTE： Cross-band isolation between n41C UL (PC3 and PC2) may need to be evaluated and specified, pending on the general MSD framework for cross-band interference between intra-band contiguous UL CA and the victim DL band.

5.x.3.3	Power class 2 for case c

For PC2 case c, due to the power class for n41 is still kept as PC3(i.e. 23dBm), comparing with PC3 CA_n39A-n41A, there are no additional cross-band isolation MSD or harmonic mixing MSD caused by PC3 UL n41 are expected.

However, due to the power class for n39 is increased to 26dBm, comparing with PC3 CA_n39A-n41A, cross-band isolation MSD or harmonic mixing MSD caused by UL PC2 n39 shall be defined.

Therefore, the harmonic mixing MSD caused by UL PC2 n39 are proposed in table 5.x.3.3-1.

Table 5.x.3.3-1. Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n39 | n41 | 5 | 15 | 25 (RBstart=0) | 5 | 10.6 | NOTE X | UL4/DL3 |
| n39 | n41 | 5 | 15 | 25 (RBstart=0) | 100 | 4.9 | NOTE X | UL4/DL3 |
| NOTE X: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

Similarily, the cross band isolation caused by UL PC2 n39 is proposed in table 5.x.3.3-2.

Table 5.x.3.3-2. Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC2 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n41 | 1900 | 40 | 15 | 216 (RBstart=0) | 2501 | 10 | 4.7 | >ACLR2 |

#### 5.x.3.4	Power class 2 for case d

For PC2 case d, the power class for both bands n39 and n41 are increased to 26dBm, therefore comparing with PC3 CA_n39A-n41A, cross-band isolation MSD or harmonic mixing MSD caused by UL PC2 n39 or PC2 n41 shall be defined.

Due to the cross-band isolation MSD or harmonic mixing MSD are the situation of one single band impact another single band, therefore the cross-band isolation MSD or harmonic mixing MSD caused UL PC2 n39 or PC2 n41 are already included in sub-clause 5.x.3.2 and sub-clause 5.x.3.3.

5.x.3.5	Power class 1.5 for single uplink n41

Since the MOP for n41 is increased to 29dBm, harmonic mixing MSD and cross band isolation MSD caused by UL PC1.5 n41 should be defined.

The proposed value for PC1.5 UL CA MSD can be found in Table 5.x.3.5-1 and Table 5.x.3.5-2:

Table 5.x.3.5-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC1.5 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n41 | n39 | 10 | 15 | 25 (RBstart=0) | 5 | 10.5 | NOTE X | UL3/DL4 |
| n41 | n39 | 10 | 15 | 25 (RBstart=0) | 40 | 3.0 | NOTE X | UL3/DL4 |
| NOTE X: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

Table 5.x.3.5-2: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC1.5 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n41 | n39 | 2546 | 100 | 30 | 270 (RBstart=3) | 1917.5 | 5 | 6.7 | >ACLR2 |

5.x.4	∆TIB and ∆RIB values

Void

## <<End Change for TR 38.899>>