# 3GPP TSG-RAN WG4 Meeting #110-bis                                                          R4-2406564

# Changsha, China, 15 April – 19 April, 2024

Agenda Item:	5.17.2

Source: 	CMCC, Huawei, HiSilicon, ZTE Corporation, Murata Manufacturing Co Ltd.

Title: 	(HPUE_FR1_TDD_NR_CADC_SUL_R18) TP for TR 38.899 to introduce PC2 and PC1.5 CA_n28A-n40A

Document for:	Approval

- Introduction

The HPUE request of CA_n28A-n40A have been approved.

In this contribution, we will analyze the MSD studies for these band combinations.

- Reference

- . TR38.899, High power UE for FR1 NR inter-band CA/DC or NR SUL band combination with y (1<y<=6) bands DL and x (x=1, 2) bands UL and power class m (m<3) and high power on TDD band(s), 0.8.0

- TP to TR 38.899

## <<Start of Change for TR 38.899>>

5.x	CA_n28A-n40A

5.x.1	Configurations

Table 5.x.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration |  | Uplink CA configuration or single uplink carrier |  | NR Band |  | Channel bandwidth (MHz) |  | Bandwidth combination set |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n28A-n40A |  | n408,9 CA_n28A-n40A8 |  | n28 |  | 5, 10, 15, 20 |  | 0 |  |
|  |  |  |  | n40 |  | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |  |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20, 25, 30 |  | 1 |  |
|  |  |  |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  |  |  | n28 |  | See n28 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  |  |  | n40 |  | See n40 channel bandwidths in Table 5.3.5-1 |  |  |  |
| NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination |  |  |  |  |  |  |  |  |  |

5.x.2	Maximum output power

Table 5.x.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n28-n40 | CA power class | Carrier n28 power class | Carrier n40 power class |
| --- | --- | --- | --- | --- |
| CA_n28-n40 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

In addition, the power class for PC2 and PC1.5 n40 single carrier for this band combination.

5.x.3	REFSENS requirements

For PC3 CA_n28A-n40A, there are harmonic mixing MSD for this band combination:

band n41 UL may fall into 3rd harmonic of band n28 DL.

5.x.3.1	Power class 2 case a

For PC2 case a, since the power configuration for each band is 23dBm, therefore comparing with PC3 CA_n28A-n40A, no additional harmonic mixing MSD are expected for this PC2 CA_n28A-n40A with 1 uplink carrier.

5.x.3.2	Power class 2 case b

For PC2 case b, due to the MOP for n28 is still kept as 23dBm. Since the MOP for n40 is increased to 26dBm, harmonic mixing MSD caused by UL PC2 n40 shall be defined.

The proposed value for PC2 UL CA MSD can be found in Table 5.x.3.2-1:

Table 5.x.3.2-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n40 | n28 | 5 | 15 | 25 (RBstart=0) | 5 | 40.8 | NOTE 7 | UL1/DL3 |
| n40 | n28 | 20 | 15 | 100 (RBstart=0) | 20 | 33.3 | NOTE 7 | UL1/DL3 |
| NOTE 7: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |

5.x.3.3	Power class 1.5 for single uplink n40

Since the MOP for n40 is increased to 29dBm, harmonic mixing MSD caused by UL PC1.5 n40 should be defined.

The proposed value for PC1.5 UL CA MSD can be found in Table 5.x.3.3-1:

Table 5.x.3.3-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC1.5 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n40 | n28 | 5 | 15 | 25 (RBstart=0) | 5 | 43.8 | NOTE X | UL1/DL3 |
| n40 | n28 | 20 | 15 | 100 (RBstart=0) | 20 | 36.3 | NOTE X | UL1/DL3 |
| NOTE X: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |

5.x.4	∆TIB and ∆RIB values

Void

## <<End Change for TR 38.899>>