3GPP TSG-RAN WG4 Meeting #110bis	R4-2406566

# Changsha , China, 15th – 19th April 2024

Title: 	WF on PC2 and PC1.5 indication for combinations in the tables

Agenda Item:	5.17

Source: 	China Telecom

Document for:	Approval

# Way forward

## WF on PC2/PC1.5 indications for higher order inter-band DL combination

During the previous meeting, the following agreements were captured in the way forward [1]: Option 1: Introduce a general note. - For single band UL in higher order DL CA combinations, introduce a general note for 3 or more band DL CA configurations stating that “PC2 or PC1.5 for single UL can be supported if it has been specified in all the fallback combinations”. - For 2UL HPUE indication in higher order combinations, introduce a general note for 4 or more band DL CA configurations stating that “PC2 for 2-band UL can be supported if it has been specified in all the fallback combinations”. Option 2: Maintain the current PC2/PC1.5 introduction process with explicit notes and encourage proponent companies for higher order inter-band DL combinations to promptly complete the PC2/PC1.5 note additions if they have been specified for all the fallback combinations. Option 3: for single band UL in higher order DL combinations, a general note for [2] or more DL CA configurations stating that PC2 or PC1.5 for single UL can be supported also if the UE complies with - an allowed exception for an UL/DL configuration corresponding to the DL CA configuration with the UL power limited to that of a lower power class e.g. PC2 or PC3 for which an exception is specified or - the standard CA REFSENS requirement also for the higher power class PC2/PC1.5 irrespective of any allowed exception.

This meeting, the following agreement has been reached as way forward:

Agreement:

- Combine Option 1 and Option 2 together to mitigate the power class disparity issue between higher order inter-band DL combination and its fallback combinations.

  - How to implement the agreements can be further discussed.

- Leave Option 3 for further discussion in the topic thread [140]

## WF on inter-band combination with and without intra-band configurations in DL

Example of same inter-band combination with and without intra-band configurations in DL.

| Same inter-band combination composed of n1 and n78 |  |  |
| --- | --- | --- |
| DL Configuration | UL Configuration | Comment |
| CA_n1A-n78A | n18 n788,9 CA_n1A-n78A8 | Lowest order |
| CA_n1A-n78(2A) | CA_n78(2A) CA_n1A-n78A | Higher order |
| CA_n1A-n78C | CA_n78C CA_n1A-n78A | Higher order |
| CA_n1(2A)-n78A | - | Higher order |

Continue to discuss the following proposals:

- Proposal 1: Introduce a general note for Inter-band CA combination with intra-band configurations in DL.

- Proposal 2: Support to maintain the current approach of using Notes in the BC configuration tables.

- Proposal 3: Improve the wording for the description of the issue and avoid using controversial words such as “same inter-band combination”, “inherit (from the lowest order combination)”.

- Proposal 4: When adding intra-band CA to a DL FDD band, new MSD analysis/requirements may be needed for PC2 FDD UL, unless the evaluation is already completed by the other HPUE FDD basket WI.

- Proposal 5: Introduce a description in general section to clarify the band combination indications.

# Reference

[1] R4-2403884, “WF on PC2 and PC1.5 indications in BC configuration tables”, Apple, 3GPP TSG RAN WG4 Meeting #110, Athens, Greece, February 26th – March 1st, 2024.

[2] R4-2404181, “On PC2 and PC1.5 indications in BC configuration tables”, Apple, 3GPP TSG RAN WG4 Meeting #110bis, Changsha, China, April 15th – 19th, 2024.

[3] R4-2404884, “Discussion on HPUE indication in band combination configuration tables”, Huawei, HiSilicon, 3GPP TSG RAN WG4 Meeting #110bis, Changsha, China, April 15th – 19th, 2024.

[4] R4-2405263, “Topic summary for [110bis][110] HPUE_Basket_inter-CA_SUL”, China Telecom, 3GPP TSG RAN WG4 Meeting #110bis, Changsha, China, April 15th – 19th, 2024.