3GPP TSG-RAN WG4 Meeting # 110-bis											R4-2406573

Changsha , China, 15th – 19th April 2024

Title: 	WF on A-MPR for FDD PC2 HPUE

Agenda Item:	5.19

Source: 	China Unicom

Document for:	Approval

1. Background

During the RAN4-110 meeting, the issues related with FDD PC2 A-MPR were discussed, and the agreements were captured in [1].

<Topic 1 A-MPR>

<Sub-topic 1-1 HPUE with dual Tx>

<Background>:

WF from [1]:

The handling of HPUE with dual Tx requires special attention. When NR-U PC3 was specified, a separate table was introduced for dual Tx. While this might offer the clearest solution in terms of explicitly defined values it creates considerable work and more cluttering.

Companies are therefore asked to provide their views and proposals to efficiently solve the challenges with specifying A-MPR for dual Tx.

<Way forward>:

- Option 1: Limit the dual-Tx UE A-MPR to be at most 3 dB higher than PC3, while allowing the extra [2] dB relaxation within this boundary.

- Option 2: Introduce dedicated A-MPR for PC2 with dual Tx (as has been done for other NS flags such as NS_60).

- Option 3:

  - Void Note 7 from Table 6.2.3.1-1,

  - Limit the single-Tx and dual-Tx PC2 FDD A-MPR to be at most 3dB higher than PC3 A-MPR,

  - Capture the dual-Tx A-MPR additional relaxation for each affected NS flag.

  - We invite companies to study how to best capture the dual-Tx additional relaxation:

    - separate table from PC2 single Tx?

    - footnote to single-Tx PC2 A-MPR for each regions?

    - any other approach not precluded.

- Other Options are not precluded.

<Agreement>:

- Limit the single-Tx and dual-Tx PC2 FDD A-MPR to be at most 3dB higher than the PC3 A-MPR.

- FFS how to capture the dual-Tx A-MPR in the spec.

<Sub-topic 1-2 NS_07 (A-MPR for n13 PC2)>

<Way forward>:

- There are two candidate results for NS_07 (A-MPR for n13 PC2) :

- Option 1: Define 1Tx PC2 A-MPR for NS_07 as proposed in “R4-2318750 On NS_07 for PC2, Apple, RAN4#109”.

- Option 2: Agree on the captured regions and A-MPR for single Tx from WF R4-2403629.

- Aim to finalize the requirements in RAN4-111.

<Sub-topic 1-3 A-MPR and related issues for n28 PC2>

- A-MPR and related issues for n28 PC2 is discussed in “R4-2406708 WF on band n28 full band duplexer”.

<Sub-topic 1-4 NS_46 (A-MPR for n7 PC2)>

<Agreement>:

- Tentatively agree the A-MPR regions and A-MPR values in Table 1 and Table 2 below for NS_46 for PC2 operation:

Table 1: A-MPR regions for NS_46 for PC2

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | 2550 ≤ FC ≤ 2565 | ≥ 7.92 | ≥ 5.4 | A6 |
|  |  | ≥ 7.92 | < 2.16 | A5 |
| 15 MHz | [2560.5] ≤ FC ≤ 2562.5 | ≥ 11.16 | ≥ 5.4 | A6 |
|  |  | ≥ 11.16 | < 5.4 | A5 |
| 20 MHz | [2552] ≤ FC ≤ 2560 | ≥12.6, < 15.3 | > 10.8 | A6 |
|  |  | ≥ 15.3 | > 8.64 | A9 |
|  |  | ≥ 14.4 | ≤ 8.64 | A10 |
| 25 MHz | [2534.5] ≤ FC ≤ 2557.5 | < 14.4 | >max (0, 12*SCS*RBend - 2.7) | A5 |
|  |  | ≥ 14.4, <18.9 | > 12.6 | A6 |
|  |  | ≥ 18.9 | > 16.02 | A9 |
|  |  | ≥ 18.9 | ≤ 16.02, ≥ 9.72 | A6 |
|  |  | ≥ 17.64 | < 9.72 | A10 |
| 30 MHz | 2515 ≤ FC ≤ 2555 | ≥0, <1.44 | >0 | A4 |
|  |  | ≥1.44, <13.5 | >max (0, 12*SCS*RBend -1.8) | A5 |
|  |  | ≥13.5, <19.8 | >11.52 | A6 |
|  |  | ≥19.8, <25.92 | >6.3 | A7 |
|  |  | ≥25.92 | >0 | A8 |
|  |  | > 20.7 , < 25.92 | ≤ 6.3 | A10 |
| 35 MHz | 2517.5 ≤ FC ≤ 2552.5 | ≥0, <3.42 | >0 | A4 |
|  |  | ≥3.42, <15.84 | >max (0, 12*SCS*RBend - 3.06) | A5 |
|  |  | ≥15.84, <22.68 | >12.6 | A6 |
|  |  | ≥22.68, <28.8 | >9.0 | A7 |
|  |  | ≥28.8 | >0 | A8 |
|  |  | > 24.3 , < 28.8 | ≤ 9.0 | A10 |
| 40 MHz | 2520 ≤ FC ≤ 2550 | ≥0, < 4.5 | >0 | A4 |
|  |  | ≥ 4.14, <5.04 | >max (0, 12*SCS*RBend - 4.5) | A11 |
|  |  | ≥5.04, <18 | >max (0, 12*SCS*RBend - 4.5) | A5 |
|  |  | ≥18, <25.74 | >13.5 | A6 |
|  |  | ≥25.74, <32.4 | >12.6 | A7 |
|  |  | ≥32.4 | >0 | A8 |
|  |  | > 27.9 , < 32.4 | ≤ 12.6 | A10 |
| 50 MHz | 2525 ≤ FC ≤ 2545 | ≥0, <9 | >0 | A4 |
|  |  | ≥9, <21.6 | >max (0, 12*SCS*RBend - 7.2) | A5 |
|  |  | ≥21.6, <31.5 | >18 | A6 |
|  |  | ≥31.5, <39.6 | >16.2 | A7 |
|  |  | ≥39.6 | >0 | A8 |
|  |  | > 33.84, < 39.6 | ≤ 16.2 | A10 |

Table 2: A-MPR for NS_46 for PC2

| Modulation/Waveform |  | A4 | A5 | A6 | A7 | A8 | A9 | A10 | A11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 6.5 | 2.5 | 3.5 | 7 | 10 | 6.5 | 3.0 | 5.0 |
|  | QPSK | 6.5 | 2.5 | 3.5 | 7 | 10 | 6.5 | 3.0 | 5.0 |
|  | 16 QAM | 6.5 | 2.5 | 3.5 | 7 | 10 | 6.5 | 3.5 | 5.0 |
|  | 64 QAM | 6.5 | 2.5 | 3.5 | 7 | 10 | 6.5 | 3.5 | 5.0 |
|  | 256 QAM | 6.5 |  |  | 7 | 10 | 6.5 |  | 5.0 |
| CP-OFDM | QPSK | 6.5 | 3.5 | 5.5 | 8 | 11 | 8 | 4.5 | 6.5 |
|  | 16 QAM | 6.5 | 3.5 | 5.5 | 8 | 11 | 8 | 4.5 | 6.5 |
|  | 64 QAM | 6.5 | 3.5 | 5.5 | 8 | 11 | 8 | 4.5 | 6.5 |
|  | 256 QAM | 6.5 |  |  | 8 | 11 | 8 |  |  |

- Aim to finalize the requirements in RAN4-111

<Sub-topic 1-5 NS_12, NS_13, NS_14, NS_15 (A-MPR for n26 PC2)>

<Background [1]>:

Set a starting point for NS_12, NS_13, NS_14, NS_15 by merging the A-MPR regions and values in R4-2402214 and R4-2315374. New inputs can be still taken into consideration.

<Way forward>:

- Aim to finalize the requirements in RAN4#111.

Reference

[1] R4-2403629	WF on HPUE for FDD bands	China Unicom, Skyworks		RAN4-110

[2] R4-2405264	Topic summary for [110bis][111] HPUE_Basket_FDD	Moderator (China Unicom)	RAN4-110-bis