3GPP TSG-RAN WG4 Meeting #110bis	R4-2406583

Changsha, China, 14-19 April, 2024

Title: 	WF on HPUE UE for UL CA and EN-DC

Agenda Item:	9.1.2

Source: 	Samsung

Document for:	Approval

1. General aspect

1.1 New proposed BC configurations for intra-band CA which are not included in the WI objectives

Agreement based online and further clarification in AH meeting:

- For the lower order power class cases (PC2 and PC3) of same PC1.5 band combination

  - Have the Rel-19 basket WIs to cover the necessary requirements

    - FFS on whether to have independent basket WIs to cover the requirements in RAN plenary

- For the new UL intra-band CA configurations for PC1.5

  - Have the Rel-19 basket WIs to cover the necessary requirements

    - FFS on whether to have independent basket WIs to cover the requirements in RAN plenary

  - Discuss the requirements for those new UL intra-band CA configurations for PC1.5 after the general requirements are finalized in this WI

1.2 SAR solution

Way forward:

FFS on following options SAR solution with consideration the P-MPR is applicable.

-	Option 1: Duty-cycle solution is not considered for any HPUE scenarios in this WI

-  Option 2：Duty-cycle solution is considered for all HPUE scenarios in this WI

-  Option 3：Duty-cycle solution is considered for all HPUE scenarios in this WI except for “increasing UE transmission power limit”

-	Option 4: Duty-cycle solution is considered for limited scenarios, further study which scenarios to consider

2. PC1.5 for intra-band contiguous and non-contiguous UL CA

NOTE 1: Intra-band contiguous UL CA includes cases with and without UL MIMO, Intra-band non-contiguous UL CA includes only w/o UL MIMO

NOTE 2: Unless otherwise stated, the issues discussed below are applicable for both contiguous and non-contiguous CA

2.1 Assumed UE architectures for MPR/A-MPR evaluation

-  Option 1: (Xiaomi, Samsung, Huawei)

| Architecture | Description | Indicated capability | Support UL MIMO | Applicable cases | Note |
| --- | --- | --- | --- | --- | --- |
| #1 | 2x26 dBm PA + 2 LO with 100MHz BW | dualPA-Architecture | No | C/NC CA | No Frequency Separation limitation |
| #2 | 2x26 dBm PA + 1 LO with 200MHz BW | TxD | Yes | C/NC CA | Frequency Separation ≤ 200MHz For NC CA, FFS whether BW Gap size＜CC1 +CC2 CBW |

NOTE: support UL MIMO here doesn’t mean UE has to support the feature, it depends on UE capability reporting

- Option 2: (Apple, vivo, Skyworks, Meta)

| Architecture | Description | Indicated capability | Support UL MIMO | Applicable cases | Note |
| --- | --- | --- | --- | --- | --- |
| #1 | 2x26 dBm PA + 2 LO with 100MHz BW | dualPA-Architecture | No | NC CA | No Frequency Separation limitation |
| #2 | 2x26 dBm PA + 1 LO with 200MHz BW | TxD | Yes | Contiguous CA |  |

Way forward:

Both options are considered for next meeting discussion, FFS on down selection of UE architectures

- Prioritize UL contiguous CA

2.2 General consideration for MPR/PCMAX/PCMAX,c

Way forward:

-	For PC1.5, intra-band contiguous ULCA with UL MIMO can share the MPR requirements of intra-band contiguous ULCA without UL MIMO and with dual Tx (TxD)

-  Study and if necessary, specify the MPR requirements for both small and large form factor UEs, i.e. minimum antenna isolation = 10 or 20dB

-  For R19 PC1.5 intra-band non-contiguous /contiguous ULCA with two 26dBm PAs and one PA per CC, consider PCMAX,,c limitation (i.e., 26dBm) for each component carrier, which have impact on MOP, PCMAX tolerance and MPR/A-MPR evaluation

-  FFS For R19 PC1.5 intra-band non-contiguous /contiguous ULCA with two 26dBm PAs and one PA per CC, the PCMAX is modified as follows to account for RB BW imbalances

PCmax=10*log(10^(26/10) + 10^((26-10*log(LCRB1*SCS1/(LCRB2*SCS2)))/10))

-	Other options are not precluded

2.3 MOP tolerance

Agreement: +2/-3 dB, in addition adding note into MOP table to reflect PC1.5 is achieved by dualPA-Architecture and/or TxD.



  - PCMAX tolerance

Way forward: The following values as starting point

- TLOW=3dB

- THIGH=2dB

2.5 ACLR

Agreement: NR ALCR as 31dB, UTRA ALCR not needed for the example combos

3. 2-band Inter-band UL NR-CA/EN-DC with 2Tx and/or 3Tx

Note 1: Only PC3 is considered for LTE FDD in EN-DC, per the WID

Note 2: Only PC1.5 is considered for inter-band NR-CA, per the WID

  - Applicability of 3Tx requirements for FWA and handheld UE

Agreement online:

- 3Tx NR-CA configurations and corresponding requirements defined in Rel-18 are applied to handheld UE in Rel-19.

- 3Tx EN-DC configurations and corresponding requirements defined in Rel-18 are applied to handheld UE in Rel-19

3.2 MOP tolerance

Agreement: For 2Tx PC1.5 MOP tolerance, reuse the 3Tx PC1.5 MOP tolerance, i.e., +2/-3dB

3.3 PCMAX tolerance

Agreement: No new requirement required. When 3Tx PC1.5 was introduced in Rel-18, the configured transmitted power clause referred to 2Tx configured transmitted power clause.

3.4 Configuration for 2Tx inter-band NR-CA/EN-DC

Way forward:

- For 2Tx inter-band NR-CA, strive to define general requirements in a band-combination configuration agnostic way

- For 2Tx inter-band EN-DC, strive to define general requirements in a band-combination configuration agnostic way, further check whether there is demand on FDD-FDD (PC2 in total)

Note: General requirements here do not include band-combination specific requirements like MSD.

3.5 Configuration for 3Tx inter-band NR-CA/EN-DC

Way forward:

- No restriction on the power/MIMO configurations of each band in the band combination.

- Only PC3 is considered for LTE FDD in EN-DC, per the WID

- Both PC2 and PC1.5 are considered for inter-band NR-CA for 3Tx

- In the following basket WI, have the following limitation,

- If the power class is not specified for the single band, this band within a BC cannot have this power class capability

- If UL MIMO is not specified for the single band, this band within a BC cannot have UL MIMO capability

3.6 MSD framework for IMD

Way forward:

- For 2 Tx PC1.5 inter-band NR-CA, assuming both of the transmitters shall be set min(+26 dBm, PCMAX_L,f,c) as defined in clause 6.2A.4

- For 2 Tx PC1.5 inter-band EN-DC, assuming both of the transmitters shall be set min(+26 dBm, PCMAX_L,f,c) as defined in clause 6.2A.4

- For 3 Tx PC1.5 inter-band NR-CA, strive to reuse existing 2Tx/3Tx IMD framework/requirements and Rel-19 new 2Tx IMD framework/requirements, for new configurations if any

- For 3Tx PC2/PC1.5 inter-band EN-DC, follow same approach/methodology as for 3Tx PC2/PC1.5 inter-band NR-CA

4. Increasing UE transmission power

4.1 Scenarios to be considered in Rel-19

Way forward: Consider the following options and further discuss it in future meetings

- Option 1: For 2Tx, if based on existing specified combos, there seems no new scenarios to be considered so far. For 3Tx, the following scenarios may could be considered. (Samsung)

| Indicated PC for A-B (3Tx in total) | PC for band A of A-B (1Tx) | PC for band B of A-B (2Tx) | The total power (dBm) | Note |
| --- | --- | --- | --- | --- |
| PC2 | PC3 | PC2 | 27.8 | One CC per band |
| PC1.5 | PC3 | PC1.5 | 30.0 | One CC per band; For FWA only |

- Option 2: the following power class configuration could be considered in Rel 19 for UE increasing high power limit. (Xiaomi)

    - PC3 (TDD/FDD) +PC1.5

    - PC2 (TDD with TxD) +PC3 (TDD/FDD)

    - PC2 (TDD) +PC5

- Option 3: To consider the following eligible inter-band CA/ENDC band combination to enable increase higher power limit feature in Rel-19 (ZTE)

    - For PC2 2Tx inter-band NR CA and ENDC:

      - Inter-band with intra-band UL CA in one of the bands

    - For HPUE 3Tx inter-band NR CA and ENDC:

      - PC2 band combination of PC3+PC2 with single carrier in each band.

      - PC1.5 band combination of PC3+PC1.5 with single carrier in each band.

      - PC1.5 band combination of PC2+PC1.5 with single carrier in each band.

Note: Only PC3 is considered for LTE FDD in EN-DC

- Option 4: For both FWA and handheld UE, at least, the increasing UE power high limit feature should be introduced for 3Tx UL CA/EN-DC configured with PC3 band and PC2 band. Other power class configurations can also be discussed if they are included in the scope of 3Tx in Rel-19 WI. (DOCOMO)

  - On MSD impact

Way forward: Consider the following proposals and further discuss it in future meetings

- Proposal 1: Consider impact to the MSD requirements, e.g., whether to define MSD requirements for various combinations of per-band power classes such as PC1.5+PC2/3/5, PC2+PC5, etc. (Huawei)

- Proposal 2: RAN4 shall be mindful on defining MSD requirements for the new UL configurations supporting “Increasing UE power high limit for CA and DC” feature. (Apple)

- Proposal 3: For Rel-19, RAN4 to identify the additional conditions that must also be met before a new MSD test case is justified in context of a new power class aggregation for an already specified ULCA inter-band combination. For example, if an MSD test case exists for a PC1.5 CA power class UE comprising a PC3 UL band and a PC1.5 UL band, does a new case need to be defined when the same band combination is enabled for PC2 + PC1.5 UL? (Qualcomm)