3GPP TSG-RAN WG4 Meeting #110bis	R4-2406586

Changsha, China, April 15th – 19th, 2024

Title: 	WF on legacy power class related issues

Agenda Item:	11.2

Source: 	Qualcomm Inc.

Document for:	Approval

## Sub-topic 1-1: DL CA configured with single carrier UL

Agreements: For HPUE consider following options:

- Option 1: UE shall mandatorily meet ue-PowerClass at least for up to PC2, FFS for PC1.5

- Option 2: support up to highest specified single carrier power class is optional, based on UE capability indication and not restricted by notes in clause 5.5A

- For both options

  - Focus on Pcmax impact in RAN4#111

  - Strive to update specification from rel-17

  - the relation to the table notes in clause 5.5A is also considered.

  - Further consider output power for refsens and MSD requirements

## Sub-topic 1-2: Interband UL CA with single carrier UL transmission

Agreements: To increase UE output power and improve performance with one cell scheduled consider following options:

- Option 1: Allow UE to transmit higher power than specified power classes for the CA configuration up to at least PC2 single carrier power class (ue-powerClass) of the UL band subject to UE capability indication. FFS for PC 1.5.

- Option 2: For interband UL CA, specify Pcmax only for simultaneous transmission in both bands.

- For both options

  - Focus on Pcmax impact in RAN4#111

  - Strive to update specification from rel-17

## Sub-topic 1-3: Other updates in 38.101-1

Agreements:

- Consider to update the Pcmax,f,c for serving cells c of UL CA configurations to cover the cases in which

- the NR band power class ue-PowerClass/ue-PowerClass-v1610 is modified by the per-band-per-BC power class ue-PowerClassPerBandPerBC-r17; or

- the power class of the band combination (per-BC) is lower than NR band power class, excluding scenarios of sub-topic 1-1 and 1-2, or

- UE-specific P-Max is lower than the NR band power class; or

- UE-specific cell-group P-Max indications are present (can be lower than the per-BC power class)

such that the UL output power per serving cell c and the PHR become correct also for these cases.

- Consider to add a general clause to beginning of 6.2A in TS 38.101-1 describing the power class capabilities and their relationship, if necessary, noting that power-class capabilities are specified in 38.306.

## Sub-topic 1-4: Online agreements

Agreements: For the scenarios in the table below excluding scenarios in sub-topic 1-1 and 1-2 power class (powerClass) shall limit the configured maximum output power (Pcmax,f,c) when higherPowerLimit-r17 is not indicated:

|  | Scenarios |
| --- | --- |
| #1 | Intra-band DLCA with intra-band ULCA |
| #2 | Inter-band 2CC ULCA |
| #3 | Inter+intra 3CC ULCA |
| #4 | Inter+intra DLCA with intra-band ULCA |