3GPP TSG-RAN WG4 Meeting # 110bis	R4-2406606

# Changsha, China, April 15 – April 19, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | DraftCR | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for 38.101-5 to introduce clause 10.1~10.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Ericsson, Thales |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2024-04-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To introduce the following requirements in clause 10.1, 10.2 and 10.3 for Ka band VSAT. - The general description is proposed on where to verify the receiver characteristics. - For Polarization characteristics, the minimum requirements on the receiver characteristics apply under either LHCP (Left Hand Circular Polarization) or RHCP (Right Hand Circular Polarization). - OTA reference sensitivity level is introduced with full UL/DL RB allocation and EISREFSENS_50MHz + 10log10(NRB x SCS x 12 / factor). |  |  |  |  |  |  |  |  |
| Summary of change: |  | To introduce the following requirements in clause 10.1, 10.2 and 10.3 for Ka band VSAT. - The general description is proposed on where to verify the receiver characteristics. - For Polarization characteristics, the minimum requirements on the receiver characteristics apply under either LHCP (Left Hand Circular Polarization) or RHCP (Right Hand Circular Polarization). - OTA reference sensitivity level is introduced with full UL/DL RB allocation and EISREFSENS_50MHz + 10log10(NRB x SCS x 12 / factor). - Simplification and rewording of REFSENS requirement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no requirements for clause 10.1~10.3 of TS 38.101-5. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1, 10.2 and 10.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-5 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of Change for TS 38.101-5>>

10	Radiated receiver characteristics

10.1	General

Unless otherwise stated, the receiver characteristics are specified over the air (OTA) at the RIB for Ka bands fixed and mobile VSAT. The reference effective isotropic sensitivity (EIS), wanted signals and interference is defined assuming a 0 dBi reference antenna located at the center of the quiet zone.

10.2	Polarization characteristics

The minimum requirements on the receiver characteristics apply under either LHCP (Left Hand Circular Polarization) or RHCP (Right Hand Circular Polarization).

10.3	OTA reference sensitivity level

10.3.1	General

The OTA REFSENS requirement is a directional requirement and is intended to ensure the minimum OTA reference sensitivity level at the centre of the quiet zone in the RX beam peak direction. The OTA reference sensitivity power level EISREFSENS is the minimum mean power received over the air at the RIB, at which the throughput shall meet or exceed the requirements for a specified reference measurement channel.

10.3.2	Minimum requirement

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as [specified in Annexes A.2.3.2 and A.3.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.2.1) with peak reference sensitivity specified in Table 10.3.2-1 and Table 10.3.2-2]. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link Angle).

Table 10.3.2-1: OTA reference sensitivity requirement for NTN VSAT

| NTN VSAT channel bandwidth (MHz) |  | UL/DL RB allocation |  | OTA reference sensitivity level, EISREFSENS (dBm) |  |
| --- | --- | --- | --- | --- | --- |
| 50, 100, 200, 400 |  | Full RB allocation NRB as specified in clause 5.3.2 |  | EISREFSENS_50MHz + 10log10(NRB x SCS x 12 / factor) (NOTE 1) |  |
| NOTE 1: The “factor” represents the normalized factor to scale EIS for different (Channel bandwidth, SCS) configurations. The value of factor is 66 RBs x 60 kHz SCS x 12, i.e. 47520 kHz. |  |  |  |  |  |

Table 10.3.2-2: EISREFSENS_50MHz value per NTN VSAT

| Operating band | NTN VSAT class | NTN VSAT type | EISREFSENS_50MHz (dBm) |
| --- | --- | --- | --- |
| n512, n511 | Fixed VSAT | 1, 2 | -122 |
|  |  | 3 | -115.6 |
| n512, n511, n510 | Mobile VSAT | 4, 5 | -122 |

<<End of Change>>