3GPP TSG-RAN WG1 Meeting #110bis														R4-2406616

China, Changsha, April 15th – 19th , 2024

Agenda item:	9.11.5

Source: 	Qualcomm Incorporated

Title: 	AI/ML ad-hoc meeting minutes

Document for:	Approval

# Introduction

The discussion on the AI/ML study is organized under a single thread [335] in RAN4#110 bis. The ad-hoc meeting will discuss some of the topics from the moderator summary in [1].

# Discussion

## Testability and interoperability issues for beam management

### Sub-topic 2-5

Test environment limitation/requirements

Multiple companies brought up several issues regarding the test environment needed. RAN4 should come up with a list of requirements/conditions for the test environment in order to further discuss feasibility and what can be achieved with the current testing setups.

Issue 2-5: Test environment

- Proposals

  - Option 1: Test environment needs to emulate at least X beams for set B. X=8, 16

  - Option 2: Test environment must ensure spatial consistency between beams in set A and set B

  - Option 3: number of angles of arrivals (or angle of arrival range/spread)

  - Option 4: number of AoDs

  - Option 5: UE rotation

  - Option 6: others

- Recommended WF

  - To be discussed

Nokia: why do we need multiple AoDs?

Moderator: question is whether we need to test with signals coming from multiple directions or is a single direction enough.

Qualcomm: our proposal is about AoA, not AoD. we need to test with CDL with multiple AoAs. AoD does not physically exist, will determine the probe power.

Oppo: we agree with QC, we need to think about number of AoAs, that will be directly related with the number of AoDs. we use different configurations or power values to mimic beam sweeping.

Nokia: if we translate X into number of probes. we will need the number of beams in set A also.to know the ground truth do we know all set A beams to be emulated in the chamber. 32 or 64 probes. we need a solution other than creating the number the number of probes equals to the number of beams.

Apple: Option 1 is valid. we need a number. Option 2 is important, we need a relation between sets. then we need to ask feedback from TE vendors. we do not need the number of AoDs or AoAs.

Samsung: how many beams we want to have for set A and set B. set A might also need to be emulated depending on how the test is set up.

Apple: for multiple AoDs, we do not necessarily assume that the probes are transmitting simultaneously to emulate muti-path.

Moderator: we can leave the discussion on whether probes are transmitting simultaneously later, we still need a number of AoDs or AoAs.

KTL: reporting should be a one time instance

### Sub-topic 2-6

Training data

Issue 2-6:	Datasets for training

- Proposals

  - Option 1: Training Data set to be specified in RAN4(directly or through some algorithm )

  - Option 2: Training data set to be left to implementation (companies can generate it based on knowledge of the test environment)

  - Option 3: others

- Recommended WF

  - To be discussed

Discussion:

Qualcomm: if RAN4 defines the codebook and the channel model, any vendor would have enough information could generate the training data. how to generate the data set should be enough. in the test, it would be signaled that a certain codebook is used to ensure test data and training is consistent

Oppo: w]e need to consider the consistency between the training and test data. we may need something specified.

Samsung: for training the Tx side codebook will be needed. a reference will be needed, channel model will need alignment. for UE side, it will depend on the UE implementation. vendors could use to generate the training data.

Nokia:  Option 2 should be enough. option 1 will raise the questions how the UE will work in the real deployment

Apple: for the UE side, we probably do not need to define the training data set. UE would do offline training.

Oppo: we need some guidance to guarantee that UE can generate the model

Moderator: test setup(environment, channel model, etc)

Intel: this is for UE side. leaving to implementation makes sense. the model shall be generic. companies will use something only to pass the test.

Samsung: we are not sure the model should be generic enough. . do you mean the test should be generic enough? different BSs will have different Tx codebook

Apple: model can be a separate issue. we can focus on the dataset for testing. we can focus on the requirement. we do not have to mix everything.

Huawei: if UE cannot know the test environment. for CDL the results would be difficult to converge.

Intel: end goal is that we should not specify any data set. this is a chicken and egg problem. the requirements will be based on simulation. it could make sense when we define the requirement, companies would have to report some information about the training data used.

Qualcomm: we could list some parameters right now

vivo: when we define the test, we need some procedures. we need some reference model.

Nokia: currently the UE doesn’t know the Tx side codebook.

Apple:  why don’t we do requirements for the network side model.

Agreement:

Take option 2 as baseline under the assumption that enough test parameters are known(for example: Tx side codebook, channel model, etc) such that UE vendors can train the mode for the test setup

FFS on what exact parameters are needed.

## Testability and interoperability issues for positioning accuracy enhancement

### Sub-topic 3-1

Requirements for case 1

There are some proposals on whether to define requirements for case 1 (UE-based positioning with UE-side model, direct AI/ML positioning)

Issue 3-1: Requirements for case 1

- Proposals

  - Option 1: RAN4 should not define requirements for case 1

  - Option 2: RAN4 should continue to discuss how to define requirements for case 1 (including feasibility of defining such requirements)

  - Option 3: Others

- Recommended WF

To be discussed

Note: currently there are no requirements for UE based positioning

Discussion:

vivo: we think we may need to define requirements, we may need further study how to perform the test. we may need something for LCM. for example, straight line distance.

Docomo: we need requirements for this case but it would be difficult

Nokia: we also think we need requirements. accuracy is the main point. straight line between the UE report value and the ideal position.

interdigital: what is the ideal position. we assume some grid or PRU

Ericsson:  we do not think a requirement is needed. it should be motivated, why is it needed. first we need to identify the requirement

CATT: we prefer option 2. we should work out some way to do this.

Moderator: if UE position is to be reported, we would need some requirement.

Oppo: companies can also propose more concrete methods for testing.

Apple:

Agreement:

postpone discussion until reporting scheme(if defined) is clear.  if reporting scheme is introduced, RAN4 will further discuss whether to define requirements or not.

RAN4 will not define any accuracy requirements if no reporting scheme is introduced

### Sub-topic 3-2

KPIs for case 1

Several KPIs are proposed for each use case, it should be discussed which are more appropriate for each use case if requirements are to be studied/defined

Identified KPIs in the TR:

For metrics for positioning requirements/tests, the candidate options include

- Option 1: positioning accuracy: Ground truth vs. reported

  - only option available for direct positioning

- Option 2: CIR/PDP, channel estimation accuracy

- Option 3: ToA, RSTD and RSRP, and RSRPP

- Option 4: others (e.g., intermediate KPIs, LoS/NLoS)/combinations of the above

Issue 3-2: KPIs for case 1

- Proposals

  - Option 1: Option 1

  - Option 2: Option 2

  - Option 3: Option 3

  - Option 4: others

  - Option 5: combination of options

- Recommended WF

  - To be discussed

If option 5 is proposed, a concrete proposal should be presented

### Sub-topic 3-3

LOS/NLOS indicator

There are some proposals to introduce requirements/tests for the LOS/NLOS indicator

Issue 3-3: LOS/NLOS indicator

- Proposals

  - Option 1: RAN4 should discuss requirements for the LOS/NLOS indicator

  - Option 2: RAN4 should wait for progress in other groups whether LOS/NLOS reporting is defined

  - Option 3: It is not feasible for RAN4 to define requirements for such an indicator

  - Option 4: others

- Recommended WF

To be discussed

### Sub-topic 3-4

KPIs for case 3a/3b

Several KPIs are proposed for each use case, it should be discussed which are more appropriate for each use case if requirements are to be studied/defined

Identified KPIs in the TR:

For metrics for positioning requirements/tests, the candidate options include

- Option 1: positioning accuracy: Ground truth vs. reported

  - only option available for direct positioning

- Option 2: CIR/PDP, channel estimation accuracy

- Option 3: ToA, RSTD and RSRP, and RSRPP

- Option 4: others (e.g., intermediate KPIs, LoS/NLoS)/combinations of the above

Issue 3-5: KPIs for case 3a/3b

- Proposals

  - Option 1: Option 1

  - Option 2: Option 2

  - Option 3: Option 3

  - Option 4: others

  - Option 5: combination of options

- Recommended WF

  - To be discussed

If option 5 is proposed, a concrete proposal should be presented

## Testability and interoperability issues for CSI compression and CSI prediction

### Sub-topic 4-1

CSI prediction KPI

Some companies brought somewhat different options for CSI prediction test metric.

Issue 4-1: CSI Prediction Accuracy metrics

- Proposals

  - Option 1: Use relative throughput (e.g. predicted PMI vs random PMI) as the test metric

    - use Type I single-panel,

  - Option 2: Use absolute minimum throughput as metric

  - Option 3: Others

- Recommended WF

To be discussed

### Sub-topic 4-2

Reference encoder/decoder and Test encoder/decoder

There is some confusion on the terminology of reference encoder/decoder and test encoder/decoder, this should be clarified

Issue 4-2: Reference and test encoder/decoder

- Proposals

  - Option 1:

    - Reference encoder/decoder: the encoder/decoder used in RAN4 discussions for simulation alignment/requirement derivation. It could be documented (in TR, WF, etc) or captured in the specifications as necessary.

    - Test encoder/decoder: the encoder/decoder to be used in RAN4 tests. it will be captured in the specifications if necessary (for example, for Option 3 it would be captured in the specifications)

  - Option 2: Other definitions

- Recommended WF

  - Option 1

To be discussed any other changes are needed

Discussion:

vivo: we are generally ok with Option 1, for the reference, in RAN1 inter-vendor is discussed. there is a fully standardize reference model to be used in the field. encoder/decoder used in RAN4 discussion “at least for simulation/requirement”

Samsung: we should just discuss the reference from a RAN4 perspective. we still discuss the tests and we might need a reference for requirements/ tests. this might be related to RAN1 reference but not necessarily

Qualcomm: agree that this should be only for RAN4. the reference could also serve for verification of test encoder/decoder. for example, for Option 4, we will need to have some verification. we can connect the decoder with the reference encoder to test the performance

Nokia: we assume a fixed or we allow flexibility. in our view we should not fix a single reference one. each company will train their own.

Intel:test decoder will cover both Option 3 and 4.

Agreement:

Test decoder(for UE side test): the decoder to be used in RAN4 tests and implemented in TE. it will be captured in the specifications if necessary (for example, for Option 3 it would be explicitly captured in the specifications)

test decoder definition covers both Option 3 and Option 4

Proposed definition by moderator, to be further discussed:

Reference encoder/decoder: the encoder/decoder used in RAN4 discussions at least for simulation alignment/requirement derivation, test decoder derivation and/or test decoder verification. It could be documented (in TR, WF, etc) or captured in the specifications as necessary.

### Sub-topic 4-3

Option 3 split

The possibility of splitting option 3 was brought up

Issue 4-3: Option 3 split

- Proposals for parameters that should be agreed upon:

  - Option 1:

- Option 3a: The test decoder is determined for each test case, using a specific dataset collecting from the configuration/scenario of the considered test case.

- Option 3b: The test decoder is determined for more than one test cases, using a mixed dataset collecting from different configurations/scenarios of the considered test cases.

  - Option 2: Decision to split should be made later (in an eventual WI phase) based on feasibility

    - it will depend on whether it is feasible to have a “one fits all” decoder or whether it has to be scenario specific

  - Option 3: others

- Recommended WF

  - Option 2

Moderator: we are currently discussing the feasibility of deriving a test decoder, discussion/decision on whether multiple decoders would be needed for different tests can happen later if needed.

### Sub-topic 4-4

RAN4 – RAN1 Coordination

Some companies brought up the fact that the RAN1 discussion on interoperability for the 2 sided model is similar to the RAN4 discussion on Options 3/4 and some further alignment would be needed

Issue 4-4: RAN4 – RAN1 Coordination

- Proposals

  - Option 1: Further alignment is needed between RAN4 and RAN1

    - which parameters to align?

  - Option 2: Discussion in RAN4 should continue, RAN4 should determine testing feasibility of Option 3/4

  - Option 3: Option 1 and Option 2

  - Option 4: others

- Recommended WF

  - To be discussed

Concrete proposals on which parameters to align would be useful

E///: we need alignment on what is the purpose of discuss. in RAN4 the purpose is to figure out whether Option 3 or 4 works. there might be an issue if we discuss a reference encoder, hopefully there won’t be multiple references in different WGs.

Samsung: we need to know what is the purpose of feasibility study in RAN4 and RAN1. in RAN1 they are still discussing the benefits. in RAN4 we are trying to figure out testability. our suggestion is to focus on a single use case. it doesn’t necessarily have to be aligned with RAN1. study purpose is different. if there will be WI, more coordination will be needed.

Qualcomm: similar to what Samsung commented. we propose a quick agreement.

Apple: RAN4 testability will unlikely impact RAN1 design. the outcome here should depend on RAN1 work. one RAN1 solution is just to standardize the data set. if that is agree, Option 3 might not work. we can request RAN1 to inform us about decisions made.

Oppo: maybe we need to emphasize the purpose for the reference model in RAN1 and test model in RAN4. we might have a simple model with some simple requirements. a simple mode might not work in the field.

Ericsson: RAN4 delegates should also discuss with their RAN1 colleagues because RAN4 work on Option 3 might help RAN1 discussion.

Agreement:

RAN4 continues to discuss Option 3 and 4 and can review/discuss RAN1 agreements on interoperability if there will be any

RAN4 reference model and RAN1 reference model point to different things.

### Sub-topic 4-5

standardization steps for Option 3

Further steps on how Option 3 would be standardized should be discussed. This discussion should apply to Option 4, at least partially. A flow chart is provided in R4-2405653 and reproduced below for convenience:

Step-1:  Identify necessary Model Architecture Parameters

Standardization Procedure End

for a certain use case
(e.g., CSI compression for precoding matrix under certain config.)

Model architecture parameters could include: Model type, Model depth, Layer type/size, Quantization, etc.

Model training procedure, loss function, training datasets, hyperparameters, etc.

Step-3:  Companies provide two-sided model design based on their own study/preference

Step-4:  Performance comparison based on different companies’ en/decoder designs

Yes

No

Step-6:  Performance alignment by companies based on agreed model architecture/training parameters

Performance comparison in terms of metrics like NMSE, SGCS, etc.

No

Yes

No

Standardization 
Procedure Start

Step-2:  Identify necessary Model training Parameters

Test decoder is expected to be generated in this step

Reference encoder is assumed, but leave enough implementation flexibility to vendors (similar to Demod alignment for MMSE-IRC)

Step-8:  performance alignment 
for encoder design by companies 
based on assumptions on reference encoder

Yes

RAN4 performance requirement obtained (for certain reference encoder)

Step-10:  Derive RAN4 performance requirement

Step-5: RAN4
agree on two-sided model architecture
 / training parameters?

Step-7: RAN4 agree on test decoder 
(which can be fully specified in spec.)

Step-9: RAN4 achieve performance alignment?

Issue 4-5: Option 3 standardization process

- Proposals

  - Option 1: This flowchart can be used as reference, RAN4 is now discussing Steps 1-4,  RAN4 to futher work on feasibility to pass Step - 5 and move to Step - 6

  - Option 2: others

- Recommended WF

To be discussed:

Current status and further steps to align the understanding and goals among companies

Oppo:  we might need something before step 1 like test conditions

Qualcomm: flowchart is a good start, it should help up in the future to figure out what is needed. criteria for Step-4 will be complicated. step-1 and 2 is what we are doing now. we do not have agreements now.

Nokia: we will need some comparison criteria for Step-4. we also need the performance goals from RAN4. the focus on the parameters is different.

Apple: flowchart helps. the problem will be how to make a decision in Step-5

vivo: flowchart is good. in step-4 we might need a performance evaluation method, including complexity. in step-5 we might need some model structure.

Samsung: there is some explanation in the first box, for the performance comparison, it was intentional to leave this generic. we are aware that there is no criteria right now.

Xiaomi: do we need to align the data set? how we choose the decoder, performance and complexity is a thing. interoperability.

E///: step-1 and 2 will also be complicated, how do we identify?

Huawei: we agree with the flowchart. for Step-4 we will also need to figure out what is the benchmark. do we need any intermiedate KPI.are we comparing against eTypeII or Type 1.

Oppo: identify means align among companies or just different companies come with different ones?

Samsung: we are not trying to solve the problem but try to identify where we are and what needs to be done. we do not talk about the details.

Oppo: for step-2, do we need some alignment? training procedures could lead to similar results. for the training procedures, different companies can use different parameters.

Apple: we do not have to scrutinize all the details. we might need several iterations. we might need multiple results with different assumptions and try to align.

Nokia: some clarification on step-7

CATT:similar view about this task, thanks for presenting this flowchart. this is only a guidance, the details have to be discussed.

oppo: we will need to discuss whether we need step 8 to 9

### Sub-topic 4-6

Reference encoder/decoder handling

Need for standardizing a reference encoder/decoder and how to handle it was brought up in one proposal.

Issue 4-6: Reference encoder/decoder handling

- Proposals

  - Option 1: RAN4 should follow the same process of specifying parameters for designing the reference one-sided and two-sided AI/ML models as used in Option 3 for specifying the test decoder. Additionally, it should define a training dataset to ensure consistency and alignment in performance results.

  - Option 2: Discuss on a case by case basis for Option 3/4 and the “sub-options”

  - Option 3: reference encoder/decoder does not need to be specified

  - Option 4: others

- Recommended WF

  - To be discussed

Concrete proposals on which parameters to align would be useful

### Sub-topic 4-7

Option 3 for 2-sided model

Discussion on choosing parameters for the option 3 feasibility study started in the previous meeting and some agreements were captured in a table reproduced below (agreements highlighted in green, tentative agreements highlighted in yellow):

| Category | Parameter | Description/Examples |
| --- | --- | --- |
| Model architecture parametersa | Model type | Transformer, CNN, RNN, MLP |
|  | Model depth | Number of layers |
|  | Layer type | Fully connected, convolutional, activation layer, etc. |
|  | Layer size | Neuron count and configuration |
|  | Quantization method for the encoder output | Scalar, vector (with codebook) |
|  | Encoder-decoder interface | Number of bits of latent message |
|  | Fixed point representation | Int8, int16, floating point etc |
|  | Format of input to encoder/output of decoder |  |
| Model Training related parameters | Training procedure | FFS (e.g Initialization method, training duration, training completion criteria, collaboration type, encoder assumption, etc) |
|  | Loss function | SGCS, NMSE, etc. |
|  | Training datasets | Channel model, number of Tx/Rx ports Other parameters FFS (e.g. rank) |
|  | Hyperparameters | Learning rate, batch size, regularization techniques and strength, optimization algorithm, etc. |
|  | Cross-validation details | Dataset splits for training/testing/validation |
| Generalization (may be applicable to all four options) | Performance requirements on test dataset(s) | Mean SGCS, etc. |
| Scalability (may be applicable to all four options) | Supported antenna port configurations | (2,8,2), (2,4,2), etc. |
|  | Supported feedback payloads | Low, medium, high overhead (with specified number of bits) |

Proposals for the other parameters were brought in this meeting and these need to be discussed.

Issue 4-3: Option 3 for 2-sided model

- Proposals for parameters that should be agreed upon:

  - Option 1: The proposals from different companies are captured in the table below:

| Proponent | Vivo | CATT | SEU | QC | Xiaomi | Intel | E/// | Apple | Nokia | Keysight | VIAVI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Model type | Transformer and CNN | Transformer | CsiNet | MLP | Transformer | Several model types can be considered (e.g., transformer, CNN) RAN1 inputs on the best identified models in terms of performance/complexity can be requested | Transformer or CNN depending on design target **In order to decide on the model structure, it is necessary to consider RAN1 evaluations and also consider whether the RAN4 standardized test decoder should target maximum performance or some complexity limit. RAN4 should decide and agree on whether the target is purely performance or both performance and complexity. (Some checking with RAN1 may be needed). | CNN | Transformer |  |  |
| Model depth |  |  |  | Three linear layers (with one activation function) | 6 | decide upper bound of complexity based on RAN1 evaluation |  | Number of layers, CNN: Kernel/Filter Size, Padding, Stride, Pooling layers parameters, Number of channels | Several multi-head attention layers (min: [3], max: [7]) |  |  |
| Layer type |  |  |  | MLP with expansion factor N = 4, and each layer/function is described in the following • 1st linear layer: input is latent message of size Zdim and output is a vector of size nSB x nTx • Reshape: convert the vector of size nSB x nTx to nSB vectors with size nTx • 2nd Linear layer: For each subband, the input is a vector of size nTx, and the output is a vector of size N x nTx. The same linear layer is applied to each of nSB subbands. • Activation function: GELU • 3rd Linear layer: the input is a vector of size N x nTx, and the output is with a vector of size nTx. The same linear layer is applied to each of nSB subbands. |  |  |  | Fully connected, convolutional, activation layer (activation type: leakyRelu,etc), batch(group)-normalization layer,dropout layer, etc. | Fully connected layers with activation function for each attention layer/block. |  |  |
| Layer size |  |  |  |  |  |  |  | Neuron count and configuration | Note that output layer can be different. |  |  |
| Quantization method for the encoder output |  |  |  | scalar quantizer, 2 bits per dimension (element) | Scalar, 2 bits |  |  | Scalar, vector (with codebook) | Specify embedding and feedforward dimensions, number of attention heads per attention layer/block. |  |  |
| Encoder-decoder interface |  |  |  |  | 60, 120, 280bits |  | Consider 63, 110 or 230 | Number of bits of latent message | Scalar quantization |  |  |
| Fixed point representation |  |  |  |  |  |  | Int8, int16, floating point etc. | Int8, int16, floating point etc |  |  |  |
| Format of input to encoder/output of decoder |  |  |  |  |  |  | Consider pre-processing of Eigenvector using Enhanced Type 2 codebook |  |  |  |  |
| Training procedure |  |  |  |  |  |  | Convolutional: Feedback bits per transmission e.g., 10 x 4 = 40 Transformer: Optimizer, e.g., Adam | collaboration training type need to be specified | Collaboration type: Type-3 Network first training | Training completion criteria is probably one of the most important parameters assuming it will include boundaries (minimum and maximum) for the test decoder performance required. Collaboration type will determine interactions required between different stakeholders and/or different AI/ML algorithms blocks | Adopt a type 3 training collaboration approach for RAN4 testing and consider special handling (e.g. an adaptation layer) to minimise performance variations between TE vendors |
| Loss function |  |  |  | SGCS | SGCS |  | NMSE |  |  |  |  |
| Training datasets |  |  |  | Encoder input dataset should cover all the contributing companies’ encoder input data |  |  |  | Number of layers/rank? SNR, Genie/ real channel estimates (impairments)? Data format of training (depends on Collaboration training type) Size of training data set Specify channel model parameters or training data samples stored in a repository? Different Training Sets (configurations/ scenarios)? Multiple vendor training sets | Channel model for training: UMa Note that in the performance test TDL or CDL (if available) model to be used. Number of Tx/Rx ports: 4 RX, 16 or 32 TX, Note that other options should not be precluded but better to agree on a single scenario as a starting point. Rank: 1. Channel estimates: Channel eigenvectors derived from [ideal, non-ideal] channel estimates, magnitude normalized to unit length. Dataset size: Sufficient number of samples to achieve minimum performance and prevent underfitting are needed. |  |  |
| Hyperparameters | Learning rate, batch size, regularization techniques and strength, optimization algorithm, etc. Note that training procedure does not need to be fully aligned. |  |  |  | Learning rate = 0.001, batch size = 128 optimization algorithm = Adam, |  |  |  |  |  |  |
| Cross-validation details | Dataset splits for training/testing/validation |  |  |  | Dataset for training: 199,500 Dataset for Testing: 10000 Dataset for validation: 10500 |  |  | Dataset splits for training/validation/testing This testing doesn’t refer to DUT testing | 80%/20%, where training data is also used for validation. |  |  |

- Recommended WF

  - To be discussed

Likely multiple options need to be chosen, RAN4 should agree on a minimum set such that companies can continue the study

Training procedure:

Samsung: this is hard to align. it could also be done offline(just like alignment when defining demod requirements). we can first try without agreements on this and then iterate depending on the results.

Huawei: even with training procedure we could end up with different model parameters. some agreements could help in saving us time on alignment.

Apple: we might just do a first round of simulations and see what companies come up with.

E///: this table is only Option 3.

Nokia: we cannot completely avoid and agreements on the training procedure. different training criteria (types) are important. duration is not that important.

oppo: my suggestion for the table is to treat some parts of them to be aligned/agreed and companies should provide information on the other parameters used.

CATT: training procedure is not the first priority. we can focus on the architecture and constraints.

Qualcomm: this table is only for the derivation procedures. more agreements means we shrink the decoder possible decoders.

CATT: we need to consider the gain. we should discuss later the model selection criteria.

Samsung: we should not focus on the gain, we need to consider it but it’s not the main criteria. we should focus on the channel model and parameters(number of Rx/Tx, etc). companies can come with proposals

Nokia: we could select some parameters from existing CSI test cases. it would give us a concrete scenario.

oppo: we will need to have a minimum requirement.we can discuss which KPI separately.

E///: it may be a good idea to start from a CSI test case but it could lead to overfitting

Nokia: might not be good to train specifically for that. a concrete scenario would help, though

Samsung: to use existing setup of CSI tests is a good idea, channel model might be too simple. for minimum requirement, what is the minimum requirement. there is no objective about performance.

Apple: performance is also about whether it is feasible to test. it will also have to be discussed.

oppo: there is no specified minimum requirement. for the test model, different companies have different understanding about minimum performance for the channel.

Parameters:

Following parameters based on RAN1 baseline scenario captured in TR:

Channel model: []

Tx/Rx configuration: []

latent message size: []

number of subbands: []

companies to report model type used:

### Sub-topic 4-8

Option 4 for 2-sided model

Several companies brought proposal on how to further study/check the feasibility of option 4. This discussion is also related to sub-topic 4-3 as the feasibility study has many similarities

Issue 4-8: Option 4 for 2-sided model

- Proposals

  - Option 1: dataset based

    - 1. Training dataset is specified, where each training sample consists of both the raw channel matric/precoding matrix and the bit stream forwarded to the test decoder

    - 2 : (Nokia)

      - Option-2a: Freeze complete training data while leaving model architecture for implementation.

      - Option-2b: Freeze the important characteristics of training data, e.g., number of bits of latent message while leaving actual data samples and model architecture for implementation.

    - 3 : (Huawei)Model structure is not specified in RAN4. Training dataset is specified, where each training sample consists of both the raw channel matric/precoding matrix and the bit stream forwarded to the test decoder.

    - 4: (Qualcomm)

      - Step 1: RAN4 agrees a pair of encoder and decoder with full details (same as fully specified decoder discussion) and an encoder input data generation procedure.

      - Step 2: RAN4 uses this encoder/decoder pair and the generation procedure to generate a set of decoder input and output data and captures this dataset in the specification.

      - Step 3: RAN4 specifies a test decoder verification procedure based on the specified dataset.

  - Option 2: aggregated data set: (Qualcomm), (Apple)

    - Step 1: RAN4 achieves some agreements (e.g., part of but not all the parameters in the test decoder parameter table in the previous meeting WF[1]) for the test decoder.

    - Step 2: Interested companies can design their own encoder/decoder pairs based on the agreements to contribute the (decoder input, decoder output) dataset to RAN4

    - Step 3: RAN4 aggregates the datasets from all the contributing companies, and capture the aggregated dataset in the specification

    - Step 4: RAN4 specifies a test decoder verification procedure based on the specified dataset.

  - Option 3: Model structure/architecture based:

    - 1: (Nokia) (Model architecture-based):

      - Option-1a: Freeze a complete model architecture while leaving training data for implementation.

      - Option-1b: Freeze a backbone of model architecture while leaving complete training data and model architectural details for implementation.

    - 2: (Huawei)Option 4b: Model structure is specified in RAN4. Training dataset is not specified for verifying the encoder at DUT. The test decoder developed by TE vendor needs verification.

      - FFS: How to determine the test metric for test decoder developed by each TE vendor.

    - 3: (vivo) specify model structure of decoder

  - Option 4: Reference encoder

    - 1: (Ericsson) standardize a reference encoder and a test data framework and test dataset/channel model:

    - 2: (Qualcomm)

      - Step 1: RAN4 agrees a pair of encoder and decoder with full details (same as fully specified decoder discussion) and an encoder input data generation procedure

      - Step 2: RAN4 capture the encoder as a reference encoder and the encoder input data generation procedure in the specification.

      - Step 3: RAN4 specifies a test decoder verification procedure based on the reference encoder.

    - 3: (vivo) specify encoder

    - 4: (vivo) specify encoder backbone

  - Option 5: Other – “hybrid”

    - 1: (Nokia)Option 3: Freeze the important characteristics of training data, e.g., number of bits of latent message, and a backbone of model architecture while leaving actual data samples and architectural details for implementation.

  - Option 6: Others

- Recommended WF

  - To be discussed

Likely multiple options need to be chosen/combined RAN4 should agree on a minimum set such that companies can continue the study

Discussion:

E///: if we just standardize a data set, we will get similar performance but not similar latent message. we need somehow to standardize the latent space or a reference encoder

Apple: we can unify the simulations for Option 3 and 4. if we fix the parameters and overall this works, Option 4 might work overall. we should focus on the simulation campaign. once we have a scenario, each company will provide a dataset and a encoder. we do not have to standardize a reference. we cannot align the results

Samsung: we need to standardize the dataset (encoder input(same as decoder output) to latent mapping)

Qualcomm: for any of these options, we need to ensure test repatibility. we have to make sure that any encoder paired with any TE decoder should lead to same results. for option 1, we require a fully specified encoder and decoder pair in order to generate dataset.

vivo: Option 4-2  has the issue of latent message not being specified. for other options, we have to pair encoder with decoder so we need to align them. we will need some reference encoder and decoder

E///: for this Option 4, anyone should be able to take the RAN4 spec and build an encoder or decoder. it is achievable if we specify the entire set of latents but this could be more efficient to do a look up table

oppo: we need to think about the latent message. how to get that, there are different possible solutions.

Intel: we might need a reference encoder and decoder for Option 4. potentially we can align this between options 3 an 4 to work on parallel.

Apple: training would also be part of the test. or certain data set. if the test is only for inference, we might need a way to train the TE side decoder.

Samsung: we need a reference to generate the dataset. there are still differences between Option 3 and 4. for the dataset , we still need a channel model.

Huawei: in addition to encoder input, decoder output is also needed. if we want Option 4 to be testable, we need a pair of encoder and decoder. Options 3 and 4 would be equivalent.

E///: we do not a decoder output to be exactly the same as the encoder input, they should just be close enough. related to what Apple said, it’s not feasible to have this kind of training for the infra side, separately with every DUT vendor.

### Sub-topic 4-8

TE verification/validation for Option 4

Option will leave the actual test decoder implementation to the TE vendors, there might be a need to verify that the decoder is correctly implemented and has the right level of performance.

Issue 4-8: TE decoder verification/validation

- Proposals

  - Option 1: RAN4 will have to come up with a TE verification/validation procedure

    - details are FFS

  - Option 2: This should be left to RAN5

  - Option 3: TE verification/validation is not needed

  - Option 4: other options

- Recommended WF

To be discussed

# References

[1] R4-2318142, “Topic summary for [109][136] FS_NR_AIML_air”, Moderator(Qualcomm Incorporated), RAN4#109