3GPP TSG-RAN WG4 Meeting #110bis												R4-2406619

Changsha, China, 15th – 19th April, 2024

Agenda item:			9.14.5

Source:	vivo

Title:	WF on NR LP-WUS UE requirements

Document for:	Approval

Introduction

This is way forward for Rel-19 LP-WUS UE RF.

Topic #2: UE RF requirements

Sub-topic 2-1 General for UE RF

Issue 2-1-1: Operation bands for LP-WUR

Agreement:

- Focus on FR1 licensed bands

  - FR2 is not precluded

- FFS on which licensed bands will be chosen as example bands for band specific requirements

Issue 2-1-3: one or multi-Sets of requirements

Way forward:

  - Discussions could be separated for OOK-based receiver and OFDM-based receiver. And latter stage to discuss whether a generic requirement could be defined, or not.

    - Further discuss the possibility of a dedicated set of requirements for OOK LP-WUR

    - FFS whether there is any other factors should be considered for LP-WUR requirements discussion

  - SNR and NF may be different for these two types.

Issue 2-1-4: Performance metric for Rx requirements

Way forward:

  - Agreements have been reached in issue 2-2-1.

Issue 2-1-5: Rx antenna assumption for LP-WUR

Way forward:

  - 1Rx is supported in FR1. FFS RX diversity

Issue 2-1-6: CBW and RB number for LP-WUR

Way forward:

  - This can be discussed in simulation assumption for ACS/ASCS.

Issue 2-1-7: SCS for LP-WUR

Way forward:

  - The SCS of LP-WUS is considered same as in-band NR signals

Issue 2-1-8: sync raster for LP-WUR

Way forward:

  - No sync raster is needed for LP-WUR based on RAN1 design

Sub-topic 2-2 REFSENS requirements

Issue 2-2-1: Performance metric for REFSENS

Agreement:

- Use X% missed detection rate as the starting point for performance metric for LP-WUS RF requirements

  - FFS on X values

  - FFS on whether to have false alarm rate

Issue 2-2-2: How to specify REFSENS requirements

Agreement:

- Reuse legacy approach to derive REFSENS, further discuss SNR, NF, IM

  - FFS whether to design REFSENS requirements or other requirements to ensure LP-WUR meet the coverage target

  - Side condition for REFSENS test: DL test signal will only have LP-WUS signal.

Issue 2-2-3: How to decide SNR value (not requirement) for REFSENS

Way forward:

  - After concluding WUS design in RAN1, the SNR to specify REFSENS requirements should be decided in RAN4

Issue 2-2-4: How to decide NF value (not requirement) for REFSENS

Way forward:

  - Encourage companies input on NF analysis for different LP-WUR types

Issue 2-2-5: RF/Antenna Architecture considerations for LP-WUS receiver

Way forward:

  - Detailed antenna/RF architecture can be considered in issue 2-2-4 analysis

  - FFS other RAN4 impacts than REFSENS

Sub-topic 2-3 ASCS requirements

Issue 2-3-1: Simulation work for ASCS

Agreement:

- LLS simulation for ASCS is sufficient

  - The same level PSD for LP-WUS and NR signals is assumed

Issue 2-3-2: Detailed Methodology for simulation to evaluate ASCS value and guard RB

Way forward:

  - Use SI assumption as a starting point, further confirm and align the simulation parameters in the group next meeting

    - Metric for link-level simulation should be aligned

    - Alignment required on RF impairments before starting the work.

Issue 2-3-3: ASCS requirements value

Way forward:

  - FFS ASCS requirements value

Issue 2-3-4: Required number of guard RB

Way forward:

  - FFS required number of guard RB for ASCS

Issue 2-3-5: RF impairment considerations for ASCS

Way forward:

  - RF impairments can be considered and aligned for ASCS simulation

Issue 2-3-6: Side conditions for ASCS test

Way forward:

  - LP-WUS along with required guard RBs is packed with NR legacy DL signal on both sides.

Sub-topic 2-4 ACS requirements

Issue 2-4-1: coexistence System-level simulation to evaluate ACS

Agreement:

- The same interference level as for main radio is assumed for LP-WUR

  - Guard RB number needs be evaluated by link level simulation for ACS requirements

Issue 2-4-2: Link-level simulation to evaluate ACS

Way forward:

  - Use SI assumption as a starting point, further confirm and align the simulation parameters next meeting

    - Necessary update and alignment on parameters, e.g., number of WUS RB, RF impairment, ADC bit, and performance metric is required before starting the work.

Issue 2-4-3: ACS requirements value

Way forward:

  - FFS ACS requirements value

Issue 2-4-4: How to evaluate required guard RB for ACS case

Way forward:

  - FFS required number of guard RB for ACS

Issue 2-4-5: Detailed coexistence System-level simulation assumptions

Way forward:

  - System-level simulation is not needed for ACS evaluation

Sub-topic 2-5 other UE RF requirements

Issue 2-5-1: Any other Rx requirements should be specified

Agreement:

- Apart from REFSENS, ACS, Rx requirements of IBB, OBB, intermodulation as well as spurious emissions should be specified for LP-WUR.

- Other legacy receiver requirements are not precluded

Sub-topic 2-6 Testabilty for UE RF requirements

Issue 2-6-1: Test metric for LP-WUR receiver

Way forward:

  - Test metric should be aligned with performance metric for Rx requirements