3GPP TSG-RAN WG4 Meeting #110bis	R4-2406623

Changsha, China, April 15th – 19th, 2024

Agenda item:			9.10.3

Source:	Intel Corporation

Title:	WF on UE RF requirements for NR channel BW less than 5MHz for FR1 Phase 2

Document for:	Approval

# Topic #2: UE RF requirements for inter-band NR CA/DC with less than 5MHz CBW

## Issue 2-1: NR CA configuration

Agreement (Wed online discussion):

- Define new inter-band CA operating band combination

| NR CA Band | NR Band |
| --- | --- |
| CA_n100-n101 | n100, n101 |

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n100A-n101A | CA_n100A-n101A | n100 | 3 | 0 |
|  |  | n101 | 5, 10 |  |

- 5MHz bandwidth can be added for n100 in the band combination after RAN plenary agrees to add it in the WID

## Issue 2-2: NR DC configuration

Agreement (Wed online discussion):

- Define new inter-band DC configuration

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n100A-n101A | DC_n100A-n101A |

## Issue 2-3: Transmission bandwidth configuration for 3MHz and 5MHz CBW for band n100

Agreement (Wed online discussion):

- Define RF requirements agnostic of transmission bandwidth configurations for 3MHz CBW

## Issue 2-4: Power class for UL inter-band CA

Agreement (Wed online discussion):

- Define PC3 inter-band CA/DC combinations

- FFS on whether to define PC1 inter-band CA/DC combinations

## Issue 2-6: Signalling aspects

Agreement

- FFS whether to introduce new UE capability and/or other RRC signalling to support NR CA/DC with less than 5MHz CBW and support of 12 or 20 RB transmission bandwidth

## Issue 2-7: UE RF specification impact for inter-band CA (CA_n100A-n101A) for TS 38.101-1

Agreement

- Define the following requirements for NR CA operation for CA_n100A-n101A

| Requirement | Required change for NR CA |
| --- | --- |
| General |  |
| 5.2A Operating bands | Specify inter-band CA operating band |
| 5.3 UE channel bandwidth | Remove the restriction that limited 3MHz CBW to single carrier case |
| 5.5A Configurations for CA | Specify new NR CA configurations and bandwidth combinations sets for inter-band CA |
| TX requirements |  |
| 6.2A.1 UE maximum output power for CA | Specify UE Power Class for uplink inter-band CA combination (Table 6.2A.1.3-1) |
| 6.2A.4 Configured transmitted power for CA | Specify ΔTIB,c for uplink inter-band CA combination (Table 6.2A.4.2.3-1) |
| 6.5A Output RF spectrum emissions for CA | Specify spurious emissions for UE coexistence for uplink inter-band CA combination (Table 6.5A.3.2.3-1) |
| RX requirements |  |
| 7.3A.2 Reference sensitivity for CA | Specify ΔRIB,c for inter-band CA combination (Table 7.3A.3.2.1-1) |

- For the following requirements there is no impact for CA_n100A-n101A, but they may need to be assessed for other combinations involving CBW with less than 5MHz

| Requirement | Required change for NR CA |
| --- | --- |
| RX requirements |  |
| 7.6A Blocking characteristics for CA |  |

## Issue 2-8: UE RF specification impact for NR DC (DC_n100A-n101A) for TS 38.101-1

Agreement

- Define the following requirements for NR DC operation for DC_n100A-n101A.

| Requirement | Required change for NR DC |
| --- | --- |
| General |  |
| 5.5B Configurations for DC | Specify new NR DC configurations |
| TX requirements |  |
| 6.2B.1 UE maximum output power for NR-DC | Specify UE Power Class for uplink inter-band DC combination (Table 6.2B.1.3-1) |

- Other requirements for DC_n100A-n101A are same as for CA_n100-n101

## Issue 2-10: Configured transmitted power for Inter-band CA: ΔTIB,c

Agreement

- ΔTIB,c  is FFS

  - Option 1: ΔTIB,c = [0.3] dB

  - Other option are not precluded

## Issue 2-11: Reference sensitivity: ΔRIB,c

Agreement

- ΔRIB,c  = 0 dB

## Issue 2-12: Reference sensitivity: MSD requirements

Agreement

- FFS on MSD requirements