3GPP TSG-RAN WG4 Meeting #110bis	R4-2406653

# Changsha, China, 15th - 19th April 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR |  | rev | - | Current version: | 18.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TS38.101-3 ENDC for FR1 xBLTE2BNR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Softbank Corp. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | DC_R18_xBLTE_2BNR_yDL2UL |  |  |  |  |  | Date: |  |  | 2024-04-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Inter-band EN-DC configurations within FR1 (Three bands) as follows - DC_8B-1A_n77A Inter-band EN-DC configurations within FR1 (four bands) as follows. - DC_3A-8A_n1A-n77A - DC_3A-8B_n1A-n77A |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the requested Inter-band EN-DC combinations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Proposed CA/DC are not supported properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5B.4.3 Table 5.5B.4.3-1 6.2B.4.2.3.3 Table 6.2B.4.2.3.3-1 7.3B.3.3.3 Table 7.3B.3.3.3-1 7.3B.2.3.5.2 Table 7.3B.2.3.5.2-1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---Start of changes---

5.5B.4.3	Inter-band EN-DC configurations within FR1 (four bands)

Table 5.5B.4.3-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-(n)3AA-n8A | DC_1A_n3A DC_1A_n8A DC_(n)3AA1 DC_3A_n8A |
| DC_1A-3A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_3A_n3A4 DC_3A_n41A |
| DC_1A-3A_n3A-n77A2 | DC_1A_n3A DC_1A_n77A DC_3A_n3A4 DC_3A_n77A |
| DC_1A-3A_n3A-n78A2 | DC_1A_n3A DC_1A_n78A DC_3A_n3A4 DC_3A_n78A |
| DC_1A-3A-5A_n28A | DC_1A_n28A DC_3A_n28A DC_5A_n28A |
| DC_1A-3A-5A_n40A | DC_1A_n40A DC_3A_n40A DC_5A_n40A |
| DC_1A-3A_n5A-n40A | DC_1A_n5A DC_1A_n40A DC_3A_n5A DC_3A_n40A |
| DC_1A-3A-5A_n77A DC_1A-3A-5A_n77(3A) DC_1A-3A-5A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_5A_n77A |
| DC_1A-3A-5A_n78A2 DC_1A-3A-5A_n78C2 DC_1A-3C-5A_n78A DC_1A-1A-3A-5A_n78A DC_1A-1A-3C-5A_n78A | DC_1A_n78A DC_3A_n78A DC_5A_n78A |
| DC_1A-3A-5A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_5A_n78A |
| DC_1A-3A-5A_n78(A-C) | DC_1A_n78A DC_3A_n78A DC_5A_n78A |
| DC_1A-3A_n5A-n78A2 DC_1A-3C_n5A-n78A2 | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3A_n78A DC_3C_n78A |
| DC_1A-3A-5A_n79A2 | DC_1A_n79A DC_3A_n79A DC_5A_n79A |
| DC_1A-3A-7A_n1A | DC_1A_n1A DC_3A_n1A DC_7A_n1A |
| DC_1A-3A-7A_n3A DC_1A-3A-7C_n3A | DC_1A_n3A DC_3A_n3A4 DC_7A_n3A |
| DC_1A-3A-7A_n5A DC_1A-3A-7C_n5A DC_1A-3C-7A_n5A DC_1A-3C-7C_n5A | DC_1A_n5A DC_3A_n5A DC_7A_n5A DC_7C_n5A |
| DC_1A-3A-7A_n7A DC_1A-3C-7A_n7A | DC_1A_n7A DC_3A_n7A DC_7A_n7A4 |
| DC_1A-1A-3A-7A_n7A DC_1A-1A-3C-7A_n7A DC_1A-3A-3A-7A_n7A DC_1A-1A-3A-3A-7A_n7A | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_7A_n7A4 |
| DC_1A-3A-(n)7AA DC_1A-3C-(n)7AA | DC_1A_n7A DC_3A_n7A |
| DC_1A-3A-7A_n8A | DC_1A_n8A DC_3A_n8A DC_7A_n8A |
| DC_1A-3A-7A_n26A DC_1A-3A-7C_n26A DC_1A-3C-7A_n26A DC_1A-3C-7C_n26A | DC_1A_n26A DC_3A_n26A DC_3C_n26A DC_7A_n26A DC_7C_n26A |
| DC_1A-3A-7A_n28A DC_1A-3A-7C_n28A DC_1A-3C-7A_n28A DC_1A-3C-7C_n28A | DC_1A_n28A DC_3A_n28A DC_3C_n28A DC_7A_n28A DC_7C_n28A |
| DC_1A-3A-7A-7A_n28A | DC_1A_n28A DC_3A_n28A DC_7A_n28A |
| DC_1A-1A-3A-7A_n28A DC_1A-1A-3C-7A_n28A | DC_1A_n28A DC_3A_n28A DC_3C_n28A DC_7A_n28A |
| DC_1A-3A-7A_n38A12,13 | CA_1A-3A |
| DC_1A-3A-7A_n40A | DC_1A_n40A DC_3A_n40A DC_7A_n40A |
| DC_1A-3A-7A-7A_n40A | DC_1A_n40A DC_3A_n40A DC_7A_n40A |
| DC_1A-3A-7A_n77A | DC_1A_n77A DC_3A_n77A DC_7A_n77A |
| DC_1A-3A-7A_n77(2A) DC_1A-3A-7A_n77(3A) | DC_1A_n77A DC_3A_n77A DC_7A_n77A |
| DC_1A-3A-7A-7A_n77A | DC_1A_n77A DC_3A_n77A DC_7A_n77A |
| DC_1A-3A-7A-7A_n77(2A) DC_1A-3A-7A-7A_n77(3A) | DC_1A_n77A DC_3A_n77A DC_7A_n77A |
| DC_1A-3A-7A_n78A2 DC_1A-3A-7C_n78A DC_1A-3C-7A_n78A2 DC_1A-3C-7C_n78A DC_1A-3A-7A_n78C2 | DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_7C_n78A |
| DC_1A-3A-3A-7A_n78A2 | DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-3A-7A_n78(2A) DC_1A-3C-7A_n78(2A) DC_1A-3A-7C_n78(2A) DC_1A-3C-7C_n78(2A) | DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_7C_n78A |
| DC_1A-3A-7A_n78(A-C) | DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-1A-3A-7A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-1A-3A-3A-7A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-3A_n7A-n78A DC_1A-3A_n7B-n78A | DC_1A_n7A DC_1A_n78A DC_3A_n7A DC_3A_n78A |
| DC_1A-3A_n7A-n78(2A) DC_1A-3C_n7A-n78(2A) | DC_1A_n7A DC_1A_n78A DC_3A_n7A DC_3A_n78A DC_3C_n7A DC_3C_n78A |
| DC_1A-3C_n7A-n78A DC_1A-3C_n7B-n78A | DC_1A_n7A DC_1A_n78A DC_3A_n7A DC_3A_n78A DC_3C_n7A DC_3C_n78A |
| DC_1A-3A-7A-7A_n78A2 DC_1A-1A-3C-7A_n78A DC_1A-3A-7A-7A_n78C2 | DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-3A-7A-7A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-3A-7A-7A_n78(A-C) | DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-3A-3A-7A-7A_n78A2 | DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-3A-7A_n105A | DC_1A_n105A DC_3A_n105A DC_7A_n105A |
| DC_1A-3A-8A_n7A | DC_1A_n7A DC_3A_n7A DC_8A_n7A |
| DC_1A-3A-8A_n28A | DC_1A_n28A DC_3A_n28A DC_8A_n28A |
| DC_1A-3A-8A_n77A2,9 DC_1A-3C-8A_n77A2,9 | DC_1A_n77A9 DC_3A_n77A9 DC_3C_n77A DC_8A_n77A9 |
| DC_1A-3A-8A_n77(2A)2 DC_1A-3C-8A_n77(2A) | DC_1A_n77A9 DC_3A_n77A9 DC_3C_n77A DC_8A_n77A9 |
| DC_1A_n3A-n8A-n77A | DC_1A_n3A DC_1A_n8A DC_1A_n77A |
| DC_1A_n3A-n8A-n77(2A) | DC_1A_n3A DC_1A_n8A DC_1A_n77A |
| DC_1A-3A-8A_n77(3A)2 | DC_1A_n77A DC_3A_n77A DC_8A_n77A |
| DC_1A-3A_n8A-n77A DC_1A-3A_n8A-n77(2A) | DC_1A_n8A DC_1A_n77A DC_3A_n8A DC_3A_n77A |
| DC_1A-3A-8A_n78A2,9 DC_1A-3C-8A_n78A2,9 | DC_1A_n78A9 DC_3A_n78A9 DC_8A_n78A9 |
| DC_1A-3A-8A_n78(2A)2 DC_1A-3C-8A_n78(2A)2 | DC_1A_n78A9 DC_3A_n78A9 DC_8A_n78A9 |
| DC_1A-3A_n8A-n78A | DC_1A_n8A DC_1A_n78A DC_3A_n8A DC_3A_n78A |
| DC_1A-3A-8A_n79A2 | DC_1A_n79A DC_3A_n79A DC_8A_n79A |
| DC_1A-3A-11A_n28A | DC_1A_n28A DC_3A_n28A DC_11A_n28A |
| DC_1A-3A-11A_n77A2 | DC_1A_n77A DC_3A_n77A DC_11A_n77A |
| DC_1A-3A-11A_n77(2A) 2 DC_1A-3A-11A_n77(3A)2 | DC_1A_n77A DC_3A_n77A DC_11A_n77A |
| DC_1A-3A-18A_n3A | DC_1A_n3A DC_3A_n3A4 DC_18A_n3A |
| DC_1A-3A-18A_n28A | DC_1A_n28A DC_3A_n28A DC_18A_n28A |
| DC_1A-3A-18A_n41A | DC_1A_n41A DC_3A_n41A DC_18A_n41A |
| DC_1A-3A-18A_n77A | DC_1A_n77A DC_3A_n77A DC_18A_n77A |
| DC_1A-3A-18A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_18A_n77A |
| DC_1A-3A-18A_n78A | DC_1A_n78A DC_3A_n78A DC_18A_n78A |
| DC_1A-3A-18A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_18A_n78A |
| DC_1A-3A-18A_n79A | DC_1A_n79A DC_3A_n79A DC_18A_n79A |
| DC_1A-3A-19A_n77A2,9 DC_1A-3A-19A_n77C2 | DC_1A_n77A9 DC_3A_n77A9 DC_19A_n77A9 |
| DC_1A-3A-19A_n77(2A)2 | DC_1A_n77A DC_3A_n77A DC_19A_n77A |
| DC_1A-3A-19A_n78A2, 9 DC_1A-3A-19A_n78C2 | DC_1A_n78A9 DC_3A_n78A9 DC_19A_n78A9 |
| DC_1A-3A-19A_n78(2A)2 | DC_1A_n78A DC_3A_n78A DC_19A_n78A |
| DC_1A-3A-19A_n79A2,9 DC_1A-3A-19A_n79C2 | DC_1A_n79A9 DC_3A_n79A9 DC_19A_n79A9 |
| DC_1A-3A-20A_n1A | DC_1A_n1A DC_3A_n1A DC_20A_n1A |
| DC_1A-3A-20A_n3A | DC_1A_n3A DC_3A_n3A DC_20A_n3A |
| DC_1A-3A-20A_n7A | DC_1A_n7A DC_3A_n7A DC_20A_n7A |
| DC_1A-3A-20A_n8A | DC_1A_n8A DC_3A_n8A DC_20A_n8A |
| DC_1A-3A-20A_n28A3,8,14 DC_1A-3C-20A_n28A3,8,14 | DC_1A_n28A DC_3A_n28A DC_3C_n28A DC_20A_n28A |
| DC_1A-3A-20A_n38A | DC_3A_n38A DC_20A_n38A |
| DC_1A-3A-20A_n41A DC_1A-3C-20A_n41A | DC_1A_n41A DC_3A_n41A DC_3C_n41A DC_20A_n41A |
| DC_1A-3A-20A_n78A2 DC_1A-3A-20A_n78C2 | DC_1A_n78A DC_3A_n78A DC_20A_n78A |
| DC_1A-1A-3A-20A_n78A2 | DC_1A_n78A DC_3A_n78A DC_20A_n78A |
| DC_1A-3A-3A-20A_n78A2 | DC_1A_n78A DC_3A_n78A DC_20A_n78A |
| DC_1A-3A-20A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_20A_n78A |
| DC_1A-3A-21A_n77A2,9 DC_1A-3A-21A_n77C2 | DC_1A_n77A9 DC_3A_n77A9 DC_21A_n77A9 |
| DC_1A-3A-21A_n77(2A)2,9 | DC_1A_n77A9 DC_3A_n77A9 DC_21A_n77A9 |
| DC_1A-3A-21A_n78A2,9 DC_1A-3A-21A_n78C2 | DC_1A_n78A9 DC_3A_n78A9 DC_21A_n78A9 |
| DC_1A-3A-21A_n78(2A)2,9 | DC_1A_n78A9 DC_3A_n78A9 DC_21A_n78A9 |
| DC_1A-3A-21A_n79A2,9 DC_1A-3A-21A_n79C2 | DC_1A_n79A9 DC_3A_n79A9 DC_21A_n79A9 |
| DC_1A-3A-26A_n78A DC_1A-3C-26A_n78A | DC_1A_n78A DC_3A_n78A DC_26A_n78A |
| DC_1A-3A-26A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_26A_n78A |
| DC_1A-3C-26A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_26A_n78A |
| DC_1A-3A_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3A_n78A |
| DC_1A-3C_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3C_n26A DC_3A_n78A DC_3C_n78A |
| DC_1A-3A-28A_n3A | DC_1A_n3A DC_3A_n3A4 DC_28A_n3A |
| DC_1A-3A-28A_n5A DC_1A-3C-28A_n5A | DC_1A_n5A DC_3A_n5A DC_28A_n5A |
| DC_1A-3A-28A_n7A DC_1A-3C-28A_n7A DC_1A-3A-28A_n7B DC_1A-3C-28A_n7B | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_28A_n7A |
| DC_1A-3A-3A-28A_n7A DC_1A-3A-3A-28A_n7B | DC_1A_n7A DC_3A_n7A DC_28A_n7A |
| DC_1A-1A-3A-28A_n7A DC_1A-1A-3C-28A_n7A DC_1A-1A-3A-28A_n7B DC_1A-1A-3C-28A_n7B | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_28A_n7A |
| DC_1A-1A-3A-3A-28A_n7A DC_1A-1A-3A-3A-28A_n7B | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_28A_n7A |
| DC_1A-3A-28A_n38A | DC_1A_n38A DC_3A_n38A DC_28A_n38A |
| DC_1A-3A_n28A-n38A | DC_1A_n28A DC_3A_n28A DC_1A_n38A DC_3A_n38A |
| DC_1A-3A-28A_n40A | DC_1A_n40A DC_3A_n40A DC_28A_n40A |
| DC_1A-3A_n28A-n41A2 | DC_1A_n28A DC_1A_n41A DC_3A_n28A DC_3A_n41A |
| DC_1A-3A_n28A-n75A | DC_1A_n28A DC_3A_n28A |
| DC_1A-3C_n28A-n75A | DC_1A_n28A DC_3A_n28A DC_3C_n28A |
| DC_1A-3A-28A_n77A2 DC_1A-3A-28A_n77C2 | DC_1A_n77A DC_3A_n77A DC_28A_n77A |
| DC_1A-3A_n28A-n77A2 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A |
| DC_1A-3A_n28A-n77(2A) 2 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A |
| DC_1A_n3A-n28A-n77A2 | DC_1A_n3A DC_1A_n28A DC_1A_n77A |
| DC_1A_n3A-n28A-n77(2A) 2 | DC_1A_n3A DC_1A_n28A DC_1A_n77A |
| DC_1A-3A-28A_n78A2 DC_1A-3C-28A_n78A2 DC_1A-3A-28A_n78C2 DC_1A-1A-3A-28A_n78A DC_1A-1A-3C-28A_n78A DC_1A-3A-28A_n78(2A) 2 DC_1A-3C-28A_n78(2A)2 | DC_1A_n78A DC_3A_n78A DC_28A_n78A |
| DC_1A-3A-3A-28A_n78A2 | DC_1A_n78A DC_3A_n78A DC_28A_n78A |
| DC_1A-3A-28A_n79A2 DC_1A-3A-28A_n79C2 | DC_1A_n79A DC_3A_n79A DC_28A_n79A |
| DC_1A-3A_n28A-n79A2 | DC_1A_n28A DC_1A_n79A DC_3A_n28A DC_3A_n79A |
| DC_1A_n3A-n28A-n79A | DC_1A_n3A DC_1A_n28A DC_1A_n79A |
| DC_1A-3A_n28A-n78A2 DC_1A-3C_n28A-n78A2 | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3C_n28A DC_3A_n78A DC_3C_n78A |
| DC_1A-3A_n28A-n78(2A)2 | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_3C_n78A |
| DC_1A-3A-32A_n28A DC_1A-3C-32A_n28A | DC_1A_n28A DC_3A_n28A DC_3C_n28A |
| DC_1A-3A-32A_n78A DC_1A-3A-32A_n78C | DC_1A_n78A DC_3A_n78A |
| DC_1A-3A-32A_n78(2A) | DC_1A_n78A DC_3A_n78A |
| DC_1A-3C-32A_n78A | DC_1A_n78A DC_3A_n78A DC_3C_n78A |
| DC_1A-3A-38A_n28A DC_1A-3C-38A_n28A | DC_1A_n28A DC_3A_n28A DC_3C_n28A DC_38A_n28A |
| DC_1A-3A-38A_n78A | DC_1A_n78A DC_3A_n78A |
| DC_1A-3A-38A_n78(2A) DC_1A-3C-38A_n78(2A) | DC_1A_n78A DC_3A_n78A |
| DC_1A-3A_n38A-n78A | DC_1A_n38A DC_1A_n78A DC_3A_n38A DC_3A_n78A |
| DC_1A-3C-38A_n78A | DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_38A_n78A |
| DC_1A-3A_n40A-n77A | DC_1A_n40A DC_1A_n77A DC_3A_n40A DC_3A_n77A |
| DC_1A-3A_n40A-n77(2A) | DC_1A_n40A DC_1A_n77A DC_3A_n40A DC_3A_n77A |
| DC_1A-3A_n40A-n78A DC_1A-3A_n40A-n78C | DC_1A_n40A DC_1A_n78A DC_3A_n40A DC_3A_n78A |
| DC_1A-3A-40A_n78A DC_1A-3A-40C_n78A | DC_1A_n78A DC_3A_n78A DC_40A_n78A |
| DC_1A-3A_n40A-n105A | DC_1A_n40A DC_1A_n105A DC_3A_n40A DC_3A_n105A |
| DC_1A-3A-40A_n78(2A) DC_1A-3A-40C_n78(2A) | DC_1A_n78A DC_3A_n78A DC_40A_n78A |
| DC_1A-3A-41A_n3A DC_1A-3A-41C_n3A | DC_1A_n3A DC_3A_n3A4 DC_41A_n3A DC_41C_n3A |
| DC_1A-3A-41A_n28A2 DC_1A-3A-41C_n28A2 | DC_1A_n28A DC_3A_n28A DC_41A_n28A DC_41C_n28A |
| DC_1A-3A-41A_n41A | DC_1A_n41A DC_3A_n41A |
| DC_1A-3A-(n)41AA | DC_1A_n41A DC_3A_n41A |
| DC_1A-3A-41A_n77A DC_1A-3A-41C_n77A | DC_1A_n77A DC_3A_n77A DC_41A_n77A DC_41C_n77A |
| DC_1A-3A-41A_n77(2A) DC_1A-3A-41C_n77(2A) | DC_1A_n77A DC_3A_n77A DC_41A_n77A DC_41C_n77A |
| DC_1A-3A_n41A-n77A | DC_1A_n41A DC_1A_n77A DC_3A_n41A DC_3A_n77A |
| DC_1A-3A_n41A-n77(2A) | DC_1A_n41A DC_1A_n77A DC_3A_n41A DC_3A_n77A |
| DC_1A-3A-41A_n78A DC_1A-3A-41C_n78A | DC_1A_n78A DC_3A_n78A DC_41A_n78A DC_41C_n78A |
| DC_1A-3A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_3A_n41A DC_3A_n78A |
| DC_1A-3A_n41A-n78(2A) | DC_1A_n41A DC_1A_n78A DC_3A_n41A DC_3A_n78A |
| DC_1A-3A-41A_n78(2A) DC_1A-3A-41C_n78(2A) | DC_1A_n78A DC_3A_n78A DC_41A_n78A DC_41C_n78A |
| DC_1A-3A-41A_n79A2 DC_1A-3A-41C_n79A2 | DC_1A_n79A DC_3A_n79A DC_41A_n79A |
| DC_1A-3A-42A_n28A2 DC_1A-3A-42C_n28A2 | DC_1A_n28A DC_3A_n28A DC_42A_n28A DC_42C_n28A |
| DC_1A-3A-42A_n77A7,8,9 DC_1A-3A-42A_n77C7,8 DC_1A-3A-42C_n77A7,8,9 DC_1A-3A-42C_n77C7,8 DC_1A-3A-42D_n77A7,8,9 | DC_1A_n77A9 DC_3A_n77A9 |
| DC_1A-3A-42A_n77(2A) 7,8 DC_1A-3A-42C_n77(2A) 7,8 | DC_1A_n77A DC_3A_n77A |
| DC_1A-3A-42A_n78A7,8,9 DC_1A-3A-42A_n78C7,8 DC_1A-3A-42C_n78A7,8,9 DC_1A-3A-42C_n78C7,8 DC_1A-3A-42D_n78A7,8,9 | DC_1A_n78A9 DC_3A_n78A9 |
| DC_1A-3A-42A_n79A9 DC_1A-3A-42A_n79C DC_1A-3A-42C_n79A9 DC_1A-3A-42C_n79C DC_1A-3A-42D_n79A9 | DC_1A_n79A9 DC_3A_n79A9 |
| DC_1A-3A_n75A-n78A DC_1A-3C_n75A-n78A | DC_1A_n78A DC_3A_n78A DC_3C_n78A |
| DC_1A-3A_n77A-n79A9 | DC_1A_n77A9 DC_1A_n79A9 DC_3A_n77A9 DC_3A_n79A9 |
| DC_1A-(n)3AA-n77A DC_1A-(n)3AA-n77(2A) | DC_1A_n3A DC_1A_n77A DC_3A_n77A |
| DC_1A_n3A-n77A-n79A | DC_1A_n3A DC_1A_n77A DC_1A_n79A |
| DC_1A_n3A-n77(2A)-n79A | DC_1A_n3A DC_1A_n77A DC_1A_n79A |
| DC_1A-3A_n78A-n79A9 | DC_1A_n78A9 DC_1A_n79A9 DC_3A_n78A9 DC_3A_n79A9 |
| DC_1A-3A_n78A-n105A | DC_1A_n78A DC_1A_n105A DC_3A_n78A DC_3A_n105A |
| DC_1A-3A_SUL_n78A-n80A | DC_1A_n78A DC_1A_n80A DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A |
| DC_1A-5A-7A_n28A | DC_1A_n28A DC_5A_n28A DC_7A_n28A |
| DC_1A-5A-7A_n40A | DC_1A_n40A DC_5A_n40A DC_7A_n40A |
| DC_1A-5A-7A-7A_n40A | DC_1A_n40A DC_5A_n40A DC_7A_n40A |
| DC_1A-5A-7A_n77A | DC_1A_n77A DC_5A_n77A DC_7A_n77A |
| DC_1A-5A-7A_n77(2A) DC_1A-5A-7A_n77(3A) | DC_1A_n77A DC_5A_n77A DC_7A_n77A |
| DC_1A-5A-7A-7A_n77A | DC_1A_n77A DC_5A_n77A DC_7A_n77A |
| DC_1A-5A-7A-7A_n77(2A) DC_1A-5A-7A-7A_n77(3A) | DC_1A_n77A DC_5A_n77A DC_7A_n77A |
| DC_1A-5A-7A_n78A DC_1A-5A-7A_n78C DC_1A-1A-5A-7A_n78A | DC_1A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-5A-7A_n78(2A) | DC_1A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-5A-7A_n78(A-C) | DC_1A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-5A-7A-7A_n78A DC_1A-5A-7A-7A_n78C | DC_1A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-5A-7A-7A_n78(2A) | DC_1A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-5A-7A-7A_n78(A-C) | DC_1A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-5A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_5A_n28A DC_5A_n78A |
| DC_1A-5A_n40A-n77A | DC_1A_n40A DC_1A_n77A DC_5A_n40A DC_5A_n77A |
| DC_1A-5A_n40A-n77(2A) | DC_1A_n40A DC_1A_n77A DC_5A_n40A DC_5A_n77A |
| DC_1A-5A_n40A-n78A DC_1A-5A_n40A-n78C | DC_1A_n40A DC_1A_n78A DC_5A_n40A DC_5A_n78A |
| DC_1A-5A-41A_n79A | DC_1A_n79A DC_5A_n79A DC_41A_n79A |
| DC_1A-7A_n3A-n38A | DC_1A_n3A |
| DC_1A-7A_n3A-n78A DC_1A-7C_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_7A_n3A DC_7C_n3A DC_7A_n78A DC_7C_n78A |
| DC_1A-7A_n3A-n78(2A) DC_1A-7C_n3A-n78(2A) | DC_1A_n3A DC_1A_n78A DC_7A_n3A DC_7C_n3A DC_7A_n78A DC_7C_n78A |
| DC_1A-7A_n5A-n40A | DC_1A_n5A DC_1A_n40A DC_7A_n5A DC_7A_n40A |
| DC_1A-7A_n5A-n78A DC_1A-7C_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_7A_n5A DC_7A_n78A DC_7C_n5A DC_7C_n78A |
| DC_1A-7A_n38A-n78A | DC_1A_n78A |
| DC_1A-7A-8A_n3A | DC_1A_n3A DC_7A_n3A DC_8A_n3A |
| DC_1A-7A-8A_n7A | DC_1A_n7A DC_7A_n7A DC_8A_n7A |
| DC_1A-7A-8A_n20A | DC_1A_n20A DC_7A_n20A DC_8A_n20A |
| DC_1A-7A-8A_n28A | DC_1A_n28A DC_7A_n28A DC_8A_n28A |
| DC_1A-7A-7A-8A_n28A | DC_1A_n28A DC_7A_n28A DC_8A_n28A |
| DC_1A-7A_n7A-n78A | DC_1A_n7A DC_7A_n7A4 DC_1A_n78A DC_7A_n78A |
| DC_1A-7A-8A_n78A DC_1A-7A-7A-8A_n78A | DC_1A_n78A DC_7A_n78A DC_8A_n78A |
| DC_1A-7A-8A_n78(2A) | DC_1A_n78A DC_7A_n78A DC_8A_n78A |
| DC_1A-7A_n8A-n78A | DC_1A_n8A DC_1A_n78A DC_7A_n8A DC_7A_n78A |
| DC_1A-7A-20A_n3A DC_1A-7C-20A_n3A | DC_1A_n3A DC_7A_n3A DC_7C_n3A DC_20A_n3A |
| DC_1A-7A-20A_n8A | DC_1A_n8A DC_7A_n8A DC_20A_n8A |
| DC_1A-7A-20A_n28A3,8,14 | DC_1A_n28A DC_7A_n28A DC_20A_n28A |
| DC_1A-7A-20A_n38A12,13 | CA_1A-20A |
| DC_1A-7A-20A_n78A2 DC_1A-7A-20A_n78C2 | DC_1A_n78A DC_7A_n78A DC_20A_n78A |
| DC_1A-1A-7A-20A_n78A2 | DC_1A_n78A DC_7A_n78A DC_20A_n78A |
| DC_1A-7A-7A-20A_n78A2 | DC_1A_n78A DC_7A_n78A DC_20A_n78A |
| DC_1A-7A-20A_n78(2A) | DC_1A_n78A DC_7A_n78A DC_20A_n78A |
| DC_1A-7A-26A_n78A DC_1A-7C-26A_n78A | DC_1A_n78A DC_7A_n78A DC_26A_n78A |
| DC_1A-7A-26A_n78(2A) DC_1A-7C-26A_n78(2A) | DC_1A_n78A DC_7A_n78A DC_26A_n78A |
| DC_1A-7A_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_7A_n26A DC_7A_n78A |
| DC_1A-7C_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_7A_n26A DC_7C_n26A DC_7A_n78A DC_7C_n78A |
| DC_1A-7A-28A_n3A DC_1A-7C-28A_n3A | DC_1A_n3A DC_7A_n3A DC_7C_n3A DC_28A_n3A |
| DC_1A-7A-28A_n5A DC_1A-7C-28A_n5A | DC_1A_n5A DC_7A_n5A DC_7C_n5A DC_28A_n5A |
| DC_1A-7A-28A_n7A | DC_1A_n7A DC_7A_n7A4 DC_28A_n7A |
| DC_1A-1A-7A-28A_n7A | DC_1A_n7A DC_7A_n7A4 DC_28A_n7A |
| DC_1A-7A-28A_n20A | DC_1A_n20A DC_7A_n20A DC_28A_n20A |
| DC_1A-7A-28A_n38A | 1A16 28A16 |
| DC_1A-7A-28A_n40A | DC_1A_n40A DC_7A_n40A DC_28A_n40A |
| DC_1A-7A-28A_n78A DC_1A-7C-28A_n78A | DC_1A_n78A DC_7A_n78A DC_7C_n78A DC_28A_n78A |
| DC_1A-7A-28A_n78(2A) DC_1A-7C-28A_n78(2A) | DC_1A_n78A DC_7A_n78A DC_28A_n78A |
| DC_1A-1A-7A-28A_n78A | DC_1A_n78A DC_7A_n78A DC_28A_n78A |
| DC_1A-7A_n28A-n78A2 DC_1A-7C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_7A_n28A DC_7A_n78A DC_7C_n28A DC_7C_n78A |
| DC_1A-7A-32A_n3A DC_1A-7C-32A_n3A | DC_1A_n3A DC_7A_n3A |
| DC_1A-7A-32A_n8A | DC_1A_n8A DC_7A_n8A |
| DC_1A-7A-32A_n28A | DC_1A_n28A DC_7A_n28A |
| DC_1A-7A-32A_n78A | DC_1A_n78A DC_7A_n78A |
| DC_1A-7A-38A_n3A | DC_1A_n3A |
| DC_1A-7A-38A_n8A | DC_1A_n8A |
| DC_1A-7A-38A_n28A10 | DC_1A_n28A |
| DC_1A-7A-38A_n78A10 | DC_1A_n78A |
| DC_1A-7A_n40A-n77A | DC_1A_n40A DC_1A_n77A DC_7A_n40A DC_7A_n77A |
| DC_1A-7A_n40A-n77(2A) | DC_1A_n40A DC_1A_n77A DC_7A_n40A DC_7A_n77A |
| DC_1A-7A-7A_n40A-n77A | DC_1A_n40A DC_1A_n77A DC_7A_n40A DC_7A_n77A |
| DC_1A-7A-7A_n40A-n77(2A) | DC_1A_n40A DC_1A_n77A DC_7A_n40A DC_7A_n77A |
| DC_1A-7A-40A_n78A DC_1A-7A-40C_n78A | DC_1A_n78A DC_7A_n78A DC_40A_n78A |
| DC_1A-7A-40A_n78(2A) DC_1A-7A-40C_n78(2A) | DC_1A_n78A DC_7A_n78A DC_40A_n78A |
| DC_1A-7A_n40A-n78A DC_1A-7A_n40A-n78C | DC_1A_n40A DC_1A_n78A DC_7A_n40A DC_7A_n78A |
| DC_1A-7A-7A_n40A-n78A DC_1A-7A-7A_n40A-n78C | DC_1A_n40A DC_1A_n78A DC_7A_n40A DC_7A_n78A |
| DC_1A-7A_n40A-n105A | DC_1A_n40A DC_1A_n105A DC_7A_n40A DC_7A_n105A |
| DC_1A-7A_n75A-n78A | DC_1A_n78A DC_7A_n78A |
| DC_1A-7A_n78A-n105A | DC_1A_n78A DC_1A_n105A DC_7A_n78A DC_7A_n105A |
| DC_1A-8A-(n)3AA | DC_1A_n3A DC_(n)3AA4 DC_8A_n3A |
| DC_1A-8A_n3A-n28A | DC_1A_n3A DC_1A_n28A DC_8A_n3A DC_8A_n28A |
| DC_1A-8A_n3A-n77A2 DC_1A-8B_n3A-n77A2 | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A |
| DC_1A-8A_n3A-n77(2A)2 | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A |
| DC_1A-8A_n3A-n79A | DC_1A_n3A DC_1A_n79A DC_8A_n3A DC_8A_n79A |
| DC_1A-8A-11A_n3A | DC_1A_n3A DC_8A_n3A DC_11A_n3A |
| DC_1A-8A-11A_n28A | DC_1A_n28A DC_8A_n28A DC_11A_n28A |
| DC_1A-8A-11A_n77A2 | DC_1A_n77A DC_8A_n77A DC_11A_n77A |
| DC_1A-8A-11A_n77(2A)2 DC_1A-8A-11A_n77(3A)2 | DC_1A_n77A DC_8A_n77A DC_11A_n77A |
| DC_1A-8A-11A_n78A2 | DC_1A_n78A DC_8A_n78A DC_11A_n78A |
| DC_1A-8A-11A_n79A2 | DC_1A_n79A DC_8A_n79A DC_11A_n79A |
| DC_1A-8A-20A_n3A | DC_1A_n3A DC_8A_n3A DC_20A_n3A |
| DC_1A-8A-20A_n28A3,8,11,14 | DC_1A_n28A DC_8A_n28A DC_20A_n28A |
| DC_1A-8A-20A_n78A | DC_1A_n78A DC_8A_n78A DC_20A_n78A |
| DC_1A-8A-28A_n3A | DC_1A_n3A DC_8A_n3A DC_28A_n3A |
| DC_1A-8A_n28A-n77A2 | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A |
| DC_1A-8A_n28A-n77(2A)2 | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A |
| DC_1A-8A-28A_n78A | DC_1A_n78A DC_8A_n78A DC_28A_n78A |
| DC_1A-8A_n28A-n78A2 | DC_1A_n28A DC_1A_n78A DC_8A_n28A DC_8A_n78A |
| DC_1A-8A_n28A-n79A2 | DC_1A_n28A DC_1A_n79A DC_8A_n28A DC_8A_n79A |
| DC_1A-8A-32A_n3A | DC_1A_n3A DC_8A_n3A |
| DC_1A-8A-32A_n78A | DC_1A_n78A DC_8A_n78A |
| DC_1A-8A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_8A_n40A DC_8A_n78A |
| DC_1A-8A-40A_n78A DC_1A-8A-40C_n78A | DC_1A_n78A DC_8A_n78A DC_40A_n78A |
| DC_1A-8A-40A_n78(2A) DC_1A-8A-40C_n78(2A) | DC_1A_n78A DC_8A_n78A DC_40A_n78A |
| DC_1A-8A-42A_n3A2 DC_1A-8A-42C_n3A2 | DC_1A_n3A DC_8A_n3A DC_42A_n3A DC_42C_n3A |
| DC_1A-8A-42A_n28A2 DC_1A-8A-42C_n28A2 | DC_1A_n28A DC_8A_n28A DC_42A_n28A DC_42C_n28A |
| DC_1A-8A-42A_n77A7,8 DC_1A-8A-42C_n77A7,8 | DC_1A_n77A DC_8A_n77A |
| DC_1A-8A-42A_n77(2A) 7,8 DC_1A-8A-42C_n77(2A) 7,8 | DC_1A_n77A DC_8A_n77A |
| DC_1A-8A_n77A-n79A DC_1A-8A_n77(2A)-n79A | DC_1A_n77A DC_1A_n79A DC_8A_n77A DC_8A_n79A |
| DC_1A-11A_n3A-n28A | DC_1A_n3A DC_1A_n28A DC_11A_n3A DC_11A_n28A |
| DC_1A-11A_n3A-n77A2 | DC_1A_n3A DC_1A_n77A DC_11A_n3A DC_11A_n77A |
| DC_1A-11A_n3A-n77(2A) 2 | DC_1A_n3A DC_1A_n77A DC_11A_n3A DC_11A_n77A |
| DC_1A-11A_n3A-n79A | DC_1A_n3A DC_1A_n79A DC_11A_n3A DC_11A_n79A |
| DC_1A-11A-18A_n3A | DC_1A_n3A DC_11A_n3A DC_18A_n3A |
| DC_1A-11A-18A_n28A | DC_1A_n28A DC_11A_n28A DC_18A_n28A |
| DC_1A-11A-18A_n41A | DC_1A_n41A DC_11A_n41A DC_18A_n41A |
| DC_1A-11A-18A_n77A | DC_1A_n77A DC_11A_n77A DC_18A_n77A |
| DC_1A-11A-18A_n77(2A) | DC_1A_n77A DC_11A_n77A DC_18A_n77A |
| DC_1A-11A-18A_n78A | DC_1A_n78A DC_11A_n78A DC_18A_n78A |
| DC_1A-11A-18A_n78(2A) | DC_1A_n78A DC_11A_n78A DC_18A_n78A |
| DC_1A-11A_n28A-n77A2 | DC_1A_n28A DC_1A_n77A DC_11A_n28A DC_11A_n77A |
| DC_1A-11A_n28A-n77(2A) 2 | DC_1A_n28A DC_1A_n77A DC_11A_n28A DC_11A_n77A |
| DC_1A-11A_n77A-n79A | DC_1A_n77A DC_1A_n79A DC_11A_n77A DC_11A_n79A |
| DC_1A-11A_n77(2A)-n79A | DC_1A_n77A DC_1A_n79A DC_11A_n77A DC_11A_n79A |
| DC_1A-18A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_18A_n3A DC_18A_n41A |
| DC_1A-18A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_18A_n3A DC_18A_n77A |
| DC_1A-18A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_18A_n3A DC_18A_n78A |
| DC_1A-18A_n28A-n41A | DC_1A_n28A DC_1A_n41A DC_18A_n28A DC_18A_n41A |
| DC_1A-18A-28A_n77A | DC_1A_n77A DC_18A_n77A DC_28A_n77A |
| DC_1A-18A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_18A_n28A DC_18A_n77A |
| DC_1A-18A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_18A_n28A DC_18A_n77A |
| DC_1A-18A-28A_n78A | DC_1A_n78A DC_18A_n78A DC_28A_n78A |
| DC_1A-18A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_18A_n28A DC_18A_n78A |
| DC_1A-18A_n28A-n78(2A) | DC_1A_n28A DC_1A_n78A DC_18A_n28A DC_18A_n78A |
| DC_1A-18A-28A_n79A2 | DC_1A_n79A DC_18A_n79A DC_28A_n79A |
| DC_1A-18A-41A_n3A DC_1A-18A-41C_n3A | DC_1A_n3A DC_18A_n3A DC_41A_n3A DC_41C_n3A |
| DC_1A-18A-41A_n77A DC_1A-18A-41C_n77A | DC_1A_n77A DC_18A_n77A DC_41A_n77A DC_41C_n77A |
| DC_1A-18A_n41A-n77A | DC_1A_n41A DC_1A_n77A DC_18A_n41A DC_18A_n77A |
| DC_1A-18A_n41A-n77(2A) | DC_1A_n41A DC_1A_n77A DC_18A_n41A DC_18A_n77A |
| DC_1A-18A-41A_n78A DC_1A-18A-41C_n78A | DC_1A_n78A DC_18A_n78A DC_41A_n78A DC_41C_n78A |
| DC_1A-18A_n41A-n78A | DC_1A_n41A DC_18A_n41A DC_1A_n78A DC_18A_n78A |
| DC_1A-18A_n41A-n78(2A) | DC_1A_n41A DC_18A_n41A DC_1A_n78A DC_18A_n78A |
| DC_1A-18A-42A_n77A7,8 DC_1A-18A-42C_n77A7,8 | DC_1A_n77A DC_18A_n77A |
| DC_1A-18A-42A_n78A7,8 DC_1A-18A-42C_n78A7,8 | DC_1A_n78A DC_18A_n78A |
| DC_1A-18A-42A_n79A DC_1A-18A-42C_n79A | DC_1A_n79A DC_18A_n79A |
| DC_1A-19A-21A_n77A2,9 DC_1A-19A-21A_n77C2 | DC_1A_n77A9 DC_19A_n77A9 DC_21A_n77A9 |
| DC_1A-19A-21A_n77(2A) 2 | DC_1A_n77A DC_19A_n77A DC_21A_n77A |
| DC_1A-19A-21A_n78A2, 9 DC_1A-19A-21A_n78C2 | DC_1A_n78A9 DC_19A_n78A9 DC_21A_n78A9 |
| DC_1A-19A-21A_n78(2A) 2 | DC_1A_n78A DC_19A_n78A DC_21A_n78A |
| DC_1A-19A-21A_n79A2,9 DC_1A-19A-21A_n79C2 | DC_1A_n79A9 DC_19A_n79A9 DC_21A_n79A9 |
| DC_1A-19A-42A_n77A7,8,9 DC_1A-19A-42A_n77C7,8 DC_1A-19A-42C_n77A7,8,9 DC_1A-19A-42C_n77C7,8 | DC_1A_n77A9 DC_19A_n77A9 |
| DC_1A-19A-42A_n78A7,8,9 DC_1A-19A-42A_n78C7,8 DC_1A-19A-42C_n78A7,8,9 DC_1A-19A-42C_n78C7,8 | DC_1A_n78A9 DC_19A_n78A9 |
| DC_1A-19A-42A_n79A9 DC_1A-19A-42A_n79C DC_1A-19A-42C_n79A9 DC_1A-19A-42C_n79C | DC_1A_n79A9 DC_19A_n79A9 |
| DC_1A-19A_n77A-n79A9 | DC_19A_n77A9 DC_19A_n79A9 |
| DC_1A-19A_n78A-n79A9 | DC_19A_n78A9 DC_19A_n79A9 |
| DC_1A-20A_n3A-n38A | DC_1A_n3A DC_20A_n3A DC_1A_n38A DC_20A_n38A |
| DC_1A-20A_n3A-n78A | DC_1A_n3A DC_20A_n3A DC_1A_n78A DC_20A_n78A |
| DC_1A-20A_n7A-n78A | DC_1A_n7A DC_20A_n7A DC_1A_n78A DC_20A_n78A |
| DC_1A-20A_n8A-n78A | DC_1A_n8A DC_1A_n78A DC_20A_n8A DC_20A_n78A |
| DC_1A-20A-28A_n3A | DC_1A_n3A DC_20A_n3A DC_28A_n3A |
| DC_1A-20A_n28A-n75A | DC_1A_n28A DC_20A_n28A |
| DC_1A-20A-28A_n78A | DC_1A_n78A DC_20A_n78A DC_28A_n78A |
| DC_1A-20A_n28A-n78A2,3,8,14 | DC_1A_n28A DC_1A_n78A DC_20A_n28A DC_20A_n78A |
| DC_1A-20A-32A_n3A | DC_1A_n3A DC_20A_n3A |
| DC_1A-20A-32A_n8A | DC_1A_n8A DC_20A_n8A |
| DC_1A-20A-32A_n28A8,14 | DC_1A_n28A DC_20A_n28A |
| DC_1A-20A-32A_n78A | DC_1A_n78A DC_20A_n78A |
| DC_1A-20A-38A_n3A | DC_1A_n3A DC_20A_n3A |
| DC_1A-20A-(n)38AA | DC_1A_n38A DC_20A_n38A |
| DC_1A-20A-38A_n8A | DC_1A_n8A DC_20A_n8A DC_38A_n8A |
| DC_1A-20A-38A_n78A | DC_1A_n78A DC_20A_n78A |
| DC_1A-20A-38A_n78(2A) | DC_1A_n78A DC_20A_n78A |
| DC_1A-20A_n38A-n78A | DC_1A_n38A DC_20A_n38A DC_1A_n78A DC_20A_n78A |
| DC_1A-20A-40A_n78A DC_1A-20A-40C_n78A | DC_1A_n78A DC_20A_n78A DC_40A_n78A |
| DC_1A-20A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_20A_n41A DC_20A_n78A |
| DC_1A-21A-28A_n77A2 | DC_1A_n77A DC_21A_n77A DC_28A_n77A |
| DC_1A-21A_n28A-n77A2 | DC_1A_n28A DC_1A_n77A DC_21A_n28A DC_21A_n77A |
| DC_1A-21A-28A_n78A2 | DC_1A_n78A DC_21A_n78A DC_28A_n78A |
| DC_1A-21A_n28A-n78A2 | DC_1A_n28A DC_1A_n78A DC_21A_n28A DC_21A_n78A |
| DC_1A-21A-28A_n79A2 | DC_1A_n79A DC_21A_n79A DC_28A_n79A |
| DC_1A-21A_n28A-n79A2 | DC_1A_n28A DC_1A_n79A DC_21A_n28A DC_21A_n79A |
| DC_1A-21A-42A_n77A7,8,9 DC_1A-21A-42A_n77C7,8 DC_1A-21A-42C_n77A7,8,9 DC_1A-21A-42C_n77C7,8 DC_1A-21A-42D_n77A7,8 DC_1A-21A-42D_n77C7,8 | DC_1A_n77A9 DC_21A_n77A9 |
| DC_1A-21A-42A_n78A7,8,9 DC_1A-21A-42A_n78C7,8 DC_1A-21A-42C_n78A7,8,9 DC_1A-21A-42C_n78C7,8 DC_1A-21A-42D_n78A7,8 DC_1A-21A-42D_n78C7,8 | DC_1A_n78A9 DC_21A_n78A9 |
| DC_1A-21A-42A_n79A9 DC_1A-21A-42A_n79C DC_1A-21A-42C_n79A9 DC_1A-21A-42C_n79C DC_1A-21A-42D_n79A DC_1A-21A-42D_n79C | DC_1A_n79A9 DC_21A_n79A9 |
| DC_1A-21A_n77A-n79A9 | DC_1A_n77A9 DC_1A_n79A9 |
| DC_1A-21A_n78A-n79A9 | DC_1A_n78A9 DC_1A_n79A9 |
| DC_1A-28A_n3A-n77A2 | DC_28A_n3A DC_28A_n77A |
| DC_1A-28A_n3A-n78A2 | DC_1A_n3A DC_1A_n78A DC_28A_n3A DC_28A_n78A |
| DC_1A-28A_n5A-n40A | DC_1A_n5A DC_1A_n40A DC_28A_n5A DC_28A_n40A |
| DC_1A-28A_n5A-n78A2 | DC_1A_n5A DC_1A_n78A DC_28A_n5A DC_28A_n78A |
| DC_1A-28A-(n)7AA | DC_1A_n7A DC_28A_n7A |
| DC_1A-28A_n7A-n78A | DC_1A_n7A DC_28A_n7A DC_1A_n78A DC_28A_n78A |
| DC_1A-28A_n7B-n78A | DC_1A_n7A DC_1A_n7B DC_28A_n7A DC_28A_n7B DC_1A_n78A DC_28A_n78A |
| DC_1A-28A-32A_n3A | DC_1A_n3A DC_28A_n3A |
| DC_1A-28A-40A_n78A DC_1A-28A-40C_n78A | DC_1A_n78A DC_28A_n78A DC_40A_n78A |
| DC_1A-28A_n38A-n78A | DC_1A_n38A DC_1A_n78A DC_28A_n38A DC_28A_n78A |
| DC_1A-28A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_28A_n40A DC_28A_n78A |
| DC_1A-28A-42A_n77A7,8 DC_1A-28A-42A_n77C7,8 DC_1A-28A-42C_n77A7,8 DC_1A-28A-42C_n77C7,8 | DC_1A_n77A DC_28A_n77A |
| DC_1A-28A-42A_n78A7,8 DC_1A-28A-42A_n78C7,8 DC_1A-28A-42C_n78A7,8 DC_1A-28A-42C_n78C7,8 | DC_1A_n78A DC_28A_n78A |
| DC_1A-28A-42A_n79A DC_1A-28A-42A_n79C DC_1A-28A-42C_n79A DC_1A-28A-42C_n79C | DC_1A_n79A DC_28A_n79A |
| DC_1A-41A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_41A_n3A |
| DC_1A_n28A-n77A-n79A | DC_1A_n28A DC_1A_n77A DC_1A_n79A |
| DC_1A_n28A-n78A-n79A | DC_1A_n28A DC_1A_n78A DC_1A_n79A |
| DC_1A-38A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_38A_n3A DC_38A_n78A |
| DC_1A-38A_n7A-n78A | DC_1A_n78A |
| DC_1A-38A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_38A_n28A DC_38A_n78A |
| DC_1A_n40A-n78A-n105A | DC_1A_n40A DC_1A_n78A DC_1A_n105A |
| DC_1A-41A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_41A_n3A DC_41A_n77A |
| DC_1A-41C_n3A-n77A | DC_41A_n3A DC_41A_n77A DC_41C_n3A DC_41C_n77A |
| DC_1A-41A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_41A_n3A DC_41A_n78A |
| DC_1A-41C_n3A-n78A | DC_41A_n3A DC_41A_n78A DC_41C_n3A DC_41C_n78A |
| DC_1A-41A_n28A-n41A | DC_1A_n28A DC_1A_n41A DC_41A_n28A |
| DC_1A-41A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_41A_n28A DC_41A_n77A |
| DC_1A-41C_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_41A_n28A DC_41A_n77A DC_41C_n28A DC_41C_n77A |
| DC_1A-41A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_41A_n28A DC_41A_n78A |
| DC_1A-41C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_41A_n28A DC_41A_n78A DC_41C_n28A DC_41C_n78A |
| DC_1A-41A_n41A-n77A | DC_1A_n41A DC_1A_n77A DC_41A_n77A |
| DC_1A-41A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_41A_n78A |
| DC_1A-42A_n3A-n28A2 | DC_1A_n3A DC_1A_n28A DC_42A_n3A DC_42A_n28A |
| DC_1A-42C_n3A-n28A2 | DC_1A_n3A DC_1A_n28A DC_42A_n3A DC_42A_n28A DC_42C_n3A DC_42C_n28A |
| DC_1A-42A_n3A-n77A7,8 | DC_1A_n3A DC_1A_n77A DC_42A_n3A |
| DC_1A-42A_n3A-n77(2A) 7,8 | DC_1A_n3A DC_1A_n77A DC_42A_n3A |
| DC_1A-42C_n3A-n77A7,8 | DC_1A_n3A DC_1A_n77A DC_42A_n3A DC_42C_n3A |
| DC_1A-42C_n3A-n77(2A)7,8 | DC_1A_n3A DC_1A_n77A DC_42A_n3A DC_42C_n3A |
| DC_1A-42A_n28A-n77A7,8 | DC_1A_n28A DC_1A_n77A DC_42A_n28A |
| DC_1A-42A_n28A-n77(2A)7,8 | DC_1A_n28A DC_1A_n77A DC_42A_n28A |
| DC_1A-42C_n28A-n77A7,8 | DC_1A_n28A DC_1A_n77A DC_42A_n28A DC_42C_n28A |
| DC_1A-42C_n28A-n77(2A)7,8 | DC_1A_n28A DC_1A_n77A DC_42A_n28A DC_42C_n28A |
| DC_1A-41A-42A_n77A7,8 DC_1A-41A-42C_n77A7,8 DC_1A-41C-42A_n77A7,8 DC_1A-41C-42C_n77A7,8 | DC_1A_n77A DC_41A_n77A |
| DC_1A-41A-42A_n77(2A)7,8 DC_1A-41A-42C_n77(2A)7,8 | DC_1A_n77A DC_41A_n77A |
| DC_1A-41A-42A_n78A7,8 DC_1A-41A-42C_n78A7,8 DC_1A-41C-42A_n78A7,8 DC_1A-41C-42C_n78A7,8 | DC_1A_n78A DC_41A_n78A |
| DC_1A-41A-42A_n79A DC_1A-41A-42C_n79A DC_1A-41C-42A_n79A DC_1A-41C-42C_n79A | DC_1A_n79A DC_41A_n79A |
| DC_1A-42A_n77A-n79A7,8,9 DC_1A-42C_n77A-n79A7,8,9 | DC_1A_n77A9 DC_1A_n79A9 |
| DC_1A-42A_n78A-n79A7,8,9 DC_1A-42C_n78A-n79A7,8,9 | DC_1A_n78A9 DC_1A_n79A9 |
| DC_2A-4A-7A_n28A | DC_2A_n28A DC_4A_n28A DC_7A_n28A |
| DC_2A-4A-7A_n78A DC_2A-4A-7C_n78A | DC_2A_n78A DC_4A_n78A |
| DC_2A-5A_n2A-n41A | DC_2A_n2A4 DC_2A_n41A DC_5A_n2A DC_5A_n41A |
| DC_2A-5A_n2A-n66A | DC_2A_n2A4 DC_2A_n66A DC_5A_n2A DC_5A_n66A |
| DC_2A-5A_n2A-n77A DC_2A-5A_n2A-n77C | DC_2A_n77A DC_5A_n2A DC_5A_n77A |
| DC_2A-5A_n2A-n78A | DC_5A_n2A DC_2A_n78A DC_5A_n78A |
| DC_2A-5A_n5A-n77A DC_2A-5A_n5A-n77C | DC_2A_n5A DC_2A_n77A DC_5A_n77A |
| DC_2A-5A-7A_n2A | DC_5A_n2A DC_7A_n2A |
| DC_2A-5A-7A_n7A | DC_2A_n7A DC_5A_n7A DC_7A_n7A4 |
| DC_2A-5A-7A_n66A DC_2A-5A-7C_n66A | DC_2A_n66A DC_5A_n66A DC_7A_n66A |
| DC_2A-5A-7A_n77A | DC_2A_n77A DC_5A_n77A DC_7A_n77A |
| DC_2A-5A-7A_n77(2A) | DC_2A_n77A DC_5A_n77A DC_7A_n77A |
| DC_2A-5A-7A_n78A | DC_2A_n78A DC_5A_n78A DC_7A_n78A |
| DC_2A-2A-5A-7A_n66A DC_2A-5A-7A-7A_n66A | DC_2A_n66A DC_5A_n66A DC_7A_n66A |
| DC_2A-5A-7A-(n)66AA DC_2A-5A-7C-(n)66AA | DC_2A_n66A DC_5A_n66A DC_7A_n66A DC_(n)66AA4 |
| DC_2A-5A-7A-7A-(n)66AA | DC_2A_n66A DC_5A_n66A DC_7A_n66A DC_(n)66AA4 |
| DC_2A-5A-7A_n78(2A) | DC_2A_n78A DC_5A_n78A DC_7A_n78A |
| DC_2A-5A-(n)12AA | DC_5A_n12A DC_2A_n12A DC_(n)12AA4 |
| DC_2A-5A_n41A-n66A | DC_2A_n41A DC_2A_n66A DC_5A_n41A DC_5A_n66A |
| DC_2A-12A-(n)5AA | DC_2A_n5A DC_12A_n5A DC_(n)5AA4 |
| DC_2A-5A-30A_n2A | DC_2A_n2A4 DC_5A_n2A DC_30A_n2A |
| DC_2A-5A-30A_n5A | DC_2A_n5A DC_30A_n5A |
| DC_2A-5A-30A_n66A | DC_2A_n66A DC_5A_n66A DC_30A_n66A |
| DC_2A-2A-5A-30A_n66A | DC_2A_n66A DC_5A_n66A DC_30A_n66A |
| DC_2A-5A-30A_n77A9 DC_2A-2A-5A-30A_n77A9 | DC_2A_n77A9 DC_5A_n77A9 DC_30A_n77A9 |
| DC_2A-5A-30A_n77(2A) 9 | DC_2A_n77A9 DC_5A_n77A9 DC_30A_n77A9 |
| DC_2A-5A-48A_n12A | DC_2A_n12A DC_5A_n12A DC_48A_n12A |
| DC_2A-5A-48A_n77A7,8,9 DC_2A-5A-48A_n77C7,8,9 DC_2A-5A-48C_n77A7,8,9 DC_2A-5A-48C_n77C7,8,9 | DC_2A_n77A DC_5A_n77A |
| DC_2A-5A-66A_n2A DC_2A-5B-66A_n2A | DC_2A_n2A4 DC_5A_n2A DC_66A_n2A |
| DC_2A-5A-5A-66A_n2A | DC_2A_n2A4 DC_5A_n2A DC_66A_n2A |
| DC_2A-5A-66A-66A_n2A DC_2A-5B-66A-66A_n2A | DC_2A_n2A4 DC_5A_n2A DC_66A_n2A |
| DC_2A-5A-5A-66A-66A_n2A | DC_2A_n2A4 DC_5A_n2A DC_66A_n2A |
| DC_2A-5A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-2A-5A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-2A-5A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-5A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-5A-66A_n7A | DC_2A_n7A DC_5A_n7A DC_66A_n7A |
| DC_2A-5A-66A-66A_n7A | DC_2A_n7A DC_5A_n7A DC_66A_n7A |
| DC_2A-5A-66A_n12A | DC_2A_n12A DC_5A_n12A DC_66A_n12A |
| DC_2A-5A-66A_n30A | DC_2A_n30A DC_5A_n30A DC_66A_n30A |
| DC_2A-2A-5A-66A_n30A | DC_2A_n30A DC_5A_n30A DC_66A_n30A |
| DC_2A-5A-66A-66A_n30A | DC_2A_n30A DC_5A_n30A DC_66A_n30A |
| DC_2A-5A-66A_n41A DC_2A-2A-5A-66A_n41A | DC_2A_n41A DC_5A_n41A DC_66A_n41A |
| DC_2A-5A-66A_n48A DC_2A-5A-66A_n48B | DC_2A_n48A DC_5A_n48A DC_66A_n48A |
| DC_2A-5A-66A-66A_n48A DC_2A-5A-66A-66A_n48B | DC_2A_n48A DC_5A_n48A DC_66A_n48A |
| DC_2A-5A-66A_n66A DC_2A-5B-66A_n66A | DC_2A_n66A DC_5A_n66A DC_66A_n66A4 |
| DC_2A-5A-(n)66AA | DC_2A_n66A DC_5A_n66A DC_(n)66AA4 |
| DC_2A-2A-5A-(n)66AA | DC_2A_n66A DC_5A_n66A DC_(n)66AA4 |
| DC_2A-5A-5A-66A_n66A | DC_2A_n66A DC_5A_n66A |
| DC_2A-2A-5A-66A_n66A | DC_2A_n66A DC_5A_n66A |
| DC_2A-5A-66A-66A_n66A DC_2A-5B-66A-66A_n66A | DC_2A_n66A DC_5A_n66A |
| DC_2A-5A-66A-(n)66AA | DC_2A_n66A DC_5A_n66A DC_66A_n66A4 DC_(n)66AA4 |
| DC_2A-2A-5A-66A-(n)66AA | DC_2A_n66A DC_5A_n66A DC_66A_n66A4 DC_(n)66AA4 |
| DC_2A-2A-5A-66A-66A_n66A | DC_2A_n66A DC_5A_n66A |
| DC_2A-5A-5A-66A-66A_n66A | DC_2A_n66A DC_5A_n66A |
| DC_2A-5A-66A_n71A | DC_2A_n71A DC_5A_n71A DC_66A_n71A |
| DC_2A-5A-66A_n77A9 DC_2A-5A-66A_n77C9 DC_2A-2A-5A-66A_n77C9 DC_2A-5A-66A-66A_n77C9 | DC_2A_n77A9 DC_5A_n77A9 DC_66A_n77A9 |
| DC_2A-5A-66A_n77(2A) 9 | DC_2A_n77A9 DC_5A_n77A9 DC_66A_n77A9 |
| DC_2A-2A-5A-66A_n77A9 | DC_2A_n77A9 DC_5A_n77A9 DC_66A_n77A9 |
| DC_2A-5A-66A-66A_n77A9 | DC_2A_n77A9 DC_5A_n77A9 DC_66A_n77A9 |
| DC_2A-5A-66A_n78A | DC_2A_n78A DC_5A_n78A DC_66A_n78A |
| DC_2A-5A-66A_n78(2A) | DC_2A_n78A DC_5A_n78A DC_66A_n78A |
| DC_2A-5A_n66A-n77A DC_2A-5A_n66A-n77C | DC_2A_n66A DC_2A_n77A DC_5A_n66A DC_5A_n77A |
| DC_2A-5A_n66A-n78A | DC_2A_n66A DC_5A_n66A DC_2A_n78A DC_5A_n78A |
| DC_2A-7A_n2A-n66A | DC_2A_n2A4 DC_2A_n66A DC_7A_n2A DC_7A_n66A |
| DC_2A-7A_n2A-n66A | DC_2A_n2A4 DC_2A_n66A DC_7A_n2A DC_7A_n66A |
| DC_2A-7A_n2A-n71A | DC_2A_n71A DC_7A_n2A DC_7A_n71A |
| DC_2A-7A_n2A-n77A | DC_2A_n2A4 DC_2A_n77A DC_7A_n2A DC_7A_n77A |
| DC_2A-7A_n2A-n78A | DC_7A_n2A DC_2A_n78A DC_7A_n78A |
| DC_2A-7A-12A_n2A | DC_7A_n2A DC_12A_n2A |
| DC_2A-7A-12A_n66A | DC_2A_n66A DC_7A_n66A DC_12A_n66A |
| DC_2A-2A-7A-12A_n66A | DC_2A_n66A DC_7A_n66A DC_12A_n66A |
| DC_2A-7A-12A_n77A | DC_2A_n77A DC_7A_n77A DC_12A_n77A |
| DC_2A-7A_n12A-n77A | DC_2A_n12A DC_2A_n77A DC_7A_n12A DC_7A_n77A |
| DC_2A-7A-12A_n77(2A) | DC_2A_n77A DC_7A_n77A DC_12A_n77A |
| DC_2A-7A-12A_n78A | DC_2A_n78A DC_7A_n78A DC_12A_n78A |
| DC_2A-2A-7A-12A_n78A | DC_2A_n78A DC_7A_n78A DC_12A_n78A |
| DC_2A-7A-12A_n78(2A) | DC_2A_n78A DC_7A_n78A DC_12A_n78A |
| DC_2A-7A-13A_n25A7,8 | DC_7A_n25A DC_13A_n25A |
| DC_2A-7A-7A-13A_n25A7,8 | DC_7A_n25A DC_13A_n25A |
| DC_2A-7C-13A_n25A7,8 | DC_7A_n25A DC_13A_n25A |
| DC_2A-7A-13A_n66A DC_2A-7C-13A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A |
| DC_2A-7C-13A-(n)66AA | DC_2A_n66A DC_7A_n66A DC_13A_n66A DC_(n)66AA4 |
| DC_2A-2A-7C-13A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A |
| DC_2A-7A-7A-13A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A |
| DC_2A-2A-7A-13A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A |
| DC_2A-2A-7A-7A-13A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A |
| DC_2A-7A_n25A-n66A8,14 | DC_2A_n66A DC_7A_n25A DC_7A_n66A |
| DC_2A-7A-7A_n25A-n66A8,14 | DC_2A_n66A DC_7A_n25A DC_7A_n66A |
| DC_2A-7C_n25A-n66A8,14 | DC_2A_n66A DC_7A_n25A DC_7A_n66A |
| DC_2A-7A-28A_n7A | DC_2A_n7A DC_7A_n7A4 DC_28A_n7A |
| DC_2A-7A-28A_n66A DC_2A-7C-28A_n66A | DC_2A_n66A DC_7A_n66A DC_28A_n66A |
| DC_2A-7A-28A_n78A DC_2A-7C-28A_n78A | DC_2A_n78A DC_7A_n78A DC_7C_n78A DC_28A_n78A |
| DC_2A-7A-28A_n78(2A) DC_2A-7C-28A_n78(2A) | DC_2A_n78A DC_28A_n78A |
| DC_2A-7A_n38A-n66A DC_2A-7C_n38A-n66A | DC_2A_n38A DC_2A_n66A DC_7A_n66A |
| DC_2A-7A-7A_n38A-n66A | DC_2A_n38A DC_2A_n66A DC_7A_n66A |
| DC_2A-7A-29A_n78A DC_2A-7C-29A_n78A | DC_2A_n78A DC_7A_n78A |
| DC_2A-7A-7A-29A_n78A | DC_2A_n78A DC_7A_n78A |
| DC_2A-7A-38A_n78A DC_2A-7C-38A_n78A | DC_2A_n78A |
| DC_2A-7A_n38A-n78A DC_2A-7C_n38A-n78A | DC_2A_n78A |
| DC_2A-7A-7A_n38A-n78A | DC_2A_n78A |
| DC_2A-7A-66A_n2A | DC_7A_n2A DC_66A_n2A |
| DC_2A-7A-66A_n7A | DC_2A_n7A DC_7A_n7A4 DC_66A_n7A |
| DC_2A-7A-66A-66A_n7A | DC_2A_n7A DC_7A_n7A4 DC_66A_n7A |
| DC_2A-7A-66A_n12A | DC_2A_n12A DC_7A_n12A DC_66A_n12A |
| DC_2A-7A-66A_n25A7,8 | DC_7A_n25A DC_66A_n25A |
| DC_2A-7A-7A-66A_n25A7,8 | DC_7A_n25A DC_66A_n25A |
| DC_2A-7C-66A_n25A7,8 | DC_7A_n25A DC_66A_n25A |
| DC_2A-7A-66A_n28A | DC_2A_n28A DC_7A_n28A DC_66A_n28A |
| DC_2A-7A-66A_n38A | 2A5 66A5 |
| DC_2A-2A-7A-66A_n38A | 2A5 66A5 |
| DC_2A-7A-66A_n66A DC_2A-7C-66A_n66A | DC_2A_n66A DC_7A_n66A DC_66A_n66A4 |
| DC_2A-7A-(n)66AA DC_2A-7C-(n)66AA | DC_2A_n66A DC_7A_n66A DC_(n)66AA4 |
| DC_2A-7A-7A-(n)66AA | DC_2A_n66A DC_7A_n66A DC_(n)66AA4 |
| DC_2A-7A-7A-66A_n66A | DC_2A_n66A DC_7A_n66A DC_66A_n66A4 |
| DC_2A-7A-66A-66A_n66A | DC_2A_n66A DC_7A_n66A DC_66A_n66A4 |
| DC_2A-7A-66A-(n)66AA | DC_2A_n66A DC_7A_n66A DC_66A_n66A4 DC_(n)66AA4 |
| DC_2A-7A-7A-66A-(n)66AA | DC_2A_n66A DC_7A_n66A DC_66A_n66A4 DC_(n)66AA4 |
| DC_2A-7A-7A-66A-66A_n66A | DC_2A_n66A DC_7A_n66A DC_66A_n66A4 |
| DC_2A-7A-66A_n71A | DC_2A_n71A DC_7A_n71A DC_66A_n71A |
| DC_2A-2A-7A-66A_n71A | DC_2A_n71A DC_7A_n71A DC_66A_n71A |
| DC_2A-7A_n66A-n71A | DC_2A_n66A DC_2A_n71A DC_7A_n66A DC_7A_n71A |
| DC_2A-7A-66A_n77A DC_2A-7C-66A_n77A | DC_2A_n77A DC_7A_n77A DC_66A_n77A |
| DC_2A-7A-66A_n77(2A) DC_2A-7C-66A_n77(2A) | DC_2A_n77A DC_7A_n77A DC_66A_n77A |
| DC_2A-7A-7A-66A_n77A | DC_2A_n77A DC_7A_n77A DC_66A_n77A |
| DC_2A-7A-7A-66A_n77(2A) | DC_2A_n77A DC_7A_n77A DC_66A_n77A |
| DC_2A-7A_n66A-n77A DC_2A-7C_n66A-n77A DC_2A-7A-7A_n66A-n77A | DC_2A_n66A DC_7A_n66A DC_2A_n77A DC_7A_n77A |
| DC_2A-7A-66A_n78A9 DC_2A-7C-66A_n78A9 | DC_2A_n78A9 DC_7A_n78A9 DC_66A_n78A9 |
| DC_2A-2A-7A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-7A_n66A-n78A DC_2A-7C_n66A-n78A | DC_2A_n66A DC_2A_n78A DC_7A_n66A DC_7A_n78A |
| DC_2A-7A-66A_n78(2A)9 DC_2A-7C-66A_n78(2A)9 | DC_2A_n78A9 DC_7A_n78A9 DC_66A_n78A9 |
| DC_2A-7A-7A_n66A-n78A | DC_2A_n66A DC_2A_n78A DC_7A_n66A DC_7A_n78A |
| DC_2A-7A-7A-66A_n78A9 | DC_2A_n78A9 DC_7A_n78A9 DC_66A_n78A9 |
| DC_2A-7A-66A-66A_n78A9 DC_2A-7C-66A-66A_n78A9 | DC_2A_n78A9 DC_7A_n78A9 DC_66A_n78A9 |
| DC_2A-7A-66A-66A_n78(2A) 9 DC_2A-7C-66A-66A_n78(2A) 9 | DC_2A_n78A9 DC_7A_n78A9 DC_66A_n78A9 |
| DC_2A-7A-7A-66A_n78(2A) 9 | DC_2A_n78A9 DC_7A_n78A9 DC_66A_n78A9 |
| DC_2A-7A-7A-66A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-7A-7A-66A-66A_n78(2A) | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-7A-71A_n2A | DC_7A_n2A DC_71A_n2A |
| DC_2A-7A-71A_n66A | DC_2A_n66A DC_7A_n66A DC_71A_n66A |
| DC_2A-2A-7A-71A_n66A | DC_2A_n66A DC_7A_n66A DC_71A_n66A |
| DC_2A-7A-71A_n77A | DC_2A_n77A DC_7A_n77A DC_71A_n77A |
| DC_2A-7A-71A_n77(2A) | DC_2A_n77A DC_7A_n77A DC_71A_n77A |
| DC_2A-7A_n71A-n77A | DC_2A_n71A DC_2A_n77A DC_7A_n71A DC_7A_n77A |
| DC_2A-7A-71A_n78A | DC_2A_n78A DC_7A_n78A DC_71A_n78A |
| DC_2A-2A-7A-71A_n78A | DC_2A_n78A DC_7A_n78A DC_71A_n78A |
| DC_2A-7A-71A_n78(2A) | DC_2A_n78A DC_7A_n78A DC_71A_n78A |
| DC_2A-7A_n71A-n78A | DC_2A_n71A DC_7A_n71A DC_2A_n78A DC_7A_n78A |
| DC_2A-12A_n2A-n41A | DC_2A_n41A DC_12A_n2A DC_12A_n41A |
| DC_2A-12A_n2A-n66A | DC_2A_n2A4 DC_2A_n66A DC_12A_n2A DC_12A_n66A |
| DC_2A-12A_n2A-n77A | DC_2A_n2A4 DC_2A_n77A DC_12A_n2A DC_12A_n77A |
| DC_2A-12A_n2A-n78A | DC_12A_n2A DC_2A_n78A DC_7A_n78A |
| DC_2A-12A-30A_n2A | DC_12A_n2A DC_30A_n2A |
| DC_2A-12A_n41A-n66A | DC_2A_n41A DC_2A_n66A DC_12A_n41A DC_12A_n66A |
| DC_2A-12A-48A_n5A | DC_2A_n5A DC_12A_n5A DC_48A_n5A |
| DC_2A-12A-66A_n5A | DC_2A_n5A DC_12A_n5A DC_66A_n5A |
| DC_2A-12A-30A_n66A | DC_2A_n66A DC_12A_n66A DC_30A_n66A |
| DC_2A-2A-12A-30A_n66A | DC_2A_n66A DC_12A_n66A DC_30A_n66A |
| DC_2A-12A-30A_n77A9 DC_2A-2A-12A-30A_n77A9 | DC_2A_n77A9 DC_12A_n77A9 DC_30A_n77A9 |
| DC_2A-12A-30A_n77(2A)9 | DC_2A_n77A9 DC_12A_n77A9 DC_30A_n77A9 |
| DC_2A-12A-66A_n2A | DC_12A_n2A DC_66A_n2A |
| DC_2A-12A-66A-66A_n2A | DC_12A_n2A DC_66A_n2A |
| DC_2A-12A-66A_n7A | DC_2A_n7A DC_12A_n7A DC_66A_n7A |
| DC_2A-12A-66A_n30A | DC_2A_n30A DC_12A_n30A DC_66A_n30A |
| DC_2A-2A-12A-66A_n30A | DC_2A_n30A DC_12A_n30A DC_66A_n30A |
| DC_2A-12A-66A-66A_n30A | DC_2A_n30A DC_12A_n30A DC_66A_n30A |
| DC_2A-12A-66A_n41A | DC_2A_n41A DC_12A_n41A DC_66A_n41A |
| DC_2A-2A-12A-66A_n41A | DC_2A_n41A DC_12A_n41A DC_66A_n41A |
| DC_2A-12A-66A_n66A | DC_2A_n66A DC_12A_n66A DC_66A_n66A4 |
| DC_2A-12A-(n)66AA | DC_2A_n66A DC_12A_n66A DC_(n)66AA4 |
| DC_2A-2A-12A-(n)66AA | DC_2A_n66A DC_12A_n66A DC_(n)66AA4 |
| DC_2A-2A-12A-66A_n66A | DC_2A_n66A DC_12A_n66A DC_66A_n66A4 |
| DC_2A-12A-66A_n77A9 DC_2A-2A-12A-66A_n77A9 DC_2A-12A-66A-66A_n77A9 | DC_2A_n77A9 DC_12A_n77A9 DC_66A_n77A9 |
| DC_2A-12A-66A_n77(2A) 9 | DC_2A_n77A9 DC_12A_n77A9 DC_66A_n77A9 |
| DC_2A-12A_n66A-n77A | DC_2A_n66A DC_2A_n77A DC_12A_n66A DC_12A_n77A |
| DC_2A-12A-66A_n78A | DC_2A_n78A DC_12A_n78A DC_66A_n78A |
| DC_2A-2A-12A-66A_n78A | DC_2A_n78A DC_12A_n78A DC_66A_n78A |
| DC_2A-12A-66A_n78(2A) | DC_2A_n78A DC_12A_n78A DC_66A_n78A |
| DC_2A-12A_n66A-n78A | DC_2A_n66A DC_12A_n66A DC_2A_n78A DC_12A_n78A |
| DC_2A-13A_n2A-n77A DC_2A-13A_n2A-n77C | DC_2A_n77A DC_13A_n2A DC_13A_n77A |
| DC_2A-13A_n5A-n77A9 DC_2A-2A-13A_n5A-n77A9 DC_2A-13A_n5A-n77C9 | DC_2A_n5A DC_2A_n77A DC_13A_n77A |
| DC_2A-13A_n25A-n66A8,14 | DC_2A_n66A DC_13A_n25A DC_13A_n66A |
| DC_2A-13A-48A_n77A7,8,9 DC_2A-13A-48A_n77C7,8,9 DC_2A-13A-48C_n77A7,8,9 DC_2A-13A-48C_n77C7,8,9 | DC_2A_n77A9 DC_13A_n77A |
| DC_2A-13A-66A_n2A | DC_13A_n2A DC_66A_n2A |
| DC_2A-13A-66A-66A_n2A | DC_13A_n2A DC_66A_n2A |
| DC_2A-13A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-2A-13A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-13A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-2A-13A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-13A-66A_n48A DC_2A-13A-66A_n48B | DC_2A_n48A DC_13A_n48A DC_66A_n48A |
| DC_2A-13A-66A-66A_n48A DC_2A-13A-66A-66A_n48B | DC_2A_n48A DC_13A_n48A DC_66A_n48A |
| DC_2A-13A-66A_n66A DC_2A-2A-13A-66A_n66A DC_2A-13A-66A-66A_n66A DC_2A-2A-13A-66A-66A_n66A | DC_2A_n66A DC_13A_n66A DC_66A_n66A4 |
| DC_2A-13A-(n)66AA | DC_2A_n66A DC_13A_n66A DC_(n)66AA4 |
| DC_2A-2A-13A-(n)66AA | DC_2A_n66A DC_13A_n66A DC_(n)66AA4 |
| DC_2A-13A-66A-(n)66AA | DC_2A_n66A DC_13A_n66A DC_66A_n66A4 DC_(n)66AA4 |
| DC_2A-2A-13A-66A-(n)66AA | DC_2A_n66A DC_13A_n66A DC_66A_n66A4 DC_(n)66AA4 |
| DC_2A-13A-66B_n66A | DC_2A_n66A DC_13A_n66A |
| DC_2A-13A-66A_n77A9 DC_2A-13A-66A_n77C9 DC_2A-2A-13A-66A_n77C9 DC_2A-2A-13A-66A-66A_n77A DC_2A-13A-66A-66A_n77C9 | DC_2A_n77A9 DC_13A_n77A9 DC_66A_n77A9 |
| DC_2A-2A-13A-66A_n77A9 | DC_2A_n77A9 DC_13A_n77A9 DC_66A_n77A9 |
| DC_2A-13A-66A-66A_n77A9 | DC_2A_n77A9 DC_13A_n77A9 DC_66A_n77A9 |
| DC_2A-13A_n66A-n77A9 DC_2A-13A_n66A-n77C9 DC_2A-2A-13A_n66A-n77A9 | DC_2A_n66A DC_2A_n77A9 DC_13A_n66A DC_13A_n77A9 |
| DC_2A-14A-30A_n2A | DC_2A_n2A4 DC_14A_n2A DC_30A_n2A |
| DC_2A-14A-30A_n66A | DC_2A_n66A DC_14A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_2A-2A-14A-30A_n66A | DC_2A_n66A DC_14A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_2A-14A-30A_n77A9 DC_2A-2A-14A-30A_n77A9 | DC_2A_n77A9 DC_14A_n77A9 DC_30A_n77A9 |
| DC_2A-14A-30A_n77(2A) 9 | DC_2A_n77A9 DC_14A_n77A9 DC_30A_n77A9 |
| DC_2A-14A-66A_n2A | DC_2A_n2A4 DC_14A_n2A DC_66A_n2A |
| DC_2A-14A-66A-66A_n2A | DC_2A_n2A4 DC_14A_n2A DC_66A_n2A |
| DC_2A-14A-66A_n30A | DC_2A_n30A DC_14A_n30A DC_66A_n30A |
| DC_2A-2A-14A-66A_n30A | DC_2A_n30A DC_14A_n30A DC_66A_n30A |
| DC_2A-14A-66A-66A_n30A | DC_2A_n30A DC_14A_n30A DC_66A_n30A |
| DC_2A-14A-66A_n66A | DC_2A_n66A DC_14A_n66A DC_66A_n66A4 |
| DC_2A-2A-14A-66A_n66A | DC_2A_n66A DC_14A_n66A DC_66A_n66A4 |
| DC_2A-14A-66A_n77A9 DC_2A-2A-14A-66A_n77A9 DC_2A-14A-66A-66A_n77A9 | DC_2A_n77A9 DC_14A_n77A9 DC_66A_n77A9 |
| DC_2A-14A-66A_n77(2A) 9 | DC_2A_n77A9 DC_14A_n77A9 DC_66A_n77A9 |
| DC_2A-28A-66A_n7A | DC_2A_n7A DC_28A_n7A DC_66A_n7A |
| DC_2A-28A-66A_n66A | DC_2A_n66A DC_28A_n66A DC_66A_n66A4 |
| DC_2A-29A-30A_n2A | DC_2A_n2A4 DC_30A_n2A |
| DC_2A-29A-30A_n66A | DC_2A_n66A DC_30A_n66A |
| DC_2A-2A-29A-30A_n66A | DC_2A_n66A DC_30A_n66A |
| DC_2A-29A-30A_n77A9 DC_2A-2A-29A-30A_n77A9 | DC_2A_n77A9 DC_30A_n77A9 |
| DC_2A-29A-66A_n2A | DC_2A_n2A4 DC_66A_n2A |
| DC_2A-29A-66A-66A_n2A | DC_2A_n2A4 DC_66A_n2A |
| DC_2A-29A-66A_n30A | DC_2A_n30A DC_66A_n30A |
| DC_2A-2A-29A-66A_n30A | DC_2A_n30A DC_66A_n30A |
| DC_2A-29A-66A-66A_n30A | DC_2A_n30A DC_66A_n30A |
| DC_2A-29A-66A_n66A | DC_2A_n66A DC_66A_n66A4 |
| DC_2A-29A-(n)66AA | DC_2A_n66A DC_(n)66AA4 |
| DC_2A-2A-29A-(n)66AA | DC_2A_n66A DC_(n)66AA4 |
| DC_2A-2A-29A-66A_n66A | DC_2A_n66A DC_66A_n66A4 |
| DC_2A-29A-66A_n77A9 | DC_2A_n77A9 DC_66A_n77A9 |
| DC_2A-29A-66A_n78A | DC_2A_n78A DC_66A_n78A |
| DC_2A-30A-(n)5AA DC_2A-2A-30A-(n)5AA | DC_2A_n5A DC_30A_n5A DC_(n)5AA4 |
| DC_2A-30A-66A_n2A | DC_2A_n2A4 DC_30A_n2A DC_66A_n2A |
| DC_2A-30A-66A-66A_n2A | DC_2A_n2A4 DC_30A_n2A DC_66A_n2A |
| DC_2A-30A-66A_n5A | DC_2A_n5A DC_30A_n5A DC_66A_n5A |
| DC_2A-2A-30A-66A_n5A | DC_2A_n5A DC_30A_n5A DC_66A_n5A |
| DC_2A-30A-66A-66A_n5A | DC_2A_n5A DC_30A_n5A DC_66A_n5A |
| DC_2A-30A-66A_n66A | DC_2A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_2A-2A-30A-66A_n66A | DC_2A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_2A-30A-66A_n77A9 DC_2A-2A-30A-66A_n77A9 DC_2A-30A-66A-66A_n77A9 | DC_2A_n77A9 DC_30A_n77A9 DC_66A_n77A9 |
| DC_2A-30A-66A_n77(2A) 9 | DC_2A_n77A9 DC_30A_n77A9 DC_66A_n77A9 |
| DC_2A-46A_n41A-n66A DC_2A-46C_n41A-n66A DC_2A-46D_n41A-n66A | DC_2A_n41A DC_2A_n66A |
| DC_2A-46A_n41A-n71A DC_2A-46C_n41A-n71A DC_2A-46D_n41A-n71A | DC_2A_n41A DC_2A_n71A |
| DC_2A-46A_n41(2A)-n71A DC_2A-46C_n41(2A)-n71A DC_2A-46D_n41(2A)-n71A | DC_2A_n41A DC_2A_n71A |
| DC_2A-46A-48A_n2A DC_2A-46C-48A_n2A DC_2A-46D-48A_n2A DC_2A-46E-48A_n2A | DC_2A_n2A4 DC_48A_n2A |
| DC_2A-46A-48A_n5A DC_2A-46C-48A_n5A DC_2A-46D-48A_n5A DC_2A-46E-48A_n5A | DC_2A_n5A DC_48A_n5A |
| DC_2A-46A-48A_n66A DC_2A-46C-48A_n66A DC_2A-46D-48A_n66A DC_2A-46E-48A_n66A | DC_2A_n66A DC_48A_n66A |
| DC_2A-46A-66A_n5A DC_2A-46C-66A_n5A DC_2A-46D-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-46A-66A_n41A DC_2A-46C-66A_n41A DC_2A-46D-66A_n41A | DC_2A_n41A DC_66A_n41A |
| DC_2A-46A-66A_n41(2A) DC_2A-46C-66A_n41(2A) DC_2A-46D-66A_n41(2A) | DC_2A_n41A DC_66A_n41A |
| DC_2A-46A-66A_n71A DC_2A-46C-66A_n71A DC_2A-46D-66A_n71A | DC_2A_n71A DC_66A_n71A |
| DC_2A-48A-(n)5AA | DC_2A_n5A DC_48A_n5A DC_(n)5AA4 |
| DC_2A-46A_n66A-n71A DC_2A-46C_n66A-n71A DC_2A-46D_n66A-n71A | DC_2A_n66A DC_2A_n71A |
| DC_2A-48A_n48A-n66A | DC_2A_n48A DC_2A_n66A DC_48A_n66A |
| DC_2A-48A-66A_n2A DC_2A-48C-66A_n2A DC_2A-48D-66A_n2A DC_2A-48E-66A_n2A | DC_66A_n2A DC_48A_n2A DC_2A_n2A4 |
| DC_2A-48A-66A_n5A | DC_2A_n5A DC_48A_n5A DC_66A_n5A |
| DC_2A-48C-66A_n5A DC_2A-48D-66A_n5A DC_2A-48E-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-48A-66A_n12A | DC_2A_n12A DC_48A_n12A DC_66A_n12A |
| DC_2A-48A-66A_n66A DC_2A-48C-66A_n66A DC_2A-48D-66A_n66A DC_2A-48E-66A_n66A | DC_66A_n66A4 DC_48A_n66A DC_2A_n66A |
| DC_2A-48A-66A_n71A | DC_2A_n71A DC_48A_n71A DC_66A_n71A |
| DC_2A-48A-66A_n77A7,8,9 DC_2A-48C-66A_n77A7,8,9 DC_2A-48A-66A_n77C7,8,9 DC_2A-48C-66A_n77C7,8,9 DC_2A-48D-66A_n77A7,8,9 DC_2A-48E-66A_n77A7,8,9 | DC_2A_n77A9 DC_66A_n77A9 |
| DC_2A-66A_n2A-n41A | DC_2A_n2A4 DC_2A_n41A DC_66A_n2A DC_66A_n41A |
| DC_2A-66A_n2A-n66A | DC_2A_n2A4 DC_2A_n66A DC_66A_n2A DC_66A_n66A4 |
| DC_2A-66A_n2A-n71A | DC_2A_n2A4 DC_2A_n71A DC_66A_n2A DC_66A_n71A |
| DC_2A-66A_n2A-n77A DC_2A-66A_n2A-n77C | DC_2A_n77A DC_66A_n2A DC_66A_n77A |
| DC_2A-66A-66A_n2A-n77A | DC_2A_n77A DC_66A_n2A DC_66A_n77A |
| DC_2A-66A-(n)5AA DC_2A-2A-66A-(n)5AA DC_2A-66A-66A-(n)5AA | DC_2A_n5A DC_66A_n5A DC_(n)5AA4 |
| DC_2A-66A_n2A-n78A | DC_66A_n2A DC_2A_n78A DC_66A_n78A |
| DC_2A-66A_n5A-n77A9 DC_2A-2A-66A_n5A-n77A9 DC_2A-66A-66A_n5A-n77A9 DC_2A-66A_n5A-n77C9 | DC_2A_n5A DC_2A_n77A9 DC_5A_n77A DC_66A_n5A DC_66A_n77A9 |
| DC_2A-66A_n12A-n77A | DC_2A_n12A DC_2A_n77A DC_66A_n12A DC_66A_n77A |
| DC_2A-66A_n12A-n78A | DC_2A_n12A DC_2A_n78A DC_66A_n12A DC_66A_n78A |
| DC_2A-66A_n25A-n66A7,8 | DC_2A_n66A DC_66A_n25A |
| DC_2A-66A_n38A-n78A | DC_2A_n38A DC_2A_n78A DC_66A_n38A DC_66A_n78A |
| DC_2A-66A_n66A-n71A | DC_2A_n66A DC_2A_n71A DC_66A_n66A4 DC_66A_n71A |
| DC_2A-(n)66AA-n78A | DC_2A_n66A DC_2A_n78A DC_66A_n78A DC_(n)66AA2 |
| DC_2A-66A-71A_n38A | DC_2A_n38A DC_66A_n38A DC_71A_n38A |
| DC_2A-2A-66A-71A_n38A | DC_2A_n38A DC_66A_n38A DC_71A_n38A |
| DC_2A-66A-71A_n41A | DC_2A_n41A DC_66A_n41A DC_71A_n41A |
| DC_2A-2A-66A-71A_n41A | DC_2A_n41A DC_66A_n41A DC_71A_n41A |
| DC_2A-66A-71A_n66A | DC_2A_n66A DC_66A_n66A4 DC_71A_n66A |
| DC_2A-66A-71A_n71A | DC_2A_n71A DC_66A_n71A |
| DC_2A-66A-71A_n77A | DC_2A_n77A DC_66A_n77A DC_71A_n77A |
| DC_2A-66A-71A_n77(2A) | DC_2A_n77A DC_66A_n77A DC_71A_n77A |
| DC_2A-66A_n71A-n77A | DC_2A_n71A DC_2A_n77A DC_66A_n71A DC_66A_n77A |
| DC_2A-66A-71A_n78A | DC_2A_n78A DC_66A_n78A DC_71A_n78A |
| DC_2A-2A-66A-71A_n78A | DC_2A_n78A DC_66A_n78A DC_71A_n78A |
| DC_2A-66A-71A_n78(2A) | DC_2A_n78A DC_66A_n78A DC_71A_n78A |
| DC_2A-66A-(n)71AA DC_2A-66C-(n)71AA | DC_2A_n71A DC_66A_n71A DC_(n)71AA |
| DC_2A-66A_n41A-n71A DC_2A-66A_n41C-n71A | DC_2A_n41A DC_2A_n71A DC_66A_n41A DC_66A_n71A |
| DC_2A-66A_n41(2A)-n71A | DC_2A_n41A DC_2A_n71A DC_66A_n41A DC_66A_n71A |
| DC_2A-66A_n66A-n77A9 DC_2A-2A-66A_n66A-n77A9 DC_2A-66A_n66A-n77C9 | DC_2A_n66A DC_2A_n77A9 DC_66A_n77A9 |
| DC_2A-66A_n66A-n78A | DC_2A_n66A DC_2A_n78A DC_66A_n66A4 |
| DC_2A-66A-71A_n2A | DC_66A_n2A DC_71A_n2A |
| DC_2A-66A_n71A-n78A | DC_2A_n71A DC_66A_n71A DC_2A_n78A DC_66A_n78A |
| DC_2A-71A_n2A-n41A | DC_2A_n41A DC_71A_n2A DC_71A_n41A |
| DC_2A-71A_n2A-n66A | DC_2A_n2A4 DC_2A_n66A DC_71A_n2A DC_71A_n66A |
| DC_2A-71A_n2A-n77A | DC_2A_n2A4 DC_2A_n77A DC_71A_n2A DC_71A_n77A |
| DC_2A-71A_n2A-n78A | DC_71A_n2A DC_2A_n78A DC_7A_n78A |
| DC_2A-71A_n41A-n66A | DC_2A_n41A DC_2A_n66A DC_71A_n41A DC_71A_n66A |
| DC_2A-71A_n66A-n77A | DC_2A_n66A DC_2A_n77A DC_71A_n66A DC_71A_n77A |
| DC_2A-71A_n66A-n78A | DC_2A_n66A DC_71A_n66A DC_2A_n78A DC_71A_n78A |
| DC_3A_n1A-n8A-n78A2 | DC_3A_n1A DC_3A_n8A DC_3A_n78A |
| DC_3A-3A_n1A-n8A-n78A2 | DC_3A_n1A DC_3A_n8A DC_3A_n78A |
| DC_3A_n1A-n28A-n75A DC_3C_n1A-n28A-n75A | DC_3A_n1A DC_3C_n1A DC_3A_n28A DC_3C_n28A |
| DC_3A_n1A-n40A-n78A | DC_3A_n1A DC_3A_n40A DC_3A_n78A |
| DC_3A_n1A-n75A-n78A DC_3C_n1A-n75A-n78A | DC_3A_n1A DC_3C_n1A DC_3A_n78A DC_3C_n78A |
| DC_3A_n1A-n77A-n79A | DC_3A_n1A DC_3A_n77A DC_3A_n79A |
| DC_3A_n1A-n78A-n79A | DC_3A_n1A DC_3A_n78A DC_3A_n79A |
| DC_3A-5A-7A_n28A | DC_3A_n28A DC_5A_n28A DC_7A_n28A |
| DC_3A-5A-7A_n40A | DC_3A_n40A DC_5A_n40A DC_7A_n40A |
| DC_3A-5A-7A-7A_n40A | DC_3A_n40A DC_5A_n40A DC_7A_n40A |
| DC_3A-5A-7A_n77A | DC_3A_n77A DC_5A_n77A DC_7A_n77A |
| DC_3A-5A-7A_n77(2A) DC_3A-5A-7A_n77(3A) | DC_3A_n77A DC_5A_n77A DC_7A_n77A |
| DC_3A-5A-7A-7A_n77A | DC_3A_n77A DC_5A_n77A DC_7A_n77A |
| DC_3A-5A-7A-7A_n77(2A) DC_3A-5A-7A-7A_n77(3A) | DC_3A_n77A DC_5A_n77A DC_7A_n77A |
| DC_3A-5A-7A_n78A DC_3C-5A-7A_n78A DC_3A-5A-7A_n78C | DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_3A-5A-7A_n78(2A) | DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_3A-5A-7A_n78(A-C) | DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_3A-5A-7A-7A_n78A DC_3A-5A-7A-7A_n78C | DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_3A-5A-7A-7A_n78(2A) | DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_3A-5A-7A-7A_n78(A-C) | DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_3A-5A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_5A_n28A DC_5A_n78A |
| DC_3A-5A_n40A-n77A | DC_3A_n40A DC_3A_n77A DC_5A_n40A DC_5A_n77A |
| DC_3A-5A_n40A-n77(2A) | DC_3A_n40A DC_3A_n77A DC_5A_n40A DC_5A_n77A |
| DC_3A-5A_n40A-n78A DC_3A-5A_n40A-n78C | DC_3A_n40A DC_3A_n78A DC_5A_n40A DC_5A_n78A |
| DC_3A_n5A-n40A-n78A | DC_3A_n5A DC_3A_n40A DC_3A_n78A |
| DC_3A-7A_n1A-n8A | DC_3A_n1A DC_3A_n8A DC_7A_n1A DC_7A_n8A |
| DC_3A-3A-7A_n1A-n8A | DC_3A_n1A DC_3A_n8A DC_7A_n1A DC_7A_n8A |
| DC_3A-7A-7A_n1A-n8A | DC_3A_n1A DC_3A_n8A DC_7A_n1A DC_7A_n8A |
| DC_3A-3A-7A-7A_n1A-n8A | DC_3A_n1A DC_3A_n8A DC_7A_n1A DC_7A_n8A |
| DC_3A-7A_n1A-n28A DC_3C-7A_n1A-n28A | DC_3A_n1A DC_3A_n28A DC_3C_n1A DC_7A_n1A DC_7A_n28A |
| DC_3A-7C_n1A-n28A DC_3C-7C_n1A-n28A | DC_3A_n1A DC_3A_n28A DC_3C_n1A DC_7A_n1A DC_7A_n28A DC_7C_n1A DC_7C_n28A |
| DC_3A-7A_n1A-n40A | DC_3A_n1A DC_3A_n40A DC_7A_n1A DC_7A_n40A |
| DC_3A-7A_n1A-n78A2, 9 DC_3C-7A_n1A-n78A2 | DC_3A_n1A DC_3C_n1A DC_3A_n78A9 DC_3C_n78A DC_7A_n1A DC_7A_n78A9 |
| DC_3A-7A_n1A-n78(2A)2 DC_3C-7A_n1A-n78(2A)2 | DC_3A_n1A DC_3C_n1A DC_3A_n78A DC_3C_n78A DC_7A_n1A DC_7A_n78A |
| DC_3A-3A-7A_n1A-n78A2, 9 | DC_3A_n1A DC_3A_n78A9 DC_7A_n1A DC_7A_n78A9 |
| DC_3A-7A-7A_n1A-n78A2, 9 | DC_3A_n1A DC_3A_n78A9 DC_7A_n1A DC_7A_n78A9 |
| DC_3A-3A-7A-7A_n1A-n78A2, 9 | DC_3A_n1A DC_3A_n78A9 DC_7A_n1A DC_7A_n78A9 |
| DC_3A-7C_n1A-n78A DC_3C-7C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_3C_n1A DC_3C_n78A DC_7A_n1A DC_7A_n78A DC_7C_n1A DC_7C_n78A |
| DC_3A-7C_n1A-n78(2A)2 DC_3C-7C_n1A-n78(2A)2 | DC_3A_n1A DC_3A_n78A DC_3C_n1A DC_3C_n78A DC_7A_n1A DC_7A_n78A DC_7C_n1A DC_7C_n78A |
| DC_3A-5A-41A_n79A | DC_3A_n79A DC_5A_n79A DC_41A_n79A |
| DC_3A-7A_n1A-n75A | DC_3A_n1A DC_7A_n1A |
| DC_3C-7A_n1A-n75A | DC_3C_n1A DC_3A_n1A DC_7A_n1A |
| DC_3A-7A_n3A-n78A | DC_3A_n3A4 DC_7A_n3A DC_3A_n78A DC_7A_n78A |
| DC_3A-7C_n3A-n78A | DC_3A_n3A4 DC_7A_n3A DC_7C_n3A DC_3A_n78A DC_7C_n78A DC_7A_n78A |
| DC_3A-7A_n5A-n40A | DC_3A_n5A DC_3A_n40A DC_7A_n5A DC_7A_n40A |
| DC_3A-7A_n5A-n78A9 DC_3A-7C_n5A-n78A9 DC_3C-7A_n5A-n78A9 DC_3C-7C_n5A-n78A9 | DC_3A_n5A DC_3A_n78A9 DC_3C_n78A9 DC_7A_n5A DC_7C_n5A DC_7A_n78A9 DC_7C_n78A9 |
| DC_3A-7A_n7A-n78A2 | DC_3A_n7A DC_7A_n7A4 DC_3A_n78A DC_7A_n78A |
| DC_3A-3A-7A_n7A-n78A2 | DC_3A_n7A DC_7A_n7A4 DC_3A_n78A DC_7A_n78A |
| DC_3C-7A_n7A-n78A | DC_3A_n7A DC_3C_n7A DC_7A_n7A4 DC_3A_n78A DC_3C_n78A DC_7A_n78A |
| DC_3A-7A-8A_n1A DC_3A-7A-8B_n1A DC_3C-7A-8A_n1A | DC_3A_n1A DC_3C_n1A DC_7A_n1A DC_8A_n1A |
| DC_3A-3A-7A-8A_n1A DC_3A-3A-7A-8B_n1A | DC_3A_n1A DC_7A_n1A DC_8A_n1A |
| DC_3A-7A-7A-8A_n1A DC_3A-7A-7A-8B_n1A | DC_3A_n1A DC_7A_n1A DC_8A_n1A |
| DC_3A-3A-7A-7A-8A_n1A DC_3A-3A-7A-7A-8B_n1A | DC_3A_n1A DC_7A_n1A DC_8A_n1A |
| DC_3A-7A-8A_n7A | DC_3A_n7A DC_7A_n7A4 DC_8A_n7A |
| DC_3A-7A-8A_n28A | DC_3A_n28A DC_7A_n28A DC_8A_n28A |
| DC_3A-7A-7A-8A_n28A | DC_3A_n28A DC_7A_n28A DC_8A_n28A |
| DC_3A-7A-8A_n40A | DC_3A_n40A DC_7A_n40A DC_8A_n40A |
| DC_3A-7A-8A_n77A2 | DC_3A_n77A DC_7A_n77A DC_8A_n77A |
| DC_3A-7A-8A_n78A2, 9 DC_3A-7A-8B_n78A2 DC_3C-7A-8A_n78A | DC_3A_n78A9 DC_3C_n78A DC_7A_n78A 9 DC_8A_n78A9 DC_8B_n78A |
| DC_3A-7A-8A_n78(2A) | DC_3A_n78A, DC_7A_n78A, DC_8A_n78A |
| DC_3A-3A-7A-8A_n78A2, 9 DC_3A-3A-7A-8B_n78A2 | DC_3A_n78A9 DC_7A_n78A9 DC_8A_n78A9 DC_8B_n78A |
| DC_3A-7A-7A-8A_n78A2, 9 DC_3A-7A-7A-8B_n78A2 | DC_3A_n78A9 DC_7A_n78A9 DC_8A_n78A9 DC_8B_n78A |
| DC_3A-3A-7A-7A-8A_n78A2, 9 DC_3A-3A-7A-7A-8B_n78A2 | DC_3A_n78A9 DC_7A_n78A9 DC_8A_n78A9 DC_8B_n78A |
| DC_3A-7A_n8A-n78A2 | DC_3A_n8A DC_3A_n78A DC_7A_n8A DC_7A_n78A |
| DC_3A-3A-7A_n8A-n78A2 | DC_3A_n8A DC_3A_n78A DC_7A_n8A DC_7A_n78A |
| DC_3A-7A-7A_n8A-n78A2 | DC_3A_n8A DC_3A_n78A DC_7A_n8A DC_7A_n78A |
| DC_3A-3A-7A-7A_n8A-n78A2 | DC_3A_n8A DC_3A_n78A DC_7A_n8A DC_7A_n78A |
| DC_3A-7A-20A_n1A DC_3C-7A-20A_n1A DC_3A-7C-20A_n1A DC_3C-7C-20A_n1A | DC_3A_n1A DC_3C_n1A DC_7A_n1A DC_7C_n1A DC_20A_n1A |
| DC_3A-7A-20A_n3A | DC_3A_n3A DC_7A_n3A DC_20A_n3A |
| DC_3A-7A-20A_n8A | DC_3A_n8A DC_7A_n8A DC_20A_n8A |
| DC_3A-7A-20A_n28A3,8,14 DC_3C-7A-20A_n28A3 | DC_3A_n28A DC_7A_n28A DC_3C_n28A DC_20A_n28A |
| DC_3A-7A-20A_n38A12,13 | CA_3A-20A |
| DC_3A-7A-20A_n78A2 DC_3C-7A-20A_n78A2 DC_3A-7A-20A_n78C2 | DC_3A_n78A DC_3C_n78A DC_20A_n78A DC_7A_n78A |
| DC_3A-3A-7A-20A_n78A2 | DC_3A_n78A DC_7A_n78A DC_20A_n78A |
| DC_3A-7A-7A-20A_n78A2 | DC_3A_n78A DC_7A_n78A DC_20A_n78A |
| DC_3A-7A-20A_n78(2A) | DC_3A_n78A DC_7A_n78A DC_20A_n78A |
| DC_3A-7A-26A_n78A DC_3C-7A-26A_n78A DC_3A-7C-26A_n78A DC_3C-7C-26A_n78A | DC_3A_n78A DC_7A_n78A DC_26A_n78A |
| DC_3A-7A-26A_n78(2A) DC_3A-7C-26A_n78(2A) | DC_3A_n78A DC_7A_n78A DC_26A_n78A |
| DC_3A-7A_n26A-n78A | DC_3A_n78A DC_7A_n78A DC_3A_n26A DC_7A_n26A |
| DC_3C-7A-26A_n78(2A) DC_3C-7C-26A_n78(2A) | DC_3A_n78A DC_7A_n78A DC_26A_n78A |
| DC_3A-7C_n26A-n78A | DC_3A_n78A DC_7A_n78A DC_7C_n78A DC_3A_n26A DC_7A_n26A DC_7C_n26A |
| DC_3C-7A_n26A-n78A | DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_3A_n26A DC_3C_n26A DC_7A_n26A |
| DC_3C-7C_n26A-n78A | DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_7C_n78A DC_3A_n26A DC_3C_n26A DC_7A_n26A DC_7C_n26A |
| DC_3A-7A-28A_n1A DC_3C-7A-28A_n1A | DC_3A_n1A DC_3C_n1A DC_7A_n1A DC_28A_n1A |
| DC_3A-7A-7A-28A_n1A | DC_3A_n1A DC_7A_n1A DC_28A_n1A |
| DC_3A-7A-28A_n3A DC_3A-7C-28A_n3A | DC_3A_n3A4 DC_7A_n3A DC_7C_n3A DC_28A_n3A |
| DC_3A-7A-28A_n5A DC_3A-7C-28A_n5A DC_3C-7A-28A_n5A DC_3C-7C-28A_n5A | DC_3A_n5A DC_7A_n5A DC_7C_n5A DC_28A_n5A |
| DC_3A-7A-28A_n7A DC_3C-7A-28A_n7A | DC_3A_n7A DC_3C_n7A DC_7A_n7A4 DC_28A_n7A |
| DC_3A-3A-7A-28A_n7A | DC_3A_n7A DC_7A_n7A4 DC_28A_n7A |
| DC_3A-7A-28A_n38A | 3A17 28A17 |
| DC_3A-7A-28A_n40A | DC_3A_n40A DC_7A_n40A DC_28A_n40A |
| DC_3A-7A-28A_n78A2, 9 DC_3A-7C-28A_n78A2, 9 DC_3C-7A-28A_n78A9 DC_3C-7C-28A_n78A9 | DC_3A_n78A9 DC_3C_n78A9 DC_7A_n78A9 DC_7C_n78A9 DC_28A_n78A9 |
| DC_3A-7A-28A_n78(2A) DC_3A-7C-28A_n78(2A) DC_3C-7A-28A_n78(2A)2 DC_3C-7C-28A_n78(2A)2 | DC_3A_n78A DC_7A_n78A DC_28A_n78A |
| DC_3A-7A_n28A-n78A2, 9 DC_3A-7C_n28A-n78A9 DC_3C-7A_n28A-n78A9 DC_3C-7C_n28A-n78A9 | DC_3A_n28A DC_3C_n28A DC_3A_n78A9 DC_3C_n78A9 DC_7A_n28A DC_7A_n78A9 DC_7C_n28A DC_7C_n78A9 |
| DC_3A-7A-32A_n1A DC_3C-7A-32A_n1A | DC_3A_n1A DC_3C_n1A DC_7A_n1A |
| DC_3A-7A-32A_n28A DC_3C-7A-32A_n28A | DC_3A_n28A DC_3C_n28A DC_7A_n28A |
| DC_3A-7A-32A_n78A DC_3C-7A-32A_n78A | DC_3A_n78A DC_3C_n78A DC_7A_n78A |
| DC_3A-7A-38A_n28A10 DC_3C-7A-38A_n28A10 | DC_3A_n28A DC_3C_n28A |
| DC_3A-7A-38A_n78A10 DC_3C-7A-38A_n78A10 | DC_3A_n78A DC_3C_n78A |
| DC_3A-7A_n38A-n78A | DC_3A_n78A |
| DC_3A-7A-40A_n1A DC_3A-7A-40C_n1A | DC_3A_n1A DC_7A_n1A DC_40A_n1A |
| DC_3A-7A_n40A-n77A | DC_3A_n40A DC_3A_n77A DC_7A_n40A DC_7A_n77A |
| DC_3A-7A-7A_n40A-n77A | DC_3A_n40A DC_3A_n77A DC_7A_n40A DC_7A_n77A |
| DC_3A-7A-7A_n40A-n77(2A) | DC_3A_n40A DC_3A_n77A DC_7A_n40A DC_7A_n77A |
| DC_3A-7A_n40A-n77(2A) | DC_3A_n40A DC_3A_n77A DC_7A_n40A DC_7A_n77A |
| DC_3A-7A-40A_n78A DC_3A-7A-40C_n78A | DC_3A_n78A DC_7A_n78A DC_40A_n78A |
| DC_3A-7A-40A_n78(2A) DC_3A-7A-40C_n78(2A) | DC_3A_n78A DC_7A_n78A DC_40A_n78A |
| DC_3A-7A_n40A-n78A DC_3A-7A_n40A-n78C | DC_3A_n40A DC_3A_n78A DC_7A_n40A DC_7A_n78A |
| DC_3A-7A-7A_n40A-n78A DC_3A-7A-7A_n40A-n78C | DC_3A_n40A DC_3A_n78A DC_7A_n40A DC_7A_n78A |
| DC_3A-7A_n40A-n105A | DC_3A_n40A DC_3A_n105A DC_7A_n40A DC_7A_n105A |
| DC_3A-7A_n75A-n78A DC_3C-7A_n75A-n78A | DC_3A_n78A DC_3C_n78A DC_7A_n78A |
| DC_3A-7A_n78A-n79A | DC_3A_n78A DC_3A_n79A DC_7A_n78A DC_7A_n79A |
| DC_3A-3A-7A_n78A-n79A | DC_3A_n78A DC_3A_n79A DC_7A_n78A DC_7A_n79A |
| DC_3A-7A-7A_n78A-n79A | DC_3A_n78A DC_3A_n79A DC_7A_n78A DC_7A_n79A |
| DC_3A-3A-7A-7A_n78A-n79A | DC_3A_n78A DC_3A_n79A DC_7A_n78A DC_7A_n79A |
| DC_3A-7A_n78A-n105A | DC_3A_n78A DC_3A_n105A DC_7A_n78A DC_7A_n105A |
| DC_3A-7A_SUL_n78A-n80A DC_3C-7A_SUL_n78A-n80A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A DC_7A_n78A DC_7A_n80A |
| DC_3A-8A_n1A-n28A | DC_3A_n1A DC_8A_n1A DC_3A_n28A DC_8A_n28A |
| DC_3A-8A_n1A-n40A | DC_3A_n1A DC_8A_n1A DC_3A_n40A DC_8A_n40A |
| DC_3A-8A_n1A-n77A DC_3A-8B_n1A-n77A | DC_3A_n1A DC_3A_n77A DC_8A_n1A DC_8A_n77A |
| DC_3A-8A_n1A-n78A2,9 DC_3A-8B_n1A-n78A2 | DC_3A_n1A DC_3A_n78A9 DC_8A_n1A DC_8A_n78A9 |
| DC_3A-3A-8A_n1A-n78A2,9 DC_3A-3A-8B_n1A-n78A2 | DC_3A_n1A DC_3A_n78A9 DC_8A_n1A DC_8A_n78A9 |
| DC_(n)3AA-n8A-n77A DC_(n)3AA-n8A-n77(2A) | DC_(n)3AA4 DC_3A_n8A DC_3A_n77A |
| DC_3A-8A-11A_n28A | DC_3A_n28A DC_8A_n28A DC_11A_n28A |
| DC_3A-8A-11A_n77A2 | DC_3A_n77A DC_8A_n77A DC_11A_n77A |
| DC_3A-8A-11A_n77(2A) 2 DC_3A-8A-11A_n77(3A)2 | DC_3A_n77A DC_8A_n77A DC_11A_n77A |
| DC_3A-8A-20A_n1A | DC_3A_n1A DC_8A_n1A DC_20A_n1A |
| DC_3A-8A-20A_n28A DC_3C-8A-20A_n28A | DC_3A_n28A DC_3C_n28A DC_8A_n28A DC_20A_n28A |
| DC_3A-8A-20A_n78A | DC_3A_n78A DC_8A_n78A DC_20A_n78A |
| DC_3A-8A_n28A-n77A2 | DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A |
| DC_3A-8A_n28A-n77(2A)2 | DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A |
| DC_3A-8A-28A_n78A | DC_3A_n78A DC_8A_n78A DC_28A_n78A |
| DC_3A-8A_n28A-n78A2 | DC_3A_n28A DC_3A_n78A DC_8A_n28A DC_8A_n78A |
| DC_3A-8A-32A_n1A | DC_3A_n1A DC_8A_n1A |
| DC_3A-8A-32A_n28A DC_3C-8A-32A_n28A | DC_3A_n28A DC_8A_n28A |
| DC_3A-8A-32A_n78A | DC_3A_n78A DC_8A_n78A |
| DC_3A-8A_n40A-n41A | DC_3A_n40A DC_3A_n41A DC_8A_n40A DC_8A_n41A |
| DC_3A-8A_n40A-n78A | DC_3A_n40A DC_3A_n78A DC_8A_n40A DC_8A_n78A |
| DC_3A-8A-40A_n1A DC_3A-8A-40C_n1A | DC_3A_n1A DC_8A_n1A DC_40A_n1A |
| DC_3A-8A-40A_n78A DC_3A-8A-40C_n78A | DC_3A_n78A DC_8A_n78A DC_40A_n78A |
| DC_3A-8A-40A_n78(2A) DC_3A-8A-40C_n78(2A) | DC_3A_n78A DC_8A_n78A DC_40A_n78A |
| DC_3A-8A_n40A-n79A DC_3A-8A_n40A-n79C | DC_3A_n40A DC_3A_n79A DC_8A_n40A DC_8A_n79A |
| DC_3A-8A-41A_n1A DC_3A-8A-41C_n1A | DC_3A_n1A DC_8A_n1A DC_41A_n1A DC_41C_n1A |
| DC_3A-3A-8A-41A_n1A DC_3A-3A-8A-41C_n1A | DC_3A_n1A DC_8A_n1A DC_41A_n1A DC_41C_n1A |
| DC_3A-8A-41A_n78A DC_3A-8A-41C_n78A | DC_3A_n78A DC_8A_n78A DC_41A_n78A DC_41C_n78A |
| DC_3A-3A-8A-41A_n78A DC_3A-3A-8A-41C_n78A | DC_3A_n78A DC_8A_n78A DC_41A_n78A DC_41C_n78A |
| DC_3A-8A_n41A-n79A DC_3A-8A_n41A-n79C | DC_3A_n41A DC_3A_n79A DC_8A_n41A DC_8A_n79A |
| DC_3A-8A-42A_n77A7,8 DC_3A-8A-42C_n77A7,8 | DC_3A_n77A DC_8A_n77A |
| DC_3A-8A_n77A-n79A | DC_3A_n77A DC_3A_n79A DC_8A_n77A DC_8A_n79A |
| DC_3A-8A_SUL_n78A-n80A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A DC_8A_n78A DC_8A_n80A |
| DC_3A-11A_n28A-n77A2 | DC_3A_n28A DC_3A_n77A DC_11A_n28A DC_11A_n77A |
| DC_3A-11A_n28A-n77(2A) 2 | DC_3A_n28A DC_3A_n77A DC_11A_n28A DC_11A_n77A |
| DC_3A-18A_n3A-n41A | DC_3A_n3A4 DC_3A_n41A DC_18A_n3A DC_18A_n41A |
| DC_3A-18A_n3A-n77A | DC_3A_n3A4 DC_3A_n77A DC_18A_n3A DC_18A_n77A |
| DC_3A-18A_n3A-n78A | DC_3A_n3A4 DC_3A_n78A DC_18A_n3A DC_18A_n78A |
| DC_3A-18A_n28A-n41A | DC_3A_n28A DC_3A_n41A DC_18A_n28A DC_18A_n41A |
| DC_3A-18A_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_18A_n28A DC_18A_n77A |
| DC_3A-18A_n28A-n77(2A) | DC_3A_n28A DC_3A_n77A DC_18A_n28A DC_18A_n77A |
| DC_3A-18A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_18A_n28A DC_18A_n78A |
| DC_3A-18A_n28A-n78(2A) | DC_3A_n28A DC_3A_n78A DC_18A_n28A DC_18A_n78A |
| DC_3A-18A_n41A-n77A | DC_3A_n41A DC_3A_n77A DC_18A_n41A DC_18A_n77A |
| DC_3A-18A_n41A-n77(2A) | DC_3A_n41A DC_3A_n77A DC_18A_n41A DC_18A_n77A |
| DC_3A-18A_n41A-n78A | DC_3A_n41A DC_3A_n78A DC_18A_n41A DC_18A_n78A |
| DC_3A-18A_n41A-n78(2A) | DC_3A_n41A DC_3A_n78A DC_18A_n41A DC_18A_n78A |
| DC_3A-18A-42A_n77A7,8 DC_3A-18A-42C_n77A7,8 | DC_3A_n77A DC_18A_n77A |
| DC_3A-18A-42A_n78A7,8 DC_3A-18A-42C_n78A7,8 | DC_3A_n78A DC_18A_n78A |
| DC_3A-18A-42A_n79A DC_3A-18A-42C_n79A | DC_3A_n79A DC_18A_n79A |
| DC_3A-19A_n1A-n77A2 | DC_3A_n1A DC_3A_n77A DC_19A_n1A DC_19A_n77A |
| DC_3A-19A_n1A-n78A2 | DC_3A_n1A DC_3A_n78A DC_19A_n1A DC_19A_n78A |
| DC_3A-19A_n1A-n79A2 | DC_3A_n1A DC_3A_n79A DC_19A_n1A DC_19A_n79A |
| DC_3A-19A-21A_n77A2 DC_3A-19A-21A_n77C2 | DC_3A_n77A DC_19A_n77A DC_21A_n77A |
| DC_3A-19A-21A_n78A2 DC_3A-19A-21A_n78C2 | DC_3A_n78A DC_19A_n78A DC_21A_n78A |
| DC_3A-19A-21A_n79A2 DC_3A-19A-21A_n79C2 | DC_3A_n79A DC_19A_n79A DC_21A_n79A |
| DC_3A-19A-42A_n1A2 DC_3A-19A-42C_n1A2 | DC_3A_n1A DC_19A_n1A DC_42A_n1A |
| DC_3A-19A-42A_n77A7,8,9 DC_3A-19A-42A_n77C7,8 DC_3A-19A-42C_n77A7,8,9 DC_3A-19A-42C_n77C7,8 DC_3A-19A-42D_n77A7,8 DC_3A-19A-42D_n77C7,8 | DC_3A_n77A9 DC_19A_n77A9 |
| DC_3A-19A-42A_n78A7,8,9 DC_3A-19A-42A_n78C7,8 DC_3A-19A-42C_n78A7,8,9 DC_3A-19A-42C_n78C7,8 DC_3A-19A-42D_n78A7,8 DC_3A-19A-42D_n78C7,8 | DC_3A_n78A9 DC_19A_n78A9 |
| DC_3A-19A-42A_n79A9 DC_3A-19A-42A_n79C2 DC_3A-19A-42C_n79A9 DC_3A-19A-42C_n79C DC_3A-19A-42D_n79A DC_3A-19A-42D_n79C | DC_3A_n79A9 DC_19A_n79A9 |
| DC_3A-19A_n77A-n79A9 | DC_19A_n77A9 DC_19A_n79A9 |
| DC_3A-19A_n78A-n79A9 | DC_19A_n78A9 DC_19A_n79A9 |
| DC_3A-20A_n1A-n7A | DC_3A_n1A DC_3A_n7A DC_20A_n1A DC_20A_n7A |
| DC_3C-20A_n1A-n7A | DC_3A_n1A DC_3C_n1A DC_3A_n7A DC_3C_n7A DC_20A_n1A DC_20A_n7A |
| DC_3A-20A_n1A-n28A8,14 | DC_3A_n1A DC_3A_n28A DC_20A_n1A DC_20A_n28A |
| DC_3C-20A_n1A-n28A8,14 | DC_3A_n1A DC_3A_n28A DC_3C_n28A DC_20A_n1A DC_3C_n1A DC_20A_n28A |
| DC_3A-20A_n1A-n75A | DC_3A_n1A DC_20A_n1A |
| DC_3C-20A_n1A-n75A | DC_3A_n1A DC_3C_n1A DC_20A_n1A |
| DC_3A-20A_n1A-n78A DC_3A-3A-20A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_20A_n1A DC_20A_n78A |
| DC_3C-20A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_20A_n1A DC_20A_n78A DC_3C_n1A DC_3C_n78A |
| DC_3A-20A_n3A-n67A | DC_3A_n3A4 DC_20A_n3A |
| DC_3A-20A_n7A-n28A8,14 | DC_3A_n7A DC_3A_n28A DC_20A_n7A DC_20A_n28A |
| DC_3C-20A_n7A-n28A8,14 | DC_3A_n7A DC_3A_n28A DC_3C_n7A DC_20A_n7A DC_20A_n28A |
| DC_3A-20A_n7A-n78A | DC_3A_n7A DC_3A_n78A DC_20A_n7A DC_20A_n78A |
| DC_3A-20A_n8A-n78A | DC_3A_n8A DC_3A_n78A DC_20A_n8A DC_20A_n78A |
| DC_3A-20A-28A_n1A | DC_3A_n1A DC_20A_n1A DC_28A_n1A |
| DC_3A-20A_n28A-n75A | DC_3A_n28A DC_20A_n28A |
| DC_3C-20A_n28A-n75A | DC_20A_n28A DC_3A_n28A DC_3C_n28A |
| DC_3A-20A-28A_n78A | DC_3A_n78A DC_20A_n78A DC_28A_n78A |
| DC_3A-3A-20A-28A_n78A | DC_3A_n78A DC_20A_n78A DC_28A_n78A |
| DC_3A-20A_n28A-n78A2,3,8,14 DC_3C-20A_n28A-n78A2,3,8,14 | DC_3A_n28A DC_3C_n28A DC_3A_n78A DC_3C_n78A DC_20A_n28A DC_20A_n78A |
| DC_3A-20A-32A_n1A DC_3C-20A-32A_n1A | DC_3A_n1A DC_3C_n1A DC_20A_n1A |
| DC_3A-20A-32A_n7A | DC_3A_n7A DC_20A_n7A |
| DC_3A-20A-32A_n28A8,14 DC_3C-20A-32A_n28A8,14 | DC_3A_n28A DC_3C_n28A DC_20A_n28A |
| DC_3A-20A-32A_n78A | DC_3A_n78A DC_20A_n78A |
| DC_3A-20A-38A_n78A DC_3C-20A-38A_n78A | DC_3A_n78A DC_3C_n78A DC_20A_n78A DC_38A_n78A |
| DC_3A-20A-38A_n78(2A) | DC_3A_n78A DC_20A_n78A |
| DC_3A-20A_n38A-n78A | DC_3A_n78A DC_20A_n78A DC_3A_n38A DC_20A_n38A |
| DC_3A-20A-40A_n78A DC_3A-20A-40C_n78A | DC_3A_n78A DC_20A_n78A DC_40A_n78A |
| DC_3A-20A-40A_n78(2A) DC_3A-20A-40C_n78(2A) | DC_3A_n78A DC_20A_n78A DC_40A_n78A |
| DC_3A-20A-41A_n1A DC_3A-20A-41C_n1A | DC_3A_n1A DC_20A_n1A DC_41A_n1A DC_41C_n1A |
| DC_3A-3A-20A-41A_n1A DC_3A-3A-20A-41C_n1A | DC_3A_n1A DC_20A_n1A DC_41A_n1A DC_41C_n1A |
| DC_3A-20A_n41A-n78A | DC_3A_n41A DC_3A_n78A DC_20A_n41A DC_20A_n78A |
| DC_3A-20A-41A_n78A DC_3A-20A-41C_n78A | DC_3A_n78A DC_20A_n78A DC_41A_n78A DC_41C_n78A |
| DC_3A-3A-20A-41A_n78A DC_3A-3A-20A-41C_n78A | DC_3A_n78A DC_20A_n78A DC_41A_n78A DC_41C_n78A |
| DC_3A-20A-67A_n3A | DC_3A_n3A4 DC_20A_n3A |
| DC_3A-20A_SUL_n78A-n80A DC_3C-20A_SUL_n78A-n80A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A DC_20A_n78A DC_20A_n80A |
| DC_3A-21A_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_21A_n28A DC_21A_n77A |
| DC_3A-21A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_21A_n28A DC_21A_n78A |
| DC_3A-21A_n28A-n79A2 | DC_3A_n28A DC_3A_n79A DC_21A_n28A DC_21A_n79A |
| DC_3A-21A-42A_n1A2 DC_3A-21A-42C_n1A2 | DC_3A_n1A DC_21A_n1A DC_42A_n1A |
| DC_3A-21A_n1A-n77A2 | DC_3A_n1A DC_3A_n77A DC_21A_n1A DC_21A_n77A |
| DC_3A-21A_n1A-n78A2 | DC_3A_n1A DC_3A_n78A DC_21A_n1A DC_21A_n78A |
| DC_3A-21A_n1A-n79A2 | DC_3A_n1A DC_3A_n79A DC_21A_n1A DC_21A_n79A |
| DC_3A-21A-42A_n77A7,8,9 DC_3A-21A-42A_n77C7,8 DC_3A-21A-42C_n77A7,8,9 DC_3A-21A-42C_n77C7,8 DC_3A-21A-42D_n77A7,8 DC_3A-21A-42D_n77C7,8 | DC_3A_n77A9 DC_21A_n77A9 |
| DC_3A-21A-42A_n78A7,8,9 DC_3A-21A-42A_n78C7,8 DC_3A-21A-42C_n78A7,8,9 DC_3A-21A-42C_n78C7,8 DC_3A-21A-42D_n78A7,8 DC_3A-21A-42D_n78C7,8 | DC_3A_n78A9 DC_21A_n78A9 |
| DC_3A-21A-42A_n79A9 DC_3A-21A-42A_n79C DC_3A-21A-42C_n79A9 DC_3A-21A-42C_n79C DC_3A-21A-42D_n79A DC_3A-21A-42D_n79C | DC_3A_n79A9 DC_21A_n79A9 |
| DC_3A-21A_n77A-n79A9 | DC_3A_n77A9 DC_3A_n79A9 DC_21A_n77A9 DC_21A_n79A9 |
| DC_3A-21A_n78A-n79A9 | DC_3A_n78A9 DC_3A_n79A9 DC_21A_n78A9 DC_21A_n79A9 |
| DC_3A-28A_n1A-n40A | DC_3A_n1A DC_3A_n40A DC_28A_n1A DC_28A_n40A |
| DC_3A-28A_n1A-n78A2 | DC_3A_n1A DC_28A_n1A DC_3A_n78A DC_28A_n78A |
| DC_3A-28A_n3A-n78A2 | DC_3A_n3A4 DC_28A_n3A DC_3A_n78A DC_28A_n78A |
| DC_3A-28A_n5A-n40A | DC_3A_n5A DC_3A_n40A DC_28A_n5A DC_28A_n40A |
| DC_3A-28A_n5A-n78A2 DC_3C-28A_n5A-n78A2 | DC_3A_n5A DC_3A_n78A DC_3C_n78A DC_28A_n5A DC_28A_n78A |
| DC_3A-28A-(n)7AA DC_3C-28A-(n)7AA | DC_3A_n7A DC_28A_n7A |
| DC_3A-28A_n7A-n78A | DC_3A_n7A DC_28A_n7A DC_3A_n78A DC_28A_n78A |
| DC_3A-3A-28A_n7A-n78A | DC_3A_n7A DC_28A_n7A DC_3A_n78A DC_28A_n78A |
| DC_3A-28A_n7B-n78A | DC_3A_n7A DC_3A_n7B DC_28A_n7A DC_28A_n7B DC_3A_n78A DC_28A_n78A |
| DC_3A-3A-28A_n7B-n78A | DC_3A_n7A DC_3A_n7B DC_28A_n7A DC_28A_n7B DC_3A_n78A DC_28A_n78A |
| DC_3C-28A_n7A-n78A | DC_3A_n7A DC_3C_n7A DC_28A_n7A DC_3A_n78A DC_3C_n78A DC_28A_n78A |
| DC_3C-28A_n7B-n78A | DC_3A_n7A DC_3C_n7A DC_3A_n7B DC_28A_n7A DC_28A_n7B DC_3A_n78A DC_3C_n78A DC_28A_n78A |
| DC_3A-28A-32A_n1A | DC_3A_n1A DC_28A_n1A |
| DC_3A-28A-40A_n78A DC_3A-28A-40C_n78A | DC_3A_n78A DC_28A_n78A DC_40A_n78A |
| DC_3A-28A_n38A-n78A | DC_3A_n38A DC_3A_n78A DC_28A_n38A DC_28A_n78A |
| DC_3A-28A_n40A-n78A | DC_3A_n40A DC_3A_n78A DC_28A_n40A DC_28A_n78A |
| DC_3A-28A_n41A-n77A | DC_3A_n41A DC_28A_n41A DC_3A_n77A DC_28A_n77A |
| DC_3A-28A-41A_n78A DC_3A-28A-41C_n78A | DC_3A_n78A DC_28A_n78A DC_41A_n78A DC_41C_n78A |
| DC_3A-28A-42A_n77A7,8 DC_3A-28A-42A_n77C7,8 DC_3A-28A-42C_n77A7,8 DC_3A-28A-42C_n77C7,8 | DC_3A_n77A DC_28A_n77A |
| DC_3A-28A-42A_n78A7,8 DC_3A-28A-42A_n78C7,8 DC_3A-28A-42C_n78A7,8 DC_3A-28A-42C_n78C7,8 | DC_3A_n78A DC_28A_n78A |
| DC_3A-28A-42A_n79A DC_3A-28A-42A_n79C DC_3A-28A-42C_n79A DC_3A-28A-42C_n79C | DC_3A_n79A DC_28A_n79A |
| DC_3A_n28A-n77A-n79A | DC_3A_n28A DC_3A_n77A DC_3A_n79A |
| DC_3A_n28A-n78A-n79A | DC_3A_n28A DC_3A_n78A DC_3A_n79A |
| DC_3A-32A_n1A-n28A | DC_3A_n1A DC_3A_n28A |
| DC_3C-32A_n1A-n28A | DC_3A_n1A DC_3A_n28A DC_3C_n28A DC_3C_n1A |
| DC_3A-32A_n1A-n78A DC_3C-32A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_3C_n1A DC_3C_n78A |
| DC_3A-38A_n7A-n78A | DC_3A_n78A |
| DC_3A-32A-38A_n28A DC_3C-32A-38A_n28A | DC_3A_n28A DC_38A_n28A |
| DC_3A-38A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_38A_n28A DC_38A_n78A |
| DC_3C-38A_n28A-n78A | DC_3C_n78A DC_3A_n28A DC_3A_n78A DC_38A_n28A DC_38A_n78A |
| DC_3A-40A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_40A_n1A DC_40A_n78A |
| DC_3A-40C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_40A_n1A DC_40A_n78A |
| DC_3A_n40A-n41A-n79A | DC_3A_n40A DC_3A_n41A DC_3A_n79A |
| DC_3A_n40A-n78A-n105A | DC_3A_n40A DC_3A_n78A DC_3A_n105A |
| DC_3A-41A_n1A-n78A DC_3A-3A-41A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_41A_n1A DC_41A_n78A |
| DC_3A-41C_n1A-n78A DC_3A-3A-41C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_41A_n1A DC_41A_n78A |
| DC_3A-41A_n3A-n41A | DC_3A_n3A4 DC_3A_n41A DC_41A_n3A |
| DC_3A-41A_n3A-n77A | DC_3A_n3A4 DC_3A_n77A DC_41A_n3A DC_41A_n77A |
| DC_3A-41C_n3A-n77A | DC_3A_n3A4 DC_3A_n77A DC_41A_n3A DC_41A_n77A DC_41C_n3A DC_41C_n77A |
| DC_3A-41A_n3A-n78A | DC_3A_n3A4 DC_3A_n78A DC_41A_n3A DC_41A_n78A |
| DC_3A-41C_n3A-n78A | DC_3A_n3A4 DC_3A_n78A DC_41A_n3A DC_41A_n78A DC_41C_n3A DC_41C_n78A |
| DC_3A-41A_n28A-n41A | DC_3A_n28A DC_3A_n41A DC_41A_n28A |
| DC_3A-41A_n28A-n77A9 | DC_3A_n28A DC_3A_n77A9 DC_41A_n28A DC_41A_n77A9 |
| DC_3A-41C_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_41A_n28A DC_41A_n77A DC_41C_n28A DC_41C_n77A |
| DC_3A-41A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_41A_n28A DC_41A_n78A |
| DC_3A-41C_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_41A_n28A DC_41A_n78A DC_41C_n28A DC_41C_n78A |
| DC_3A-41A_n41A-n77A | DC_3A_n41A DC_3A_n77A DC_41A_n77A |
| DC_3A-41A_n41A-n78A | DC_3A_n41A DC_3A_n78A DC_41A_n78A |
| DC_3A-41A-42A_n77A7,8 DC_3A-41A-42C_n77A7,8 DC_3A-41C-42A_n77A7,8 DC_3A-41C-42C_n77A7,8 | DC_3A_n77A DC_41A_n77A |
| DC_3A-41A-42A_n77(2A)7,8 DC_3A-41A-42C_n77(2A)7,8 | DC_3A_n77A DC_41A_n77A |
| DC_3A-41A-42A_n78A7,8 DC_3A-41A-42C_n78A7,8 DC_3A-41C-42A_n78A7,8 DC_3A-41C-42C_n78A7,8 | DC_3A_n78A DC_41A_n78A |
| DC_3A-41A-42A_n79A DC_3A-41A-42C_n79A DC_3A-41C-42A_n79A DC_3A-41C-42C_n79A | DC_3A_n79A DC_41A_n79A |
| DC_3A-42A_n1A-n77A7,8 DC_3A-42C_n1A-n77A7,8 | DC_3A_n1A DC_3A_n77A |
| DC_3A-42A_n1A-n78A7,8 DC_3A-42C_n1A-n78A7,8 | DC_3A_n1A DC_3A_n78A |
| DC_3A-42A_n1A-n79A DC_3A-42C_n1A-n79A | DC_3A_n1A DC_3A_n79A |
| DC_3A-42A_n28A-n77A7,8 | DC_3A_n28A DC_3A_n77A DC_42A_n28A |
| DC_3A-42A_n28A-n77(2A)7,8 | DC_3A_n28A DC_3A_n77A DC_42A_n28A |
| DC_3A-42C_n28A-n77A7,8 | DC_3A_n28A DC_3A_n77A DC_42A_n28A DC_42C_n28A |
| DC_3A-42C_n28A-n77(2A)7,8 | DC_3A_n28A DC_3A_n77A DC_42A_n28A DC_42C_n28A |
| DC_3A-42A_n77A-n79A7,8,9 DC_3A-42C_n77A-n79A7,8,9 | DC_3A_n77A9 DC_3A_n79A9 |
| DC_3A-42A_n78A-n79A7,8,9 DC_3A-42C_n78A-n79A7,8,9 | DC_3A_n78A9 DC_3A_n79A9 |
| DC_5A-7A_n2A-n66A | DC_5A_n2A DC_5A_n66A DC_7A_n2A DC_7A_n66A |
| DC_5A-7A_n2A-n77A | DC_5A_n2A DC_5A_n77A DC_7A_n2A DC_7A_n77A |
| DC_5A-7A_n2A-n78A | DC_5A_n2A DC_7A_n2A DC_5A_n78A DC_7A_n78A |
| DC_5A-7A_n28A-n78A | DC_5A_n28A DC_5A_n78A DC_7A_n28A DC_7A_n78A |
| DC_5A-7A_n40A-n77A | DC_5A_n40A DC_5A_n77A DC_7A_n40A DC_7A_n77A |
| DC_5A-7A_n40A-n77(2A) | DC_5A_n40A DC_5A_n77A DC_7A_n40A DC_7A_n77A |
| DC_5A-7A-7A_n40A-n77A | DC_5A_n40A DC_5A_n77A DC_7A_n40A DC_7A_n77A |
| DC_5A-7A-7A_n40A-n77(2A) | DC_5A_n40A DC_5A_n77A DC_7A_n40A DC_7A_n77A |
| DC_5A-7A_n40A-n78A | DC_5A_n40A DC_5A_n78A DC_7A_n40A DC_7A_n78A |
| DC_5A-7A_n40A-n78C | DC_5A_n40A DC_5A_n78A DC_7A_n40A DC_7A_n78A |
| DC_5A-7A-7A_n40A-n78A DC_5A-7A-7A_n40A-n78C | DC_5A_n40A DC_5A_n78A DC_7A_n40A DC_7A_n78A |
| DC_5A-7A-66A_n2A | DC_5A_n2A DC_7A_n2A DC_66A_n2A |
| DC_5A-7A-66A_n7A | DC_5A_n7A DC_7A_n7A4 DC_66A_n7A |
| DC_5A-7A-66A-66A_n7A | DC_5A_n7A DC_7A_n7A4 DC_66A_n7A |
| DC_5A-7A-66A_n66A DC_5A-7C-66A_n66A DC_5A-7A-7A-66A_n66A | DC_5A_n66A DC_7A_n66A DC_66A_n66A4 |
| DC_5A-7A-(n)66AA DC_5A-7C-(n)66AA | DC_5A_n66A DC_7A_n66A DC_(n)66AA4 |
| DC_5A-7A-7A-(n)66AA | DC_5A_n66A DC_7A_n66A DC_(n)66AA4 |
| DC_5A-7A-66A_n77A | DC_5A_n77A DC_7A_n77A DC_66A_n77A |
| DC_5A-7A-66A_n77(2A) | DC_5A_n77A DC_7A_n77A DC_66A_n77A |
| DC_5A-7A_n66A-n77A | DC_5A_n66A DC_5A_n77A DC_7A_n66A DC_7A_n77A |
| DC_5A-7A-66A_n78A DC_5A-7C-66A_n78A | DC_5A_n78A DC_7A_n78A DC_7C_n78A DC_66A_n78A |
| DC_5A-7A-66A-66A_n78A DC_5A-7C-66A-66A_n78A | DC_5A_n78A DC_7A_n78A DC_7C_n78A DC_66A_n78A |
| DC_5A-7A-66A_n78(2A) | DC_5A_n78A DC_7A_n78A DC_66A_n78A |
| DC_5A-7A_n66A-n78A | DC_5A_n66A DC_7A_n66A DC_5A_n78A DC_7A_n78A |
| DC_5A-30A-66A_n2A DC_5A-30A-66A-66A_n2A | DC_5A_n2A DC_30A_n2A DC_66A_n2A |
| DC_5A-30A-66A_n5A DC_5A-30A-66A-66A_n5A | DC_30A_n5A DC_66A_n5A |
| DC_5A-30A-66A_n66A | DC_5A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_5A-30A-66A_n77A9 DC_5A-30A-66A-66A_n77A9 | DC_5A_n77A9 DC_30A_n77A9 DC_66A_n77A9 |
| DC_5A-30A-66A_n77(2A) 9 | DC_5A_n77A9 DC_30A_n77A9 DC_66A_n77A9 |
| DC_5A-48A-(n)12AA | DC_5A_n12A DC_48A_n12A DC_(n)12AA4 |
| DC_5A-48A-66A_n12A | DC_5A_n12A DC_48A_n12A DC_66A_n12A |
| DC_5A-48A-66A_n71A | DC_5A_n71A DC_48A_n71A DC_66A_n71A |
| DC_5A-66A_n2A-n41A | DC_5A_n2A DC_5A_n41A DC_66A_n2A DC_66A_n41A |
| DC_5A-66A_n2A-n66A | DC_5A_n2A DC_5A_n66A DC_66A_n2A DC_66A_n66A4 |
| DC_5A-66A_n2A-n77A DC_5A-66A_n2A-n77C | DC_5A_n2A DC_5A_n77A DC_66A_n2A DC_66A_n77A |
| DC_5A-66A-66A_n2A-n77A | DC_5A_n2A DC_5A_n77A DC_66A_n2A DC_66A_n77A |
| DC_5A-66A_n5A-n77A DC_5A-66A_n5A-n77C | DC_5A_n77A DC_66A_n5A DC_66A_n77A |
| DC_5A-66A-66A_n5A-n77A | DC_5A_n77A DC_66A_n5A DC_66A_n77A |
| DC_5A-48A-66A_n77A7,8,9 DC_5A-48A-66A_n77C7,8,9 DC_5A-48C-66A_n77A7,8,9 DC_5A-48C-66A_n77C7,8,9 | DC_5A_n77A DC_66A_n77A |
| DC_5A-66A_n2A-n78A | DC_5A_n2A DC_66A_n2A DC_5A_n78A DC_66A_n78A |
| DC_5A-66A-(n)12AA | DC_5A_n12A DC_66A_n12A DC_(n)12AA4 |
| DC_5A-66A_n66A-n77A9 DC_5A-66A_n66A-n77C9 | DC_5A_n66A DC_5A_n77A9 DC_66A_n77A9 |
| DC_7A_n1A-n8A-n78A2 | DC_7A_n1A DC_7A_n8A DC_7A_n78A |
| DC_7A_n1A-n40A-n78A | DC_7A_n1A DC_7A_n40A DC_7A_n78A |
| DC_7A_n1A-n75A-n78A | DC_7A_n1A DC_7A_n78A |
| DC_7A-7A_n1A-n8A-n78A2 | DC_7A_n1A DC_7A_n8A DC_7A_n78A |
| DC_7A-8A_n1A-n40A | DC_7A_n1A DC_8A_n1A DC_7A_n40A DC_8A_n40A |
| DC_7A-8A_n1A-n78A2,9 DC_7A-8B_n1A-n78A2 | DC_7A_n1A DC_7A_n78A9 DC_8A_n1A DC_8A_n78A9 |
| DC_7A-7A-8A_n1A-n78A2,9 DC_7A-7A-8B_n1A-n78A2 | DC_7A_n1A DC_7A_n78A9 DC_8A_n1A DC_8A_n78A9 |
| DC_7A-8A-20A_n1A | DC_7A_n1A DC_8A_n1A DC_20A_n1A |
| DC_7A-8A-20A_n3A | DC_7A_n3A DC_8A_n3A DC_20A_n3A |
| DC_7A-8A-20A_n28A9 | DC_7A_n28A |
| DC_7A-8A-20A_n78A | DC_7A_n78A DC_8A_n78A DC_20A_n78A |
| DC_7A-8A-32A_n1A | DC_7A_n1A DC_8A_n1A |
| DC_7A-8A-32A_n78A | DC_7A_n78A DC_8A_n78A |
| DC_7A-8A-38A_n1A | DC_8A_n1A |
| DC_7A-8A_n28A-n78A | DC_7A_n28A DC_7A_n78A DC_8A_n28A DC_8A_n78A |
| DC_7A-8A-40A_n1A DC_7A-8A-40C_n1A | DC_7A_n1A DC_8A_n1A DC_40A_n1A |
| DC_7A-8A-40A_n78A DC_7A-8A-40C_n78A | DC_7A_n78A DC_8A_n78A DC_40A_n78A |
| DC_7A-8A-40A_n78(2A) DC_7A-8A-40C_n78(2A) | DC_7A_n78A DC_8A_n78A DC_40A_n78A |
| DC_7A-8A_n40A-n78A | DC_7A_n40A DC_7A_n78A DC_8A_n40A DC_8A_n78A |
| DC_7A-12A_n2A-n66A | DC_7A_n2A DC_7A_n66A DC_12A_n2A DC_12A_n66A |
| DC_7A-12A_n2A-n77A | DC_7A_n2A DC_7A_n77A DC_12A_n2A DC_12A_n77A |
| DC_7A-12A_n2A-n78A | DC_7A_n2A DC_12A_n2A DC_7A_n78A DC_12A_n78A |
| DC_7A-12A-66A_n2A | DC_7A_n2A DC_12A_n2A DC_66A_n2A |
| DC_7A-12A-66A_n25A | DC_7A_n25A DC_12A_n25A DC_66A_n25A |
| DC_7A-12A-66A_n66A | DC_7A_n66A DC_12A_n66A DC_66A_n66A |
| DC_7A-12A-66A_n77A | DC_7A_n77A DC_12A_n77A DC_66A_n77A |
| DC_7A-12A-66A_n77(2A) | DC_7A_n77A DC_12A_n77A DC_66A_n77A |
| DC_7A-12A_n66A-n77A | DC_7A_n66A DC_7A_n77A DC_12A_n66A DC_12A_n77A |
| DC_7A-12A-66A_n78A | DC_7A_n78A DC_12A_n78A DC_66A_n78A |
| DC_7A-12A-66A_n78(2A) | DC_7A_n78A DC_12A_n78A DC_66A_n78A |
| DC_7A-12A_n66A-n78A | DC_7A_n66A DC_12A_n66A DC_7A_n78A DC_12A_n78A |
| DC_7A-12A-71A_n77A | DC_7A_n77A DC_12A_n77A DC_71A_n77A |
| DC_7A-13A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A |
| DC_7A-7A-13A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A |
| DC_7C-13A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A |
| DC_7A-13A-66A_n66A DC_7C-13A-66A_n66A | DC_7A_n66A DC_13A_n66A DC_66A_n66A4 |
| DC_7A-13A-(n)66AA DC_7C-13A-(n)66AA | DC_7A_n66A DC_13A_n66A DC_(n)66AA4 |
| DC_7A-7A-13A-(n)66AA | DC_7A_n66A DC_13A_n66A DC_(n)66AA4 |
| DC_7A-7A-13A-66A_n66A | DC_7A_n66A DC_13A_n66A DC_66A_n66A4 |
| DC_7A-20A_n1A-n75A | DC_3A_n1A DC_7A_n1A |
| DC_7A-20A_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_20A_n1A DC_20A_n78A |
| DC_7A-20A_n3A-n38A | DC_20A_n3A |
| DC_7A-20A_n3A-n78A | DC_7A_n3A DC_20A_n3A DC_7A_n78A DC_20A_n78A |
| DC_7A-20A_n8A-n78A | DC_7A_n8A DC_7A_n78A DC_20A_n8A DC_20A_n78A |
| DC_7A-20A-28A_n1A | DC_7A_n1A DC_20A_n1A DC_28A_n1A |
| DC_7A-20A-28A_n3A DC_7C-20A-28A_n3A | DC_7A_n3A DC_20A_n3A DC_28A_n3A |
| DC_7A-20A-28A_n78A2 | DC_7A_n78A DC_20A_n78A DC_28A_n78A |
| DC_7A-20A_n28A-n78A2,3 | DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A |
| DC_7A-20A-32A_n1A | DC_7A_n1A DC_20A_n1A |
| DC_7A-20A-32A_n3A DC_7C-20A-32A_n3A | DC_7A_n3A DC_20A_n3A |
| DC_7A-20A-32A_n8A | DC_7A_n8A DC_20A_n8A |
| DC_7A-20A-32A_n28A | DC_7A_n28A DC_20A_n28A |
| DC_7A-20A-32A_n78A | DC_7A_n78A DC_20A_n78A |
| DC_7A-20A-38A_n1A | DC_20A_n1A |
| DC_7A-20A-38A_n3A | DC_20A_n3A |
| DC_7A-20A-38A_n8A | DC_20A_n8A |
| DC_7A-20A-38A_n78A10 | DC_20A_n78A |
| DC_7A-20A_n38A-n78A15 | DC_20A_n78A |
| DC_7A-25A-66A_n77A DC_7C-25A-66A_n77A | DC_7A_n77A DC_25A_n77A DC_66A_n77A |
| DC_7A-7A-25A-66A_n77A | DC_7A_n77A DC_25A_n77A DC_66A_n77A |
| DC_7A-25A-25A-66A_n77A DC_7C-25A-25A-66A_n77A | DC_7A_n77A DC_25A_n77A DC_66A_n77A |
| DC_7A-7A-25A-25A-66A_n77A | DC_7A_n77A DC_25A_n77A DC_66A_n77A |
| DC_7A-25A-66A_n78A DC_7C-25A-66A_n78A | DC_7A_n78A DC_25A_n78A DC_66A_n78A |
| DC_7A-7A-25A-66A_n78A | DC_7A_n78A DC_25A_n78A DC_66A_n78A |
| DC_7A-25A-25A-66A_n78A DC_7C-25A-25A-66A_n78A | DC_7A_n78A DC_25A_n78A DC_66A_n78A |
| DC_7A-7A-25A-25A-66A_n78A | DC_7A_n78A DC_25A_n78A DC_66A_n78A |
| DC_7A-28A_n1A-n40A | DC_7A_n1A DC_7A_n40A DC_28A_n1A DC_28A_n40A |
| DC_7A-28A_n1A-n78A | DC_7A_n1A DC_28A_n1A DC_7A_n78A DC_28A_n78A |
| DC_7A-28A_n3A-n78A | DC_7A_n3A DC_28A_n3A DC_7A_n78A DC_28A_n78A |
| DC_7C-28A_n3A-n78A | DC_7A_n3A DC_7C_n3A DC_28A_n3A DC_7A_n78A DC_7C_n78A DC_28A_n78A |
| DC_7A-28A_n5A-n40A | DC_7A_n5A DC_7A_n40A DC_28A_n5A DC_28A_n40A |
| DC_7A-28A_n5A-n78A DC_7C-28A_n5A-n78A | DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A DC_28A_n5A DC_28A_n78A |
| DC_7A-28A_n7A-n78A | DC_7A_n7A4 DC_28A_n7A DC_7A_n78A DC_28A_n78A |
| DC_7A-28A-32A_n1A | DC_7A_n1A DC_28A_n1A |
| DC_7A-28A-32A_n3A DC_7C-28A-32A_n3A | DC_7A_n3A DC_28A_n3A |
| DC_7A-28A-38A_n1A | DC_28A_n1A |
| DC_7A-28A-38A_n78A DC_7C-28A-38A_n78A | DC_28A_n78A |
| DC_7A-28A_n38A-n78A15 | DC_28A_n78A |
| DC_7A-28A_n40A-n78A | DC_7A_n40A DC_7A_n78A DC_28A_n40A DC_28A_n78A |
| DC_7A-66A_n38A-n78A DC_7C-66A_n38A-n78A | DC_66A_n38A DC_66A_n78A |
| DC_7A-7A-66A_n38A-n78A | DC_66A_n38A DC_66A_n78A |
| DC_7A-7A-(n)66AA-n78A | DC_7A_n66A DC_7A_n78A DC_66A_n78A DC_(n)66AA2 |
| DC_7A-28A-66A_n7A | DC_7A_n7A4 DC_28A_n7A DC_66A_n7A |
| DC_7A-28A-66A_n66A DC_7C-28A-66A_n66A | DC_7A_n66A DC_28A_n66A DC_66A_n66A4 |
| DC_7A-29A-66A_n78A DC_7C-29A-66A_n78A | DC_7A_n78A DC_66A_n78A |
| DC_7A-7A-29A-66A_n78A | DC_7A_n78A DC_66A_n78A |
| DC_7A-32A_n1A-n78A | DC_7A_n1A DC_7A_n78A |
| DC_7A-40A_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_40A_n1A DC_40A_n78A |
| DC_7A-40C_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_40A_n1A DC_40A_n78A |
| DC_7A_n40A-n78A-n105A | DC_7A_n40A DC_7A_n78A DC_7A_n105A |
| DC_7A-66A_n2A-n66A | DC_7A_n2A DC_7A_n66A DC_66A_n2A DC_66A_n66A4 |
| DC_7A-66A_n2A-n71A | DC_7A_n2A DC_7A_n71A DC_66A_n2A DC_66A_n71A |
| DC_7A-66A_n2A-n77A | DC_7A_n2A DC_7A_n77A DC_66A_n2A DC_66A_n77A |
| DC_7A-66A_n2A-n78A | DC_7A_n2A DC_66A_n2A DC_7A_n78A DC_66A_n78A |
| DC_7A-66A_n12A-n77A | DC_7A_n12A DC_7A_n77A DC_66A_n12A DC_66A_n77A |
| DC_7A-66A_n12A-n78A | DC_7A_n12A DC_7A_n78A DC_66A_n12A DC_66A_n78A |
| DC_7A-66A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_66A_n25A |
| DC_7A-7A-66A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_66A_n25A |
| DC_7C-66A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_66A_n25A |
| DC_7A-66A_n66A-n71A | DC_7A_n66A DC_7A_n71A DC_66A_n66A4 DC_66A_n71A |
| DC_7A-66A_n66A-n77A DC_7C-66A_n66A-n77A DC_7A-7A-66A_n66A-n77A | DC_7A_n66A DC_7A_n77A DC_66A_n77A |
| DC_7A-66A_n66A-n78A DC_7C-66A_n66A-n78A | DC_7A_n66A DC_7A_n78A DC_66A_n66A4 DC_66A_n78A |
| DC_7A-(n)66AA-n78A DC_7C-(n)66AA-n78A | DC_7A_n66A DC_7A_n78A DC_66A_n78A DC_(n)66AA2 |
| DC_7A-7A-66A_n66A-n78A | DC_7A_n66A DC_7A_n78A DC_66A_n66A4 DC_66A_n78A |
| DC_7A-66A-71A_n2A | DC_7A_n2A DC_66A_n2A DC_71A_n2A |
| DC_7A-66A-71A_n25A | DC_7A_n25A DC_66A_n25A DC_71A_n25A |
| DC_7A-66A-71A_n66A | DC_7A_n66A DC_66A_n66A4 DC_71A_n66A |
| DC_7A-66A-71A_n77A | DC_7A_n77A DC_66A_n77A DC_71A_n77A |
| DC_7A-66A-71A_n77(2A) | DC_7A_n77A DC_66A_n77A DC_71A_n77A |
| DC_7A-66A_n71A-n77A | DC_7A_n71A DC_7A_n77A DC_66A_n71A DC_66A_n77A |
| DC_7A-66A-71A_n78A | DC_7A_n78A DC_66A_n78A DC_71A_n78A |
| DC_7A-66A-71A_n78(2A) | DC_7A_n78A DC_66A_n78A DC_71A_n78A |
| DC_7A-66A_n71A-n78A | DC_7A_n71A DC_66A_n71A DC_7A_n78A DC_66A_n78A |
| DC_7A-71A_n2A-n66A | DC_7A_n2A DC_7A_n66A DC_71A_n2A DC_71A_n66A |
| DC_7A-71A_n2A-n77A | DC_7A_n2A DC_7A_n77A DC_71A_n2A DC_71A_n77A |
| DC_7A-71A_n2A-n78A | DC_7A_n2A DC_71A_n2A DC_7A_n78A DC_71A_n78A |
| DC_7A-71A_n66A-n77A | DC_7A_n66A DC_7A_n77A DC_71A_n66A DC_71A_n77A |
| DC_7A-71A_n66A-n78A | DC_7A_n66A DC_71A_n66A DC_7A_n78A DC_71A_n78A |
| DC_8A_n1A-n3A-n77A | DC_8A_n1A DC_8A_n3A DC_8A_n77A |
| DC_8A-(n)3AA-n77A | DC_8A_n3A DC_8A_n77A DC_(n)3AA4 DC_3A_n77A |
| DC_8A-(n)3AA-n77(2A) | DC_8A_n3A DC_8A_n77A DC_(n)3AA4 DC_3A_n77A |
| DC_8A_n3A-n28A-n77A2 | DC_8A_n3A DC_8A_n28A DC_8A_n77A |
| DC_8A_n3A-n28A-n77(2A)2 | DC_8A_n3A DC_8A_n28A DC_8A_n77A |
| DC_8A_n3A-n28A-n79A | DC_8A_n3A DC_8A_n28A DC_8A_n79A |
| DC_8A_n3A-n77A-n79A | DC_8A_n3A DC_8A_n77A DC_8A_n79A |
| DC_8A_n3A-n77(2A)-n79A | DC_8A_n3A DC_8A_n77A DC_8A_n79A |
| DC_8A-11A_n1A-n77A DC_8B-11A_n1A-n77A | DC_8A_n1A DC_8A_n77A DC_11A_n1A DC_11A_n77A |
| DC_8A-11A_n1A-n77(2A) | DC_8A_n1A DC_8A_n77A DC_11A_n1A DC_11A_n77A |
| DC_8A-11A_n3A-n28A | DC_8A_n3A DC_8A_n28A DC_11A_n3A DC_11A_n28A |
| DC_8A-11A_n3A-n77A2 DC_8B-11A_n3A-n77A2 | DC_8A_n3A DC_8A_n77A DC_11A_n3A DC_11A_n77A |
| DC_8A-11A_n3A-n77(2A) 2 | DC_8A_n3A DC_8A_n77A DC_11A_n3A DC_11A_n77A |
| DC_8A-11A_n3A-n79A | DC_8A_n3A DC_8A_n79A DC_11A_n3A DC_11A_n79A |
| DC_8A-11A_n28A-n77A2 | DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A |
| DC_8A-11A_n28A-n77(2A) 2 | DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A |
| DC_8A-11A_n77A-n79A | DC_8A_n77A DC_8A_n79A DC_11A_n77A DC_11A_n79A |
| DC_8A-11A_n77(2A)-n79A | DC_8A_n77A DC_8A_n79A DC_11A_n77A DC_11A_n79A |
| DC_8A-20A-28A_n3A | DC_8A_n3A DC_20A_n3A DC_28A_n3A |
| DC_8A-20A-28A_n78A | DC_8A_n78A DC_20A_n78A DC_28A_n78A |
| DC_8A-20A-32A_n1A | DC_8A_n1A DC_20A_n1A |
| DC_8A-20A-32A_n3A | DC_8A_n3A DC_20A_n3A |
| DC_8A_n28A-n77A-n79A | DC_8A_n28A DC_8A_n77A DC_8A_n79A |
| DC_8A-20A-38A_n1A | DC_8A_n1A DC_20A_n1A DC_38A_n1A |
| DC_8A-32A-38A_n1A | DC_8A_n1A DC_38A_n1A |
| DC_8A_n39A-n40A-n41A | DC_8A_n39A DC_8A_n40A DC_8A_n41A |
| DC_8A_n39A-n40A-n79A | DC_8A_n39A DC_8A_n40A DC_8A_n79A |
| DC_8A_n39A-n41A-n79A | DC_8A_n39A DC_8A_n41A DC_8A_n79A |
| DC_8A_n40A-n41A-n79A | DC_8A_n40A DC_8A_n41A DC_8A_n79A |
| DC_8A-41A_n1A-n3A | DC_8A_n1A DC_8A_n3A DC_41A_n1A DC_41A_n3A |
| DC_8A-41C_n1A-n3A | DC_8A_n1A DC_8A_n3A DC_41A_n1A DC_41A_n3A |
| DC_8A-41A_n1A-n77A | DC_8A_n1A DC_8A_n77A DC_41A_n1A DC_41A_n77A |
| DC_8A-41C_n1A-n77A | DC_8A_n1A DC_8A_n77A DC_41A_n1A DC_41A_n77A |
| DC_8A-40A_n1A-n78A | DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A |
| DC_8A-40C_n1A-n78A | DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A |
| DC_8A-41A_n1A-n78A | DC_8A_n1A DC_8A_n78A DC_41A_n1A DC_41A_n78A |
| DC_8A-41C_n1A-n78A | DC_8A_n1A DC_8A_n78A DC_41A_n1A DC_41A_n78A |
| DC_8A-41A_n3A-n77A | DC_8A_n3A DC_8A_n77A DC_41A_n3A DC_41A_n77A |
| DC_8A-41C_n3A-n77A | DC_8A_n3A DC_8A_n77A DC_41A_n3A DC_41C_n3A DC_41A_n77A DC_41C_n77A |
| DC_8A-42A_n1A-n3A | DC_8A_n1A DC_8A_n3A DC_42A_n1A DC_42A_n3A |
| DC_8A-42C_n1A-n3A | DC_8A_n1A DC_8A_n3A DC_42A_n1A DC_42C_n1A DC_42A_n3A DC_42C_n3A |
| DC_8A-42A_n1A-n77A | DC_8A_n1A DC_8A_n77A DC_42A_n1A |
| DC_8A-42C_n1A-n77A | DC_8A_n1A DC_8A_n77A DC_42A_n1A DC_42C_n1A |
| DC_8A-42A_n3A-n28A2 | DC_8A_n3A DC_8A_n28A DC_42A_n3A DC_42A_n28A |
| DC_8A-42C_n3A-n28A2 | DC_8A_n3A DC_8A_n28A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_8A-42A_n3A-n77A | DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42A_n77A |
| DC_8A-42A_n3A-n77(2A) | DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42A_n77A |
| DC_8A-42C_n3A-n77A | DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n77A DC_42C_n77A |
| DC_8A-42C_n3A-n77(2A) | DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n77A DC_42C_n77A |
| DC_8A-42A_n28A-n77A | DC_8A_n28A DC_8A_n77A DC_42A_n28A |
| DC_8A-42A_n28A-n77(2A) | DC_8A_n28A DC_8A_n77A DC_42A_n28A |
| DC_8A-42C_n28A-n77A | DC_8A_n28A DC_8A_n77A DC_42A_n28A DC_42C_n28A |
| DC_8A-42C_n28A-n77(2A) | DC_8A_n28A DC_8A_n77A DC_42A_n28A DC_42C_n28A |
| DC_11A_n3A-n28A-n77A2 | DC_11A_n3A DC_11A_n28A DC_11A_n77A |
| DC_11A_n3A-n28A-n77(2A) 2 | DC_11A_n3A DC_11A_n28A DC_11A_n77A |
| DC_11A_n3A-n77A-n79A | DC_11A_n3A DC_11A_n77A DC_11A_n79A |
| DC_11A_n3A-n77(2A)-n79A | DC_11A_n3A DC_11A_n77A DC_11A_n79A |
| DC_12A-30A-66A_n2A | DC_12A_n2A DC_30A_n2A DC_66A_n2A |
| DC_12A-30A-66A-66A_n2A | DC_12A_n2A DC_30A_n2A DC_66A_n2A |
| DC_12A-30A-66A_n66A | DC_12A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_12A-30A-66A_n77A9 DC_12A-30A-66A-66A_n77A9 | DC_12A_n77A9 DC_30A_n77A9 DC_66A_n77A9 |
| DC_12A-30A-66A_n77(2A) 9 | DC_12A_n77A9 DC_30A_n77A9 DC_66A_n77A9 |
| DC_12A-48A-(n)5AA | DC_12A_n5A DC_48A_n5A DC_(n)5AA4 |
| DC_12A-48A-66A_n5A | DC_12A_n5A DC_48A_n5A DC_66A_n5A |
| DC_12A-66A-(n)5AA | DC_12A_n5A DC_66A_n5A DC_(n)5AA4 |
| DC_12A-66A_n2A-n41A | DC_12A_n2A DC_12A_n41A DC_66A_n2A DC_66A_n41A |
| DC_12A-66A_n2A-n66A | DC_12A_n2A DC_12A_n66A DC_66A_n2A DC_66A_n66A4 |
| DC_12A-66A_n2A-n77A | DC_12A_n2A DC_12A_n77A DC_66A_n2A DC_66A_n77A |
| DC_12A-66A_n2A-n78A | DC_12A_n2A DC_66A_n2A DC_12A_n78A DC_66A_n78A |
| DC_12A-66A_n66A-n77A | DC_12A_n66A DC_12A_n77A DC_66A_n66A4 DC_66A_n77A |
| DC_13A-48A-66A_n77A9 DC_13A-48C-66A_n77A9 DC_13A-48A-66A_n77C9 DC_13A-48C-66A_n77C9 | DC_13A_n77A9 DC_66A_n77A9 |
| DC_13A-66A_n2A-n77A9 DC_13A-66A-66A_n2A-n77A9 DC_13A-66A_n2A-n77C9 | DC_13A_n2A DC_13A_n77A9 DC_66A_n2A DC_66A_n77A9 |
| DC_13A-66A_n5A-n48A | DC_13A_n48A DC_66A_n5A DC_66A_n48A |
| DC_13A-66A_n5A-n77A9 DC_13A-66A-66A_n5A-n77A9 DC_13A-66A_n5A-n77C9 DC_13A-66A-66A_n5A-n77C9 | DC_66A_n5A DC_13A_n77A DC_66A_n77A |
| DC_13A-66A_n66A-n77A9 DC_13A-66A_n66A-n77C | DC_13A_n66A DC_13A_n77A9 DC_66A_n77A9 |
| DC_14A-30A-66A_n2A | DC_14A_n2A DC_30A_n2A DC_66A_n2A |
| DC_14A-30A-66A-66A_n2A | DC_14A_n2A DC_30A_n2A DC_66A_n2A |
| DC_14A-30A-66A_n66A | DC_14A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_14A-30A-66A_n77A9 DC_14A-30A-66A-66A_n77A9 | DC_14A_n77A9 DC_30A_n77A9 DC_66A_n77A9 |
| DC_14A-30A-66A_n77(2A) 9 | DC_14A_n77A9 DC_30A_n77A9 DC_66A_n77A9 |
| DC_18A-41A_n3A-n77A | DC_18A_n3A DC_18A_n77A DC_41A_n3A DC_41A_n77A |
| DC_18A-41C_n3A-n77A | DC_18A_n3A DC_18A_n77A DC_41A_n3A DC_41A_n77A DC_41C_n3A DC_41C_n77A |
| DC_18A-41A_n3A-n78A | DC_18A_n3A DC_18A_n78A DC_41A_n3A DC_41A_n78A |
| DC_18A-41C_n3A-n78A | DC_18A_n3A DC_18A_n78A DC_41A_n3A DC_41A_n78A DC_41C_n3A DC_41C_n78A |
| DC_19A_n1A-n77A-n79A | DC_19A_n1A DC_19A_n77A DC_19A_n79A |
| DC_19A_n1A-n78A-n79A | DC_19A_n1A DC_19A_n78A DC_19A_n79A |
| DC_19A-21A_n1A-n77A2 | DC_19A_n1A DC_19A_n77A DC_21A_n1A DC_21A_n77A |
| DC_19A-21A_n1A-n78A2 | DC_19A_n1A DC_19A_n78A DC_21A_n1A DC_21A_n78A |
| DC_19A-21A_n1A-n79A2 | DC_19A_n1A DC_19A_n79A DC_21A_n1A DC_21A_n79A |
| DC_19A-21A-42A_n1A2 DC_19A-21A-42C_n1A2 | DC_19A_n1A DC_21A_n1A DC_42A_n1A |
| DC_19A-21A-42A_n77A9 DC_19A-21A-42A_n77C DC_19A-21A-42C_n77A9 DC_19A-21A-42C_n77C | DC_19A_n77A9 DC_21A_n77A9 |
| DC_19A-21A-42A_n78A9 DC_19A-21A-42A_n78C DC_19A-21A-42C_n78A9 DC_19A-21A-42C_n78C | DC_19A_n78A9 DC_21A_n78A9 |
| DC_19A-21A-42A_n79A9 DC_19A-21A-42A_n79C DC_19A-21A-42C_n79A9 DC_19A-21A-42C_n79C | DC_19A_n79A9 DC_21A_n79A9 |
| DC_19A-21A_n77A-n79A9 | DC_19A_n77A9 DC_19A_n79A9 |
| DC_19A-21A_n78A-n79A9 | DC_19A_n78A9 DC_19A_n79A9 |
| DC_19A-42A_n1A-n77A DC_19A-42C_n1A-n77A | DC_19A_n1A DC_19A_n77A |
| DC_19A-42A_n1A-n78A DC_19A-42C_n1A-n78A | DC_19A_n1A DC_19A_n78A |
| DC_19A-42A_n1A-n79A DC_19A-42C_n1A-n79A | DC_19A_n1A DC_19A_n79A |
| DC_19A-42A_n77A-n79A9 DC_19A-42C_n77A-n79A9 | DC_19A_n77A9 DC_19A_n79A9 |
| DC_19A-42A_n78A-n79A9 DC_19A-42C_n78A-n79A9 | DC_19A_n78A9 DC_19A_n79A9 |
| DC_20A_n1A-n28A-n75A | DC_20A_n1A DC_20A_n28A |
| DC_20A-(n)3AA-n67A | DC_(n)3AA4 DC_20A_n3A |
| DC_20A-28A-32A_n1A | DC_20A_n1A DC_28A_n1A |
| DC_20A-28A-32A_n3A | DC_20A_n3A DC_28A_n3A |
| DC_20A-28A-38A_n1A | DC_20A_n1A DC_28A_n1A DC_38A_n1A |
| DC_20A-32A_n1A-n28A | DC_20A_n1A DC_20A_n28A |
| DC_20A-32A-38A_n1A | DC_20A_n1A DC_38A_n1A |
| DC_20A-38A_n3A-n78A | DC_20A_n3A DC_20A_n78A DC_38A_n3A DC_38A_n78A |
| DC_20A-41A_n1A-n78A | DC_20A_n1A DC_20A_n78A DC_41A_n1A DC_41A_n78A |
| DC_20A-41C_n1A-n78A | DC_20A_n1A DC_20A_n78A DC_41A_n1A DC_41A_n78A |
| DC_20A-67A-(n)3AA | DC_(n)3AA4 DC_20A_n3A |
| DC_21A_n1A-n77A-n79A | DC_21A_n1A DC_21A_n77A DC_21A_n79A |
| DC_21A_n1A-n78A-n79A | DC_21A_n1A DC_21A_n78A DC_21A_n79A |
| DC_21A-28A-42A_n77A DC_21A-28A-42C_n77A | DC_21A_n77A DC_28A_n77A |
| DC_21A-28A-42A_n78A DC_21A-28A-42C_n78A | DC_21A_n78A DC_28A_n78A |
| DC_21A-28A-42A_n79A DC_21A-28A-42C_n79A | DC_21A_n79A DC_28A_n79A |
| DC_21A_n28A-n77A-n79A | DC_21A_n28A DC_21A_n77A DC_21A_n79A |
| DC_21A_n28A-n78A-n79A | DC_21A_n28A DC_21A_n78A DC_21A_n79A |
| DC_21A-42A_n1A-n77A DC_21A-42C_n1A-n77A | DC_21A_n1A DC_21A_n77A |
| DC_21A-42A_n1A-n78A DC_21A-42C_n1A-n78A | DC_21A_n1A DC_21A_n78A |
| DC_21A-42A_n1A-n79A DC_21A-42C_n1A-n79A | DC_21A_n1A DC_21A_n79A |
| DC_21A-42A_n77A-n79A9 DC_21A-42C_n77A-n79A9 | DC_21A_n77A9 DC_21A_n79A9 |
| DC_21A-42A_n78A-n79A9 DC_21A-42C_n78A-n79A9 | DC_21A_n78A9 DC_21A_n79A9 |
| DC_28A_n1A-n40A-n78A | DC_28A_n1A DC_28A_n40A DC_28A_n78A |
| DC_28A_n5A-n40A-n78A | DC_28A_n5A DC_28A_n40A DC_28A_n78A |
| DC_28A-32A-38A_n1A | DC_28A_n1A DC_38A_n1A |
| DC_28A-41A-42A_n78A DC_28A-41C-42A_n78A DC_28A-41A-42C_n78A DC_28A-41C-42C_n78A | DC_28A_n78A DC_41A_n78A DC_41C_n78A |
| DC_29A-30A-66A_n2A | DC_30A_n2A DC_66A_n2A |
| DC_29A-30A-66A-66A_n2A | DC_30A_n2A DC_66A_n2A |
| DC_29A-30A-66A_n66A | DC_30A_n66A DC_66A_n66A4 |
| DC_29A-30A-66A_n77A9 | DC_30A_n77A9 DC_66A_n77A9 |
| DC_30A-66A-(n)5AA | DC_30A_n5A DC_66A_n5A DC_(n)5AA4 |
| DC_42A_n1A-n77A-n79A7,8 | N/A |
| DC_42A_n1A-n78A-n79A7,8 | N/A |
| DC_42A_n3A-n28A-n77A7,8 | DC_42A_n3A DC_42A_n28A |
| DC_42A_n3A-n28A-n77(2A)7,8 | DC_42A_n3A DC_42A_n28A |
| DC_42C_n3A-n28A-n77A7,8 | DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_42C_n3A-n28A-n77(2A)7,8 | DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_46A-66A_n25A-n41A DC_46C-66A_n25A-n41A DC_46D-66A_n25A-n41A | DC_66A_n25A DC_66A_n41A |
| DC_46A-66A_n25A-n71A DC_46C-66A_n25A-n71A DC_46D-66A_n25A-n71A | DC_66A_n25A DC_66A_n71A |
| DC_46A-66A_n41A-n71A DC_46C-66A_n41A-n71A DC_46D-66A_n41A-n71A | DC_66A_n41A DC_66A_n71A |
| DC_46A-66A_n41(2A)-n71A DC_46C-66A_n41(2A)-n71A DC_46D-66A_n41(2A)-n71A | DC_66A_n41A DC_66A_n71A |
| DC_48A-66A_n25A-n48A | DC_48A_n25A DC_66A_n25A DC_66A_n48A |
| DC_66A-71A_n2A-n41A | DC_66A_n2A DC_66A_n41A DC_71A_n2A DC_71A_n41A |
| DC_66A-71A_n2A-n66A | DC_66A_n2A DC_66A_n66A4 DC_71A_n2A DC_71A_n66A |
| DC_66A-71A_n2A-n77A | DC_66A_n2A DC_66A_n77A DC_71A_n2A DC_71A_n77A |
| DC_66A-71A_n2A-n78A | DC_66A_n2A DC_71A_n2A DC_66A_n78A DC_71A_n78A |
| DC_66A-71A_n66A-n77A | DC_66A_n66A4 DC_66A_n77A DC_71A_n66A DC_71A_n77A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 3: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. NOTE 4: Only single switched UL is supported. NOTE 5: UL carrier shall be supported in Band 2 or band 66 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. NOTE 6: The combination is not used alone as fall back mode of other band combinations in which UL in Band 42 is not used. NOTE 7: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42/48 and Band n77/n78 combination. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. NOTE 8: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. NOTE 9: Minimum requirements for PC2 are applicable for this uplink EN-DC configuration in this downlink/uplink EN-DC configuration. NOTE 10: Band 7 and Band 38 are restricted as DL Scell. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. NOTE 11: The implementation with 3 low-band antennas is targeted for FWA form factor for this band combination in Release 17. NOTE 12: Void. NOTE 13: Power imbalance between downlink carriers on Band 7 and band n38 is assumed to be within 6dB. The power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 14: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec between overlapping or partially overlapping DL bands contained in different cell groups. NOTE 15: Band 7 and Band n38 are restricted as DL Scell. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. NOTE 16: UL carrier shall be supported in Band 1 or band 28 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. NOTE 17: UL carrier shall be supported in Band 3 or band 28 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. |  |

6.2B.4.2.3	Inter-band EN-DC within FR1

6.2B.4.2.3.3	ΔTIB,c for EN-DC four bands

Table 6.2B.4.2.3.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_1-(n)3-n8 | 0.3 | 0.3 | 0.3 | 0.3 |
| DC_1-3_n3-n41 | 0.5 | 0.5 | 0.5 | 0.34/0.85 |
| DC_1-3_n3-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-3_n3-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-3_n5-n40 | 0.6 | 0.6 | 0.6 | 0.9 |
| DC_1-3-5_n28 | 0.3 | 0.3 | 0.7 | 0.7 |
| DC_1-3-5_n40 | 0.6 | 0.6 | 0.6 | 0.9 |
| DC_1-3-5_n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-3-5_n78 | 0.6 | 0.6 | 0.3 | 0.8 |
| DC_1-3-5_n79 | 0.3 | 0.3 | 0.3 | - |
| DC_1-3-7_n3 | 0.6 | 0.6 | 0.6 | 0.6 |
| DC_1-3-7_n1 | 0.6 | 0.6 | 0.6 | 0.6 |
| DC_1-3-7_n5 | 0.6 | 0.6 | 0.6 | 0.3 |
| DC_1-3-7_n7 DC_1-3-(n)7 | 0.6 | 0.6 | 0.6 | 0.6 |
| DC_1-3-7_n8 | 0.6 | 0.6 | 0.6 | 0.3 |
| DC_1-3-7_n26 | 0.6 | 0.6 | 0.6 | 0.3 |
| DC_1-3-7_n28 DC_1-3-7-7_n28 | 0.6 | 0.6 | 0.6 | 0.6 |
| DC_1-3-7_n38 | 0.6 | 0.6 | N/A | N/A |
| DC_1-3-7_n40 DC_1-3-7-7_n40 | 0.6 | 0.6 | 0.8 | 0.9 |
| DC_1-3-7_n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-3-7_n78 DC_1-3-3-7_n78 DC_1-3-3-7-7_n78 DC_1-3-7-7_n78 DC_1-1-3-3-7_n78 | 0.7 | 0.7 | 0.7 | 0.8 |
| DC_1-3_n7-n78 | 0.7 | 0.7 | 0.7 | 0.8 |
| DC_1-3-7_n105 | 0.6 | 0.6 | 0.6 | 0.6 |
| DC_1-3-8_n7 | 0.6 | 0.6 | 0.6 | 0.6 |
| DC_1-3-8_n28 | 0.3 | 0.3 | 0.6 | 0.6 |
| DC_1-3-8_n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1_n3-n8-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-3-8_n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-3_n8-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-3-8_n79 | 0.3 | 0.3 | 0.3 | - |
| DC_1-3-11_n28 | 0.3 | 0.8 | 0.9 | 0.6 |
| DC_1-3-11_n77 | 0.6 | 0.8 | 0.9 | 0.8 |
| DC_1-3-18_n3 | 0.3 | 0.3 | 0.3 | 0.3 |
| DC_1-3-18_n28 | 0.3 | 0.3 | 0.3 | 0.6 |
| DC_1-3-18_n41 | 0.3 | 0.3 | 0.3 | 0.34 |
| DC_1-3-28_n3 | 0.3 | 0.3 | 0.6 | 0.3 |
| DC_1-3-18_n77 | 0.6 | 0.6 | 0.3 | 0.8 |
| DC_1-3-18_n78 | 0.6 | 0.6 | 0.3 | 0.8 |
| DC_1-3-18_n79 | 0.3 | 0.3 | 0.3 | - |
| DC_1-3-19_n78 | 0.6 | 0.6 | 0.3 | 0.8 |
| DC_1-3-19_n79 | 0.3 | 0.3 | 0.3 | - |
| DC_1-3-20_n1 | 0.3 | 0.3 | 0.3 | 0.3 |
| DC_1-3-20_n3 | 0.3 | 0.3 | 0.3 | 0.3 |
| DC_1-3-20_n7 | 0.3 | 0.5 | 0.3 | 0.5 |
| DC_1-3-20_n8 | 0.6 | 0.6 | 0.6 | 0.6 |
| DC_1-3-20_n28 | 0.3 | 0.3 | 0.6 | 0.6 |
| DC_1-3-20_n38 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_1-3-20_n41 | 0.5 | 0.5 | 0.3 | 0.84 / 1.35 |
| DC_1-3-20_n78 DC_1-1-3-20_n78 DC_1-3-3-20_n78 | 0.6 | 0.6 | 0.3 | 0.8 |
| DC_1-3-21_n77 | 0.6 | 0.8 | 0.9 | 0.8 |
| DC_1-3-21_n78 | 0.6 | 0.8 | 0.9 | 0.8 |
| DC_1-3-21_n79 | 0.3 | 0.8 | 0.9 | - |
| DC_1-3-26_n78 | 0.6 | 0.6 | 0.3 | 0.8 |
| DC_1-3_n26-n78 | 0.6 | 0.6 | 0.3 | 0.8 |
| DC_1-3-28_n5 | 0.3 | 0.3 | 0.6 | 0.6 |
| DC_1-3-28_n7 | 0.6 | 0.6 | 0.6 | 0.6 |
| DC_1-3-28_n38 | 0.6 | 0.6 | 0.6 | 0.6 |
| DC_1-3-28_n40 | 0.5 | 0.5 | 0.6 | 0.5 |
| DC_1-3_n28-n75 | 0.3 | 0.3 | 0.6 | N/A |
| DC_1-3-28_n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1_n3-n28-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-3-28_n78 DC_1-3-3-28_n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-3_n28-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-3-28_n79 | 0.6 | 0.6 | 0.6 | - |
| DC_1_n3-n28-n79 | 0.6 | 0.6 | 0.6 | - |
| DC_1-3_n28-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-3_n28-n79 | 0.3 | 0.3 | 0.6 | - |
| DC_1-3-32_n28 | 0.3 | 0.3 | N/A | 0.6 |
| DC_1-3-38_n28 | 0.5 | 0.5 | 0.5 | 0.6 |
| DC_1-3-32_n78 | 0.6 | 0.6 | - | 0.8 |
| DC_1-3-38_n78 | 0.6 | 0.6 | 0.5 | 0.8 |
| DC_1-3_n38-n78 | 0.5 | 0.6 | 0.6 | 0.8 |
| DC_1-3_n40-n77 | 0.5 | 0.6 | 0.36 | 0.86 |
| DC_1-3_n40-n78 | 0.5 | 0.6 | 0.36 | 0.86 |
| DC_1-3-40_n78 | 0.6 | 0.6 | 0.39 | 0.89 |
| DC_1-3_n40-n105 | 0.6 | 0.6 | 0.6 | 0.5 |
| DC_1-3-41_n3 | 0.5 | 0.5 | 0.34 / 0.85 | 0.5 |
| DC_1-3-41_n28 | 0.5 | 0.5 | 0.34 / 0.85 | 0.6 |
| DC_1-3-41_n41 | 0.5 | 0.5 | 0.34 / 0.85 | 0.34 / 0.85 |
| DC_1-3_(n)41 | 0.5 | 0.5 | 0.34 / 0.85 | 0.34 / 0.85 |
| DC_1-3-41_n77 | 0.6 | 0.6 | 0.5 | 0.8 |
| DC_1-3_n41-n77 | 0.6 | 0.6 | 0.5 | 0.8 |
| DC_1-3-41_n78 | 0.6 | 0.6 | 0.5 | 0.8 |
| DC_1-3_n41-n78 | 0.6 | 0.6 | 0.5 | 0.8 |
| DC_1-3-41_n79 | 0.5 | 0.5 | 0.34 / 0.85 | - |
| DC_1-3-42_n28 | 0.6 | 0.6 | 0.8 | 0.8 |
| DC_1-3-42_n77 | 0.6 | 0.6 | N/A | 0.8 |
| DC_1-3-42_n78 | 0.6 | 0.6 | N/A | 0.8 |
| DC_1-3-42_n79 | 0.6 | 0.6 | N/A | - |
| DC_1-3_n75-n78 | 0.6 | 0.6 | N/A | 0.8 |
| DC_1-3_n77-n79 | 0.6 | 0.6 | 0.8 | - |
| DC_1_n3-n77-n79 | 0.6 | 0.6 | 0.8 | - |
| DC_1-3_n78-n79 | 0.6 | 0.6 | 0.8 | - |
| DC_1-3_n78-n105 | 0.6 | 0.6 | 0.8 | 0.6 |
| DC_1-3_SUL_n78-n80 | 0.6 | 0.6 | 0.8 | 0.6 |
| DC_1-5-7_n28 | 0.5 | 0.7 | 0.6 | 0.7 |
| DC_1-5-7_n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-5-7_n78 DC_1-5-7-7_n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-5_n28-n78 | 0.3 | 0.6 | 0.6 | 0.9 |
| DC_1-5_n40-n77 | 0.6 | 0.6 | 0.5 | 0.8 |
| DC_1-5_n40-n78 | 0.6 | 0.6 | 0.5 | 0.8 |
| DC_1-5-41_n79 | 0.5 | 0.3 | 0.5 | - |
| DC_1-7_n3-n38 | 0.6 | 0.6 | 0.6 | 0.5 |
| DC_1-7_n3-n78 | 0.5 | 0.2 | 0.6 | 0.8 |
| DC_1-7_n5-n40 | 0.6 | 0.8 | 0.6 | 0.9 |
| DC_1-7_n7-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-7-8_n3 | 0.6 | 0.6 | 0.3 | 0.6 |
| DC_1-7-8_n7 | 0.5 | 0.6 | 0.6 | 0.6 |
| DC_1-7-8_n20 | 0.5 | 0.6 | 0.6 | 0.6 |
| DC_1-7-8_n28 DC_1-7-7-8_n28 | 0.5 | 0.6 | 0.6 | 0.6 |
| DC_1-7-8_n78 DC_1-7-7-8_n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-7_n8-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-7-20_n3 | 0.3 | 0.5 | 0.3 | 0.5 |
| DC_1-7-20_n8 | 0.6 | 0.6 | 0.6 | 0.6 |
| DC_1-7-20_n28 | 0.5 | 0.6 | 0.6 | 0.6 |
| DC_1-7-20_n38 | 0.5 | N/A | 0.3 | N/A |
| DC_1-7-20_n78 DC_1-1-7-20_n78 DC_1-7-7-20_n78 | 0.6 | 0.7 | 0.4 | 0.8 |
| DC_1-7-26_n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-7_n26-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-7-28_n3 | 0.6 | 0.6 | 0.6 | 0.6 |
| DC_1-7-28_n5 | 0.3 | 0.3 | 0.6 | 0.6 |
| DC_1-7-28_n7 | 0.5 | 0.6 | 0.6 | 0.6 |
| DC_1-7-28_n20 | 0.5 | 0.6 | 0.6 | 0.6 |
| DC_1-7-28_n38 | 0.5 | N/A | 0.6 | N/A |
| DC_1-7-28_n40 | 0.6 | 0.8 | 0.6 | 0.9 |
| DC_1-7-28_n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-7_n28-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-7-32_n3 | 0.6 | 0.6 | N/A | 0.6 |
| DC_1-7-32_n8 | 0.7 | 0.7 | N/A | 0.6 |
| DC_1-7-32_n28 | 0.5 | 0.6 | N/A | 0.7 |
| DC_1-7_n40-n77 DC_1-7-7_n40-n77 | 0.6 | 0.5 | 0.5 | 0.8 |
| DC_1-7_n40-n105 | 0.6 | 0.5 | 0.5 | 0.5 |
| DC_1-7-38_n3 | 0.6 | N/A | N/A | 0.6 |
| DC_1-7-38_n78 | 0.3 | N/A | N/A | 0.8 |
| DC_1-7-32_n78 | 0.6 | 0.6 | - | 0.8 |
| DC_1-7-38_n8 | 0.5 | N/A | N/A | 0.5 |
| DC_1-7-38_n28 | 0.3 | N/A | N/A | 0.6 |
| DC_1-7-40_n78 | 0.6 | 0.5 | 0.39 | 0.89 |
| DC_1-7_n40-n78 DC_1-7-7_n40-n78 | 0.6 | 0.5 | 0.5 | 0.8 |
| DC_1-7_n75-n78 | 0.2 | 0.2 | N/A | 0.5 |
| DC_1-7_n78-n105 | 0.6 | 0.6 | 0.8 | 0.6 |
| DC_1-8-(n)3 | 0.3 | 0.3 | 0.3 | 0.3 |
| DC_1-8_n3-n28 | 0.3 | 0.6 | 0.3 | 0.6 |
| DC_1-8_n3-n77 | 0.6 | 0.6 | 0.8 | 0.8 |
| DC_1-8_n3-n79 | 0.3 | 0.3 | 0.3 | 0.8 |
| DC_1-8-11_n3 | 0.3 | 0.3 | 0.8 | 0.9 |
| DC_1-8-11_n28 | 0.3 | 0.6 | 0.4 | 0.6 |
| DC_1-8-11_n77 | 0.6 | 0.6 | 0.4 | 0.8 |
| DC_1-8-11_n78 | 0.3 | 0.6 | 0.4 | 0.8 |
| DC_1-8-11_n79 | 0.3 | 0.3 | 0.4 | - |
| DC_1-8-20_n3 | 0.3 | 0.4 | 0.4 | 0.3 |
| DC_1-8-20_n28 | 0.3 | 0.6 | 0.6 | 0.6 |
| DC_1-8-20_n78 | 0.3 | 0.6 | 0.6 | 0.8 |
| DC_1-8-28_n3 | 0.3 | 0.6 | 0.6 | 0.3 |
| DC_1-8_n28-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-8-28_n78 | 0.3 | 0.6 | 0.6 | 0.8 |
| DC_1-8_n28-n78 | 0.3 | 0.6 | 0.5 | 0.8 |
| DC_1-8_n28-n79 | 0.3 | 0.6 | 0.6 | 0.8 |
| DC_1-8-32_n3 | 0.5 | 0.3 | N/A | 0.8 |
| DC_1-8-32_n78 | 0.5 | 0.6 | N/A | 0.8 |
| DC_1-8-40_n78 | 0.6 | 0.6 | 0.39 | 0.89 |
| DC_1-8-42_n3 | 0.3 | 0.6 | 0.8 | 0.6 |
| DC_1-8-42_n28 | 0.3 | 0.6 | 0.8 | 0.8 |
| DC_1-8_n40-n78 | 0.5 | 0.3 | 0.5 | 0.8 |
| DC_1-8-42_n77 | 0.6 | 0.6 | N/A | 0.8 |
| DC_1-8_n77-n79 | 0.6 | 0.6 | 0.8 | 0.5 |
| DC_1-11_n3-n28 | 0.3 | 0.8 | 0.9 | 0.6 |
| DC_1-11_n3-n77 | 0.6 | 0.8 | 0.9 | 0.8 |
| DC_1-11_n3-n79 | 0.3 | 0.8 | 0.9 | 0.8 |
| DC_1-11-18_n3 | 0.3 | 0.9 | 0.3 | 0.8 |
| DC_1-11-18_n28 | 0.3 | 0.4 | 0.4 | 0.6 |
| DC_1-11-18_n41 | 0.5 | 0.4 | 0.3 | 0.5 |
| DC_1-11-18_n77 | 0.6 | 0.4 | 0.3 | 0.8 |
| DC_1-11-18_n78 | 0.3 | 0.4 | 0.3 | 0.8 |
| DC_1-11_n77-n79 | 0.6 | 0.4 | 0.8 | - |
| DC_1-18_n3-n41 | 0.3 | 0.3 | 0.3 | 0.34 |
| DC_1-18_n3-n77 | 0.6 | 0.3 | 0.6 | 0.8 |
| DC_1-18_n3-n78 | 0.6 | 0.3 | 0.6 | 0.8 |
| DC_1-18_n28-n41 | 0.3 | 0.3 | 0.5 | 0.34 |
| DC_1-18-28_n77 | 0.3 | 0.5 | 0.5 | 0.8 |
| DC_1-18_n28-n77 | 0.3 | 0.5 | 0.5 | 0.8 |
| DC_1-18-28_n78 | 0.3 | 0.5 | 0.5 | 0.8 |
| DC_1-18_n28-n78 | 0.3 | 0.5 | 0.5 | 0.8 |
| DC_1-18-28_n79 | 0.3 | 0.5 | 0.5 | - |
| DC_1-18-41_n77 | 0.6 | 0.3 | 0.5 | 0.8 |
| DC_1-18_n41-n77 | 0.6 | 0.3 | 0.5 | 0.8 |
| DC_1-18-41_n78 | 0.5 | 0.3 | 0.5 | 0.8 |
| DC_1-18_n41-n78 | 0.5 | 0.3 | 0.5 | 0.8 |
| DC_1-18-42_n77 | 0.3 | 0.3 | N/A | 0.8 |
| DC_1-18-42_n78 | 0.3 | 0.3 | N/A | 0.8 |
| DC_1-18-42_n79 | 0.3 | 0.3 | N/A | - |
| DC_1-19-42_n77 | 0.6 | 0.3 | N/A | 0.8 |
| DC_1-19-42_n78 | 0.3 | 0.3 | N/A | 0.8 |
| DC_1-19-42_n79 | 0.3 | 0.3 | N/A | - |
| DC_1-19_n77-n79 | 0.3 | 0.3 | 0.8 | - |
| DC_1-19_n78-n79 | 0.3 | 0.3 | 0.8 | - |
| DC_1-20_n3-n38 | 0.5 | 0.3 | 0.3 | 0.5 |
| DC_1-20_n3-n78 | 0.3 | 0.6 | 0.3 | 0.8 |
| DC_1-20_n7-n78 | 0.5 | 0.3 | 0.3 | 0.8 |
| DC_1-20_n8-n78 | 0.3 | 0.6 | 0.6 | 0.8 |
| DC_1-20-28_n3 | 0.3 | 0.6 | 0.6 | 0.3 |
| DC_1-20_n28-n75 | 0.3 | 0.6 | 0.7 | N/A |
| DC_1-20-28_n78 | 0.3 | 0.6 | 0.6 | 0.8 |
| DC_1-20_n28-n78 | 0.3 | 0.6 | 0.6 | 0.8 |
| DC_1-20-32_n3 | 0.5 | 0.3 | N/A | 0.5 |
| DC_1-20-32_n28 | 0.3 | 0.6 | N/A | 0.7 |
| DC_1-20-32_n78 | 0.3 | 0.3 | N/A | 0.8 |
| DC_1-20-38_n3 | 0.3 | 0.3 | 0.3 | 0.3 |
| DC_1-20_(n)38 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_1-20-38_n8 | 0.5 | 0.5 | 0.5 | 0.6 |
| DC_1-20-38_n78 | 0.3 | 0.6 | - | 0.8 |
| DC_1-20-40_n78 | 0.5 | 0.3 | 0.59 | 0.89 |
| DC_1-20_n41-n78 | 0.5 | 0.3 | 0.5 | 0.8 |
| DC_1-21-28_n77 | 0.6 | 0.4 | 0.6 | 0.8 |
| DC_1-21-28_n78 | 0.3 | 0.4 | 0.6 | 0.8 |
| DC_1-21-28_n79 | 0.3 | 0.4 | 0.6 | - |
| DC_1-21_n28-n77 | 0.3 | 0.4 | 0.6 | 0.8 |
| DC_1-21_n28-n78 | 0.6 | 0.4 | 0.6 | 0.8 |
| DC_1-21_n28-n79 | 0.3 | 0.4 | 0.6 | - |
| DC_1-21-42_n77 | 0.6 | 0.4 | N/A | 0.8 |
| DC_1-21-42_n78 | 0.3 | 0.4 | N/A | 0.8 |
| DC_1-21-42_n79 | 0.3 | 0.4 | N/A | - |
| DC_1-21_n77-n79 | 0.3 | 0.3 | 0.8 |  |
| DC_1-21_n78-n79 | 0.3 | 0.3 | 0.8 |  |
| DC_1-28_n3-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-28_n3-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-28_n5-n40 | 0.6 | 0.6 | 0.6 | 0.9 |
| DC_1-28-(n)7 | 0.5 | 0.6 | 0.6 | 0.6 |
| DC_1-28_n7-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-28-32_n3 | 0.5 | 0.6 | N/A | 0.5 |
| DC_1-28-40_n78 | 0.5 | 0.5 | 0.36 | 0.86 |
| DC_1-28_n40-n78 | 0.5 | 0.5 | 0.36 | 0.86 |
| DC_1-28-42_n77 | 0.6 | 0.6 | N/A | 0.8 |
| DC_1-28-42_n78 | 0.3 | 0.6 | N/A | 0.8 |
| DC_1-28-42_n79 | 0.3 | 0.6 | N/A |  |
| DC_1_n28-n77-n79 | 0.6 | 0.6 | 0.8 | 0.5 |
| DC_1_n28-n78-n79 | 0.3 | 0.6 | 0.8 | 0.5 |
| DC_1-38_n3-n78 | 0.5 | 0.6 | 0.6 | 0.8 |
| DC_1-38_n7-n78 | 0.6 | 0.5 | 0.6 | 0.8 |
| DC_1-38_n28-n78 | 0.5 | 0.5 | 0.5 | 0.8 |
| DC_1_n40-n78-n105 | 0.5 | 0.5 | 0.8 | 0.5 |
| DC_1-41_n3-n41 | 0.5 | 0.34 / 0.85 | 0.5 | 0.34 / 0.85 |
| DC_1-41_n3-n77 | 0.6 | 0.34 / 0.85 | 0.6 | 0.8 |
| DC_1-41_n3-n78 | 0.6 | 0.34 / 0.85 | 0.6 | 0.8 |
| DC_1-41_n28-n41 | 0.5 | 0.34 / 0.85 | 0.5 | 0.34 / 0.85 |
| DC_1-41_n28-n77 | 0.6 | 0.5 | 0.5 | 0.8 |
| DC_1-41_n28-n78 | 0.5 | 0.5 | 0.5 | 0.8 |
| DC_1-41_n41-n77 | 0.5 | 0.5 | 0.5 | 0.8 |
| DC_1-41_n41-n78 | 0.5 | 0.5 | 0.5 | 0.8 |
| DC_1-41-42_n77 | 0.5 | 0.5 | N/A | 0.8 |
| DC_1-41-42_n78 | 0.5 | 0.5 | N/A | 0.8 |
| DC_1-41-42_n79 | 0.5 | 0.5 | N/A | - |
| DC_1-42_n3-n28 | 0.3 | 0.8 | 0.6 | 0.8 |
| DC_1-42_n3-n77 | 0.6 | N/A | 0.6 | 0.8 |
| DC_1-42_n77-n79 | 0.6 | N/A | 0.8 | - |
| DC_1-42_n28-n77 | 0.6 | N/A | 0.8 | 0.8 |
| DC_1-42_n78-n79 | 0.3 | N/A | 0.8 | - |
| DC_2-4-7_n28 | 0.5 | 0.5 | 0.5 | 0.6 |
| DC_2-4-7_n78 | 0.6 | 0.6 | 0.5 | 0.8 |
| DC_2-5_n2-n41 | 0.5 | 0.6 | 0.5 | 0.41 / 0.92 |
| DC_2-5_n2-n66 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_2-5_n2-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-5_n2-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-5_n5-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-5-7_n2 | 0.5 | 0.3 | 0.5 | 0.3 |
| DC_2-5-7_n7 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_2-5-7_n66 DC_2-2-5-7_n66 DC_2-5-7-7_n66 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_2-5-7_n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-5-7_n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-5_(n)12 | 0.3 | 0.8 | 0.4 | 0.4 |
| DC_2-12_(n)5 | - | 0.5 | 0.3 | 0.5 |
| DC_2-5-30_n2 | 0.5 | 0.3 | 0.3 | 0.5 |
| DC_2-5-30_n66 | 0.5 | 0.3 | 0.3 | 0.5 |
| DC_2-5-30_n77 DC_2-2-5-30_n77 | 0.6 | 0.6 | 0.3 | 0.8 |
| DC_2-5_n41-n66 | 0.5 | 0.6 | 0.51 / 12 | 0.5 |
| DC_2-5-48_n12 | 0.6 | 0.8 | 0.8 | 0.4 |
| DC_2-5-48_n71 | 0.6 | 0.5 | 0.8 | 0.5 |
| DC_2-5-48_n77 | 0.6 | 0.6 | N/A | 0.8 |
| DC_2-5-66_n2 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_2-5-66_n5 | 0.5 | 0.3 | 0.5 | 0.3 |
| DC_2-5-66_n7 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_2-5-66_n12 | 0.3 | 0.5 | 0.5 | 0.3 |
| DC_2-5-66_n30 DC_2-2-5-66_n30 DC_2-5-66-66_n30 | 0.5 | 0.3 | 0.5 | 0.3 |
| DC_2-5-66_n41 DC_2-2-5-66_n41 | 0.5 | 0.6 | 0.5 | 0.81 / 1.32 |
| DC_2-5-66_n48 DC_2-5-66-66_n48 | 0.5 | 0.3 | 0.5 | 0.8 |
| DC_2-2-5-(n)66 DC_2-5-66-(n)66 DC_2-5-(n)66 DC_2-5-66_n66 DC_2-5-5-66_n66 DC_2-5-66-66_n66 DC_2-2-5-66-(n)66DC_2-2-5-66-66_n66 DC_2-5-5-66-66_n66 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_2-5-66_n71 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_2-5-66_n77 DC_2-2-5-66_n77 DC_2-5-66-66_n77 | 0.5 | 0.3 | 0.5 | 0.8 |
| DC_2-5-66_n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-5_n66-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-5_n66-n78 | 0.5 | 0.3 | 0.5 | 0.8 |
| DC_2-7_n2-n66 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_2-7_n2-n71 | 0.5 | 0.5 | 0.5 | 0.6 |
| DC_2-7_n2-n77 | 0.6 | 0.5 | 0.6 | 0.8 |
| DC_2-7_n2-n78 | 0.6 | 0.5 | 0.6 | 0.8 |
| DC_2-7-12_n2 | 0.5 | 0.5 | 0.3 | 0.5 |
| DC_2-7-12_n66 DC_2-2-7-12_n66 | 0.5 | 0.5 | 0.8 | 0.5 |
| DC_2-7-12_n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-7_n12-n77 | 0.6 | 0.5 | 0.3 | 0.8 |
| DC_2-7-12_n78 DC_2-2-7-12_n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-7-13_n25 | 0.5 | 0.5 | 0.3 | 0.5 |
| DC_2-7-13_n66 DC_2-7-7-13_n66 DC_2-2-7-7-13_n66 | 0.5 | 0.5 | 0.3 | 0.5 |
| DC_2-7_n25-n66 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_2-7-28_n7 | 0.5 | 0.5 | 0.3 | 0.5 |
| DC_2-7-28_n66 | 0.5 | 0.5 | 0.6 | 0.5 |
| DC_2-7-28_n78 | 0.5 | 0.5 | 0.3 | 0.8 |
| DC_2-7-29_n78 DC_2-7-7-29_n78 | 0.6 | 0.5 | N/A | 0.8 |
| DC_2-7_n38-n66 DC_2-7-7_n38-n66 | 0.5 | N/A | N/A | 0.5 |
| DC_2-7-38_n78 | 0.6 | N/A | N/A | 0.8 |
| DC_2-7_n38-n78 DC_2-7-7_n38-n78 | 0.6 | N/A | N/A | 0.8 |
| DC_2-7-66_n2 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_2-7-66_n7 DC_2-7-66-66_n7 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_2-7-66_n12 | 0.5 | 0.5 | 0.5 | 0.8 |
| DC_2-7-66_n25 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_2-7-66_n28 | 0.5 | 0.5 | 0.5 | 0.6 |
| DC_2-7-66_n38 DC_2-2-7-66_n38 | 0.5 | N/A | 0.5 | N/A |
| DC_2-7-(n)66 DC_2-7-66_n66 DC_2-7-7-(n)66 DC_2-7-7-66_n66 DC_2-7-7-66-(n)66 DC_2-7-66-(n)66 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_2-7-66_n71 DC_2-2-7-66_n71 | 0.5 | 0.5 | 0.5 | 0.3 |
| DC_2-7_n66-n71 | 0.5 | 0.5 | 0.5 | 0.3 |
| DC_2-7-66_n77 | 0.6 | 0.5 | 0.6 | 0.8 |
| DC_2-7-66_n78 DC_2-7-7-66_n78 DC_2-7-66-66_n78 DC_2-7-7-66-66_n78 DC_2-7_n66-n78 DC_2-7-7_n66-n78 | 0.6 | 0.5 | 0.6 | 0.8 |
| DC_2-7_n66-n77 | 0.6 | 0.5 | 0.6 | 0.8 |
| DC_2-7-71_n2 | 0.5 | 0.5 | 0.6 | 0.5 |
| DC_2-7-71_n66 DC_2-2-7-71_n66 | 0.5 | 0.5 | 0.3 | 0.5 |
| DC_2-7-71_n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-7_n71-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-7-71_n78 DC_2-2-7 -71_n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-7_n71-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-12_n2-n41 | 0.5 | 0.3 | 0.5 | 0.41 / 0.92 |
| DC_2-12_n2-n66 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_2-12_n2-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-12_n2-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-12-30_n2 | 0.5 | 0.3 | 0.3 | 0.5 |
| DC_2-12-30_n66 | 0.5 | 0.8 | 0.3 | 0.5 |
| DC_2-12-30_n77 DC_2-2-12-30_n77 | 0.6 | 0.5 | 0.3 | 0.8 |
| DC_2-12_n41-n66 | 0.5 | 0.8 | 0.51 / 12 | 0.5 |
| DC_2-12-48_n5 | 0.6 | 0.4 | 0.8 | 0.8 |
| DC_2-12-66_n2 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_2-12-66_n5 | 0.5 | 0.8 | 0.5 | 0.8 |
| DC_2-12-66_n7 | 0.5 | 0.6 | 0.5 | 0.8 |
| DC_2-12-66_n30 DC_2-2-12-66_n30 DC_2-12-66-66_n30 | 0.5 | 0.8 | 0.5 | 0.3 |
| DC_2-2-12-(n)66 DC_2-12-(n)66 DC_2-12-66_n66 | 0.5 | 0.8 | 0.5 | 0.5 |
| DC_2-12-66_n77 DC_2-2-12-66_n77 DC_2-12-66-66_n77 | 0.6 | 0.8 | 0.6 | 0.8 |
| DC_2-12_n66-n77 | 0.6 | 0.8 | 0.6 | 0.8 |
| DC_2-12-66_n78 DC_2-2-12-66_n78 | 0.6 | 0.3 | 0.6 | 0.8 |
| DC_2-12_n66-n78 | 0.5 | 0.3 | 0.5 | 0.8 |
| DC_2-13_n25-n66 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_2-13-48_n77 | 0.6 | 0.5 | N/A | 0.8 |
| DC_2-13-66_n2 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_2-13-66_n5 | 0.5 | 0.3 | 0.5 | 0.3 |
| DC_2-13-66_n48 | 0.6 | 0.3 | 0.6 | 0.8 |
| DC_2-13-(n)66 DC_2-2-13-(n)66 DC_2-13-66_n66 DC_2-13-66-(n)66 DC_2-2-13-66-(n)66 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_2-13-66_n77 DC_2-2-13-66_n77 DC_2-2-13-66-66_n77 DC_2-13-66-66_n77 | 0.5 | 0.3 | 0.5 | 0.8 |
| DC_2-13_n66-n77 | 0.6 | 0.3 | 0.6 | 0.8 |
| DC_2-14-30_n2 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_2-14-30_n66 | 0.5 | 0.3 | 0.3 | 0.5 |
| DC_2-14-30_n77 DC_2-2-14-30_n77 | 0.6 | 0.5 | 0.3 | 0.8 |
| DC_2-14-66_n2 DC_2-14-66-66_n2 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_2-14-66_n30 DC_2-2-14-66_n30 DC_2-14-66-66_n30 | 0.5 | 0.3 | 0.5 | 0.3 |
| DC_2-14-66_n66 DC_2-2-14-66_n66 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_2-14-66_n77 DC_2-2-14-66_n77 DC_2-14-66-66_n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-28-66_n7 | 0.5 | 0.6 | 0.5 | 0.5 |
| DC_2-28-66_n66 | 0.5 | 0.6 | 0.5 | 0.5 |
| DC_2-29-30_n2 | 0.5 | N/A | 0.3 | 0.5 |
| DC_2-29-30_n66 | 0.5 | N/A | 0.3 | 0.5 |
| DC_2-29-30_n77 DC_2-2-29-30_n77 | 0.6 | N/A | 0.3 | 0.8 |
| DC_2-29-66_n2 DC_2-29-66-66_n2 | 0.5 | N/A | 0.5 | 0.5 |
| DC_2-29-66_n30 DC_2-2-29-66_n30 DC_2-29-66-66_n30 | 0.5 | N/A | 0.5 | 0.3 |
| DC_2-29-(n)66 DC_2-2-29-(n)66 DC_2-29-66_n66 | 0.5 | N/A | 0.5 | 0.5 |
| DC_2-29-66_n77 | 0.6 | N/A | 0.6 | 0.8 |
| DC_2-29-66_n78 | 0.6 | N/A | 0.6 | 0.8 |
| DC_2-30-(n)5 DC_2-2-30-(n)5 | 0.5 | 0.3 | 0.3 | 0.3 |
| DC_2-30-66_n2 DC_2-30-66-66_n2 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_2-30-66_n5 | 0.5 | 0.3 | 0.5 | 0.3 |
| DC_2-30-66_n66 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_2-30-66_n77 DC_2-2-30-66_n77 DC_2-30-66-66_n77 | 0.6 | 0.3 | 0.6 | 0.8 |
| DC_2-46_n41-n66 | 0.5 | N/A | 0.5 | 0.5 |
| DC_2-46_n41-n71 | 0.5 | N/A | 0.5 | 0.6 |
| DC_2-46-48_n2 | 0.6 | N/A | 0.8 | 0.6 |
| DC_2-46-48_n5 | 0.6 | N/A | 0.8 | 0.3 |
| DC_2-46-48_n66 | 0.6 | N/A | 0.8 | 0.6 |
| DC_2-46-66_n5 | 0.5 | N/A | 0.5 | 0.3 |
| DC_2-46-66_n41 | 0.5 | N/A | 0.5 | 0.81 / 1.32 |
| DC_2-46-66_n71 | - | N/A | 0.3 | 0.3 |
| DC_2-48-66_n77 | 0.6 | N/A | 0.6 | 0.8 |
| DC_2-48_n48-n66 | 0.6 | 0.8 | 0.8 | 0.6 |
| DC_2-48_(n)5 | 0.6 | 0.3 | 0.8 | 0.3 |
| DC_2-46_n66_n71 | 0.5 | N/A | 0.5 | 0.3 |
| DC_2-48-66_n2 | 0.6 | 0.8 | 0.6 | 0.6 |
| DC_2-48-66_n5 | 0.6 | 0.8 | 0.6 | - |
| DC_2-48-66_n12 | 0.6 | 0.8 | 0.6 | 0.3 |
| DC_2-48-66_n66 | 0.6 | 0.8 | 0.6 | 0.6 |
| DC_2-48-66_n71 | 0.6 | 0.8 | 0.6 | 0.3 |
| DC_2-66_n2-n41 | 0.5 | 0.5 | 0.5 | 0.51 / 12 |
| DC_2-66_n2-n66 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_2-66_n2-n71 | 0.5 | 0.5 | 0.5 | 0.3 |
| DC_2-66_n2-n77 DC_2-66-66_n2-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-66_n2-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-66_(n)5 DC_2-2-66_(n)5 DC_2-66-66_(n)5 | 0.5 | 0.3 | 0.5 | 0.3 |
| DC_2-66_n5-n77 | 0.6 | 0.6 | 0.3 | 0.8 |
| DC_2-66_n12-n77 | 0.6 | 0.6 | 0.8 | 0.8 |
| DC_2-66_n12-n78 | 0.6 | 0.6 | 0.8 | 0.8 |
| DC_2-66_n25-n66 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_2-66_n38-n78 | 0.6 | 0.6 | 0.9 | 0.8 |
| DC_2-66_n41-n71 | 0.5 | 0.5 | 0.81 / 1.32 | 0.8 |
| DC_2-66_n66-n71 | 0.5 | 0.5 | 0.5 | 0.3 |
| DC_2-66_n66-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-(n)66-n78 DC_2-66_n66-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-66-71_n2 | 0.5 | 0.5 | 0.3 | 0.5 |
| DC_2-66-71_n38 DC_2-2-66-71_n38 | 0.5 | 0.5 | 0.3 | 0.5 |
| DC_2-66-71_n41 DC_2-2-66-71_n41 | 0.5 | 0.5 | 0.8 | 0.81 / 1.32 |
| DC_2-66-71_n66 | 0.5 | 0.5 | 0.3 | 0.5 |
| DC_2-66-(n)71 | 0.5 | 0.5 | 0.3 | 0.3 |
| DC_2-66-71_n71 | 0.5 | 0.5 | 0.3 | 0.3 |
| DC_2-66-71_n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-66_n71-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-66-71_n78 DC_2-2-66-71_n78 | 0.5 | 0.5 | 0.3 | 0.5 |
| DC_2-66_n71-n78 | 0.5 | 0.5 | 0.3 | 0.5 |
| DC_2-71_n2-n41 | 0.5 | 0.6 | 0.5 | 0.41 / 0.92 |
| DC_2-71_n2-n66 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_2-71_n2-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-71_n2-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-71_n41-n66 | 0.5 | 0.8 | 0.81 / 1.32 | 0.8 |
| DC_2-71_n66-n77 | 0.5 | 0.3 | 0.5 | 0.8 |
| DC_2-71_n66-n78 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_3_n1-n28-n75 | 0.3 | 0.3 | 0.7 | N/A |
| DC_3_n1-n75-n78 | 0.6 | 0.6 | N/A | 0.8 |
| DC_3_n1-n40-n78 | 0.6 | 0.6 | 0.5 | 0.8 |
| DC_3_n1-n77-n79 | 0.6 | 0.6 | 0.8 | 0.5 |
| DC_3_n1-n78-n79 | 0.6 | 0.3 | 0.8 | 0.5 |
| DC_3-5-7_n28 | 0.5 | 0.7 | 0.5 | 0.7 |
| DC_3-5-7_n40 DC_3-5-7-7_n40 | 0.6 | 0.6 | 0.8 | 0.9 |
| DC_3-5-7_n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-5-7_n78 DC_3-5-7-7_n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-5_n28-n78 | 0.5 | 0.6 | 0.5 | 0.9 |
| DC_3-5_n40-n77 | 0.6 | 0.6 | 0.5 | 0.8 |
| DC_3-5_n40-n78 | 0.6 | 0.6 | 0.5 | 0.8 |
| DC_3_n5-n40-n78 | 0.6 | 0.6 | 0.5 | 0.8 |
| DC_3-5-41_n79 | 0.5 | 0.33 | 0.34 / 0.85 | - |
| DC_3-7_n1-n8 DC_3-3-7_n1-n8 DC_3-3-7-7_n1-n8 | 0.6 | 0.6 | 0.6 | 0.6 |
| DC_3-7_n1-n28 | 0.6 | 0.6 | 0.6 | 0.5 |
| DC_3-7_n1-n40 | 0.6 | 0.8 | 0.6 | 0.9 |
| DC_3-7_n1-n75 | 0.6 | 0.6 | 0.6 | N/A |
| DC_3-7_n1-n78 | 0.7 | 0.7 | 0.7 | 0.8 |
| DC_3-7_n3-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-7_n5-n40 | 0.6 | 0.8 | 0.6 | 0.9 |
| DC_3-7_n7-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-7-8_n1 DC_3-3-7-8_n1 DC_3-7-7-8_n1 DC_3-3-7-7-8_n1 | 0.6 | 0.6 | 0.6 | 0.6 |
| DC_3-7-8_n7 | 0.5 | 0.5 | 0.6 | 0.5 |
| DC_3-7-8_n28 DC_3-7-7-8_n28 | 0.5 | 0.5 | 0.6 | 0.5 |
| DC_3-7-8_n40 | 0.5 | 0.5 | 0.6 | 0.6 |
| DC_3-7-8_n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-7-8_n78 DC_3-3-7-8_n78 DC_3-7-7-8_n78 DC_3-3-7-7-8_n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-7_n8-n78 DC_3-3-7_n8-n78 DC_3-7-7_n8-n78 DC_3-3-7-7_n8-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-7-20_n1 | 0.6 | 0.6 | 0.3 | 0.6 |
| DC_3-7-20_n3 | 0.5 | 0.5 | 0.3 | 0.5 |
| DC_3-7-20_n8 | 0.6 | 0.6 | 0.6 | 0.6 |
| DC_3-7-20_n28 | 0.5 | 0.5 | 0.6 | 0.5 |
| DC_3-7-20_n38 | 0.5 | N/A | 0.3 | N/A |
| DC_3-7-20_n78 DC_3-3-7-20_n78 DC_3-7-7-20_n78 | 0.6 | 0.6 | 0.3 | 0.8 |
| DC_3-7-26_n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-7_n26-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-7-28_n1 DC_3-7-7-28_n1 | 0.6 | 0.6 | 0.5 | 0.6 |
| DC_3-7-28_n3 | 0.5 | 0.5 | 0.3 | 0.5 |
| DC_3-7-28_n5 | 0.5 | 0.5 | 0.4 | 0.4 |
| DC_3-7-28_n7 | 0.5 | 0.5 | 0.3 | 0.5 |
| DC_3-7-28_n38 | 0.5 | 0.5 | 0.3 | 0.5 |
| DC_3-7-28_n40 | 0.6 | 0.8 | 0.3 | 0.9 |
| DC_3-7-28_n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-7_n28-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-7-32_n1 | 0.6 | 0.6 | N/A | 0.6 |
| DC_3-7-32_n28 | 0.5 | 0.5 | N/A | 0.3 |
| DC_3-7-32_n78 | 0.6 | 0.6 | N/A | 0.8 |
| DC_3-7-38_n28 | 0.3 | N/A | N/A | 0.3 |
| DC_3-7-38_n78 | 0.6 | N/A | N/A | 0.8 |
| DC_3-7-40_n1 | 0.6 | 0.8 | 0.9 | 0.6 |
| DC_3-7_n40-n77 DC_3-7-7_n40-n77 | 0.6 | 0.5 | 0.5 | 0.8 |
| DC_3-7-40_n78 | 0.6 | 0.5 | 0.39 | 0.89 |
| DC_3-7_n40-n78 DC_3-7-7_n40-n78 | 0.6 | 0.5 | 0.5 | 0.8 |
| DC_3-7_n40-n105 | 0.6 | 0.5 | 0.5 | 0.5 |
| DC_3-7_n75-n78 | 0.6 | 0.6 | N/A | 0.8 |
| DC_3-7_n78-n79 DC_3-3-7_n78-n79 DC_3-7-7_n78-n79 DC_3-3-7-7_n78-n79 | 0.6 | 0.6 | 0.8 | 0.5 |
| DC_3-7_n78-n105 | 0.6 | 0.6 | 0.8 | 0.6 |
| DC_3-7_SUL_n78-n80 | 0.6 | 0.6 | 0.8 | 0.6 |
| DC_3-8_n1-n28 | 0.3 | 0.6 | 0.3 | 0.6 |
| DC_3-3-8_n1-n78 | 0.5 | 0.5 | 0.5 | 0.6 |
| DC_3_n1-n8-n78 | 0.5 | 0.5 | 0.5 | 0.6 |
| DC_3-8_n1-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-8_n1-n78 DC_3-3-8_n1-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-8-11_n28 | 0.8 | 0.6 | 0.9 | 0.6 |
| DC_3-8-11_n77 | 0.8 | 0.6 | 0.9 | 0.8 |
| DC_3-8-20_n1 | 0.3 | 0.4 | 0.4 | 0.3 |
| DC_3-8-20_n28 | 0.3 | 0.6 | 0.5 | 0.5 |
| DC_3-8-20_n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-8_n28-n77 | 0.6 | 0.6 | 0.5 | 0.8 |
| DC_3-8-28_n78 | 0.6 | 0.6 | 0.5 | 0.8 |
| DC_3-8_n28-n78 | 0.6 | 0.6 | 0.5 | 0.8 |
| DC_3-8-32_n1 | 0.5 | 0.3 | N/A | 0.8 |
| DC_3-8-32_n28 | 0.3 | 0.3 | N/A | 0.6 |
| DC_3-8-32_n78 | 0.8 | 0.6 | N/A | 0.8 |
| DC_3-8-40_n1 | 0.5 | 0.5 | 0.6 | 0.5 |
| DC_3-8-40_n78 | 0.6 | 0.6 | 0.39 | 0.89 |
| DC_3-8_n40-n41 | 0.5 | 0.3 | 0.5 | 0.54/0.85 |
| DC_3-8_n40-n78 | 0.6 | 0.3 | 0.5 | 0.8 |
| DC_3-8_n40-n79 | 0.5 | 0.3 | 0.5 | - |
| DC_3-8-41_n1 DC_3-3-8-41_n1 | 0.6 | 0.6 | 0.6 | 0.6 |
| DC_3-8-41_n78 DC_3-3-8-41_ n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-8_n41-n79 | 0.6 | 0.3 | 0.34/0.85 | - |
| DC_3-8-42_n77 | 0.6 | 0.6 | N/A | 0.8 |
| DC_(n)3-n8-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-8_n77-n79 | 0.6 | 0.6 | 0.8 | 0.5 |
| DC_3-8_SUL_n78-n80 | 0.6 | 0.6 | 0.8 | 0.6 |
| DC_3-11_n28-n77 | 0.8 | 0.9 | 0.6 | 0.8 |
| DC_3-18_n3-n41 | 0.6 | 0.3 | 0.6 | 0.34 / 0.85 |
| DC_3-18_n3-n77 | 0.6 | 0.3 | 0.6 | 0.8 |
| DC_3-18_n3-n78 | 0.6 | 0.3 | 0.6 | 0.8 |
| DC_3-18_n28-n41 | 0.6 | 0.3 | 0.5 | 0.34 / 0.85 |
| DC_3-18_n28-n77 | 0.6 | 0.3 | 0.5 | 0.8 |
| DC_3-18_n28-n78 | 0.6 | 0.3 | 0.5 | 0.8 |
| DC_3-18_n41-n77 | 0.6 | 0.3 | 0.5 | 0.8 |
| DC_3-18_n41-n78 | 0.6 | 0.3 | 0.6 | 0.8 |
| DC_3-18-42_n77 | 0.3 | 0.3 | N/A | 0.8 |
| DC_3-18-42_n78 | 0.3 | 0.3 | N/A | 0.8 |
| DC_3-18-42_n79 | 0.6 | 0.3 | N/A | - |
| DC_3-19_n1-n77 | 0.6 | 0.3 | 0.6 | 0.8 |
| DC_3-19_n1-n78 | 0.6 | 0.3 | 0.6 | 0.8 |
| DC_3-19_n1-n79 | 0.3 | 0.3 | 0.3 | - |
| DC_3-19-21_n77 | 0.8 | 0.3 | 0.9 | 0.8 |
| DC_3-19-21_n78 | 0.8 | 0.3 | 0.9 | 0.8 |
| DC_3-19-21_n79 | 0.8 | 0.3 | 0.9 | - |
| DC_3-19-42_n1 | 0.6 | 0.3 | 0.8 | - |
| DC_3-19-42_n77 | 0.6 | 0.3 | N/A | 0.8 |
| DC_3-19-42_n78 | 0.6 | 0.3 | N/A | 0.8 |
| DC_3-19-42_n79 | 0.6 | 0.3 | N/A | - |
| DC_3-19_n77-n79 | 0.6 | 0.3 | 0.8 | - |
| DC_3-19_n78-n79 | 0.6 | 0.3 | 0.8 | - |
| DC_3-20_n1-n7 | 0.6 | 0.3 | 0.6 | 0.6 |
| DC_3-20_n1-n28 | 0.6 | 0.3 | 0.6 | 0.8 |
| DC_3-20_n1-n75 | 0.5 | 0.3 | 0.5 | N/A |
| DC_3-20_n1-n78 | 0.6 | 0.3 | 0.6 | 0.8 |
| DC_3-20_n3-n67 | 0.3 | 0.5 | 0.3 | N/A |
| DC_3-20_n7-n28 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_3-20_n8-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-20-28_n1 | 0.3 | 0.6 | 0.6 | 0.3 |
| DC_3-20_n28-n75 | 0.3 | 0.5 | 0.5 | N/A |
| DC_3-20-28_n78 DC_3-3-20-28_n78 | 0.6 | 0.6 | 0.5 | 0.8 |
| DC_3-20_n28-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-20-32_n1 | 0.5 | 0.3 | N/A | 0.5 |
| DC_3-20-32_n7 | 0.7 | 0.3 | N/A | 0.7 |
| DC_3-20-32_n28 | 0.3 | 0.5 | N/A | 0.5 |
| DC_3-20-32_n78 | 0.5 | 0.3 | N/A | 0.8 |
| DC_3-20-38_n78 | 0.6 | 0.6 | 0.5 | 0.8 |
| DC_3-20_n38-n78 | 0.6 | 0.6 | 0.5 | 0.8 |
| DC_3-20-40_n78 | 0.6 | 0.5 | 0.36 | 0.86 |
| DC_3-20-41_n1 DC_3-3-20-41_n1 | 0.6 | 0.3 | 0.6 | 0.6 |
| DC_3-20-41_n78 DC_3-3-20-41_n78 DC_3-20_n41-n78 | 0.5 | 0.3 | 0.5 | 0.8 |
| DC_3-20-67_n3 | 0.3 | 0.5 | N/A | 0.3 |
| DC_3_20_SUL_n78-n80 | 0.5 | 0.3 | 0.8 | 0.5 |
| DC_3-21_n1-n77 | 0.8 | 0.9 | 0.6 | 0.8 |
| DC_3-21_n1-n78 | 0.8 | 0.9 | 0.6 | 0.8 |
| DC_3-21_n1-n79 | 0.8 | 0.9 | 0.3 | - |
| DC_3-21_n28-n77 | 0.8 | 0.9 | 0.5 | 0.8 |
| DC_3-21_n28-n78 | 0.8 | 0.9 | 0.5 | 0.8 |
| DC_3-21_n28-n79 | 0.8 | 0.9 | 0.3 | - |
| DC_3-21-42_n1 | 0.8 | 0.9 | 0.8 | 0.6 |
| DC_3-21-42_n77 | 0.8 | 0.9 | N/A | 0.8 |
| DC_3-21-42_n78 | 0.8 | 0.9 | N/A | 0.8 |
| DC_3-21-42_n79 | 0.8 | 0.9 | N/A | - |
| DC_3-21_n77-n79 | 0.8 | 0.9 | 0.8 | - |
| DC_3-21_n78-n79 | 0.8 | 0.9 | 0.8 | - |
| DC_3-28_n1-n40 | 0.5 | 0.6 | 0.5 | 0.5 |
| DC_3-28_n1-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-28_n3-n78 | 0.5 | 0.3 | 0.5 | 0.8 |
| DC_3-28_n5-n40 | 0.6 | 0.6 | 0.6 | 0.9 |
| DC_3-28-(n)7 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_3-28_n7-n78 DC_3-3-28_n7-n78 | 1.0 | 0.5 | 0.8 | 0.8 |
| DC_3-28-32_n1 | 0.3 | 0.6 | N/A | 0.3 |
| DC_3-28-40_n78 | 0.6 | 0.5 | 0.36 | 0.86 |
| DC_3-28_n40-n78 | 0.6 | 0.5 | 0.36 | 0.86 |
| DC_3-28_n41-n77 | 1.0 | 0.5 | 0.34 / 0.85 | 0.8 |
| DC_3-28-41_n78 | 1.0 | 0.5 | 0.34 / 0.85 | 0.8 |
| DC_3-28-42_n77 | 0.6 | 0.5 | N/A | 0.8 |
| DC_3-28-42_n78 | 0.6 | 0.5 | N/A | 0.8 |
| DC_3-28-42_n79 | 0.6 | 0.5 | N/A | - |
| DC_3_n28-n77-n79 | 0.6 | 0.5 | 0.8 | 0.5 |
| DC_3_n28-n78-n79 | 0.6 | 0.5 | 0.8 | 0.5 |
| DC_3-32_n1-n28 | 0.3 | N/A | 0.3 | 0.6 |
| DC_3-32_n1-n78 | 0.6 | N/A | 0.6 | 0.8 |
| DC_3-38_n7-n78 | 0.6 | N/A | N/A | 0.8 |
| DC_3-32-38_n28 | 0.7 | N/A | 0.7 | 0.6 |
| DC_3-38_n28-n78 | 1.0 | 0.3 | 0.5 | 0.8 |
| DC_3-40_n1-n78 | 0.6 | 0.36 | 0.5 | 0.86 |
| DC_3_n40-n41-n79 | 0.5 | 0.5 | 0.54/0.85 | 0.8 |
| DC_3_n40-n78-n105 | 0.5 | 0.5 | 0.8 | 0.5 |
| DC_3-41_n1-n78 DC_3-3-41_n1-n78 | 0.6 | 0.5 | 0.6 | 0.8 |
| DC_3-41_n3-n41 | 0.5 | 0.34 / 0.85 | 0.5 | 0.34 / 0.85 |
| DC_3-41_n3-n77 | 0.6 | 0.34 / 0.85 | 0.6 | 0.8 |
| DC_3-41_n3-n78 | 0.6 | 0.34 / 0.85 | 0.6 | 0.8 |
| DC_3-41_n28-n41 | 0.6 | 0.34 / 0.85 | 0.5 | 0.34 / 0.85 |
| DC_3-41_n28-n77 | 0.6 | 0.34 / 0.85 | 0.5 | 0.8 |
| DC_3-41_n28-n78 | 1.0 | 0.34 / 0.85 | 0.5 | 0.8 |
| DC_3-41_n41-n77 | 0.6 | 0.34 / 0.85 | 0.34 / 0.85 | 0.8 |
| DC_3-41_n41-n78 | 0.6 | 0.34 / 0.85 | 0.34 / 0.85 | 0.8 |
| DC_3-41-42_n77 | 1.0 | 0.34 / 0.85 | N/A | 0.8 |
| DC_3-41-42_n78 | 1.0 | 0.34 / 0.85 | N/A | 0.8 |
| DC_3-41-42_n79 | 1.0 | 0.34 / 0.85 | N/A | - |
| DC_3-42_n1-n77 | 0.6 | N/A | 0.6 | 0.8 |
| DC_3-42_n1-n78 | 0.6 | N/A | 0.6 | 0.8 |
| DC_3-42_n1-n79 | 0.6 | 0.8 | 0.6 | - |
| DC_3-42_n28-n77 | 0.6 | N/A | 0.8 | 0.8 |
| DC_3-42_n77-n79 | 0.6 | N/A | 0.8 | - |
| DC_3-42_n78-n79 | 0.6 | N/A | 0.8 | - |
| DC_5-7_n2-n66 | 0.3 | 0.5 | 0.5 | 0.5 |
| DC_5-7_n2-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_5-7_n2-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_5-7_n28-n78 | 0.6 | 0.6 | 0.5 | 0.9 |
| DC_5-7_n40-n77 DC_5-7-7_n40-n77 | 0.3 | 0.5 | 0.5 | 0.8 |
| DC_5-7_n40-n78 DC_5-7-7_n40-n78 | 0.3 | 0.5 | 0.5 | 0.8 |
| DC_5-7-66_n2 | 0.3 | 0.5 | 0.5 | 0.5 |
| DC_5-7-66_n7 DC_5-7-66-66_n7 | 0.3 | 0.5 | 0.5 | 0.5 |
| DC_5-7-(n)66 DC_5-7-7-(n)66 DC_5-7-66_n66 DC_5-7-7-66_n66 | 0.3 | 0.5 | 0.5 | 0.5 |
| DC_5-7-66_n77 | 0.3 | 0.5 | 0.5 | 0.8 |
| DC_5-7_n66-n77 | 0.5 | 0.8 | 1.0 | 0.8 |
| DC_5-7_n66-n78 | 0.5 | 0.8 | 1.0 | 0.8 |
| DC_5-7-66_n78 | 0.3 | 0.5 | 0.5 | 0.8 |
| DC_5-30-66_n2 | 0.3 | 0.3 | 0.5 | 0.5 |
| DC_5-30-66_n66 | 0.3 | 0.3 | 0.5 | 0.5 |
| DC_5-30-66_n77 DC_5-30-66-66_n77 | 0.6 | 0.3 | 0.6 | 0.8 |
| DC_5-48_(n)12 | 0.8 | 0.4 | 0.3 | 0.8 |
| DC_5-48-66_n12 | 0.8 | 0.8 | 0.6 | 0.4 |
| DC_5-48-66_n71 | 0.5 | 0.8 | 0.6 | 0.5 |
| DC_5-48-66_n77 | 0.6 | N/A | 0.6 | 0.8 |
| DC_5-66_n2-n41 | 0.6 | 0.5 | 0.5 | 0.81 / 1.32 |
| DC_5-66_n2-n66 | 0.3 | 0.5 | 0.5 | 0.5 |
| DC_5-66_n2-n77 DC_5-66-66_n2-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_5-66_n2-n78 | 0.3 | 0.5 | 0.5 | 0.8 |
| DC_5-66_n5-n77 DC_5-66-66_n5-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_5-66_(n)12 | 0.3 | 0.8 | 0.8 | 0.8 |
| DC_5-66_n66-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_7_n1-n40-n78 | 0.3 | 0.2 | 0.8 | 0.5 |
| DC_7_n1-n40-n78 | 0.8 | 0.6 | 0.9 | 0.8 |
| DC_7_n1-n75-n78 | 0.6 | 0.6 | - | 0.8 |
| DC_7-8_n1-n40 | 0.8 | 0.6 | 0.6 | 0.9 |
| DC_7-8_n1-n78 DC_7-7-8_n1-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_7-8-20_n1 | 0.6 | 0.6 | 0.6 | 0.5 |
| DC_7-8-20_n3 | 0.5 | 0.6 | 0.4 | 0.5 |
| DC_7-8_n28-n78 | 0.5 | 0.6 | 0.5 | 0.8 |
| DC_7-8-32_n1 | 0.7 | 0.6 | N/A | 0.7 |
| DC_7-8-32_n78 | 0.7 | 0.6 | N/A | 0.8 |
| DC_7-8-38_n1 | N/A | 0.5 | N/A | 0.5 |
| DC_7-8-40_n1 | 0.8 | 0.6 | 0.9 | 0.6 |
| DC_7-8-40_n78 | 0.5 | 0.6 | 0.39 | 0.89 |
| DC_7-8_n40-n78 | 0.5 | 0.3 | 0.5 | 0.8 |
| DC_7-12_n2-n66 | 0.5 | 0.8 | 0.5 | 0.5 |
| DC_7-12_n2-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_7-12_n2-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_7-12-66_n2 | 0.5 | 0.8 | 0.5 | 0.5 |
| DC_7-12-66_n25 | 0.5 | 0.8 | 0.5 | 0.5 |
| DC_7-12-66_n66 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_7-12-66_n77 | 0.8 | 0.5 | 1.0 | 0.8 |
| DC_7-12_n66-n77 | 0.8 | 0.5 | 1.0 | 0.8 |
| DC_7-12-66_n78 | 0.8 | 0.5 | 1.0 | 0.8 |
| DC_7-12_n66-n78 | 0.8 | 0.5 | 1.0 | 0.8 |
| DC_7-12-71_n77 | 0.5 | 0.5 | 0.8 | 0.8 |
| DC_7-13_n25-n66 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_7-7-13-(n)66 DC_7-13-(n)66 DC_7-13-66_n66 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_7-20_n1-n75 | 0.7 | 0.3 | 0.7 | N/A |
| DC_7-20_n1-n78 | 0.7 | 0.4 | 0.6 | 0.8 |
| DC_7-20_n3-n38 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_7-20_n3-n78 | 0.5 | 0.6 | 0.5 | 0.8 |
| DC_7-20_n8-n78 | 0.5 | 0.6 | 0.6 | 0.8 |
| DC_7-20-28_n1 | 0.6 | 0.6 | 0.6 | 0.5 |
| DC_7-20-28_n3 | 0.5 | 0.6 | 0.5 | 0.5 |
| DC_7-20_n28-n78 | 0.3 | 0.6 | 0.6 | 0.8 |
| DC_7-20-32_n1 | 0.6 | 0.3 | N/A | 0.5 |
| DC_7-20-32_n3 | 0.7 | 0.3 | N/A | 0.3 |
| DC_7-20-32_n8 | 0.7 | 0.6 | N/A | 0.6 |
| DC_7-20-32_n28 | 0.3 | 0.5 | N/A | 0.7 |
| DC_7-20-32_n78 | 0.7 | 0.5 | N/A | 0.8 |
| DC_7-20-38_n1 | N/A | 0.3 | N/A | 0.5 |
| DC_7-20-38_n3 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_7-20-38_n8 | N/A | 0.6 | N/A | 0.6 |
| DC_7-20-38_n78 | N/A | 0.6 | N/A | 0.8 |
| DC_7-28_n1-n40 | 0.3 | 0.2 | - | 0.8 |
| DC_7-28_n1-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_7-28_n3-n78 | 0.8 | 0.5 | 1.0 | 0.8 |
| DC_7-28_n5-n40 | 0.8 | 0.6 | 0.6 | 0.9 |
| DC_7-28_n7-n78 | 0.3 | 0.3 | 0.3 | 0.8 |
| DC_7-28-66_n7 | 0.5 | 0.6 | 0.5 | 0.5 |
| DC_7-28-66_n66 | 0.5 | 0.6 | 0.5 | 0.5 |
| DC_7-28-32_n1 | 0.7 | 0.6 | N/A | 0.7 |
| DC_7-28-32_n3 | 0.7 | 0.3 | N/A | 0.7 |
| DC_7-28-38_n1 | N/A | 0.6 | N/A | 0.5 |
| DC_7-28-38_n78 | N/A | 0.3 | N/A | 0.8 |
| DC_7-28_n38-n78 | N/A | 0.3 | N/A | 0.8 |
| DC_7-28_n40-n78 | 0.5 | 0.3 | 0.5 | 0.8 |
| DC_7-29-66_n78 | 0.5 | N/A | 0.6 | 0.8 |
| DC_7-32_n1-n78 | 0.2 | - | 0.2 | 0.5 |
| DC_7-38_n3-n78 | N/A | N/A | 0.6 | 0.8 |
| DC_7-40_n1-n78 | 0.5 | 0.56 | 0.6 | 0.86 |
| DC_7_n40-n78-n105 | 0.5 | 0.5 | 0.8 | 0.5 |
| DC_7-66_n2-n66 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_7-66_n2-n71 | 0.5 | 0.5 | 0.5 | 0.6 |
| DC_7-66_n2-n77 | 0.5 | 0.6 | 0.6 | 0.8 |
| DC_7-66_n2-n78 | 0.5 | 0.6 | 0.6 | 0.8 |
| DC_7-66_n12-n77 | 0.8 | 1.0 | 0.5 | 0.8 |
| DC_7-66_n12-n78 | 0.8 | 1.0 | 0.5 | 0.8 |
| DC_7-66_n25-n66 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_7-66_n38-n78 DC_7-7-66_n38-n78 | N/A | 0.6 | N/A | 0.8 |
| DC_7-66_n66-n71 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_7-66-71_n25 | 0.5 | 0.5 | 0.6 | 0.5 |
| DC_7-66_n66-n77 | 0.5 | 0.6 | 0.6 | 0.8 |
| DC_7-(n)66-n78 DC_7-7-(n)66-n78 DC_7-66_n66-n78 DC_7-7-66_n66-n78 | 0.5 | 0.6 | 0.6 | 0.8 |
| DC_7-66-71_n2 | 0.5 | 0.5 | 0.3 | 0.5 |
| DC_7-66-71_n77 | 0.6 | 0.6 | 0.3 | 0.8 |
| DC_7-66_n71-n77 | 0.6 | 0.6 | 0.3 | 0.8 |
| DC_7-66-71_n78 | 0.6 | 0.6 | 0.3 | 0.8 |
| DC_7-66_n71-n78 | 0.6 | 0.6 | 0.3 | 0.8 |
| DC_7-71_n2-n66 | 0.6 | 0.6 | 0.6 | 0.5 |
| DC_7-71_n2-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_7-71_n2-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_7-71_n66-n77 | 0.6 | 0.3 | 0.6 | 0.8 |
| DC_7-71_n66-n78 | 0.6 | 0.3 | 0.6 | 0.8 |
| DC_8_n1-n3-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_8-(n)3-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_8_n3-n28-n77 | 0.6 | 0.6 | 0.5 | 0.8 |
| DC_8_n3-n28-n79 | 0.6 | 0.5 | 0.3 | 0.8 |
| DC_8_n3-n77-n79 | 0.6 | 0.6 | 0.8 | 0.8 |
| DC_8-11_n1-n77 | 0.6 | 0.4 | 0.6 | 0.8 |
| DC_8-11_n3-n28 | 0.6 | 0.8 | 0.9 | 0.6 |
| DC_8-11_n3-n77 | 0.6 | 0.8 | 0.9 | 0.8 |
| DC_8-11_n3-n79 | 0.3 | 0.8 | 0.9 | 0.8 |
| DC_8-11_n28-n77 | 0.6 | 0.4 | 0.6 | 0.8 |
| DC_8-11_n77-n79 | 0.6 | 0.4 | 0.8 | 0.5 |
| DC_8-20-28_n3 | 0.6 | 0.5 | 0.5 | 0.3 |
| DC_8-20-28_n78 | 0.6 | 0.6 | 0.5 | 0.8 |
| DC_8-20-32_n1 | 0.4 | 0.4 | N/A | 0.5 |
| DC_8-20-32_n3 | 0.4 | 0.5 | N/A | 0.3 |
| DC_8-20-38_n1 | 0.6 | 0.5 | 0.5 | 0.5 |
| DC_8_n28-n77-n79 | 0.6 | 0.5 | 0.8 | 0.8 |
| DC_8-32-38_n1 | 0.3 | N/A | 0.5 | 0.5 |
| DC_8_n39-n40-n41 | 0.3 | 0.3 | 0.3 | 0.3 |
| DC_8_n39-n40-n79 | 0.3 | 0.3 | 0.3 | 0.8 |
| DC_8_n40-n41-n79 | 0.3 | 0.3 | 0.3 | - |
| DC_8-40_n1-n78 | 0.3 | 0.56 | 0.5 | 0.86 |
| DC_8-41_n1-n3 | 0.3 | 0.54 / 0.85 | 0.5 | 0.5 |
| DC_8-41_n1-n77 | 0.6 | 0.5 | 0.6 | 0.8 |
| DC_8-41_n1-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_8-41_n3-n77 | 0.6 | 0.310 / 0.811 | 0.6 | 0.8 |
| DC_8-42_n1-n3 | 0.6 | 0.8 | 0.3 | 0.6 |
| DC_8-42_n1-n77 | 0.6 | N/A | 0.6 | 0.8 |
| DC_8-42_n3-n28 | 0.6 | 0.8 | 0.6 | 0.8 |
| DC_8-42_n3-n77 | 0.6 | 0.8 | 0.6 | 0.8 |
| DC_8-42_n28-n77 | 0.6 | 0.8 | 0.8 | 0.8 |
| DC_11_n3-n28-n77 | 0.8 | 0.9 | 0.6 | 0.8 |
| DC_11_n3-n77-n79 | 0.8 | 0.9 | 0.8 | 0.8 |
| DC_12-30-66_n2 | 0.8 | 0.3 | 0.5 | 0.5 |
| DC_12-30-66_n66 | 0.8 | 0.3 | 0.5 | 0.5 |
| DC_12-30-66_n77 DC_12-30-66-66_n77 | 0.8 | 0.3 | 0.6 | 0.8 |
| DC_12-48_(n)5 | 0.8 | 0.4 | 0.3 | 0.8 |
| DC_12-48-66_n5 | 0.8 | 0.8 | 0.8 | 0.3 |
| DC_12-66_(n)5 | 0.3 | 0.8 | 0.8 | 0.3 |
| DC_12-66_n2-n41 | 0.8 | 0.5 | 0.5 | 0.51 / 12 |
| DC_12-66_n2-n66 | 0.8 | 0.5 | 0.5 | 0.5 |
| DC_12-66_n2-n77 | 0.3 | 0.5 | 0.5 | 0.8 |
| DC_12-66_n2-n78 | 0.3 | 0.5 | 0.5 | 0.8 |
| DC_12-66_n66-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_13-48-66_n77 | 0.3 | N/A | 0.6 | 0.8 |
| DC_13-66_n2-n77 | 0.3 | 0.6 | 0.6 | 0.8 |
| DC_13-66_n5-n48 | 0.4 | 0.6 | 0.8 | 0.8 |
| DC_13-66_n5-n77 DC_13-66-66_n5-n77 | 0.5 | 0.6 | 0.6 | 0.8 |
| DC_13-66_n66-n77 | 0.3 | 0.6 | 0.6 | 0.8 |
| DC_14-30-66-n2 | 0.3 | 0.3 | 0.5 | 0.5 |
| DC_14-30-66_n66 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_14-30-66_n77 DC_14-30-66-66_n77 | 0.6 | 0.3 | 0.6 | 0.8 |
| DC_18-41_n3-n77 | 0.3 | 0.34 / 0.85 | 0.6 | 0.8 |
| DC_18-41_n3-n78 | 0.3 | 0.34 / 0.85 | 0.6 | 0.8 |
| DC_19_n1-n77-n79 | 0.3 | 0.6 | 0.8 | 0.5 |
| DC_19_n1-n78-n79 | 0.3 | 0.3 | 0.8 | 0.5 |
| DC_19-21_n1-n77 | 0.3 | 0.4 | 0.3 | 0.8 |
| DC_19-21_n1-n78 | 0.3 | 0.4 | 0.6 | 0.8 |
| DC_19-21_n1-n79 | 0.3 | 0.4 | 0.3 | - |
| DC_19-21-42_n1 | 0.3 | 0.4 | 0.8 | 0.3 |
| DC_19-21-42_n77 | 0.3 | 0.4 | N/A | 0.8 |
| DC_19-21-42_n78 | 0.3 | 0.4 | N/A | 0.8 |
| DC_19-21-42_n79 | 0.3 | 0.4 | N/A | - |
| DC_19-21_n77-n79 | 0.3 | 0.4 | 0.8 | - |
| DC_19-21_n78-n79 | 0.3 | 0.4 | 0.8 | - |
| DC_19-42_n1-n77 | 0.3 | N/A | 0.6 | 0.8 |
| DC_19-42_n1-n78 | 0.3 | N/A | 0.3 | 0.8 |
| DC_19-42_n1-n79 | 0.3 | 0.8 | 0.3 | - |
| DC_19-42_n77-n79 | 0.3 | N/A | 0.8 | - |
| DC_19-42_n78-n79 | 0.3 | N/A | 0.8 | - |
| DC_20-(n)3-n67 | 0.5 | 0.3 | 0.3 | N/A |
| DC_20-28-32_n1 | 0.6 | 0.6 | N/A | 0.5 |
| DC_20-28-32_n3 | 0.5 | 0.6 | N/A | 0.5 |
| DC_20-28-38_n1 | 0.6 | 0.6 | 0.5 | 0.5 |
| DC_20-32_n1-n28 | 0.6 | N/A | 0.3 | 0.7 |
| DC_20-32-38_n1 | 0.3 | N/A | 0.5 | 0.5 |
| DC_20-38_n3-n78 | 0.6 | 0.5 | 0.6 | 0.8 |
| DC_20-41_n1-n78 | 0.3 | 0.5 | 0.5 | 0.8 |
| DC_20-67-(n)3 | 0.5 | 0.3 | N/A | 0.3 |
| DC_21_n1-n77-n79 | 0.4 | 0.6 | 0.8 | 0.5 |
| DC_21_n1-n78-n79 | 0.4 | 0.6 | 0.8 | 0.5 |
| DC_21-28-42_n77 | 0.4 | 0.5 | N/A | 0.8 |
| DC_21-28-42_n78 | 0.4 | 0.5 | N/A | 0.8 |
| DC_21-28-42_n79 | 0.4 | 0.5 | N/A | - |
| DC_21_n28-n77-n79 | 0.4 | 0.5 | 0.8 | 0.5 |
| DC_21_n28-n78-n79 | 0.4 | 0.5 | 0.8 | - |
| DC_21-42_n1-n77 | 0.4 | N/A | 0.6 | 0.8 |
| DC_21-42_n1-n78 | 0.4 | N/A | 0.3 | 0.8 |
| DC_21-42_n1-n79 | 0.4 | 0.8 | 0.3 | - |
| DC_21-42_n77-n79 | 0.4 | N/A | 0.8 | - |
| DC_21-42_n78-n79 | 0.4 | N/A | 0.8 | - |
| DC_28_n1-n40-n78 | 0.2 | 0.2 | 0.8 | 0.5 |
| DC_28_n5-n40-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_28-32-38_n1 | 0.7 | N/A | 0.5 | 0.5 |
| DC_28-41-42_n78 | 0.5 | 0.3 | N/A | 0.8 |
| DC_29-30-66_n2 DC_29-30-66-66_n2 | N/A | 0.3 | 0.5 | 0.5 |
| DC_29-30-66_n66 | N/A | 0.3 | 0.5 | 0.5 |
| DC_29-30-66_n77 | N/A | 0.3 | 0.6 | 0.8 |
| DC_30-66-(n)5 | 0.3 | 0.3 | 0.5 | 0.3 |
| DC_42_n1-n77-n79 | 0.8 | 0.6 | 0.8 | - |
| DC_42_n1-n78-n79 | 0.8 | 0.3 | 0.8 | - |
| DC_42_n3-n28-n77 | 0.8 | 0.6 | 0.8 | 0.8 |
| DC_46-66_n25-n41 | N/A | 0.5 | 0.5 | 0.41 / 0.92 |
| DC_46-66_n25-n71 | N/A | 0.5 | 0.5 | 0.3 |
| DC_46-66_n41-n71 | N/A | 0.5 | 0.41 / 0.92 | 0.6 |
| DC_48-66_n25-n48 | 0.8 | 0.6 | 0.6 | 0.8 |
| DC_66-71_n2-n41 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_66-71_n2-n66 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_66-71_n2-n77 | 0.5 | 0.3 | 0.5 | 0.8 |
| DC_66-71_n2-n78 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_66-71_n66-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. NOTE 3: The values in the table reflect what can be achieved with the present state of the art technology. They shall be reconsidered when the state of the art technology progresses. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 6: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. NOTE 7: Void. NOTE 8: Void. NOTE 9: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx NOTE 10: The requirement is applied for UE transmitting on the frequency range of 2515 - 2690 MHz. NOTE 11: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.3B.3.3	Inter-band EN-DC within FR1

7.3B.3.3.3	ΔRIB,c for EN-DC four bands

Table 7.3B.3.3.3-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_1-(n)3-n8 | - | - | - | - |
| DC_1-3_n3-n41 | - | - | - | 03 / 0.54 |
| DC_1-3_n3-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-3-5_n28 | - | - | 0.2 | 0.2 |
| DC_1-3_n5-n40 | - | - | 0.2 | 0.8 |
| DC_1-3-5_n40 | - | - | 0.2 | 0.8 |
| DC_1-3-5_n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-3_n3-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-3-5_n78 | 0.2 | 0.2 | - | 0.5 |
| DC_1-3-7_n28 DC_1-3-7-7_n28 | - | - | - | 0.2 |
| DC_1-3-7_n40 | - | - | 0.3 | 0.8 |
| DC_1-3-7_n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-3-7_n78 DC_1-3-3-7_n78 DC_1-3-3-7-7_n78 DC_1-3-7-7_n78 DC_1-1-3-3-7_n78 | 0.3 | 0.3 | 0.3 | 0.5 |
| DC_1-3_n7-n78 | 0.3 | 0.3 | 0.3 | 0.5 |
| DC_1-3-7_n105 | - | - | - | 0.3 |
| DC_1-3-8_n7 | - | - | 0.2 | - |
| DC_1-3-8_n28 | - | - | 0.2 | 0.2 |
| DC_1-3-8_n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1_n3-n8-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-8_n3-n79 | - | - | - | 0.5 |
| DC_1-3-8_n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-3_n8-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-3-11_n28 | - | 0.3 | 0.5 | 0.2 |
| DC_1-3-11_n77 | 0.2 | 0.3 | 0.5 | 0.5 |
| DC_1-3-18_n28 | - | - | - | 0.2 |
| DC_1-3-18_n41 | - | - | - | 0.26 |
| DC_1-3-28_n3 | - | - | 0.2 | - |
| DC_1-3-18_n77 | 0.2 | 0.2 | - | 0.5 |
| DC_1-3-18_n78 | 0.2 | 0.2 | - | 0.5 |
| DC_1-3-19_n78 | 0.2 | 0.2 | - | 0.5 |
| DC_1-3-20_n28 | - | - | 0.2 | 0.2 |
| DC_1-3-20_n41 | - | - | - | 01 / 0.54 |
| DC_1-3-20_n78 DC_1-1-3-20_n78 DC_1-3-3-20_n78 | 0.2 | 0.2 | - | 0.5 |
| DC_1-3-21_n77 | 0.2 | 0.3 | 0.5 | 0.5 |
| DC_1-3-21_n78 | 0.2 | 0.3 | 0.5 | 0.5 |
| DC_1-3-21_n79 | - | 0.3 | 0.5 | - |
| DC_1-3-26_n78 | 0.6 | 0.6 | 0.3 | 0.8 |
| DC_1-3_n26-n78 | 0.2 | 0.2 | - | 0.5 |
| DC_1-3-28_n5 | - | - | 0.2 | 0.2 |
| DC_1-3-28_n7 | - | - | 0.2 | - |
| DC_1-3-28_n38 | - | - | 0.2 | - |
| DC_1-3-28_n40 | - | - | 0.2 | - |
| DC_1-3_n28-n75 | 0.2 | - | 0.2 | - |
| DC_1-3-28_n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-3_n28-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1_n3-n28-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-3-28_n78 DC_1-3-3-28_n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-3_n28-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-3-28_n79 | 0.2 | 0.2 | 0.2 | - |
| DC_1-3_n28-n79 | 0.2 | 0.2 | 0.2 | - |
| DC_1_n3-n28-n79 | 0.2 | 0.2 | 0.2 | - |
| DC_1-3-32_n28 | - | 0.5 | - | 0.5 |
| DC_1-3-32_n78 | - | - | - | 0.5 |
| DC_1-3-38_n28 | - | - | - | 0.2 |
| DC_1-3_n38-n78 | - | 0.2 | - | 0.5 |
| DC_1-3-38_n78 | 0.2 | 0.2 | 0.4 | 0.5 |
| DC_1-3_n40-n77 | - | 0.2 | 0.45 | 0.55 |
| DC_1-3-40_n78 | 0.2 | 0.2 | 0.48 | 0.58 |
| DC_1-3_n40-n78 | - | 0.2 | 0.45 | 0.55 |
| DC_1-3_n40-n105 | - | 0.2 | 0.4 | 0.3 |
| DC_1-3-41_n3 | - | - | 03 / 0.54 | - |
| DC_1-3-41_n28 | - | - | 03 / 0.54 | 0.2 |
| DC_1-3-41_n41 | - | - | 03 / 0.54 | 03 / 0.54 |
| DC_1-3_(n)41 | - | - | 03 / 0.54 | 03 / 0.54 |
| DC_1-3-41_n77 | 0.2 | 0.2 | - | 0.5 |
| DC_1-3_n41-n77 | 0.2 | 0.2 | - | 0.5 |
| DC_1-3-41_n78 | 0.2 | 0.2 | - | 0.5 |
| DC_1-3_n41-n78 | 0.2 | 0.2 | - | 0.5 |
| DC_1-3-41_n79 | - | - | 03 / 0.54 | - |
| DC_1-3-42_n28 | 0.2 | 0.2 | 0.5 | 0.5 |
| DC_1-3-42_n77 | 0.2 | 0.2 | 0.5 | 0.5 |
| DC_1-3-42_n78 | 0.2 | 0.2 | 0.5 | 0.5 |
| DC_1-3-42_n79 | 0.2 | 0.2 | 0.5 | - |
| DC_1-3_n75-n78 | - | - | - | 0.5 |
| DC_1-3_n77-n79 | 0.2 | 0.2 | 0.5 | - |
| DC_1_n3-n77-n79 | 0.2 | 0.2 | 0.5 | - |
| DC_1-3_n78-n79 | 0.2 | 0.2 | 0.5 | - |
| DC_1-3_n78-n105 | 0.2 | 0.2 | 0.5 | 0.3 |
| DC_1-3_SUL_n78-n80 | 0.2 | 0.2 | 0.5 | - |
| DC_1-5-7_n28 | - | 0.2 | - | 0.2 |
| DC_1-5-7_n40 DC_1-5-7-7_n40 | - | 0.2 | 0.3 | 0.8 |
| DC_1-5-7_n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-5-7_n40 DC_1-5-7-7_n40 | 0.6 | 0.6 | 0.8 | 0.9 |
| DC_1-5-7_n78 DC_1-5-7-7_n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-5_n28-n78 | - | 0.2 | 0.2 | 0.8 |
| DC_1-5_n40-n77 | 0.2 | 0.2 | 0.48 | 0.5 |
| DC_1-5_n40-n78 | 0.2 | 0.2 | 0.48 | 0.5 |
| DC_1-7_n3-n38 | - | - | - | 0.2 |
| DC_1-7_n3-n78 | - | - | - | 0.5 |
| DC_1-7_n5-n40 | - | 0.3 | 0.2 | 0.8 |
| DC_1-7_n7-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-7-8_n7 | 0.2 | 0.2 | 0.2 | 0.2 |
| DC_1-7-8_n20 | - | - | 0.2 | 0.2 |
| DC_1-7-8_n28 DC_1-7-7-8_n28 | - | - | 0.2 | 0.2 |
| DC_1-7-8_n78 DC_1-7-7-8_n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-7_n8-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-7-20_n28 | - | - | 0.2 | 0.2 |
| DC_1-7-20_n38 | - | - | - | 0.2 |
| DC_1-7-20_n78 DC_1-1-7-20_n78 DC_1-7-7-20_n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-7-26_n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-7_n26-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-7-28_n3 | - | - | 0.2 | - |
| DC_1-7-28_n5 | - | - | 0.2 | 0.2 |
| DC_1-7-28_n7 | - | - | 0.2 | - |
| DC_1-7-28_n20 | - | - | 0.2 | 0.2 |
| DC_1-7-28_n38 | - | - | 0.2 | - |
| DC_1-7-28_n40 | - | 0.3 | 0.2 | 0.8 |
| DC_1-7-28_n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-7_n28-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-7-32_n8 | - | - | - | 0.2 |
| DC_1-7-32_n28 | - | - | - | 0.2 |
| DC_1-7-32_n78 | 0.2 | 0.2 | - | 0.5 |
| DC_1-7-38_n8 | - | - | 0.2 | - |
| DC_1-7-38_n28 | - | - | 0.2 | 0.2 |
| DC_1-7-38_n78 | 0.6 | 0.6 | - | 0.8 |
| DC_1-7_n40-n77 DC_1-7-7_n40-n77 | 0.2 | - | 0.4 | 0.5 |
| DC_1-7-40_n78 | 0.2 | - | 0.48 | 0.58 |
| DC_1-7_n40-n78 DC_1-7-7_n40-n78 | 0.2 | - | 0.4 | 0.5 |
| DC_1-7_n40-n105 | 0.2 | - | 0.4 | 0.3 |
| DC_1-7_n75-n78 | 0.6 | 0.6 | - | 0.8 |
| DC_1-7_n78-n105 | 0.6 | 0.6 | 0.5 | 0.3 |
| DC_1-8_n3-n28 | - | 0.2 | - | 0.2 |
| DC_1-8_n3-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-8-11_n3 | - | - | 0.3 | 0.5 |
| DC_1-8-11_n28 | - | 0.2 | - | 0.2 |
| DC_1-8-11_n77 | 0.2 | 0.2 | - | 0.5 |
| DC_1-8-11_n78 | - | 0.2 | - | 0.5 |
| DC_1-8-20_n28 | - | 0.2 | 0.2 | 0.2 |
| DC_1-8-20_n78 | - | 0.2 | - | 0.5 |
| DC_1-8-28_n3 | - | 0.2 | 0.2 | - |
| DC_1-8_n28-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-8-28_n78 | - | 0.2 | 0.2 | 0.5 |
| DC_1-8_n28-n78 | - | 0.2 | 0.2 | 0.5 |
| DC_1-8_n28-n79 | 0.3 | 0.3 | 0.6 | 0.5 |
| DC_1-8-32_n3 | - | - | 0.5 | 0.3 |
| DC_1-8-32_n78 | - | 0.2 | - | 0.5 |
| DC_1-8_n40-n78 | - | 0.2 | 0.4 | 0.5 |
| DC_1-8-40_n78 | 0.2 | 0.2 | 0.48 | 0.58 |
| DC_1-8-42_n3 | - | 0.2 | 0.5 | 0.2 |
| DC_1-8-42_n28 | - | 0.2 | 0.5 | 0.5 |
| DC_1-8-42_n77 | 0.2 | 0.2 | 0.5 | 0.5 |
| DC_1-8_n77-n79 | 0.2 | 0.2 | 0.5 | - |
| DC_1-11_n3-n28 | - | 0.3 | 0.5 | 0.2 |
| DC_1-11_n3-n77 | 0.2 | 0.3 | 0.5 | 0.5 |
| DC_1-11-18_n77 | 0.2 | - | - | 0.5 |
| DC_1-11-18_n78 | - | - | - | 0.5 |
| DC_1-11_n28-n77 | 0.2 | - | 0.2 | 0.5 |
| DC_1-18_n3-n77 | 0.2 | - | 0.2 | 0.5 |
| DC_1-18_n3-n78 | 0.2 | - | 0.2 | 0.5 |
| DC_1-11_n3-n79 | - | 0.3 | 0.5 | 0.5 |
| DC_1-11-18_n3 | - | 0.5 | - | 0.3 |
| DC_1-11-18_n28 | - | - | - | 0.1 |
| DC_1-11_n77-n79 | 0.2 | - | 0.5 | - |
| DC_1-18_n28-n41 | - | - | 0.2 | - |
| DC_1-18-28_n77 | - | - | - | 0.5 |
| DC_1-18_n28-n77 | - | - | - | 0.5 |
| DC_1-18-28_n78 | - | - | - | 0.5 |
| DC_1-18_n28-n78 | - | - | - | 0.5 |
| DC_1-18-41_n3 | - | - | 03 / 0.54 | - |
| DC_1-18-41_n77 | 0.2 | - | - | 0.5 |
| DC_1-18_n41-n77 | 0.2 | - | - | 0.5 |
| DC_1-18-41_n78 | - | - | - | 0.5 |
| DC_1-18_n41-n78 | - | - | - | 0.5 |
| DC_1-18-42_n77 | - | - | 0.5 | 0.5 |
| DC_1-18-42_n78 | - | - | 0.5 | 0.5 |
| DC_1-18-42_n79 | - | - | 0.5 | - |
| DC_1-19-42_n77 | 0.2 | - | 0.5 | 0.5 |
| DC_1-19-42_n78 | - | - | 0.5 | 0.5 |
| DC_1-19-42_n79 | - | - | 0.5 | - |
| DC_1-19_n77-n79 | 0.3 | 0.3 | 0.5 | - |
| DC_1-19_n78-n79 | 0.3 | 0.3 | 0.5 | - |
| DC_1-20_n3-n78 | - | - | - | 0.5 |
| DC_1-20_n7-n78 | - | - | - | 0.5 |
| DC_1-20_n8-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-20-28_n3 | - | 0.2 | 0.2 | - |
| DC_1-20_n28-n75 | - | 0.2 | 0.2 | - |
| DC_1-20-28_n78 | - | 0.2 | 0.2 | 0.5 |
| DC_1-20_n28-n78 | - | 0.2 | 0.2 | 0.5 |
| DC_1-20-32_n8 | 0.5 | 0.4 | - | 0.4 |
| DC_1-20-32_n28 | - | 0.2 | - | 0.2 |
| DC_1-20-32_n78 | - | - | - | 0.5 |
| DC_1-20-38_n78 | - | - | 0.4 | 0.5 |
| DC_1-20-40_n78 | - | - | - | 0.88 |
| DC_1-20_n41-n78 | - | - | - | 0.5 |
| DC_1-21_n28-n77 | 0.2 | - | 0.2 | 0.5 |
| DC_1-21_n28-n78 | 0.2 | - | 0.2 | 0.5 |
| DC_1-21_n28-n79 | 0.3 | - | 0.3 | - |
| DC_1-21-42_n77 | 0.2 | - | 0.5 | 0.5 |
| DC_1-21-42_n78 | - | - | 0.5 | 0.5 |
| DC_1-21-42_n79 | - | - | 0.5 | - |
| DC_1-21_n77-n79 | - | - | 0.5 | - |
| DC_1-21_n78-n79 | - | - | 0.5 | - |
| DC_1-28_n3-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-28_n3-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-28_n5-n40 | - | 0.2 | 0.2 | 0.8 |
| DC_1-28-(n)7 | - | 0.2 | - | - |
| DC_1-28_n7-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-28-32_n3 | - | 0.2 | - | - |
| DC_1-28-40_n78 | - | 0.2 | 0.45 | 0.55 |
| DC_1-28_n40-n78 | - | 0.2 | 0.45 | 0.55 |
| DC_1-28-42_n77 | 0.2 | 0.2 | 0.5 | 0.5 |
| DC_1-28-42_n78 | - | 0.2 | 0.5 | 0.5 |
| DC_1-28-42_n79 | - | 0.2 | 0.5 | - |
| DC_1_n28-n77-n79 | 0.3 | 0.3 | 0.5 | - |
| DC_1_n28-n78-n79 | 0.3 | 0.3 | 0.5 | - |
| DC_1-38_n3-n78 | - | - | 0.2 | 0.5 |
| DC_1-38_n7-n78 | 0.2 | - | 0.2 | 0.5 |
| DC_1-38_n28-n78 | - | - | 0.2 | 0.5 |
| DC_1_n40-n78-n105 | - | 0.4 | 0.5 | 0.3 |
| DC_1-41_n3-n41 | - | 03 / 0.54 | - | 03 / 0.54 |
| DC_1-41_n3-n77 | 0.2 | 03 / 0.54 | 0.2 | 0.5 |
| DC_1-41_n3-n78 | 0.2 | 03 / 0.54 | 0.2 | 0.5 |
| DC_1-41_n28-n41 | - | 03 / 0.54 | - | 03 / 0.54 |
| DC_1-41_n28-n77 | 0.2 | - | 0.2 | 0.5 |
| DC_1-41_n28-n78 | - | - | 0.2 | 0.5 |
| DC_1-41_n41-n77 | - | - | - | 0.5 |
| DC_1-41_n41-n78 | - | - | - | 0.5 |
| DC_1-41-42_n77 | - | - | 0.5 | 0.5 |
| DC_1-41-42_n78 | - | - | 0.5 | 0.5 |
| DC_1-41-42_n79 | - | - | 0.5 | - |
| DC_1-41-42_n79 | - | - | 0.5 | - |
| DC_1-42_n3-n28 | - | 0.5 | 0.2 | 0.5 |
| DC_1-42_n3-n77 | 0.2 | 0.5 | 0.2 | 0.5 |
| DC_1-42_n28-n77 | 0.2 | 0.5 | 0.5 | 0.5 |
| DC_1-42_n77-n79 | 0.2 | 0.5 | 0.5 | - |
| DC_1-42_n78-n79 | 0.2 | 0.5 | 0.5 | - |
| DC_2-4-7_n28 | 0.3 | 0.5 | 0.5 | 0.2 |
| DC_2-4-7_n78 | 0.3 | 0.3 | - | 0.8 |
| DC_2-5_n2-n41 | - | 0.2 | - | - |
| DC_2-5_n2-n66 | 0.3 | - | 0.3 | 0.3 |
| DC_2-5_n2-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_2-5_n2-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_2-5_n5-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_2-5-7_n66 DC_2-2-5-7_n66 DC_2-5-7-7_n66 | 0.3 | - | 0.5 | 0.5 |
| DC_2-5-7_n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_2-5-7_n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_2-5_(n)12 | - | 0.5 | 0.3 | 0.3 |
| DC_2-12_(n)5 | - | 0.5 | 0.5 | - |
| DC_2-5-30_n2 | 0.4 | - | 0.5 | 0.4 |
| DC_2-5-30_n66 | 0.4 | - | 0.5 | 0.4 |
| DC_2-5-30_n77 DC_2-2-5-30_n77 | 0.2 | 0.2 | - | 0.5 |
| DC_2-5_n41-n66 | 0.3 | 0.2 | 0.51 / 12 | 0.5 |
| DC_2-5-48_n12 | 0.2 | 0.5 | 0.5 | 0.3 |
| DC_2-5-48_n71 | 0.2 | - | 0.5 | - |
| DC_2-5-48_n77 | 0.2 | 0.2 | 0.5 | 0.5 |
| DC_2-5-66_n2 | 0.3 | - | 0.3 | 0.3 |
| DC_2-5-66_n5 | 0.3 | - | 0.3 | - |
| DC_2-5-66_n7 | 0.3 | - | 0.5 | 0.5 |
| DC_2-5-66_n12 | 0.2 | 0.5 | 0.5 | 0.3 |
| DC_2-5-66_n30 DC_2-2-5-66_n30 DC_2-5-66-66_n30 | 0.4 | - | 0.4 | 0.5 |
| DC_2-5-66_n41 DC_2-2-5-66_n41 | 0.3 | - | 0.5 | 0.51 / 12 |
| DC_2-5-66_n48 DC_2-5-66-66_n48 | 0.3 | - | 0.3 | 0.5 |
| DC_2-2-5-(n)66 DC_2-2-5-66-(n)66 DC_2-5-(n)66 DC_2-5-66_n66 DC_2-5-66-(n)66 | 0.3 | - | 0.3 | 0.3 |
| DC_2-5-66_n71 | 0.3 | - | 0.3 | - |
| DC_2-5-66_n77 DC_2-2-5-66_n77 DC_2-5-66-66_n77 | 0.3 | - | 0.3 | 0.5 |
| DC_2-5_n66-n77 | 0.3 | 0.2 | 0.3 | 0.5 |
| DC_2-5-66_n78 | 0.3 | 0.5 | 0.3 | 0.5 |
| DC_2-5_n66-n78 | 0.3 | - | 0.3 | 0.5 |
| DC_2-7_n2-n66 | 0.3 | 0.5 | 0.3 | 0.5 |
| DC_2-7_n2-n71 | - | - | - | 0.2 |
| DC_2-7_n2-n77 | 0.2 | 0.5 | 0.2 | 0.5 |
| DC_2-7_n2-n78 | 0.2 | 0.5 | 0.2 | 0.5 |
| DC_2-7-12_n66 DC_2-2-7-12_n66 | 0.3 | 0.5 | 0.5 | 0.3 |
| DC_2-7-12_n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_2-7_n12-n77 | 0.2 | 0.2 | 0.5 | 0.5 |
| DC_2-7-12_n78 DC_2-2-7-12_n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_2-7-13_n66 DC_2-7-7-13_n66 DC_2-2-7-7-13_n66 | 0.3 | 0.5 | - | 0.5 |
| DC_2-7_n25-n66 | 0.3 | 0.5 | 0.3 | 0.5 |
| DC_2-7-28_n66 | 0.3 | 0.5 | 0.2 | 0.5 |
| DC_2-7-28_n78 | 0.3 | 0.5 | 0.2 | 0.5 |
| DC_2-7-29_n78 DC_2-7-7-29_n78 | 0.2 | 0.5 | 0.2 | 0.5 |
| DC_2-7_n38-n66 DC_2-7-7_n38-n66 | 0.3 | - | - | 0.5 |
| DC_2-7-38_n78 | 0.2 | - | - | 0.5 |
| DC_2-7_n38-n78 DC_2-7-7_n38-n78 | 0.2 | - | - | 0.5 |
| DC_2-7-66_n2 | 0.3 | 0.5 | 0.5 | 0.3 |
| DC_2-7-66_n7 DC_2-7-66-66_n7 | 0.3 | 0.5 | 0.5 | 0.5 |
| DC_2-7-66_n12 | 0.3 | 0.5 | 0.5 | 0.5 |
| DC_2-7-66_n25 | 0.3 | 0.5 | 0.5 | 0.5 |
| DC_2-7-66_n28 | 0.3 | 0.5 | 0.5 | 0.2 |
| DC_2-7-66_n38 DC_2-2-7-66_n38 | 0.3 | 0.5 | 0.5 | 0.5 |
| DC_2-7-(n)66 DC_2-7-66_n66 DC_2-7-7-(n)66 DC_2-7-7-66_n66 DC_2-7-7-66-(n)66 DC_2-7-66-(n)66 | 0.3 | 0.5 | 0.5 | 0.5 |
| DC_2-7-66_n71 DC_2-2-7-66_n71 | 0.3 | 0.5 | 0.5 | - |
| DC_2-7_n66-n71 | 0.3 | 0.5 | 0.5 | - |
| DC_2-7-66_n77 | 0.2 | 0.5 | 0.5 | 0.5 |
| DC_2-7_n66-n77 | 0.3 | 0.5 | 0.5 | 0.5 |
| DC_2-7-66_n78 DC_2-2-7-66_n78 DC_2-7-7-66_n78 DC_2-7-66-66_n78 DC_2-7-7-66-66_n78 | 0.3 | - | 0.3 | 0.5 |
| DC_2-7_n66-n78 DC_2-7-7_n66-n78 | 0.3 | 0.5 | 0.5 | 0.5 |
| DC_2-7-71_n2 | - | - | 0.2 | - |
| DC_2-7-71_n66 DC_2-2-7-71_n66 | 0.3 | 0.5 | - | 0.3 |
| DC_2-7-71_n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_2-7_n71-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_2-7-71_n78 DC_2-2-7 -71_n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_2-7_n71-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_2-12_n2-n66 | 0.3 | 0.5 | 0.3 | 0.3 |
| DC_2-12_n2-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_2-12_n2-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_2-12-30_n2 | 0.4 | - | 0.5 | 0.4 |
| DC_2-12-30_n66 | 0.4 | 0.5 | 0.5 | 0.4 |
| DC_2-12-30_n77 DC_2-2-12-30_n77 | 0.2 | 0.2 | - | 0.5 |
| DC_2-12_n41-n66 | 0.3 | 0.5 | 0.5 | 0.3 |
| DC_2-12-48_n5 | 0.3 | 0.3 | 0.5 | 0.5 |
| DC_2-12-66_n2 | 0.3 | 0.5 | 0.3 | 0.3 |
| DC_2-12-66_n5 | 0.3 | 0.5 | 0.5 | 0.3 |
| DC_2-12-66_n7 | 0.3 | 0.5 | 0.3 | 0.3 |
| DC_2-12-66_n30 DC_2-2-12-66_n30 DC_2-12-66-66_n30 | 0.4 | 0.5 | 0.4 | 0.5 |
| DC_2-12-66_n41 DC_2-2-12-66_n41 | 0.5 | 0.8 | 0.5 | 0.5 |
| DC_2-2-12-(n)66 DC_2-12-(n)66 DC_2-12-66_n66 | 0.3 | 0.5 | 0.3 | 0.3 |
| DC_2-12-66_n77 DC_2-2-12-66_n77 DC_2-12-66-66_n77 | 0.2 | 0.5 | 0.5 | 0.5 |
| DC_2-12_n66-n77 | 0.2 | 0.5 | 0.5 | 0.5 |
| DC_2-12-66_n78 DC_2-2-12-66_n78 | 0.3 | - | 0.3 | 0.5 |
| DC_2-12_n66-n78 | 0.3 | - | 0.3 | 0.5 |
| DC_2-13_n2-n77 | 0.2 | - | 0.2 | 0.5 |
| DC_2-13_n5-n77 DC_2-2-13_n5-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_2-13_n25-n66 | 0.3 | - | 0.3 | 0.3 |
| DC_2-13-48_n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_2-13-66_n2 | 0.3 | - | 0.3 | 0.3 |
| DC_2-13-66_n5 | 0.3 | - | 0.3 | - |
| DC_2-13-66_n48 | 0.3 | - | 0.3 | 0.5 |
| DC_2-2-13-(n)66 DC_2-2-13-66-(n)66 DC_2-13-(n)66 DC_2-13-66_n66 DC_2-13-66-(n)66 | 0.3 | - | 0.3 | 0.3 |
| DC_2-13-66_n77 DC_2-2-13-66_n77 DC_2-2-13-66-66_n77 DC_2-13-66-66_n77 | 0.3 | - | 0.3 | 0.5 |
| DC_2-13_n66-n77 | 0.3 | - | 0.3 | 0.5 |
| DC_2-14-30_n2 | 0.3 | - | 0.3 | 0.3 |
| DC_2-14-30_n66 | 0.4 | - | 0.5 | 0.4 |
| DC_2-14-30_n77 DC_2-2-14-30_n77 | 0.2 | 0.2 | - | 0.5 |
| DC_2-14-66_n2 DC_2-14-66-66_n2 | 0.3 | - | 0.3 | 0.3 |
| DC_2-14-66_n30 DC_2-2-14-66_n30 DC_2-14-66-66_n30 | 0.4 | - | 0.4 | 0.5 |
| DC_2-14-66_n66 DC_2-2-14-66_n66 | 0.3 | - | 0.3 | 0.3 |
| DC_2-14-66_n77 DC_2-2-14-66_n77 DC_2-14-66-66_n77 | 0.2 | 0.2 | 0.5 | 0.5 |
| DC_2-28-66_n7 | 0.3 | 0.2 | 0.5 | 0.5 |
| DC_2-28-66_n66 | 0.3 | 0.2 | 0.3 | 0.3 |
| DC_2-29-30_n2 | 0.4 | - | 0.5 | 0.4 |
| DC_2-29-30_n66 | 0.4 | - | 0.5 | 0.4 |
| DC_2-29-30_n77 DC_2-2-29-30_n77 | 0.2 | 0.2 | - | 0.5 |
| DC_2-29-66_n2 DC_2-29-66-66_n2 | 0.3 | - | 0.3 | 0.3 |
| DC_2-29-66_n30 DC_2-2-29-66_n30 DC_2-29-66-66_n30 | 0.4 | - | 0.4 | 0.5 |
| DC_2-29-(n)66 DC_2-2-29-(n)66 DC_2-29-66_n66 | 0.3 | - | 0.3 | 0.3 |
| DC_2-29-66_n77 | 0.2 | 0.5 | 0.5 | 0.5 |
| DC_2-29-66_n78 | 0.3 | - | 0.3 | 0.5 |
| DC_2-30-(n)5 DC_2-2-30-(n)5 | 0.4 | - | 0.5 | - |
| DC_2-30-66_n2 DC_2-30-66-66_n2 | 0.4 | 0.5 | 0.4 | 0.4 |
| DC_2-30-66_n5 | 0.4 | 0.5 | 0.4 | - |
| DC_2-30-66_n66 | 0.4 | 0.5 | 0.4 | 0.4 |
| DC_2-30-66_n77 DC_2-2-30-66_n77 DC_2-30-66-66_n77 | 0.2 | 0.5 | 0.4 | 0.5 |
| DC_2-46_n41-n66 | 0.3 | - | 0.5 | 0.5 |
| DC_2-46_n41-n71 | - | - | - | 0.2 |
| DC_2-46-48_n2 | 0.3 | - | 0.5 | 0.3 |
| DC_2-46-48_n5 | 0.2 | - | 0.5 | - |
| DC_2-46-48_n66 | 0.3 | - | 0.5 | 0.3 |
| DC_2-46-66_n5 | 0.3 | - | 0.3 | - |
| DC_2-46-66_n41 | 0.3 | - | 0.5 | 0.51 / 12 |
| DC_2-48_(n)5 | 0.2 | - | 0.5 | - |
| DC_2-48_n48-n66 | 0.3 | 0.4 | 0.4 | 0.3 |
| DC_2-48-66_n2 | 0.3 | 0.5 | 0.3 | 0.3 |
| DC_2-48-66_n5 | 0.3 | 0.5 | 0.3 | - |
| DC_2-48-66_n12 | 0.3 | 0.5 | 0.3 | - |
| DC_2-48-66_n66 | 0.3 | 0.5 | 0.3 | 0.3 |
| DC_2-48-66_n71 | 0.3 | 0.5 | 0.3 | - |
| DC_2-48-66_n77 | 0.3 | 0.5 | 0.3 | 0.5 |
| DC_2-66_n2-n41 | 0.3 | 0.5 | 0.3 | 0.5 |
| DC_2-66_n2-n66 | 0.3 | 0.3 | 0.3 | 0.3 |
| DC_2-66_n2-n71 | 0.3 | 0.3 | 0.3 | - |
| DC_2-66_n2-n77 DC_2-66-66_n2-n77 | 0.2 | 0.3 | 0.3 | 0.5 |
| DC_2-66_n2-n78 | 0.3 | 0.3 | 0.3 | 0.5 |
| DC_2-66_(n)5 DC_2-2-66_(n)5 DC_2-66-66_(n)5 | 0.3 | - | 0.3 | - |
| DC_2-66_n5-n77 | 0.3 | 0.3 | - | 0.5 |
| DC_2-66_n12-n77 | 0.2 | 0.5 | 0.5 | 0.5 |
| DC_2-66_n12-n78 | 0.3 | 0.3 | - | 0.5 |
| DC_2-66_n25-n66 | 0.3 | 0.3 | 0.3 | 0.3 |
| DC_2-66_n38-n78 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_2-66_n41-n71 | 0.3 | 0.3 | 0.51 / 12 | 0.5 |
| DC_2-66_n66-n71 | 0.3 | 0.3 | 0.3 | - |
| DC_2-66-71_n38 DC_2-2-66-71_n38 | 0.3 | 0.5 | - | 0.5 |
| DC_2-66-71_n41 DC_2-2-66-71_n41 | 0.3 | 0.3 | 0.5 | 0.51 / 12 |
| DC_2-66-71_n66 | 0.3 | 0.3 | - | 0.3 |
| DC_2-66-(n)71 | 0.3 | 0.3 | - | - |
| DC_2-66-71_n71 | 0.3 | 0.3 | - | - |
| DC_2-66-71_n77 | 0.3 | 0.3 | 0.2 | 0.5 |
| DC_2-66_n71-n77 | 0.3 | 0.3 | 0.2 | 0.5 |
| DC_2-66-71_n78 DC_2-2-66-71_n78 | 0.3 | 0.5 | - | 0.5 |
| DC_2-66_n71-n78 | 0.3 | 0.5 | - | 0.5 |
| DC_2-66_n66-n77 | 0.3 | 0.3 | 0.3 | 0.5 |
| DC_2-(n)66-n78 DC_2-66_n66-n78 | 0.3 | 0.3 | 0.3 | 0.5 |
| DC_2-66-71_n2 | 0.3 | 0.3 | - | 0.3 |
| DC_2-71_n2-n41 | - | 0.2 | - | - |
| DC_2-71_n2-n66 | 0.3 | - | 0.3 | 0.3 |
| DC_2-71_n2-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_2-71_n2-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_2-71_n41-n66 | 0.3 | 0.5 | 0.51 / 12 | 0.3 |
| DC_2-71_n66-n77 | 0.3 | - | 0.5 | 0.5 |
| DC_2-71_n66-n78 | 0.3 | - | 0.5 | 0.5 |
| DC_3_n1-n28-n75 | 0.3 | 0.3 | 0.7 | - |
| DC_3_n1-n75-n78 | 0.6 | 0.6 | - | 0.8 |
| DC_3_n1-n40-n78 | 0.2 | 0.2 | 0.48 | 0.5 |
| DC_3_n1-n77-n79 | 0.2 | 0.2 | 0.5 | - |
| DC_3_n1-n78-n79 | 0.2 | 0.2 | 0.5 | - |
| DC_3-5-7_n28 | - | 0.2 | - | 0.2 |
| DC_3-5-7_n40 DC_3-5-7-7_n40 | - | 0.2 | 0.3 | 0.8 |
| DC_3-5-7_n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-5-7_n78 DC_3-5-7-7_n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-5_n28-n78 | 0.2 | 0.2 | 0.2 | 0.8 |
| DC_3-5_n40-n77 | 0.2 | 0.2 | 0.48 | 0.5 |
| DC_3-5_n40-n78 | 0.2 | 0.2 | 0.48 | 0.5 |
| DC_3_n5-n40-n78 | 0.2 | 0.2 | 0.48 | 0.5 |
| DC_3-5-41_n79 | - | - | 03 / 0.54 | - |
| DC_3-7_n1-n8 DC_3-3-7_n1-n8 DC_3-7-7_n1-n8 DC_3-3-7-7_n1-n8 | - | - | - | 0.2 |
| DC_3-7_n1-n28 | - | - | - | 0.2 |
| DC_3-7_n1-n40 | - | 0.3 | - | 0.8 |
| DC_3-7_n1-n78 | 0.3 | 0.3 | 0.3 | 0.5 |
| DC_3-7_n3-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-7_n5-n40 | - | 0.3 | 0.2 | 0.8 |
| DC_3-7-7_n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-7-8_n1 DC_3-3-7-8_n1 DC_3-7-7-8_n1 DC_3-3-7-7-8_n1 | - | - | 0.2 | - |
| DC_3-7-8_n7 | - | - | 0.2 | - |
| DC_3-7-8_n28 DC_3-7-7-8_n28 | - | - | 0.2 | 0.1 |
| DC_3-7-8_n40 | - | - | 0.2 | 0.5 |
| DC_3-7-8_n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-7-8_n78 DC_3-3-7-8_n78 DC_3-7-7-8_n78 DC_3-3-7-7-8_n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-7_n8-n78, DC_3-3-7_n8-n78, DC_3-7-7_n8-n78, DC_3-3-7-7_n8-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-7_n7-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-7-20_n28 | - | - | 0.2 | 0.1 |
| DC_3-7-20_n38 | - | - | - | 0.2 |
| DC_3-7-20_n78 DC_3-3-7-20_n78 DC_3-7-7-20_n78 | 0.2 | 0.2 | - | 0.5 |
| DC_3-7_n26-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-7-26_n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-7-28_n1 DC_3-7-7-28_n1 | - | - | 0.2 | - |
| DC_3-7-28_n40 | - | 0.3 | - | 0.8 |
| DC_3-7-28_n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-7_n28-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-7-32_n28 | 0.5 | - | - | 0.5 |
| DC_3-7-32_n78 | 0.2 | 0.2 | - | 0.5 |
| DC_3-7-38_n28 | - | - | 0.2 | 0.2 |
| DC_3-7-38_n78 | 0.2 | - | - | 0.5 |
| DC_3-7-40_n1 | - | 0.3 | 0.8 | - |
| DC_3-7_n40-n77 DC_3-7-7_n40-n77 | 0.2 | - | 0.48 | 0.58 |
| DC_3-7_n40-n78 DC_3-7-7_n40-n78 | 0.2 | - | 0.48 | 0.58 |
| DC_3-7_n40-n105 | 0.2 | - | 0.4 | 0.3 |
| DC_3-7_n75-n78 | 0.2 | 0.2 | - | 0.5 |
| DC_3-7_n78-n79 DC_3-3-7_n78-n79 DC_3-7-7_n78-n79 DC_3-3-7-7_n78-n79 | 0.2 | 0.2 | 0.5 | 0.5 |
| DC_3-7_n78-n105 | 0.2 | 0.2 | 0.5 | 0.3 |
| DC_3-7_SUL_n78-n80 | 0.2 | 0.2 | 0.5 | - |
| DC_3-8_n77-n79 | 0.6 | 0.3 | 0.8 | - |
| DC_3-8_n1-n28 | - | 0.2 | - | 0.2 |
| DC_3-8_n1-n40 | - | - | 0.1 | 0.2 |
| DC_3-8_n1-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-8_n1-n78 DC_3-3-8_n1-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-8-11_n28 | 0.3 | 0.2 | 0.5 | 0.2 |
| DC_3-8-11_n77 | 0.3 | 0.2 | 0.5 | 0.5 |
| DC_3-8-20_n28 | - | 0.2 | 0.1 | 0.1 |
| DC_3-8-20_n78 | 0.2 | 0.2 | - | 0.5 |
| DC_3-8_n28-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-8-28_n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-8_n28-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-8-32_n1 | - | - | 0.5 | 0.3 |
| DC_3-8-32_n28 | - | - | - | 0.2 |
| DC_3-8-32_n78 | 0.3 | 0.2 | 0.5 | 0.5 |
| DC_3-8_n40-n41 | - | - | - | 03/0.54 |
| DC_3-8-41_n78 DC_3-3-8-41_n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-8-40_n1 | - | - | 0.2 | 0.1 |
| DC_3-8-40_n78 | 0.2 | 0.2 | 0.48 | 0.58 |
| DC_3-8_n40-n78 | 0.2 | 0.2 | 0.4 | 0.5 |
| DC_3-8-41_n1 DC_3-3-8-41_n1 | - | 0.2 | - | - |
| DC_3-8_n41-n79 | 0.2 | - | 03/0.54 | - |
| DC_3-8-42_n77 | 0.2 | 0.2 | 0.5 | 0.5 |
| DC_(n)3-n8-n77 | 0.6 | 0.6 | 0.3 | 0.8 |
| DC_3-8_SUL_n78-n80 | 0.2 | 0.2 | 0.5 | - |
| DC_3-11_n28-n77 | 0.3 | 0.5 | 0.2 | 0.5 |
| DC_3-18_n3-n41 | 0.2 | - | 0.2 | - |
| DC_3-18_n3-n77 | 0.2 | - | 0.2 | 0.5 |
| DC_3-18_n3-n78 | 0.2 | - | 0.2 | 0.5 |
| DC_3-18_n28-n41 | 0.2 | - | 0.2 | - |
| DC_3-18_n28-n77 | 0.2 | - | 0.2 | 0.5 |
| DC_3-18_n28-n78 | 0.2 | - | 0.2 | 0.5 |
| DC_3-18_n41-n77 | 0.2 | - | - | 0.5 |
| DC_3-18_n41-n78 | 0.2 | - | - | 0.5 |
| DC_3-18-42_n77 | - | - | 0.5 | 0.5 |
| DC_3-18-42_n78 | - | - | 0.5 | 0.5 |
| DC_3-18-42_n79 | 0.2 | - | 0.5 | - |
| DC_3-19_n1-n77 | 0.2 | - | 0.2 | 0.5 |
| DC_3-19_n1-n78 | 0.2 | - | 0.2 | 0.5 |
| DC_3-19-21_n77 | 0.3 | - | 0.5 | 0.5 |
| DC_3-19-21_n78 | 0.3 | - | 0.5 | 0.5 |
| DC_3-19-21_n79 | 0.3 | - | 0.5 | - |
| DC_3-19-42_n1 | 0.2 | - | 0.5 | 0.2 |
| DC_3-19-42_n77 | 0.2 | - | 0.5 | 0.5 |
| DC_3-19-42_n78 | 0.2 | - | 0.5 | 0.5 |
| DC_3-19-42_n79 | 0.2 | - | 0.5 | - |
| DC_3-19_n77-n79 | 0.2 | - | 0.5 | - |
| DC_3-19_n78-n79 | 0.2 | - | 0.5 | - |
| DC_3-20_n1-n28 | - | - | 0.2 | 0.2 |
| DC_3-20_n1-n78 | 0.2 | - | 0.2 | 0.5 |
| DC_3-20_n7-n28 | - | 0.1 | - | 0.1 |
| DC_3-20_n3-n67 | - | 0.1 | - | 0.1 |
| DC_3-20_n8-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-20-28_n1 | - | 0.2 | 0.2 | - |
| DC_3-20_n28-n75 | 0.5 | - | 0.5 | - |
| DC_3-20-28_n78 DC_3-3-20-28_n78 | 0.2 | 0.1 | 0.2 | 0.5 |
| DC_3-20_n28-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-20-32_n28 | 0.5 | - | - | 0.5 |
| DC_3-20-32_n78 | 0.2 | - | - | 0.5 |
| DC_3-20-38_n78 | 0.2 | 0.2 | 0.4 | 0.5 |
| DC_3-20_n38-n78 | 0.2 | 0.2 | 0.4 | 0.5 |
| DC_3-20-40_n78 | 0.2 | 0.2 | 0.45 | 0.55 |
| DC_3-20-41_n1 DC_3-3-20-41_n1 | 0.2 | - | 0.2 | 0.5 |
| DC_3-20-41_n78 DC_3-3-20-41_n78 DC_3-20_n41-n78 | - | - | - | 0.5 |
| DC_3-20-67_n3 | - | 0.1 | 0.1 | - |
| DC_3_20_SUL_n78-n80 | 0.2 | - | 0.5 | - |
| DC_3-21_n1-n77 | 0.3 | 0.5 | 0.2 | 0.5 |
| DC_3-21_n1-n78 | 0.3 | 0.5 | 0.2 | 0.5 |
| DC_3-21_n1-n79 | 0.3 | 0.5 | - | - |
| DC_3-21_n28-n77 | 0.3 | 0.5 | 0.2 | 0.5 |
| DC_3-21_n28-n78 | 0.3 | 0.5 | 0.2 | 0.5 |
| DC_3-21_n28-n79 | 0.3 | 0.5 | 0.3 | - |
| DC_3-21-42_n1 | 0.3 | 0.5 | 0.5 | 0.2 |
| DC_3-21-42_n77 | 0.3 | 0.5 | 0.5 | 0.5 |
| DC_3-21-42_n78 | 0.3 | 0.5 | 0.5 | 0.5 |
| DC_3-21-42_n79 | 0.3 | 0.5 | 0.5 | - |
| DC_3-21_n77-n79 | 0.3 | 0.5 | 0.5 | - |
| DC_3-21_n78-n79 | 0.3 | 0.5 | 0.5 | - |
| DC_3-28_n1-n40 | - | 0.2 | - | - |
| DC_3-28_n1-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-28_n3-n78 | - | 0.2 | - | 0.5 |
| DC_3-28_n5-n40 | - | 0.2 | 0.2 | 0.8 |
| DC_3-28_n7-n78 DC_3-3-28_n7-n78 | 0.5 | 0.2 | 0.4 | 0.5 |
| DC_3-28-32_n1 | 0.5 | 0.5 | - | - |
| DC_3-28-40_n78 | 0.2 | 0.2 | 0.45 | 0.55 |
| DC_3-28_n40-n78 | 0.2 | 0.2 | 0.45 | 0.55 |
| DC_3-28_n41-n77 | 0.5 | 0.2 | 0.43 / 0.54 | 0.5 |
| DC_3-28-41_n78 | 0.5 | 0.2 | 0.43 / 0.54 | 0.5 |
| DC_3-28-42_n77 | 0.2 | 0.2 | 0.5 | 0.5 |
| DC_3-28-42_n78 | 0.2 | 0.2 | 0.5 | 0.5 |
| DC_3-28-42_n79 | 0.2 | 0.2 | 0.5 | - |
| DC_3_n28-n77-n79 | 0.2 | 0.2 | 0.5 | - |
| DC_3_n28-n78-n79 | 0.2 | 0.2 | 0.5 | - |
| DC_3-32_n1-n28 | - | - | 0.2 | 0.2 |
| DC_3-32_n1-n78 | - | - | - | 0.5 |
| DC_3-38_n7-n78 | 0.2 | 0.4 | 0.2 | 0.5 |
| DC_3-32-38_n28 | - | - | - | 0.2 |
| DC_3-38_n28-n78 | 0.5 | 0.4 | 0.2 | 0.5 |
| DC_3-40_n1-n78 | 0.2 | 0.45 | - | 0.55 |
| DC_3_n40-n41-n79 | - | - | 03/0.54 | 0.5 |
| DC_3_n40-n78-n105 | - | 0.4 | 0.8 | 0.3 |
| DC_3-41_n1-n78 DC_3-3-41_n1-n78 | 0.2 | - | 0.2 | 0.5 |
| DC_3-41_n3-n41 | - | 03 / 0.54 | - | 03 / 0.54 |
| DC_3-41_n3-n77 | 0.2 | 03 / 0.54 | 0.2 | 0.5 |
| DC_3-41_n3-n78 | 0.2 | 03 / 0.54 | 0.2 | 0.5 |
| DC_3-41_n28-n41 | 0.2 | 03 / 0.54 | 0.2 | 03 / 0.54 |
| DC_3-41_n28-n77 | 0.2 | 03 / 0.54 | 0.2 | 0.5 |
| DC_3-41_n28-n78 | 0.5 | 0.43 / 0.54 | 0.2 | 0.5 |
| DC_3-41_n41-n77 | 0.2 | 03 / 0.54 | 03 / 0.54 | 0.5 |
| DC_3-41_n41-n78 | 0.2 | 03 / 0.54 | 03 / 0.54 | 0.5 |
| DC_3-41-42_n77 | 0.5 | 03 / 0.54 | 0.5 | 0.5 |
| DC_3-41-42_n78 | 0.5 | 03 / 0.54 | 0.5 | 0.5 |
| DC_3-41-42_n79 | 0.5 | 03 / 0.54 | 0.5 | - |
| DC_3-42_n1-n77 | 0.2 | 0.5 | 0.2 | 0.5 |
| DC_3-42_n1-n78 | 0.2 | 0.5 | 0.2 | 0.5 |
| DC_3-42_n1-n79 | 0.2 | 0.5 | 0.2 | - |
| DC_3-42_n28-n77 | 0.2 | 0.5 | 0.5 | 0.5 |
| DC_3-42_n77-n79 | 0.2 | 0.5 | 0.5 | - |
| DC_3-42_n78-n79 | 0.2 | 0.5 | 0.5 | - |
| DC_5-7_n2-n66 | - | 0.5 | 0.3 | 0.5 |
| DC_5-7_n2-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_5-7_n2-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_5-7-7_n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_5-7_n28-n78 | 0.2 | - | 0.2 | 0.8 |
| DC_5-7_n40-n77 DC_5-7-7_n40-n77 | 0.2 | - | 0.4 | 0.5 |
| DC_5-7_n40-n78 DC_5-7-7_n40-n78 | 0.2 | - | 0.4 | 0.5 |
| DC_5-7-66_n2 | - | 0.5 | 0.5 | 0.3 |
| DC_5-7-66_n7 DC_5-7-66-66_n7 | - | 0.5 | 0.5 | 0.5 |
| DC_5-7-(n)66 DC_5-7-7-(n)66 DC_5-7-66_n66 DC_5-7-7-66_n66 | 0.3 | - | 0.3 | 0.3 |
| DC_5-7-66_n77 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_5-7_n66-n77 | 0.2 | 0.5 | 0.5 | 0.5 |
| DC_5-7_n66-n78 | 0.2 | 0.5 | 0.5 | 0.5 |
| DC_5-7-66_n78 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_5-30-66_n2 | - | 0.5 | 0.4 | 0.4 |
| DC_5-30-66_n66 | - | 0.5 | 0.4 | 0.4 |
| DC_5-30-66_n77 DC_5-30-66-66_n77 | 0.2 | 0.5 | 0.4 | 0.5 |
| DC_5-48_(n)12 | 0.5 | 0.3 | - | 0.5 |
| DC_5-48-66_n12 | 0.5 | 0.5 | 0.2 | 0.3 |
| DC_5-48-66_n71 | - | 0.5 | 0.2 | - |
| DC_5-48-66_n77 | 0.2 | 0.5 | 0.2 | 0.5 |
| DC_5-66_n2-n41 | 0.2 | 0.3 | 0.5 | 0.51 / 12 |
| DC_5-66_n2-n66 | - | 0.3 | 0.3 | 0.3 |
| DC_5-66_n2-n77 DC_5-66-66_n2-n77 | 0.2 | 0.3 | 0.3 | 0.5 |
| DC_5-66_n2-n78 | - | 0.3 | 0.3 | 0.5 |
| DC_5-66_n5-n77 DC_5-66-66_n5-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_5-66_(n)12 | - | 0.5 | 0.5 | 0.5 |
| DC_5-66_n66-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_7_n1-n75-n78 | 0.2 | 0.2 | - | 0.5 |
| DC_7-8_n1-n78 | 0.3 | 0.2 | - | 0.8 |
| DC_7_n1-n8-n78 | 0.3 | - | 0.2 | 0.8 |
| DC_7-8_n1-n78 DC_7-7-8_n1-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_7-8-20_n1 | - | 0.2 | 0.2 | - |
| DC_7-8-20_n3 | - | 0.2 | - | - |
| DC_7-8_n28-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_7-8-32_n1 | - | 0.2 | - | - |
| DC_7-8-32_n78 | - | 0.2 | - | 0.5 |
| DC_7-8-38_n1 | - | - | 0.2 | - |
| DC_7-8-40_n1 | 0.3 | 0.2 | 0.8 | - |
| DC_7-8-40_n78 | - | 0.2 | 0.48 | 0.58 |
| DC_7-8_n40-n78 | - | 0.2 | 0.4 | 0.5 |
| DC_7-12_n2-n66 | 0.5 | 0.5 | 0.3 | 0.3 |
| DC_7-12_n2-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_7-12_n2-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_7-12-66_n2 | 0.5 | 0.5 | 0.3 | 0.3 |
| DC_7-12-66_n25 | 0.3 | 0.5 | 0.5 | 0.5 |
| DC_7-12-66_n66 | 0.5 | - | 0.5 | 0.5 |
| DC_7-12-66_n77 | 0.5 | 0.2 | 0.5 | 0.5 |
| DC_7-12_n66-n77 | 0.5 | 0.2 | 0.5 | 0.5 |
| DC_7-12-66_n78 | 0.5 | 0.2 | 0.5 | 0.5 |
| DC_7-12_n66-n78 | 0.5 | 0.2 | 0.5 | 0.5 |
| DC_7-12-71_n77 | 0.2 | 0.5 | 0.5 | 0.5 |
| DC_7-13_n25-n66 | 0.5 | - | 0.3 | 0.5 |
| DC_7-7-13-(n)66 DC_7-13-(n)66 DC_7-13-66_n66 | 0.5 | - | 0.5 | 0.5 |
| DC_7-20_n1-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_7-20_n3-n38 | - | 0.2 | - | 0.2 |
| DC_7-20_n3-n78 | - | - | - | 0.5 |
| DC_7-20_n8-n78 | - | 0.2 | 0.2 | 0.5 |
| DC_7-20-28_n1 | - | 0.2 | 0.2 | - |
| DC_7-20-28_n3 | - | 0.2 | 0.1 | - |
| DC_7-20-28_n78 | 0.3 | 0.6 | 0.6 | 0.8 |
| DC_7-20_n28-n78 | - | 0.2 | 0.2 | 0.5 |
| DC_7-20-32_n8 | - | 0.2 | - | 0.2 |
| DC_7-20-32_n28 | - | - | - | 0.2 |
| DC_7-20-32_n78 | - | - | - | 0.5 |
| DC_7-20-38_n3 | - | - | 0.2 | - |
| DC_7-20-38_n8 | - | 0.2 | 0.2 | 0.2 |
| DC_7-20-38_n78 | - | - | 0.4 | 0.6 |
| DC_7-28_n1-n40 | 0.3 | 0.2 | - | 0.8 |
| DC_7-28_n3-n78 | 0.5 | 0.2 | 0.5 | 0.5 |
| DC_7-28_n5-n40 | 0.3 | 0.2 | 0.2 | 0.8 |
| DC_7-28_n7-n78 | - | - | - | 0.5 |
| DC_7-28-32_n1 | - | 0.2 | - | - |
| DC_7-28-38_n1 | - | 0.2 | 0.2 | - |
| DC_7-28-38_n78 | - | 0.2 | 0.4 | 0.5 |
| DC_7-28_n38-n78 | - | 0.2 | 0.4 | 0.5 |
| DC_7-28_n40-n78 | - | 0.2 | 0.4 | 0.5 |
| DC_7-29-66_n78 | 0.5 | - | 0.5 | 0.5 |
| DC_7-32_n1-n78 | 0.6 | - | 0.6 | 0.8 |
| DC_7-38_n3-n78 | 0.5 | 0.5 | 0.2 | 0.5 |
| DC_7_n40-n78-n105 | - | 0.4 | 0.8 | 0.3 |
| DC_7-66_n38-n78 DC_7-7-66_n38-n78 | - | 0.2 | - | 0.5 |
| DC_7-28_n1-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_7-28-66_n7 | 0.5 | 0.2 | 0.5 | 0.5 |
| DC_7-28-66_n66 | 0.5 | 0.2 | 0.5 | 0.5 |
| DC_7-40_n1-n78 | - | 0.45 | 0.2 | 0.55 |
| DC_7-66_n2-n66 | 0.5 | 0.3 | 0.3 | 0.5 |
| DC_7-66_n2-n71 | 0.5 | 0.5 | 0.3 | 0.2 |
| DC_7-66_n2-n77 | - | 0.3 | 0.3 | 0.5 |
| DC_7-66_n2-n78 | - | 0.3 | 0.3 | 0.5 |
| DC_7-66_n12-n77 | 0.5 | 0.5 | 0.2 | 0.5 |
| DC_7-66_n12-n77 | 0.5 | 0.5 | 0.2 | 0.5 |
| DC_7-66_n25-n66 | 0.5 | 0.5 | 0.3 | 0.5 |
| DC_7-66_n66-n71 | 0.5 | 0.5 | 0.5 | 0.1 |
| DC_7-66_n66-n77 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_7-(n)66-n78 DC_7-7-(n)66-n78 DC_7-66_n66-n78 DC_7-7-66_n66-n78 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_7-66-71_n2 | 0.5 | 0.5 | - | 0.3 |
| DC_7-66-71_n25 | 0.3 | 0.5 | - | 0.5 |
| DC_7-66-71_n77 | 0.2 | 0.2 | - | 0.5 |
| DC_7-66_n71-n77 | 0.2 | 0.2 | - | 0.5 |
| DC_7-66-71_n78 | 0.2 | 0.2 | - | 0.5 |
| DC_7-66_n71-n78 | 0.2 | 0.2 | - | 0.5 |
| DC_7-71_n2-n66 | 0.5 | 0.2 | 0.2 | 0.5 |
| DC_7-71_n2-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_7-71_n2-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_7-71_n66-n77 | 0.2 | - | 0.2 | 0.5 |
| DC_7-71_n66-n78 | 0.2 | - | 0.2 | 0.5 |
| DC_8_n1-n3-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_8_n3-n28-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_8_n3-n77-n79 | 0.2 | 0.2 | 0.5 | 0.5 |
| DC_8-11_n1-n77 | 0.2 | - | 0.2 | 0.5 |
| DC_8-11_n3-n28 | 0.2 | 0.3 | 0.5 | 0.2 |
| DC_8-11_n3-n77 | 0.2 | 0.3 | 0.5 | - |
| DC_8-11_n3-n79 | - | 0.3 | 0.5 | 0.5 |
| DC_8-11_n28-n77 | 0.2 | - | 0.2 | 0.5 |
| DC_8-11_n77-n79 | 0.2 | - | 0.5 | - |
| DC_8-20-28_n78 | 0.2 | 0.1 | 0.2 | 0.5 |
| DC_8-20-32_n3 | - | - | 0.5 | 0.3 |
| DC_8_n28-n77-n79 | 0.2 | 0.2 | 0.5 | 0.5 |
| DC_8_n39-n40-n79 | - | 0.3 | 0.3 | 0.5 |
| DC_8-40_n1-n78 | 0.2 | 0.45 | - | 0.55 |
| DC_8-41_n1-n3 | - | 03 / 0.54 | - | - |
| DC_8-41_n1-n77 | 0.2 | - | 0.2 | 0.5 |
| DC_8-41_n1-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_8-41_n3-n77 | 0.2 | 09 / 0.510 | 0.2 | 0.5 |
| DC_8-42_n1-n3 | 0.2 | 0.5 | - | 0.2 |
| DC_8-42_n1-n77 | 0.2 | 0.5 | 0.2 | 0.5 |
| DC_8-42_n3-n28 | 0.2 | 0.5 | 0.2 | 0.5 |
| DC_8-42_n3-n77 | 0.2 | 0.5 | 0.2 | 0.5 |
| DC_8-42_n28-n77 | 0.2 | 0.5 | 0.5 | 0.5 |
| DC_11_n3-n28-n77 | 0.3 | 0.5 | 0.2 | 0.5 |
| DC_11_n3-n77-n79 | 0.3 | 0.5 | 0.5 | 0.5 |
| DC_12-30-66_n2 | 0.5 | 0.5 | 0.4 | 0.4 |
| DC_12-30-66_n66 | 0.5 | 0.5 | 0.4 | 0.4 |
| DC_12-30-66_n77 DC_12-30-66-66_n77 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_12-48_(n)5 | 0.5 | 0.3 | - | 0.5 |
| DC_12-48-66_n5 | 0.5 | 0.5 | 0.5 | - |
| DC_12-66_(n)5 | - | 0.5 | 0.5 | - |
| DC_12-66_n2-n41 | 0.5 | 0.3 | 0.3 | 0.5 |
| DC_12-66_n2-n66 | 0.5 | 0.3 | 0.3 | 0.3 |
| DC_12-66_n2-n77 | - | 0.3 | 0.3 | 0.5 |
| DC_12-66_n2-n78 | - | 0.3 | 0.3 | 0.5 |
| DC_12-66_n66-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_13-48-66_n77 | - | 0.5 | 0.2 | 0.5 |
| DC_13-66_n2-n77 | - | 0.2 | 0.2 | 0.5 |
| DC_13-66_n5-n48 | 0.3 | 0.2 | 0.5 | 0.5 |
| DC_13-66_n5-n77 DC_13-66-66_n5-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_13-66_n66-n77 | - | 0.2 | 0.2 | 0.5 |
| DC_14-30-66-n2 | - | 0.5 | 0.4 | 0.4 |
| DC_14-30-66_n66 | - | 0.5 | 0.4 | 0.4 |
| DC_14-30-66_n77 DC_14-30-66-66_n77 | 0.2 | 0.5 | 0.5 | 0.5 |
| DC_18-41_n3-n77 | 0.2 | 03 / 0.54 | 0.2 | 0.5 |
| DC_18-41_n3-n78 | 0.2 | 03 / 0.54 | 0.2 | 0.5 |
| DC_19_n1-n77-n79 | 0.3 | 0.3 | 0.5 | - |
| DC_19_n1-n78-n79 | 0.3 | 0.3 | 0.5 | - |
| DC_19-21_n1-n77 | - | - | - | 0.5 |
| DC_19-21_n1-n78 | - | - | 0.2 | 0.5 |
| DC_19-21-42_n1 | - | - | 0.5 | - |
| DC_19-21-42_n77 | - | - | 0.5 | 0.5 |
| DC_19-21-42_n78 | - |  | 0.5 | 0.5 |
| DC_19-21-42_n79 | - | - | 0.5 | - |
| DC_19-21_n77-n79 | - | - | 0.5 | - |
| DC_19-21_n78-n79 | - | - | 0.5 | - |
| DC_19-42_n1-n77 | - | 0.5 | 0.2 | 0.5 |
| DC_19-42_n1-n78 | - | 0.5 | - | 0.5 |
| DC_19-42_n1-n79 | - | 0.5 | - | - |
| DC_19-42_n77-n79 | - | 0.5 | 0.5 | - |
| DC_19-42_n78-n79 | - | 0.5 | 0.5 | - |
| DC_20-(n)3-n67 | 0.1 | - | - | 0.1 |
| DC_20-28-32_n1 | 0.2 | 0.2 | - | - |
| DC_20-28-32_n3 | 0.3 | 0.2 | - | 0.3 |
| DC_20-28-38_n1 | 0.2 | 0.2 | - | - |
| DC_20-32_n1-n28 | 0.2 | - | - | 0.2 |
| DC_20-38_n3-n78 | 0.2 | 0.4 | 0.2 | 0.5 |
| DC_20-41_n1-n78 | - | - | - | 0.5 |
| DC_20-67-(n)3 | 0.1 | 0.1 | - | - |
| DC_21_n1-n77-n79 | - | 0.2 | 0.5 | - |
| DC_21_n1-n78-n79 | - | 0.2 | 0.5 | - |
| DC_21-28-42_n77 | - | 0.2 | 0.5 | 0.5 |
| DC_21-28-42_n78 | - | 0.2 | 0.5 | 0.5 |
| DC_21-28-42_n79 | - | 0.2 | 0.5 | - |
| DC_21_n28-n77-n79 | - | 0.2 | 0.5 | - |
| DC_21_n28-n78-n79 | - | 0.2 | 0.5 | - |
| DC_21-42_n1-n77 | - | 0.5 | 0.2 | 0.5 |
| DC_21-42_n1-n78 | - | 0.5 | - | 0.5 |
| DC_21-42_n1-n79 | - | 0.5 | - | - |
| DC_21-42_n77-n79 | - | 0.5 | 0.5 | - |
| DC_21-42_n78-n79 | - | 0.5 | 0.5 | - |
| DC_28_n5-n40-n78 | 0.2 | 0.2 | 0.5 | 0.5 |
| DC_28-32-38_n1 | 0.2 | - | - | - |
| DC_28-41-42_n78 | 0.2 | 0.4 | 0.5 | 0.5 |
| DC_29-30-66_n2 DC_29-30-66-66_n2 | - | 0.5 | 0.4 | 0.4 |
| DC_29-30-66_n66 | - | 0.5 | 0.3 | 0.3 |
| DC_29-30-66_n77 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_30-66-(n)5 | 0.5 | - | 0.4 | 0.5 |
| DC_42_n1-n77-n79 | 0.5 | 0.2 | 0.5 | - |
| DC_42_n1-n78-n79 | 0.5 | 0.2 | 0.5 | - |
| DC_42_n3-n28-n77 | 0.5 | 0.2 | 0.5 | 0.5 |
| DC_46-66_n25-n41 | - | 0.3 | 0.3 | 0.51 / 12 |
| DC_46-66_n41-n71 | - | 0.3 | 0.51 / 12 | 0.2 |
| DC_48-66_n25-n48 | 0.4 | 0.3 | 0.3 | 0.4 |
| DC_66-71_n2-n41 | 0.5 | - | 0.3 | 0.5 |
| DC_66-71_n2-n66 | 0.3 | - | 0.3 | 0.3 |
| DC_2-5-66_n2-n66 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_66-71_n2-n77 | 0.5 | - | 0.3 | 0.5 |
| DC_66-71_n2-n78 | 0.5 | - | 0.3 | 0.5 |
| DC_66-71_n66-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 - 2690 MHz NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. NOTE 6: Void. NOTE 7: Void. NOTE 8: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. NOTE 9: The requirement is applied for UE transmitting on the frequency range of 2515 - 2690 MHz. NOTE 10: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.3B.2.3.5.2	MSD test points for intermodulation interference due to dual uplink operation for EN-DC in NR FR1 involving three bands

Table 7.3B.2.3.5.2-0: MSD test points for Pcell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A-(n)71AA | 66 | 1750 | 5 | 25 | 2150 | 5 | IMD4 |
|  | n71 | 678 | 10 | 10 (RBstart =0) | 632 | N/A | N/A |
| NOTE 1: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 2: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. |  |  |  |  |  |  |  |

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) |  | UL/DL BW (MHz) |  | UL LCRB |  | DL Fc (MHz) |  | MSD (dB) |  | IMD order |  |  |
| DC_1A-3A_n1A | n1 | 1950 | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | 3 | 1750 | 5 |  | 25 |  | 1845 |  | N/A |  | N/A |  |  |
|  | 1 | N/A | 5 |  | N/A |  | 2150 |  | 23 |  | IMD3 |  |  |
| DC_1A-3A_n28A DC_1A-3C_n28A | 1 | 1975 |  | 5 |  | 25 |  | 2165 |  | N/A |  | N/A |  |  |
|  | 3 | N/A |  | 5 |  | N/A |  | 1818.5 |  | 4.0 |  | IMD5 |  |  |
|  | n28 | 710.5 |  | 5 |  | 25 |  | 765.5 |  | N/A |  | N/A |  |  |
|  | 1 | N/A |  | 5 |  | N/A |  | 2139 |  | 11.0 |  | IMD4 |  |  |
|  | 3 | 1780 |  | 5 |  | 25 |  | 1875 |  | N/A |  | N/A |  |  |
|  | n28 | 710.5 |  | 5 |  | 25 |  | 765.5 |  | N/A |  | N/A |  |  |
| DC_1A-3A_n71A DC_1A-3A_n71B | 1 | N/A |  | 5 |  | N/A |  | 2150 |  | 5 |  | IMD4 |  |  |
|  | 3 | 1750 |  | 5 |  | 25 |  | 1845 |  | N/A |  | N/A |  |  |
|  | n71 | 675 |  | 5 |  | 25 |  | 629 |  | N/A |  | N/A |  |  |
| DC_1A_n3A-n28A | 1 | 1975 |  | 5 |  | 25 |  | 2165 |  | N/A |  | N/A |  |  |
|  | n3 | N/A |  | 5 |  | N/A |  | 1818.5 |  | 4.0 |  | IMD5 |  |  |
|  | n28 | 710.5 |  | 5 |  | 25 |  | 765.5 |  | N/A |  | N/A |  |  |
| DC_1A_n3A-n41A | 1 | 1977.5 |  | 5 |  | 25 |  | 2167.5 |  | N/A |  | N/A |  |  |
|  | n3 | 1712.5 |  | 5 |  | 25 |  | 1807.5 |  | N/A |  | N/A |  |  |
|  | n41 | N/A |  | 5 |  | N/A |  | 2507.5 |  | 5.0 |  | IMD5 |  |  |
| DC_1A_n3A-n75A | n75 | N/A |  | 5 |  | N/A |  | 1480 |  | 15.2 |  | IMD34 |  |  |
|  | n3 | 1720 |  | 5 |  | 25 |  | 1815 |  | N/A |  | N/A |  |  |
|  | 1 | 1960 |  | 5 |  | 25 |  | 2150 |  | N/A |  | N/A |  |  |
| DC_1A_n3A-n79A | 1 | 1930 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
|  | n3 | 1720 |  | 5 |  | 25 |  | 1815 |  | N/A |  | N/A |  |  |
|  | n79 | N/A |  | 40 |  | N/A |  | 4950 |  | 4.7 |  | IMD5 |  |  |
| DC_1A_n5A-n40A | 1 | 1977.5 |  | 5 |  | 25 |  | 2167.5 |  | N/A |  | N/A |  |  |
|  | n5 | 826.5 |  | 5 |  | 25 |  | 871.5 |  | N/A |  | N/A |  |  |
|  | n40 | N/A |  | 10 |  | N/A |  | 2305 |  | 9.0 |  | IMD4 |  |  |
|  | 1 | 1945 |  | 5 |  | 25 |  | 2135 |  | N/A |  | N/A |  |  |
|  | n5 | N/A |  | 5 |  | N/A |  | 880 |  | 8.5 |  | IMD4 |  |  |
|  | n40 | 2385 |  | 5 |  | 25 |  | 2385 |  | N/A |  | N/A |  |  |
| DC_1A-7A_n28A DC_1A-7C_n28A DC_1A-7A-7A_n28A | 1 | 1935 |  | 5 |  | 25 |  | 2125 |  | N/A |  | N/A |  |  |
|  | n28 | 718 |  | 5 |  | 25 |  | 773 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 10 |  | N/A |  | 2653 |  | 30.0 |  | IMD2 |  |  |
| DC_1A-7A_n40A | 1 | 1970 |  | 5 |  | 25 |  | 2160 |  | N/A |  | N/A |  |  |
| DC_1A-7A-7A_n40A | 7 | N/A |  | 5 |  | N/A |  | 2630 |  | 23 |  | IMD3 |  |  |
|  | n40 | 2390 |  | 5 |  | 25 |  | 2390 |  | N/A |  | N/A |  |  |
|  | 1 | N/A |  | 5 |  | N/A |  | 2120 |  | 16.4 |  | IMD3 |  |  |
|  | 7 | 2530 |  | 5 |  | 25 |  | 2650 |  | N/A |  | N/A |  |  |
|  | n40 | 2310 |  | 5 |  | 25 |  | 2310 |  | N/A |  | N/A |  |  |
| DC_1A_n8A-n77A | 1 | 1955 |  | 5 |  | 25 |  | 2145 |  | N/A |  | N/A |  |  |
| DC_1A_n8A-n77(2A) | n8 | 910 |  | 5 |  | 25 |  | 955 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 3410 |  | 1.5 |  | IMD5 |  |  |
| DC_1A_n8A-n77A | n8 | 910 |  | 5 |  | 25 |  | 955 |  | N/A |  | N/A |  |  |
| DC_1A_n8A-n77(2A) | 1 | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 3960 |  | 8.8 |  | IMD3 |  |  |
| DC_1A_n8A-n77A | 1 | 1955 |  | 5 |  | 25 |  | 2145 |  | N/A |  | N/A |  |  |
| DC_1A_n8A-n77(2A) | n77 | 3410 |  | 10 |  | 50 |  | 3410 |  | N/A |  | N/A |  |  |
|  | n8 | N/A |  | 5 |  | N/A |  | 955 |  | 3.3 |  | IMD5 |  |  |
| DC_1A-8A_n78A | 1 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
| DC_1A_n8A-n77(2A) | 8 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | IMD5 |  |  |
|  | n78 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
| DC_1A-3A_n77A DC_1A-3A_n77(2A) DC_1A-3A_n77(3A) DC_1A-3C_n77A DC_1A-3A_n77C DC_1A-3C_n77(2A) | 1 | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | 3 | N/A |  | 5 |  | N/A |  | 1807.5 |  | 31.5 |  | IMD2 |  |  |
|  | n77 | 3757.5 |  | 10 |  | 50 |  | 3757.5 |  | N/A |  | N/A |  |  |
|  | 1 | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | 3 | N/A |  | 5 |  | N/A |  | 1870 |  | 8.5 |  | IMD4 |  |  |
|  | n77 | 3980 |  | 10 |  | 50 |  | 3980 |  | N/A |  | N/A |  |  |
|  | 1 | N/A |  | 5 |  | N/A |  | 2140 |  | 31.0 |  | IMD2 |  |  |
|  | 3 | 1775 |  | 5 |  | 25 |  | 1870 |  | N/A |  | N/A |  |  |
|  | n77 | 3915 |  | 10 |  | 50 |  | 3915 |  | N/A |  | N/A |  |  |
| DC_1A-3A_n78A DC_1A-3A-3A_n78A DC_1A-3C_n78A DC_1A-3A_n78C DC_1A-3A_n78(2A) DC_1A-3C_n78(2A) DC_1A-3A_n78(A-C) | 1 | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | 3 | N/A |  | 5 |  | N/A |  | 1807.5 |  | 31.2 |  | IMD2 |  |  |
|  | n78 | 3757.5 |  | 10 |  | 50 |  | 3757.5 |  | N/A |  | N/A |  |  |
|  | 1 | N/A |  | 5 |  | N/A |  | 2125 |  | 2.8 |  | IMD5 |  |  |
|  | 3 | 1775 |  | 5 |  | 25 |  | 1870 |  | N/A |  | N/A |  |  |
|  | n78 | 3725 |  | 10 |  | 50 |  | 3725 |  | N/A |  | N/A |  |  |
| DC_1A_n3A-n77A DC_1A_n3A-n77(2A) | 1 | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | n3 | 1750 |  | 5 |  | 25 |  | 1845 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 3700 |  | 28.4 |  | IMD2 |  |  |
|  | 1 | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | n3 | 1770 |  | 5 |  | 25 |  | 1865 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 3360 |  | 11.2 |  | IMD4 |  |  |
|  | 1 | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | n3 | N/A |  | 5 |  | N/A |  | 1807.5 |  | 31.5 |  | IMD2 |  |  |
|  | n77 | 3757.5 |  | 10 |  | 50 |  | 3757.5 |  | N/A |  | N/A |  |  |
|  | 1 | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | n3 | N/A |  | 5 |  | N/A |  | 1870 |  | 8.5 |  | IMD4 |  |  |
|  | n77 | 3980 |  | 10 |  | 50 |  | 3980 |  | N/A |  | N/A |  |  |
| DC_1A_n3A-n78A | 1 | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | n3 | 1750 |  | 5 |  | 25 |  | 1845 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3700 |  | 28.4 |  | IMD2 |  |  |
|  | 1 | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | n3 | N/A |  | 5 |  | N/A |  | 1830 |  | 27.9 |  | IMD2 |  |  |
|  | n78 | 3780 |  | 10 |  | 50 |  | 3780 |  | N/A |  | N/A |  |  |
| DC_1A-3A_n105A | 1 | 1970 |  | 5 |  | 25 |  | 2160 |  | N/A |  | N/A |  |  |
|  | 3 | N/A |  | 5 |  | N/A |  | 1855 |  | 4 |  | IMD5 |  |  |
|  | n105 | 695 |  | 5 |  | 25 |  | 644 |  | N/A |  | N/A |  |  |
|  | 1 | N/A |  | 5 |  | N/A |  | 2160 |  | 5 |  | IMD4 |  |  |
|  | 3 | 1775 |  | 5 |  | 25 |  | 1870 |  | N/A |  | N/A |  |  |
|  | n105 | 695 |  | 5 |  | 25 |  | 644 |  | N/A |  | N/A |  |  |
| DC_1A-5A_n77A DC_1A-5A_n77(2A) DC_1A-5A_n77(3A) | 1 | N/A |  | 5 |  | N/A |  | 2122 |  | 18.1 |  | IMD3 |  |  |
|  | 5 | 829 |  | 5 |  | 25 |  | 874 |  | N/A |  | N/A |  |  |
|  | n77 | 3780 |  | 10 |  | 50 |  | 3780 |  | N/A |  | N/A |  |  |
|  | 1 | 1975 |  | 5 |  | 25 |  | 2165 |  | N/A |  | N/A |  |  |
|  | 5 | N/A |  | 5 |  | N/A |  | 885 |  | 3.1 |  | IMD5 |  |  |
|  | n77 | 3405 |  | 10 |  | 50 |  | 3405 |  | N/A |  | N/A |  |  |
| DC_1A-3A_n77A | 1 | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | 3 | N/A |  | 5 |  | N/A |  | 1807.5 |  | 37.5 |  | IMD21 |  |  |
|  | n77 | 3757.5 |  | 10 |  | 50 |  | 3757.5 |  | N/A |  | N/A |  |  |
|  | 1 | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | 3 | N/A |  | 5 |  | N/A |  | 1870 |  | 20.5 |  | IMD41 |  |  |
|  | n77 | 3980 |  | 10 |  | 50 |  | 3980 |  | N/A |  | N/A |  |  |
|  | 1 | N/A |  | 5 |  | N/A |  | 2140 |  | 37.0 |  | IMD21 |  |  |
|  | 3 | 1775 |  | 5 |  | 25 |  | 1870 |  | N/A |  | N/A |  |  |
|  | n77 | 3915 |  | 10 |  | 50 |  | 3915 |  | N/A |  | N/A |  |  |
| DC_1A-5A_n78A DC_1A-5A_n78C DC_1A-5A_n78(A-C) | 1 | N/A |  | 5 |  | N/A |  | 2122 |  | 18.1 |  | IMD3 |  |  |
|  | 5 | 829 |  | 5 |  | 25 |  | 874 |  | N/A |  | N/A |  |  |
|  | n78 | 3780 |  | 10 |  | 50 |  | 3780 |  | N/A |  | N/A |  |  |
|  | 1 | 1975 |  | 5 |  | 25 |  | 2165 |  | N/A |  | N/A |  |  |
|  | 5 | N/A |  | 5 |  | N/A |  | 885 |  | 3.1 |  | IMD5 |  |  |
|  | n78 | 3405 |  | 10 |  | 50 |  | 3405 |  | N/A |  | N/A |  |  |
| DC_1A_n5A-n78A | 1 | 1932 |  | 5 |  | 25 |  | 2122 |  | N/A |  | N/A |  |  |
|  | n5 | 829 |  | 5 |  | 25 |  | 874 |  | N/A |  | N/A |  |  |
|  | n78 | 3583 |  | 10 |  | 50 |  | 3583 |  | 18.1 |  | IMD3 |  |  |
|  | 1 | 1975 |  | 5 |  | 25 |  | 2165 |  | N/A |  | N/A |  |  |
|  | n5 | 840 |  | 5 |  | 25 |  | 885 |  | 3.1 |  | IMD5 |  |  |
|  | n78 | 3405 |  | 10 |  | 50 |  | 3405 |  | N/A |  | N/A |  |  |
| DC_1A-7A_n77A DC_1A-7A_n77(2A) DC_1A-7A_n77(3A) DC_1A-7A-7A_n77A DC_1A-7A-7A_n77(2A) DC_1A-7A-7A_n77(3A) | 1 | 1977.5 |  | 5 |  | 25 |  | 2167.5 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 5 |  | N/A |  | 2627.5 |  | 9.1 |  | IMD44 |  |  |
|  | n77 | 3305 |  | 10 |  | 50 |  | 3305 |  | N/A |  | N/A |  |  |
|  | 1 | N/A |  | 5 |  | N/A |  | 2140 |  | 8.7 |  | IMD4 |  |  |
|  | 7 | 2510 |  | 10 |  | 50 |  | 2630 |  | N/A |  | N/A |  |  |
|  | n77 | 3580 |  | 10 |  | 50 |  | 3580 |  | N/A |  | N/A |  |  |
| DC_1A-7A_n78A DC_1A-7C_n78A DC_1A-7A_n78(2A) DC_1A-7C_n78(2A) DC_1A-7A_n78C DC_1A-7A_n78(A-C) DC_1A-1A-7A_n78A DC_1A-7A-7A_n78C DC_1A-7A-7A_n78(A-C) | 1 | 1977.5 |  | 5 |  | 25 |  | 2167.5 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 5 |  | N/A |  | 2627.5 |  | 9.1 |  | IMD4 |  |  |
|  | n78 | 3305 |  | 10 |  | 50 |  | 3305 |  | N/A |  | N/A |  |  |
|  | 1 | N/A |  | 5 |  | N/A |  | 2140 |  | 8.7 |  | IMD4 |  |  |
|  | 7 | 2510 |  | 10 |  | 50 |  | 2630 |  | N/A |  | N/A |  |  |
|  | n78 | 3580 |  | 10 |  | 50 |  | 3580 |  | N/A |  | N/A |  |  |
| DC_1A_n7A-n78A DC_1A_n7B-n78A DC_1A_n7A-n78(2A) | 1 | 1977.5 |  | 5 |  | 25 |  | 2167.5 |  | N/A |  | N/A |  |  |
|  | n7 | N/A |  | 5 |  | N/A |  | 2627.5 |  | 9.1 |  | IMD4 |  |  |
|  | n78 | 3305 |  | 10 |  | 50 |  | 3305 |  | N/A |  | N/A |  |  |
|  | 1 | 1970 |  | 5 |  | 25 |  | 2160 |  | N/A |  | N/A |  |  |
|  | n7 | 2520 |  | 5 |  | 25 |  | 2640 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3390 |  | 10.1 |  | IMD4 |  |  |
| DC_1A-3A_n79A | 1 | N/A |  | 5 |  | N/A |  | 2140 |  | 3.6 |  | IMD5 |  |  |
|  | 3 | 1750 |  | 5 |  | 25 |  | 1845 |  | N/A |  | N/A |  |  |
|  | n79 | 4860 |  | 40 |  | 216 |  | 4860 |  | N/A |  | N/A |  |  |
| DC_1A-5A_n28A | 1 | N/A |  | 5 |  | N/A |  | 2123 |  | 4 |  | IMD5 |  |  |
|  | 5 | 829 |  | 5 |  | 25 |  | 874 |  | N/A |  | N/A |  |  |
|  | n28 | 738 |  | 5 |  | 25 |  | 793 |  | N/A |  | N/A |  |  |
|  | 1 | 1965 |  | 5 |  | 25 |  | 2155 |  | N/A |  | N/A |  |  |
|  | 5 | N/A |  | 5 |  | N/A |  | 875 |  | 4.6 |  | IMD5 |  |  |
|  | n28 | 710 |  | 5 |  | 25 |  | 765 |  | N/A |  | N/A |  |  |
| DC_1A-5A_n40A | 1 | N/A |  | 5 |  | N/A |  | 2144 |  | 4.0 |  | IMD5 |  |  |
|  | 5 | 832 |  | 5 |  | 25 |  | 877 |  | N/A |  | N/A |  |  |
|  | n40 | 2320 |  | 5 |  | 25 |  | 2320 |  | N/A |  | N/A |  |  |
|  | 1 | 1945 |  | 5 |  | 25 |  | 2135 |  | N/A |  | N/A |  |  |
|  | 5 | N/A |  | 5 |  | N/A |  | 880 |  | 8.0 |  | IMD4 |  |  |
|  | n40 | 2385 |  | 5 |  | 25 |  | 2385 |  | N/A |  | N/A |  |  |
| DC_1A-5A_n79A | 1 | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | 5 | N/A |  | 5 |  | N/A |  | 882.5 |  | 18.3 |  | IMD3 |  |  |
|  | n79 | 4782.5 |  | 40 |  | 216 |  | 4782.5 |  | N/A |  | N/A |  |  |
|  | 1 | 1930 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
|  | 5 | N/A |  | 5 |  | N/A |  | 882.5 |  | 8.9 |  | IMD4 |  |  |
|  | n79 | 4907.5 |  | 40 |  | 216 |  | 4907.5 |  | N/A |  | N/A |  |  |
|  | 1 | N/A |  | 5 |  | N/A |  | 2140 |  | 8.1 |  | IMD4 |  |  |
|  | 5 | 837.5 |  | 5 |  | 25 |  | 882.5 |  | N/A |  | N/A |  |  |
|  | n79 | 4652.5 |  | 40 |  | 216 |  | 4652.5 |  | N/A |  | N/A |  |  |
| DC_1A-7A_n105A | 1 | 1975 |  | 5 |  | 25 |  | 2165 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 5 |  | N/A |  | 2673 |  | 30 |  | IMD2 |  |  |
|  | n105 | 698 |  | 5 |  | 25 |  | 647 |  | N/A |  | N/A |  |  |
| DC_1A-8A_n7A | 1 | 1977.5 |  | 5 |  | 25 |  | 2167.5 |  | N/A |  | N/A |  |  |
|  | n7 | 2502.5 |  | 5 |  | 25 |  | 2622.5 |  | N/A |  | N/A |  |  |
|  | 8 | N/A |  | 5 |  | N/A |  | 927.5 |  | 1.0 |  | IMD5 |  |  |
| DC_1A-8A_n28A | 1 | 1970 |  | 5 |  | 25 |  | 2160 |  | N/A |  | N/A |  |  |
|  | n28 | 730 |  | 5 |  | 25 |  | 785 |  | N/A |  | N/A |  |  |
|  | 8 | N/A |  | 5 |  | N/A |  | 950 |  | 3.3 |  | IMD5 |  |  |
| DC_1A-8A_n40A | 1 | 1930 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
|  | 8 | N/A |  | 5 |  | N/A |  | 930 |  | 8.0 |  | IMD4 |  |  |
|  | n40 | 2395 |  | 5 |  | 25 |  | 2395 |  | N/A |  | N/A |  |  |
|  | 1 | N/A |  | 5 |  | N/A |  | 2135 |  | 5.3 |  | IMD5 |  |  |
|  | 8 | 885 |  | 5 |  | 25 |  | 930 |  | N/A |  | N/A |  |  |
|  | n40 | 2395 |  | 5 |  | 25 |  | 2395 |  | N/A |  | N/A |  |  |
| DC_1A-8A_n77A | 1 | 1955 |  | 5 |  | 25 |  | 2145 |  | N/A |  | N/A |  |  |
| DC_1A-8A_n77(2A) DC_1A-8A_n77(3A) DC_1A-8B_n77A DC_1A-8B_n77(2A) | n77 | 3410 |  | 10 |  | 50 |  | 3410 |  | N/A |  | N/A |  |  |
|  | 8 | N/A |  | 5 |  | N/A |  | 955 |  | 3.3 |  | IMD5 |  |  |
| DC_1A-8A_n77A | 8 | 910 |  | 5 |  | 25 |  | 955 |  | N/A |  | N/A |  |  |
| DC_1A-8A_n77(2A) DC_1A-8A_n77(3A) | n77 | 3960 |  | 10 |  | 50 |  | 3960 |  | N/A |  | N/A |  |  |
| DC_1A-8B_n77A DC_1A-8B_n77(2A) | 1 | N/A |  | 5 |  | N/A |  | 2140 |  | 14.4 |  | IMD3 |  |  |
| DC_1A-8A_n79A | 1 | 1935 |  | 5 |  | 25 |  | 2125 |  | N/A |  | N/A |  |  |
|  | n79 | 4815 |  | 40 |  | 216 |  | 4815 |  | N/A |  | N/A |  |  |
|  | 8 | N/A |  | 5 |  | N/A |  | 945 |  | 15.8 |  | IMD3 |  |  |
| DC_1A-8A_n79A | 8 | 900 |  | 5 |  | 25 |  | 945 |  | N/A |  | N/A |  |  |
|  | n79 | 4845 |  | 40 |  | 216 |  | 4845 |  | N/A |  | N/A |  |  |
|  | 1 | N/A |  | 5 |  | N/A |  | 2145 |  | 8.2 |  | IMD4 |  |  |
| DC_1A_n8A-n40A | 1 | 1930 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
|  | n8 | N/A |  | 5 |  | N/A |  | 930 |  | 8.0 |  | IMD4 |  |  |
|  | n40 | 2395 |  | 5 |  | 25 |  | 2395 |  | N/A |  | N/A |  |  |
| DC_1A_n8A-n78A | 1 | 1945 |  | 5 |  | 25 |  | 2135 |  | N/A |  | N/A |  |  |
|  | n8 | 900 |  | 5 |  | 25 |  | 945 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3745 |  | 14.9 |  | IMD3 |  |  |
|  | 1 | 1940 |  | 5 |  | 25 |  | 2130 |  | N/A |  | N/A |  |  |
|  | n8 | N/A |  | 5 |  | N/A |  | 940 |  | 3.3 |  | IMD5 |  |  |
|  | n78 | 3380 |  | 10 |  | 50 |  | 3330 |  | N/A |  | N/A |  |  |
| DC_1A-11A_n3A | 1 | 1960 |  | 5 |  | 25 |  | 2150 |  | N/A |  | N/A |  |  |
|  | n3 | 1720 |  | 5 |  | 25 |  | 1815 |  | N/A |  | N/A |  |  |
|  | 11 | N/A |  | 5 |  | N/A |  | 1480 |  | 15.2 |  | IMD3 |  |  |
| DC_1A-11A_n28A | 11 | 1440 |  | 5 |  | 25 |  | 1488 |  | N/A |  | N/A |  |  |
|  | n28 | 710 |  | 5 |  | 25 |  | 765 |  | N/A |  | N/A |  |  |
|  | 1 | N/A |  | 5 |  | N/A |  | 2150 |  | 28.3 |  | IMD21 |  |  |
| DC_1A-11A_n41A | 11 | 1442 |  | 5 |  | 25 |  | 1490 |  | N/A |  | N/A |  |  |
|  | n41 | 2520 |  | 10 |  | 50 |  | 2520 |  | N/A |  | N/A |  |  |
|  | 1 | N/A |  | 5 |  | N/A |  | 2156 |  | 10.2 |  | IMD4 |  |  |
|  | 1 | 1940 |  | 5 |  | 25 |  | 2130 |  | N/A |  | N/A |  |  |
|  | n41 | 2685 |  | 10 |  | 50 |  | 2685 |  | N/A |  | N/A |  |  |
|  | 11 | N/A |  | 5 |  | N/A |  | 1490 |  | 10.6 |  | IMD4 |  |  |
| DC_1A-11A_n77A DC_1A-11A_n77(2A) DC_1A-11A_n77(3A) | 1 | 1955 |  | 5 |  | 25 |  | 2145 |  | N/A |  | N/A |  |  |
|  | 11 | N/A |  | 5 |  | N/A |  | 1486 |  | 31.4 |  | IMD2 |  |  |
|  | n77 | 3441 |  | 10 |  | 50 |  | 3441 |  | N/A |  | N/A |  |  |
|  | 1 | N/A |  | 5 |  | N/A |  | 2140 |  | 30.8 |  | IMD2 |  |  |
|  | 11 | 1438 |  | 5 |  | 25 |  | 1486 |  | N/A |  | N/A |  |  |
|  | n77 | 3578 |  | 10 |  | 50 |  | 3578 |  | N/A |  | N/A |  |  |
| DC_1A-11A_n78A DC_1A-11A_n78(2A) | 1 | 1955 |  | 5 |  | 25 |  | 2145 |  | N/A |  | N/A |  |  |
|  | 11 | N/A |  | 5 |  | N/A |  | 1486 |  | 31.4 |  | IMD2 |  |  |
|  | n78 | 3441 |  | 10 |  | 50 |  | 3441 |  | N/A |  | N/A |  |  |
|  | 1 | N/A |  | 5 |  | N/A |  | 2140 |  | 30.8 |  | IMD2 |  |  |
|  | 11 | 1438 |  | 5 |  | 25 |  | 1486 |  | N/A |  | N/A |  |  |
|  | n78 | 3578 |  | 10 |  | 50 |  | 3578 |  | N/A |  | N/A |  |  |
| DC_1A-11A_n79A | 1 | 1970 |  | 5 |  | 25 |  | 2160 |  | N/A |  | N/A |  |  |
|  | 11 | N/A |  | 5 |  | N/A |  | 1483 |  | 10.2 |  | IMD4 |  |  |
|  | n79 | 4427 |  | 40 |  | 216 |  | 4427 |  | N/A |  | N/A |  |  |
|  | 1 | N/A |  | 5 |  | N/A |  | 2118 |  | 15.6 |  | IMD3 |  |  |
|  | 11 | 1431 |  | 5 |  | 25 |  | 1479 |  | N/A |  | N/A |  |  |
|  | n79 | 4980 |  | 40 |  | 216 |  | 4980 |  | N/A |  | N/A |  |  |
| DC_1A-18A_n77A DC_1A-18A_n77(2A) | 1 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
|  | 18 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | IMD5 |  |  |
|  | n77 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
|  | 1 | N/A |  | 5 |  | N/A |  | 2120 |  | 16.4 |  | IMD3 |  |  |
|  | 18 | 825 |  | 5 |  | 25 |  | 870 |  | N/A |  | N/A |  |  |
|  | n77 | 3770 |  | 10 |  | 50 |  | 3770 |  | N/A |  | N/A |  |  |
| DC_1A-18A_n78A DC_1A-18A_n78(2A) | 1 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
|  | 18 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | IMD5 |  |  |
|  | n78 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
|  | 1 | N/A |  | 5 |  | N/A |  | 2120 |  | 16.4 |  | IMD3 |  |  |
|  | 18 | 819 |  | 5 |  | 25 |  | 864 |  | N/A |  | N/A |  |  |
|  | n78 | 3758 |  | 10 |  | 50 |  | 3758 |  | N/A |  | N/A |  |  |
| DC_1A-18A_n79A | 1 | 1935 |  | 5 |  | 25 |  | 2125 |  | N/A |  | N/A |  |  |
|  | 18 | N/A |  | 5 |  | N/A |  | 867.5 |  | 18.3 |  | IMD3 |  |  |
|  | n79 | 4737.5 |  | 40 |  | 216 |  | 4737.5 |  | N/A |  | N/A |  |  |
|  | 1 | 1930 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
|  | 18 | N/A |  | 5 |  | N/A |  | 865 |  | 8.9 |  | IMD4 |  |  |
|  | n79 | 4925 |  | 40 |  | 216 |  | 4925 |  | N/A |  | N/A |  |  |
|  | 1 | N/A |  | 5 |  | N/A |  | 2125 |  | 8.1 |  | IMD4 |  |  |
|  | 18 | 822.5 |  | 5 |  | 25 |  | 867.5 |  | N/A |  | N/A |  |  |
|  | n79 | 4592.5 |  | 40 |  | 216 |  | 4592.5 |  | N/A |  | N/A |  |  |
| DC_1A-19A_n77A DC_1A-19A_n78A | 1 | N/A |  | 5 |  | N/A |  | 2130 |  | 17.8 |  | IMD3 |  |  |
|  | 19 | 832.5 |  | 5 |  | 25 |  | 877.5 |  | N/A |  | N/A |  |  |
|  | n77, n78 | 3795 |  | 10 |  | 50 |  | 3795 |  | N/A |  | N/A |  |  |
|  | 1 | 1940 |  | 5 |  | 25 |  | 2130 |  | N/A |  | N/A |  |  |
|  | 19 | N/A |  | 5 |  | N/A |  | 880 |  | 5.1 |  | IMD5 |  |  |
|  | n77, n78 | 3350 |  | 10 |  | 50 |  | 3350 |  | N/A |  | N/A |  |  |
| DC_1A-19A_n79A | 1 | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | 19 | N/A |  | 5 |  | N/A |  | 882.5 |  | 18.3 |  | IMD3 |  |  |
|  | n79 | 4782.5 |  | 40 |  | 216 |  | 4782.5 |  | N/A |  | N/A |  |  |
|  | 1 | N/A |  | 5 |  | N/A |  | 2140 |  | 8.1 |  | IMD4 |  |  |
|  | 19 | 837.5 |  | 5 |  | 25 |  | 882.5 |  | N/A |  | N/A |  |  |
|  | n79 | 4652.5 |  | 40 |  | 216 |  | 4652.5 |  | N/A |  | N/A |  |  |
| DC_1A-20A_n1A | n1 | 1930 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
|  | 20 | 850 |  | 5 |  | 25 |  | 809 |  | N/A |  | N/A |  |  |
|  | 1 | N/A |  | 5 |  | N/A |  | 2160 |  | 6 |  | IMD4 |  |  |
| DC_1A_n28A-n41A | 1 | 1935 |  | 5 |  | 25 |  | 2125 |  | N/A |  | N/A |  |  |
|  | n28 | 718 |  | 5 |  | 25 |  | 773 |  | N/A |  | N/A |  |  |
|  | n41 | N/A |  | 10 |  | N/A |  | 2653 |  | 30.1 |  | IMD2 |  |  |
|  | 1 | 1923 |  | 5 |  | 25 |  | 2113 |  | N/A |  | N/A |  |  |
|  | n28 | N/A |  | 5 |  | N/A |  | 762 |  | 29.3 |  | IMD2 |  |  |
|  | n41 | 2685 |  | 10 |  | 50 |  | 2685 |  | N/A |  | N/A |  |  |
|  | 1 | 1935 |  | 5 |  | 25 |  | 2125 |  | N/A |  | N/A |  |  |
|  | n28 | N/A |  | 10 |  | N/A |  | 785 |  | 4.5 |  | IMD5 |  |  |
|  | n41 | 2510 |  | 10 |  | 50 |  | 2510 |  | N/A |  | N/A |  |  |
| DC_1A-20A_n7A | 1 | 1940 |  | 5 |  | 25 |  | 2130 |  | N/A |  | N/A |  |  |
|  | 20 | N/A |  | 10 |  | N/A |  | 800 |  | 4.5 |  | IMD5 |  |  |
|  | n7 | 2510 |  | 10 |  | 50 |  | 2630 |  | N/A |  | N/A |  |  |
| DC_1A-20A_n8A | 1 | 1925 |  | 5 |  | 25 |  | 2115 |  | N/A |  | N/A |  |  |
|  | 20 | N/A |  | 5 |  | N/A |  | 805 |  | 11.5 |  | IMD4 |  |  |
|  | n8 | 910 |  | 5 |  | 25 |  | 955 |  | N/A |  | N/A |  |  |
| DC_1A-20A_n38A | 1 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
|  | 20 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | IMD5 |  |  |
|  | n38 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
| DC_1A-20A_n78A DC_1A-1A-20A_n78A | 1 | N/A |  | 5 |  | N/A |  | 2120 |  | 20.3 |  | IMD3 |  |  |
| DC_1A-20A_n78(2A) | 20 | 835 |  | 5 |  | 25 |  | 794 |  | N/A |  | N/A |  |  |
| DC_1A-20A_n78C | n78 | 3790 |  | 10 |  | 50 |  | 3790 |  | N/A |  | N/A |  |  |
|  | 1 | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | 20 | N/A |  | 5 |  | N/A |  | 810 |  | 3.0 |  | IMD5 |  |  |
|  | n78 | 3330 |  | 10 |  | 50 |  | 3330 |  | N/A |  | N/A |  |  |
| DC_1A-21A_n28A10 | 1 | N/A |  | 5 |  | N/A |  | 2165.3 |  | 16.1 |  | IMD3 |  |  |
|  | 21 | 1450.4 |  | 5 |  | 25 |  | 1498.4 |  | N/A |  | N/A |  |  |
|  | n28 | 735.5 |  | 5 |  | 25 |  | 790.5 |  | N/A |  | N/A |  |  |
| DC_1A-21A_n77A DC_1A-21A_n78A | 1 | N/A |  | 5 |  | N/A |  | 2154.6 |  | 30.6 |  | IMD2 |  |  |
|  | 21 | 1450.4 |  | 5 |  | 25 |  | 1498.4 |  | N/A |  | N/A |  |  |
|  | n77, n78 | 3605 |  | 10 |  | 50 |  | 3605 |  | N/A |  | N/A |  |  |
|  | 1 | N/A |  | 5 |  | N/A |  | 2154.6 |  | 3.6 |  |  | IMD5 |  |
|  | 21 | 1450.4 |  | 5 |  | 25 |  | 1498.4 |  | N/A |  |  | N/A |  |
|  | n77, n78 | 3647 |  | 10 |  | 50 |  | 3647 |  | N/A |  |  | N/A |  |
|  | 1 | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | 21 | N/A |  | 5 |  | N/A |  | 1500 |  | 31.5 |  | IMD2 |  |  |
|  | n77, n78 | 3450 |  | 10 |  | 50 |  | 3450 |  | N/A |  | N/A |  |  |
|  | 1 | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | 21 | N/A |  | 5 |  | N/A |  | 1500 |  | 2.9 |  | IMD5 |  |  |
|  | n77, n78 | 3675 |  | 10 |  | 50 |  | 3675 |  | N/A |  | N/A |  |  |
| DC_1A-21A_n79A | 1 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
|  | 21 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | IMD4 |  |  |
|  | n79 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
| DC_1A-26A_n78A | 1 | N/A |  | 5 |  | N/A |  | 2122 |  | 18.1 |  | IMD3 |  |  |
|  | 26 | 829 |  | 5 |  | 25 |  | 874 |  | N/A |  | N/A |  |  |
|  | n78 | 3780 |  | 10 |  | 50 |  | 3780 |  | N/A |  | N/A |  |  |
|  | 1 | 1975 |  | 5 |  | 25 |  | 2165 |  | N/A |  | N/A |  |  |
|  | 26 | N/A |  | 5 |  | N/A |  | 885 |  | 3.1 |  | IMD5 |  |  |
|  | n78 | 3405 |  | 10 |  | 50 |  | 3405 |  | N/A |  | N/A |  |  |
| DC_1A_n26A-n78A | n1 | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | n26 | 830 |  | 5 |  | 25 |  | 875 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3610 |  | 15.7 |  | IMD3 |  |  |
|  | 1 | 1975 |  | 5 |  | 25 |  | 2165 |  | N/A |  | N/A |  |  |
|  | n26 | N/A |  | 5 |  | N/A |  | 885 |  | 3.1 |  | IMD5 |  |  |
|  | n78 | 3405 |  | 10 |  | 50 |  | 3405 |  | N/A |  | N/A |  |  |
| DC_1A-28A_n3A | 1 | N/A |  | 5 |  | N/A |  | 2139 |  | 11.0 |  | IMD4 |  |  |
|  | 28 | 710.5 |  | 5 |  | 25 |  | 765.5 |  | N/A |  | N/A |  |  |
|  | n3 | 1780 |  | 5 |  | 25 |  | 1875 |  | N/A |  | N/A |  |  |
| DC_1A-28A_n7A DC_1A-1A-28A_n7A DC_1A-28A_n7B DC_1A-1A-28A_n7B | 1 | 1935 |  | 5 |  | 25 |  | 2125 |  | N/A |  | N/A |  |  |
|  | 28 | N/A |  | 10 |  | N/A |  | 785 |  | 4.5 |  | IMD5 |  |  |
|  | n7 | 2510 |  | 10 |  | 50 |  | 2630 |  | N/A |  | N/A |  |  |
| DC_1A-28A_n40A | 1 | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
| DC_1A-28A_n38A | 1 | 1975 |  | 5 |  | 25 |  | 2165 |  | N/A |  | N/A |  |  |
|  | 28 | N/A |  | 5 |  | N/A |  | 765 |  | 4.5 |  | IMD5 |  |  |
|  | n38 | 2580 |  | 5 |  | 25 |  | 2580 |  | N/A |  | N/A |  |  |
|  | 28 | N/A |  | 5 |  | N/A |  | 780 |  | 8.9 |  | IMD4 |  |  |
|  | n40 | 2340 |  | 5 |  | 25 |  | 2340 |  | N/A |  | N/A |  |  |
| DC_1A-28A_n77A | 1 | N/A |  | 5 |  | N/A |  | 2150 |  | 15.7 |  | IMD3 |  |  |
|  | 28 | 740 |  | 5 |  | 25 |  | 795 |  | N/A |  | N/A |  |  |
|  | n77 | 3630 |  | 10 |  | 50 |  | 3630 |  | N/A |  | N/A |  |  |
|  | 1 | 1970 |  | 5 |  | 25 |  | 2160 |  | N/A |  | N/A |  |  |
|  | 28 | N/A |  | 5 |  | N/A |  | 794 |  | 4.2 |  | IMD5 |  |  |
|  | n77 | 3352 |  | 10 |  | 50 |  | 3352 |  | N/A |  | N/A |  |  |
| DC_1A-28A_n78A | 1 | N/A |  | 5 |  | N/A |  | 2150 |  | 15.7 |  | IMD3 |  |  |
|  | 28 | 740 |  | 5 |  | 25 |  | 795 |  | N/A |  | N/A |  |  |
|  | n78 | 3630 |  | 10 |  | 50 |  | 3630 |  | N/A |  | N/A |  |  |
|  | 1 | 1970 |  | 5 |  | 25 |  | 2160 |  | N/A |  | N/A |  |  |
|  | 28 | N/A |  | 5 |  | N/A |  | 794 |  | 4.2 |  | IMD5 |  |  |
|  | n78 | 3352 |  | 10 |  | 50 |  | 3352 |  | N/A |  | N/A |  |  |
| DC_1A-28A_n79A | 1 | 1930 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
|  | 28 | N/A |  | 5 |  | N/A |  | 788 |  | 15.2 |  | IMD3 |  |  |
|  | n79 | 4648 |  | 40 |  | 216 |  | 4648 |  | N/A |  | N/A |  |  |
|  | 1 | 1925 |  | 5 |  | 25 |  | 2115 |  | N/A |  | N/A |  |  |
|  | 28 | N/A |  | 5 |  | N/A |  | 795 |  | 10.0 |  | IMD4 |  |  |
|  | n79 | 4980 |  | 40 |  | 216 |  | 4980 |  | N/A |  | N/A |  |  |
|  | 1 | N/A |  | 5 |  | N/A |  | 2167.5 |  | 1.2 |  | IMD4 |  |  |
|  | 28 | 745.5 |  | 5 |  | 25 |  | 800.5 |  | N/A |  | N/A |  |  |
|  | n79 | 4420 |  | 40 |  | 216 |  | 4420 |  | N/A |  | N/A |  |  |
|  | 1 | N/A |  | 5 |  | N/A |  | 2125 |  | 4.5 |  | IMD5 |  |  |
|  | 28 | 718 |  | 5 |  | 25 |  | 773 |  | N/A |  | N/A |  |  |
|  | n79 | 4807 |  | 40 |  | 216 |  | 4807 |  | N/A |  | N/A |  |  |
| DC_1A_n28A-n40A | 1 | 1930 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
|  | n28 | 743 |  | 5 |  | 25 |  | 798 |  | N/A |  | N/A |  |  |
|  | n40 | N/A |  | 5 |  | N/A |  | 2374 |  | 10.1 |  | IMD4 |  |  |
|  | 1 | 1930 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
|  | n28 | N/A |  | 5 |  | N/A |  | 768 |  | 8.6 |  | IMD4 |  |  |
|  | n40 | 2314 |  | 5 |  | 25 |  | 2314 |  | N/A |  | N/A |  |  |
| DC_1A_n28A-n77A DC_1A_n28A-n78A | 1 | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | n28 | 733 |  | 5 |  | 25 |  | 788 |  | N/A |  | N/A |  |  |
|  | n77/n78 | N/A |  | 10 |  | N/A |  | 3416 |  | 15.7 |  | IMD3 |  |  |
|  | 1 | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | n77/n78 | 3320 |  | 10 |  | 50 |  | 3320 |  | N/A |  | N/A |  |  |
|  | n28 | N/A |  | 5 |  | N/A |  | 790 |  | 4.2 |  | IMD5 |  |  |
| DC_1A_n28A-n79A | 1 | 1930 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
|  | n28 | N/A |  | 5 |  | N/A |  | 788 |  | 15.2 |  | IMD39 |  |  |
|  | n79 | 4648 |  | 40 |  | 216 |  | 4648 |  | N/A |  | N/A |  |  |
|  | 1 | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | n28 | 730 |  | 5 |  | 25 |  | 785 |  | N/A |  | N/A |  |  |
|  | n79 | N/A |  | 40 |  | N/A |  | 4630 |  | 14.9 |  | IMD34 |  |  |
| DC_1A-32A_n3A | n3 | 1720 |  | 5 |  | 25 |  | 1815 |  | N/A |  | N/A |  |  |
|  | 32 | N/A |  | 5 |  | N/A |  | 1480 |  | 15.2 |  | IMD34 |  |  |
|  | 1 | 1960 |  | 5 |  | 25 |  | 2150 |  | N/A |  | N/A |  |  |
| DC_1A-32A_n78A DC_1A-32A_n78C DC_1A-32A_n78(2A) | 1 | 1930 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
|  | 32 | N/A |  | 5 |  | N/A |  | 1470 |  | 31.8 |  | IMD2 |  |  |
|  | n78 | 3400 |  | 10 |  | 50 |  | 3400 |  | N/A |  | N/A |  |  |
|  | 1 | 1930 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
|  | 32 | N/A |  | 5 |  | N/A |  | 1470 |  | 0 |  | IMD5 |  |  |
|  | n78 | 3630 |  | 10 |  | 50 |  | 3630 |  | N/A |  | N/A |  |  |
| DC_1A-38A_n78A DC_1A-38A_n78(2A) | 1 | 1970 |  | 5 |  | 25 |  | 2160 |  | N/A |  | N/A |  |  |
|  | 38 | N/A |  | 5 |  | N/A |  | 2590 |  | 12.7 |  | IMD4 |  |  |
|  | n78 | 3320 |  | 10 |  | 50 |  | 3320 |  | N/A |  | N/A |  |  |
| DC_1A_n38A-n78A | 1 | 1970 |  | 5 |  | 25 |  | 2160 |  | N/A |  | N/A |  |  |
|  | n38 | N/A |  | 10 |  | N/A |  | 2590 |  | 12.7 |  | IMD4 |  |  |
|  | n78 | 3320 |  | 10 |  | 50 |  | 3320 |  | N/A |  | N/A |  |  |
| DC_1A_n40A-n77A | 1 | 1930 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
| DC_1A_n40A-n77(2A) | n40 | 2340 |  | 5 |  | 25 |  | 2340 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 3450 |  | 9.8 |  | IMD4 |  |  |
|  | 1 | 1960 |  | 5 |  | 25 |  | 2150 |  | N/A |  | N/A |  |  |
|  | n40 | N/A |  | 5 |  | N/A |  | 2360 |  | 10.6 |  | IMD4 |  |  |
|  | n77 | 3520 |  | 10 |  | 50 |  | 3520 |  | N/A |  | N/A |  |  |
| DC_1A-40A_n78A DC_1A-40C_n78A | 1 | 1930 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
|  | 40 | N/A |  | 5 |  | N/A |  | 2340 |  | 10.6 |  | IMD4 |  |  |
|  | n78 | 3450 |  | 10 |  | 50 |  | 3450 |  | N/A |  | N/A |  |  |
|  | 1 | N/A |  | 5 |  | N/A |  | 2140 |  | 9.1 |  | IMD4 |  |  |
|  | 40 | 2360 |  | 5 |  | 25 |  | 2360 |  | N/A |  | N/A |  |  |
|  | n78 | 3430 |  | 10 |  | 50 |  | 3430 |  | N/A |  | N/A |  |  |
| DC_1A_n40A-n78A DC_1A_n40A-n78(2A) | 1 | 1930 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
| DC_1A_n40A-n78C | n40 | 2340 |  | 5 |  | 25 |  | 2340 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3450 |  | 9.8 |  | IMD4 |  |  |
|  | 1 | 1960 |  | 5 |  | 25 |  | 2150 |  | N/A |  | N/A |  |  |
|  | n40 | N/A |  | 5 |  | N/A |  | 2360 |  | 10.6 |  | IMD4 |  |  |
|  | n78 | 3520 |  | 10 |  | 50 |  | 3520 |  | N/A |  | N/A |  |  |
| DC_1_n40-n105 | 1 | 1977 |  | 5 |  | 25 |  | 2167 |  | N/A |  | N/A |  |  |
|  | n40 | 2305 |  | 10 |  | 50 |  | 2305 |  | N/A |  | N/A |  |  |
|  | n105 | 700 |  | 5 |  | 25 |  | 649 |  | 1 |  | IMD4 |  |  |
| DC_1A-41A_n3A DC_1A-41C_n3A | 1 | 1977.5 |  | 5 |  | 25 |  | 2167.5 |  | N/A |  | N/A |  |  |
|  | 41 | N/A |  | 5 |  | N/A |  | 2507.5 |  | 5.0 |  | IMD5 |  |  |
|  | n3 | 1712.5 |  | 5 |  | 25 |  | 1807.5 |  | N/A |  | N/A |  |  |
| DC_1A-41A_n28A | 1 | 1935 |  | 5 |  | 25 |  | 2125 |  | N/A |  | N/A |  |  |
|  | 41 | N/A |  | 10 |  | N/A |  | 2653 |  | 30 |  | IMD2 |  |  |
|  | n28 | 718 |  | 5 |  | 25 |  | 773 |  | N/A |  | N/A |  |  |
| DC_1A-41A_n77A DC_1A-41C_n77A DC_1A-41A_n77(2A) DC_1A-41C_n77(2A) | 1 | 1970 |  | 5 |  | 25 |  | 2160 |  | N/A |  | N/A |  |  |
|  | 41 | 2510 |  | 5 |  | 25 |  | 2510 |  | 11.0 |  | IMD4 |  |  |
|  | n77 | 3400 |  | 10 |  | 50 |  | 3400 |  | N/A |  | N/A |  |  |
|  | 1 | N/A |  | 5 |  | N/A |  | 2140 |  | 9.3 |  | IMD4 |  |  |
|  | 41 | 2640 |  | 5 |  | 25 |  | 2640 |  | N/A |  | N/A |  |  |
|  | n77 | 3710 |  | 10 |  | 50 |  | 3710 |  | N/A |  | N/A |  |  |
|  | 1 | 1930 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
|  | 41 | 2510 |  | 5 |  | 25 |  | 2510 |  | 3.6 |  | IMD5 |  |  |
|  | n77 | 4150 |  | 10 |  | 50 |  | 4150 |  | N/A |  | N/A |  |  |
| DC_1A_n41A-n77A DC_1A_n41A-n77(2A) | 1 | 1975 |  | 5 |  | 25 |  | 2165 |  | N/A |  | N/A |  |  |
|  | n41 | N/A |  | 10 |  | N/A |  | 2515 |  | 11.5 |  | IMD44 |  |  |
|  | n77 | 3410 |  | 10 |  | 50 |  | 3410 |  | N/A |  | N/A |  |  |
|  | 1 | 1970 |  | 5 |  | 25 |  | 2160 |  | N/A |  | N/A |  |  |
|  | n41 | 2650 |  | 10 |  | 25 |  | 2650 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 3330 |  | 19.6 |  | IMD34,9 |  |  |
| DC_1A-41A_n78A DC_1A-41C_n78A DC_1A-41A_n78(2A) DC_1A-41C_n78(2A) | 1 | N/A |  | 5 |  | N/A |  | 2140 |  | 9.3 |  | IMD4 |  |  |
|  | 41 | 2640 |  | 5 |  | 25 |  | 2640 |  | N/A |  | N/A |  |  |
|  | n78 | 3710 |  | 10 |  | 50 |  | 3710 |  | N/A |  | N/A |  |  |
|  | 1 | 1975 |  | 5 |  | 25 |  | 2165 |  | N/A |  | N/A |  |  |
|  | 41 | N/A |  | 5 |  | N/A |  | 2515 |  | 12 |  | IMD4 |  |  |
|  | n78 | 3410 |  | 10 |  | 50 |  | 3410 |  | N/A |  | N/A |  |  |
| DC_1A_n41A-n78A DC_1A_n41A-n78(2A) | 1 | 1975 |  | 5 |  | 25 |  | 2165 |  | N/A |  | N/A |  |  |
|  | n41 | N/A |  | 10 |  | N/A |  | 2515 |  | 11.5 |  | IMD44 |  |  |
|  | n78 | 3410 |  | 10 |  | 50 |  | 3410 |  | N/A |  | N/A |  |  |
|  | 1 | 1970 |  | 5 |  | 25 |  | 2160 |  | N/A |  | N/A |  |  |
|  | n41 | 2650 |  | 10 |  | 25 |  | 2650 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3330 |  | 19.6 |  | IMD34,9 |  |  |
| DC_1A-41A_n79A | 1 | 1970 |  | 5 |  | 25 |  | 2160 |  | N/A |  | N/A |  |  |
|  | 41 | N/A |  | 5 |  | N/A |  | 2530 |  | 29.4 |  | IMD2 |  |  |
|  | n79 | 4500 |  | 40 |  | 216 |  | 4500 |  | N/A |  | N/A |  |  |
| DC_1A-42A_n3A | 1 | 1922.5 |  | 5 |  | 25 |  | 2112.5 |  | N/A |  | N/A |  |  |
|  | n3 | 1782.5 |  | 5 |  | 25 |  | 1877.5 |  | N/A |  | N/A |  |  |
|  | 42 | N/A |  | 5 |  | N/A |  | 3425 |  | 13.0 |  | IMD4 |  |  |
| DC_1A-42A_n28A | 1 | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | n28 | 733 |  | 5 |  | 25 |  | 788 |  | N/A |  | N/A |  |  |
|  | 42 | N/A |  | 5 |  | N/A |  | 3416 |  | 15.7 |  | IMD3 |  |  |
| DC_1A-42A_n28A | 42 | 3580 |  | 5 |  | 25 |  | 3580 |  | N/A |  | N/A |  |  |
|  | n28 | 723 |  | 5 |  | 25 |  | 778 |  | N/A |  | N/A |  |  |
|  | 1 | N/A |  | 5 |  | N/A |  | 2134 |  | 15.7 |  | IMD3 |  |  |
| DC_1A-42A_n79A | 1 | 1977.5 |  | 5 |  | 25 |  | 2167.5 |  | N/A |  | N/A |  |  |
|  | n79 | 4420 |  | 40 |  | 216 |  | 4420 |  | N/A |  | N/A |  |  |
|  | 42 | N/A |  | 5 |  | N/A |  | 3490 |  | 4.8 |  | IMD5 |  |  |
|  | 42 | 3402.5 |  | 5 |  | 25 |  | 3402.5 |  | N/A |  | N/A |  |  |
|  | n79 | 4640 |  | 40 |  | 216 |  | 4640 |  | N/A |  | N/A |  |  |
|  | 1 | N/A |  | 5 |  | N/A |  | 2165 |  | 15.5 |  | IMD3 |  |  |
|  | 42 | 3450 |  | 5 |  | 25 |  | 3450 |  | N/A |  | N/A |  |  |
|  | n79 | 4520 |  | 40 |  | 216 |  | 4520 |  | N/A |  | N/A |  |  |
|  | 1 | N/A |  | 5 |  | N/A |  | 2140 |  | 9.3 |  | IMD4 |  |  |
| DC_1A_SUL_n77A-n80A | 1 | N/A |  | 5 |  | N/A |  | 2140 |  | 23 |  | IMD3 |  |  |
|  | n80 | 1760 |  | 5 |  | 25 |  |  |  | N/A |  | N/A |  |  |
| DC_1A_SUL_n77A-n80A | 1 | 1922.5 |  | 5 |  | 25 |  | 2112.5 |  | N/A |  | N/A |  |  |
|  | n80 | 1782.5 |  | 5 |  | 25 |  |  |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3425 |  | 13.0 |  | IMD4 |  |  |
| DC_1A_n75A-n78A DC_1A_n75A-n78(2A) | 1 | 1930 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
|  | n75 | N/A |  | 5 |  | N/A |  | 1470 |  | 30.4 |  | IMD2 |  |  |
|  | n78 | 3400 |  | 10 |  | 50 |  | 3400 |  | N/A |  | N/A |  |  |
| DC_1A_n78A-n79A | 1 | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | n78 | 3410 |  | 10 |  | 50 |  | 3410 |  | N/A |  | N/A |  |  |
|  | n79 | N/A |  | 40 |  | N/A |  | 4870 |  | 15.9 |  | IMD3 |  |  |
|  | 1 | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | n79 | 4670 |  | 40 |  | 216 |  | 4670 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3490 |  | 4.6 |  | IMD5 |  |  |
| DC_1A_SUL_n78A-n80A | 1 | 1950 |  | 5 |  | 25 |  | 2140 |  | 23 |  | IMD3 |  |  |
|  | n80 | 1760 |  | 5 |  | 25 |  |  |  | N/A |  | N/A |  |  |
|  | 1 | 1922.5 |  | 5 |  | 25 |  | 2112.5 |  | N/A |  | N/A |  |  |
|  | n80 | 1782.5 |  | 5 |  | 25 |  |  |  | N/A |  | N/A |  |  |
|  | n78 | 3425 |  | 10 |  | 50 |  | 3425 |  | 13.0 |  | IMD4 |  |  |
| DC_1_n78-n105 | 1 | 1970 |  | 5 |  | 25 |  | 2160 |  | N/A |  | N/A |  |  |
|  | n78 | 3305 |  | 10 |  | 50 |  | 3305 |  | N/A |  | N/A |  |  |
|  | n105 | 686 |  | 5 |  | 25 |  | 635 |  | 15.2 |  | IMD3 |  |  |
|  | 1 | 1970 |  | 5 |  | 25 |  | 2160 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3342 |  | 15.7 |  | IMD3 |  |  |
|  | n105 | 686 |  | 5 |  | 25 |  | 635 |  | N/A |  | N/A |  |  |
| DC_2A-(n)66AA | 2 | 1883.3 |  | 5 |  | 25 |  | 1963.3 |  | N/A |  | N/A |  |  |
|  | 66 | N/A |  | 5 |  | N/A |  | 2145 |  | 2.8 |  | IMD5 |  |  |
|  | n66 | 1750 |  | 5 |  | 25 |  | 2150 |  | 4 |  | IMD5 |  |  |
| DC_2A_n2A-n66A | 2 | 1875 |  | 5 |  | 25 |  | 1955 |  | N/A |  | N/A |  |  |
|  | n2 | N/A |  | 5 |  | N/A |  | 1975 |  | 20 |  | IMD3 |  |  |
|  | n66 | 1775 |  | 5 |  | 25 |  | 2175 |  | N/A |  | N/A |  |  |
| DC_2A_n2A-n77A | 2 | 1875 |  | 5 |  | 25 |  | 1955 |  | N/A |  | N/A |  |  |
|  | n2 | N/A |  | 5 |  | N/A |  | 1935 |  | 26 |  | IMD2 |  |  |
|  | n77 | 3810 |  | 10 |  | 50 |  | 3810 |  | N/A |  | N/A |  |  |
|  | 2 | 1900 |  | 5 |  | 25 |  | 1980 |  | N/A |  | N/A |  |  |
|  | n2 | N/A |  | 5 |  | N/A |  | 1965 |  | 8.0 |  | IMD44 |  |  |
|  | n77 | 3735 |  | 10 |  | 50 |  | 3735 |  | N/A |  | N/A |  |  |
| DC_2A_n2A-n78A | 2 | 1852.5 |  | 5 |  | 25 |  | 1932.5 |  | N/A |  | N/A |  |  |
|  | n2 | N/A |  | 5 |  | N/A |  | 1942.5 |  | 26 |  | IMD24 |  |  |
|  | n78 | 3795 |  | 10 |  | 50 |  | 3795 |  | N/A |  | N/A |  |  |
| DC_2A-4A_n28A | 2 | N/A |  | 5 |  | N/A |  | 1960 |  | 11.0 |  | IMD4 |  |  |
|  | 4 | 1720 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
|  | n28 | 740 |  | 5 |  | 25 |  | 795 |  | N/A |  | N/A |  |  |
| DC_2A-4A_n41A | 2 | N/A |  | 5 |  | N/A |  | 1940 |  | 11.0 |  | IMD4 |  |  |
|  | 4 | 1715 |  | 5 |  | 25 |  | 2115 |  | N/A |  | N/A |  |  |
|  | n41 | 2685 |  | 10 |  | 50 |  | 2685 |  | N/A |  | N/A |  |  |
| DC_2A-4A_n78A | 2 | 1875 |  | 5 |  | 25 |  | 1955 |  | N/A |  | N/A |  |  |
|  | 4 | N/A |  | 5 |  | N/A |  | 2145 |  | 10.3 |  | IMD4 |  |  |
|  | n78 | 3480 |  | 10 |  | 50 |  | 3480 |  | N/A |  | N/A |  |  |
|  | 2 | N/A |  | 5 |  | N/A |  | 1960 |  | 32.1 |  | IMD2 |  |  |
|  | 4 | 1740 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | n78 | 3700 |  | 10 |  | 50 |  | 3700 |  | N/A |  | N/A |  |  |
|  | 2 | N/A |  | 5 |  | N/A |  | 1940 |  | 9.1 |  | IMD4 |  |  |
|  | 4 | 1750 |  | 5 |  | 25 |  | 2150 |  | N/A |  | N/A |  |  |
|  | n78 | 3310 |  | 10 |  | 50 |  | 3310 |  | N/A |  | N/A |  |  |
|  | 2 | N/A |  | 5 |  | N/A |  | 1950 |  | 2.1 |  | IMD5 |  |  |
|  | 4 | 1750 |  | 5 |  | 25 |  | 2150 |  | N/A |  | N/A |  |  |
|  | n78 | 3600 |  | 10 |  | 50 |  | 3600 |  | N/A |  | N/A |  |  |
| DC_2A-5A_n12A8 | 2 | N/A |  | 5 |  | N/A |  | 1980 |  | 5.9 |  | IMD5 |  |  |
|  | 5 | 840 |  | 5 |  | 25 |  | 885 |  | N/A |  | N/A |  |  |
|  | n12 | 705 |  | 5 |  | 25 |  | 735 |  | N/A |  | N/A |  |  |
| DC_2A-5A_n30A | 2 | 1870 |  | 5 |  | 25 |  | 1959 |  | N/A |  | N/A |  |  |
|  | 5 | N/A |  | 5 |  | N/A |  | 880 |  | 9.7 |  | IMD4 |  |  |
|  | n30 | 2310 |  | 10 |  | 50 |  | 2355 |  | N/A |  | N/A |  |  |
| DC_2A-5A_n48A DC_2A-5A_n48B | 2 | N/A |  | 5 |  | N/A |  | 1962 |  | 15.6 |  | IMD3 |  |  |
|  | 5 | 839 |  | 5 |  | 25 |  | 884 |  | N/A |  | N/A |  |  |
|  | n48 | 3640 |  | 5 |  | 25 |  | 3640 |  | N/A |  | N/A |  |  |
| DC_2A-5A_n71A | 2 | 1855 |  | 5 |  | 25 |  | 1935 |  | N/A |  | N/A |  |  |
|  | n71 | 686.5 |  | 5 |  | 25 |  | 640.5 |  | N/A |  | N/A |  |  |
|  | 5 | N/A |  | 5 |  | N/A |  | 891.5 |  | 4.2 |  | IMD5 |  |  |
| DC_2A_n5A-n77A | 2 | 1880 |  | 5 |  | 25 |  | 1960 |  | N/A |  | N/A |  |  |
|  | n5 | 830 |  | 5 |  | 25 |  | 875 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 3540 |  | 16.0 |  | IMD3 |  |  |
| DC_2A_n5A-n77A11 | 2 | 1907 |  | 5 |  | 25 |  | 1987 |  | N/A |  | N/A |  |  |
|  | n5 | N/A |  | 5 |  | N/A |  | 889 |  | 3.8 |  | IMD5 |  |  |
|  | n77 | 3305 |  | 10 |  | 50 |  | 3305 |  | N/A |  | N/A |  |  |
| DC_2A-5A_n77A11 | 2 | 1907.5 |  | 5 |  | 25 |  | 1987.5 |  | N/A |  | N/A |  |  |
| DC_2A-5A_n77C11 DC_2A-5A_n77(2A)11 DC_2A-2A-5A_n77A11 | 5 | N/A |  | 5 |  | N/A |  | 887.5 |  | 3.8 |  | IMD5 |  |  |
| DC_2A-2A-5A_n77C11 DC_2A-2A-5A_n77(2A)11 | n77 | 3305 |  | 10 |  | 50 |  | 3305 |  | N/A |  | N/A |  |  |
|  | 2 | N/A |  | 5 |  | N/A |  | 1987 |  | 16.5 |  | IMD3 |  |  |
|  | 5 | 846.5 |  | 5 |  | 25 |  | 891.5 |  | N/A |  | N/A |  |  |
|  | n77 | 3680 |  | 10 |  | 50 |  | 3680 |  | N/A |  | N/A |  |  |
| DC_2A-5A_n78A DC_2A-2A-5A_n78A DC_2A-5A_n78(2A) | 2 | 1907.5 |  | 5 |  | 25 |  | 1987.5 |  | N/A |  | N/A |  |  |
|  | 5 | N/A |  | 5 |  | N/A |  | 887.5 |  | 3.8 |  | IMD5 |  |  |
|  | n78 | 3305 |  | 10 |  | 50 |  | 3305 |  | N/A |  | N/A |  |  |
|  | 2 | N/A |  | 5 |  | N/A |  | 1987 |  | 16.5 |  | IMD3 |  |  |
|  | 5 | 846.5 |  | 5 |  | 25 |  | 891.5 |  | N/A |  | N/A |  |  |
|  | n78 | 3680 |  | 10 |  | 50 |  | 3680 |  | N/A |  | N/A |  |  |
| DC_2A-7A_n5A DC_2A-7C_n5A DC_2A-7A-7A_n5A | 2 | 1855 |  | 10 |  | 50 |  | 1935 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 10 |  | N/A |  | 2685 |  | 30.0 |  | IMD2 |  |  |
|  | n5 | 830 |  | 5 |  | 25 |  | 875 |  | N/A |  | N/A |  |  |
| DC_2A-7A_n12A | 2 | 1907.5 | 5 |  | 25 |  | 1987.5 |  | N/A |  | N/A |  |  |
| DC_2A-2A-7A_n12A | 7 | 2502.5 | 5 |  | 25 |  | 2622.5 |  | 30.8 |  | IMD2 |  |  |
|  | n12 | 713.5 | 5 |  | 25 |  | 743.5 |  | N/A |  | N/A |  |  |
| DC_2A-7A_n28A DC_2A-7C_n28A | 2 | 1880 |  | 5 |  | 25 |  | 1960 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 5 |  | N/A |  | 2120 |  | 29.0 |  | IMD2 |  |  |
|  | n28 | 740 |  | 5 |  | 25 |  | 795 |  | N/A |  | N/A |  |  |
| DC_2A-7A_n77A DC_2A-2A-7A_n77A DC_2A-7C_n77A DC_2A-7A-7A_n77A DC_2A-7A_n77(2A) DC_2A-7C_n77(2A) DC_2A-7A-7A_n77(2A) | 2 | N/A |  | 5 |  | N/A |  | 1950 |  | 8.6 |  | IMD4 |  |  |
|  | 7 | 2550 |  | 5 |  | 25 |  | 2685 |  | N/A |  | N/A |  |  |
|  | n77 | 3525 |  | 10 |  | 50 |  | 3475 |  | N/A |  | N/A |  |  |
|  | 2 | 1860 |  | 5 |  | 25 |  | 1940 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 5 |  | N/A |  | 2660 |  | 3.4 |  | IMD5 |  |  |
|  | n77 | 4120 |  | 10 |  | 50 |  | 4120 |  | N/A |  | N/A |  |  |
| DC_2A-7A_n78A DC_2A-2A-7A_n78A DC_2A-7C_n78A DC_2A-7A-7A_n78A DC_2A-7A_n78(2A) DC_2A-7C_n78(2A) DC_2A-7A-7A_n78(2A) | 2 | N/A |  | 5 |  | N/A |  | 1950 |  | 8.6 |  | IMD4 |  |  |
|  | 7 | 2550 |  | 5 |  | 25 |  | 2685 |  | N/A |  | N/A |  |  |
|  | n78 | 3525 |  | 10 |  | 50 |  | 3475 |  | N/A |  | N/A |  |  |
| DC_2A_n7A-n78A, DC_2A_n7(2A)-n78A DC_2A_n7A-n78(2A) DC_2A_n7(2A)-n78(2A) | 2 | 1900 |  | 5 |  | 25 |  | 1980 |  | N/A |  | N/A |  |  |
|  | n7 | 2525 |  | 5 |  | 25 |  | 2645 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3775 |  | 4.2 |  | IMD5 |  |  |
| DC_2-8_n2 | 2 | N/A |  | 5 |  | N/A |  | 1940 |  | 4 |  | IMD4 |  |  |
|  | 8 | 910 |  | 5 |  | 25 |  | 955 |  | N/A |  | N/A |  |  |
|  | n2 | 1880 |  | 5 |  | 25 |  | 1960 |  | N/A |  | N/A |  |  |
| DC_2A-12A_n5A DC_2A-2A-12A_n5A | 2 | N/A |  | 5 |  | N/A |  | 1980 |  | 5.9 |  | IMD5 |  |  |
|  | 12 | 705 |  | 5 |  | 25 |  | 735 |  | N/A |  | N/A |  |  |
|  | n5 | 840 |  | 5 |  | 25 |  | 885 |  | N/A |  | N/A |  |  |
| DC_2A-12A_n7A DC_2A-12A_n7(2A) | 2 | 1907.5 |  | 5 |  | 25 |  | 1987.5 |  | N/A |  | N/A |  |  |
| DC_2A-2A-12A_n7A | 12 | N/A |  | 5 |  | N/A |  | 731.5 |  | 4.5 |  | IMD5 |  |  |
|  | n7 | 2502.5 |  | 5 |  | 25 |  | 2622.5 |  | N/A |  | N/A |  |  |
| DC_2A-12A_n41A DC_2A-2A-12A_n41A | 2 | N/A |  | 5 |  | N/A |  | 1952 |  | 26 |  | IMD2 |  |  |
|  | 12 | 708 |  | 5 |  | 50 |  | 738 |  | N/A |  | N/A |  |  |
|  | n41 | 2660 |  | 10 |  | 50 |  | 2660 |  | N/A |  | N/A |  |  |
|  | 2 | 1900 |  | 5 |  | 25 |  | 1980 |  | N/A |  | N/A |  |  |
|  | 12 | N/A |  | 5 |  | N/A |  | 738 |  | 28.7 |  | IMD24 |  |  |
|  | n41 | 2638 |  | 10 |  | 50 |  | 2638 |  | N/A |  | N/A |  |  |
| DC_2A-12A_n66A | 2 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | IMD4 |  |  |
|  | 12 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
|  | n66 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
| DC_2A-12A_n77A DC_2A-12A_n77(2A) | 2 | N/A |  | 5 |  | N/A |  | 1960 |  | 16.5 |  | IMD39,11 |  |  |
| DC_2A-2A-12A_n77A DC_2A-2A-12A_n77(2A) | 12 | 707.5 |  | 5 |  | 25 |  | 737.5 |  | N/A |  | N/A |  |  |
|  | n77 | 3375 |  | 10 |  | 50 |  | 3375 |  | N/A |  | N/A |  |  |
| DC_2A_n12A-n77A DC_2A-2A_n12A-n77A | 2 | 1900 |  | 5 |  | 25 |  | 1980 |  | N/A |  | N/A |  |  |
|  | n12 | 707.5 |  | 5 |  | 25 |  | 737.5 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 3315 |  | 16.0 |  | IMD34,9,11 |  |  |
| DC_2A-12A_n78A DC_2A-2A-12A_n78A DC_2A-12A_n78(2A) | 2 | N/A |  | 5 |  | N/A |  | 1954 |  | 16.5 |  | IMD3 |  |  |
|  | 12 | 708 |  | 5 |  | 25 |  | 738 |  | N/A |  | N/A |  |  |
|  | n78 | 3370 |  | 10 |  | 50 |  | 3370 |  | N/A |  | N/A |  |  |
| DC_2A_n12A-n78A | 2 | 1900 |  | 5 |  | 25 |  | 1980 |  | N/A |  | N/A |  |  |
|  | n12 | 707.5 |  | 5 |  | 25 |  | 737.5 |  | N/A |  | N/A |  |  |
|  | n78 | 3315 |  | 10 |  | 50 |  | 3315 |  | 16.0 |  | IMD3 |  |  |
| DC_2A-13A_n48A DC_2A-13A_n48B | 2 | N/A |  | 5 |  | N/A |  | 1983.5 |  | 15.6 |  | IMD3 |  |  |
|  | 13 | 784.5 |  | 5 |  | 25 |  | 753.5 |  | N/A |  | N/A |  |  |
|  | n48 | 3552.5 |  | 5 |  | 25 |  | 3552.5 |  | N/A |  | N/A |  |  |
| DC_2A-13A_n66A DC_2A-2A-13A_n66A | 2 | N/A |  | 5 |  | N/A |  | 1940 |  | 6.2 |  | IMD4 |  |  |
|  | 13 | 780 |  | 10 |  | 50 |  | 749 |  | N/A |  | N/A |  |  |
|  | n66 | 1750 |  | 5 |  | 25 |  | 2150 |  | N/A |  | N/A |  |  |
| DC_2A-13A_n77A | 2 | N/A |  | 5 |  | N/A |  | 1944 |  | 16.0 |  | IMD3 |  |  |
| DC_2A-13A_n77C | 13 | 783 |  | 10 |  | 50 |  | 752 |  | N/A |  | N/A |  |  |
| DC_2A-2A-13A_n77A DC_2A-2A-13A_n77C | n77 | 3510 |  | 5 |  | 25 |  | 3510 |  | N/A |  | N/A |  |  |
| DC_2A-14A_n77A DC_2A-14A_n77(2A) | 2 | N/A |  | 5 |  | N/A |  | 1954 |  | 16.5 |  | IMD3 |  |  |
| DC_2A-2A-14A_n77A DC_2A-2A-14A_n77(2A) | 14 | 793 |  | 5 |  | 25 |  | 763 |  | N/A |  | N/A |  |  |
|  | n77 | 3540 |  | 10 |  | 50 |  | 3540 |  | N/A |  | N/A |  |  |
| DC_2_n25-n66 | 2 | 1855 |  | 5 |  | 25 |  | 1935 |  | N/A |  | N/A |  |  |
|  | n25 | N/A |  | 5 |  | N/A |  | 1935 |  | 20 |  | IMD3 |  |  |
|  | n66 | 1775 |  | 5 |  | 25 |  | 2175 |  | N/A |  | N/A |  |  |
|  | 2 | 1883.3 |  | 5 |  | 25 |  | 1963.3 |  | N/A |  | N/A |  |  |
|  | n25 | N/A |  | 5 |  | N/A |  | 1963.3 |  | 4 |  | IMD5 |  |  |
|  | n66 | 1750 |  | 5 |  | 25 |  | 2150 |  | N/A |  | N/A |  |  |
| DC_2A_n38A-n71A | 2 | 1900 |  | 5 |  | 25 |  | 1980 |  | N/A |  | N/A |  |  |
|  | n38 | N/A |  | 10 |  | N/A |  | 2586 |  | 29.2 |  | IMD2 |  |  |
|  | n71 | 686 |  | 5 |  | 25 |  | 640 |  | N/A |  | N/A |  |  |
| DC_2A_n38A-n78A | 2 | 1870 |  | 5 |  | 25 |  | 1950 |  | N/A |  | N/A |  |  |
|  | n38 | 2610 |  | 10 |  | 50 |  | 2610 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3350 |  | 14.8 |  | IMD3 |  |  |
| DC_2A-14A_n66A | 2 | N/A |  | 5 |  | N/A |  | 1954 |  | 7.2 |  | IMD4 |  |  |
|  | 14 | 793 |  | 5 |  | 25 |  | 763 |  | N/A |  | N/A |  |  |
|  | 66 | 1770 |  | 5 |  | 25 |  | 2170 |  | N/A |  | N/A |  |  |
| DC_2A-28A_n66A | 2 | N/A |  | 5 |  | N/A |  | 1980 |  | 11 |  | IMD4 |  |  |
|  | 28 | 730 |  | 5 |  | 25 |  | 785 |  | N/A |  | N/A |  |  |
|  | n66 | 1720 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
| DC_2A-30A_n77A DC_2A-30A_n77(2A) | 2 | N/A |  | 5 |  | N/A |  | 1986 |  | 8.6 |  | IMD411 |  |  |
| DC_2A-2A-30A_n77A DC_2A-2A-30A_n77(2A) | 30 | 2312 |  | 5 |  | 25 |  | 2357 |  | N/A |  | N/A |  |  |
|  | n77 | 3305 |  | 10 |  | 50 |  | 3305 |  | N/A |  | N/A |  |  |
|  | 2 | 1905 |  | 5 |  | 25 |  | 1985 |  | N/A |  | N/A |  |  |
|  | 30 | N/A |  | 5 |  | N/A |  | 2354 |  | 10.6 |  | IMD411 |  |  |
|  | n77 | 3361 |  | 10 |  | 50 |  | 3361 |  | N/A |  | N/A |  |  |
|  | 2 | 1860 |  | 5 |  | 25 |  | 1940 |  | N/A |  | N/A |  |  |
|  | 30 | N/A |  | 5 |  | N/A |  | 2354 |  | 3.4 |  | IMD5 |  |  |
|  | n77 | 3967 |  | 10 |  | 50 |  | 3967 |  | N/A |  | N/A |  |  |
| DC_2A-38A_n78A | 2 | N/A |  | 5 |  | N/A |  | 1932.5 |  | 16 |  | IMD39 |  |  |
|  | 38 | 2617.5 |  | 5 |  | 25 |  | 2617.5 |  | N/A |  | N/A |  |  |
|  | n78 | 3305 |  | 10 |  | 50 |  | 3305 |  | N/A |  | N/A |  |  |
| DC_2A_n41A-n71A DC_2A-2A_n41A-n71A | 2 | 1900 |  | 5 |  | 25 |  | 1980 |  | N/A |  | N/A |  |  |
|  | n41 | 2530 |  | 10 |  | 50 |  | 2530 |  | N/A |  | N/A |  |  |
|  | n71 | N/A |  | 5 |  | N/A |  | 630 |  | 28.7 |  | IMD2 |  |  |
|  | 2 | 1900 |  | 5 |  | 25 |  | 1980 |  | N/A |  | N/A |  |  |
|  | n41 | N/A |  | 10 |  | N/A |  | 2586 |  | 29.2 |  | IMD2 |  |  |
|  | n71 | 686 |  | 5 |  | 50 |  | 640 |  | N/A |  | N/A |  |  |
| DC_2A-46A_n5A5 DC_2A-46C_n5A5 DC_2A-46D_n5A5 DC_2A-46E_n5A5 | 2 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
| DC_2A-2A-46A_n5A5 DC_2A-2A-46C_n5A5 DC_2A-2A-46D_n5A5 | 46 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | IMD4, IMD5 |  |  |
|  | n5 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
| DC_2A-46A_n66A5 DC_2A-46C_n66A5 DC_2A-46D_n66A5 DC_2A-46E_n66A5 | 2 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
|  | 46 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | IMD3, IMD5 |  |  |
|  | n66 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
| DC_2A-46A_n77A5 DC_2A-46A-46A_n77A5 | 2 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
|  | 46 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | IMD2, IMD3 |  |  |
|  | n77 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
| DC_2A-48A_n2A DC_2A-48C_n2A DC_2A-48D_n2A DC_2A-48E_n2A | n2 | 1853 |  | 5 |  | 25 |  | 1933 |  | N/A |  | N/A |  |  |
|  | 48 | 3590 |  | 20 |  | 100 |  | 3590 |  | N/A |  | N/A |  |  |
|  | 2 | N/A |  | 5 |  | N/A |  | 1969 |  | 12 |  | IMD4 |  |  |
| DC_2A-48A_n5A | 2 | N/A |  | 5 |  | N/A |  | 1950 |  | 16.9 |  | IMD3 |  |  |
| DC_2A-48C_n5A | 48 | 3610 |  | 10 |  | 50 |  | 3610 |  | N/A |  | N/A |  |  |
| DC_2A-48D_n5A | n5 | 830 |  | 5 |  | 25 |  | 875 |  | N/A |  | N/A |  |  |
| DC_2A-48E_n5A | 2 | 1890 |  | 5 |  | 25 |  | 1970 |  | N/A |  | N/A |  |  |
|  | 48 | N/A |  | 5 |  | N/A |  | 3570 |  | 16.2 |  | IMD3 |  |  |
|  | n5 | 840 |  | 5 |  | 25 |  | 885 |  | N/A |  | N/A |  |  |
| DC_2A-48A_n66A DC_2A-48C_n66A DC_2A-48D_n66A | 2 | 1880 |  | 5 |  | 25 |  | 1960 |  | N/A |  | N/A |  |  |
|  | 48 | N/A |  | 10 |  | N/A |  | 3620 |  | 29.4 |  | IMD2 |  |  |
|  | n66 | 1740 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | 2 | N/A |  | 5 |  | N/A |  | 1960 |  | 28.3 |  | IMD2 |  |  |
|  | 48 | 3695 |  | 5 |  | 25 |  | 3695 |  | N/A |  | N/A |  |  |
|  | n66 | 1735 |  | 5 |  | 25 |  | 2135 |  | N/A |  | N/A |  |  |
| DC_2A_n48A-n66A | 2 | 1880 |  | 5 |  | 25 |  | 1960 |  | N/A |  | N/A |  |  |
| DC_2A-48E_n66A | n48 | N/A |  | 10 |  | N/A |  | 3620 |  | 29.4 |  | IMD2 |  |  |
|  | n66 | 1740 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | 2 | N/A |  | 5 |  | N/A |  | 1980 |  | 20 |  | IMD3 |  |  |
| DC_2A-66A_n2A | 66 | 1730 |  | 5 |  | 25 |  | 2130 |  | N/A |  | N/A |  |  |
| DC_2A-66A-66A_n2A | n2 | 1855 |  | 5 |  | 25 |  | 1935 |  | N/A |  | N/A |  |  |
| DC_2A-66A_n5A | 2 | 1900 |  | 5 |  | 25 |  | 1980 |  | N/A |  | N/A |  |  |
|  | 66 | N/A |  | 5 |  | N/A |  | 2140 |  | 7.2 |  | IMD4 |  |  |
|  | n5 | 830 |  | 5 |  | 25 |  | 875 |  | N/A |  | N/A |  |  |
| DC_2A-66A_n25A | 2 | N/A |  | 5 |  | N/A |  | 1935 |  | 20 |  | IMD3 |  |  |
|  | 66 | 1775 |  | 5 |  | 25 |  | 2175 |  | N/A |  | N/A |  |  |
|  | n25 | N/A |  | 5 |  | N/A |  | 1935 |  | 20 |  | IMD3 |  |  |
|  | 2 | 1883.3 |  | 5 |  | 25 |  | 1963.3 |  | N/A |  | N/A |  |  |
|  | 66 | N/A |  | 5 |  | N/A |  | 2150 |  | 4 |  | IMD5 |  |  |
|  | n25 | 1883.3 |  | 5 |  | 25 |  | 1963.3 |  | N/A |  | N/A |  |  |
|  | 2 | 1883.3 |  | 5 |  | 25 |  | 1963.3 |  | N/A |  | N/A |  |  |
|  | 66 | N/A |  | 5 |  | N/A |  | 2112.5 |  | 23 |  | IMD3 |  |  |
|  | n25 | 1912.5 |  | 5 |  | 25 |  | 1992.5 |  | N/A |  | N/A |  |  |
| DC_2A-66A_n28A | 2 | N/A |  | 5 |  | N/A |  | 1960 |  | 11.0 |  | IMD4 |  |  |
|  | 66 | 1720 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
|  | n28 | 740 |  | 5 |  | 25 |  | 795 |  | N/A |  | N/A |  |  |
| DC_2A-66A_n41A DC_2A-66A_n41C DC_2A-66A_n41(2A) | 2 | N/A |  | 5 |  | N/A |  | 1940 |  | 11.0 |  | IMD4 |  |  |
|  | 66 | 1715 |  | 5 |  | 25 |  | 2115 |  | N/A |  | N/A |  |  |
|  | n41 | 2685 |  | 5 |  | 25 |  | 2685 |  | N/A |  | N/A |  |  |
| DC_2A-66A_n48A DC_2A-66A_n48B DC_2A-66A-66A_n48A DC_2A-66A-66A_n48B | 2 | 1905 |  | 5 |  | 25 |  | 1985 |  | N/A |  | N/A |  |  |
|  | 66 | N/A |  | 5 |  | N/A |  | 2155 |  | 12.1 |  | IMD4 |  |  |
|  | n48 | 3560 |  | 5 |  | 25 |  | 3560 |  | N/A |  | N/A |  |  |
| DC_2A-66A_n48A DC_2A-66A_n48B DC_2A-66A-66A_n48A DC_2A-66A-66A_n48B | 2 | N/A |  | 5 |  | N/A |  | 1960 |  | 28.3 |  | IMD5 |  |  |
|  | 66 | 1735 |  | 5 |  | 25 |  | 2135 |  | N/A |  | N/A |  |  |
|  | n48 | 3695 |  | 5 |  | 25 |  | 3695 |  | N/A |  | N/A |  |  |
| DC_2A-66A_n77A | 2 | 1855 |  | 5 |  | 25 |  | 1935 |  | N/A |  | N/A |  |  |
| DC_2A-66A_n77C DC_2A-66A_n77(2A) DC_2A-2A-66A_n77A DC_2A-2A-66A_n77C DC_2A-2A-66A_n77(2A) DC_2A-66A-66A_n77A DC_2A-66A-66A_n77C DC_2A-66A-66A_n77(2A) DC_2A-2A-66A-66A_n77A DC_2A-2A-66A-66A_n77C | 66 | N/A |  | 5 |  | N/A |  | 2115 |  | 29.2 |  | IMD2 |  |  |
|  | n77 | 3970 |  | 10 |  | 50 |  | 3970 |  | N/A |  | N/A |  |  |
|  | 2 | 1880 |  | 5 |  | 25 |  | 1960 |  | N/A |  | N/A |  |  |
|  | 66 | N/A |  | 5 |  | N/A |  | 2140 |  | 10.4 |  | IMD4 |  |  |
|  | n77 | 3500 |  | 10 |  | 50 |  | 3500 |  | N/A |  | N/A |  |  |
|  | 2 | 1885 |  | 5 |  | 25 |  | 1965 |  | N/A |  | N/A |  |  |
|  | 66 | N/A |  | 5 |  | N/A |  | 2175 |  | 4.0 |  | IMD5 |  |  |
|  | n77 | 3915 |  | 10 |  | 50 |  | 3915 |  | N/A |  | N/A |  |  |
|  | 2 | N/A |  | 5 |  | N/A |  | 1960 |  | 32.1 |  | IMD2 |  |  |
|  | 66 | 1760 |  | 5 |  | 25 |  | 2160 |  | N/A |  | N/A |  |  |
|  | n77 | 3720 |  | 10 |  | 50 |  | 3720 |  | N/A |  | N/A |  |  |
| DC_2A-66A_n77A11 DC_2A-66A_n77C11 DC_2A-66A_n77(2A)11 DC_2A-2A-66A_n77A11 DC_2A-2A-66A_n77C11 DC_2A-66A-66A_n77A11 DC_2A-66A-66A_n77C11 DC_2A-2A-66A-66A_n77A11 DC_2A-2A-66A-66A_n77C11 | 2 | N/A |  | 5 |  | N/A |  | 1960 |  | 32.1 |  | IMD2 |  |  |
|  | 66 | 1745 |  | 5 |  | 25 |  | 2145 |  | N/A |  | N/A |  |  |
|  | n77 | 3705 |  | 10 |  | 50 |  | 3705 |  | N/A |  | N/A |  |  |
| DC_2A_n66A-n77A11 DC_2A-2A_n66A-n77A11 | 2 | 1855 |  | 5 |  | 25 |  | 1935 |  | N/A |  | N/A |  |  |
|  | n66 | N/A |  | 5 |  | N/A |  | 2115 |  | 29.2 |  | IMD2 |  |  |
|  | n77 | 3970 |  | 10 |  | 50 |  | 3970 |  | N/A |  | N/A |  |  |
|  | 2 | 1853 |  | 5 |  | 25 |  | 1933 |  | N/A |  | N/A |  |  |
|  | n66 | 1713 |  | 5 |  | 25 |  | 2113 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 3566 |  | 29.4 |  | IMD2 |  |  |
| DC_2A-66A_n78A DC_2A-66A_n78(2A) DC_2A-66A-66A_n78A DC_2A-66A-66A_n78(2A) | 2 | 1880 |  | 5 |  | 25 |  | 1960 |  | N/A |  | N/A |  |  |
| DC_2A-2A-66A_n78A | 66 | N/A |  | 5 |  | N/A |  | 2160 |  | 10.3 |  | IMD4 |  |  |
|  | n78 | 3480 |  | 10 |  | 50 |  | 3480 |  | N/A |  | N/A |  |  |
|  | 2 | N/A |  | 5 |  | N/A |  | 1960 |  | 32.1 |  | IMD2 |  |  |
|  | 66 | 1740 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | n78 | 3700 |  | 10 |  | 50 |  | 3700 |  | N/A |  | N/A |  |  |
|  | 2 | N/A |  | 5 |  | N/A |  | 1960 |  | 9.1 |  | IMD4 |  |  |
|  | 66 | 1770 |  | 5 |  | 25 |  | 2170 |  | N/A |  | N/A |  |  |
|  | n78 | 3350 |  | 10 |  | 50 |  | 3350 |  | N/A |  | N/A |  |  |
|  | 2 | N/A |  | 5 |  | N/A |  | 1960 |  | 2.1 |  | IMD5 |  |  |
|  | 66 | 1760 |  | 5 |  | 25 |  | 2160 |  | N/A |  | N/A |  |  |
|  | n78 | 3620 |  | 10 |  | 50 |  | 3620 |  | N/A |  | N/A |  |  |
| DC_2A_n66A-n78A DC_2A_n66(2A)-n78A DC_2A_n66(2A)-n78(2A) | 2 | 1880 |  | 5 |  | 25 |  | 1960 |  | N/A |  | N/A |  |  |
|  | n66 | 1740 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3620 |  | 29.4 |  | IMD2 |  |  |
|  | 2 | 1880 |  | 5 |  | 25 |  | 1960 |  | N/A |  | N/A |  |  |
|  | n66 | N/A |  | 5 |  | N/A |  | 2140 |  | 10.3 |  | IMD4 |  |  |
|  | n78 | 3500 |  | 10 |  | 50 |  | 3500 |  | N/A |  | N/A |  |  |
|  | 2 | 1880 |  | 5 |  | 25 |  | 1960 |  | N/A |  | N/A |  |  |
|  | n66 | 1740 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3340 |  | 8.9 |  | IMD4 |  |  |
| DC_2A-71A_n7A | 2 | 1900 |  | 5 |  | 25 |  | 1980 |  | N/A |  | N/A |  |  |
| DC_2A-2A-71A_n7A | 71 | N/A |  | 5 |  | N/A |  | 630 |  | 28.7 |  | IMD24 |  |  |
|  | n7 | 2530 |  | 10 |  | 50 |  | 2650 |  | N/A |  | N/A |  |  |
| DC_2A-71A_n38A DC_2A-2A-71A_n38A | 2 | N/A |  | 5 |  | N/A |  | 1942 |  | 26 |  | IMD2 |  |  |
|  | 71 | 668 |  | 5 |  | 25 |  | 622 |  | N/A |  | N/A |  |  |
|  | n38 | 2610 |  | 10 |  | 50 |  | 2610 |  | N/A |  | N/A |  |  |
| DC_2A-71A_n41A DC_2A-2A-71A_n41A | 2 | N/A |  | 5 |  | N/A |  | 1942 |  | 26 |  | IMD2 |  |  |
|  | 71 | 668 |  | 5 |  | 25 |  | 622 |  | N/A |  | N/A |  |  |
|  | n41 | 2610 |  | 10 |  | 50 |  | 2610 |  | N/A |  | N/A |  |  |
|  | 2 | 1900 |  | 5 |  | 25 |  | 1980 |  | N/A |  | N/A |  |  |
|  | 71 | N/A |  | 5 |  | N/A |  | 630 |  | 28.7 |  | IMD24 |  |  |
|  | n41 | 2530 |  | 10 |  | 50 |  | 2530 |  | N/A |  | N/A |  |  |
| DC_2A-71A_n77A DC_2A-2A-71A_n77A DC_2A-71A_n77(2A) | 2 | N/A |  | 5 |  | N/A |  | 1954 |  | 16.5 |  | IMD39 |  |  |
|  | 71 | 693 |  | 5 |  | 25 |  | 647 |  | N/A |  | N/A |  |  |
|  | n77 | 3340 |  | 10 |  | 50 |  | 3340 |  | N/A |  | N/A |  |  |
| DC_2A_n71A-n77A DC_2A-2A_n71A-n77A DC_2A_n71A-n77(2A) | 2 | 1907.5 |  | 5 |  | 25 |  | 1987.5 |  | N/A |  | N/A |  |  |
|  | n71 | 695.5 |  | 5 |  | 25 |  | 649.5 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 3305 |  | 8 |  | IMD3 |  |  |
| DC_2A-71A_n78A DC_2A-2A-71A_n78A | 2 | N/A |  | 5 |  | N/A |  | 1954 |  | 16.5 |  | IMD3 |  |  |
|  | 71 | 693 |  | 5 |  | 25 |  | 647 |  | N/A |  | N/A |  |  |
|  | n78 | 3340 |  | 10 |  | 50 |  | 3340 |  | N/A |  | N/A |  |  |
| DC_2A_n71A-n78A DC_2A-2A_n71A-n78A | 2 | 1907.5 |  | 5 |  | 25 |  | 1987.5 |  | N/A |  | N/A |  |  |
|  | n71 | 695.5 |  | 5 |  | 25 |  | 649.5 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3305 |  | 8 |  | IMD3 |  |  |
| DC_3A_n1A-n28A DC_3C_n1A-n28A | 3 | 1780 |  | 5 |  | 25 |  | 1875 |  | N/A |  | N/A |  |  |
|  | n28 | 710.5 |  | 5 |  | 25 |  | 765.5 |  | N/A |  | N/A |  |  |
|  | n1 | N/A |  | 5 |  | N/A |  | 2139 |  | 11.0 |  | IMD4 |  |  |
| DC_3A_n1A-n40A | n1 | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | 3 | 1735 |  | 5 |  | 25 |  | 1830 |  | N/A |  | N/A |  |  |
|  | 40 | N/A |  | 5 |  | N/A |  | 2380 |  | 8.0 |  | IMD5 |  |  |
| DC_3A_n1A-n41A | 3 | 1712.5 |  | 5 |  | 25 |  | 1807.5 |  | N/A |  | N/A |  |  |
|  | n1 | 1977.5 |  | 5 |  | 25 |  | 2167.5 |  | N/A |  | N/A |  |  |
|  | n41 | N/A |  | 5 |  | N/A |  | 2507.5 |  | 5.0 |  | IMD5 |  |  |
| DC_3A_n1A-n75A | n75 | N/A |  | 5 |  | N/A |  | 1480 |  | 15.2 |  | IMD34,19 |  |  |
| DC_3C_n1A-n75A | n1 | 1960 |  | 5 |  | 25 |  | 2150 |  | N/A |  | N/A |  |  |
|  | 3 | 1720 |  | 5 |  | 25 |  | 1815 |  | N/A |  | N/A |  |  |
| DC_3A_n1A-n77A | 3 | 1750 |  | 5 |  | 25 |  | 1845 |  | N/A |  | N/A |  |  |
|  | n1 | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 3700 |  | 28.4 |  | IMD2 |  |  |
|  | 3 | 1775 |  | 5 |  | 25 |  | 1870 |  | N/A |  | N/A |  |  |
|  | n1 | N/A |  | 5 |  | N/A |  | 2140 |  | 31.0 |  | IMD2 |  |  |
|  | n77 | 3915 |  | 10 |  | 50 |  | 3915 |  | N/A |  | N/A |  |  |
| DC_3A_n1A-n78A DC_3C_n1A-n78A DC_3A-3A_n1A-n78A | 3 | 1750 |  | 5 |  | 25 |  | 1845 |  | N/A |  | N/A |  |  |
|  | n1 | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3700 |  | 28.4 |  | IMD2 |  |  |
|  | 3 | 1770 |  | 5 |  | 25 |  | 1865 |  | N/A |  | N/A |  |  |
|  | n1 | N/A |  | 5 |  | N/A |  | 2130 |  | 3.5 |  | IMD5 |  |  |
|  | n78 | 3720 |  | 10 |  | 50 |  | 3720 |  | N/A |  | N/A |  |  |
| DC_3A_n1A-n79A | 3 | 1720 |  | 5 |  | 25 |  | 1815 |  | N/A |  | N/A |  |  |
|  | n1 | 1930 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
|  | n79 | 4950 |  | 40 |  | 216 |  | 4950 |  | 4.7 |  | IMD5 |  |  |
|  | 3 | 1750 |  | 5 |  | 25 |  | 1845 |  | N/A |  | N/A |  |  |
|  | n1 | 1950 |  | 40 |  | 216 |  | 2140 |  | 3.6 |  | IMD5 |  |  |
|  | n79 | 4860 |  | 5 |  | 25 |  | 4860 |  | N/A |  | N/A |  |  |
| DC_(n)3AA-n8A | n8 | 897.5 |  | 5 |  | 25 |  | 942.5 |  | N/A |  | N/A |  |  |
|  | 3 | N/A |  | 5 |  | N/A |  | 1837.5 |  | 4.5 |  | IMD5 |  |  |
|  | n3 | 1747.5 |  | 5 |  | 25 |  | 1842.5 |  | 6.4 |  | IMD5 |  |  |
| DC_3A_n3A-n41A | 3 | 1725 |  | 5 |  | 25 |  | 1820 |  | N/A |  | N/A |  |  |
|  | n3 | N/A |  | 5 |  | N/A |  | 1865 |  | 8.2 |  | IMD4 |  |  |
|  | n41 | 2657.5 |  | 5 |  | 25 |  | 2657.5 |  | N/A |  | N/A |  |  |
| DC_(n)3AA-n78A | 3 | 1740 |  | 5 |  | 25 |  | 1835 |  | 31.9 |  | IMD24 |  |  |
| DC_(n)3AA-n78(2A) | n3 | N/A |  | 5 |  | N/A |  | 1840 |  | [28.9] |  | IMD24 |  |  |
|  | n78 | 3575 |  | 10 |  | 50 |  | 3575 |  | N/A |  | N/A |  |  |
| DC_3A-5A_n28A | 3 | N/A |  | 5 |  | N/A |  | 1829.5 |  | 8.7 |  | IMD4 |  |  |
|  | 5 | 845 |  | 5 |  | 25 |  | 890 |  | N/A |  | N/A |  |  |
|  | n28 | 705.5 |  | 5 |  | 25 |  | 760.5 |  | N/A |  | N/A |  |  |
| DC_3A-5A_n77A DC_3A-5A_n77(2A) DC_3A-5A_n77(3A) | 3 | N/A |  | 5 |  | N/A |  | 1820 |  | 17.3 |  | IMD3 |  |  |
|  | 5 | 845 |  | 5 |  | 25 |  | 804 |  | N/A |  | N/A |  |  |
|  | n77 | 3510 |  | 10 |  | 50 |  | 3510 |  | N/A |  | N/A |  |  |
| DC_3A-5A_n78A DC_3A-5A_n78(A-C) | 3 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | IMD3 |  |  |
|  | 5 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
| DC_3A_n5A-n78A DC_3C_n5A-n78A | 3 | 1730 |  | 5 |  | 25 |  | 1825 |  | N/A |  | N/A |  |  |
|  | n5 | 845 |  | 5 |  | 25 |  | 890 |  | N/A |  | N/A |  |  |
|  | n78 | 3420 |  | 10 |  | 52 |  | 3420 |  | 16.1 |  | IMD3 |  |  |
| DC_3A-5A_n79A | 3 | 1775 |  | 5 |  | 25 |  | 1870 |  | N/A |  | N/A |  |  |
|  | 5 | N/A |  | 5 |  | N/A |  | 885 |  | 18.5 |  | IMD3 |  |  |
|  | n79 | 4435 |  | 40 |  | 216 |  | 4435 |  | N/A |  | N/A |  |  |
|  | 3 | N/A |  | 5 |  | N/A |  | 1877.5 |  | 0.2 |  | IMD4 |  |  |
|  | 5 | 842.5 |  | 5 |  | 25 |  | 887.5 |  | N/A |  | N/A |  |  |
|  | n79 | 4420 |  | 40 |  | 216 |  | 4420 |  | N/A |  | N/A |  |  |
| DC_3A-7A_n5A | 3 | 1780 |  | 10 |  | 50 |  | 1875 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 10 |  | N/A |  | 2625 |  | 30.0 |  | IMD21 |  |  |
|  | n5 | 845 |  | 5 |  | 25 |  | 890 |  | N/A |  | N/A |  |  |
| DC_3A-(n)7AA DC_3C-(n)7AA | 3 | 1730 |  | 5 |  | 25 |  | 1825 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 5 |  | N/A |  | 2647.5 |  | 6.9 |  | IMD4 |  |  |
|  | n7 | 2535 |  | 10 |  | 50 |  | 2655 |  | 10.2 |  | IMD4 |  |  |
| DC_3A-7A_n8A | 3 | 1780 |  | 5 |  | 25 |  | 1875 |  | N/A |  | N/A |  |  |
|  | n8 | 890 |  | 5 |  | 25 |  | 935 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 10 |  | N/A |  | 2670 |  | 29.0 |  | IMD2 IMD33 |  |  |
| DC_3A-7A_n26A | 3 | 1780 |  | 10 |  | 50 |  | 1875 |  | N/A |  | N/A |  |  |
| DC_3A-7C_n26A | 7 | N/A |  | 10 |  | N/A |  | 2625 |  | 30.0 |  | IMD2 |  |  |
| DC_3C-7A_n26A DC_3C-7C_n26A | n26 | 845 |  | 5 |  | 25 |  | 890 |  | N/A |  | N/A |  |  |
|  | 3 | 1760 |  | 5 |  | 25 |  | 1855 |  | N/A |  | N/A |  |  |
|  | 7 | 2555 |  | 10 |  | N/A |  | 2675 |  | 16.9 |  | IMD319 |  |  |
|  | n26 | 845 |  | 5 |  | 25 |  | 890 |  | N/A |  | N/A |  |  |
| DC_3A-7A_n28A DC_3A-7C_n28A DC_3C-7A_n28A DC_3C-7C_n28A | 3 | 1712.5 |  | 5 |  | 25 |  | 1807.5 |  | N/A |  | N/A |  |  |
| DC_3A-7A-7A_n28A | n28 | 743 |  | 5 |  | 25 |  | 798 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 10 |  | N/A |  | 2682 |  | 16.9 |  | IMD3 |  |  |
|  | 7 | 2543 |  | 10 |  | 50 |  | 2663 |  | N/A |  | N/A |  |  |
|  | n28 | 710.5 |  | 5 |  | 25 |  | 765.5 |  | N/A |  | N/A |  |  |
|  | 3 | N/A |  | 5 |  | N/A |  | 1832.5 |  | 26.0 |  | IMD2 |  |  |
| DC_3A-18A_n3A | 3 | N/A |  | 5 |  | N/A |  | 1814 |  | 4 |  | IMD4 ¦2*fn3-2*fB18¦ |  |  |
|  | 18 | 823 |  | 5 |  | 25 |  | 868 |  | N/A |  | N/A |  |  |
|  | n3 | 1730 |  | 5 |  | 25 |  | 1825 |  | N/A |  | N/A |  |  |
| DC_3-18_n41 | 18 | N/A |  | 5 |  | N/A |  | 865 |  | 28.9 |  | IMD2 |  |  |
|  | 3 | 1765 |  | 5 |  | 25 |  | 1860 |  | N/A |  | N/A |  |  |
|  | n41 | 2630 |  | 10 |  | 50 |  | 2630 |  | N/A |  | N/A |  |  |
|  | 18 | N/A |  | 5 |  | N/A |  | 865 |  | 19.0 |  | IMD3 |  |  |
|  | 3 | 1725 |  | 5 |  | 25 |  | 1820 |  | N/A |  | N/A |  |  |
|  | n41 | 2585 |  | 5 |  | 25 |  | 2585 |  | N/A |  | N/A |  |  |
|  | 3 | N/A |  | 5 |  | N/A |  | 1850 |  | 28.8 |  | IMD2 |  |  |
|  | n41 | 2670 |  | 10 |  | 50 |  | 2670 |  | N/A |  | N/A |  |  |
|  | 18 | 820 |  | 5 |  | 25 |  | 865 |  | MSD |  | N/A |  |  |
| DC_3A-18A_n77A DC_3A-18A_n77(2A) DC_3A-18A_n78A DC_3A-18A_n78(2A) | 3 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | IMD3 |  |  |
|  | 18 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
|  | n77, n78 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
| DC_3A-19A_n77A DC_3A-19A_n78A | 3 | N/A |  | 5 |  | N/A |  | 1850 |  | 17.3 |  | IMD3 |  |  |
|  | 19 | 835 |  | 5 |  | 25 |  | 880 |  | N/A |  | N/A |  |  |
|  | n77, n78 | 3520 |  | 10 |  | 50 |  | 3520 |  | N/A |  | N/A |  |  |
| DC_3A_n7A-n28A | 3 | 1747 |  | 5 |  | 25 |  | 1842 |  | N/A |  | N/A |  |  |
| DC_3C_n7A-n28A | n7 | 2543 |  | 5 |  | 25 |  | 2663 |  | N/A |  | N/A |  |  |
|  | n28 | N/A |  | 5 |  | N/A |  | 796.0 |  | 20.0 |  | IMD2 |  |  |
|  | 3 | 1712.5 |  | 5 |  | 25 |  | 1807.5 |  | N/A |  | N/A |  |  |
|  | n7 | N/A |  | 5 |  | N/A |  | 2682 |  | 17.0 |  | IMD3 |  |  |
|  | n28 | 743 |  | 5 |  | 25 |  | 798 |  | N/A |  | N/A |  |  |
| DC_3A-7A_n40A | 3 | N/A |  | 5 |  | N/A |  | 1866.6 |  | 3.4 |  | IMD5 |  |  |
| DC_3A-7A-7A_n40A | 7 | 2530 |  | 5 |  | 25 |  | 2650 |  | N/A |  | N/A |  |  |
|  | n40 | 2310 |  | 5 |  | 25 |  | 2310 |  | N/A |  | N/A |  |  |
| DC_3A-7A_n77A | 3 | N/A |  | 5 |  | N/A |  | 1820 |  | 17.6 |  | IMD3 |  |  |
| DC_3A-7A_n77(2A) DC_3A-7A_n77(3A) | 7 | 2565 |  | 5 |  | 25 |  | 2685 |  | N/A |  | N/A |  |  |
| DC_3A-7A-7A_n77(2A) | n77 | 3310 |  | 10 |  | 50 |  | 3310 |  | N/A |  | N/A |  |  |
| DC_3A-7A-7A_n77(3A) | 3 | N/A |  | 5 |  | N/A |  | 1820 |  | 8.6 |  | IMD4 |  |  |
|  | 7 | 2565 |  | 5 |  | 25 |  | 2685 |  | N/A |  | N/A |  |  |
|  | n77 | 3475 |  | 10 |  | 50 |  | 3475 |  | N/A |  | N/A |  |  |
|  | 3 | 1715 |  | 5 |  | 25 |  | 1810 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 5 |  | N/A |  | 2670 |  | 5.2 |  | IMD5 |  |  |
|  | n77 | 4190 |  | 10 |  | 50 |  | 4190 |  | N/A |  | N/A |  |  |
|  | 3 | 1720 |  | 5 |  | 25 |  | 1815 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 5 |  | N/A |  | 2640 |  | 3.4 |  | IMD5 |  |  |
|  | n77 | 3900 |  | 10 |  | 50 |  | 3900 |  | N/A |  | N/A |  |  |
| DC_3A-7A_n78A DC_3C-7A_n78A DC_3C-7C_n78A DC_3A-3A-7A_n78A DC_3A-3A-7A-7A_n78A DC_3A-7A_SUL_n78A-n80A DC_3C-7A_SUL_n78A-n80A DC_3A-7A_n78(2A) DC_3C-7A_n78(2A) DC_3A-7C_n78(2A) DC_3C-7C_n78(2A) DC_3A-7A_n78C DC_3A-7A_n78(A-C) DC_3A-7A-7A_n78C | 3 | N/A |  | 5 |  | N/A |  | 1820 |  | 17.6 |  | IMD3 |  |  |
| DC_3A-7A-7A_n78(A-C) | 7 | 2565 |  | 5 |  | 25 |  | 2685 |  | N/A |  | N/A |  |  |
|  | n78 | 3310 |  | 10 |  | 50 |  | 3310 |  | N/A |  | N/A |  |  |
|  | 3 | N/A |  | 5 |  | N/A |  | 1820 |  | 8.6 |  | IMD4 |  |  |
|  | 7 | 2565 |  | 5 |  | 25 |  | 2685 |  | N/A |  | N/A |  |  |
|  | n78 | 3475 |  | 10 |  | 50 |  | 3475 |  | N/A |  | N/A |  |  |
| DC_3A-7A_n79A | 3 | 1770 |  | 5 |  | 25 |  | 1865 |  | N/A |  | N/A |  |  |
| DC_3A-3A-7A_n79A | n79 | 4440 |  | 10 |  | 50 |  | 4440 |  | N/A |  | N/A |  |  |
| DC_3A-7A-7A_n79A | 7 | N/A |  | 5 |  | N/A |  | 2670 |  | 30.2 |  | IMD2 |  |  |
| DC_3A-3A-7A-7A_n79A | 3 | 1770 |  | 5 |  | 25 |  | 1865 |  | N/A |  | N/A |  |  |
|  | n79 | 4440 |  | 10 |  | 50 |  | 4440 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 5 |  | N/A |  | 2640 |  | 5.0 |  | IMD5 |  |  |
|  | 7 | 2565 |  | 5 |  | 25 |  | 2685 |  | N/A |  | N/A |  |  |
|  | n79 | 4420 |  | 10 |  | 50 |  | 4420 |  | N/A |  | N/A |  |  |
|  | 3 | N/A |  | 5 |  | N/A |  | 1855 |  | 29.4 |  | IMD2 |  |  |
|  | 7 | 2550 |  | 5 |  | 25 |  | 2670 |  | N/A |  | N/A |  |  |
|  | n79 | 4745 |  | 10 |  | 50 |  | 4745 |  | N/A |  | N/A |  |  |
|  | 3 | N/A |  | 5 |  | N/A |  | 1840 |  | 4.8 |  | IMD5 |  |  |
| DC_3A-7A_n105A | 3 | N/A |  | 5 |  | N/A |  | 1875 |  | 16.5 |  | IMD2 |  |  |
|  | 7 | 2550 |  | 5 |  | 25 |  | 2670 |  | N/A |  | N/A |  |  |
|  | n105 | 675 |  | 5 |  | 25 |  | 624 |  | N/A |  | N/A |  |  |
| DC_3A-8A_n7A | 3 | 1735 |  | 5 |  | 25 |  | 1830 |  | N/A |  | N/A |  |  |
|  | n7 | 2530 |  | 10 |  | 50 |  | 2650 |  | N/A |  | N/A |  |  |
|  | 8 | N/A |  | 5 |  | N/A |  | 940 |  | 18.0 |  | IMD3 |  |  |
| DC_3A-8A_n40A | 3 | N/A |  | 5 |  | N/A |  | 1874 |  | 4 |  | IMD5 |  |  |
|  | 8 | 912 |  | 5 |  | 25 |  | 957 |  | N/A |  | N/A |  |  |
|  | n40 | 2305 |  | 5 |  | 25 |  | 2305 |  | N/A |  | N/A |  |  |
| DC_3A-8A_n41A | 3 | 1725 |  | 5 |  | 25 |  | 1820 |  | N/A |  | N/A |  |  |
|  | 8 | N/A |  | 5 |  | N/A |  | 945 |  | 26.0 |  | IMD215 |  |  |
|  | n41 | 2670 |  | 10 |  | 50 |  | 2670 |  | N/A |  | N/A |  |  |
|  | 3 | N/A |  | 5 |  | N/A |  | 1807.5 |  | 25 |  | IMD2x |  |  |
|  | 8 | 882.5 |  | 5 |  | 25 |  | 927.5 |  | N/A |  | N/A |  |  |
|  | n41 | 2685 |  | 10 |  | 50 |  | 2685 |  | N/A |  | N/A |  |  |
| DC_3A_n8A-n41A | 3 | 1722.5 |  | 5 |  | 25 |  | 1817.5 |  | N/A |  | N/A |  |  |
|  | n8 | 887.5 |  | 5 |  | 25 |  | 932.5 |  | N/A |  | N/A |  |  |
|  | n41 | N/A |  | 10 |  | N/A |  | 2610 |  | 28.0 |  | IMD216 |  |  |
|  | 3 | 1725 |  | 5 |  | 25 |  | 1820 |  | N/A |  | N/A |  |  |
|  | n8 | N/A |  | 5 |  | N/A |  | 945 |  | 26.0 |  | IMD216 |  |  |
|  | n41 | 2516 |  | 10 |  | 50 |  | 2516 |  | N/A |  | N/A |  |  |
| DC_3A-8A_n77A DC_3A-8A_n77(2A) DC_3A-8A_n77(3A) DC_3C-8A_n77A DC_3C-8A_n77(2A) | 3 | 1715 |  | 5 |  | 25 |  | 1810 |  | N/A |  | N/A |  |  |
| DC_3A-8B_n77A | n77 | 4190 |  | 10 |  | 50 |  | 4190 |  | N/A |  | N/A |  |  |
|  | 8 | N/A |  | 5 |  | N/A |  | 955 |  | 9.7 |  | IMD4 |  |  |
| DC_3A-8A_n77A DC_3A-8A_n77(2A) DC_3A-8A_n77(3A) DC_3C-8A_n77A DC_3C-8A_n77(2A) | 8 | 910 |  | 5 |  | 25 |  | 955 |  | N/A |  | N/A |  |  |
| DC_3A-8B_n77A | n77 | 3640 |  | 10 |  | 50 |  | 3640 |  | N/A |  | N/A |  |  |
|  | 3 | N/A |  | 5 |  | N/A |  | 1820 |  | 16.5 |  | IMD3 |  |  |
| DC_3A-8A_n78A DC_3A-3A-8A_n78A | 8 | 910 |  | 5 |  | 25 |  | 955 |  | N/A |  | N/A |  |  |
| DC_3A-8B_n78A DC_3A-3A-8B_n78A DC_3A-8A_n78(2A) DC_3C-8A_n78(2A) | n78 | 3640 |  | 10 |  | 50 |  | 3640 |  | N/A |  | N/A |  |  |
|  | 3 | N/A |  | 5 |  | N/A |  | 1820 |  | 16.5 |  | IMD319 |  |  |
| DC_3A_n8A-n78A | 3 | 1740 |  | 5 |  | 25 |  | 1835 |  | N/A |  | N/A |  |  |
|  | n8 | 900 |  | 5 |  | 25 |  | 945 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3540 |  | 16.3 |  | IMD3 |  |  |
| DC_3A-8A_n79A | 3 | 1755 |  | 5 |  | 25 |  | 1850 |  | N/A |  | N/A |  |  |
|  | n79 | 4465 |  | 40 |  | 216 |  | 4465 |  | N/A |  | N/A |  |  |
|  | 8 | N/A |  | 5 |  | N/A |  | 955 |  | 15.3 |  | IMD3 |  |  |
| DC_3A-8A_n79A | 8 | 910 |  | 5 |  | 25 |  | 955 |  | N/A |  | N/A |  |  |
|  | n79 | 4580 |  | 40 |  | 216 |  | 4580 |  | N/A |  | N/A |  |  |
|  | 3 | N/A |  | 5 |  | N/A |  | 1850 |  | 8.8 |  | IMD4 |  |  |
| DC_3A_n7A-n78A DC_3A_n7B-n78A DC_3C_n7A-n78A DC_3C_n7B-n78A | 3 | 1730 |  | 5 |  | 25 |  | 1825 |  | N/A |  | N/A |  |  |
| DC_3A_n7A-n78(2A) | n7 | 2560 |  | 5 |  | 25 |  | 2680 |  | N/A |  | N/A |  |  |
| DC_3C_n7A-n78(2A) | n78 | N/A |  | 10 |  | N/A |  | 3390 |  | 16.1 |  | IMD3 |  |  |
| DC_3A-11A_n77A DC_3A-11A_n77(2A) | 3 | 1720 |  | 5 |  | 25 |  | 1815 |  | N/A |  | N/A |  |  |
|  | n77 | 3675 |  | 10 |  | 50 |  | 3675 |  | N/A |  | N/A |  |  |
|  | 11 | N/A |  | 5 |  | N/A |  | 1491 |  | 8.8 |  | IMD4 |  |  |
|  | 11 | 1435.4 |  | 5 |  | 25 |  | 1483.4 |  | N/A |  | N/A |  |  |
|  | n77 | 3905 |  | 10 |  | 50 |  | 3905 |  | N/A |  | N/A |  |  |
|  | 3 | N/A |  | 5 |  | N/A |  | 1848 |  | 3.4 |  | IMD57 |  |  |
| DC_3A-11A_n79A | 3 | 1720 |  | 5 |  | 25 |  | 1815 |  | N/A |  | N/A |  |  |
|  | 11 | N/A |  | 5 |  | 25 |  | 1480 |  | 16.1 |  | IMD3 |  |  |
|  | n79 | 4920 |  | 40 |  | 216 |  | 4920 |  | N/A |  | N/A |  |  |
| DC_3A-19A_n79A | 3 | 1775 |  | 5 |  | 25 |  | 1870 |  | N/A |  | N/A |  |  |
|  | 19 | N/A |  | 5 |  | N/A |  | 885 |  | 18.5 |  | IMD3 |  |  |
|  | n79 | 4435 |  | 40 |  | 216 |  | 4435 |  | N/A |  | N/A |  |  |
|  | 3 | N/A |  | 5 |  | N/A |  | 1877.5 |  | 5.5 |  | IMD4 |  |  |
|  | 19 | 842.5 |  | 5 |  | 25 |  | 887.5 |  | N/A |  | N/A |  |  |
|  | n79 | 4420 |  | 40 |  | 216 |  | 4420 |  | N/A |  | N/A |  |  |
| DC_3A-20A_n3A | 3 | N/A |  | 5 |  | N/A |  | 1870 |  | 4 |  | IMD4 |  |  |
|  | 20 | 835 |  | 5 |  | 25 |  | 794 |  | N/A |  | N/A |  |  |
|  | n3 | 1765 |  | 5 |  | 25 |  | 1860 |  | N/A |  | N/A |  |  |
| DC_3A-20A_n7A DC_3C-20A_n7A | 3 | 1737 |  | 5 |  | 25 |  | 1832 |  | N/A |  | N/A |  |  |
|  | 20 | N/A |  | 10 |  | N/A |  | 806 |  | 10.5 |  | IMD2 |  |  |
|  | n7 | 2543 |  | 10 |  | 50 |  | 2663 |  | N/A |  | N/A |  |  |
| DC_3A-20A_n8A | 3 | 1720 |  | 5 |  | 25 |  | 1815 |  | N/A |  | N/A |  |  |
|  | n8 | 910 |  | 5 |  | 25 |  | 955 |  | N/A |  | N/A |  |  |
|  | 20 | N/A |  | 5 |  | N/A |  | 810 |  | 27 |  | IMD2 |  |  |
|  | 3 | N/A |  | 5 |  | N/A |  | 1860 |  | 14.5 |  | IMD4 |  |  |
|  | n8 | 900 |  | 5 |  | 25 |  | 945 |  | N/A |  | N/A |  |  |
|  | 20 | 840 |  | 5 |  | 25 |  | 799 |  | N/A |  | N/A |  |  |
| DC_3A-20A_n28A DC_3C-20A_n28A | 20 | 852 |  | 5 |  | 25 |  | 811 |  | N/A |  | N/A |  |  |
|  | n28 | 728 |  | 5 |  | 25 |  | 783 |  | N/A |  | N/A |  |  |
|  | 3 | N/A |  | 5 |  | N/A |  | 1828 |  | 9.4 |  | IMD4 |  |  |
| DC_3A-20A_n38A | 3 | 1779 |  | 5 |  | 25 |  | 1874 |  | N/A |  | N/A |  |  |
|  | 20 | N/A |  | 10 |  | N/A |  | 811 |  | 26.0 |  | IMD21 |  |  |
|  | n38 | 2590 |  | 10 |  | 50 |  | 2590 |  | N/A |  | N/A |  |  |
| DC_3A-20A_n41A DC_3C-20A_n41A | 3 | N/A |  | 5 |  | N/A |  | 1839 |  | 26.0 |  | IMD2 |  |  |
|  | n41 | 2680 |  | 10 |  | 50 |  | 2680 |  | N/A |  | N/A |  |  |
|  | 20 | 841 |  | 10 |  | 50 |  | 800 |  | N/A |  | N/A |  |  |
|  | 3 | 1779 |  | 5 |  | 25 |  | 1874 |  | N/A |  | N/A |  |  |
|  | n41 | 2590 |  | 10 |  | 50 |  | 2590 |  | N/A |  | N/A |  |  |
|  | 20 | N/A |  | 10 |  | N/A |  | 811 |  | 26.0 |  | IMD2 |  |  |
|  | 3 | 1730 |  | 5 |  | 25 |  | 1825 |  | N/A |  | N/A |  |  |
|  | n41 | 2660 |  | 10 |  | 50 |  | 2660 |  | N/A |  | N/A |  |  |
|  | 20 | N/A |  | 5 |  | N/A |  | 800 |  | 12.5 |  | IMD3 |  |  |
| DC_3_n20-n67 | 3 | 1765 |  | 5 |  | 25 |  | 1860 |  | N/A |  | N/A |  |  |
|  | n20 | 837 |  | 5 |  | 25 |  | 796 |  | N/A |  | N/A |  |  |
|  | n67 | N/A |  | 5 |  | N/A |  | 746 |  | 10.1 |  | IMD4 |  |  |
| DC_3A_20A_SUL_n78A-n80A DC_3C_20A_SUL_n78A-n80A | 3 | N/A |  | 5 |  | N/A |  | 1820 |  | 17.3 |  | IMD3 |  |  |
|  | 20 | 845 |  | 5 |  | 25 |  | 804 |  | N/A |  | N/A |  |  |
|  | n78 | 3510 |  | 10 |  | 50 |  | 3510 |  | N/A |  | N/A |  |  |
| DC_3A_n20A-n78A | 3 | 1730 |  | 5 |  | 25 |  | 1825 |  | N/A |  | N/A |  |  |
|  | n20 | 845 |  | 5 |  | 25 |  | 804 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3420 |  | 16.1 |  | IMD3 |  |  |
| DC_3A-20A_n78A DC_3C-20A_n78A DC_3A-20A_n78(2A) | 3 | N/A |  | 5 |  | N/A |  | 1820 |  | 17.3 |  | IMD3 |  |  |
|  | 20 | 845 |  | 5 |  | 25 |  | 804 |  | N/A |  | N/A |  |  |
|  | n78 | 3510 |  | 10 |  | 50 |  | 3510 |  | N/A |  | N/A |  |  |
| DC_3A-21A_n77A DC_3A-21A_n78A | 3 | 1767.5 |  | 5 |  | 25 |  | 1862.5 |  | N/A |  | N/A |  |  |
|  | 21 | N/A |  | 5 |  | N/A |  | 1507.5 |  | 8.8 |  | IMD4 |  |  |
|  | n77, n78 | 3795 |  | 10 |  | 50 |  | 3795 |  | N/A |  | N/A |  |  |
|  | 3 | N/A |  | 5 |  | N/A |  | 1862.5 |  | 30.8 |  | IMD2 |  |  |
|  | 21 | 1459.5 |  | 5 |  | 25 |  | 1507.5 |  | N/A |  | N/A |  |  |
|  | n77, n78 | 3322 |  | 10 |  | 50 |  | 3322 |  | N/A |  | N/A |  |  |
| DC_3A-21A_n77A | 3 | N/A |  | 5 |  | N/A |  | 1866.6 |  | 3.4 |  | IMD5 |  |  |
|  | 21 | 1450.4 |  | 5 |  | 25 |  | 1498.4 |  | N/A |  | N/A |  |  |
|  | n77 | 3935 |  | 10 |  | 50 |  | 3935 |  | N/A |  | N/A |  |  |
| DC_3A-21A_n79A | 3 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
|  | 21 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | IMD3 |  |  |
|  | n79 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
|  | 3 | N/A |  | 5 |  | N/A |  | 1869.2 |  | 17.8 |  | IMD3 |  |  |
|  | 21 | 1450.4 |  | 5 |  | 25 |  | 1498.4 |  | N/A |  | N/A |  |  |
|  | n79 | 4770 |  | 40 |  | 216 |  | 4770 |  | N/A |  | N/A |  |  |
| DC_3A-26A_n78A DC_3C-26A_n78A | 3 | N/A |  | 5 |  | N/A |  | 1862 |  | 15.7 |  | IMD3 |  |  |
|  | 26 | 839 |  | 5 |  | 25 |  | 884 |  | N/A |  | N/A |  |  |
|  | n78 | 3540 |  | 10 |  | 50 |  | 3540 |  | N/A |  | N/A |  |  |
| DC_3A-28A_n1A DC_3C-28A_n1A | 3 | N/A |  | 5 |  | N/A |  | 1820 |  | 4 |  | IMD5 |  |  |
|  | 28 | 710 |  | 5 |  | 25 |  | 765 |  | N/A |  | N/A |  |  |
|  | n1 | 1975 |  | 5 |  | 25 |  | 2165 |  | N/A |  | N/A |  |  |
| DC_3A-28A_n5A DC_3C-28A_n5A | 3 | N/A |  | 5 |  | N/A |  | 1830 |  | 8.7 |  | IMD4 |  |  |
|  | 28 | 705 |  | 5 |  | 25 |  | 798 |  | N/A |  | N/A |  |  |
|  | n5 | 845 |  | 5 |  | 25 |  | 874 |  | N/A |  | N/A |  |  |
|  | 3 | 1750 |  | 5 |  | 25 |  | 1845 |  | N/A |  | N/A |  |  |
|  | 28 | N/A |  | 5 |  | N/A |  | 785 |  | 9.4 |  | IMD4 |  |  |
|  | n5 | 845 |  | 5 |  | 25 |  | 874 |  | N/A |  | N/A |  |  |
| DC_3A-28A_n7A DC_3C-28A_n7A DC_3A-3A-28A_n7A DC_3A-28A_n7B DC_3C-28A_n7B DC_3A-3A-28A_n7B | 3 | N/A |  | 5 |  | N/A |  | 1832.5 |  | 26.0 |  | IMD2 |  |  |
|  | 28 | 710.5 |  | 5 |  | 25 |  | 765.5 |  | N/A |  | N/A |  |  |
|  | n7 | 2543 |  | 10 |  | 50 |  | 2663 |  | N/A |  | N/A |  |  |
|  | 3 | 1747 |  | 5 |  | 25 |  | 1842 |  | N/A |  | N/A |  |  |
|  | 28 | N/A |  | 5 |  | N/A |  | 796.0 |  | 20.0 |  | IMD2 |  |  |
|  | n7 | 2543 |  | 5 |  | 25 |  | 2663 |  | N/A |  | N/A |  |  |
| DC_3A-28A_n77A | 3 | 1712.5 |  | 5 |  | 25 |  | 1807.5 |  | N/A |  | N/A |  |  |
|  | 28 | N/A |  | 5 |  | N/A |  | 770 |  | 15.3 |  | IMD3 |  |  |
|  | n77 | 4195 |  | 10 |  | 50 |  | 4195 |  | N/A |  | N/A |  |  |
|  | 3 | N/A |  | 5 |  | N/A |  | 1850 |  | 17.0 |  | IMD3 |  |  |
|  | 28 | 735 |  | 5 |  | 25 |  | 790 |  | N/A |  | N/A |  |  |
|  | n77 | 3320 |  | 10 |  | 50 |  | 3320 |  | N/A |  | N/A |  |  |
| DC_3A_n28A-n75A DC_3C_n28A-n75A | B3 | 1780 |  | 5 |  | 25 |  | 1875 |  | N/A |  | N/A |  |  |
|  | n28 | 708 |  | 5 |  | 25 |  | 763 |  | N/A |  | N/A |  |  |
|  | n75 | N/A |  | - |  | N/A |  | 1436 |  | 3.3 |  | IMD5 |  |  |
| DC_3A_n28A-n77A | 3 | 1720 |  | 5 |  | 25 |  | 1815 |  | N/A |  | N/A |  |  |
|  | 28 | 733 |  | 5 |  | 25 |  | 788 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 4173 |  | 15.9 |  | IMD3 |  |  |
|  | 3 | 1712.5 |  | 5 |  | 25 |  | 1807.5 |  | N/A |  | N/A |  |  |
|  | 28 | N/A |  | 5 |  | N/A |  | 770 |  | 15.3 |  | IMD3 |  |  |
|  | n77 | 4195 |  | 10 |  | 50 |  | 4195 |  | N/A |  | N/A |  |  |
| DC_3A-28A_n38A | 3 | N/A |  | 5 |  | N/A |  | 1870 |  | 26.0 |  | IMD2 |  |  |
|  | 28 | 710 |  | 5 |  | 25 |  | 765 |  | N/A |  | N/A |  |  |
|  | n38 | 2580 |  | 5 |  | 25 |  | 2580 |  | N/A |  | N/A |  |  |
|  | 3 | 1780 |  | 5 |  | 25 |  | 1875 |  | N/A |  | N/A |  |  |
|  | 28 | N/A |  | 5 |  | N/A |  | 800 |  | 20.0 |  | IMD21 |  |  |
|  | n38 | 2580 |  | 5 |  | 25 |  | 2580 |  | N/A |  | N/A |  |  |
| DC_3A-28A_n41A | 3 | 1720 |  | 5 |  | 25 |  | 1815 |  | N/A |  | N/A |  |  |
|  | n41 | 2510 |  | 5 |  | 25 |  | 2510 |  | N/A |  | N/A |  |  |
|  | 28 | N/A |  | 5 |  | N/A |  | 790 |  | 26.0 |  | IMD21 |  |  |
|  | 3 | N/A |  | 5 |  | N/A |  | 1832.5 |  | 26.0 |  | IMD2 |  |  |
|  | n41 | 2543 |  | 10 |  | 50 |  | 2543 |  | N/A |  | N/A |  |  |
|  | 28 | 710.5 |  | 5 |  | 25 |  | 765.5 |  | N/A |  | N/A |  |  |
| DC_3A_n28A-n41A | 3 | 1720 |  | 5 |  | 25 |  | 1815 |  | N/A |  | N/A |  |  |
|  | n28 | N/A |  | 5 |  | N/A |  | 790 |  | 261 |  | IMD2 ¦fn41-fB3¦ |  |  |
|  | n41 | 2510 |  | 5 |  | 25 |  | 2510 |  | N/A |  | N/A |  |  |
|  | 3 | 1780 |  | 5 |  | 25 |  | 1875 |  | N/A |  | N/A |  |  |
|  | n28 | 738 |  | 5 |  | 25 |  | 793 |  | N/A |  | N/A |  |  |
|  | n41 | N/A |  | 5 |  | N/A |  | 2518 |  | 27.4 |  | IMD2 ¦fB3+fn28¦ |  |  |
|  | 3 | 1715 |  | 5 |  | 25 |  | 1810 |  | N/A |  | N/A |  |  |
|  | n28 | 743 |  | 5 |  | 25 |  | 798 |  | N/A |  | N/A |  |  |
|  | n41 | N/A |  | 5 |  | N/A |  | 2687 |  | 15.9 |  | IMD3 ¦2*fB3-fn28¦ |  |  |
| DC_3A_n26A-n78A | 3 | 1730 |  | 5 |  | 25 |  | 1825 |  | N/A |  | N/A |  |  |
| DC_3C_n26A-n78A | n26 | 839 |  | 5 |  | 25 |  | 884 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3408 |  | 16.1 |  | IMD3 |  |  |
|  | 3 | 1730 |  | 5 |  | 25 |  | 1825 |  | N/A |  | N/A |  |  |
|  | n26 | 839 |  | 5 |  | 25 |  | 884 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3512 |  | 4.5 |  | IMD5 |  |  |
| DC_3A-28A_n78A DC_3C-28A_n78A DC_3A-3A-28A_n78A | 3 | N/A |  | 5 |  | N/A |  | 1870 |  | 17.3 |  | IMD3 |  |  |
|  | 28 | 740 |  | 5 |  | 25 |  | 760 |  | N/A |  | N/A |  |  |
|  | n78 | 3350 |  | 10 |  | 25 |  | 3350 |  | N/A |  | N/A |  |  |
| DC_3A-28A_n79A | 3 | 1770 |  | 5 |  | 25 |  | 1865 |  | N/A |  | N/A |  |  |
|  | 28 | N/A |  | 5 |  | N/A |  | 780 |  | 10.3 |  | IMD4 |  |  |
|  | n79 | 4530 |  | 40 |  | 216 |  | 4530 |  | N/A |  | N/A |  |  |
|  | 3 | N/A |  | 5 |  | N/A |  | 1870 |  | 5.7 |  | IMD5 |  |  |
|  | 28 | 725 |  | 5 |  | 25 |  | 780 |  | N/A |  | N/A |  |  |
|  | n79 | 4770 |  | 40 |  | 216 |  | 4770 |  | N/A |  | N/A |  |  |
| DC_3A_n28A-n78A DC_3C_n28A-n78A | 3 | 1750 |  | 5 |  | 25 |  | 1845 |  | N/A |  | N/A |  |  |
|  | n28 | 743 |  | 5 |  | 25 |  | 798 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3764 |  | 4.5 |  | IMD5 |  |  |
| DC_3A_n28A-n79A | 3 | 1770 |  | 5 |  | 25 |  | 1865 |  | N/A |  | N/A |  |  |
|  | n28 | N/A |  | 5 |  | N/A |  | 780 |  | 10.3 |  | IMD4 |  |  |
|  | n79 | 4530 |  | 40 |  | 216 |  | 4530 |  | N/A |  | N/A |  |  |
|  | 3 | 1770 |  | 5 |  | 25 |  | 1865 |  | N/A |  | N/A |  |  |
|  | n28 | 725 |  | 5 |  | 25 |  | 780 |  | N/A |  | N/A |  |  |
|  | n79 | N/A |  | 40 |  | N/A |  | 4585 |  | 9.4 |  | IMD44 |  |  |
| DC_3A_n40A-n77A | 3 | 1720 |  | 5 |  | 25 |  | 1815 |  | N/A |  | N/A |  |  |
| DC_3A_n40A-n77(2A) | n40 | 2350 |  | 5 |  | 25 |  | 2350 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 4070 |  | 30.3 |  | IMD2 |  |  |
|  | 3 | 1730 |  | 5 |  | 25 |  | 1825 |  | N/A |  | N/A |  |  |
|  | n40 | 2360 |  | 5 |  | 25 |  | 2360 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 3620 |  | 4.8 |  | IMD5 |  |  |
|  | 3 | 1745 |  | 5 |  | 25 |  | 1840 |  | N/A |  | N/A |  |  |
|  | n40 | N/A |  | 5 |  | N/A |  | 2355 |  | 29,2 |  | IMD2 |  |  |
|  | n77 | 4100 |  | 10 |  | 50 |  | 4100 |  | N/A |  | N/A |  |  |
|  | 3 | 1720 |  | 5 |  | 25 |  | 1815 |  | N/A |  | N/A |  |  |
|  | n40 | N/A |  | 5 |  | N/A |  | 2360 |  | 4.4 |  | IMD5 |  |  |
|  | n77 | 3760 |  | 10 |  | 50 |  | 3760 |  | N/A |  | N/A |  |  |
| DC_3A_SUL_n77A-n84A | 3 | 1782.5 |  | 5 |  | 25 |  | 1877.5 |  | N/A |  | N/A |  |  |
|  | n84 | 1922.5 |  | 5 |  | 25 |  |  |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 3425 |  | 13.0 |  | IMD4 |  |  |
| DC_3A_n40A-n78A | 3 | 1730 |  | 5 |  | 25 |  | 1825 |  | N/A |  | N/A |  |  |
| DC_3A_n40A-n78C | n40 | 2360 |  | 5 |  | 25 |  | 2360 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3620 |  | 4.8 |  | IMD5 |  |  |
|  | 3 | 1720 |  | 5 |  | 25 |  | 1815 |  | N/A |  | N/A |  |  |
|  | n40 | N/A |  | 5 |  | N/A |  | 2360 |  | 4.4 |  | IMD5 |  |  |
|  | n78 | 3760 |  | 10 |  | 50 |  | 3760 |  | N/A |  | N/A |  |  |
| DC_3A_n40A-n79A | 3 | 1720 |  | 5 |  | 25 |  | 1815 |  | N/A |  | N/A |  |  |
|  | n40 | 2330 |  | 5 |  | 25 |  | 2330 |  | N/A |  | N/A |  |  |
|  | n79 | N/A |  | 40 |  | N/A |  | 4550 |  | 4.7 |  | IMD5 |  |  |
|  | 3 | 1720 |  | 5 |  | 25 |  | 1815 |  | N/A |  | N/A |  |  |
|  | n40 | N/A |  | 5 |  | N/A |  | 2330 |  | 3.2 |  | IMD5 |  |  |
|  | n79 | 4550 |  | 40 |  | 216 |  | 4550 |  | N/A |  | N/A |  |  |
| DC_3_n40-n105 | 3 | 1745 |  | 5 |  | 25 |  | 1840 |  | N/A |  | N/A |  |  |
|  | n40 | 2380 |  | 10 |  | 50 |  | 2380 |  | N/A |  | N/A |  |  |
|  | n105 | N/A |  | 5 |  | N/A |  | 635 |  | 26.0 |  | IMD2 |  |  |
|  | 3 | 1777.5 |  | 5 |  | 25 |  | 1872.5 |  | N/A |  | N/A |  |  |
|  | n40 | 2350 |  | 10 |  | 50 |  | 2350 |  | N/A |  | N/A |  |  |
|  | n105 | N/A |  | 5 |  | N/A |  | 632.5 |  | 4.5 |  | IMD5 |  |  |
|  | 3 | 1720 |  | 5 |  | 25 |  | 1815 |  | N/A |  | N/A |  |  |
|  | n40 | N/A |  | 10 |  | N/A |  | 2388 |  | 26.0 |  | IMD2 |  |  |
|  | n105 | 668 |  | 5 |  | 25 |  | 617 |  | N/A |  | N/A |  |  |
| DC_3A_n41A-n79A | 3 | 1770 |  | 5 |  | 25 |  | 1865 |  | N/A |  | N/A |  |  |
|  | n41 | 2670 |  | 10 |  | 50 |  | 2670 |  | N/A |  | N/A |  |  |
|  | n79 | N/A |  | 40 |  | N/A |  | 4440 |  | 30.8 |  | IMD24 |  |  |
| DC_3A-42A_n1A DC_3A-42C_n1A | 3 | 1782.5 |  | 5 |  | 25 |  | 1877.5 |  | N/A |  | N/A |  |  |
|  | 42 | N/A |  | 5 |  | N/A |  | 3425 |  | 13.0 |  | IMD4 |  |  |
|  | n1 | 1922.5 |  | 5 |  | 25 |  | 2112.5 |  | N/A |  | N/A |  |  |
| DC_3A_n75A-n78A DC_3C_n75A-n78A DC_3A_n75A-n78(2A) | 3 | 1782.5 |  | 5 |  | 25 |  | 1877.5 |  | N/A |  | N/A |  |  |
|  | n78 | 3305 |  | 10 |  | 50 |  | 3305 |  | N/A |  | N/A |  |  |
|  | n75 | N/A |  | - |  | N/A |  | 1514.5 |  | 10.0 |  | IMD2 |  |  |
| DC_3A_n78A-n79A | 3 | 1770 |  | 5 |  | 25 |  | 1865 |  | N/A |  | N/A |  |  |
| DC_3A-3A_n78A-n79A | n78 | 3340 |  | 10 |  | 50 |  | 3340 |  | N/A |  | N/A |  |  |
|  | n79 | N/A |  | 40 |  | N/A |  | 4910 |  | 16.3 |  | IMD3 |  |  |
|  | 3 | 1770 |  | 5 |  | 25 |  | 1865 |  | N/A |  | N/A |  |  |
|  | n79 | 4510 |  | 40 |  | 216 |  | 4510 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3710 |  | 4.2 |  | IMD5 |  |  |
| DC_3A_SUL_n78A-n82A | 3 | N/A |  | 5 |  | N/A |  | 1870 |  | 4 |  | IMD4 |  |  |
|  | n82 | 840 |  | 5 |  | 25 |  |  |  | N/A |  | N/A |  |  |
| DC_3A_SUL_n78A-n84A | 3 | 1782.5 |  | 5 |  | 25 |  | 1877.5 |  | N/A |  | N/A |  |  |
|  | n84 | 1922.5 |  | 5 |  | 25 |  |  |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3425 |  | 13.0 |  | IMD4 |  |  |
| DC_3A-32A_n1A | 3 | 1720 |  | 5 |  | 25 |  | 1815 |  | N/A |  | N/A |  |  |
| DC_3C-32A_n1A | 32 | N/A |  | 5 |  | N/A |  | 1480 |  | 15.2 |  | IMD34, 19 |  |  |
|  | n1 | 1960 |  | 5 |  | 25 |  | 2150 |  | N/A |  | N/A |  |  |
| DC_3A-32A_n7A | 3 | 1775 |  | 5 |  | 25 |  | 1870 |  | N/A |  | N/A |  |  |
|  | 32 | N/A |  | 5 |  | N/A |  | 1470 |  | 10.5 |  | IMD4 |  |  |
|  | n7 | 2510 |  | 10 |  | 50 |  | 2630 |  | N/A |  | N/A |  |  |
| DC_3A-32A_n78A DC_3C-32A_n78A DC_3A-32A_n78C DC_3A-32A_n78(2A) | 3 | 1730 |  | 5 |  | 25 |  | 1825 |  | N/A |  | N/A |  |  |
|  | 32 | N/A |  | 5 |  | N/A |  | 1470 |  | 4.9 |  | IMD4 |  |  |
|  | n78 | 3720 |  | 10 |  | 50 |  | 3720 |  | N/A |  | N/A |  |  |
|  | 3 | 1775 |  | 5 |  | 25 |  | 1870 |  | N/A |  | N/A |  |  |
|  | 32 | N/A |  | 5 |  | N/A |  | 1475 |  | 0 |  | IMD5 |  |  |
|  | n78 | 3400 |  | 10 |  | 50 |  | 3400 |  | N/A |  | N/A |  |  |
| DC_3A-38A_n28A DC_3C-38A_n28A | 38 | 2575 |  | 5 |  | 25 |  | 2575 |  | N/A |  | N/A |  |  |
|  | n28 | 725 |  | 5 |  | 25 |  | 780 |  | N/A |  | N/A |  |  |
|  | 3 | N/A |  | 5 |  | N/A |  | 1850 |  | 26 |  | IMD2 |  |  |
| DC_3A-38A_n78A DC_3C-38A_n78A | 3 | N/A |  | 5 |  | N/A |  | 1830 |  | 16.4 |  | IMD35 |  |  |
|  | 38 | 2615 |  | 5 |  | 25 |  | 2615 |  | N/A |  | N/A |  |  |
|  | n78 | 3400 |  | 10 |  | 50 |  | 3400 |  | N/A |  | N/A |  |  |
| DC_3A-40A_n1A DC_3A-40C_n1A | n1 | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | 3 | 1735 |  | 5 |  | 25 |  | 1830 |  | N/A |  | N/A |  |  |
|  | 40 | N/A |  | 5 |  | N/A |  | 2380 |  | 8.0 |  | IMD5 |  |  |
| DC_3A-40A_n77A DC_3A-40C_n77A | 3 | 1720 | 5 |  | 25 |  | 1815 |  | N/A |  | N/A |  |  |
|  | 40 | 2310 | 5 |  | 25 |  | 2310 |  | 29.4 |  | IMD2 |  |  |
|  | n77 | 4030 | 10 |  | 50 |  | 4030 |  | N/A |  | N/A |  |  |
|  | 3 | 1720 | 5 |  | 25 |  | 1815 |  | N/A |  | N/A |  |  |
|  | 40 | 2350 | 5 |  | 25 |  | 2350 |  | 5.3 |  | IMD5 |  |  |
|  | n77 | 3755 | 10 |  | 50 |  | 3755 |  | N/A |  | N/A |  |  |
|  | 3 | 1725 | 5 |  | 25 |  | 1820 |  | 29.9 |  | IMD29 |  |  |
|  | 40 | 2310 | 5 |  | 25 |  | 2310 |  | N/A |  | N/A |  |  |
|  | n77 | 4130 | 10 |  | 50 |  | 4130 |  | N/A |  | N/A |  |  |
| DC_3A-40A_n78A DC_3A-40C_n78A | 3 | N/A |  | 5 |  | N/A |  | 1870 |  | 9.1 |  | IMD4 |  |  |
|  | 40 | 2390 |  | 5 |  | 25 |  | 2390 |  | N/A |  | N/A |  |  |
|  | n78 | 3325 |  | 10 |  | 50 |  | 3325 |  | N/A |  | N/A |  |  |
|  | 3 | 1720 |  | 5 |  | 25 |  | 1815 |  | N/A |  | N/A |  |  |
|  | 40 | N/A |  | 5 |  | N/A |  | 2360 |  | 4.4 |  | IMD5 |  |  |
|  | n78 | 3760 |  | 10 |  | 50 |  | 3760 |  | N/A |  | N/A |  |  |
| DC_3A-41A_n1A DC_3A-41C_n1A DC_3A-3A-41A_n1A DC_3A-3A-41C_n1A | n1 | 1977.5 |  | 5 |  | 25 |  | 2167.5 |  | N/A |  | N/A |  |  |
|  | 3 | 1712.5 |  | 5 |  | 25 |  | 1807.5 |  | N/A |  | N/A |  |  |
|  | 41 | N/A |  | 5 |  | N/A |  | 2507.5 |  | 5.0 |  | IMD5 |  |  |
| DC_3A-41A_n3A DC_3A-41C_n3A | 3 | N/A |  | 5 |  | N/A |  | 1865 |  | 8.2 |  | IMD4 ¦2*fB41-2*fn3¦ |  |  |
|  | 41 | 2657.5 |  | 5 |  | 25 |  | 2657.5 |  | N/A |  | N/A |  |  |
|  | n3 | 1725 |  | 5 |  | 25 |  | 1820 |  | N/A |  | N/A |  |  |
| DC_3A-41A_n28A DC_3A-41C_n28A | 41 | 2543 |  | 10 |  | 50 |  | 2543 |  | N/A |  | N/A |  |  |
|  | n28 | 710.5 |  | 5 |  | 25 |  | 765.5 |  | N/A |  | N/A |  |  |
|  | 3 | N/A |  | 5 |  | N/A |  | 1832.5 |  | 26 |  | IMD2 |  |  |
|  | 3 | 1780 |  | 5 |  | 25 |  | 1875 |  | N/A |  | N/A |  |  |
|  | n28 | 738 |  | 5 |  | 25 |  | 793 |  | N/A |  | N/A |  |  |
|  | 41 | N/A |  | 5 |  | N/A |  | 2518 |  | 27.4 |  | IMD2 |  |  |
|  | 3 | 1715 |  | 5 |  | 25 |  | 1810 |  | N/A |  | N/A |  |  |
|  | n28 | 743 |  | 5 |  | 25 |  | 798 |  | N/A |  | N/A |  |  |
|  | 41 | N/A |  | 5 |  | N/A |  | 2687 |  | 15.9 |  | IMD3 |  |  |
| DC_3A-41A_n77A DC_3A-41C_n77A DC_3A-41A_n77(2A) DC_3A-41C_n77(2A) DC_3A_n41A-n77A DC_3A_n41A-n77(2A) | 3 | 1720 |  | 5 |  | 25 |  | 1815 |  | N/A |  | N/A |  |  |
|  | n77 | 3900 |  | 10 |  | 50 |  | 3900 |  | N/A |  | N/A |  |  |
|  | 41/n41 | N/A |  | 5 |  | N/A |  | 2640 |  | 5.3 |  | IMD5 |  |  |
|  | 41/n41 | 2620 |  | 5 |  | 25 |  | 2620 |  | N/A |  | N/A |  |  |
|  | n77 | 3400 |  | 10 |  | 50 |  | 3400 |  | N/A |  | N/A |  |  |
|  | 3 | N/A |  | 5 |  | N/A |  | 1840 |  | 16.4 |  | IMD3 |  |  |
| DC_3A-41A_n78A DC_3A-41C_n78A DC_3A-41A_n78(2A) DC_3A-41C_n78(2A) | 41 | 2620 |  | 5 |  | 25 |  | 2620 |  | N/A |  | N/A |  |  |
|  | n78 | 3400 |  | 10 |  | 50 |  | 3400 |  | N/A |  | N/A |  |  |
|  | 3 | N/A |  | 5 |  | N/A |  | 1840 |  | 16.4 |  | IMD3 |  |  |
| DC_3A_n41A-n78A DC_3A_n41A-n78(2A) | 3 | 1730 |  | 5 |  | 25 |  | 1825 |  | N/A |  | N/A |  |  |
|  | n41 | 2560 |  | 10 |  | 50 |  | 2560 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3390 |  | 16.4 |  | IMD3 |  |  |
| DC_3A-41A_n79A | 3 | 1770 |  | 5 |  | 25 |  | 1865 |  | N/A |  | N/A |  |  |
|  | n79 | 4440 |  | 40 |  | 216 |  | 4440 |  | N/A |  | N/A |  |  |
|  | 41 | N/A |  | 5 |  | N/A |  | 2670 |  | 30.2 |  | IMD2 |  |  |
|  | 41 | 2570 |  | 5 |  | 25 |  | 2570 |  | N/A |  | N/A |  |  |
|  | n79 | 4420 |  | 40 |  | 216 |  | 4420 |  | N/A |  | N/A |  |  |
|  | 3 | N/A |  | 5 |  | N/A |  | 1850 |  | 29.4 |  | IMD2 |  |  |
| DC_3_n78-n105 | 3 | 1715 |  | 5 |  | 25 |  | 1810 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3725 |  | 13 |  | IMD44 |  |  |
|  | n105 | 670 |  | 5 |  | 25 |  | 619 |  | N/A |  | N/A |  |  |
| DC_4A-5A_n78A | 4 | N/A |  | 5 |  | N/A |  | 2122 |  | 18.1 |  | IMD3 |  |  |
|  | 5 | 829 |  | 5 |  | 25 |  | 874 |  | N/A |  | N/A |  |  |
|  | n78 | 3780 |  | 10 |  | 50 |  | 3780 |  | N/A |  | N/A |  |  |
| DC_4A-7A_n28A | 4 | 1715 |  | 5 |  | 25 |  | 2115 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 5 |  | N/A |  | 2685 |  | 18.0 |  | IMD3 |  |  |
|  | n28 | 745 |  | 5 |  | 25 |  | 800 |  | N/A |  | N/A |  |  |
| DC_4A-7A_n78A | 4 | N/A |  | 5 |  | N/A |  | 2150 |  | 8.7 |  | IMD4 |  |  |
|  | 7 | 2550 |  | 5 |  | 25 |  | 2670 |  | N/A |  | N/A |  |  |
|  | n78 | 3625 |  | 10 |  | 50 |  | 3625 |  | N/A |  | N/A |  |  |
| DC_5_n1-n78 | 5 | 829 |  | 5 |  | 25 |  | 874 |  | N/A |  | N/A |  |  |
|  | n1 | N/A |  | 5 |  | N/A |  | 2122 |  | 18.1 |  | IMD3 |  |  |
|  | n78 | 3780 |  | 10 |  | 50 |  | 3780 |  | N/A |  | N/A |  |  |
|  | 5 | 830 |  | 5 |  | 25 |  | 875 |  | N/A |  | N/A |  |  |
|  | n1 | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3610 |  | 15.7 |  | IMD3 |  |  |
| DC_5A_n1A-n28A | 5 | 829 |  | 5 |  | 25 |  | 874 |  | N/A |  | N/A |  |  |
|  | n1 | N/A |  | 5 |  | N/A |  | 2123 |  | 4 |  | IMD5 |  |  |
|  | n28 | 738 |  | 5 |  | 25 |  | 793 |  | N/A |  | N/A |  |  |
| DC_5A_n2A-n41A | 5 | 830 |  | 5 |  | 25 |  | 875 |  | N/A |  | N/A |  |  |
|  | n2 | 1855 |  | 10 |  | 50 |  | 1935 |  | N/A |  | N/A |  |  |
|  | n41 | 2685 |  | 10 |  | 50 |  | 2685 |  | 30.0 |  | IMD2 |  |  |
| DC_5A_n2A-n66A | 5 | 830 |  | 5 |  | 25 |  | 875 |  | N/A |  | N/A |  |  |
|  | n2 | 1900 |  | 5 |  | 25 |  | 1980 |  | N/A |  | N/A |  |  |
|  | n66 | 1740 |  | 5 |  | 25 |  | 2140 |  | 7.2 |  | IMD4 |  |  |
| DC_5A_n2A-n77A11 | n2 | N/A |  | 5 |  | N/A |  | 1987 |  | 16.5 |  | IMD3 |  |  |
|  | 5 | 846.5 |  | 5 |  | 25 |  | 891.5 |  | N/A |  | N/A |  |  |
|  | n77 | 3680 |  | 10 |  | 50 |  | 3680 |  | N/A |  | N/A |  |  |
| DC_5A_n3A-n28A | 5 | 845 |  | 5 |  | 25 |  | 890 |  | N/A |  | N/A |  |  |
|  | n3 | N/A |  | 5 |  | N/A |  | 1829.5 |  | 8.7 |  | IMD4 |  |  |
|  | n28 | 705.5 |  | 5 |  | 25 |  | 760.5 |  | N/A |  | N/A |  |  |
|  | 5 | 827 |  | 5 |  | 25 |  | 872 |  | N/A |  | N/A |  |  |
|  | n3 | 1713 |  | 5 |  | 25 |  | 1808 |  | N/A |  | N/A |  |  |
|  | n28 | N/A |  | 5 |  | N/A |  | 768 |  | 9.4 |  | IMD4 |  |  |
| DC_5A_n5A-n77A11 | 5 | 834 |  | 5 |  | 25 |  | 879 |  | N/A |  | N/A |  |  |
|  | n5 | N/A |  | 5 |  | N/A |  | 889 |  | 8.3 |  | IMD4 |  |  |
|  | n77 | 3391 |  | 10 |  | 50 |  | 3391 |  | N/A |  | N/A |  |  |
|  | 5 | 826.5 |  | 5 |  | 25 |  | 871.5 |  | N/A |  | N/A |  |  |
|  | n5 | N/A |  | 5 |  | N/A |  | 882 |  | 5.5 |  | IMD5 |  |  |
|  | n77 | 4188 |  | 10 |  | 50 |  | 4188 |  | N/A |  | N/A |  |  |
| DC_5A-7A_n7A | 5 | N/A |  | 5 |  | N/A |  | 879 |  | 12 |  | IMD34 |  |  |
|  | 7 | 2527 |  | 10 |  | 50 |  | 2647 |  | N/A |  | N/A |  |  |
|  | n7 | 2547 |  | 10 |  | 50 |  | 2667 |  | N/A |  | N/A |  |  |
| DC_5A_n2A-n78A | 5 | 830 |  | 5 |  | 25 |  | 875 |  | N/A |  | N/A |  |  |
|  | n2 | 1880 |  | 5 |  | 25 |  | 1960 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3540 |  | 16.0 |  | IMD3 |  |  |
|  | 5 | 846.5 |  | 5 |  | 25 |  | 891.5 |  | N/A |  | N/A |  |  |
|  | n2 | N/A |  | 5 |  | N/A |  | 1987 |  | 16.5 |  | IMD3 |  |  |
|  | n78 | 3680 |  | 10 |  | 25 |  | 3680 |  | N/A |  | N/A |  |  |
| DC_5A_n3A-n78A | 5 | 839 |  | 5 |  | 25 |  | 884 |  | N/A |  | N/A |  |  |
|  | n3 | 1730 |  | 5 |  | 25 |  | 1825 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3408 |  | 16.1 |  | IMD3 |  |  |
|  | 5 | 839 |  | 5 |  | 25 |  | 884 |  | N/A |  | N/A |  |  |
|  | n3 | 1730 |  | 5 |  | 25 |  | 1825 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3512 |  | 4.5 |  | IMD5 |  |  |
|  | 5 | 839 |  | 5 |  | 25 |  | 884 |  | N/A |  | N/A |  |  |
|  | n3 | N/A |  | 5 |  | N/A |  | 1862 |  | 15.7 |  | IMD3 |  |  |
|  | n78 | 3540 |  | 10 |  | 50 |  | 3540 |  | N/A |  | N/A |  |  |
| DC_5A-7A_n25A | 5 | 1855 |  | 5 |  | 25 |  | 1935 |  | N/A |  | N/A |  |  |
|  | 7 | 2565 |  | 5 |  | 25 |  | 2685 |  | 30.0 |  | IMD2 |  |  |
|  | n25 | 830 |  | 5 |  | 25 |  | 875 |  | N/A |  | N/A |  |  |
| DC_5A-7A_n28A | 5 | 842 |  | 5 |  | 25 |  | 887 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 5 |  | N/A |  | 2640 |  | 5.9 |  | IMD5 |  |  |
|  | n28 | 728 |  | 5 |  | 25 |  | 783 |  | N/A |  | N/A |  |  |
| DC_5A-7A_n66A DC_5A-7C_n66A DC_5A-7A-7A_n66A | 5 | N/A |  | 5 |  | N/A |  | 880 |  | 17.8 |  | IMD3 |  |  |
|  | 7 | 2560 |  | 5 |  | 25 |  | 2680 |  | N/A |  | N/A |  |  |
|  | 66 | 1720 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
|  | 5 | 846.5 |  | 5 |  | 25 |  | 891.5 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 5 |  | N/A |  | 2624 |  | 29.0 |  | IMD21 |  |  |
|  | 66 | 1777.5 |  | 5 |  | 25 |  | 2177.5 |  | N/A |  | N/A |  |  |
| DC_5A-7A_n71A | 5 | 835 |  | 5 |  | 25 |  | 880 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 5 |  | N/A |  | 2660 |  | 6.5 |  | IMD5 |  |  |
|  | n71 | 680 |  | 5 |  | 25 |  | 634 |  | N/A |  | N/A |  |  |
|  | 5 | 844 |  | 5 |  | 25 |  | 889 |  | N/A |  | N/A |  |  |
| DC_5A-7A_n77A | 7 | N/A |  | 5 |  | N/A |  | 2645 |  | 30.1 |  | IMD2 |  |  |
| DC_5A-7A_n77(2A) DC_5A-7A_n77(3A) | n77 | 3489 |  | 10 |  | 50 |  | 3489 |  | N/A |  | N/A |  |  |
| DC_5A-7A-7A_n77A | 5 | N/A |  | 5 |  | N/A |  | 879 |  | 30.2 |  | IMD21 |  |  |
| DC_5A-7A-7A_n77(2A) DC_5A-7A-7A_n77(3A) | 7 | 2550 |  | 5 |  | 25 |  | 2670 |  | N/A |  | N/A |  |  |
|  | n77 | 3429 |  | 10 |  | 50 |  | 3429 |  | N/A |  | N/A |  |  |
| DC_5A-7A_n78A DC_5A-7A_n78C DC_5A-7A_n78(A-C) DC_5A-7A-7A_n78C | 5 | 844 |  | 5 |  | 25 |  | 889 |  | N/A |  | N/A |  |  |
| DC_5A-7A-7A_n78(A-C) | 7 | N/A |  | 5 |  | N/A |  | 2645 |  | 30.1 |  | IMD2 |  |  |
|  | n78 | 3489 |  | 10 |  | 50 |  | 3489 |  | N/A |  | N/A |  |  |
|  | 5 | N/A |  | 5 |  | N/A |  | 879 |  | 30.2 |  | IMD2 |  |  |
|  | 7 | 2550 |  | 5 |  | 25 |  | 2670 |  | N/A |  | N/A |  |  |
|  | n78 | 3429 |  | 10 |  | 50 |  | 3429 |  | N/A |  | N/A |  |  |
|  | 5 | N/A |  | 5 |  | N/A |  | 875 |  | 3.3 |  | IMD5 |  |  |
|  | 7 | 2525 |  | 5 |  | 25 |  | 2645 |  | N/A |  | N/A |  |  |
|  | n78 | 3350 |  | 10 |  | 50 |  | 3350 |  | N/A |  | N/A |  |  |
| DC_5A_n7A-n78A, DC_5A_n7(2A)-n78A DC_5A_n7A-n78(2A) DC_5A_n7(2A)-n78(2A) | 5 | 844 |  | 5 |  | 25 |  | 889 |  | N/A |  | N/A |  |  |
|  | n7 | N/A |  | 5 |  | N/A |  | 2645 |  | 30.1 |  | IMD2 |  |  |
|  | n78 | 3489 |  | 10 |  | 50 |  | 3489 |  | N/A |  | N/A |  |  |
|  | 5 | 835 |  | 5 |  | 25 |  | 880 |  | N/A |  | N/A |  |  |
|  | n7 | 2540 |  | 5 |  | 25 |  | 2660 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3375 |  | 29.7 |  | IMD2 |  |  |
| DC_5A-13A_n66A | 5 | 840 |  | 5 |  | 25 |  | 885 |  | N/A |  | N/A |  |  |
|  | 13 | N/A |  | 5 |  | N/A |  | 750 |  | 9.4 |  | IMD4 |  |  |
|  | n66 | 1770 |  | 5 |  | 25 |  | 2170 |  | N/A |  | N/A |  |  |
| DC_5A-13A_n77A11 | 5 | 840 |  | 5 |  | 25 |  | 885 |  | N/A |  | N/A |  |  |
| DC_5A-13A_n77C11 | n77 | 4110 |  | 10 |  | 50 |  | 4110 |  | N/A |  | N/A |  |  |
|  | 13 | N/A |  | 5 |  | N/A |  | 750 |  | 4.4 |  | IMD5 |  |  |
|  | 13 | 782 |  | 5 |  | 20 |  | 751 |  | N/A |  | N/A |  |  |
|  | n77 | 4013 |  | 10 |  | 50 |  | 4013 |  | N/A |  | N/A |  |  |
|  | 5 | N/A |  | 5 |  | N/A |  | 885 |  | 4.5 |  | IMD5 |  |  |
| DC_5A_n28A-n77A DC_5A_n28A-n77C | 5 | 846.5 |  | 5 |  | 25 |  | 891.5 |  | N/A |  | N/A |  |  |
|  | n28 | 710.5 |  | 5 |  | 25 |  | 765.5 |  | 11.6 |  | IMD4 |  |  |
|  | n77 | 3305 |  | 10 |  | 50 |  | 3305 |  | N/A |  | N/A |  |  |
|  | 5 | 835 |  | 5 |  | 25 |  | 880 |  | N/A |  | N/A |  |  |
|  | n28 | 710 |  | 5 |  | 25 |  | 765 |  | 4.4 |  | IMD5 |  |  |
|  | n77 | 4105 |  | 10 |  | 50 |  | 4105 |  | N/A |  | N/A |  |  |
| DC_5A_n28A-n78A DC_5A_n28A-n78C | 5 | 846.5 |  | 5 |  | 25 |  | 891.5 |  | N/A |  | N/A |  |  |
|  | n28 | 715.5 |  | 5 |  | 25 |  | 765.5 |  | 11.6 |  | IMD4 |  |  |
|  | n78 | 3305 |  | 10 |  | 50 |  | 3305 |  | N/A |  | N/A |  |  |
|  | 5 | 830 |  | 5 |  | 25 |  | 875 |  | N/A |  | N/A |  |  |
|  | n28 | 707 |  | 5 |  | 25 |  | 762 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3658 |  | 4.0 |  | IMD5 |  |  |
| DC_5A-30A_n2A | 5 | N/A |  | 5 |  | N/A |  | 880 |  | 8 |  | IMD4 |  |  |
|  | 30 | 2310 |  | 5 |  | 25 |  | 2355 |  | N/A |  | N/A |  |  |
|  | n2 | 1870 |  | 5 |  | 25 |  | 1950 |  | N/A |  | N/A |  |  |
| DC_5A-30A_n77A DC_5A-30A_n77(2A) | 5 | 835 |  | 5 |  | 25 |  | 880 |  | 15.2 |  | IMD34 |  |  |
|  | 30 | N/A |  | 5 |  | N/A |  | 2355 |  | N/A |  | N/A |  |  |
|  | n77 | 3740 |  | 10 |  | 50 |  | 3740 |  | N/A |  | N/A |  |  |
|  | 5 | 835 |  | 5 |  | 25 |  | 880 |  | N/A |  | N/A |  |  |
|  | 30 | N/A |  | 5 |  | N/A |  | 2355 |  | 13.2 |  | IMD311 |  |  |
|  | n77 | 4025 |  | 10 |  | 50 |  | 4025 |  | N/A |  | N/A |  |  |
| DC_5A_n38A-n66A | 5 | 830 |  | 5 |  | 25 |  | 875 |  | N/A |  | N/A |  |  |
|  | n66 | 1760 |  | 5 |  | 25 |  | 2160 |  | N/A |  | N/A |  |  |
|  | n38 | N/A |  | 10 |  | N/A |  | 2590 |  | 28.9 |  | IMD2 |  |  |
| DC_5A-40A_n77A | 5 | 835 | 5 |  | 25 |  | 880 |  | N/A |  | N/A |  |  |
| DC_5A-40C_n77A | 40 | 2355 | 5 |  | 25 |  | 2355 |  | 13.2 |  | IMD3 |  |  |
| DC_5A-40A_n77C | n77 | 4025 | 10 |  | 50 |  | 4025 |  | N/A |  | N/A |  |  |
| DC_5A-40C_n77C | 5 | 835 | 5 |  | 25 |  | 880 |  | 15.2 |  | IMD34 |  |  |
|  | 40 | 2310 | 5 |  | 25 |  | 2310 |  | N/A |  | N/A |  |  |
|  | n77 | 3740 | 10 |  | 50 |  | 3740 |  | N/A |  | N/A |  |  |
| DC_5A_n41A-n66A | 5 | 846.5 |  | 5 |  | 25 |  | 891.5 |  | N/A |  | N/A |  |  |
|  | n41 | 2624 |  | 10 |  | 50 |  | 2624 |  | 29.0 |  | IMD2 |  |  |
|  | n66 | 1777.5 |  | 5 |  | 25 |  | 2177.5 |  | N/A |  | N/A |  |  |
|  | 5 | 830 |  | 5 |  | 25 |  | 875 |  | N/A |  | N/A |  |  |
|  | n41 | 2600 |  | 10 |  | 50 |  | 2600 |  | 18 |  | IMD3 |  |  |
|  | n66 | 1715 |  | 5 |  | 25 |  | 2115 |  | N/A |  | N/A |  |  |
| DC_5A_n40A-n77A | 5 | 840 |  | 5 |  | 25 |  | 885 |  | N/A |  | N/A |  |  |
| DC_5A_n40A-n77(2A) | n40 | 2310 |  | 5 |  | 25 |  | 2310 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 3780 |  | 16.1 |  | IMD3 |  |  |
|  | 5 | 835 |  | 5 |  | 25 |  | 880 |  | N/A |  | N/A |  |  |
|  | n40 | N/A |  | 5 |  | N/A |  | 2355 |  | 13.2 |  | IMD3 |  |  |
|  | n77 | 4025 |  | 10 |  | 50 |  | 4025 |  | N/A |  | N/A |  |  |
| DC_5A-40A_n78A DC_5A-40C_n78A | 5 | 835 |  | 5 |  | 25 |  | 880 |  | 15.2 |  | IMD3 |  |  |
| DC_5A-40A_n78C | 40 | 2310 |  | 5 |  | 25 |  | 2310 |  | N/A |  | N/A |  |  |
| DC_5A-40C_n78C | n78 | 3740 |  | 10 |  | 50 |  | 3740 |  | N/A |  | N/A |  |  |
| DC_5A_n40A-n78A | 5 | 840 |  | 5 |  | 25 |  | 885 |  | N/A |  | N/A |  |  |
| DC_5A_n40A-n78C | n40 | 2310 |  | 5 |  | 25 |  | 2310 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3780 |  | 16.1 |  | IMD3 |  |  |
| DC_5A_41A_n78A | 5 | N/A |  | 5 |  | N/A |  | 885 |  | 30.2 |  | IMD2 |  |  |
|  | 41 | 2615 |  | 5 |  | 25 |  | 2615 |  | N/A |  | N/A |  |  |
|  | n78 | 3500 |  | 10 |  | 50 |  | 3500 |  | N/A |  | N/A |  |  |
|  | 5 | N/A |  | 5 |  | N/A |  | 881.5 |  | 3.1 |  | IMD5 |  |  |
|  | 41 | 2620.5 |  | 5 |  | 25 |  | 2620.5 |  | N/A |  | N/A |  |  |
|  | n78 | 3490 |  | 10 |  | 50 |  | 3490 |  | N/A |  | N/A |  |  |
| DC_5A-41A_n79A | 5 | N/A |  | 5 |  | N/A |  | 880 |  | 23.9 |  | IMD3 |  |  |
|  | 41 | 2665 |  | 5 |  | 25 |  | 2665 |  | N/A |  | N/A |  |  |
|  | n79 | 4450 |  | 40 |  | 216 |  | 4450 |  | N/A |  | N/A |  |  |
|  | 5 | 826.5 |  | 5 |  | 25 |  | 871.5 |  | N/A |  | N/A |  |  |
|  | 41 | N/A |  | 5 |  | N/A |  | 2517.5 |  | 1.8 |  | IMD4 |  |  |
|  | n79 | 4980 |  | 40 |  | 216 |  | 4980 |  | N/A |  | N/A |  |  |
| DC_5A-46A_n66A | 5 | 847 |  | 5 |  | 25 |  | 892 |  | N/A |  | N/A |  |  |
|  | 46 | N/A |  | 10 |  | N/A |  | 5163 |  | 9.04 |  | IMD4 ¦2*fB5+2*fn66¦ |  |  |
|  | n66 | 1775 |  | 5 |  | 25 |  | 2175 |  | N/A |  | N/A |  |  |
| DC_5A-48A_n12A | 5 | 830 |  | 5 |  | 25 |  | 875 |  | N/A |  | N/A |  |  |
|  | 48 | N/A |  | 5 |  | N/A |  | 3650 |  | 4.4 |  | IMD5 |  |  |
|  | n12 | 705 |  | 5 |  | 25 |  | 735 |  | N/A |  | N/A |  |  |
|  | 5 | N/A |  | 5 |  | N/A |  | 875 |  | 5.9 |  | IMD5 |  |  |
|  | 48 | 3695 |  | 5 |  | 25 |  | 3695 |  | N/A |  | N/A |  |  |
|  | n12 | 705 |  | 5 |  | 25 |  | 735 |  | N/A |  | N/A |  |  |
| DC_5A-48A_n71A | 5 | 830 |  | 5 |  | 25 |  | 875 |  | N/A |  | N/A |  |  |
|  | 48 | N/A |  | 5 |  | N/A |  | 3590 |  | 4.4 |  | IMD5 |  |  |
|  | n71 | 690 |  | 5 |  | 25 |  | 644 |  | N/A |  | N/A |  |  |
|  | 5 | N/A |  | 5 |  | N/A |  | 880 |  | 5.9 |  | IMD5 |  |  |
|  | 48 | 3600 |  | 5 |  | 25 |  | 3600 |  | N/A |  | N/A |  |  |
|  | n71 | 680 |  | 5 |  | 25 |  | 634 |  | N/A |  | N/A |  |  |
| DC_5A-66A_n2A DC_5B-66A_n2A DC_5A-5A-66A_n2A DC_5A-66A-66A_n2A DC_5B-66A-66A_n2A DC_5A-5A-66A-66A_n2A | 5 | 834 |  | 5 |  | 25 |  | 879 |  | N/A |  | N/A |  |  |
| DC_5A-66B_n2A DC_5A-66A_n2(2A) | 66 | N/A |  | 5 |  | N/A |  | 2132 |  | 7.2 |  | IMD4 |  |  |
|  | n2 | 1900 |  | 5 |  | 25 |  | 1980 |  | N/A |  | N/A |  |  |
| DC_5A-66A_n7A DC_5A-66A-66A_n7A | 5 | N/A |  | 5 |  | N/A |  | 880 |  | 18.0 |  | IMD3 |  |  |
|  | 66 | 1720 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
|  | n7 | 2560 |  | 5 |  | 25 |  | 2680 |  | N/A |  | N/A |  |  |
| DC_5A-66A_n25A | 5 | 834 |  | 5 |  | 25 |  | 879 |  | N/A |  | N/A |  |  |
|  | 66 | 1732 |  | 5 |  | 25 |  | 2132 |  | 7.2 |  | IMD4 |  |  |
|  | n25 | 1900 |  | 5 |  | 25 |  | 1980 |  | N/A |  | N/A |  |  |
| DC_5A-66A_n30A | 5 | 830 |  | 5 |  | 25 |  | 875 |  | N/A |  | N/A |  |  |
|  | 66 | N/A |  | 5 |  | N/A |  | 2125 |  | 4 |  | IMD5 |  |  |
|  | n30 | 2307.5 |  | 5 |  | 50 |  | 2352.5 |  | N/A |  | N/A |  |  |
| DC_5A-66A_n41A | 5 | 830 |  | 5 |  | 25 |  | 875 |  | 28.9 |  | IMD2 |  |  |
|  | 66 | 1765 |  | 5 |  | 25 |  | 2165 |  | N/A |  | N/A |  |  |
|  | n41 | 2640 |  | 10 |  | 50 |  | 2640 |  | N/A |  | N/A |  |  |
|  | 5 | 835 |  | 5 |  | 25 |  | 880 |  | 18.0 |  | IMD3 |  |  |
|  | 66 | 1720 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
|  | n41 | 2560 |  | 10 |  | 50 |  | 2560 |  | N/A |  | N/A |  |  |
| DC_5A-66A_n71A | 5 | 830 |  | 5 |  | 25 |  | 875 |  | N/A |  | N/A |  |  |
|  | 66 | N/A |  | 5 |  | N/A |  | 2161 |  | 13 |  | IMD3 |  |  |
|  | n71 | 665.5 |  | 5 |  | 25 |  | 619.5 |  | N/A |  | N/A |  |  |
|  | 5 | N/A |  | 5 |  | N/A |  | 891.5 |  | 4.2 |  | IMD5 |  |  |
|  | 66 | 1770 |  | 5 |  | 25 |  | 2170 |  | N/A |  | N/A |  |  |
|  | n71 | 665.5 |  | 5 |  | 25 |  | 619.5 |  | N/A |  | N/A |  |  |
| DC_5A-66A_n77A | 5 | 826.5 |  | 5 |  | 25 |  | 871.5 |  | N/A |  | N/A |  |  |
| DC_5A-66A_n77C DC_5A-66A_n77(2A) DC_5A-66A-66A_n77A DC_5A-66A-66A_n77C | 66 | N/A |  | 5 |  | N/A |  | 2142 |  | 13.2 |  | IMD3 |  |  |
| DC_5A-66A-66A_n77(2A) | n77 | 3795 |  | 10 |  | 50 |  | 3795 |  | N/A |  | N/A |  |  |
| DC_5A-66A_n78A DC_5A-66A_n78(2A) | 5 | 826.5 |  | 5 |  | 25 |  | 871.5 |  | N/A |  | N/A |  |  |
| DC_5A-66A-66A_n78A | 66 | N/A |  | 5 |  | N/A |  | 2142 |  | 13.2 |  | IMD3 |  |  |
|  | n78 | 3795 |  | 10 |  | 50 |  | 3795 |  | N/A |  | N/A |  |  |
| DC_5A_n66A-n77A | 5 | 826.5 |  | 5 |  | 25 |  | 871.5 |  | N/A |  | N/A |  |  |
|  | n66 | N/A |  | 5 |  | N/A |  | 2142 |  | 13.2 |  | IMD3 |  |  |
|  | n77 | 3795 |  | 10 |  | 50 |  | 3795 |  | N/A |  | N/A |  |  |
|  | 5 | 845 |  | 5 |  | 25 |  | 890 |  | N/A |  | N/A |  |  |
|  | n66 | 1785 |  | 5 |  | 25 |  | 2185 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 3475 |  | 16.1 |  | IMD3 |  |  |
| DC_5A_n66A-n78A | 5 | 830 |  | 5 |  | 25 |  | 875 |  | N/A |  | N/A |  |  |
|  | n66 | 1760 |  | 5 |  | 25 |  | 2160 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3420 |  | 16.6 |  | IMD3 |  |  |
|  | 5 | 826.5 |  | 5 |  | 25 |  | 871.5 |  | N/A |  | N/A |  |  |
|  | n66 | N/A |  | 5 |  | N/A |  | 2142 |  | 13.2 |  | IMD3 |  |  |
|  | n78 | 3795 |  | 10 |  | 50 |  | 3795 |  | N/A |  | N/A |  |  |
| DC_7A_n1A-n28A | 7 | 2535 |  | 5 |  | 25 |  | 2655 |  | N/A |  | N/A |  |  |
| DC_7C-n1A-n28A | n1 | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | n28 | N/A |  | 5 |  | N/A |  | 780 |  | 4.3 |  | IMD5 |  |  |
| DC_7A_n1A-n40A | 7 | 2540 |  | 5 |  | 25 |  | 2660 |  | N/A |  | N/A |  |  |
|  | n40 | 2335 |  | 5 |  | 25 |  | 2335 |  | N/A |  | N/A |  |  |
|  | n1 | N/A |  | 5 |  | N/A |  | 2130 |  | 15.2 |  | IMD3 |  |  |
| DC_7A_n1A-n75A | n1 | 1977.5 |  | 5 |  | 25 |  | 2167.5 |  | N/A |  | N/A |  |  |
|  | 7 | 2502.5 |  | 5 |  | 25 |  | 2622.5 |  | N/A |  | N/A |  |  |
|  | 75 | N/A |  | 5 |  | N/A |  | 1454.5 |  | 15.2 |  | IMD3 |  |  |
| DC_7A_n1A-n78A | 7 | 2520 |  | 5 |  | 25 |  | 2640 |  | N/A |  | N/A |  |  |
| DC_7C_n1A-n78A | n1 | 1970 |  | 5 |  | 25 |  | 2160 |  | N/A |  | N/A |  |  |
| DC_7A_n1A-n78(2A) | n78 | N/A |  | 10 |  | N/A |  | 3390 |  | 10.1 |  | IMD4 |  |  |
| DC_7C_n1A-n78(2A) | 7 | 2530 |  | 5 |  | 25 |  | 2650 |  | N/A |  | N/A |  |  |
|  | n1 | N/A |  | 5 |  | N/A |  | 2160 |  | 9.0 |  | IMD4 |  |  |
|  | n78 | 3610 |  | 10 |  | 50 |  | 3610 |  | N/A |  | N/A |  |  |
| DC_7A_n2A-n71A | 7 | 2530 |  | 5 |  | 25 |  | 2650 |  | N/A |  | N/A |  |  |
|  | n2 | 1900 |  | 5 |  | 25 |  | 1980 |  | N/A |  | N/A |  |  |
|  | n71 | N/A |  | 5 |  | N/A |  | 630 |  | 28.7 |  | IMD2 |  |  |
| DC_7A_n2A-n77A | 7 | 2550 |  | 5 |  | 25 |  | 2685 |  | N/A |  | N/A |  |  |
|  | n2 | 1870 |  | 5 |  | 25 |  | 1950 |  | 8.6 |  | IMD4 |  |  |
|  | n77 | 3525 |  | 10 |  | 50 |  | 3525 |  | N/A |  | N/A |  |  |
|  | 7 | 2525 |  | 5 |  | 25 |  | 2645 |  | N/A |  | N/A |  |  |
|  | n2 | 1900 |  | 5 |  | 25 |  | 1980 |  | N/A |  | N/A |  |  |
|  | n77 | 3775 |  | 10 |  | 50 |  | 3775 |  | 4.2 |  | IMD5 |  |  |
| DC_7A_n2A-n78A | 7 | 2550 |  | 5 |  | 25 |  | 2685 |  | N/A |  | N/A |  |  |
|  | n2 | N/A |  | 5 |  | N/A |  | 1950 |  | 8.6 |  | IMD4 |  |  |
|  | n78 | 3525 |  | 10 |  | 50 |  | 3525 |  | N/A |  | N/A |  |  |
|  | 7 | 2525 |  | 5 |  | 25 |  | 2645 |  | N/A |  | N/A |  |  |
|  | n2 | 1900 |  | 5 |  | 25 |  | 1980 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3775 |  | 4.2 |  | IMD5 |  |  |
| DC_7A_n3A-n78A | 7 | 2560 |  | 5 |  | 25 |  | 2680 |  | N/A |  | N/A |  |  |
| DC_7C_n3A-n78A | n3 | 1730 |  | 5 |  | 25 |  | 1825 |  | N/A |  | N/A |  |  |
| DC_7A_n3A-n78(2A) | n78 | N/A |  | 10 |  | N/A |  | 3390 |  | 16.1 |  | IMD3 |  |  |
| DC_7C_n3A-n78(2A) | 7 | 2565 |  | 5 |  | 25 |  | 2685 |  | N/A |  | N/A |  |  |
|  | n3 | N/A |  | 5 |  | N/A |  | 1820 |  | 15.6 |  | IMD3 |  |  |
|  | n78 | 3310 |  | 10 |  | 50 |  | 3310 |  | N/A |  | N/A |  |  |
| DC_7A_n8A-n40A | 7 | 2530 |  | 5 |  | 25 |  | 2650 |  | N/A |  | N/A |  |  |
|  | n8 | 905 |  | 5 |  | 25 |  | 950 |  | N/A |  | N/A |  |  |
|  | n40 | N/A |  | 5 |  | N/A |  | 2345 |  | 3.0 |  | IMD5 |  |  |
| DC_7A-8A_n3A | n3 | 1735 |  | 5 |  | 25 |  | 1830 |  | N/A |  | N/A |  |  |
|  | 7 | 2530 |  | 10 |  | 50 |  | 2650 |  | N/A |  | N/A |  |  |
|  | 8 | N/A |  | 5 |  | N/A |  | 940 |  | 18.0 |  | IMD3 |  |  |
| DC_7A-8A_n3A | n3 | 1780 |  | 5 |  | 25 |  | 1875 |  | N/A |  | N/A |  |  |
|  | 8 | 890 |  | 5 |  | 25 |  | 935 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 10 |  | N/A |  | 2670 |  | 29.0 |  | IMD2+IMD33 |  |  |
| DC_7A-8A_n20A | 7 | N/A |  | 5 |  | N/A |  | 2640 |  | 21.1 |  | IMD34,15 |  |  |
|  | 8 | 900 |  | 5 |  | 25 |  | 945 |  | N/A |  | N/A |  |  |
|  | n20 | 840 |  | 5 |  | 25 |  | 799 |  | N/A |  | N/A |  |  |
|  | 7 | 2503 |  | 5 |  | 25 |  | 2623 |  | N/A |  | N/A |  |  |
|  | n20 | 859 |  | 5 |  | 25 |  | 818 |  | N/A |  | N/A |  |  |
|  | 8 | N/A |  | 5 |  | N/A |  | 933 |  | 4.4 |  | IMD5 |  |  |
| DC_7A-8A_n77A | 7 | 2520 |  | 5 |  | 25 |  | 2640 |  | N/A |  | N/A |  |  |
|  | 8 | N/A |  | 5 |  | N/A |  | 940 |  | 3.1 |  | IMD5 |  |  |
|  | n77 | 3310 |  | 10 |  | 50 |  | 3310 |  | N/A |  | N/A |  |  |
|  | 7 | 2530 |  | 5 |  | 25 |  | 2650 |  | N/A |  | N/A |  |  |
|  | 8 | N/A |  | 5 |  | N/A |  | 940 |  | 30.5 |  | IMD2 |  |  |
|  | n77 | 3470 |  | 10 |  | 50 |  | 3470 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 5 |  | N/A |  | 2650 |  | 28 |  | IMD2 |  |  |
|  | 8 | 895 |  | 5 |  | 25 |  | 940 |  | N/A |  | N/A |  |  |
|  | n77 | 3545 |  | 10 |  | 50 |  | 3545 |  | N/A |  | N/A |  |  |
| DC_7A-8A_n78A | 7 | 2530 |  | 5 |  | 25 |  | 2650 |  | N/A |  | N/A |  |  |
| DC_7A-8B_n78A | 8 | N/A |  | 5 |  | N/A |  | 940 |  | 30.5 |  | IMD2 |  |  |
| DC_7A-7A-8B_n78A | n78 | 3470 |  | 10 |  | 50 |  | 3470 |  | N/A |  | N/A |  |  |
|  | 7 | 2520 |  | 5 |  | 25 |  | 2640 |  | N/A |  | N/A |  |  |
|  | 8 | N/A |  | 5 |  | N/A |  | 940 |  | 3.1 |  | IMD5 |  |  |
|  | n78 | 3310 |  | 10 |  | 50 |  | 3310 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 5 |  | N/A |  | 2650 |  | 28 |  | IMD2 |  |  |
|  | 8 | 895 |  | 5 |  | 25 |  | 940 |  | N/A |  | N/A |  |  |
|  | n78 | 3545 |  | 10 |  | 50 |  | 3545 |  | N/A |  | N/A |  |  |
| DC_7A_n8A-n78A | 7 | 2555 |  | 5 |  | 25 |  | 2675 |  | N/A |  | N/A |  |  |
|  | n8 | 900 |  | 5 |  | 25 |  | 945 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3455 |  | 28.5 |  | IMD2 |  |  |
|  | 7 | 2555 |  | 5 |  | 25 |  | 2675 |  | N/A |  | N/A |  |  |
|  | n8 | N/A |  | 5 |  | N/A |  | 945 |  | 29.7 |  | IMD2 |  |  |
|  | n78 | 3500 |  | 10 |  | 50 |  | 3500 |  | N/A |  | N/A |  |  |
| DC_7A-12A_n2A | 7 | 2502.5 |  | 5 |  | 25 |  | 2622.5 |  | N/A |  | N/A |  |  |
| DC_7A-12A_n2(2A) | 12 | N/A |  | 5 |  | N/A |  | 731.5 |  | 5.3 |  | IMD5 |  |  |
|  | n2 | 1907.5 |  | 5 |  | 25 |  | 1987.5 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 5 |  | N/A |  | 2621 |  | 30.8 |  | IMD2 |  |  |
|  | 12 | 713.5 |  | 5 |  | 25 |  | 743.5 |  | N/A |  | N/A |  |  |
|  | n2 | 1907.5 |  | 5 |  | 25 |  | 1987.5 |  | N/A |  | N/A |  |  |
| DC_7A-12A_n25A | 7 | 2502.5 |  | 5 |  | 25 |  | 2622.5 |  | N/A |  | N/A |  |  |
|  | 12 | N/A |  | 5 |  | N/A |  | 731.5 |  | 5.3 |  | IMD5 |  |  |
|  | n25 | 1907.5 |  | 5 |  | 25 |  | 1987.5 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 5 |  | N/A |  | 2622.5 |  | 30.8 |  | IMD2 |  |  |
|  | 12 | 713.5 |  | 5 |  | 25 |  | 743.5 |  | N/A |  | N/A |  |  |
|  | n25 | 1907.5 |  | 5 |  | 25 |  | 1987.5 |  | N/A |  | N/A |  |  |
| DC_7A-12A_n66A | 7 | 2515 |  | 5 |  | 25 |  | 2635 |  | N/A |  | N/A |  |  |
|  | 12 | N/A |  | 5 |  | N/A |  | 742 |  | 31 |  | IMD2 |  |  |
|  | n66 | 1773 |  | 5 |  | 25 |  | 2173 |  | N/A |  | N/A |  |  |
| DC_7A_n12A-n77A | 7 | 2565 |  | 5 |  | 25 |  | 2685 |  | N/A |  | N/A |  |  |
|  | n12 | N/A |  | 5 |  | N/A |  | 740 |  | 30.8 |  | IMD2 |  |  |
|  | n77 | 3305 |  | 10 |  | 50 |  | 3305 |  | N/A |  | N/A |  |  |
|  | 7 | 2505 |  | 5 |  | 25 |  | 2625 |  | N/A |  | N/A |  |  |
|  | n12 | 702 |  | 5 |  | 25 |  | 732 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 3909 |  | 16.0 |  | IMD3 |  |  |
| DC_7A-12A_n77A DC_7A-12A_n77(2A) | 7 | N/A |  | 5 |  | N/A |  | 2662 |  | 29.6 |  | IMD21 |  |  |
|  | 12 | 708 |  | 5 |  | 25 |  | 738 |  | N/A |  | N/A |  |  |
|  | n77 | 3370 |  | 10 |  | 50 |  | 3370 |  | N/A |  | N/A |  |  |
|  | 7 | 2565 |  | 5 |  | 25 |  | 2685 |  | N/A |  | N/A |  |  |
|  | 12 | N/A |  | 5 |  | N/A |  | 740 |  | 30.8 |  | IMD2 |  |  |
|  | n77 | 3305 |  | 10 |  | 50 |  | 3305 |  | N/A |  | N/A |  |  |
| DC_7A-12A_n78A DC_7A-12A_n78(2A) | 7 | N/A |  | 5 |  | N/A |  | 2662 |  | 29.6 |  | IMD2 |  |  |
|  | 12 | 708 |  | 5 |  | 25 |  | 738 |  | N/A |  | N/A |  |  |
|  | n78 | 3370 |  | 10 |  | 50 |  | 3370 |  | N/A |  | N/A |  |  |
|  | 7 | 2565 |  | 5 |  | 25 |  | 2685 |  | N/A |  | N/A |  |  |
|  | 12 | N/A |  | 5 |  | N/A |  | 740 |  | 30.8 |  | IMD24 |  |  |
|  | n78 | 3305 |  | 10 |  | 50 |  | 3305 |  | N/A |  | N/A |  |  |
| DC_7A_n12A-n78A | 7 | 2565 |  | 5 |  | 25 |  | 2685 |  | N/A |  | N/A |  |  |
|  | n12 | 710 |  | 5 |  | 25 |  | 740 |  | 30.8 |  | IMD24 |  |  |
|  | n78 | 3305 |  | 10 |  | 50 |  | 3305 |  | N/A |  | N/A |  |  |
|  | 7 | 2505 |  | 5 |  | 25 |  | 2625 |  | N/A |  | N/A |  |  |
|  | n12 | 673 |  | 5 |  | 25 |  | 732 |  | N/A |  | N/A |  |  |
|  | n78 | 3664 |  | 10 |  | 50 |  | 3664 |  | 10.3 |  | IMD4 |  |  |
| DC_7A-13A_n66A | 7 | 2520 |  | 5 |  | 25 |  | 2640 |  | N/A |  | N/A |  |  |
|  | 13 | N/A |  | 5 |  | N/A |  | 750 |  | 31 |  | IMD2 |  |  |
|  | n66 | 1770 |  | 5 |  | 25 |  | 2170 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 5 |  | N/A |  | 2660 |  | 18 |  | IMD3 |  |  |
|  | 13 | 780 |  | 5 |  | 25 |  | 749 |  | N/A |  | N/A |  |  |
|  | n66 | 1720 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
| DC_7A-13A_n25A DC_7A-7A-13A_n25A DC_7C-13A_n25A | 7 | N/A |  | 10 |  | N/A |  | 2662 |  | 27.6 |  | IMD2 |  |  |
|  | 13 | 782 |  | 5 |  | 25 |  | 751 |  | N/A |  | N/A |  |  |
|  | n25 | 1880 |  | 5 |  | 25 |  | 1960 |  | N/A |  | N/A |  |  |
| DC_7A-20A_n1A DC_7C-20A_n1A | 7 | 2510 |  | 10 |  | 50 |  | 2630 |  | N/A |  | N/A |  |  |
|  | 20 | N/A |  | 10 |  | N/A |  | 800 |  | 4.5 |  | IMD5 |  |  |
|  | n1 | 1940 |  | 5 |  | 25 |  | 2130 |  | N/A |  | N/A |  |  |
| DC_7A-20A_n3A | 7 | 2543 |  | 10 |  | 50 |  | 2663 |  | N/A |  | N/A |  |  |
|  | 20 | N/A |  | 10 |  | N/A |  | 806 |  | 10.5 |  | IMD2 |  |  |
|  | n3 | 1737 |  | 5 |  | 25 |  | 1832 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 10 |  | N/A |  | 2630 |  | 26.0 |  | IMD21 |  |  |
|  | 20 | 855 |  | 5 |  | 25 |  | 896 |  | N/A |  | N/A |  |  |
|  | n3 | 1775 |  | 10 |  | 50 |  | 1870 |  | N/A |  | N/A |  |  |
| DC_7A-20A_n8A | 7 | 2565 |  | 5 |  | 25 |  | 2685 |  | N/A |  | N/A |  |  |
|  | n8 | 885 |  | 5 |  | 25 |  | 930 |  | N/A |  | N/A |  |  |
|  | 20 | N/A |  | 5 |  | N/A |  | 795 |  | 17.4 |  | IMD3 |  |  |
|  | 7 | N/A |  | 5 |  | N/A |  | 2640 |  | 21.1 |  | IMD3 |  |  |
|  | n8 | 900 |  | 5 |  | 25 |  | 945 |  | N/A |  | N/A |  |  |
|  | 20 | 840 |  | 5 |  | 25 |  | 799 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 5 |  | N/A |  | 2624 |  | 18.8 |  | IMD3 |  |  |
|  | n8 | 910 |  | 5 |  | 25 |  | 955 |  | N/A |  | N/A |  |  |
|  | 20 | 857 |  | 5 |  | 25 |  | 816 |  | N/A |  | N/A |  |  |
| DC_7A-20A_n28A | 20 | 842 |  | 5 |  | 25 |  | 801 |  | N/A |  | N/A |  |  |
|  | n28 | 728 |  | 5 |  | 25 |  | 783 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 10 |  | N/A |  | 2640 |  | 5.9 |  | IMD5 |  |  |
| DC_7A-20A_n78A DC_7A-7A-20A_n78A | 7 | 2560 |  | 5 |  | 25 |  | 2680 |  | N/A |  | N/A |  |  |
| DC_7A-20A_n78(2A) | 20 | N/A |  | 5 |  | N/A |  | 810 |  | 30.5 |  | IMD2 |  |  |
| DC_7A-20A_n78C | n78 | 3370 |  | 10 |  | 50 |  | 3370 |  | N/A |  | N/A |  |  |
|  | 7 | 2560 |  | 5 |  | 25 |  | 2680 |  | N/A |  | N/A |  |  |
|  | 20 | N/A |  | 5 |  | N/A |  | 810 |  | 3.0 |  | IMD5 |  |  |
|  | n78 | 3435 |  | 10 |  | 50 |  | 3435 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 5 |  | N/A |  | 2675 |  | 30.8 |  | IMD2 |  |  |
|  | 20 | 845 |  | 5 |  | 25 |  | 804 |  | N/A |  | N/A |  |  |
|  | n78 | 3520 |  | 10 |  | 50 |  | 3520 |  | N/A |  | N/A |  |  |
| DC_7A_n25A-n71A | 7 | 2530 |  | 5 |  | 25 |  | 2650 |  | N/A |  | N/A |  |  |
|  | n25 | 1900 |  | 5 |  | 25 |  | 1980 |  | N/A |  | N/A |  |  |
|  | n71 | N/A |  | 5 |  | N/A |  | 630 |  | 28.7 |  | IMD2 |  |  |
|  | 7 | 2550 |  | 5 |  | 25 |  | 2670 |  | N/A |  | N/A |  |  |
|  | n25 | 1910 |  | 5 |  | 25 |  | 1990 |  | N/A |  | N/A |  |  |
|  | n71 | N/A |  | 5 |  | N/A |  | 630 |  | 5 |  | IMD5 |  |  |
| DC_7A-25A_n77A DC_7A-7A-25A_n77A DC_7C-25A_n77A DC_7C-25A-25A_n77A DC_7A-25A-25A_n77A DC_7A-7A-25A-25A_n77A | 7 | 2550 |  | 5 |  | 25 |  | 2670 |  | N/A |  | N/A |  |  |
|  | 25 | N/A |  | 5 |  | N/A |  | 1950 |  | 8.6 |  | IMD4 |  |  |
|  | n77 | 3525 |  | 10 |  | 50 |  | 3525 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 5 |  | N/A |  | 2660 |  | 3.4 |  | IMD5 |  |  |
|  | 25 | 1860 |  | 5 |  | 25 |  | 1940 |  | N/A |  | N/A |  |  |
|  | n77 | 4120 |  | 10 |  | 50 |  | 4120 |  | N/A |  | N/A |  |  |
| DC_7A-25A_n78A DC_7A-7A-25A_n78A DC_7C-25A_n78A DC_7A-25A-25A_n78A DC_7A-7A-25A-25A_n78A DC_7C-25A-25A_n78A | 7 | 2550 |  | 5 |  | 25 |  | 2670 |  | N/A |  | N/A |  |  |
|  | 25 | N/A |  | 5 |  | N/A |  | 1950 |  | 8.6 |  | IMD4 |  |  |
|  | n78 | 3525 |  | 10 |  | 50 |  | 3525 |  | N/A |  | N/A |  |  |
| DC_7A-26A_n78A | 7 | 2525 |  | 5 |  | 25 |  | 2645 |  | 30.1 |  | IMD2 |  |  |
| DC_7C-26A_n78A | 26 | 844 |  | 5 |  | 25 |  | 889 |  | N/A |  | N/A |  |  |
|  | n78 | 3489 |  | 10 |  | 50 |  | 3489 |  | N/A |  | N/A |  |  |
|  | 7 | 2550 |  | 5 |  | 25 |  | 2670 |  | N/A |  | N/A |  |  |
|  | 26 | 834 |  | 5 |  | 25 |  | 879 |  | 30.2 |  | IMD2 |  |  |
|  | n78 | 3429 |  | 10 |  | 50 |  | 3429 |  | N/A |  | N/A |  |  |
|  | 7 | 2525 |  | 5 |  | 25 |  | 2645 |  | N/A |  | N/A |  |  |
|  | 26 | 830 |  | 5 |  | 25 |  | 875 |  | 3.3 |  | IMD5 |  |  |
|  | n78 | 3350 |  | 10 |  | 50 |  | 3350 |  | N/A |  | N/A |  |  |
| DC_7A_n26A-n78A | 7 | 2550 |  | 5 |  | 25 |  | 2670 |  | N/A |  | N/A |  |  |
| DC_7C_n26A-n78A | n26 | N/A |  | 5 |  | N/A |  | 879 |  | 30.2 |  | IMD2 |  |  |
|  | n78 | 3429 |  | 10 |  | 50 |  | 3429 |  | N/A |  | N/A |  |  |
|  | 7 | 2525 |  | 5 |  | 25 |  | 2645 |  | N/A |  | N/A |  |  |
|  | n26 | N/A |  | 5 |  | N/A |  | 875 |  | 3.3 |  | IMD5 |  |  |
|  | n78 | 3350 |  | 10 |  | 50 |  | 3350 |  | N/A |  | N/A |  |  |
|  | 7 | 2540 |  | 5 |  | 25 |  | 2660 |  | N/A |  | N/A |  |  |
|  | n26 | 835 |  | 5 |  | 25 |  | 880 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3375 |  | 29.7 |  | IMD2 |  |  |
|  | 7 | 2550 |  | 5 |  | 25 |  | 2670 |  | N/A |  | N/A |  |  |
|  | n26 | 835 |  | 5 |  | 25 |  | 880 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3430 |  | 9.7 |  | IMD4 |  |  |
| DC_7A-28A_n1A | 7 | 2535 |  | 5 |  | 25 |  | 2655 |  | N/A |  | N/A |  |  |
| DC_7A-7A-28A_n1A | 28 | N/A |  | 5 |  | N/A |  | 780 |  | 4.3 |  | IMD5 |  |  |
|  | n1 | 1950 |  | 5 |  | 25 |  | 2165 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 5 |  | N/A |  | 2665 |  | 29.0 |  | IMD2 |  |  |
|  | 28 | 730 |  | 5 |  | 25 |  | 785 |  | N/A |  | N/A |  |  |
|  | n1 | 1935 |  | 5 |  | 25 |  | 2125 |  | N/A |  | N/A |  |  |
| DC_7A-28A_n2A | 7 | N/A |  | 10 |  | N/A |  | 2630 |  | 27.6 |  | IMD2 |  |  |
|  | 28 | 730 |  | 5 |  | 25 |  | 785 |  | N/A |  | N/A |  |  |
|  | n2 | 1900 |  | 5 |  | 25 |  | 1980 |  | N/A |  | N/A |  |  |
| DC_7A-28A_n3A DC_7C-28A_n3A | 7 | 2543 |  | 5 |  | 25 |  | 2663 |  | N/A |  | N/A |  |  |
|  | 28 | N/A |  | 5 |  | N/A |  | 796.0 |  | 20.0 |  | IMD2 |  |  |
|  | n3 | 1747 |  | 5 |  | 25 |  | 1842 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 5 |  | N/A |  | 2685 |  | 18 |  | IMD3 |  |  |
|  | 28 | 745 |  | 5 |  | 25 |  | 800 |  | N/A |  | N/A |  |  |
|  | n3 | 1715 |  | 5 |  | 25 |  | 1810 |  | N/A |  | N/A |  |  |
| DC_7A-28A_n5A DC_7C-28A_n5A | 7 | 2540 |  | 5 |  | 25 |  | 2725 |  | N/A |  | N/A |  |  |
|  | 28 | N/A |  | 5 |  | N/A |  | 776 |  | 4.4 |  | IMD5 |  |  |
|  | n5 | 829 |  | 5 |  | 25 |  | 854 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 5 |  | N/A |  | 2630 |  | 5.9 |  | IMD5 |  |  |
|  | 28 | 730 |  | 5 |  | 25 |  | 785 |  | N/A |  | N/A |  |  |
|  | n5 | 840 |  | 5 |  | 25 |  | 874 |  | N/A |  | N/A |  |  |
| DC_7A-28A_n20A | 7 | N/A |  | 5 |  | N/A |  | 2640 |  | 5.9 |  | IMD5 |  |  |
|  | 28 | 728 |  | 5 |  | 25 |  | 783 |  | N/A |  | N/A |  |  |
|  | n20 | 842 |  | 5 |  | 25 |  | 801 |  | N/A |  | N/A |  |  |
|  | 7 | 2505 |  | 5 |  | 25 |  | 2625 |  | N/A |  | N/A |  |  |
|  | n20 | 859 |  | 5 |  | 25 |  | 818 |  | N/A |  | N/A |  |  |
|  | 28 | N/A |  | 5 |  | N/A |  | 787 |  | 17.4 |  | IMD3 |  |  |
| DC_7A-28A_n40A | 7 | N/A |  | 5 |  | N/A |  | 2630 |  | 5.9 |  | IMD5 |  |  |
|  | 28 | 743 |  | 5 |  | 25 |  | 798 |  | N/A |  | N/A |  |  |
|  | n40 | 2310 |  | 5 |  | 25 |  | 2310 |  | N/A |  | N/A |  |  |
| DC_7A-28A_n66A DC_7C-28A_n66A | 7 | N/A |  | 10 |  | N/A |  | 2682 |  | 16.9 |  | IMD3 |  |  |
|  | 28 | 743 |  | 5 |  | 25 |  | 798 |  | N/A |  | N/A |  |  |
|  | n66 | 1712.5 |  | 5 |  | 25 |  | 2112.5 |  | N/A |  | N/A |  |  |
|  | 7 | 2543 |  | 5 |  | 25 |  | 2663 |  | N/A |  | N/A |  |  |
|  | 28 | N/A |  | 5 |  | N/A |  | 796 |  | 20.0 |  | IMD2 |  |  |
|  | n66 | 1747 |  | 5 |  | 25 |  | 2147 |  | N/A |  | N/A |  |  |
| DC_7A-28A_n78A | 7 | 2567.5 |  | 5 |  | 25 |  | 2687.5 |  | N/A |  | N/A |  |  |
|  | 28 | N/A |  | 5 |  | N/A |  | 782.5 |  | 28.8 |  | IMD2 |  |  |
|  | n78 | 3350 |  | 10 |  | 50 |  | 3350 |  | N/A |  | N/A |  |  |
|  | 7 | 2567.5 |  | 5 |  | 25 |  | 2687.5 |  | N/A |  | N/A |  |  |
|  | 28 | N/A |  | 5 |  | N/A |  | 782.5 |  | 3.0 |  | IMD5 |  |  |
|  | n78 | 3460 |  | 10 |  | 50 |  | 3460 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 5 |  | N/A |  | 2650 |  | 30.5 |  | IMD2 |  |  |
|  | 28 | 740 |  | 5 |  | 25 |  | 795 |  | N/A |  | N/A |  |  |
|  | n78 | 3390 |  | 10 |  | 50 |  | 3390 |  | N/A |  | N/A |  |  |
| DC_7A_n28A-n78A DC_7C_n28A-n78A | 7 | 2565 |  | 5 |  | 25 |  | 2685 |  | N/A |  | N/A |  |  |
|  | n28 | 745 |  | 5 |  | 25 |  | 800 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3310 |  | 29.7 |  | IMD2 |  |  |
|  | 7 | 2565 |  | 5 |  | 25 |  | 2685 |  | N/A |  | N/A |  |  |
|  | n78 | 3365 |  | 10 |  | 50 |  | 3365 |  | N/A |  | N/A |  |  |
|  | n28 | N/A |  | 5 |  | N/A |  | 800 |  | 28.8 |  | IMD2 |  |  |
| DC_7A-29A_n78A DC_7C-29A_n78A DC_7A-7A-29A_n78A | 7 | 2540 |  | 5 |  | 25 |  | 2660 |  | N/A |  | N/A |  |  |
|  | 29 | N/A |  | N/A |  | N/A |  | 720 |  | 3.0 |  | IMD5 |  |  |
|  | n78 | 3450 |  | 10 |  | 50 |  | 3450 |  | N/A |  | N/A |  |  |
| DC_7A-32A_n1A | n1 | 1977.5 |  | 5 |  | 25 |  | 2167.5 |  | N/A |  | N/A |  |  |
|  | 7 | 2502.5 |  | 5 |  | 25 |  | 2622.5 |  | N/A |  | N/A |  |  |
|  | 32 | N/A |  | 5 |  | N/A |  | 1454.5 |  | 15.2 |  | IMD3 |  |  |
| DC_7A-32A_n3A | 7 | 1775 |  | 5 |  | 25 |  | 1870 |  | N/A |  | N/A |  |  |
|  | n3 | 2510 |  | 10 |  | 50 |  | 2630 |  | N/A |  | N/A |  |  |
|  | 32 | N/A |  | 5 |  | N/A |  | 1470 |  | 10.5 |  | IMD4 |  |  |
| DC_7A-32A_n78A | n78 | 3560.5 |  | 10 |  | 50 |  | 3560.5 |  | N/A |  | N/A |  |  |
|  | 7 | 2517.5 |  | 5 |  | 25 |  | 2637.5 |  | N/A |  | N/A |  |  |
|  | 32 | N/A |  | 5 |  | N/A |  | 1474.5 |  | 17.6 |  | IMD3 |  |  |
|  | n78 | 3311 |  | 10 |  | 50 |  | 3311 |  | N/A |  | N/A |  |  |
|  | 7 | 2565 |  | 5 |  | 25 |  | 2685 |  | N/A |  | N/A |  |  |
|  | 32 | N/A |  | 5 |  | N/A |  | 1492 |  | 4.9 |  | IMD4 |  |  |
| DC_7A-40A_n1A DC_7A-40C_n1A | n1 | 1970 |  | 5 |  | 25 |  | 2160 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 5 |  | N/A |  | 2650 |  | 32.1 |  | IMD3 |  |  |
|  | 40 | 2310 |  | 5 |  | 25 |  | 2310 |  | N/A |  | N/A |  |  |
| DC_7A_n40A-n77A | 7 | 2520 |  | 5 |  | 25 |  | 2640 |  | N/A |  | N/A |  |  |
| DC_7A_n40A-n77(2A) | n40 | N/A |  | 5 |  | N/A |  | 2360 |  | 9.2 |  | IMD4 |  |  |
| DC_7A-7A_n40A-n77A DC_7A-7A_n40A-n77(2A) | n77 | 3700 |  | 10 |  | 50 |  | 3700 |  | N/A |  | N/A |  |  |
| DC_7A-40A_n78A DC_7A-40C_n78A | 7 | N/A |  | 5 |  | N/A |  | 2630 |  | 10.1 |  | IMD4 |  |  |
|  | 40 | 2310 |  | 5 |  | 25 |  | 2310 |  | N/A |  | N/A |  |  |
|  | n78 | 3625 |  | 10 |  | 50 |  | 3625 |  | N/A |  | N/A |  |  |
|  | 7 | 2510 |  | 5 |  | 25 |  | 2630 |  | N/A |  | N/A |  |  |
|  | 40 | N/A |  | 5 |  | N/A |  | 2310 |  | 8.7 |  | IMD4 |  |  |
|  | n78 | 3785 |  | 10 |  | 50 |  | 3785 |  | N/A |  | N/A |  |  |
| DC_7A_n40A-n78A | 7 | 2520 |  | 10 |  | 50 |  | 2640 |  | N/A |  | N/A |  |  |
| DC_7A_n40A-n78C | n40 | N/A |  | 5 |  | N/A |  | 2360 |  | 8.7 |  | IMD4 |  |  |
| DC_7A-7A_n40A-n78A DC_7A-7A_n40A-n78C | n78 | 3700 |  | 10 |  | 50 |  | 3700 |  | N/A |  | N/A |  |  |
| DC_7A-46A_n78A6 | 7 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
|  | 46 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | IMD2, IMD5 |  |  |
|  | n78 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
| DC_7A-66A_n5A DC_7C-66A_n5A DC_7A-66A-66A_n5A DC_7C-66A-66A_n5A DC_7A-7A-66A_n5A DC_7A-7A-66A-66A_n5A | 7 | N/A |  | 10 |  | N/A |  | 2625 |  | 30.0 |  | IMD26 |  |  |
|  | 66 | 1775 |  | 10 |  | 50 |  | 2175 |  | N/A |  | N/A |  |  |
|  | n5 | 846.5 |  | 5 |  | 25 |  | 891.5 |  | N/A |  | N/A |  |  |
| DC_7A-66A_n7A DC_7A-66A-66A_n7A | 7 | N/A |  | 10 |  | N/A |  | 2675 |  | 15 |  | IMD4 |  |  |
|  | 66 | 1730 |  | 5 |  | 25 |  | 2130 |  | N/A |  | N/A |  |  |
|  | n7 | 2515 |  | 10 |  | 50 |  | 2635 |  | N/A |  | N/A |  |  |
| DC_7A-66A_n28A | 7 | N/A |  | 5 |  | N/A |  | 2685 |  | 18.0 |  | IMD3 |  |  |
|  | 66 | 1715 |  | 5 |  | 25 |  | 2115 |  | N/A |  | N/A |  |  |
|  | n28 | 745 |  | 5 |  | 25 |  | 800 |  | N/A |  | N/A |  |  |
| DC_7A-66A_n77A DC_7A-7A-66A_n77A DC_7A-7A-66A_n77(2A) DC_7A-66A_n77(2A) DC_7C-66A_n77A DC_7C-66A_n77(2A) | 7 | 2550 |  | 5 |  | 25 |  | 2685 |  | N/A |  | N/A |  |  |
|  | 66 | N/A |  | 5 |  | N/A |  | 2150 |  | 8.7 |  | IMD4 ¦2*fB7-2*fn77¦ |  |  |
|  | n77 | 3625 |  | 10 |  | 50 |  | 3475 |  | N/A |  | N/A |  |  |
|  | 66 | 1715 |  | 5 |  | 25 |  | 2115 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 5 |  | N/A |  | 2670 |  | 5.2 |  | IMD5 |  |  |
|  | n77 | 4190 |  | 10 |  | 50 |  | 4190 |  | N/A |  | N/A |  |  |
|  | 66 | 1720 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 5 |  | N/A |  | 2640 |  | 3.4 |  | IMD5 |  |  |
|  | n77 | 3900 |  | 10 |  | 50 |  | 3900 |  | N/A |  | N/A |  |  |
| DC_7A_n66A-n77A DC_7A-7A_n66A-n77A DC_7C_n66A-n77A | 7 | 2550 |  | 5 |  | 25 |  | 2685 |  | N/A |  | N/A |  |  |
|  | n66 | N/A |  | 5 |  | N/A |  | 2150 |  | 8.7 |  | IMD4 |  |  |
|  | n77 | 3625 |  | 10 |  | 50 |  | 3625 |  | N/A |  | N/A |  |  |
|  | 7 | 2542 |  | 5 |  | 25 |  | 2662 |  | N/A |  | N/A |  |  |
|  | n66 | 1740 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 3344 |  | 16.0 |  | IMD3 |  |  |
|  | 7 | 2520 |  | 5 |  | 25 |  | 2640 |  | N/A |  | N/A |  |  |
|  | n66 | 1760 |  | 5 |  | 25 |  | 2160 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 4040 |  | 4.2 |  | IMD5 |  |  |
| DC_7A-66A_n78A DC_7C-66A_n78A DC_7A-7A-66A_n78A DC_7A-66A-66A_n78A DC_7A-7A-66A-66A_n78A DC_7C-66A-66A_n78A DC_7A_n66A-n78A DC_7A-7A_n66A-n78A DC_7C_n66A-n78A DC_7A-66A_n78(2A) DC_7C-66A_n78(2A) DC_7A-7A-66A_n78(2A) DC_7A-66A-66A_n78(2A) DC_7A-7A-66A-66A_n78(2A) DC_7C-66A-66A_n78(2A) | 7 | 2550 |  | 5 |  | 25 |  | 2685 |  | N/A |  | N/A |  |  |
|  | 66/n66 | N/A |  | 5 |  | N/A |  | 2150 |  | 8.7 |  | IMD4 |  |  |
|  | n78 | 3625 |  | 10 |  | 50 |  | 3475 |  | N/A |  | N/A |  |  |
| DC_7A_n66A-n78A DC_7A-7A_n66A-n78A DC_7C_n66A-n78A | 7 | 2542 |  | 5 |  | 25 |  | 2662 |  | N/A |  | N/A |  |  |
|  | n66 | 1740 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3344 |  | 16.0 |  | IMD3 |  |  |
| DC_7A-71A_n2A | n2 | 1859 |  | 5 |  | 25 |  | 1933 |  | N/A |  | N/A |  |  |
| DC_7A-71A_n2(2A) | 7 | 2505 |  | 5 |  | 25 |  | 2625 |  | N/A |  | N/A |  |  |
|  | 71 | N/A |  | 5 |  | N/A |  | 646 |  | 30.8 |  | IMD2 |  |  |
| DC_7A-71A_n25A | 7 | 2530 |  | 5 |  | 25 |  | 2530 |  | N/A |  | N/A |  |  |
|  | 71 | N/A |  | 5 |  | N/A |  | 630 |  | 28.7 |  | IMD24 |  |  |
|  | n25 | 1900 |  | 5 |  | 25 |  | 1980 |  | N/A |  | N/A |  |  |
| DC_7A-71A_n77A DC_7A-71A_n77(2A) | 7 | N/A |  | 5 |  | N/A |  | 2670 |  | 29.6 |  | IMD21 |  |  |
|  | 71 | 680 |  | 5 |  | 25 |  | 634 |  | N/A |  | N/A |  |  |
|  | n77 | 3350 |  | 10 |  | 50 |  | 3350 |  | N/A |  | N/A |  |  |
| DC_7A_n71A-n77A | 7 | 2505 |  | 5 |  | 25 |  | 2625 |  | N/A |  | N/A |  |  |
|  | n71 | 666 |  | 5 |  | 25 |  | 620 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 3837 |  | 16.0 |  | IMD3 |  |  |
| DC_7A-71A_n78A DC_7A-71A_n78(2A) | 7 | N/A |  | 5 |  | N/A |  | 2670 |  | 29.6 |  | IMD2 |  |  |
|  | 71 | 680 |  | 5 |  | 25 |  | 634 |  | N/A |  | N/A |  |  |
|  | n78 | 3350 |  | 10 |  | 50 |  | 3350 |  | N/A |  | N/A |  |  |
|  | 7 | 2540 |  | 5 |  | 25 |  | 2660 |  | N/A |  | N/A |  |  |
|  | 71 | N/A |  | 5 |  | N/A |  | 640 |  | 3.0 |  | IMD5 |  |  |
|  | n78 | 3490 |  | 10 |  | 50 |  | 3490 |  | N/A |  | N/A |  |  |
| DC_7A_n71A-n78A | 7 | 2550 |  | 5 |  | 25 |  | 2670 |  | N/A |  | N/A |  |  |
|  | n71 | 693 |  | 5 |  | 25 |  | 647 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3714 |  | 9.7 |  | IMD4 |  |  |
|  | 7 | 2555 |  | 5 |  | 25 |  | 2675 |  | N/A |  | N/A |  |  |
|  | n78 | 3520 |  | 10 |  | 50 |  | 3520 |  | N/A |  | N/A |  |  |
|  | n71 | N/A |  | 5 |  | N/A |  | 625 |  | 3.9 |  | IMD5 |  |  |
| DC_7A_n75A-n78A | n78 | 3560.5 |  | 10 |  | 50 |  | 3560.5 |  | N/A |  | N/A |  |  |
|  | 7 | 2517.5 |  | 5 |  | 25 |  | 2637.5 |  | N/A |  | N/A |  |  |
|  | n75 | N/A |  | 5 |  | N/A |  | 1474.5 |  | 17.6 |  | IMD3 |  |  |
|  | n78 | 3311 |  | 10 |  | 50 |  | 3311 |  | N/A |  | N/A |  |  |
|  | 7 | 2565 |  | 5 |  | 25 |  | 2685 |  | N/A |  | N/A |  |  |
|  | n75 | N/A |  | 5 |  | N/A |  | 1492 |  | 4.9 |  | IMD4 |  |  |
| DC_7A_n78A-n79A DC_7A_n78A-n79C DC_7A-7A_n78A-n79A | 7 | 2520 |  | 5 |  | 25 |  | 2640 |  | N/A |  | N/A |  |  |
|  | n78 | 3600 |  | 10 |  | 50 |  | 3600 |  | N/A |  | N/A |  |  |
|  | n79 | N/A |  | 10 |  | N/A |  | 4680 |  | 20.6 |  | IMD34,9,13 |  |  |
|  | 7 | 2565 |  | 5 |  | 25 |  | 2685 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3770 |  | 6.4 |  | IMD413 |  |  |
|  | n79 | 4450 |  | 10 |  | 50 |  | 4450 |  | N/A |  | N/A |  |  |
| DC_7A_SUL_n78A-n80A | n80 | 1730 |  | 5 |  | 25 |  |  |  | N/A |  | N/A |  |  |
|  | 7 | N/A |  | 10 |  | N/A |  | 2655 |  | 13 |  | IMD4 |  |  |
| DC_7_n78-n105 | 7 | 2520 |  | 5 |  | 25 |  | 2640 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3700 |  | 9.7 |  | IMD4 |  |  |
|  | n105 | 670 |  | 5 |  | 25 |  | 619 |  | N/A |  | N/A |  |  |
|  | 7 | 2555 |  | 5 |  | 25 |  | 2675 |  | N/A |  | N/A |  |  |
|  | n78 | 3520 |  | 10 |  | 50 |  | 3520 |  | N/A |  | N/A |  |  |
|  | n105 | N/A |  | 5 |  | N/A |  | 625 |  | 3.9 |  | IMD5 |  |  |
| DC_8A_n1A-n28A | 8 | 910 |  | 5 |  | 25 |  | 955 |  | N/A |  | N/A |  |  |
|  | n1 | 1965 |  | 5 |  | 25 |  | 2155 |  | N/A |  | N/A |  |  |
|  | n28 | N/A |  | 5 |  | N/A |  | 765 |  | 11.6 |  | IMD4 |  |  |
| DC_8A_n1A-n40A | 8 | 885 |  | 5 |  | 25 |  | 930 |  | N/A |  | N/A |  |  |
|  | n40 | 2395 |  | 5 |  | 25 |  | 2395 |  | N/A |  | N/A |  |  |
|  | n1 | N/A |  | 5 |  | N/A |  | 2135 |  | 3.3 |  | IMD5 |  |  |
| DC_8A_n1A-n77A DC_8B_n1A-n77A | 8 | 900 |  | 5 |  | 25 |  | 945 |  | N/A |  | N/A |  |  |
|  | n1 | 1945 |  | 5 |  | 25 |  | 2135 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 3745 |  | 14.9 |  | IMD31 |  |  |
|  | 8 | 910 |  | 5 |  | 25 |  | 955 |  | N/A |  | N/A |  |  |
|  | n77 | 3960 |  | 10 |  | 50 |  | 3960 |  | N/A |  | N/A |  |  |
|  | n1 | N/A |  | 5 |  | N/A |  | 2140 |  | 14.4 |  | IMD3 |  |  |
| DC_8A_n1A-n78A DC_8B_n1A-n78A | 8 | 900 |  | 5 |  | 25 |  | 945 |  | N/A |  | N/A |  |  |
|  | n1 | 1945 |  | 5 |  | 25 |  | 2135 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3745 |  | 14.9 |  | IMD3 |  |  |
| DC_8A_n1A-n79A | 8 | 900 |  | 5 |  | 25 |  | 945 |  | N/A |  | N/A |  |  |
|  | n1 | 1955 |  | 5 |  | 25 |  | 2145 |  | 8.2 |  | IMD4 |  |  |
|  | n79 | 4845 |  | 40 |  | 216 |  | 4845 |  | N/A |  | N/A |  |  |
| DC_8A-(n)3AA | 8 | 897.5 |  | 5 |  | 25 |  | 942.5 |  | N/A |  | N/A |  |  |
|  | 3 | N/A |  | 5 |  | N/A |  | 1835 |  | 4.5 |  | IMD5 |  |  |
|  | n3 | 1747.5 |  | 10 |  | 50 |  | 1842.5 |  | 6.4 |  | IMD5 |  |  |
| DC_8A_n3A-n28A | 8 | 912.5 |  | 5 |  | 25 |  | 957.5 |  | N/A |  | N/A |  |  |
|  | n3 | 1712.5 |  | 5 |  | 25 |  | 1807.5 |  | N/A |  | N/A |  |  |
|  | n28 | N/A |  | 5 |  | N/A |  | 800 |  | 30.4 |  | IMD2 |  |  |
| DC_8A_n3A-n77A DC_8A_n3A-n77(2A) | 8 | 900 |  | 5 |  | 25 |  | 945 |  | N/A |  | N/A |  |  |
| DC_8B_n3A-n77A | n3 | 1740 |  | 5 |  | 25 |  | 1835 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 3540 |  | 16.3 |  | IMD3 |  |  |
|  | 8 | 910 |  | 5 |  | 25 |  | 955 |  | N/A |  | N/A |  |  |
|  | n77 | 3640 |  | 10 |  | 50 |  | 3640 |  | N/A |  | N/A |  |  |
|  | n3 | N/A |  | 5 |  | N/A |  | 1820 |  | 16.5 |  | IMD3 |  |  |
| DC_8A_n3A-n78A | 8 | 910 |  | 5 |  | 25 |  | 955 |  | N/A |  | N/A |  |  |
|  | n3 | 1730 |  | 5 |  | 25 |  | 1825 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3550 |  | 16.1 |  | IMD3 |  |  |
|  | 8 | 910 |  | 5 |  | 25 |  | 955 |  | N/A |  | N/A |  |  |
|  | n3 | 1730 |  | 5 |  | 25 |  | 1825 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3370 |  | 4.5 |  | IMD5 |  |  |
|  | 8 | 910 |  | 5 |  | 25 |  | 955 |  | N/A |  | N/A |  |  |
|  | n3 | N/A |  | 5 |  | N/A |  | 1820 |  | 15.7 |  | IMD3 |  |  |
|  | n78 | 3640 |  | 10 |  | 50 |  | 3640 |  | N/A |  | N/A |  |  |
| DC_8A_n3A-n79A | 8 | 885 |  | 5 |  | 25 |  | 930 |  | N/A |  | N/A |  |  |
|  | n3 | 1770 |  | 5 |  | 25 |  | 1865 |  | N/A |  | N/A |  |  |
|  | n79 | N/A |  | 40 |  | N/A |  | 4425 |  | 15.7 |  | IMD39 |  |  |
|  | 8 | 910 |  | 5 |  | 25 |  | 955 |  | N/A |  | N/A |  |  |
|  | n79 | 4580 |  | 40 |  | 216 |  | 4580 |  | N/A |  | N/A |  |  |
|  | n3 | N/A |  | 5 |  | N/A |  | 1850 |  | 8.8 |  | IMD4 |  |  |
| DC_8A-11A_n1A | 11 | 1435 |  | 5 |  | 25 |  | 1483 |  | N/A |  | N/A |  |  |
| DC_8B-11A_n1A | n1 | 1940 |  | 5 |  | 25 |  | 2130 |  | N/A |  | N/A |  |  |
|  | 8 | N/A |  | 5 |  | N/A |  | 930 |  | 16.6 |  | IMD35 |  |  |
| DC_8A-11A_n77A | 8 | 910 |  | 5 |  | 25 |  | 955 |  | N/A |  | N/A |  |  |
| DC_8A-11A_n77(2A) DC_8B-11A_n77A DC_8B-11A_n77(2A) | n77 | 3311 |  | 10 |  | 50 |  | 3311 |  | N/A |  | N/A |  |  |
|  | 11 | N/A |  | 5 |  | N/A |  | 1491 |  | 18.8 |  | IMD3 |  |  |
|  | 11 | 1430.5 |  | 5 |  | 25 |  | 1478.5 |  | N/A |  | N/A |  |  |
|  | n77 | 3791 |  | 10 |  | 50 |  | 3791 |  | N/A |  | N/A |  |  |
|  | 8 | N/A |  | 5 |  | N/A |  | 930 |  | 18.2 |  | IMD3 |  |  |
| DC_8A-11A_n78A | 8 | 910 |  | 5 |  | 25 |  | 955 |  | N/A |  | N/A |  |  |
|  | n78 | 3311 |  | 10 |  | 50 |  | 3311 |  | N/A |  | N/A |  |  |
|  | 11 | N/A |  | 5 |  | N/A |  | 1491 |  | 18.8 |  | IMD3 |  |  |
|  | 11 | 1430.5 |  | 5 |  | 25 |  | 1478.5 |  | N/A |  | N/A |  |  |
|  | n78 | 3791 |  | 10 |  | 50 |  | 3791 |  | N/A |  | N/A |  |  |
|  | 8 | N/A |  | 5 |  | N/A |  | 930 |  | 18.2 |  | IMD3 |  |  |
| DC_8A-11A_n79A | 8 | 882.5 |  | 5 |  | 25 |  | 927.5 |  | N/A |  | N/A |  |  |
|  | n79 | 4980 |  | 40 |  | 216 |  | 4980 |  | N/A |  | N/A |  |  |
|  | 11 | N/A |  | 5 |  | N/A |  | 1478.4 |  | 1.2 |  | IMD5 |  |  |
|  | 11 | 1435 |  | 5 |  | 25 |  | 1483 |  | N/A |  | N/A |  |  |
|  | n79 | 4810 |  | 40 |  | 216 |  | 4810 |  | N/A |  | N/A |  |  |
|  | 8 | N/A |  | 5 |  | N/A |  | 930 |  | 2.8 |  | IMD5 |  |  |
| DC_8-20_n1 | n1 | 1925 |  | 5 |  | 25 |  | 2115 |  | N/A |  | N/A |  |  |
|  | 8 | 910 |  | 5 |  | 25 |  | 955 |  | N/A |  | N/A |  |  |
|  | 20 | N/A |  | 5 |  | N/A |  | 805 |  | 11.5 |  | IMD4 |  |  |
| DC_8-20_n3 | n3 | 1720 |  | 5 |  | 25 |  | 1815 |  | N/A |  | N/A |  |  |
|  | 8 | 910 |  | 5 |  | 25 |  | 955 |  | N/A |  | N/A |  |  |
|  | 20 | N/A |  | 5 |  | N/A |  | 810 |  | 27 |  | IMD24 |  |  |
|  | n3 | 1770 |  | 5 |  | 25 |  | 1865 |  | N/A |  | N/A |  |  |
|  | 8 | 890 |  | 5 |  | 25 |  | 930 |  | 27 |  | IMD24 |  |  |
|  | 20 | 840 |  | 5 |  | 25 |  | 799 |  | N/A |  | N/A |  |  |
| DC_8A-20A_n28A | 8 | N/A |  | 5 |  | N/A |  | 946 |  | [23.5] |  | IMD3 |  |  |
|  | 20 | 837 |  | 5 |  | 25 |  | 796 |  | N/A |  | N/A |  |  |
|  | n28 | 728 |  | 5 |  | 25 |  | 773 |  | N/A |  | N/A |  |  |
| DC_8A-20A_n78A | 8 | 890 |  | 5 |  | 25 |  | 935 |  | N/A |  | N/A |  |  |
|  | n78 | 3470 |  | 10 |  | 50 |  | 3470 |  | N/A |  | N/A |  |  |
|  | 20 | N/A |  | 5 |  | N/A |  | 800 |  | 12.1 |  | IMD4 |  |  |
|  | 8 | N/A |  | 5 |  | N/A |  | 940 |  | 12.1 |  | IMD4 |  |  |
|  | n78 | 3481 |  | 10 |  | 50 |  | 3481 |  | N/A |  | N/A |  |  |
|  | 20 | 847 |  | 5 |  | 25 |  | 806 |  | N/A |  | N/A |  |  |
| DC_8A-28A_n3A | 8 | 912.5 |  | 5 |  | 25 |  | 957.5 |  | N/A |  | N/A |  |  |
|  | 28 | N/A |  | 5 |  | N/A |  | 800 |  | 30.4 |  | IMD24 |  |  |
|  | n3 | 1712.5 |  | 5 |  | 25 |  | 1807.5 |  | N/A |  | N/A |  |  |
| DC_8A_n28A-n77A | 8 | 910 |  | 5 |  | 25 |  | 955 |  | N/A |  | N/A |  |  |
|  | n28 | 743 |  | 5 |  | 25 |  | 798 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 3473 |  | 10.3 |  | IMD4 |  |  |
|  | 8 | 910 |  | 5 |  | 25 |  | 955 |  | N/A |  | N/A |  |  |
|  | n28 | N/A |  | 5 |  | N/A |  | 765 |  | 11.6 |  | IMD4 |  |  |
|  | n77 | 3495 |  | 10 |  | 50 |  | 3495 |  | N/A |  | N/A |  |  |
| DC_8A_n28A-n78A | 8 | 910 |  | 5 |  | 25 |  | 955 |  | N/A |  | N/A |  |  |
|  | n28 | 725 |  | 5 |  | 25 |  | 780 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3455 |  | 10.3 |  | IMD4 |  |  |
|  | 8 | 910 |  | 5 |  | 25 |  | 955 |  | N/A |  | N/A |  |  |
|  | n28 | N/A |  | 5 |  | N/A |  | 765 |  | 11.6 |  | IMD4 |  |  |
|  | n78 | 3495 |  | 10 |  | 50 |  | 3495 |  | N/A |  | N/A |  |  |
| DC_8A_n28A-n79A | 8 | 912.5 |  | 5 |  | 25 |  | 957.5 |  | N/A |  | N/A |  |  |
|  | n28 | 745.5 |  | 5 |  | 25 |  | 800.5 |  | N/A |  | N/A |  |  |
|  | n79 | N/A |  | 40 |  | N/A |  | 4420 |  | 0.0 |  | IMD5 |  |  |
|  | 8 | 905 |  | 5 |  | 25 |  | 950 |  | N/A |  | N/A |  |  |
|  | n79 | 4420 |  | 40 |  | 216 |  | 4420 |  | N/A |  | N/A |  |  |
|  | n28 | N/A |  | 5 |  | N/A |  | 800 |  | 3.9 |  | IMD5 |  |  |
| DC_8A-32A_n78A | 8 | 910 |  | 5 |  | 25 |  | 955 |  | N/A |  | N/A |  |  |
|  | n78 | 3311 |  | 10 |  | 50 |  | 3311 |  | N/A |  | N/A |  |  |
|  | 32 | N/A |  | 5 |  | N/A |  | 1491 |  | 18.8 |  | IMD3 |  |  |
| DC_8A-39A_n40A | 8 | 895 |  | 5 |  | 25 |  | 940 |  | 8.6 |  | IMD4 |  |  |
|  | 39 | 1900 |  | 5 |  | 25 |  | 1900 |  | N/A |  | N/A |  |  |
|  | n40 | 2370 |  | 5 |  | 25 |  | 2370 |  | N/A |  | N/A |  |  |
|  | 8 | 885 |  | 5 |  | 25 |  | 930 |  | 4.9 |  | IMD5 |  |  |
|  | 39 | 1890 |  | 5 |  | 25 |  | 1890 |  | N/A |  | N/A |  |  |
|  | n40 | 2370 |  | 5 |  | 25 |  | 2370 |  | N/A |  | N/A |  |  |
| DC_8-39_n79 | 8 | 897.5 |  | 5 |  | 25 |  | 942.5 |  | N/A |  | N/A |  |  |
|  | 39 | N/A |  | 5 |  | N/A |  | 1907.5 |  | 13.8 |  | IMD4 |  |  |
|  | n79 | 4600 |  | 40 |  | 216 |  | 4600 |  | N/A |  | N/A |  |  |
|  | 8 | 895 |  | 5 |  | 25 |  | 940 |  | 15.1 |  | IMD3 |  |  |
|  | 39 | 1900 |  | 10 |  | 50 |  | 1900 |  | N/A |  | N/A |  |  |
|  | n79 | 4740 |  | 40 |  | 216 |  | 4740 |  | N/A |  | N/A |  |  |
|  | 8 | N/A |  | 5 |  | N/A |  | 940 |  | 7.1 |  | IMD4 |  |  |
|  | 39 | 1900 |  | 5 |  | 25 |  | 1900 |  | N/A |  | N/A |  |  |
|  | n79 | 4760 |  | 40 |  | 216 |  | 4760 |  | N/A |  | N/A |  |  |
| DC_8A_n39A-n79A | 8 | 900 |  | 5 |  | 25 |  | 945 |  | N/A |  | N/A |  |  |
|  | n39 | 1890 |  | 10 |  | 50 |  | 1890 |  | N/A |  | N/A |  |  |
|  | n79 | N/A |  | 40 |  | N/A |  | 4680 |  | 15.9 |  | IMD3 |  |  |
|  | 8 | 890 |  | 5 |  | 25 |  | 935 |  | N/A |  | N/A |  |  |
|  | n39 | 1890 |  | 10 |  | 50 |  | 1890 |  | N/A |  | N/A |  |  |
|  | n79 | N/A |  | 40 |  | N/A |  | 4560 |  | 12.1 |  | IMD4 |  |  |
|  | 8 | 897.5 |  | 5 |  | 25 |  | 942.5 |  | N/A |  | N/A |  |  |
|  | n39 | N/A |  | 10 |  | N/A |  | 1907.5 |  | 13.8 |  | IMD4 |  |  |
|  | n79 | 4600 |  | 40 |  | 216 |  | 4600 |  | N/A |  | N/A |  |  |
| DC_8A-40A_n1A DC_8A-40C_n1A | 8 | N/A |  | 5 |  | N/A |  | 930 |  | 8.0 |  | IMD4 |  |  |
|  | 40 | 2395 |  | 5 |  | 25 |  | 2395 |  | N/A |  | N/A |  |  |
|  | n1 | 1930 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
| DC_8A-40A_n78A DC_8A-40C_n78A | 8 | N/A |  | 5 |  | N/A |  | 950 |  | 30.5 |  | IMD2 |  |  |
|  | 40 | 2380 |  | 5 |  | 25 |  | 2380 |  | N/A |  | N/A |  |  |
|  | n78 | 3330 |  | 10 |  | 50 |  | 3330 |  | N/A |  | N/A |  |  |
|  | 8 | N/A |  | 5 |  | N/A |  | 935 |  | 19.8 |  | IMD3 |  |  |
|  | 40 | 2320 |  | 5 |  | 25 |  | 2320 |  | N/A |  | N/A |  |  |
|  | n78 | 3705 |  | 10 |  | 50 |  | 3705 |  | N/A |  | N/A |  |  |
|  | 8 | 910 |  | 5 |  | 25 |  | 955 |  | N/A |  | N/A |  |  |
|  | 40 | N/A |  | 5 |  | N/A |  | 2395 |  | 28 |  | IMD2 |  |  |
|  | n78 | 3305 |  | 10 |  | 50 |  | 3305 |  | N/A |  | N/A |  |  |
| DC_8A_n40A-n79A | 8 | 885 |  | 5 |  | 25 |  | 930 |  | N/A |  | N/A |  |  |
|  | n40 | 2305 |  | 5 |  | 25 |  | 2305 |  | N/A |  | N/A |  |  |
|  | n79 | N/A |  | 40 |  | N/A |  | 4960 |  | 10.7 |  | IMD4 |  |  |
|  | 8 | 885 |  | 5 |  | 25 |  | 930 |  | N/A |  | N/A |  |  |
|  | n40 | N/A |  | 5 |  | N/A |  | 2305 |  | 9.2 |  | IMD4 |  |  |
|  | n79 | 4960 |  | 40 |  | 216 |  | 4960 |  | N/A |  | N/A |  |  |
| DC_8A-41A_n1A | 41 | 2500 |  | 5 |  | 25 |  | 2500 |  | N/A |  | N/A |  |  |
| DC_8A-41C_n1A | n1 | 1977 |  | 5 |  | 25 |  | 2167 |  | N/A |  | N/A |  |  |
|  | 8 | N/A |  | 5 |  | N/A |  | 931 |  | 4.5 |  | IMD5 |  |  |
| DC_8A-41A_n3A | n3 | 1780 |  | 5 |  | 25 |  | 1875 |  | N/A |  | N/A |  |  |
| DC_8A-41C_n3A | 8 | 885 |  | 5 |  | 25 |  | 930 |  | N/A |  | N/A |  |  |
|  | 41 | N/A |  | 5 |  | N/A |  | 2665 |  | 27.4 |  | IMD21 |  |  |
|  | n3 | 1715 |  | 5 |  | 25 |  | 1810 |  | N/A |  | N/A |  |  |
|  | 8 | N/A |  | 5 |  | N/A |  | 950 |  | 28.9 |  | IMD21 |  |  |
|  | 41 | 2665 |  | 5 |  | 25 |  | 2665 |  | N/A |  | N/A |  |  |
| DC_8A-41A_n77A | 8 | N/A |  | 5 |  | N/A |  | 950 |  | 29.1 |  | IMD21, 4 |  |  |
| DC_8A-41C_n77A | 41 | 2630 |  | 10 |  | 50 |  | 2630 |  | N/A |  | N/A |  |  |
|  | n77 | 3580 |  | 10 |  | 50 |  | 3580 |  | N/A |  | N/A |  |  |
|  | 8 | 895 |  | 5 |  | 25 |  | 940 |  | N/A |  | N/A |  |  |
|  | 41 | N/A |  | 5 |  | N/A |  | 2650 |  | 28.0 |  | IMD2 |  |  |
|  | n77 | 3545 |  | 10 |  | 50 |  | 3545 |  | N/A |  | N/A |  |  |
| DC_8A-41A_n78A | 8 | N/A |  | 5 |  | N/A |  | 950 |  | 29.1 |  | IMD24 |  |  |
| DC_8A-41C_n78A | 41 | 2630 |  | 5 |  | 25 |  | 2630 |  | N/A |  | N/A |  |  |
|  | n78 | 3580 |  | 10 |  | 50 |  | 3580 |  | N/A |  | N/A |  |  |
|  | 8 | 895 |  | 5 |  | 25 |  | 940 |  | N/A |  | N/A |  |  |
|  | 41 | N/A |  | 5 |  | N/A |  | 2650 |  | 28.0 |  | IMD2 |  |  |
|  | n78 | 3545 |  | 10 |  | 50 |  | 3545 |  | N/A |  | N/A |  |  |
| DC_8A_n41A-n79A | 8 | 910 |  | 5 |  | 25 |  | 955 |  | N/A |  | N/A |  |  |
|  | n41 | 2650 |  | 10 |  | 50 |  | 2650 |  | N/A |  | N/A |  |  |
|  | n79 | N/A |  | 40 |  | N/A |  | 4470 |  | 16.3 |  | IMD3 |  |  |
|  | 8 | 910 |  | 5 |  | 25 |  | 955 |  | N/A |  | N/A |  |  |
|  | n41 | N/A |  | 10 |  | N/A |  | 2650 |  | 15.5 |  | IMD3 |  |  |
|  | n79 | 4470 |  | 40 |  | 216 |  | 4470 |  | N/A |  | N/A |  |  |
| DC_8A-42A_n1A | 42 | 3405 |  | 10 |  | 50 |  | 3405 |  | N/A |  | N/A |  |  |
| DC_8A-42C_n1A | n1 | 1955 |  | 5 |  | 25 |  | 2145 |  | N/A |  | N/A |  |  |
|  | 8 | N/A |  | 5 |  | N/A |  | 945 |  | 3.3 |  | IMD5 |  |  |
| DC_8A-42A_n3A | 8 | 900 |  | 5 |  | 25 |  | 945 |  | N/A |  | N/A |  |  |
|  | n3 | 1740 |  | 5 |  | 25 |  | 1835 |  | N/A |  | N/A |  |  |
|  | 42 | N/A |  | 5 |  | N/A |  | 3540 |  | 16.3 |  | IMD3 |  |  |
| DC_8A-42A_n28A | 8 | 900 |  | 5 |  | 25 |  | 945 |  | N/A |  | N/A |  |  |
|  | n28 | 743 |  | 5 |  | 25 |  | 798 |  | N/A |  | N/A |  |  |
|  | 42 | N/A |  | 5 |  | N/A |  | 3443 |  | 8.7 |  | IMD4 |  |  |
| DC_8A-42A_n79A | 8 | 900 |  | 5 |  | 25 |  | 945 |  | N/A |  | N/A |  |  |
|  | n79 | 4470 |  | 40 |  | 216 |  | 4470 |  | N/A |  | N/A |  |  |
|  | 42 | N/A |  | 5 |  | N/A |  | 3570 |  | 28.8 |  | IMD2 |  |  |
| DC_8A_SUL_n78A-n80A | n80 | 1755 |  | 10 |  | 50 |  |  |  | N/A |  | N/A |  |  |
|  | 8 | N/A |  | 5 |  | N/A |  | 945 |  | 8 |  | IMD4 |  |  |
|  | n80 | 1750 |  | 10 |  | 50 |  |  |  | N/A |  | N/A |  |  |
|  | 8 | 900 |  | 5 |  | 25 |  | 945 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3550 |  | 8 |  | IMD33 |  |  |
| DC_11A_n1A-n77A DC_11A_n1A-n77(2A) | 11 | 1435 |  | 5 |  | 25 |  | 1483 |  | N/A |  | N/A |  |  |
|  | n1 | 1940 |  | 5 |  | 25 |  | 2130 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 3375 |  | 29.6 |  | IMD21 |  |  |
|  | 11 | 1438 |  | 5 |  | 25 |  | 1486 |  | N/A |  | N/A |  |  |
|  | n77 | 3578 |  | 10 |  | 50 |  | 3578 |  | N/A |  | N/A |  |  |
|  | n1 | N/A |  | 5 |  | N/A |  | 2140 |  | 30.8 |  | IMD21 |  |  |
| DC_11A_n3A-n28A | 11 | 1435 |  | 5 |  | 25 |  | 1483 |  | N/A |  | N/A |  |  |
|  | n3 | 1753 |  | 5 |  | 25 |  | 1848 |  | N/A |  | N/A |  |  |
|  | n28 | N/A |  | 5 |  | N/A |  | 800 |  | 3.0 |  | IMD5 |  |  |
| DC_11A_n3A-n77A DC_11A_n3A-n77(2A) | 11 | 1440 |  | 5 |  | 25 |  | 1488 |  | N/A |  | N/A |  |  |
|  | n3 | 1740 |  | 5 |  | 25 |  | 1835 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 3780 |  | 10.8 |  | IMD4 |  |  |
|  | 11 | 1440 |  | 5 |  | 25 |  | 1488 |  | N/A |  | N/A |  |  |
|  | n3 | N/A |  | 5 |  | N/A |  | 1870 |  | 29.0 |  | IMD2 |  |  |
|  | n77 | 3310 |  | 10 |  | 50 |  | 3310 |  | N/A |  | N/A |  |  |
| DC_11A_n3A-n79A | 11 | 1435 |  | 5 |  | 25 |  | 1483 |  | N/A |  | N/A |  |  |
|  | n3 | 1770 |  | 5 |  | 25 |  | 1865 |  | N/A |  | N/A |  |  |
|  | n79 | N/A |  | 40 |  | N/A |  | 4640 |  | 16.2 |  | IMD3 |  |  |
|  | 11 | 1435 |  | 5 |  | 25 |  | 1483 |  | N/A |  | N/A |  |  |
|  | n79 | 4735 |  | 40 |  | 216 |  | 4735 |  | N/A |  | N/A |  |  |
|  | n3 | N/A |  | 5 |  | N/A |  | 1865 |  | 17.8 |  | IMD3 |  |  |
| DC_11A-18A_n77A | 11 | 1443 |  | 5 |  | 25 |  | 1491 |  | N/A |  | N/A |  |  |
| DC_11A-18A_n77(2A) | n77 | 3706 |  | 10 |  | 50 |  | 3706 |  | N/A |  | N/A |  |  |
|  | 18 | N/A |  | 5 |  | N/A |  | 865 |  | 18.7 |  | IMD3 |  |  |
| DC_11A-18A_n78A | 11 | 1443 |  | 5 |  | 25 |  | 1491 |  | N/A |  | N/A |  |  |
| DC_11A-18A_n78(2A) | n78 | 3706 |  | 10 |  | 50 |  | 3706 |  | N/A |  | N/A |  |  |
|  | 18 | N/A |  | 5 |  | N/A |  | 865 |  | 18.7 |  | IMD3 |  |  |
| DC_11A_n28A-n77A DC_11A_n28A-n77(2A) | 11 | 1443 |  | 5 |  | 25 |  | 1491 |  | N/A |  | N/A |  |  |
|  | n28 | 743 |  | 5 |  | 25 |  | 798 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 3629 |  | 17.5 |  | IMD3 |  |  |
|  | 11 | 1443 |  | 5 |  | 25 |  | 1491 |  | N/A |  | N/A |  |  |
|  | n77 | 3684 |  | 10 |  | 50 |  | 3684 |  | N/A |  | N/A |  |  |
|  | n28 | N/A |  | 5 |  | N/A |  | 798 |  | 15.8 |  | IMD3 |  |  |
| DC_12A_n2A-n38A | 12 | 708 |  | 5 |  | 25 |  | 738 |  | N/A |  | N/A |  |  |
|  | n2 | 1900 |  | 5 |  | 25 |  | 1980 |  | N/A |  | N/A |  |  |
|  | n38 | N/A |  | 10 |  | N/A |  | 2608 |  | 28.7 |  | IMD2 |  |  |
| DC_12A_n2A-n41A | 12 | 708 |  | 5 |  | 25 |  | 738 |  | N/A |  | N/A |  |  |
|  | n2 | 1900 |  | 5 |  | 25 |  | 1980 |  | N/A |  | N/A |  |  |
|  | n41 | N/A |  | 5 |  | N/A |  | 2608 |  | 28.7 |  | IMD2 |  |  |
| DC_12A_n2A-n66A | 12 | 713.5 |  | 5 |  | 25 |  | 743.5 |  | N/A |  | N/A |  |  |
|  | n2 | 1907.5 |  | 5 |  | 25 |  | 1987.5 |  | 2 |  | IMD4 |  |  |
|  | n66 | 1712.5 |  | 5 |  | 25 |  | 2112.5 |  | N/A |  | N/A |  |  |
| DC_12A_n2A-n77A | 12 | 707.5 |  | 5 |  | 25 |  | 737.5 |  | N/A |  | N/A |  |  |
|  | n2 | 1880 |  | 5 |  | 25 |  | 1960 |  | 16.5 |  | IMD3 |  |  |
|  | n77 | 3375 |  | 10 |  | 50 |  | 3375 |  | N/A |  | N/A |  |  |
|  | 12 | 710 |  | 5 |  | 25 |  | 740 |  | N/A |  | N/A |  |  |
|  | n2 | 1890 |  | 5 |  | 25 |  | 1970 |  | 12 |  | IMD4 |  |  |
|  | n77 | 4100 |  | 10 |  | 50 |  | 4100 |  | N/A |  | N/A |  |  |
|  | 12 | 707.5 |  | 5 |  | 25 |  | 737.5 |  | N/A |  | N/A |  |  |
|  | n2 | 1900 |  | 5 |  | 25 |  | 1980 |  | N/A |  | N/A |  |  |
|  | n77 | 3315 |  | 10 |  | 50 |  | 3315 |  | 16.0 |  | IMD34, |  |  |
|  | 12 | 710 |  | 5 |  | 25 |  | 740 |  | N/A |  | N/A |  |  |
|  | n2 | 1870 |  | 5 |  | 25 |  | 1950 |  | N/A |  | N/A |  |  |
|  | n77 | 4000 |  | 10 |  | 50 |  | 4000 |  | 12 |  | IMD4 |  |  |
| DC_12_n2-n78 | 12 | 707.5 |  | 5 |  | 25 |  | 737.5 |  | N/A |  | N/A |  |  |
|  | n2 | N/A |  | 5 |  | N/A |  | 1960 |  | 16.5 |  | IMD3 |  |  |
|  | n78 | 3375 |  | 10 |  | 50 |  | 3375 |  | N/A |  | N/A |  |  |
|  | 12 | 707.5 |  | 5 |  | 25 |  | 737.5 |  | N/A |  | N/A |  |  |
|  | n2 | 1900 |  | 5 |  | 25 |  | 1980 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3315 |  | 16.0 |  | IMD3 |  |  |
| DC_12A_n7A-n78A, DC_12A_n7(2A)-n78A DC_12A_n7A-n78(2A) DC_12A_n7(2A)-n78(2A) | 12 | 708 |  | 5 |  | 25 |  | 738 |  | N/A |  | N/A |  |  |
|  | n7 | 2520 |  | 5 |  | 25 |  | 2640 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3624 |  | 9 |  | IMD4 |  |  |
|  | 12 | 708 |  | 5 |  | 25 |  | 738 |  | N/A |  | N/A |  |  |
|  | n78 | 3370 |  | 10 |  | 50 |  | 3370 |  | N/A |  | N/A |  |  |
|  | n7 | N/A |  | 5 |  | N/A |  | 2662 |  | 29.6 |  | IMD2 |  |  |
| DC_12A_n25A-n41A | 12 | 708 |  | 5 |  | 25 |  | 738 |  | N/A |  | N/A |  |  |
|  | n25 | 1900 |  | 5 |  | 25 |  | 1980 |  | N/A |  | N/A |  |  |
|  | n41 | N/A |  | 5 |  | N/A |  | 2608 |  | 28.7 |  | IMD2 |  |  |
|  | 12 | 710 |  | 5 |  | 25 |  | 740 |  | N/A |  | N/A |  |  |
|  | n25 | N/A |  | 5 |  | N/A |  | 1970 |  | 26 |  | IMD2 |  |  |
|  | n41 | 2680 |  | 10 |  | 50 |  | 2680 |  | N/A |  | N/A |  |  |
| DC_12A_n25A-n77A | 12 | 707.5 |  | 5 |  | 25 |  | 737.5 |  | N/A |  | N/A |  |  |
|  | n25 | N/A |  | 5 |  | N/A |  | 1960 |  | 16.5 |  | IMD3 |  |  |
|  | n77 | 3375 |  | 10 |  | 50 |  | 3375 |  | N/A |  | N/A |  |  |
|  | 12 | 710 |  | 5 |  | 25 |  | 740 |  | N/A |  | N/A |  |  |
|  | n25 | N/A |  | 5 |  | N/A |  | 1970 |  | 12.5 |  | IMD4 |  |  |
|  | n77 | 4100 |  | 10 |  | 50 |  | 4100 |  | N/A |  | N/A |  |  |
|  | 12 | 707.5 |  | 5 |  | 25 |  | 737.5 |  | N/A |  | N/A |  |  |
|  | n25 | 1900 |  | 5 |  | 25 |  | 1980 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 3315 |  | 16.0 |  | IMD3 |  |  |
|  | 12 | 710 |  | 5 |  | 25 |  | 740 |  | N/A |  | N/A |  |  |
|  | n25 | 1870 |  | 5 |  | 25 |  | 1950 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 4000 |  | 12 |  | IMD4 |  |  |
| DC_12A-30A_n2A | 12 | 708.5 |  | 5 |  | 25 |  | 738.5 |  | N/A |  | N/A |  |  |
|  | 30 | N/A |  | 5 |  | N/A |  | 2353 |  | 12.0 |  | IMD4 |  |  |
|  | n2 | 1885 |  | 5 |  | 25 |  | 1965 |  | N/A |  | N/A |  |  |
| DC_12A-30A_n5A | 12 | 702 |  | 5 |  | 25 |  | 732 |  | N/A |  | N/A |  |  |
|  | 30 | N/A |  | 5 |  | N/A |  | 2355 |  | 18.8 |  | IMD3 |  |  |
|  | n5 | 826.5 |  | 5 |  | 25 |  | 871.5 |  | N/A |  | N/A |  |  |
| DC_12A-30A_n77A DC_12A-30A_n77(2A) | 12 | N/A |  | 5 |  | N/A |  | 740 |  | 15.2 |  | IMD34 |  |  |
|  | 30 | 2310 |  | 5 |  | 25 |  | 2355 |  | N/A |  | N/A |  |  |
|  | n77 | 3880 |  | 10 |  | 50 |  | 3880 |  | N/A |  | N/A |  |  |
|  | 12 | 707.5 |  | 5 |  | 25 |  | 737.5 |  | N/A |  | N/A |  |  |
|  | 30 | N/A |  | 5 |  | N/A |  | 2355 |  | 13.2 |  | IMD3 |  |  |
|  | n77 | 3770 |  | 10 |  | 50 |  | 3770 |  | N/A |  | N/A |  |  |
| DC_12A_n41A-n66A | 12 | 713.5 |  | 5 |  | 25 |  | 743.5 |  | N/A |  | N/A |  |  |
|  | n41 | 2501 |  | 10 |  | 50 |  | 2501 |  | 20.0 |  | IMD218 |  |  |
|  | n66 | 1777.5 |  | 5 |  | 25 |  | 2177.5 |  | N/A |  | N/A |  |  |
| DC_12A-66A_n5A DC_12A-66A-66A_n5A | 12 | N/A |  | 5 |  | N/A |  | 742 |  | 9.4 |  | IMD4 |  |  |
|  | 66 | 1745 |  | 5 |  | 25 |  | 2145 |  | N/A |  | N/A |  |  |
|  | n5 | 829 |  | 5 |  | 25 |  | 874 |  | N/A |  | N/A |  |  |
| DC_12A-66A_n7A | 12 | N/A |  | 5 |  | N/A |  | 742 |  | 31 |  | IMD2 |  |  |
|  | 66 | 1773 |  | 5 |  | 25 |  | 2173 |  | N/A |  | N/A |  |  |
|  | n7 | 2515 |  | 5 |  | 25 |  | 2635 |  | N/A |  | N/A |  |  |
| DC_12A-66A_n25A | 12 | 708.5 |  | 5 |  | 25 |  | 738.5 |  | N/A |  | N/A |  |  |
|  | 66 | 1775 |  | 5 |  | 25 |  | 2175 |  | N/A |  | N/A |  |  |
|  | n25 | N/A |  | 5 |  | N/A |  | 1935 |  | 20 |  | IMD3 |  |  |
|  | 12 | 708.5 |  | 5 |  | 25 |  | 738.5 |  | N/A |  | N/A |  |  |
|  | 66 | N/A |  | 5 |  | N/A |  | 2150 |  | 4 |  | IMD5 |  |  |
|  | n25 | 1883.3 |  | 5 |  | 25 |  | 1963.3 |  | N/A |  | N/A |  |  |
|  | 12 | 708.5 |  | 5 |  | 25 |  | 738.5 |  | N/A |  | N/A |  |  |
|  | 66 | N/A |  | 5 |  | N/A |  | 2112.5 |  | 23 |  | IMD3 |  |  |
|  | n25 | 1912.5 |  | 5 |  | 25 |  | 1992.5 |  | N/A |  | N/A |  |  |
| DC_12A-66A_n41A | 12 | N/A |  | 5 |  | N/A |  | 742 |  | 31 |  | IMD2 |  |  |
|  | 66 | 1773 |  | 5 |  | 25 |  | 2173 |  | N/A |  | N/A |  |  |
|  | n41 | 2515 |  | 5 |  | 25 |  | 2515 |  | N/A |  | N/A |  |  |
| DC_12A-66A_n77A DC_12A-66A_n77(2A) | 12 | N/A |  | 5 |  | N/A |  | 740 |  | 15.2 |  | IMD311 |  |  |
| DC_12A-66A-66A_n77A | 66 | 1720 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
| DC_12A-66A-66A_n77(2A) | n77 | 4180 |  | 10 |  | 50 |  | 4180 |  | N/A |  | N/A |  |  |
|  | 12 | 707 |  | 5 |  | 25 |  | 737 |  | N/A |  | N/A |  |  |
|  | 66 | N/A |  | 5 |  | N/A |  | 2126 |  | 13.2 |  | IMD3 |  |  |
|  | n77 | 3540 |  | 10 |  | 50 |  | 3540 |  | N/A |  | N/A |  |  |
| DC_12A_n66A-n77A | 12 | 707 |  | 5 |  | 25 |  | 737 |  | N/A |  | N/A |  |  |
|  | n66 | 1726 |  | 5 |  | 25 |  | 2126 |  | 13.2 |  | IMD3 |  |  |
|  | n77 | 3540 |  | 10 |  | 50 |  | 3540 |  | N/A |  | N/A |  |  |
|  | 12 | 704 |  | 5 |  | 25 |  | 734 |  | N/A |  | N/A |  |  |
|  | n66 | 1723 |  | 5 |  | 25 |  | 2123 |  | N/A |  | N/A |  |  |
|  | n77 | 4150 |  | 10 |  | 50 |  | 4150 |  | 16.0 |  | IMD32,4 |  |  |
|  | 12 | 709 |  | 5 |  | 25 |  | 739 |  | N/A |  | N/A |  |  |
|  | n66 | 1715 |  | 5 |  | 25 |  | 2115 |  | N/A |  | N/A |  |  |
|  | n77 | 3842 |  | 10 |  | 50 |  | 3842 |  | 9 |  | IMD4 |  |  |
| DC_12A-66A_n78A | 12 | 710 |  | 5 |  | 25 |  | 740 |  | N/A |  | N/A |  |  |
|  | 66 | N/A |  | 5 |  | N/A |  | 2160 |  | 17.1 |  | IMD3 |  |  |
|  | n78 | 3580 |  | 5 |  | 25 |  | 3580 |  | N/A |  | N/A |  |  |
| DC_12A_n66A-n78A DC_12A_n66(2A)-n78A DC_12A_n66A-n78(2A) DC_12A_n66(2A)-n78(2A) | 12 | 703 |  | 5 |  | 25 |  | 733 |  | N/A |  | N/A |  |  |
|  | n66 | N/A |  | 5 |  | N/A |  | 2140 |  | 16.5 |  | IMD3 |  |  |
|  | n78 | 3546 |  | 10 |  | 50 |  | 3546 |  | N/A |  | N/A |  |  |
| DC_12A_n66A-n78A DC_12A_n66(2A)-n78A DC_12A_n66A-n78(2A) DC_12A_n66(2A)-n78(2A) | 12 | 703 |  | 5 |  | 25 |  | 733 |  | N/A |  | N/A |  |  |
|  | n66 | 1720 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3754 |  | 4.1 |  | IMD5 |  |  |
| DC_13A_n2A-n77A | 13 | 782 |  | 5 |  | 25 |  | 751 |  | N/A |  | N/A |  |  |
|  | n2 | 1896 |  | 5 |  | 25 |  | 1976 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 3460 |  | 17.3 |  | IMD3 |  |  |
|  | 13 | 782 |  | 5 |  | 25 |  | 751 |  | N/A |  | N/A |  |  |
|  | n2 | N/A |  | 5 |  | N/A |  | 1960 |  | 16.0 |  | IMD3 |  |  |
|  | n77 | 3524 |  | 10 |  | 50 |  | 3524 |  | N/A |  | N/A |  |  |
| DC_13A_n5A-n77A11 | 13 | 782 |  | 5 |  | 25 |  | 751 |  | N/A |  | N/A |  |  |
|  | n77 | 4013 |  | 10 |  | 50 |  | 4013 |  | N/A |  | N/A |  |  |
|  | n5 | N/A |  | 5 |  | N/A |  | 885 |  | 4.5 |  | IMD5 |  |  |
| DC_13A_n7A-n78A | 13 | 782 |  | 5 |  | 25 |  | 751 |  | N/A |  | N/A |  |  |
|  | n78 | 3432 |  | 10 |  | 50 |  | 3432 |  | N/A |  | N/A |  |  |
|  | n7 | N/A |  | 5 |  | N/A |  | 2650 |  | 27.9 |  | IMD2 |  |  |
|  | 13 | 749 |  | 5 |  | 25 |  | 780 |  | N/A |  | N/A |  |  |
|  | n7 | 2560 |  | 5 |  | 25 |  | 2680 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3622 |  | 9 |  | IMD4 |  |  |
|  | 13 | 782 |  | 5 |  | 25 |  | 751 |  | N/A |  | N/A |  |  |
|  | n7 | 2530 |  | 5 |  | 25 |  | 2650 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3312 |  | 29.0 |  | IMD2 |  |  |
| DC_13A_n25A-n66A | 13 | 782 |  | 5 |  | 25 |  | 751 |  | N/A |  | N/A |  |  |
|  | n25 | 1860 |  | 5 |  | 25 |  | 1940 |  | N/A |  | N/A |  |  |
|  | n66 | N/A |  | 5 |  | N/A |  | 2156 |  | 7.2 |  | IMD4 |  |  |
| DC_13A_n25A-n66A | 13 | 780 |  | 5 |  | 25 |  | 749 |  | N/A |  | N/A |  |  |
|  | n25 | N/A |  | 5 |  | N/A |  | 1940 |  | 6.2 |  | IMD4 |  |  |
|  | n66 | 1750 |  | 5 |  | 25 |  | 2150 |  | N/A |  | N/A |  |  |
| DC_13A-46A_n2A5 | 13 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
|  | 46 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | IMD4 |  |  |
|  | n2 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
| DC_13A-46A_n66A5 | 13 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
|  | 46 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | IMD4, IMD5 |  |  |
|  | n66 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
| DC_13A-46A_n77A5 DC_13A-46A-46A_n77A5 | 13 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
|  | 46 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | IMD3, IMD4, IMD5 |  |  |
|  | n77 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
| DC_13A_n48A-n66A | 13 | 782 |  | 5 |  | 25 |  | 751 |  | N/A |  | N/A |  |  |
|  | n48 | N/A |  | 5 |  | N/A |  | 3584 |  | 2.8 |  | IMD5 |  |  |
|  | n66 | 1716 |  | 5 |  | 25 |  | 2116 |  | N/A |  | N/A |  |  |
|  | 13 | 782 |  | 5 |  | 25 |  | 751 |  | N/A |  | N/A |  |  |
|  | n48 | 3695 |  | 5 |  | 25 |  | 3695 |  | N/A |  | N/A |  |  |
|  | n66 | N/A |  | 5 |  | N/A |  | 2131 |  | 17.1 |  | IMD3 |  |  |
| DC_13A-66A_n2A DC_13A-66A-66A_n2A | 13 | 782 |  | 5 |  | 25 |  | 751 |  | N/A |  | N/A |  |  |
| DC_13A-66B_n2A | 66 | N/A |  | 5 |  | N/A |  | 2156 |  | 7..2 |  | IMD4 |  |  |
| DC_13A-66C_n2A | n2 | 1860 |  | 5 |  | 25 |  | 1940 |  | N/A |  | N/A |  |  |
| DC_13A-66A_n5A | 13 | N/A |  | 5 |  | N/A |  | 750 |  | 9.4 |  | IMD4 |  |  |
| DC_13A-66A-66A_n5A | 66 | 1770 |  | 5 |  | 25 |  | 2170 |  | N/A |  | N/A |  |  |
|  | n5 | 840 |  | 5 |  | 25 |  | 885 |  | N/A |  | N/A |  |  |
| DC_13A-66A_n48A DC_13A-66A_n48B DC_13A-66A-66A_n48A DC_13A-66A-66A_n48B | 13 | 782 |  | 5 |  | 25 |  | 751 |  | N/A |  | N/A |  |  |
|  | 66 | N/A |  | 5 |  | N/A |  | 2131 |  | 17.1 |  | IMD3 |  |  |
|  | n48 | 3695 |  | 5 |  | 25 |  | 3695 |  | N/A |  | N/A |  |  |
| DC_13A-66A_n77A | 13 | 782 |  | 5 |  | 25 |  | 751 |  | N/A |  | N/A |  |  |
| DC_13A-66A_n77C DC_13A-66A-66A_n77A DC_13A-66A-66A_n77C | 66 | N/A |  | 5 |  | N/A |  | 2156 |  | 17.1 |  | IMD3 |  |  |
|  | n77 | 3720 |  | 10 |  | 50 |  | 3720 |  | N/A |  | N/A |  |  |
| DC_13A-66A_n77A11 | 13 | N/A |  | 5 |  | N/A |  | 750 |  | 15.2 |  | IMD3 |  |  |
| DC_13A-66A_n77C11 DC_13A-66A-66A_n77A11 DC_13A-66A-66A_n77C11 | 66 | 1710 |  | 5 |  | 25 |  | 2110 |  | N/A |  | N/A |  |  |
|  | n77 | 4170 |  | 10 |  | 50 |  | 4170 |  | N/A |  | N/A |  |  |
| DC_14A-30A_n5A | 14 | 795 |  | 5 |  | 25 |  | 765 |  | N/A |  | N/A |  |  |
|  | 30 | N/A |  | 5 |  | N/A |  | 2353 |  | 5.9 |  | IMD5 |  |  |
|  | n5 | 827 |  | 5 |  | 25 |  | 872 |  | N/A |  | N/A |  |  |
| DC_14A-30A_n77A DC_14A-30A_n77(2A) | 14 | N/A |  | 5 |  | N/A |  | 763 |  | 15.2 |  | IMD34 |  |  |
|  | 30 | 2310 |  | 5 |  | 25 |  | 2355 |  | N/A |  | N/A |  |  |
|  | n77 | 3857 |  | 10 |  | 50 |  | 3857 |  | N/A |  | N/A |  |  |
|  | 14 | 793 |  | 5 |  | 25 |  | 763 |  | N/A |  | N/A |  |  |
|  | 30 | N/A |  | 5 |  | N/A |  | 2355 |  | 13.2 |  | IMD3 |  |  |
|  | n77 | 3941 |  | 10 |  | 50 |  | 3941 |  | N/A |  | N/A |  |  |
| DC_14A-66A_n2A DC_14A-66A-66A_n2A | 14 | 793 |  | 5 |  | 25 |  | 763 |  | N/A |  | N/A |  |  |
|  | 66 | N/A |  | 5 |  | N/A |  | 2162 |  | 7.6 |  | IMD4 |  |  |
|  | n2 | 1874 |  | 5 |  | 25 |  | 1954 |  | N/A |  | N/A |  |  |
| DC_14A-66A_n5A | 14 | N/A |  | 5 |  | N/A |  | 762 |  | 9.4 |  | IMD4 |  |  |
|  | 66 | 1740 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | n5 | 834 |  | 5 |  | 25 |  | 879 |  | N/A |  | N/A |  |  |
| DC_14A-66A_n77A DC_14A-66A_n77(2A) | 14 | N/A |  | 5 |  | N/A |  | 763 |  | 15.2 |  | IMD311 |  |  |
| DC_14A-66A-66A_n77A | 66 | 1712.5 |  | 5 |  | 25 |  | 2112.5 |  | N/A |  | N/A |  |  |
| DC_14A-66A-66A_n77(2A) | n77 | 4188 |  | 10 |  | 50 |  | 4188 |  | N/A |  | N/A |  |  |
|  | 14 | 793 |  | 5 |  | 25 |  | 763 |  | N/A |  | N/A |  |  |
|  | 66 | N/A |  | 5 |  | N/A |  | 2155 |  | 13.2 |  | IMD3 |  |  |
|  | n77 | 3741 |  | 10 |  | 50 |  | 3741 |  | N/A |  | N/A |  |  |
| DC_18A_n3A-n41A | 18 | 820 |  | 5 |  | 25 |  | 865 |  | N/A |  | N/A |  |  |
|  | n3 | 1720 |  | 5 |  | 25 |  | 1815 |  | N/A |  | N/A |  |  |
|  | n41 | N/A |  | 10 |  | N/A |  | 2540 |  | 29.4 |  | IMD2 |  |  |
|  | 18 | 820 |  | 5 |  | 25 |  | 865 |  | N/A |  | N/A |  |  |
|  | n41 | 2670 |  | 10 |  | 50 |  | 2670 |  | N/A |  | N/A |  |  |
|  | n3 | N/A |  | 5 |  | N/A |  | 1850 |  | 28.2 |  | IMD2 |  |  |
| DC_18A_n3A-n77A | 18 | 820 |  | 5 |  | 25 |  | 865 |  | N/A |  | N/A |  |  |
|  | n3 | 1770 |  | 5 |  | 25 |  | 1865 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 3410 |  | 16.3 |  | IMD3 |  |  |
|  | 18 | 820 |  | 5 |  | 25 |  | 865 |  | N/A |  | N/A |  |  |
|  | n3 | N/A |  | 5 |  | N/A |  | 1865 |  | 15.7 |  | IMD3 |  |  |
|  | n77 | 3505 |  | 10 |  | 50 |  | 3505 |  | N/A |  | N/A |  |  |
| DC_18A_n3A-n78A | 18 | 820 |  | 5 |  | 25 |  | 865 |  | N/A |  | N/A |  |  |
|  | n3 | 1750 |  | 5 |  | 25 |  | 1845 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3390 |  | 15.2 |  | IMD33 |  |  |
| DC_18A-28A_n77A DC_18A_n28A-n77A | 18 | 820 |  | 5 |  | 25 |  | 865 |  | N/A |  | N/A |  |  |
|  | 28/n28 | N/A |  | 5 |  | N/A |  | 778 |  | 4.4 |  | IMD5 |  |  |
|  | n77 | 4058 |  | 10 |  | 50 |  | 4058 |  | N/A |  | N/A |  |  |
| DC_18A-28A_n77A | 18 | N/A |  | 5 |  | N/A |  | 865 |  | 3.9 |  | IMD5 |  |  |
|  | 28 | 723 |  | 5 |  | 25 |  | 778 |  | N/A |  | N/A |  |  |
|  | n77 | 3757 |  | 10 |  | 50 |  | 3757 |  | N/A |  | N/A |  |  |
| DC_18A-28A_n78A | 18 | N/A |  | 5 |  | N/A |  | 864 |  | 3.8 |  | IMD5 |  |  |
|  | 28 | 723 |  | 5 |  | 25 |  | 778 |  | N/A |  | N/A |  |  |
|  | n78 | 3756 |  | 10 |  | 50 |  | 3756 |  | N/A |  | N/A |  |  |
| DC_18A_n28A-n77A DC_18A_n28A-n78A | 18 | 820 |  | 5 |  | 25 |  | 865 |  | N/A |  | N/A |  |  |
|  | n28 | 710 |  | 5 |  | 25 |  | 765 |  | N/A |  | N/A |  |  |
|  | n77/n78 | N/A |  | 10 |  | N/A |  | 3770 |  | 4.0 |  | IMD5 |  |  |
| DC_18A-41A_n3A DC_18A-41C_n3A | 18 | 820 |  | 5 |  | 25 |  | 865 |  | N/A |  | N/A |  |  |
|  | n3 | 1725 |  | 5 |  | 25 |  | 1820 |  | N/A |  | N/A |  |  |
|  | 41 | N/A |  | 5 |  | N/A |  | 2630 |  | 16.0 |  | IMD3 |  |  |
|  | 18 | N/A |  | 5 |  | N/A |  | 865 |  | 28.9 |  | IMD21 |  |  |
|  | n3 | 1765 |  | 5 |  | 25 |  | 1860 |  | N/A |  | N/A |  |  |
|  | 41 | 2630 |  | 5 |  | 25 |  | 2630 |  | N/A |  | N/A |  |  |
| DC_18A_n41A-n77A DC_18A_n41A-n77(2A) DC_18A_n41A-n78A DC_18A_n41A-n78(2A) | 18 | N/A |  | 5 |  | N/A |  | 865 |  | 3.4 |  | IMD5 |  |  |
|  | n77 | 3527.5 |  | 10 |  | 50 |  | 3527.5 |  | N/A |  | N/A |  |  |
|  | 41 | 2640 |  | 5 |  | 25 |  | 2640 |  | N/A |  | N/A |  |  |
| DC_18A_n41A-n77A DC_18A_n41A-n78A | 18 | 820 |  | 5 |  | 25 |  | 865 |  | N/A |  | N/A |  |  |
|  | n41 | 2570 |  | 5 |  | 25 |  | 2570 |  | N/A |  | N/A |  |  |
|  | n77/n78 | N/A |  | 10 |  | N/A |  | 3390 |  | 30.1 |  | IMD2 |  |  |
|  | 18 | 820 |  | 5 |  | 25 |  | 865 |  | N/A |  | N/A |  |  |
|  | n77/n78 | 3450 |  | 10 |  | 50 |  | 3450 |  | N/A |  | N/A |  |  |
|  | n41 | N/A |  | 5 |  | N/A |  | 2630 |  | 28.5 |  | IMD2 |  |  |
| DC_18A-41A_n78A DC_18A-41C_n78A | 18 | N/A |  | 5 |  | N/A |  | 865 |  | 3.4 |  | IMD5 |  |  |
|  | n78 | 3527.5 |  | 10 |  | 50 |  | 3527.5 |  | N/A |  | N/A |  |  |
|  | 41 | 2640 |  | 5 |  | 25 |  | 2640 |  | N/A |  | N/A |  |  |
| DC_19A_n1A-n77A DC_19A_n1A-n78A | 19 | 840 |  | 5 |  | 25 |  | 885 |  | N/A |  | N/A |  |  |
|  | n1 | 1975 |  | 5 |  | 25 |  | 2165 |  | N/A |  | N/A |  |  |
|  | n77/n78 | N/A |  | 10 |  | N/A |  | 3655 |  | [21.4] |  | IMD3 |  |  |
|  | 19 | 832.5 |  | 5 |  | 25 |  | 877.5 |  | N/A |  | N/A |  |  |
|  | n1 | N/A |  | 5 |  | N/A |  | 2130 |  | 17.8 |  | IMD3 |  |  |
|  | n77/n78 | 3795 |  | 10 |  | 50 |  | 3795 |  | N/A |  | N/A |  |  |
| DC_19A_n1A-n79A20 |  |  |  |  |  | N/A |  |  |  |  |  |  |  |  |
| DC_19A-21A_n77A DC_19A-21A_n78A | 19 | N/A |  | 5 |  | N/A |  | 882.5 |  | 18.7 |  | IMD3 |  |  |
|  | 21 | 1450.4 |  | 5 |  | 25 |  | 1498.4 |  | N/A |  | N/A |  |  |
|  | n77, n78 | 3783.3 |  | 10 |  | 50 |  | 3783.3 |  | N/A |  | N/A |  |  |
|  | 19 | N/A |  | 5 |  | N/A |  | 882.5 |  | 13.2 |  | IMD4 |  |  |
|  | 21 | 1450.4 |  | 5 |  | 25 |  | 1498.4 |  | N/A |  | N/A |  |  |
|  | n77, n78 | 3468.7 |  | 10 |  | 50 |  | 3468.7 |  | N/A |  | N/A |  |  |
| DC_19A-21A_n77A | 19 | 837.5 |  | 5 |  | 25 |  | 882.5 |  | N/A |  | N/A |  |  |
|  | 21 | N/A |  | 5 |  | N/A |  | 1502.5 |  | 9.0 |  | IMD4 |  |  |
|  | n77 | 4015 |  | 10 |  | 50 |  | 4015 |  | N/A |  | N/A |  |  |
| DC_19A-21A_n79A | 19 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | IMD5 |  |  |
|  | 21 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
|  | n79 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
|  | 19 | 837.5 |  | 5 |  | 25 |  | 882.2 |  | N/A |  | N/A |  |  |
|  | 21 | N/A |  | 5 |  | N/A |  | 1500 |  | 3.8 |  | IMD5 |  |  |
|  | n79 | 4850 |  | 40 |  | 216 |  | 4850 |  | N/A |  | N/A |  |  |
| DC_19A_n78A-n79A | 19 | 835 |  | 5 |  | 25 |  | 880 |  | N/A |  | N/A |  |  |
|  | n78 | 3680 |  | 10 |  | 50 |  | 3680 |  | N/A |  | N/A |  |  |
|  | n79 | N/A |  | 40 |  | N/A |  | 4515 |  | 29.3 |  | IMD2 |  |  |
|  | 19 | 835 |  | 5 |  | 25 |  | 880 |  | N/A |  | N/A |  |  |
|  | n79 | 4550 |  | 40 |  | 216 |  | 4550 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3715 |  | 28.8 |  | IMD2 |  |  |
| DC_20A-n1A_n75A | n1 | 1950.5 |  | 5 |  | 50 |  | 2140.5 |  | N/A |  | N/A |  |  |
|  | 20 | 852.5 |  | 5 |  | 25 |  | 811.5 |  | N/A |  | N/A |  |  |
|  | n75 | N/A |  | 5 |  | N/A |  | 1459.5 |  | 4.0 |  | IMD5 |  |  |
| DC_20A_n1A-n78A | 20 | 845 |  | 5 |  | 25 |  | 804 |  | N/A |  | N/A |  |  |
|  | n1 | 1940 |  | 5 |  | 25 |  | 2130 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3630 |  | 16.0 |  | IMD3 |  |  |
|  | 20 | 835 |  | 5 |  | 25 |  | 794 |  | N/A |  | N/A |  |  |
|  | n1 | N/A |  | 5 |  | N/A |  | 2120 |  | 15.3 |  | IMD3 |  |  |
|  | n78 | 3790 |  | 10 |  | 50 |  | 3790 |  | N/A |  | N/A |  |  |
| DC_20A-(n)3AA | 3 | N/A |  | 5 |  | N/A |  | 1865 |  | 3 |  | IMD4 |  |  |
|  | n3 | 1775 |  | 5 |  | 25 |  | 1870 |  | 4 |  | IMD4 |  |  |
|  | 20 | 840 |  | 5 |  | 25 |  | 799 |  | N/A |  | N/A |  |  |
| DC_20_n3-n67 | 20 | 837 |  | 5 |  | 25 |  | 796 |  | N/A |  | N/A |  |  |
|  | n3 | 1765 |  | 5 |  | 25 |  | 1860 |  | N/A |  | N/A |  |  |
|  | n67 | N/A |  | 5 |  | N/A |  | 746 |  | 9.4 |  | IMD4 |  |  |
| DC_20A_n3A-n78A | 20 | 845 |  | 5 |  | 25 |  | 804 |  | N/A |  | N/A |  |  |
|  | n3 | 1730 |  | 5 |  | 25 |  | 1825 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3420 |  | 16.1 |  | IMD3 |  |  |
|  | 20 | 845 |  | 5 |  | 25 |  | 804 |  | N/A |  | N/A |  |  |
|  | n3 | N/A |  | 5 |  | N/A |  | 1860 |  | 15.7 |  | IMD3 |  |  |
|  | n78 | 3550 |  | 10 |  | 50 |  | 3550 |  | N/A |  | N/A |  |  |
| DC_20A_n7A-n28A | 20 | 857 |  | 5 |  | 25 |  | 816 |  | N/A |  | N/A |  |  |
|  | n7 | 2512 |  | 5 |  | 25 |  | 2632 |  | N/A |  | N/A |  |  |
|  | n28 | N/A |  | 5 |  | N/A |  | 798 |  | 13.9 |  | IMD3 |  |  |
|  | 20 | 852 |  | 5 |  | 25 |  | 811 |  | N/A |  | N/A |  |  |
|  | n7 | N/A |  | 10 |  | N/A |  | 2670 |  | 5.9 |  | IMD5 |  |  |
|  | n28 | 738 |  | 5 |  | 25 |  | 793 |  | N/A |  | N/A |  |  |
| DC_20A_n7A-n78A | 20 | 845 |  | 5 |  | 25 |  | 804 |  | N/A |  | N/A |  |  |
|  | n7 | N/A |  | 5 |  | N/A |  | 2675 |  | 30.8 |  | IMD2 |  |  |
|  | n78 | 3520 |  | 10 |  | 50 |  | 3520 |  | N/A |  | N/A |  |  |
|  | 20 | 850 |  | 5 |  | 25 |  | 809 |  | N/A |  | N/A |  |  |
|  | n7 | 2550 |  | 10 |  | 50 |  | 2675 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3400 |  | 28.8 |  | IMD21 |  |  |
| DC_20A_n8A-n78A | n8 | 910 |  | 5 |  | 25 |  | 955 |  | N/A |  | N/A |  |  |
|  | 20 | 837 |  | 5 |  | 25 |  | 796 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3567 |  | 10.3 |  | IMD4 |  |  |
|  | n8 | N/A |  | 5 |  | N/A |  | 940 |  | 12.1 |  | IMD4 |  |  |
|  | n78 | 3481 |  | 10 |  | 50 |  | 3481 |  | N/A |  | N/A |  |  |
|  | 20 | 847 |  | 5 |  | 25 |  | 806 |  | N/A |  | N/A |  |  |
| DC_20A-28A_n3A | 20 | 845 |  | 5 |  | 25 |  | 804 |  | N/A |  | N/A |  |  |
|  | 28 | N/A |  | 5 |  | N/A |  | 785 |  | 9.4 |  | IMD4 |  |  |
|  | n3 | 1750 |  | 5 |  | 25 |  | 1845 |  | N/A |  | N/A |  |  |
| DC_20A-28A_n78A | 20 | 837 |  | 5 |  | 25 |  | 796 |  | N/A |  | N/A |  |  |
|  | 28 | N/A |  | 5 |  | N/A |  | 799 |  | 9.4 |  | IMD4 |  |  |
|  | n78 | 3310 |  | 10 |  | 50 |  | 3310 |  | N/A |  | N/A |  |  |
|  | 20 | N/A |  | 5 |  | N/A |  | 808 |  | 3.8 |  | IMD5 |  |  |
|  | 28 | 705.5 |  | 5 |  | 25 |  | 760.5 |  | N/A |  | N/A |  |  |
|  | n78 | 3630 |  | 10 |  | 50 |  | 3630 |  | N/A |  | N/A |  |  |
| DC_20A_n28A-n78A, DC_20A_SUL_n78A-n83A | 20 | 857 |  | 5 |  | 25 |  | 816 |  | N/A |  | N/A |  |  |
|  | n28, n83 | 743 |  | 5 |  | 25 |  | 798 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3314 |  | 8.7 |  | IMD4 |  |  |
|  | 20 | 837 |  | 5 |  | 25 |  | 796 |  | N/A |  | N/A |  |  |
|  | n78 | 3310 |  | 10 |  | 50 |  | 3310 |  | N/A |  | N/A |  |  |
|  | n28 | N/A |  | 5 |  | N/A |  | 799 |  | 9.4 |  | IMD4 |  |  |
| DC_20A-32A_n1A | n1 | 1950.5 |  | 5 |  | 50 |  | 2140.5 |  | N/A |  | N/A |  |  |
|  | 20 | 852.5 |  | 5 |  | 25 |  | 811.5 |  | N/A |  | N/A |  |  |
|  | 32 | N/A |  | 5 |  | N/A |  | 1459.5 |  | 4.0 |  | IMD5 |  |  |
| DC_20A-38A_n1A | n1 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
|  | 20 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | IMD5 |  |  |
|  | 38 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
| DC_20A-38A_n3A | 20 | 850 |  | 5 |  | 25 |  | 809 |  | N/A |  | N/A |  |  |
|  | 38 | N/A |  | 5 |  | N/A |  | 2610 |  | 28.4 |  | IMD21 |  |  |
|  | n3 | 1760 |  | 5 |  | 25 |  | 1855 |  | N/A |  | N/A |  |  |
| DC_20A-38A_n78A DC_20A-38A_n78(2A | 20 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | IMD2 |  |  |
|  | 38 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
|  | 20 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
|  | 38 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | IMD2 |  |  |
|  | n78 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
| DC_20A_n38A-n78A | 20 | 850 |  | 5 |  | 25 |  | 809 |  | N/A |  | N/A |  |  |
|  | n38 | N/A |  | 10 |  | N/A |  | 2600 |  | 30.9 |  | IMD2 |  |  |
|  | n78 | 3450 |  | 10 |  | 50 |  | 3450 |  | N/A |  | N/A |  |  |
| DC_20A-40A_n1A DC_20A-40C_n1A | 20 | N/A |  | 5 |  | N/A |  | 800 |  | 8.0 |  | IMD4 |  |  |
|  | 40 | 2330 |  | 5 |  | 25 |  | 2330 |  | N/A |  | N/A |  |  |
|  | n1 | 1930 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
| DC_20A-40A_n78A DC_20A-40C_n78A DC_20A-40A_n78(2A) DC_20A-40C_n78(2A) | 20 | N/A |  | 5 |  | N/A |  | 815 |  | 19.8 |  | IMD3 |  |  |
|  | 40 | 2302.5 |  | 5 |  | 25 |  | 2302.5 |  | N/A |  | N/A |  |  |
|  | n78 | 3790 |  | 10 |  | 50 |  | 3790 |  | N/A |  | N/A |  |  |
| DC_20A-41A_n1A | 20 | N/A |  | 5 |  | N/A |  | 800 |  | 4.5 |  | IMD5 |  |  |
| DC_20A-41C_n1A | 41 | 2510 |  | 10 |  | 50 |  | 2510 |  | N/A |  | N/A |  |  |
|  | n1 | 1940 |  | 5 |  | 25 |  | 2130 |  | N/A |  | N/A |  |  |
| DC_20A-41A_n78A | 20 | 845 |  | 5 |  | 25 |  | 804 |  | N/A |  | N/A |  |  |
| DC_20A-41C_n78A | 41 | N/A |  | 10 |  | N/A |  | 2675 |  | 29.8 |  | IMD2 |  |  |
|  | n78 | 3520 |  | 10 |  | 50 |  | 3520 |  | N/A |  | N/A |  |  |
|  | 20 | N/A |  | 5 |  | N/A |  | 798 |  | 30.8 |  | IMD24 |  |  |
|  | 41 | 2642 |  | 10 |  | 50 |  | 2642 |  | N/A |  | N/A |  |  |
|  | n78 | 3440 |  | 10 |  | 50 |  | 3440 |  | N/A |  | N/A |  |  |
| DC_20A_n41A-n78A | 20 | 845 |  | 5 |  | 25 |  | 804 |  | N/A |  | N/A |  |  |
|  | n41 | N/A |  | 10 |  | N/A |  | 2675 |  | 29.8 |  | IMD2 |  |  |
|  | n78 | 3520 |  | 10 |  | 50 |  | 3520 |  | N/A |  | N/A |  |  |
|  | 20 | 850 |  | 5 |  | 25 |  | 809 |  | N/A |  | N/A |  |  |
|  | n41 | 2550 |  | 10 |  | 50 |  | 2550 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3400 |  | 28.8 |  | IMD2 |  |  |
| DC_20A-67A_n3A | 20 | 837 |  | 5 |  | 25 |  | 796 |  | N/A |  | N/A |  |  |
|  | 67 | N/A |  | 5 |  | N/A |  | 746 |  | 9.4 |  | IMD4 |  |  |
|  | n3 | 1765 |  | 5 |  | 25 |  | 1860 |  | N/A |  | N/A |  |  |
| DC_20A_SUL_n78A-n80A | 20 | N/A |  | 5 |  | N/A |  | 806 |  | 9 |  | IMD4 |  |  |
|  | n80 | 1735 |  | 5 |  | 25 |  |  |  | N/A |  | N/A |  |  |
| DC_21A_n1A-n77A DC_21A_n1A-n78A | 21 | 1450.4 |  | 5 |  | 25 |  | 1498.4 |  | N/A |  | N/A |  |  |
|  | n1 | N/A |  | 5 |  | N/A |  | 2154.6 |  | 30.6 |  | IMD24 |  |  |
|  | n77/n78 | 3605 |  | 10 |  | 50 |  | 3605 |  | N/A |  | N/A |  |  |
| DC_21A_n1A-n79A20 |  |  |  |  |  | N/A |  |  |  |  |  |  |  |  |
| DC_21A-28A_n77A DC_21A-28A_n78A | 21 | 1452 |  | 5 |  | 25 |  | 1500 |  | N/A |  | N/A |  |  |
|  | 28 | N/A |  | 5 |  | N/A |  | 785.5 |  | 16.9 |  | IMD3 |  |  |
|  | n77/n78 | 3689.5 |  | 10 |  | 50 |  | 3689.5 |  | N/A |  | N/A |  |  |
|  | 21 | N/A |  | 5 |  | N/A |  | 1498.5 |  | 9.9 |  | IMD4 |  |  |
|  | 28 | 730.5 |  | 5 |  | 25 |  | 785.5 |  | N/A |  | N/A |  |  |
|  | n77/n78 | 3690 |  | 10 |  | 50 |  | 3690 |  | N/A |  | N/A |  |  |
| DC_21A-28A_n79A | 21 | N/A |  | 5 |  | N/A |  | 1498 |  | 5.2 |  | IMD5 |  |  |
|  | 28 | 730.5 |  | 5 |  | 25 |  | 785.5 |  | N/A |  | N/A |  |  |
|  | n79 | 4420 |  | 40 |  | 216 |  | 4420 |  | N/A |  | N/A |  |  |
| DC_21A_n28A-n77A | 21 | 1452 |  | 5 |  | 25 |  | 1500 |  | N/A |  | N/A |  |  |
| DC_21A_n28A-n78A | n28 | N/A |  | 5 |  | N/A |  | 785.5 |  | 16.9 |  | IMD39 |  |  |
|  | n77/n78 | 3689.5 |  | 10 |  | 50 |  | 3689.5 |  | N/A |  | N/A |  |  |
|  | 21 | 1452 |  | 5 |  | 25 |  | 1500 |  | N/A |  | N/A |  |  |
|  | n28 | 730.5 |  | 5 |  | 25 |  | 785.5 |  | N/A |  | N/A |  |  |
|  | n77/n78 | N/A |  | 10 |  | N/A |  | 3634.5 |  | 17.3 |  | IMD39 |  |  |
| DC_21A_n28A-n79A 17 | 21 | 1450.4 |  | 5 |  | 25 |  | 1498.4 |  | N/A |  | N/A |  |  |
|  | n28 | N/A |  | 5 |  | N/A |  | 790.5 |  | 2.8 |  | IMD5 |  |  |
|  | n79 | 4980 |  | 40 |  | 216 |  | 4980 |  | N/A |  | N/A |  |  |
|  | 21 | 1460.4 |  | 5 |  | 25 |  | 1508.4 |  | N/A |  | N/A |  |  |
|  | n28 | 735.5 |  | 5 |  | 25 |  | 790.5 |  | N/A |  | N/A |  |  |
|  | n79 | N/A |  | 40 |  | N/A |  | 4420 |  | [6.3] |  | IMD44 |  |  |
| DC_21A-42A_n1A | 21 | N/A |  | 5 |  | N/A |  | 1500 |  | 31.4 |  | IMD2 |  |  |
|  | 42 | 3450 |  | 10 |  | 50 |  | 3450 |  | N/A |  | N/A |  |  |
|  | n1 | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
| DC_21A_n78A-n79A | 21 | 1453 |  | 5 |  | 25 |  | 1501 |  | N/A |  | N/A |  |  |
|  | n78 | 3420 |  | 10 |  | 50 |  | 3420 |  | N/A |  | N/A |  |  |
|  | n79 | N/A |  | 40 |  | N/A |  | 4873 |  | 30.1 |  | IMD2 |  |  |
|  | 21 | 1453 |  | 5 |  | 25 |  | 1501 |  | N/A |  | N/A |  |  |
|  | n79 | 4940 |  | 40 |  | 216 |  | 4940 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3487 |  | 29.8 |  | IMD2 |  |  |
| DC_25A-41A_n41A DC_25A-41C_n41A DC_25A-41D_n41A | 25 | N/A |  | 5 |  | N/A |  | 1992.5 |  | 8.5 |  | IMD7 |  |  |
| DC_25A-25A-41A_n41A DC_25A-25A-41C_n41A DC_25A-25A-41D_n41A | 41 | 2502.5 |  | 5 |  | 1 (RBstart=0) |  | 2502.5 |  | N/A |  | N/A |  |  |
| DC_25A-(n)41CA DC_25A-(n)41DA DC_25A-25A-(n)41CA DC_25A-25A-(n)41DA | n41 | 2670 |  | 5 |  | 1 (RBstart=9) |  | 2670 |  | N/A |  | N/A |  |  |
| DC_25A-66A_n77A DC_25A-25A-66A_n77A | 25 | 1855 |  | 5 |  | 25 |  | 1935 |  | N/A |  | N/A |  |  |
|  | 66 | N/A |  | 5 |  | N/A |  | 2115 |  | 29.2 |  | IMD2 |  |  |
|  | n77 | 3970 |  | 10 |  | 25 |  | 3970 |  | N/A |  | N/A |  |  |
|  | 25 | 1880 |  | 5 |  | 25 |  | 1960 |  | N/A |  | N/A |  |  |
|  | 66 | N/A |  | 5 |  | N/A |  | 2140 |  | 10.4 |  | IMD4 |  |  |
|  | n77 | 3500 |  | 10 |  | 25 |  | 3500 |  | N/A |  | N/A |  |  |
|  | 25 | 1885 |  | 5 |  | 25 |  | 1965 |  | N/A |  | N/A |  |  |
|  | 66 | N/A |  | 5 |  | N/A |  | 2175 |  | 4.0 |  | IMD5 |  |  |
|  | n77 | 3915 |  | 10 |  | 25 |  | 3915 |  | N/A |  | N/A |  |  |
|  | 25 | N/A |  | 5 |  | N/A |  | 1960 |  | 32.1 |  | IMD2 |  |  |
|  | 66 | 1760 |  | 5 |  | 25 |  | 2160 |  | N/A |  | N/A |  |  |
|  | n77 | 3720 |  | 10 |  | 25 |  | 3720 |  | N/A |  | N/A |  |  |
|  | 25 | N/A |  | 5 |  | N/A |  | 1940 |  | 9.1 |  | IMD411 |  |  |
|  | 66 | 1775 |  | 5 |  | 25 |  | 2175 |  | N/A |  | N/A |  |  |
|  | n77 | 3385 |  | 10 |  | 25 |  | 3385 |  | N/A |  | N/A |  |  |
|  | 25 | N/A |  | 5 |  | N/A |  | 1935 |  | 4.2 |  | IMD5 |  |  |
|  | 66 | 1715 |  | 5 |  | 25 |  | 2115 |  | N/A |  | N/A |  |  |
|  | n77 | 3540 |  | 10 |  | 25 |  | 3540 |  | N/A |  | N/A |  |  |
| DC_25A-66A_n78A DC_25A-25A-66A_n78A | 25 | 1880 |  | 5 |  | 25 |  | 1960 |  | N/A |  | N/A |  |  |
|  | 66 | N/A |  | 5 |  | N/A |  | 2160 |  | 10.4 |  | IMD4 |  |  |
|  | n78 | 3480 |  | 10 |  | 50 |  | 3480 |  | N/A |  | N/A |  |  |
|  | 25 | N/A |  | 5 |  | N/A |  | 1960 |  | 32.1 |  | IMD2 |  |  |
|  | 66 | 1740 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | n78 | 3700 |  | 10 |  | 50 |  | 3700 |  | N/A |  | N/A |  |  |
|  | 25 | N/A |  | 5 |  | N/A |  | 1960 |  | 9.1 |  | IMD4 |  |  |
|  | 66 | 1770 |  | 5 |  | 25 |  | 2170 |  | N/A |  | N/A |  |  |
|  | n78 | 3350 |  | 10 |  | 50 |  | 3350 |  | N/A |  | N/A |  |  |
|  | 25 | N/A |  | 5 |  | N/A |  | 1980 |  | 4.2 |  | IMD5 |  |  |
|  | 66 | 1770 |  | 5 |  | 25 |  | 2170 |  | N/A |  | N/A |  |  |
|  | n78 | 3645 |  | 10 |  | 25 |  | 3645 |  | N/A |  | N/A |  |  |
| DC_28A_n1A-n40A | 28 | 743 |  | 5 |  | 25 |  | 798 |  | N/A |  | N/A |  |  |
|  | n1 | 1930 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
|  | n40 | N/A |  | 5 |  | N/A |  | 2374 |  | 10.1 |  | IMD4 |  |  |
| DC_28A_n1A-n78A | 28 | 733 |  | 5 |  | 25 |  | 788 |  | N/A |  | N/A |  |  |
|  | n1 | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3416 |  | 15.7 |  | IMD3 |  |  |
|  | 28 | 740 |  | 5 |  | 25 |  | 795 |  | N/A |  | N/A |  |  |
|  | n1 | N/A |  | 5 |  | N/A |  | 2150 |  | 15.7 |  | IMD3 |  |  |
|  | n78 | 3630 |  | 10 |  | 50 |  | 3630 |  | N/A |  | N/A |  |  |
| DC_28A_n3A-n77A | 28 | 735 |  | 5 |  | 25 |  | 790 |  | N/A |  | N/A |  |  |
|  | n3 | N/A |  | 5 |  | N/A |  | 1850 |  | 17.0 |  | IMD3 |  |  |
|  | n77 | 3320 |  | 10 |  | 50 |  | 3320 |  | N/A |  | N/A |  |  |
|  | 28 | 733 |  | 5 |  | 25 |  | 788 |  | N/A |  | N/A |  |  |
|  | n3 | 1720 |  | 5 |  | 25 |  | 1815 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 4173 |  | 15.9 |  | IMD3 |  |  |
| DC_28A_n3A-n78A | 28 | 735 |  | 5 |  | 25 |  | 790 |  | N/A |  | N/A |  |  |
|  | n3 | 1755 |  | 5 |  | 25 |  | 1850 |  | 17.0 |  | IMD3 |  |  |
|  | n78 | 3320 |  | 10 |  | 50 |  | 3320 |  | N/A |  | N/A |  |  |
|  | n3 | 1750 |  | 5 |  | 25 |  | 1845 |  | N/A |  | N/A |  |  |
|  | 28 | 743 |  | 5 |  | 25 |  | 798 |  | N/A |  | N/A |  |  |
|  | n78 | 3764 |  | 10 |  | 50 |  | 3764 |  | 4.5 |  | IMD5 |  |  |
| DC_28A_n5A-n40A | 28 | 712 |  | 5 |  | 25 |  | 767 |  | N/A |  | N/A |  |  |
|  | n5 | 826.5 |  | 5 |  | 25 |  | 871.5 |  | N/A |  | N/A |  |  |
|  | n40 | N/A |  | 5 |  | N/A |  | 2365 |  | 18.8 |  | IMD3 |  |  |
|  | 28 | 720 |  | 5 |  | 25 |  | 775 |  | N/A |  | N/A |  |  |
|  | n5 | N/A |  | 5 |  | N/A |  | 880 |  | 17.0 |  | IMD3 |  |  |
|  | n40 | 2320 |  | 5 |  | 25 |  | 2320 |  | N/A |  | N/A |  |  |
| DC_28A_n5A-n78A | 28 | 723 |  | 5 |  | 25 |  | 778 |  | N/A |  | N/A |  |  |
|  | n5 | 829 |  | 5 |  | 25 |  | 874 |  | 3.8 |  | IMD5 |  |  |
|  | n78 | 3766 |  | 10 |  | 50 |  | 3766 |  | N/A |  | N/A |  |  |
|  | 28 | 723 |  | 5 |  | 25 |  | 778 |  | N/A |  | N/A |  |  |
|  | n5 | 829 |  | 5 |  | 25 |  | 874 |  | N/A |  | N/A |  |  |
|  | n78 | 3721 |  | 10 |  | 50 |  | 3721 |  | 4.7 |  | IMD5 |  |  |
| DC_28A_n7A-n78A DC_28A_n7B-n78A | 28 | 745 |  | 5 |  | 25 |  | 800 |  | N/A |  | N/A |  |  |
|  | n7 | 2565 |  | 5 |  | 25 |  | 2685 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3310 |  | 29.7 |  | IMD2 |  |  |
|  | 28 | 740 |  | 5 |  | 25 |  | 795 |  | N/A |  | N/A |  |  |
|  | n7 | N/A |  | 5 |  | N/A |  | 2650 |  | 30.5 |  | IMD2 |  |  |
|  | n78 | 3390 |  | 10 |  | 50 |  | 3390 |  | N/A |  | N/A |  |  |
| DC_28A_n8A-n78A | 28 | 728 |  | 5 |  | 25 |  | 783 |  | N/A |  | N/A |  |  |
|  | n8 | 910 |  | 5 |  | 25 |  | 955 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3458 |  | 9.1 |  | IMD4 |  |  |
|  | 28 | 713 |  | 5 |  | 25 |  | 768 |  | N/A |  | N/A |  |  |
|  | n8 | N/A |  | 5 |  | N/A |  | 935 |  | 4.3 |  | IMD5 |  |  |
|  | n78 | 3787 |  | 10 |  | 50 |  | 3787 |  | N/A |  | N/A |  |  |
| DC_28A-38A_n1A | n1 | 1975 |  | 5 |  | 25 |  | 2165 |  | N/A |  | N/A |  |  |
|  | 28 | N/A |  | 5 |  | N/A |  | 775 |  | 4.5 |  | IMD5 |  |  |
|  | 38 | 2575 |  | 5 |  | 25 |  | 2575 |  | N/A |  | N/A |  |  |
| DC_28A-38A_n78A | 28 | 738 |  | 5 |  | 25 |  | 793 |  | N/A |  | N/A |  |  |
|  | 38 | N/A |  | 5 |  | N/A |  | 2582 |  | 29.5 |  | IMD2 |  |  |
|  | n78 | 3320 |  | 10 |  | 50 |  | 3320 |  | N/A |  | N/A |  |  |
|  | 28 | N/A |  | 5 |  | N/A |  | 793 |  | 30.8 |  | IMD24 |  |  |
|  | 38 | 2582 |  | 5 |  | 25 |  | 2582 |  | N/A |  | N/A |  |  |
|  | n78 | 3375 |  | 10 |  | 50 |  | 3375 |  | N/A |  | N/A |  |  |
| DC_28A-40A_n78A DC_28A-40C_n78A | 28 | N/A |  | 5 |  | N/A |  | 800.5 |  | 11 |  | IMD3 |  |  |
|  | 40 | 2302.5 |  | 5 |  | 25 |  | 2302.5 |  | N/A |  | N/A |  |  |
|  | n78 | 3795 |  | 10 |  | 50 |  | 3795 |  | N/A |  | N/A |  |  |
|  | 28 | 715 |  | 5 |  | 25 |  | 770 |  | N/A |  | N/A |  |  |
|  | 40 | N/A |  | 5 |  | N/A |  | 2320 |  | 15.7 |  | IMD3 |  |  |
|  | n78 | 3750 |  | 10 |  | 50 |  | 3750 |  | N/A |  | N/A |  |  |
| DC_28A-41A_n77A | 28 | 738 |  | 5 |  | 25 |  | 793 |  | N/A |  | N/A |  |  |
|  | n77 | 3380 |  | 10 |  | 50 |  | 3380 |  | N/A |  | N/A |  |  |
|  | 41 | N/A |  | 5 |  | N/A |  | 2642 |  | 29.5 |  | IMD2 |  |  |
|  | 41 | 2642 |  | 5 |  | 25 |  | 2642 |  | N/A |  | N/A |  |  |
|  | n77 | 3440 |  | 10 |  | 50 |  | 3440 |  | N/A |  | N/A |  |  |
|  | 28 | N/A |  | 5 |  | N/A |  | 798 |  | 30.8 |  | IMD2 |  |  |
|  | 41 | 2567.5 |  | 10 |  | 50 |  | 2567.5 |  | N/A |  | N/A |  |  |
|  | n77 | 3460 |  | 10 |  | 50 |  | 3460 |  | N/A |  | N/A |  |  |
|  | 28 | N/A |  | 5 |  | N/A |  | 782.5 |  | 3.0 |  | IMD5 |  |  |
| DC_28A-41A_n78A | 28 | 738 |  | 5 |  | 25 |  | 793 |  | N/A |  | N/A |  |  |
|  | n78 | 3380 |  | 10 |  | 50 |  | 3380 |  | N/A |  | N/A |  |  |
|  | 41 | N/A |  | 5 |  | N/A |  | 2642 |  | 29.5 |  | IMD2 |  |  |
|  | 41 | 2642 |  | 5 |  | 25 |  | 2642 |  | N/A |  | N/A |  |  |
|  | n78 | 3440 |  | 10 |  | 50 |  | 3440 |  | N/A |  | N/A |  |  |
|  | 28 | N/A |  | 5 |  | N/A |  | 798 |  | 30.8 |  | IMD2 |  |  |
| DC_28A-41A_n79A | 28 | 743 |  | 5 |  | 25 |  | 798 |  | N/A |  | N/A |  |  |
|  | n79 | 4739 |  | 40 |  | 216 |  | 4739 |  | N/A |  | N/A |  |  |
|  | 41 | N/A |  | 5 |  | N/A |  | 2510 |  | 8.6 |  | IMD4 |  |  |
|  | 41 | 2650 |  | 5 |  | 25 |  | 2650 |  | N/A |  | N/A |  |  |
|  | n79 | 4502 |  | 40 |  | 216 |  | 4502 |  | N/A |  | N/A |  |  |
|  | 28 | N/A |  | 5 |  | N/A |  | 798 |  | 15.9 |  | IMD3 |  |  |
| DC_28A-42A_n79A DC_28A-42A_n79C DC_28A-42C_n79A DC_28A-42C_n79C | 28 | 730 |  | 5 |  | 25 |  | 785 |  | N/A |  | N/A |  |  |
|  | 42 | N/A |  | 5 |  | N/A |  | 3420 |  | 15.3 |  | IMD3 |  |  |
|  | n79 | 4880 |  | 40 |  | 216 |  | 4880 |  | N/A |  | N/A |  |  |
|  | 28 | N/A |  | 5 |  | N/A |  | 800 |  | 16.2 |  | IMD2 |  |  |
|  | 42 | 3597.5 |  | 5 |  | 25 |  | 3597.5 |  | N/A |  | N/A |  |  |
|  | n79 | 4420 |  | 40 |  | 216 |  | 4420 |  | N/A |  | N/A |  |  |
| DC_28A-66A_n7A | 28 | N/A |  | 5 |  | N/A |  | 790 |  | 27.6 |  | IMD2 |  |  |
|  | 66 | 1715 |  | 5 |  | 25 |  | 2115 |  | N/A |  | N/A |  |  |
|  | n7 | 2505 |  | 5 |  | 50 |  | 2625 |  | N/A |  | N/A |  |  |
| DC_28A-66A_n66A | 28 | 710.5 |  | 5 |  | 25 |  | 765.5 |  | N/A |  | N/A |  |  |
|  | 66 | N/A |  | 5 |  | N/A |  | 2129 |  | 11.0 |  | IMD4 |  |  |
|  | n66 | 1775 |  | 5 |  | 25 |  | 2175 |  | N/A |  | N/A |  |  |
| DC_29A-30A_n66A | 29 | N/A |  | 5 |  | 25 |  | 719.5 |  | 4.5 |  | IMD5 |  |  |
|  | 30 | 2307.5 |  | 5 |  | 25 |  | 2352.5 |  | N/A |  | N/A |  |  |
|  | n66 | 1777.5 |  | 5 |  | 25 |  | 2177.5 |  | N/A |  | N/A |  |  |
| DC_29A-30A_n77A | 29 | N/A |  | 5 |  | N/A |  | 722 |  | 15.2 |  | IMD34 |  |  |
|  | 30 | 2310 |  | 5 |  | 25 |  | 2355 |  | N/A |  | N/A |  |  |
|  | n77 | 3898 |  | 10 |  | 50 |  | 3898 |  | N/A |  | N/A |  |  |
| DC_29A-66A_n77A | 29 | N/A |  | 5 |  | N/A |  | 722 |  | 15.2 |  | IMD311 |  |  |
| DC_29A-66A-66A_n77A | 66 | 1734 |  | 5 |  | 25 |  | 2134 |  | N/A |  | N/A |  |  |
|  | n77 | 4190 |  | 10 |  | 50 |  | 4190 |  | N/A |  | N/A |  |  |
| DC_30A-66A_n5A, DC_30A-66A-66A_n5A, DC_30A-66A-66A-66A_n5A | 30 | 2310 |  | 5 |  | 25 |  | 2355 |  | N/A |  | N/A |  |  |
|  | 66 | N/A |  | 5 |  | N/A |  | 2130 |  | 2.5 |  | IMD5 |  |  |
|  | n5 | 830 |  | 5 |  | 25 |  | 875 |  | N/A |  | N/A |  |  |
| DC_30A-66A_n77A DC_30A-66A_n77(2A) | 30 | N/A |  | 5 |  | N/A |  | 2355 |  | 29.2 |  | IMD211 |  |  |
| DC_30A-66A-66A_n77A | 66 | 1745 |  | 5 |  | 25 |  | 2145 |  | N/A |  | N/A |  |  |
| DC_30A-66A-66A_n77(2A) | n77 | 4100 |  | 10 |  | 50 |  | 4100 |  | N/A |  | N/A |  |  |
|  | 30 | N/A |  | 5 |  | N/A |  | 2355 |  | 3.4 |  | IMD5 |  |  |
|  | 66 | 1735 |  | 5 |  | 25 |  | 2135 |  | N/A |  | N/A |  |  |
|  | n77 | 3780 |  | 10 |  | 50 |  | 3780 |  | N/A |  | N/A |  |  |
|  | 30 | 2310 |  | 5 |  | 25 |  | 2355 |  | N/A |  | N/A |  |  |
|  | 66 | N/A |  | 5 |  | N/A |  | 2160 |  | 8.7 |  | IMD411 |  |  |
|  | n77 | 3390 |  | 10 |  | 50 |  | 3390 |  | N/A |  | N/A |  |  |
| DC_38A_n28A-n78A | 38 | 2615 |  | 5 |  | 25 |  | 2615 |  | N/A |  | N/A |  |  |
|  | n28 | 745 |  | 5 |  | 25 |  | 798 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3360 |  | 28.2 |  | IMD29 |  |  |
|  | 38 | 2615 |  | 5 |  | 25 |  | 2615 |  | N/A |  | N/A |  |  |
|  | n28 | N/A |  | 5 |  | N/A |  | 785 |  | 30.8 |  | IMD24 |  |  |
|  | n78 | 3400 |  | 10 |  | 25 |  | 3400 |  | N/A |  | N/A |  |  |
| DC_39A_n40A-n79A | 39 | 1917.5 |  | 5 |  | 25 |  | 1917.5 |  | N/A |  | N/A |  |  |
|  | n40 | 2302.5 |  | 5 |  | 25 |  | 2302.5 |  | N/A |  | N/A |  |  |
|  | n79 | N/A |  | 40 |  | N/A |  | 4980 |  | 5.8 |  | IMD4 |  |  |
| DC_39A_n41A-n79A | 39 | 1900 |  | 5 |  | 25 |  | 1900 |  | N/A |  | N/A |  |  |
|  | n41 | 2620 |  | 10 |  | 50 |  | 2620 |  | N/A |  | N/A |  |  |
|  | n79 | N/A |  | 40 |  | N/A |  | 4520 |  | 29.8 |  | IMD24 |  |  |
|  | 39 | 1900 |  | 5 |  | 25 |  | 1900 |  | N/A |  | N/A |  |  |
|  | n41 | N/A |  | 10 |  | N/A |  | 2620 |  | 30.2 |  | IMD24 |  |  |
|  | n79 | 4520 |  | 40 |  | 216 |  | 4520 |  | N/A |  | N/A |  |  |
| DC_40A_n1A-n78A | 40 | 2340 |  | 5 |  | 25 |  | 2340 |  | N/A |  | N/A |  |  |
|  | n1 | 1930 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3450 |  | 9.8 |  | IMD4 |  |  |
|  | 40 | 2360 |  | 5 |  | 25 |  | 2360 |  | N/A |  | N/A |  |  |
|  | n1 | N/A |  | 5 |  | N/A |  | 2140 |  | 9.1 |  | IMD4 |  |  |
|  | n78 | 3430 |  | 10 |  | 50 |  | 3430 |  | N/A |  | N/A |  |  |
| DC_41A_n1A-n77A | 41 | 2650 |  | 5 |  | 25 |  | 2650 |  | N/A |  | TDD |  |  |
| DC_41C_n1A-n77A | n1 | 1970 |  | 5 |  | 25 |  | 2160 |  | N/A |  | FDD |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 3330 |  | 19.6 |  | TDD |  |  |
|  | 41 | 2510 |  | 5 |  | 25 |  | 2510 |  | N/A |  | TDD |  |  |
|  | n77 | 4150 |  | 10 |  | 50 |  | 4150 |  | N/A |  | TDD |  |  |
|  | n1 | N/A |  | 5 |  | N/A |  | 2120 |  | 11.0 |  | FDD |  |  |
| DC_41A_n3A-n77A DC_41C_n3A-n77A DC_41A_n3A-n78A DC_41C_n3A-n78A | 41 | 2620 |  | 5 |  | 25 |  | 2620 |  | N/A |  | N/A |  |  |
|  | n3 | N/A |  | 5 |  | N/A |  | 1840 |  | 16.4 |  | IMD3 |  |  |
|  | n77/n78 | 3400 |  | 10 |  | 50 |  | 3400 |  | N/A |  | N/A |  |  |
|  | 41 | 2580 |  | 5 |  | 25 |  | 2580 |  | N/A |  | N/A |  |  |
|  | n3 | 1720 |  | 5 |  | 25 |  | 1815 |  | N/A |  | N/A |  |  |
|  | n77/n78 | N/A |  | 10 |  | N/A |  | 3440 |  | 16.8 |  | IMD34 |  |  |
| DC_41A_n28A-n77A DC_41C_n28A-n77A DC_41A_n28A-n78A DC_41C_n28A-n78A | 41 | 2580 |  | 5 |  | 25 |  | 2580 |  | N/A |  | N/A |  |  |
|  | n28 | 743 |  | 5 |  | 25 |  | 798 |  | N/A |  | N/A |  |  |
|  | n77/n78 | N/A |  | 10 |  | N/A |  | 3323 |  | 28.2 |  | IMD21 |  |  |
|  | 41 | 2642 |  | 5 |  | 25 |  | 2642 |  | N/A |  | N/A |  |  |
|  | n28 | N/A |  | 5 |  | N/A |  | 798 |  | 30.8 |  | IMD21 |  |  |
|  | n77/n78 | 3440 |  | 10 |  | 50 |  | 3440 |  | N/A |  | N/A |  |  |
| DC_46A-48A_n5A5 DC_46C-48A_n5A5 DC_46D-48A_n5A5 DC_46E-48A_n5A5 | 46 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | IMD2, IMD3 |  |  |
|  | 48 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
|  | n5 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
| DC_46A-48A_n66A5 DC_46C-48A_n66A5 DC_46D-48A_n66A5 DC_46E-48A_n66A5 | 46 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | IMD2, IMD3 |  |  |
|  | 48 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
|  | n66 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
| DC_46A-66A_n5A | 46 | N/A |  | 10 |  | N/A |  | 5163 |  | 9.0 |  | IMD4 |  |  |
| DC_46C-66A_n5A DC_46D-66A_n5A DC_46E-66A_n5A DC_46A-66A-66A_n5A DC_46C-66A-66A_n5A DC_46D-66A-66A_n5A | 66 | 1775 |  | 5 |  | 25 |  | 2175 |  | N/A |  | N/A |  |  |
|  | n5 | 847 |  | 5 |  | 25 |  | 892 |  | N/A |  | N/A |  |  |
| DC_46A-66A_n25A4 DC_46C-66A_n25A4 DC_46D-66A_n25A4 | 46 | N/A |  | 10 |  | N/A |  | 5505 |  | 16.1 |  | IMD3 |  |  |
|  | 66 | 1775 |  | 5 |  | 25 |  | 2175 |  | N/A |  | N/A |  |  |
|  | n25 | N/A |  | 5 |  | N/A |  | 1935 |  | 20 |  | IMD3 |  |  |
|  | 46 | N/A |  | 10 |  | N/A |  | 5505 |  | 16.1 |  | IMD3 |  |  |
|  | 66 | N/A |  | 5 |  | N/A |  | 2150 |  | 4 |  | IMD5 |  |  |
|  | n25 | 1883.3 |  | 5 |  | 25 |  | 1963.3 |  | N/A |  | N/A |  |  |
|  | 46 | N/A |  | 10 |  | N/A |  | 5505 |  | 16.1 |  | IMD3 |  |  |
|  | 66 | N/A |  | 5 |  | N/A |  | 2112.5 |  | 23 |  | IMD3 |  |  |
|  | n25 | 1912.5 |  | 5 |  | 25 |  | 1992.5 |  | N/A |  | N/A |  |  |
| DC_46A-66A_n77A5 DC_46A-46A-66A_n77A5 | 46 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | IMD2, IMD3 |  |  |
|  | 66 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |  |
| DC_48A-(n)12AA | 48 | 3557.5 |  | 10 |  | 50 |  | 3557.5 |  | N/A |  | N/A |  |  |
|  | 12 | N/A |  | 5 |  | N/A |  | 740.5 |  | 5.5 |  | IMD5 |  |  |
|  | n12 | 705.5 |  | 5 |  | 25 |  | 735.5 |  | 5.5 |  | IMD5 |  |  |
| DC_48A-66A_n2A DC_48C-66A_n2A DC_48D-66A_n2A DC_48E-66A_n2A | n2 | 1880 |  | 5 |  | 25 |  | 1960 |  | N/A |  | N/A |  |  |
|  | 48 | N/A |  | 10 |  | N/A |  | 3620 |  | 29.4 |  | IMD2 |  |  |
|  | 66 | 1740 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | 48 | 3560 |  | 5 |  | 25 |  | 3560 |  | N/A |  | N/A |  |  |
|  | 66 | N/A |  | 5 |  | N/A |  | 2155 |  | 12.1 |  | IMD4 |  |  |
|  | n2 | 1905 |  | 5 |  | 25 |  | 1985 |  | N/A |  | N/A |  |  |
| DC_48A-66A_n12A | 48 | 3580 |  | 5 |  | 25 |  | 3580 |  | N/A |  | N/A |  |  |
|  | 66 | N/A |  | 5 |  | N/A |  | 2160 |  | 17.1 |  | IMD3 |  |  |
|  | n12 | 710 |  | 5 |  | 25 |  | 740 |  | N/A |  | N/A |  |  |
| DC_48A-66A_n25A DC_48C-66A_n25A DC_48D-66A_n25A | 48 | 3630 |  | 20 |  | 100 |  | 3630 |  | N/A |  | N/A |  |  |
|  | 66 | N/A |  | 5 |  | N/A |  | 2130 |  | 8.3 |  | IMD4 |  |  |
|  | n25 | 1883.3 |  | 5 |  | 25 |  | 1963.3 |  | N/A |  | N/A |  |  |
|  | 48 | N/A |  | 10 |  | N/A |  | 3620 |  | 29.4 |  | IMD2 |  |  |
|  | 66 | 1740 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | n25 | 1880 |  | 5 |  | 25 |  | 1960 |  | N/A |  | N/A |  |  |
| DC_48A-66A_n66A DC_48C-66A_n66A | 48 | 3660 |  | 20 |  | 100 |  | 3660 |  | N/A |  | N/A |  |  |
| DC_48D-66A_n66A | 66 | N/A |  | 5 |  | N/A |  | 2175 |  | 4.0 |  | IMD5 |  |  |
| DC_48E-66A_n66A | n66 | 1715 |  | 5 |  | 25 |  | 2115 |  | N/A |  | N/A |  |  |
| DC_48A-66A_n71A | 48 | 3560 |  | 5 |  | 25 |  | 3560 |  | N/A |  | N/A |  |  |
|  | 66 | N/A |  | 5 |  | N/A |  | 2174 |  | 15.8 |  | IMD3 |  |  |
|  | n71 | 693 |  | 5 |  | 25 |  | 647 |  | N/A |  | N/A |  |  |
|  | 48 | N/A |  | 5 |  | N/A |  | 3697.5 |  | 13.0 |  | IMD4 |  |  |
|  | 66 | 1712.5 |  | 5 |  | 25 |  | 2112.5 |  | N/A |  | N/A |  |  |
|  | n71 | 665.5 |  | 5 |  | 25 |  | 619.5 |  | N/A |  | N/A |  |  |
| DC_66A_n2A-n41A | 66 | 1715 |  | 5 |  | 25 |  | 2115 |  | N/A |  | N/A |  |  |
|  | n2 | 1860 |  | 5 |  | 25 |  | 1940 |  | 11.0 |  | IMD4 |  |  |
|  | n41 | 2685 |  | 5 |  | 25 |  | 2685 |  | N/A |  | N/A |  |  |
| DC_66A_n2A-n66A | 66 | 1775 |  | 5 |  | 25 |  | 2175 |  | N/A |  | N/A |  |  |
|  | n2 | N/A |  | 5 |  | N/A |  | 1935 |  | 20 |  | IMD3 |  |  |
|  | n66 | 1720 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
|  | 66 | 1720 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
|  | n2 | 1870 |  | 5 |  | 25 |  | 1950 |  | N/A |  | N/A |  |  |
|  | n66 | N/A |  | 5 |  | N/A |  | 2170 |  | 4.0 |  | IMD5 |  |  |
| DC_66A_n2A-n77A | n2 | N/A |  | 5 |  | N/A |  | 1960 |  | 32.1 |  | IMD2 |  |  |
|  | 66 | 1760 |  | 5 |  | 25 |  | 2160 |  | N/A |  | N/A |  |  |
|  | n77 | 3720 |  | 10 |  | 50 |  | 3720 |  | N/A |  | N/A |  |  |
|  | n2 | 1853 |  | 5 |  | 25 |  | 1933 |  | N/A |  | N/A |  |  |
|  | 66 | 1713 |  | 5 |  | 25 |  | 2113 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 3566 |  | 29.4 |  | IMD24 |  |  |
| DC_66A_n2A-n78A | 66 | 1760 |  | 5 |  | 25 |  | 2160 |  | N/A |  | N/A |  |  |
|  | n2 | N/A |  | 5 |  | N/A |  | 1960 |  | 32.1 |  | IMD2 |  |  |
|  | n78 | 3720 |  | 10 |  | 50 |  | 3720 |  | N/A |  | N/A |  |  |
|  | 66 | 1740 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | n2 | 1880 |  | 5 |  | 25 |  | 1960 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3620 |  | 34.9 |  | IMD2 |  |  |
|  | 66 | 1740 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | n2 | 1880 |  | 5 |  | 25 |  | 1960 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3340 |  | 20.9 |  | IMD4 |  |  |
|  | 66 | 1770 |  | 5 |  | 25 |  | 2170 |  | N/A |  | N/A |  |  |
|  | n2 | N/A |  | 5 |  | N/A |  | 1960 |  | 21.1 |  | IMD4 |  |  |
|  | n78 | 3350 |  | 10 |  | 50 |  | 3350 |  | N/A |  | N/A |  |  |
|  | 66 | 1760 |  | 5 |  | 25 |  | 2160 |  | N/A |  | N/A |  |  |
|  | n2 | N/A |  | 5 |  | N/A |  | 1960 |  | 2.1 |  | IMD5 |  |  |
|  | n78 | 3620 |  | 10 |  | 50 |  | 3620 |  | N/A |  | N/A |  |  |
| DC_66A-(n)5AA | 66 | 1721 |  | 5 |  | 25 |  | 2121 |  | N/A |  | N/A |  |  |
|  | 5 | N/A |  | 5 |  | N/A |  | 878 |  | 25 |  | IMD2 |  |  |
|  | n5 | 838 |  | 5 |  | 25 |  | 883 |  | 30 |  | IMD2 |  |  |
| DC_66A_n5A-n48A | 66 | 1750 |  | 5 |  | 25 |  | 2150 |  | N/A |  | N/A |  |  |
|  | n5 | 834 |  | 5 |  | 25 |  | 879 |  | N/A |  | N/A |  |  |
|  | n48 | N/A |  | 5 |  | N/A |  | 3582 |  | 3.3 |  | IMD5 |  |  |
| DC_66A_n5A-n77A | 66 | 1770 |  | 5 |  | 25 |  | 2170 |  | N/A |  | N/A |  |  |
|  | n5 | 845 |  | 5 |  | 25 |  | 890 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 3460 |  | 16.6 |  | IMD39 |  |  |
| DC_66A_n7A-n78A, DC_66A-66A_n7A-n78 DC_66A_n7(2A)-n78A DC_66A-66A_n7(2A)-n78A DC_66A_n7A-n78(2A) DC_66A-66A_n7A-n78(2A) DC_66A-66A_n7(2A)-n78(2A) | 66 | 1730 |  | 5 |  | 25 |  | 2130 |  | N/A |  | N/A |  |  |
|  | n7 | 2560 |  | 5 |  | 25 |  | 2680 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3390 |  | 16.1 |  | IMD3 |  |  |
| DC_66A_n12A-n77A | 66 | 1720 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
|  | n12 | N/A |  | 5 |  | N/A |  | 740 |  | 15.2 |  | IMD311 |  |  |
|  | n77 | 4180 |  | 10 |  | 50 |  | 4180 |  | N/A |  | N/A |  |  |
|  | 66 | 1723 |  | 5 |  | 25 |  | 2123 |  | N/A |  | N/A |  |  |
|  | n12 | 704 |  | 5 |  | 25 |  | 734 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 4150 |  | 16.0 |  | IMD34,9,11 |  |  |
| DC_66A_n25A-n41A | 66 | 1715 |  | 5 |  | 25 |  | 2115 |  | N/A |  | N/A |  |  |
|  | n41 | 2685 |  | 10 |  | 50 |  | 2685 |  | N/A |  | N/A |  |  |
|  | n25 | 1860 |  | 5 |  | 25 |  | 1940 |  | 5 |  | 11.0 |  |  |
| DC_66A_n25A-n48A | 66 | 1740 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |  |
|  | n25 | 1880 |  | 5 |  | 25 |  | 1960 |  | N/A |  | N/A |  |  |
|  | n48 | N/A |  | 10 |  | N/A |  | 3620 |  | 29.4 |  | IMD2 |  |  |
|  | 66 | 1735 |  | 5 |  | 25 |  | 2135 |  | N/A |  | N/A |  |  |
|  | n25 | N/A |  | 5 |  | N/A |  | 1960 |  | 28.3 |  | IMD2 |  |  |
|  | n48 | 3695 |  | 5 |  | 25 |  | 3695 |  | N/A |  | N/A |  |  |
| DC_66A_n25A-n66A | 66 | 1712.5 |  | 5 |  | 25 |  | 2112.5 |  | N/A |  | N/A |  |  |
|  | n25 | 1912.5 |  | 5 |  | 25 |  | 1992.5 |  | N/A |  | N/A |  |  |
|  | n66 | N/A |  | 5 |  | N/A |  | 2117.5 |  | 23 |  | IMD3 |  |  |
|  | 66 | 1750 |  | 5 |  | 25 |  | 2150 |  | N/A |  | N/A |  |  |
|  | n25 | 1873 |  | 5 |  | 25 |  | 1953 |  | N/A |  | N/A |  |  |
|  | n66 | N/A |  | 5 |  | N/A |  | 2119 |  | 4 |  | IMD5 |  |  |
| DC_66A_n38A-n78A | 66 | 1760 |  | 5 |  | 25 |  | 2160 |  | N/A |  | N/A |  |  |
|  | n38 | 2610 |  | 10 |  | 50 |  | 2610 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3460 |  | 15.0 |  | IMD3 |  |  |
| DC_66A_n66A-n71A | 66 | 1752 |  | 5 |  | 25 |  | 2152 |  | N/A |  | N/A |  |  |
|  | n66 | N/A |  | 5 |  | N/A |  | 2118 |  | 5.0 |  | IMD4 |  |  |
|  | n71 | 693 |  | 5 |  | 25 |  | 647 |  | N/A |  | N/A |  |  |
| DC_66A_n66A-n77A | 66 | 1730 |  | 5 |  | 25 |  | 2130 |  | N/A |  | N/A |  |  |
|  | n66 | N/A |  | 5 |  | N/A |  | 2170 |  | 31 |  | IMD2 |  |  |
|  | n77 | 3900 |  | 10 |  | 50 |  | 3900 |  | N/A |  | N/A |  |  |
| DC_66A_n66A-n78A | 66 | 1775 |  | 5 |  | 25 |  | 2175 |  | N/A |  | N/A |  |  |
|  | n66 | N/A |  | 5 |  | N/A |  | 2125 |  | 2.8 |  | IMD5 |  |  |
|  | n78 | 3725 |  | 10 |  | 50 |  | 3725 |  | N/A |  | N/A |  |  |
| DC_66A-71A_n77A DC_66A-71A_n77(2A) | 66 | N/A |  | 5 |  | N/A |  | 2160 |  | 15.5 |  | IMD39 |  |  |
|  | 71 | 693 |  | 5 |  | 25 |  | 647 |  | N/A |  | N/A |  |  |
|  | n77 | 3546 |  | 10 |  | 50 |  | 3546 |  | N/A |  | N/A |  |  |
|  | 66 | 1720 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
|  | 71 | N/A |  | 5 |  | N/A |  | 640 |  | 15.3 |  | IMD311 |  |  |
|  | n77 | 4080 |  | 10 |  | 50 |  | 4080 |  | N/A |  | N/A |  |  |
| DC_66A_n71A-n77A | 66 | 1720 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
|  | n71 | 668 |  | 5 |  | 25 |  | 622 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 4108 |  | 15.9 |  | IMD34,9,11 |  |  |
|  | 66 | 1720 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
|  | n71 | N/A |  | 5 |  | N/A |  | 640 |  | 15.3 |  | IMD311 |  |  |
|  | n77 | 4080 |  | 10 |  | 50 |  | 4080 |  | N/A |  | N/A |  |  |
| DC_66A_n71A-n78A | 66 | 1712.5 |  | 5 |  | 25 |  | 2112.5 |  | N/A |  | N/A |  |  |
|  | n71 | 665.5 |  | 5 |  | 25 |  | 619.5 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3709 |  | 13.0 |  | IMD4 |  |  |
| DC_71A_n2A-n41A | n2 | 1900 |  | 5 |  | 25 |  | 1980 |  | N/A |  | N/A |  |  |
|  | n41 | N/A |  | 5 |  | N/A |  | 2586 |  | 29.2 |  | IMD2 |  |  |
|  | 71 | 686 |  | 5 |  | 50 |  | 640 |  | N/A |  | N/A |  |  |
|  | n2 | N/A |  | 5 |  | N/A |  | 1942 |  | 26 |  | IMD2 |  |  |
|  | n41 | 2610 |  | 5 |  | 25 |  | 2610 |  | N/A |  | N/A |  |  |
|  | 71 | 668 |  | 5 |  | 25 |  | 622 |  | N/A |  | N/A |  |  |
| DC_71A_n2A-n78A | n2 | 1907.5 |  | 5 |  | 25 |  | 1987.5 |  | N/A |  | N/A |  |  |
|  | 71 | 695.5 |  | 5 |  | 25 |  | 649.5 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3305 |  | 8.0 |  | IMD3 |  |  |
|  | n2 | N/A |  | 5 |  | N/A |  | 1954 |  | 16.5 |  | IMD3 |  |  |
|  | 71 | 693 |  | 5 |  | 25 |  | 647 |  | N/A |  | N/A |  |  |
|  | n78 | 3340 |  | 10 |  | 50 |  | 3340 |  | N/A |  | N/A |  |  |
| DC_71A_n2A-n77A | 71 | 695.5 |  | 5 |  | 25 |  | 649.5 |  | N/A |  | N/A |  |  |
|  | n2 | 1907.5 |  | 5 |  | 25 |  | 1987.5 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 3305 |  | 8.0 |  | IMD34,9,11 |  |  |
|  | 71 | 693 |  | 5 |  | 25 |  | 647 |  | N/A |  | N/A |  |  |
|  | n2 | N/A |  | 5 |  | N/A |  | 1954 |  | 16.5 |  | IMD39,11 |  |  |
|  | n77 | 3340 |  | 10 |  | 50 |  | 3340 |  | N/A |  | N/A |  |  |
| DC_71A_n25A-n41A | n25 | 1900 |  | 5 |  | 25 |  | 1980 |  | N/A |  | N/A |  |  |
|  | n41 | N/A |  | 5 |  | N/A |  | 2586 |  | 29.2 |  | IMD29 |  |  |
|  | 71 | 686 |  | 5 |  | 50 |  | 640 |  | N/A |  | N/A |  |  |
|  | n25 | N/A |  | 5 |  | N/A |  | 1942 |  | 26 |  | IMD2 |  |  |
|  | n41 | 2610 |  | 5 |  | 25 |  | 2610 |  | N/A |  | N/A |  |  |
|  | 71 | 668 |  | 5 |  | 25 |  | 622 |  | N/A |  | N/A |  |  |
| DC_71A_n25A-n77A | 71 | 693 |  | 5 |  | 25 |  | 647 |  | N/A |  | N/A |  |  |
|  | n25 | N/A |  | 5 |  | N/A |  | 1954 |  | 16.5 |  | IMD3 |  |  |
|  | n77 | 3340 |  | 10 |  | 50 |  | 3340 |  | N/A |  | N/A |  |  |
|  | 71 | 666 |  | 5 |  | 25 |  | 620 |  | N/A |  | N/A |  |  |
|  | n25 | N/A |  | 5 |  | N/A |  | 1982 |  | 12.5 |  | IMD4 |  |  |
|  | n77 | 3980 |  | 10 |  | 50 |  | 3980 |  | N/A |  | N/A |  |  |
|  | 71 | 695.5 |  | 5 |  | 25 |  | 649.5 |  | N/A |  | N/A |  |  |
|  | n25 | 1907.5 |  | 5 |  | 25 |  | 1987.5 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 3298.5 |  | 16 |  | IMD34 |  |  |
|  | 71 | 666 |  | 5 |  | 25 |  | 620 |  | N/A |  | N/A |  |  |
|  | n25 | 1890 |  | 5 |  | 25 |  | 1970 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 3888 |  | 12 |  | IMD4 |  |  |
| DC_71A_n38A-n78A | 71 | 693 |  | 5 |  | 25 |  | 647 |  | N/A |  | N/A |  |  |
|  | n38 | 2615 |  | 10 |  | 50 |  | 2615 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3308 |  | 29.1 |  | IMD2 |  |  |
|  | 71 | 693 |  | 5 |  | 25 |  | 647 |  | N/A |  | N/A |  |  |
|  | n78 | 3308 |  | 10 |  | 50 |  | 3308 |  | N/A |  | N/A |  |  |
|  | n38 | N/A |  | 10 |  | N/A |  | 2615 |  | 28.7 |  | IMD2 |  |  |
| DC_71A_n66A-n77A | 71 | 668 |  | 5 |  | 25 |  | 622 |  | N/A |  | N/A |  |  |
|  | n66 | 1720 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |  |
|  | n77 | N/A |  | 10 |  | N/A |  | 4108 |  | 15.9 |  | IMD34,9,11 |  |  |
|  | 71 | 690 |  | 5 |  | 25 |  | 644 |  | N/A |  | N/A |  |  |
|  | n66 | N/A |  | 5 |  | N/A |  | 2150 |  | 15.5 |  | IMD39,11 |  |  |
|  | n77 | 3530 |  | 10 |  | 50 |  | 3530 |  | N/A |  | N/A |  |  |
| DC_71A_n66A-n78A | 71 | 693 |  | 5 |  | 25 |  | 647 |  | N/A |  | N/A |  |  |
|  | n78 | 3546 |  | 10 |  | 50 |  | 3546 |  | N/A |  | N/A |  |  |
|  | n66 | N/A |  | 5 |  | N/A |  | 2160 |  | 15.5 |  | IMD3 |  |  |
|  | 71 | 665.5 |  | 5 |  | 25 |  | 619.5 |  | N/A |  | N/A |  |  |
|  | n78 | N/A |  | 10 |  | N/A |  | 3697.5 |  | 13.0 |  | IMD4 |  |  |
|  | n66 | 1712.5 |  | 5 |  | 25 |  | 2112.5 |  | N/A |  | N/A |  |  |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. NOTE 2: For DC_3A_n3A-n77A, DC_3A_n3A-n78A paired with UL_DC_3A_n3A, the 3rd DL bands n77/n78 are subject to IMD2 which MSD is not specified NOTE 3: This MSD requirement apply with both IMD2 and IMD3 products should be generated. NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: When Band 46 have self-interference problems by dual uplink CA/EN-DC, then the requirements do not apply in exclusion zone which is frequency range within (harmonics frequency region + FHD) and IMD frequency region as follow. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. DC_2A-46A_n66A; DC_2A_n66A; 2*fc_2A + fc_n66A; 2*BW_2A + BW_n66A. DC_2A-46A_n66A; DC_2A_n66A; fc_2A + 2*fc_n66A; BW_2A + 2*BW_n66A. DC_2A-46A_n77A; DC_2A_n77A; fc_2A + fc_n77A; BW_2A + BW_n77A. DC_2A-46A_n77A; DC_2A_n77A; -fc_2A + 2*fc_n77A; -BW_2A + 2*BW_n77A. DC_13A-46A_n77A; DC_13A_n77A; 2*fc_13A + fc_n77A; 2*BW_13A + BW_n77A. DC_13A-46A_n77A; DC_13A_n77A; 3*fc_13A + fc_n77A; 3*BW_13A + BW_n77A. DC_13A-46A_n2A; DC_13A_n2A; 2*fc_n2A + 2*fc_13A; 2*BW_n2A+2*BW_13A. DC_13A-46A_n77A; DC_13A_n77A; -3*fc_13A + 2*fc_n77A; -3*BW_13A + 2*BW_n77A. DC_46A-66A_n77A; DC_66A_n77A; fc_66A + fc_n77A; BW_66A + BW_n77A. DC_46A-66A_n77A; DC_66A_n77A; -fc_66A + 2*fc_n77A; -BW_66A + 2*BW_n77A. DC_13A-46A_n66A; DC_13A_n66A; 3*fc_13A + fc_n66A; BW_13A + 2*BW_n66A. DC_13A-46A_n66A; DC_13A_n66A; 2*fc_13A + 3*fc_n66A; BW_13A + 2*BW_n66A. DC_46-48A_n66A; DC_48A_n66A; fc_48A + fc_n66A; BW_48A + 2*BW_n66A. DC_46-48A_n66A; DC_48A_n66A; 2*fc_48A + fc_n66A; 2*BW_48A + BW_n66A. DC_2A-46_n5A; DC_2A_n5A; 2*fc_2A + 2*fc_n5A; BW_2A + 2*BW_n5A. DC_2A-46_n5A; DC_2A_n5A; fc_2A + 4*fc_n5A; BW_2*2A + BW_n5A. DC_46-48A_n5A; DC_48A_n5A; 2*fc_48A + fc_n5A; BW_48A + 2*BW_n5A. DC_46-48A_n5A; DC_48A_n5A; 2*fc_48A + 2*fc_n5A; BW_2*48A + BW_n5A. NOTE 6: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 7: This band is also subject to IMD2 which is not specified. The frequency range below 3400MHz in n77 is not used for this combination. NOTE 8: Band 5 is also affected by IMD5 from UL DC_2A_n12A, but MSD value is not specified as there is only partial overlap of IMD5 with DL carrier. NOTE 9: This band is subject to IMD4 also which MSD is not specified. NOTE 10: The frequency range in band n28 is restricted for this band combination to 728 – 738 MHz for the UL and 783 – 793 MHz for the DL. This band is subject to IMD2 fall in B1 also which MSD is not specified. NOTE 11: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 12: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. NOTE 13: Void NOTE 14: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 15: This band is subject to additional IMD3 for which MSD is not specified. NOTE 16: This band is subject to IMD3 also which MSD is not specified. NOTE 17: The frequency range in band n28 is restricted for this band combination to 728 – 738 MHz for the UL and 783 – 793 MHz for the DL. NOTE 18: In the MSD test configuration, the IMD center does not fall into the DL victim Fc. NOTE 19: This band is subject to 1st order triple-beat IMD3 where MSD is not specified when the UL configuration includes intra-band uplink CCs. NOTE 20: No MSD test points are specified for this combination and verification of IMD impact is not required. |

Table 7.3B.2.3.5.2-1a: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band |  |  | UL Fc (MHz) |  |  | UL/DL BW (MHz) |  |  | UL LCRB |  |  | DL Fc (MHz) |  |  | MSD (dB) |  |  | IMD order |  |  |
| DC_1A-3A_n77A DC_1A-3A_n77(2A) DC_1A-3C_n77A DC_1A-3C_n77(2A) | 1 |  |  | 1950 |  |  | 5 |  |  | 25 |  |  | 2140 |  |  | N/A |  |  | N/A |  |  |
|  | 3 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1807.5 |  |  | 37.5 |  |  | IMD21 |  |  |
|  | n77 |  |  | 3757.5 |  |  | 10 |  |  | 50 |  |  | 3757.5 |  |  | N/A |  |  | N/A |  |  |
|  | 1 |  |  | 1950 |  |  | 5 |  |  | 25 |  |  | 2140 |  |  | N/A |  |  | N/A |  |  |
|  | 3 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1870 |  |  | 20.5 |  |  | IMD41 |  |  |
|  | n77 |  |  | 3980 |  |  | 10 |  |  | 50 |  |  | 3980 |  |  | N/A |  |  | N/A |  |  |
|  | 1 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2140 |  |  | 37.0 |  |  | IMD21 |  |  |
|  | 3 |  |  | 1775 |  |  | 5 |  |  | 25 |  |  | 1870 |  |  | N/A |  |  | N/A |  |  |
|  | n77 |  |  | 3915 |  |  | 10 |  |  | 50 |  |  | 3915 |  |  | N/A |  |  | N/A |  |  |
| DC_1A-3A_n78A DC_1A-3A_n78(2A) DC_1A-3C_n78A DC_1A-3C_n78(2A) | 1 |  |  | 1950 |  |  | 5 |  |  | 25 |  |  | 2140 |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1807.5 |  |  | 37.2 |  |  |  | IMD21 |  |
|  | n78 |  |  | 3757.5 |  |  | 10 |  |  | 50 |  |  | 3757.5 |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2125 |  |  | 17.8 |  |  |  | IMD5 |  |
|  | 3 |  |  | 1775 |  |  | 5 |  |  | 25 |  |  | 1870 |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  |  | 3725 |  |  | 10 |  |  | 50 |  |  | 3725 |  |  | N/A |  |  |  | N/A |  |
| DC_1A-3A_n79A | 1 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2140 |  |  | 24.6 |  |  |  | IMD5 |  |
|  | 3 |  |  | 1750 |  |  | 5 |  |  | 25 |  |  | 1845 |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  |  | 4860 |  |  | 10 |  |  | 50 |  |  | 4860 |  |  | N/A |  |  |  | N/A |  |
| DC_1A-5A_n78A | 1 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2120 |  |  | 19.2 |  |  | IMD4 |  |  |
|  | 5 |  |  | 844 |  |  | 5 |  |  | 25 |  |  | 889 |  |  | N/A |  |  | N/A |  |  |
|  | n78 |  |  | 3670 |  |  | 10 |  |  | 52 |  |  | 3670 |  |  | N/A |  |  | N/A |  |  |
|  | 1 |  |  | 1950 |  |  | 5 |  |  | 25 |  |  | 2140 |  |  | N/A |  |  | N/A |  |  |
|  | 5 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 889 |  |  | 19.2 |  |  | IMD4 |  |  |
|  | n78 |  |  | 3421 |  |  | 10 |  |  | 52 |  |  | 3421 |  |  | N/A |  |  | N/A |  |  |
|  | 1 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2122 |  |  | 27.0 |  |  | IMD3 |  |  |
|  | 5 |  |  | 829 |  |  | 5 |  |  | 25 |  |  | 874 |  |  | N/A |  |  | N/A |  |  |
|  | n78 |  |  | 3780 |  |  | 10 |  |  | 52 |  |  | 3780 |  |  | N/A |  |  | N/A |  |  |
|  | 1 |  |  | 1975 |  |  | 5 |  |  | 25 |  |  | 2165 |  |  | N/A |  |  | N/A |  |  |
|  | 5 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 885 |  |  | 13.2 |  |  | IMD5 |  |  |
|  | n78 |  |  | 3405 |  |  | 10 |  |  | 52 |  |  | 3405 |  |  | N/A |  |  | N/A |  |  |
| DC_1A-7A_n78A | 1 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2120 |  |  | 19.2 |  |  | IMD4 |  |  |
|  | 7 |  |  | 2550 |  |  | 5 |  |  | 25 |  |  | 2670 |  |  | N/A |  |  | N/A |  |  |
|  | n78 |  |  | 3670 |  |  | 10 |  |  | 52 |  |  | 3670 |  |  | N/A |  |  | N/A |  |  |
|  | 1 |  |  | 1977.5 |  |  | 5 |  |  | 25 |  |  | 2167.5 |  |  | N/A |  |  | N/A |  |  |
|  | 7 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2627.5 |  |  | 20.2 |  |  | IMD4 |  |  |
|  | n78 |  |  | 3305 |  |  | 10 |  |  | 52 |  |  | 3305 |  |  | N/A |  |  | N/A |  |  |
|  | 1 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2140 |  |  | 19.7 |  |  | IMD4 |  |  |
|  | 7 |  |  | 2510 |  |  | 10 |  |  | 50 |  |  | 2630 |  |  | N/A |  |  | N/A |  |  |
|  | n78 |  |  | 3580 |  |  | 10 |  |  | 52 |  |  | 3580 |  |  | N/A |  |  | N/A |  |  |
| DC_1A-8A_n77A | 1 |  |  | 1955 |  |  | 5 |  |  | 25 |  |  | 2145 |  |  | N/A |  |  |  | N/A |  |
| DC_1A-8A_n77(2A) | 8 |  |  | 910 |  |  | 5 |  |  | 25 |  |  | 955 |  |  | 15.7 |  |  |  | IMD5 |  |
|  | n77 |  |  | 3410 |  |  | 10 |  |  | 50 |  |  | 3410 |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  |  | 1950 |  |  | 5 |  |  | 25 |  |  | 2140 |  |  | 23.4 |  |  |  | IMD3 |  |
|  | 8 |  |  | 910 |  |  | 5 |  |  | 25 |  |  | 955 |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  |  | 3960 |  |  | 10 |  |  | 50 |  |  | 3960 |  |  | N/A |  |  |  | N/A |  |
| DC_1A-8A_n78A | 1 |  |  | 1955 |  |  | 5 |  |  | 25 |  |  | 2145 |  |  | N/A |  |  |  | N/A |  |
| DC_1A-8A_n78(2A) | 8 |  |  | 910 |  |  | 5 |  |  | 25 |  |  | 955 |  |  | 15.7 |  |  |  | IMD5 |  |
|  | n78 |  |  | 3410 |  |  | 10 |  |  | 50 |  |  | 3410 |  |  | N/A |  |  |  | N/A |  |
| DC_1A-19A_n77A DC_1A-19A_n77(2A) | 1 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2130 |  |  | 26.7 |  |  |  | IMD3 |  |
|  | 19 |  |  | 832.5 |  |  | 5 |  |  | 25 |  |  | 877.5 |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  |  | 3795 |  |  | 10 |  |  | 50 |  |  | 3795 |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  |  | 1940 |  |  | 5 |  |  | 25 |  |  | 2130 |  |  | N/A |  |  |  | N/A |  |
|  | 19 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 880 |  |  | 18.5 |  |  |  | IMD5 |  |
|  | n77 |  |  | 3350 |  |  | 10 |  |  | 50 |  |  | 3350 |  |  | N/A |  |  |  | N/A |  |
| DC_1A-19A_n78A DC_1A-19A_n78(2A) | 1 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2130 |  |  | 26.7 |  |  |  | IMD3 |  |
|  | 19 |  |  | 832.5 |  |  | 5 |  |  | 25 |  |  | 877.5 |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  |  | 3795 |  |  | 10 |  |  | 50 |  |  | 3795 |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  |  | 1940 |  |  | 5 |  |  | 25 |  |  | 2130 |  |  | N/A |  |  |  | N/A |  |
|  | 19 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 880 |  |  | 18.5 |  |  |  | IMD5 |  |
|  | n78 |  |  | 3350 |  |  | 10 |  |  | 50 |  |  | 3350 |  |  | N/A |  |  |  | N/A |  |
| DC_1A-19A_n79A | 1 |  |  | 1950 |  |  | 5 |  |  | 25 |  |  | 2140 |  |  | N/A |  |  |  | N/A |  |
|  | 19 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 882.5 |  |  | 33.3 |  |  |  | IMD35 |  |
|  | n79 |  |  | 4782.5 |  |  | 10 |  |  | 50 |  |  | 4782.5 |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2140 |  |  | 26.1 |  |  |  | IMD4 |  |
|  | 19 |  |  | 837.5 |  |  | 5 |  |  | 25 |  |  | 882.5 |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  |  | 4652.5 |  |  | 10 |  |  | 50 |  |  | 4652.5 |  |  | N/A |  |  |  | N/A |  |
| DC_1A-21A_n77A DC_1A-21A_n77(2A) | 1 |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  |
|  | 21 |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | IMD2 |  |  |
|  | n77 |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  |
|  | 1 |  |  | 1950 |  |  | 5 |  |  | 25 |  |  | 2140 |  |  | N/A |  |  | N/A |  |  |
|  | 21 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1500 |  |  | 17.9 |  |  | IMD5 |  |  |
|  | n77 |  |  | 3605 |  |  | 10 |  |  | 50 |  |  | 3605 |  |  | N/A |  |  | N/A |  |  |
|  | 1 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2154.6 |  |  | 36.6 |  |  | IMD21 |  |  |
|  | 21 |  |  | 1450.4 |  |  | 5 |  |  | 25 |  |  | 1498.4 |  |  | N/A |  |  | N/A |  |  |
|  | n77 |  |  | 3605 |  |  | 10 |  |  | 50 |  |  | 3605 |  |  | N/A |  |  | N/A |  |  |
| DC_1A-21A_n78A DC_1A-21A_n78(2A) | 1 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2154.6 |  |  | 36.6 |  |  | IMD2 |  |  |
|  | 21 |  |  | 1450.4 |  |  | 5 |  |  | 25 |  |  | 1498.4 |  |  | N/A |  |  | N/A |  |  |
|  | n78 |  |  | 3605 |  |  | 10 |  |  | 50 |  |  | 3605 |  |  | N/A |  |  | N/A |  |  |
|  | 1 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2154.6 |  |  | 16.2 |  |  | IMD5 |  |  |
|  | 21 |  |  | 1450.4 |  |  | 5 |  |  | 25 |  |  | 1498.4 |  |  | N/A |  |  | N/A |  |  |
|  | n78 |  |  | 3647 |  |  | 10 |  |  | 50 |  |  | 3647 |  |  | N/A |  |  | N/A |  |  |
|  | 1 |  |  | 1950 |  |  | 5 |  |  | 25 |  |  | 2140 |  |  | N/A |  |  | N/A |  |  |
|  | 21 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1500 |  |  | 37.5 |  |  | IMD2 |  |  |
|  | n78 |  |  | 3450 |  |  | 10 |  |  | 50 |  |  | 3450 |  |  | N/A |  |  | N/A |  |  |
|  | 1 |  |  | 1950 |  |  | 5 |  |  | 25 |  |  | 2140 |  |  | N/A |  |  | N/A |  |  |
|  | 21 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1500 |  |  | 14.9 |  |  | IMD5 |  |  |
|  | n78 |  |  | 3675 |  |  | 10 |  |  | 50 |  |  | 3675 |  |  | N/A |  |  | N/A |  |  |
| DC_1A-21A_n79A7,8 | 1 |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  |  | N/A |  |
|  | 21 |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  |  | IMD4 |  |
|  | n79 |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  |  | N/A |  |
| DC_1A_n28A-n77A | 1 |  |  | 1950 |  |  | 5 |  |  | 25 |  |  | 2140 |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  |  | 3320 |  |  | 10 |  |  | 50 |  |  | 3320 |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  |  | N/A |  |  | 5 |  |  | 25 |  |  | 790 |  |  | 18.7 |  |  |  | IMD5 |  |
| DC_1A-41A_n77A | 1 |  |  | 1970 |  |  | 5 |  |  | 25 |  |  | 2160 |  |  | N/A |  |  |  | N/A |  |
| DC_1A-41C_n77A | 41 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2510 |  |  | 22.5 |  |  |  | IMD4 |  |
|  | n77 |  |  | 3400 |  |  | 10 |  |  | 50 |  |  | 3400 |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  |  | 1930 |  |  | 5 |  |  | 25 |  |  | 2120 |  |  | N/A |  |  |  | N/A |  |
|  | 41 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2510 |  |  | 15.6 |  |  |  | IMD5 |  |
|  | n77 |  |  | 4150 |  |  | 10 |  |  | 50 |  |  | 4150 |  |  | N/A |  |  |  | N/A |  |
| DC_1A_n41A-n77A | 1 |  |  | 1975 |  |  | 5 |  |  | 25 |  |  | 2165 |  |  | N/A |  |  |  | N/A |  |
|  | n41 |  |  | N/A |  |  | 10 |  |  | N/A |  |  | 2515 |  |  | 22.0 |  |  |  | IMD41 |  |
|  | n77 |  |  | 3410 |  |  | 10 |  |  | 50 |  |  | 3410 |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  |  | 1970 |  |  | 5 |  |  | 25 |  |  | 2160 |  |  | N/A |  |  |  | N/A |  |
|  | n41 |  |  | 2650 |  |  | 10 |  |  | 25 |  |  | 2650 |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  |  | N/A |  |  | 10 |  |  | N/A |  |  | 3330 |  |  | 28.2 |  |  |  | IMD31,5 |  |
| DC_1A-42A_n79A DC_1A-42C_n79A DC_1A-42D_n79A DC_1A-42E_n79A | 1 |  |  | 1977.5 |  |  | 5 |  |  | 25 |  |  | 2167.5 |  |  | N/A |  |  |  | N/A |  |
|  | 42 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 3490 |  |  | 25.8 |  |  |  | IMD5 |  |
|  | n79 |  |  | 4420 |  |  | 10 |  |  | 50 |  |  | 4420 |  |  | N/A |  |  |  | N/A |  |
| DC_1A_n78A-n79A | 1 |  |  | 1950 |  |  | 5 |  |  | 25 |  |  | 2140 |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  |  | 3410 |  |  | 10 |  |  | 50 |  |  | 3410 |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  |  | N/A |  |  | 10 |  |  | N/A |  |  | 4870 |  |  | 24.9 |  |  |  | IMD31 |  |
|  | 1 |  |  | 1950 |  |  | 5 |  |  | 25 |  |  | 2140 |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  |  | N/A |  |  | 10 |  |  | N/A |  |  | 3490 |  |  | 22.6 |  |  |  | IMD5 |  |
|  | n79 |  |  | 4670 |  |  | 10 |  |  | 50 |  |  | 4670 |  |  | N/A |  |  |  | N/A |  |
| DC_2A_n2A-n77A DC_2A_n2A-n77C | 2 |  |  | 1875 |  |  | 5 |  |  | 25 |  |  | 1955 |  |  | N/A |  |  | N/A |  |  |
|  | n2 |  |  | 1855 |  |  | 5 |  |  | 25 |  |  | 1935 |  |  | 32.0 |  |  | IMD2 |  |  |
|  | n77 |  |  | 3810 |  |  | 10 |  |  | 50 |  |  | 3810 |  |  | N/A |  |  | N/A |  |  |
|  | 2 |  |  | 1895 |  |  | 5 |  |  | 25 |  |  | 1975 |  |  | N/A |  |  | N/A |  |  |
|  | n2 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1975 |  |  | 20.0 |  |  | IMD41 |  |  |
|  | n77 |  |  | 3710 |  |  | 10 |  |  | 50 |  |  | 3710 |  |  | N/A |  |  | N/A |  |  |
| DC_2A-5A_n77A2 DC_2A-5A_n77(2A)2 DC_2A-2A-5A_n77A2 DC_2A-2A-5A_n77(2A)2 DC_2A-5A_n77C2 DC_2A-2A-5A_n77C2 | 2 |  |  | 1907.5 |  |  | 5 |  |  | 25 |  |  | 1987.5 |  |  | N/A |  |  | N/A |  |  |
|  | 5 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 887.5 |  |  | 13.6 |  |  | IMD5 |  |  |
|  | n77 |  |  | 3305 |  |  | 10 |  |  | 50 |  |  | 3305 |  |  | N/A |  |  | N/A |  |  |
|  | 2 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1987 |  |  | 24.8 |  |  | IMD3 |  |  |
|  | 5 |  |  | 846.5 |  |  | 5 |  |  | 25 |  |  | 891.5 |  |  | N/A |  |  | N/A |  |  |
|  | n77 |  |  | 3680 |  |  | 10 |  |  | 50 |  |  | 3680 |  |  | N/A |  |  | N/A |  |  |
| DC_2A_n5A-n77A2 DC_2A-2A_n5A-n77A2 DC_2A_n5A-n77C2 DC_2A-2A_n5A-n77C2 | 2 |  |  | 1907 |  |  | 5 |  |  | 25 |  |  | 1987 |  |  | N/A |  |  | N/A |  |  |
|  | n5 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 889 |  |  | 13.6 |  |  | IMD52 |  |  |
|  | n77 |  |  | 3305 |  |  | 10 |  |  | 50 |  |  | 3305 |  |  | N/A |  |  | N/A |  |  |
| DC_2A-7A_n78A | 2 |  |  | 1870 |  |  | 5 |  |  | 25 |  |  | 1950 |  |  | 20.0 |  |  |  | IMD4 |  |
|  | 7 |  |  | 2550 |  |  | 5 |  |  | 25 |  |  | 2685 |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  |  | 3525 |  |  | 10 |  |  | 50 |  |  | 3525 |  |  | N/A |  |  |  | N/A |  |
| DC_2A-12A_n77A DC_2A-12A_n77(2A) DC_2A-2A-12A_n77A DC_2A-2A-12A_n77(2A) | 2 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1960 |  |  | 24.8 |  |  |  | IMD32, 5 |  |
|  | 12 |  |  | 707.5 |  |  | 5 |  |  | 25 |  |  | 737.5 |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  |  | 3375 |  |  | 10 |  |  | 50 |  |  | 3375 |  |  | N/A |  |  |  | N/A |  |
| DC_2A-13A_n77A DC_2A-2A-13A_n77A DC_2A-13A_n77C DC_2A-2A-13A_n77C | 2 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1944 |  |  | 24.2 |  |  | IMD3 |  |  |
|  | 13 |  |  | 783 |  |  | 5 |  |  | 25 |  |  | 752 |  |  | N/A |  |  | N/A |  |  |
|  | n77 |  |  | 3510 |  |  | 10 |  |  | 50 |  |  | 3510 |  |  | N/A |  |  | N/A |  |  |
| DC_2A-14A_n77A DC_2A-14A_n77(2A) DC_2A-2A-14A_n77A DC_2A-2A-14A_n77(2A) | 2 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1954 |  |  | 24.8 |  |  |  | IMD3 |  |
|  | 14 |  |  | 793 |  |  | 5 |  |  | 25 |  |  | 763 |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  |  | 3540 |  |  | 10 |  |  | 50 |  |  | 3540 |  |  | N/A |  |  |  | N/A |  |
| DC_2A-30A_n77A DC_2A-30A_n77(2A) DC_2A-2A-30A_n77A DC_2A-2A-30A_n77(2A) | 2 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1986 |  |  | 19.3 |  |  |  | IMD42 |  |
|  | 30 |  |  | 2312 |  |  | 5 |  |  | 25 |  |  | 2357 |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  |  | 3305 |  |  | 10 |  |  | 50 |  |  | 3305 |  |  | N/A |  |  |  | N/A |  |
|  | 2 |  |  | 1905 |  |  | 5 |  |  | 25 |  |  | 1985 |  |  | N/A |  |  |  | N/A |  |
|  | 30 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2354 |  |  | 22.2 |  |  |  | IMD42 |  |
|  | n77 |  |  | 3361 |  |  | 10 |  |  | 50 |  |  | 3361 |  |  | N/A |  |  |  | N/A |  |
|  | 2 |  |  | 1860 |  |  | 5 |  |  | 25 |  |  | 1940 |  |  | N/A |  |  |  | N/A |  |
|  | 30 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2354 |  |  | 12.9 |  |  |  | IMD5 |  |
|  | n77 |  |  | 3967 |  |  | 10 |  |  | 50 |  |  | 3967 |  |  | N/A |  |  |  | N/A |  |
| DC_2A-66A_n41A | 2 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1940 |  |  | 22.6 |  |  | IMD4 |  |  |
|  | 66 |  |  | 1715 |  |  | 5 |  |  | 25 |  |  | 2115 |  |  | N/A |  |  | N/A |  |  |
|  | n41 |  |  | 2685 |  |  | 5 |  |  | 25 |  |  | 2685 |  |  | N/A |  |  | N/A |  |  |
| DC_2A-66A_n77A DC_2A-66A_n77(2A) DC_2A-2A-66A_n77A DC_2A-2A-66A_n77(2A) DC_2A-66A-66A_n77A DC_2A-66A-66A_n77(2A) DC_2A-2A-66A-66A_n77A DC_2A-66A_n77C DC_2A-66A-66A_n77C DC_2A-2A-66A-66A_n77C | 2 |  |  | 1855 |  |  | 5 |  |  | 25 |  |  | 1935 |  |  | N/A |  |  | N/A |  |  |
|  | 66 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2115 |  |  | 34.7 |  |  | IMD2 |  |  |
|  | n77 |  |  | 3970 |  |  | 10 |  |  | 50 |  |  | 3970 |  |  | N/A |  |  | N/A |  |  |
|  | 2 |  |  | 1880 |  |  | 5 |  |  | 25 |  |  | 1960 |  |  | M/A |  |  | N/A |  |  |
|  | 66 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2140 |  |  | 21.1 |  |  | IMD41 |  |  |
|  | n77 |  |  | 3500 |  |  | 10 |  |  | 50 |  |  | 3500 |  |  | N/A |  |  | N/A |  |  |
|  | 2 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1960 |  |  | 37.6 |  |  | IMD2 |  |  |
|  | 66 |  |  | 1760 |  |  | 5 |  |  | 25 |  |  | 2160 |  |  | N/A |  |  | N/A |  |  |
|  | n77 |  |  | 3720 |  |  | 10 |  |  | 50 |  |  | 3720 |  |  | N/A |  |  | N/A |  |  |
|  | 2 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1940 |  |  | 19.8 |  |  | IMD41,2 |  |  |
|  | 66 |  |  | 1775 |  |  | 5 |  |  | 25 |  |  | 2195 |  |  | N/A |  |  | N/A |  |  |
|  | n77 |  |  | 3385 |  |  | 10 |  |  | 50 |  |  | 3385 |  |  | N/A |  |  | N/A |  |  |
| DC_2A_n66A-n77A DC_2A-2A_n66A-n77A DC_2A_n66A-n77C DC_2A-2A_n66A-n77C | 2 |  |  | 1855 |  |  | 5 |  |  | 25 |  |  | 1935 |  |  | N/A |  |  | N/A |  |  |
|  | n66 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2115 |  |  | 35.2 |  |  | IMD2 |  |  |
|  | n77 |  |  | 3970 |  |  | 10 |  |  | 50 |  |  | 3970 |  |  | N/A |  |  | N/A |  |  |
|  | 2 |  |  | 1900 |  |  | 5 |  |  | 25 |  |  | 1980 |  |  | N/A |  |  | N/A |  |  |
|  | n66 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2160 |  |  | 22.3 |  |  | IMD43 |  |  |
|  | n77 |  |  | 3540 |  |  | 10 |  |  | 50 |  |  | 3540 |  |  | N/A |  |  | N/A |  |  |
| DC_2A-66A_n78A | 2 |  |  | 1880 |  |  | 5 |  |  | 25 |  |  | 1960 |  |  | M/A |  |  | N/A |  |  |
|  | 66 |  |  | 1740 |  |  | 5 |  |  | 25 |  |  | 2140 |  |  | 21.1 |  |  | IMD4 |  |  |
|  | n78 |  |  | 3500 |  |  | 10 |  |  | 50 |  |  | 3500 |  |  | N/A |  |  | N/A |  |  |
|  | 2 |  |  | 1880 |  |  | 5 |  |  | 25 |  |  | 1960 |  |  | 37.6 |  |  | IMD2 |  |  |
|  | 66 |  |  | 1760 |  |  | 5 |  |  | 25 |  |  | 2160 |  |  | N/A |  |  | N/A |  |  |
|  | n78 |  |  | 3720 |  |  | 10 |  |  | 50 |  |  | 3720 |  |  | N/A |  |  | N/A |  |  |
|  | 2 |  |  | 1860 |  |  | 5 |  |  | 25 |  |  | 1940 |  |  | 19.8 |  |  | IMD4 |  |  |
|  | 66 |  |  | 1775 |  |  | 5 |  |  | 25 |  |  | 2195 |  |  | N/A |  |  | N/A |  |  |
|  | n78 |  |  | 3385 |  |  | 10 |  |  | 50 |  |  | 3385 |  |  | N/A |  |  | N/A |  |  |
|  | 2 |  |  | 1880 |  |  | 5 |  |  | 25 |  |  | 1960 |  |  | 13.2 |  |  | IMD5 |  |  |
|  | 66 |  |  | 1760 |  |  | 5 |  |  | 25 |  |  | 2160 |  |  | N/A |  |  | N/A |  |  |
|  | n78 |  |  | 3620 |  |  | 10 |  |  | 50 |  |  | 3620 |  |  | N/A |  |  | N/A |  |  |
| DC_3A_n1A-n78A DC_3A-3A_n1A-n78A | 3 |  |  | 1770 |  |  | 5 |  |  | 25 |  |  | 1865 |  |  | N/A |  |  | N/A |  |  |
|  | n1 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2130 |  |  | 17.8 |  |  | IMD5 |  |  |
|  | n78 |  |  | 3720 |  |  | 10 |  |  | 50 |  |  | 3720 |  |  | N/A |  |  | N/A |  |  |
| DC_3A-7A_n78A DC_3A-3A-7A_n78A DC_3A-7A-7A_n78A DC_3A-3A-7A-7A_n78A | 3 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1820 |  |  | 26.5 |  |  | IMD35 |  |  |
|  | 7 |  |  | 2565 |  |  | 5 |  |  | 25 |  |  | 2685 |  |  | N/A |  |  | N/A |  |  |
|  | n78 |  |  | 3310 |  |  | 10 |  |  | 50 |  |  | 3310 |  |  | N/A |  |  | N/A |  |  |
| DC_3A-8A_n77A | 3 |  |  | 1715 |  |  | 5 |  |  | 25 |  |  | 1810 |  |  | N/A |  |  | N/A |  |  |
| DC_3C-8A_n77A | 8 |  |  | 910 |  |  | 5 |  |  | 25 |  |  | 955 |  |  | 21.2 |  |  | IMD4 |  |  |
| DC_3A-8A_n77(2A) | n77 |  |  | 4190 |  |  | 10 |  |  | 50 |  |  | 4190 |  |  | N/A |  |  | N/A |  |  |
| DC_3C-8A_n77(2A) | 3 |  |  | 1725 |  |  | 5 |  |  | 25 |  |  | 1820 |  |  | 24.8 |  |  | IMD3 |  |  |
|  | 8 |  |  | 910 |  |  | 5 |  |  | 25 |  |  | 955 |  |  | N/A |  |  | N/A |  |  |
|  | n77 |  |  | 3640 |  |  | 10 |  |  | 50 |  |  | 3640 |  |  | N/A |  |  | N/A |  |  |
| DC_3A-8A_n78A DC_3A-3A-8A_n78A DC_3C-8A_n78A DC_3A-8A_n78(2A) | 8 |  |  | 910 |  |  | 5 |  |  | 25 |  |  | 955 |  |  | N/A |  |  | N/A |  |  |
|  | n78 |  |  | 3640 |  |  | 10 |  |  | 50 |  |  | 3640 |  |  | N/A |  |  | N/A |  |  |
|  | 3 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1820 |  |  | 24.8 |  |  | IMD3 |  |  |
| DC_3A-19A_n77A DC_3A-19A_n77(2A) | 3 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1850 |  |  | 26.3 |  |  | IMD3 |  |  |
|  | 19 |  |  | 835 |  |  | 5 |  |  | 25 |  |  | 880 |  |  | N/A |  |  | N/A |  |  |
|  | n77 |  |  | 3520 |  |  | 10 |  |  | 50 |  |  | 3520 |  |  | N/A |  |  | N/A |  |  |
| DC_3A-19A_n78A DC_3A-19A_n78(2A) | 3 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1850 |  |  | 26.3 |  |  | IMD3 |  |  |
|  | 19 |  |  | 835 |  |  | 5 |  |  | 25 |  |  | 880 |  |  | N/A |  |  | N/A |  |  |
|  | n78 |  |  | 3520 |  |  | 10 |  |  | 50 |  |  | 3520 |  |  | N/A |  |  | N/A |  |  |
| DC_3A-19A_n79A | 3 |  |  | 1775 |  |  | 5 |  |  | 25 |  |  | 1870 |  |  | N/A |  |  | N/A |  |  |
|  | 19 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 885 |  |  | 27.5 |  |  | IMD35 |  |  |
|  | n79 |  |  | 4435 |  |  | 40 |  |  | 216 |  |  | 4435 |  |  | N/A |  |  | N/A |  |  |
|  | 3 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1877.5 |  |  | 16.2 |  |  | IMD4 |  |  |
|  | 19 |  |  | 842.5 |  |  | 5 |  |  | 25 |  |  | 887.5 |  |  | N/A |  |  | N/A |  |  |
|  | n79 |  |  | 4420 |  |  | 40 |  |  | 216 |  |  | 4420 |  |  | N/A |  |  | N/A |  |  |
| DC_3A-21A_n77A DC_3A-21A_n77(2A) | 3 |  |  | 1767.5 |  |  | 5 |  |  | 25 |  |  | 1862.5 |  |  | N/A |  |  | N/A |  |  |
|  | 21 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1507.5 |  |  | 20.8 |  |  | IMD4 |  |  |
|  | n77 |  |  | 3795 |  |  | 10 |  |  | 50 |  |  | 3795 |  |  | N/A |  |  | N/A |  |  |
|  | 3 |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | IMD2 |  |  |
|  | 21 |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  |
|  | n77 |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  |
|  | 3 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1866.6 |  |  | 18.4 |  |  | IMD5 |  |  |
|  | 21 |  |  | 1450.4 |  |  | 5 |  |  | 25 |  |  | 1498.4 |  |  | N/A |  |  | N/A |  |  |
|  | n77 |  |  | 3935 |  |  | 10 |  |  | 50 |  |  | 3935 |  |  | N/A |  |  | N/A |  |  |
| DC_3A-21A_n78A DC_3A-21A_n78(2A) | 3 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1862.5 |  |  | 36.6 |  |  | IMD2 |  |  |
|  | 21 |  |  | 1459.5 |  |  | 5 |  |  | 25 |  |  | 1507.5 |  |  | N/A |  |  | N/A |  |  |
|  | n78 |  |  | 3322 |  |  | 10 |  |  | 50 |  |  | 3322 |  |  | N/A |  |  | N/A |  |  |
|  | 3 |  |  | 1767.5 |  |  | 5 |  |  | 25 |  |  | 1862.5 |  |  | N/A |  |  | N/A |  |  |
|  | 21 |  |  | 1459.5 |  |  | 5 |  |  | 25 |  |  | 1507.5 |  |  | 23.2 |  |  | IMD4 |  |  |
|  | n78 |  |  | 3795 |  |  | 10 |  |  | 50 |  |  | 3795 |  |  | N/A |  |  | N/A |  |  |
|  | 3 |  |  | 1767.5 |  |  | 5 |  |  | 25 |  |  | 1862.5 |  |  | N/A |  |  | N/A |  |  |
|  | 21 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1503.5 |  |  | 9.5 |  |  | IMD5 |  |  |
|  | n78 |  |  | 3403 |  |  | 10 |  |  | 50 |  |  | 3403 |  |  | N/A |  |  | N/A |  |  |
| DC_3A-21A_n79A7 | 3 |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  |
|  | 21 |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | IMD3 |  |  |
|  | n79 |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  |
|  | 3 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1869.2 |  |  | 32.8 |  |  | IMD3 |  |  |
|  | 21 |  |  | 1450.4 |  |  | 5 |  |  | 25 |  |  | 1498.4 |  |  | N/A |  |  | N/A |  |  |
|  | n79 |  |  | 4770 |  |  | 10 |  |  | 50 |  |  | 4770 |  |  | N/A |  |  | N/A |  |  |
| DC_3A-28A_n41A | 3 |  |  | 1720 |  |  | 5 |  |  | 25 |  |  | 1815 |  |  | N/A |  |  | N/A |  |  |
|  | n41 |  |  | 2510 |  |  | 5 |  |  | 25 |  |  | 2510 |  |  | N/A |  |  | N/A |  |  |
|  | 28 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 790 |  |  | 32 |  |  | IMD211 |  |  |
|  | 3 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1832.5 |  |  | 32 |  |  | IMD2 |  |  |
|  | n41 |  |  | 2543 |  |  | 10 |  |  | 50 |  |  | 2543 |  |  | N/A |  |  | N/A |  |  |
|  | 28 |  |  | 710.5 |  |  | 5 |  |  | 25 |  |  | 765.5 |  |  | N/A |  |  | N/A |  |  |
| DC_3A-28A_n77A | 3 |  |  | 1712.5 |  |  | 5 |  |  | 25 |  |  | 1807.5 |  |  | N/A |  |  | N/A |  |  |
|  | 28 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 770 |  |  | 24.2 |  |  | IMD3 |  |  |
|  | n77 |  |  | 4195 |  |  | 10 |  |  | 50 |  |  | 4195 |  |  | N/A |  |  | N/A |  |  |
|  | 3 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1850 |  |  | 25.8 |  |  | IMD35 |  |  |
|  | 28 |  |  | 735 |  |  | 5 |  |  | 25 |  |  | 790 |  |  | N/A |  |  | N/A |  |  |
|  | n77 |  |  | 3320 |  |  | 10 |  |  | 50 |  |  | 3320 |  |  | N/A |  |  | N/A |  |  |
| DC_3A_n28A-n77A | 3 |  |  | 1712.5 |  |  | 5 |  |  | 25 |  |  | 1807.5 |  |  | N/A |  |  | N/A |  |  |
|  | n28 |  |  | 715 |  |  | 5 |  |  | 25 |  |  | 770 |  |  | 24.2 |  |  | IMD3 |  |  |
|  | n77 |  |  | 4195 |  |  | 10 |  |  | 50 |  |  | 4195 |  |  | N/A |  |  | N/A |  |  |
| DC_3A-28A_n78A | 3 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1850 |  |  | 25.9 |  |  | IMD3 |  |  |
|  | 28 |  |  | 735 |  |  | 5 |  |  | 25 |  |  | 790 |  |  | N/A |  |  | N/A |  |  |
|  | n78 |  |  | 3320 |  |  | 10 |  |  | 50 |  |  | 3320 |  |  | N/A |  |  | N/A |  |  |
| DC_3A_n41A-n77A | 3 |  |  | 1720 |  |  | 5 |  |  | 25 |  |  | 1815 |  |  | N/A |  |  | N/A |  |  |
|  | n41 |  |  | 2580 |  |  | 5 |  |  | 25 |  |  | 2580 |  |  | N/A |  |  | N/A |  |  |
|  | n77 |  |  | N/A |  |  | 10 |  |  | N/A |  |  | 3440 |  |  | 25.6 |  |  | IMD31 |  |  |
|  | 3 |  |  | 1720 |  |  | 5 |  |  | 25 |  |  | 1815 |  |  | N/A |  |  | N/A |  |  |
|  | n41 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2640 |  |  | 13 |  |  | IMD5 |  |  |
|  | n77 |  |  | 3900 |  |  | 10 |  |  | 50 |  |  | 3900 |  |  | N/A |  |  | N/A |  |  |
| DC_3A-42A_n79A9 DC_3A-42C_n79A9 DC_3A-42D_n79A9 DC_3A-42E_n79A9 | 3 |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  |
|  | 42 |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | IMD5 |  |  |
|  | n79 |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  |
| DC_3A_n78A-n79A | 3 |  |  | 1770 |  |  | 5 |  |  | 25 |  |  | 1865 |  |  | N/A |  |  | N/A |  |  |
|  | n78 |  |  | 3340 |  |  | 10 |  |  | 50 |  |  | 3340 |  |  | N/A |  |  | N/A |  |  |
|  | n79 |  |  | N/A |  |  | 10 |  |  | N/A |  |  | 4910 |  |  | 25.3 |  |  | IMD3 |  |  |
|  | 3 |  |  | 1770 |  |  | 5 |  |  | 25 |  |  | 1865 |  |  | N/A |  |  | N/A |  |  |
|  | n78 |  |  | N/A |  |  | 10 |  |  | N/A |  |  | 3710 |  |  | 25.2 |  |  | IMD5 |  |  |
|  | n79 |  |  | 4510 |  |  | 10 |  |  | 50 |  |  | 4510 |  |  | N/A |  |  | N/A |  |  |
| DC_5A_n2A-n77A2 DC_5A_n2A-n77C2 | n2 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1987 |  |  | 25.5 |  |  | IMD3 |  |  |
|  | 5 |  |  | 846.5 |  |  | 5 |  |  | 25 |  |  | 891.5 |  |  | N/A |  |  | N/A |  |  |
|  | n77 |  |  | 3680 |  |  | 10 |  |  | 50 |  |  | 3680 |  |  | N/A |  |  | N/A |  |  |
| DC_5A_n5A-n77A2 DC_5A_n5A-n77C2 | 5 |  |  | 834 |  |  | 5 |  |  | 25 |  |  | 879 |  |  | N/A |  |  | N/A |  |  |
|  | n5 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 889 |  |  | 20.3 |  |  | IMD41 |  |  |
|  | n77 |  |  | 3391 |  |  | 10 |  |  | 50 |  |  | 3391 |  |  | N/A |  |  | N/A |  |  |
| DC_5A-13A_n77A2 DC_5A-13A_n77C2 | 5 |  |  | 840 |  |  | 5 |  |  | 25 |  |  | 885 |  |  | N/A |  |  |  | N/A |  |
|  | 13 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 750 |  |  | 19.4 |  |  |  | IMD5 |  |
|  | n77 |  |  | 4110 |  |  | 10 |  |  | 50 |  |  | 4110 |  |  | N/A |  |  |  | N/A |  |
|  | 5 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 885 |  |  | 19.5 |  |  |  | IMD5 |  |
|  | 13 |  |  | 782 |  |  | 5 |  |  | 20 |  |  | 751 |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  |  | 4013 |  |  | 10 |  |  | 50 |  |  | 4013 |  |  | N/A |  |  |  | N/A |  |
| DC_5A-30A_n77A DC_5A-30A_n77(2A) | 5 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 880 |  |  | 23.5 |  |  |  | IMD31 |  |
|  | 30 |  |  | 2310 |  |  | 5 |  |  | 25 |  |  | 2355 |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  |  | 3740 |  |  | 10 |  |  | 50 |  |  | 3740 |  |  | N/A |  |  |  | N/A |  |
|  | 5 |  |  | 835 |  |  | 5 |  |  | 25 |  |  | 880 |  |  | N/A |  |  |  | N/A |  |
|  | 30 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2355 |  |  | 21.4 |  |  |  | IMD32 |  |
|  | n77 |  |  | 4025 |  |  | 10 |  |  | 50 |  |  | 4025 |  |  | N/A |  |  |  | N/A |  |
| DC_5A-66A_n77A DC_5A-66A_n77(2A) DC_5A-66A-66A_n77A DC_5A-66A-66A_n77(2A) | 5 |  |  | 826.5 |  |  | 5 |  |  | 25 |  |  | 871.5 |  |  | N/A |  |  |  | N/A |  |
|  | 66 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2142 |  |  | 22.2 |  |  |  | IMD3 |  |
|  | n77 |  |  | 3795 |  |  | 10 |  |  | 50 |  |  | 3795 |  |  | N/A |  |  |  | N/A |  |
| DC_5A_n66A-n77A DC_5A_n66A-n77C | 5 |  |  | 826.5 |  |  | 5 |  |  | 25 |  |  | 871.5 |  |  | N/A |  |  | N/A |  |  |
|  | n66 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2142 |  |  | 22.2 |  |  | IMD3 |  |  |
|  | n77 |  |  | 3795 |  |  | 10 |  |  | 50 |  |  | 3795 |  |  | N/A |  |  | N/A |  |  |
| DC_7A_n1A-n78A DC_7A-7A_n1A-n78A | 1 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2140 |  |  | 19.7 |  |  | IMD4 |  |  |
|  | 7 |  |  | 2510 |  |  | 10 |  |  | 50 |  |  | 2630 |  |  | N/A |  |  | N/A |  |  |
|  | n78 |  |  | 3580 |  |  | 10 |  |  | 50 |  |  | 3580 |  |  | N/A |  |  | N/A |  |  |
| DC_7A_n5A-n78A | 7 |  |  | 2555 |  |  | 5 |  |  | 25 |  |  | 2675 |  |  | N/A |  |  | N/A |  |  |
|  | n5 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 881 |  |  | 34.7 |  |  | IMD21 |  |  |
|  | n78 |  |  | 3436 |  |  | 10 |  |  | 50 |  |  | 3436 |  |  | N/A |  |  | N/A |  |  |
| DC_7A-8A_n78A DC_7A-7A-8A_n78A | 7 |  |  | 2530 |  |  | 5 |  |  | 25 |  |  | 2650 |  |  | N/A |  |  |  | N/A |  |
|  | 8 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 940 |  |  | 35.5 |  |  |  | IMD21 |  |
|  | n78 |  |  | 3470 |  |  | 10 |  |  | 50 |  |  | 3470 |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2650 |  |  | 33 |  |  |  | IMD2 |  |
|  | 8 |  |  | 895 |  |  | 5 |  |  | 25 |  |  | 940 |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  |  | 3545 |  |  | 10 |  |  | 50 |  |  | 3545 |  |  | N/A |  |  |  | N/A |  |
| DC_7A-28A_n78A | 7 |  |  | 2567.5 |  |  | 5 |  |  | 25 |  |  | 2687.5 |  |  | N/A |  |  |  | N/A |  |
|  | 28 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 782.5 |  |  | 33.8 |  |  |  | IMD21 |  |
|  | n78 |  |  | 3350 |  |  | 10 |  |  | 50 |  |  | 3350 |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2650 |  |  | 35.5 |  |  |  | IMD2 |  |
|  | 28 |  |  | 740 |  |  | 5 |  |  | 25 |  |  | 795 |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  |  | 3390 |  |  | 10 |  |  | 50 |  |  | 3390 |  |  | N/A |  |  |  | N/A |  |
| DC_7A_n28A-n78A | 7 |  |  | 2565 |  |  | 5 |  |  | 25 |  |  | 2685 |  |  | N/A |  |  | N/A |  |  |
|  | n78 |  |  | 3365 |  |  | 10 |  |  | 50 |  |  | 3365 |  |  | N/A |  |  | N/A |  |  |
|  | n28 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 800 |  |  | 33.8 |  |  | IMD21 |  |  |
| DC_7A-66A_n78A | 7 |  |  | 2540 |  |  | 5 |  |  | 25 |  |  | 2660 |  |  | N/A |  |  | N/A |  |  |
|  | 66 |  |  | 1760 |  |  | 5 |  |  | 25 |  |  | 2160 |  |  | 20.5 |  |  | IMD4 |  |  |
|  | n78 |  |  | 3620 |  |  | 10 |  |  | 50 |  |  | 3620 |  |  | N/A |  |  | N/A |  |  |
| DC_8A_n1A-n77A | 8 |  |  | 910 |  |  | 5 |  |  | 25 |  |  | 955 |  |  | N/A |  |  | N/A |  |  |
|  | n1 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2140 |  |  | 27.5 |  |  | IMD3 |  |  |
|  | n77 |  |  | 3960 |  |  | 10 |  |  | 50 |  |  | 3960 |  |  | N/A |  |  | N/A |  |  |
| DC_8A_n1A-n79A | 8 |  |  | 900 |  |  | 5 |  |  | 25 |  |  | 945 |  |  | N/A |  |  |  | N/A |  |
|  | n1 |  |  | N/A |  |  | 5 |  |  | 25 |  |  | 2145 |  |  | 25.7 |  |  |  | IMD4 |  |
|  | n79 |  |  | 4845 |  |  | 40 |  |  | 216 |  |  | 4845 |  |  | N/A |  |  |  | N/A |  |
| DC_8A_n3A-n77A | 8 |  |  | 910 |  |  | 5 |  |  | 25 |  |  | 955 |  |  | N/A |  |  | N/A |  |  |
|  | n3 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1820 |  |  | 24.5 |  |  | IMD3 |  |  |
|  | n77 |  |  | 3640 |  |  | 10 |  |  | 50 |  |  | 3640 |  |  | N/A |  |  | N/A |  |  |
| DC_8A_n3A-n79A |  | 8 |  |  | 910 |  |  | 5 |  |  | 25 |  |  | 955 |  |  | N/A |  |  | N/A |  |
|  |  | n3 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1850 |  |  | 22.7 |  |  | IMD4 |  |
|  |  | n79 |  |  | 4580 |  |  | 40 |  |  | 216 |  |  | 4580 |  |  | N/A |  |  | N/A |  |
| DC_8A_n28A-n77A | 8 |  |  | 910 |  |  | 5 |  |  | 25 |  |  | 955 |  |  | N/A |  |  | N/A |  |  |
|  | n28 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 765 |  |  | 23 |  |  | IMD4 |  |  |
|  | n77 |  |  | 3495 |  |  | 10 |  |  | 50 |  |  | 3495 |  |  | N/A |  |  | N/A |  |  |
| DC_8A_n28A-n79A |  |  | 8 |  |  | 905 |  |  | 5 |  |  | 25 |  |  | 950 |  |  | N/A |  |  | N/A |  |
|  |  |  | n79 |  |  | 4420 |  |  | 40 |  |  | 216 |  |  | 4420 |  |  | N/A |  |  | N/A |  |
|  |  |  | n28 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 800 |  |  | 24.0 |  |  | IMD5 |  |
| DC_12A-30A_n77A DC_12A-30A_n77(2A) | 12 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 740 |  |  | 23.5 |  |  |  | IMD31 |  |
|  | 30 |  |  | 2310 |  |  | 5 |  |  | 25 |  |  | 2355 |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  |  | 3880 |  |  | 10 |  |  | 50 |  |  | 3880 |  |  | N/A |  |  |  | N/A |  |
|  | 12 |  |  | 707.5 |  |  | 5 |  |  | 25 |  |  | 737.5 |  |  | N/A |  |  |  | N/A |  |
|  | 30 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2355 |  |  | 21.4 |  |  |  | IMD3 |  |
|  | n77 |  |  | 3770 |  |  | 10 |  |  | 50 |  |  | 3770 |  |  | N/A |  |  |  | N/A |  |
| DC_12A-66A_n77A DC_12A-66A_n77(2A) DC_12A-66A-66A_n77A DC_12A-66A-66A_n77(2A) | 12 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 740 |  |  | 23.5 |  |  |  | IMD32 |  |
|  | 66 |  |  | 1720 |  |  | 5 |  |  | 25 |  |  | 2120 |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  |  | 4180 |  |  | 10 |  |  | 50 |  |  | 4180 |  |  | N/A |  |  |  | N/A |  |
|  | 12 |  |  | 707 |  |  | 5 |  |  | 25 |  |  | 737 |  |  | N/A |  |  |  | N/A |  |
|  | 66 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2126 |  |  | 21.4 |  |  |  | IMD3 |  |
|  | n77 |  |  | 3540 |  |  | 10 |  |  | 50 |  |  | 3540 |  |  | N/A |  |  |  | N/A |  |
| DC_12A-71A_n2A | 12 |  |  | 713.5 |  |  | 5 |  |  | 25 |  |  | 743.5 |  |  | 4.2 |  |  |  | IMD5 |  |
|  | 71 |  |  | 665.5 |  |  | 5 |  |  | 25 |  |  | 619.5 |  |  | N/A |  |  |  | N/A |  |
|  | n2 |  |  | 1907.5 |  |  | 5 |  |  | 25 |  |  | 1987.5 |  |  | N/A |  |  |  | N/A |  |
| DC_12A-71A_n77A | 12 |  |  | 702 |  |  | 5 |  |  | 25 |  |  | 732 |  |  | 4.4 |  |  |  | IMD5 |  |
|  | 71 |  |  | 667 |  |  | 5 |  |  | 25 |  |  | 621 |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  |  | 3400 |  |  | 10 |  |  | 50 |  |  | 3400 |  |  | N/A |  |  |  | N/A |  |
|  | 12 |  |  | 701.5 |  |  | 5 |  |  | 25 |  |  | 731.5 |  |  | N/A |  |  |  | N/A |  |
|  | 71 |  |  | 690 |  |  | 5 |  |  | 25 |  |  | 644 |  |  | 3.9 |  |  |  | IMD5 |  |
|  | n77 |  |  | 3450 |  |  | 10 |  |  | 50 |  |  | 3450 |  |  | N/A |  |  |  | N/A |  |
| DC_13A_n2A-n77A DC_13A_n2A-n77C | 13 |  |  | 782 |  |  | 5 |  |  | 25 |  |  | 751 |  |  | N/A |  |  | N/A |  |  |
|  | n2 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1960 |  |  | 25.0 |  |  | IMD3 |  |  |
|  | n77 |  |  | 3524 |  |  | 10 |  |  | 50 |  |  | 3524 |  |  | N/A |  |  | N/A |  |  |
| DC_13A_n5A-n77A2 DC_13A_n5A-n77C2 | n5 |  |  | 840 |  |  | 5 |  |  | 25 |  |  | 885 |  |  | 19.5 |  |  | IMD5 |  |  |
|  | 13 |  |  | 782 |  |  | 5 |  |  | 20 |  |  | 751 |  |  | N/A |  |  | N/A |  |  |
|  | n77 |  |  | 4013 |  |  | 10 |  |  | 50 |  |  | 4013 |  |  | N/A |  |  | N/A |  |  |
| DC_13A-66A_n77A DC_13A-66A-66A_n77A DC_13A-66A_n77C DC_13A-66A-66A_n77C | 13 |  |  | 782 |  |  | 5 |  |  | 25 |  |  | 751 |  |  | N/A |  |  | N/A |  |  |
|  | 66 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2156 |  |  | 25.3 |  |  | IMD3 |  |  |
|  | n77 |  |  | 3720 |  |  | 10 |  |  | 50 |  |  | 3720 |  |  | N/A |  |  | N/A |  |  |
|  | 13 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 750 |  |  | 23.4 |  |  | IMD32 |  |  |
|  | 66 |  |  | 1720 |  |  | 5 |  |  | 25 |  |  | 2120 |  |  | N/A |  |  | N/A |  |  |
|  | n77 |  |  | 4190 |  |  | 10 |  |  | 50 |  |  | 4190 |  |  | N/A |  |  | N/A |  |  |
| DC_13A_n66A-n77A DC_13A_n66A-n77C | 13 |  |  | 782 |  |  | 5 |  |  | 25 |  |  | 751 |  |  | N/A |  |  | N/A |  |  |
|  | n66 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2156 |  |  | 26.1 |  |  | IMD3 |  |  |
|  | n77 |  |  | 3720 |  |  | 10 |  |  | 50 |  |  | 3720 |  |  | N/A |  |  | N/A |  |  |
| DC_14A-30A_n77A DC_14A-30A_n77(2A) | 14 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 763 |  |  | 23.5 |  |  |  | IMD31 |  |
|  | 30 |  |  | 2310 |  |  | 5 |  |  | 25 |  |  | 2355 |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  |  | 3857 |  |  | 10 |  |  | 50 |  |  | 3857 |  |  | N/A |  |  |  | N/A |  |
|  | 14 |  |  | 793 |  |  | 5 |  |  | 25 |  |  | 763 |  |  | N/A |  |  |  | N/A |  |
|  | 30 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2355 |  |  | 21.4 |  |  |  | IMD3 |  |
|  | n77 |  |  | 3941 |  |  | 10 |  |  | 50 |  |  | 3941 |  |  | N/A |  |  |  | N/A |  |
| DC_14A-66A_n77A DC_14A-66A_n77(2A) DC_14A-66A-66A_n77A DC_14A-66A-66A_n77(2A) | 14 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 763 |  |  | 23.5 |  |  |  | IMD32 |  |
|  | 66 |  |  | 1712.5 |  |  | 5 |  |  | 25 |  |  | 2112.5 |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  |  | 4188 |  |  | 10 |  |  | 50 |  |  | 4188 |  |  | N/A |  |  |  | N/A |  |
|  | 14 |  |  | 793 |  |  | 5 |  |  | 25 |  |  | 763 |  |  | N/A |  |  |  | N/A |  |
|  | 66 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2155 |  |  | 21.4 |  |  |  | IMD3 |  |
|  | n77 |  |  | 3741 |  |  | 10 |  |  | 50 |  |  | 3741 |  |  | N/A |  |  |  | N/A |  |
| DC_18A_n28A-n77A | 18 |  |  | 820 |  |  | 5 |  |  | 25 |  |  | 865 |  |  | N/A |  |  | N/A |  |  |
|  | n28 |  |  | 723 |  |  | 5 |  |  | 25 |  |  | 778 |  |  | 17.5 |  |  | IMD5 |  |  |
|  | n77 |  |  | 4058 |  |  | 10 |  |  | 50 |  |  | 4058 |  |  | N/A |  |  | N/A |  |  |
| DC_19A-21A_n77A DC_19A-21A_n77(2A) | 19 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 882.5 |  |  | 27.7 |  |  |  | IMD3 |  |
|  | 21 |  |  | 1450.4 |  |  | 5 |  |  | 25 |  |  | 1498.4 |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  |  | 3783.3 |  |  | 10 |  |  | 50 |  |  | 3783.3 |  |  | N/A |  |  |  | N/A |  |
|  | 19 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 882.5 |  |  | 25.2 |  |  |  | IMD4 |  |
|  | 21 |  |  | 1450.4 |  |  | 5 |  |  | 25 |  |  | 1498.4 |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  |  | 3468.7 |  |  | 10 |  |  | 50 |  |  | 3468.7 |  |  | N/A |  |  |  | N/A |  |
|  | 19 |  |  | 837.5 |  |  | 5 |  |  | 25 |  |  | 882.5 |  |  | N/A |  |  |  | N/A |  |
|  | 21 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1502.5 |  |  | 21.0 |  |  |  | IMD4 |  |
|  | n77 |  |  | 4015 |  |  | 10 |  |  | 50 |  |  | 4015 |  |  | N/A |  |  |  | N/A |  |
| DC_19A-21A_n78A DC_19A-21A_n78(2A) | 19 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 882.5 |  |  | 27.7 |  |  |  | IMD3 |  |
|  | 21 |  |  | 1450.4 |  |  | 5 |  |  | 25 |  |  | 1498.4 |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  |  | 3783.3 |  |  | 10 |  |  | 50 |  |  | 3783.3 |  |  | N/A |  |  |  | N/A |  |
|  | 19 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 882.5 |  |  | 25.2 |  |  |  | IMD4 |  |
|  | 21 |  |  | 1450.4 |  |  | 5 |  |  | 25 |  |  | 1498.4 |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  |  | 3468.7 |  |  | 10 |  |  | 50 |  |  | 3468.7 |  |  | N/A |  |  |  | N/A |  |
| DC_19A-21A_n79A7 | 19 |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  |  | IMD5 |  |
|  | 21 |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  |  | N/A |  |
|  | 19 |  |  | 837.5 |  |  | 5 |  |  | 25 |  |  | 882.2 |  |  | N/A |  |  |  | N/A |  |
|  | 21 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1500 |  |  | 24.8 |  |  |  | IMD5 |  |
|  | n79 |  |  | 4850 |  |  | 10 |  |  | 50 |  |  | 4850 |  |  | N/A |  |  |  | N/A |  |
| DC_19A-42A_n79A10 DC_19A-42C_n79A10 | 19 |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  |  | N/A |  |
|  | 42 |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  |  | IMD2 |  |
|  | n79 |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  |  | N/A |  |
| DC_19A_n78A-n79A | 19 |  |  | 835 |  |  | 5 |  |  | 25 |  |  | 880 |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  |  | 3680 |  |  | 10 |  |  | 50 |  |  | 3680 |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  |  | N/A |  |  | 40 |  |  | N/A |  |  | 4515 |  |  | 35.3 |  |  |  | IMD2 |  |
|  | 19 |  |  | 835 |  |  | 5 |  |  | 25 |  |  | 880 |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  |  | N/A |  |  | 10 |  |  | N/A |  |  | 3715 |  |  | 34.8 |  |  |  | IMD2 |  |
|  | n79 |  |  | 4550 |  |  | 40 |  |  | 216 |  |  | 4550 |  |  | N/A |  |  |  | N/A |  |
| DC_21A-42A_n79A10 DC_21A-42C_n79A10 | 21 |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  |  | N/A |  |
|  | 42 |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  |  | IMD2 |  |
|  | n79 |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  |  | N/A |  |
| DC_21A_n78A-n79A | 21 |  |  | 1453 |  |  | 5 |  |  | 25 |  |  | 1501 |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  |  | 3420 |  |  | 10 |  |  | 50 |  |  | 3420 |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  |  | N/A |  |  | 10 |  |  | N/A |  |  | 4873 |  |  | 36.1 |  |  |  | IMD25 |  |
|  | 21 |  |  | 1453 |  |  | 5 |  |  | 25 |  |  | 1501 |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  |  | N/A |  |  | 10 |  |  | N/A |  |  | 3487 |  |  | 38.8 |  |  |  | IMD2 |  |
|  | n79 |  |  | 4940 |  |  | 10 |  |  | 50 |  |  | 4940 |  |  | N/A |  |  |  | N/A |  |
| DC_29A-30A_n77A | 29 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 722 |  |  | 23.5 |  |  |  | IMD31 |  |
|  | 30 |  |  | 2310 |  |  | 5 |  |  | 25 |  |  | 2355 |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  |  | 3898 |  |  | 10 |  |  | 50 |  |  | 3898 |  |  | N/A |  |  |  | N/A |  |
| DC_29A-66A_n77A DC_29A-66A-66A_n77A | 29 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 722 |  |  | 23.5 |  |  |  | IMD32 |  |
|  | 66 |  |  | 1734 |  |  | 5 |  |  | 25 |  |  | 2134 |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  |  | 4190 |  |  | 10 |  |  | 50 |  |  | 4190 |  |  | N/A |  |  |  | N/A |  |
| DC_30A-66A_n77A DC_30A-66A_n77(2A) DC_30A-66A-66A_n77A DC_30A-66A-66A_n77(2A) | 30 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2355 |  |  | 34.2 |  |  |  | IMD22 |  |
|  | 66 |  |  | 1745 |  |  | 5 |  |  | 25 |  |  | 2145 |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  |  | 4100 |  |  | 10 |  |  | 50 |  |  | 4100 |  |  | N/A |  |  |  | N/A |  |
|  | 30 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2355 |  |  | 12.9 |  |  |  | IMD5 |  |
|  | 66 |  |  | 1735 |  |  | 5 |  |  | 25 |  |  | 2135 |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  |  | 3780 |  |  | 10 |  |  | 50 |  |  | 3780 |  |  | N/A |  |  |  | N/A |  |
|  | 30 |  |  | 2310 |  |  | 5 |  |  | 25 |  |  | 2355 |  |  | N/A |  |  |  | N/A |  |
|  | 66 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2160 |  |  | 19.2 |  |  |  | IMD42 |  |
|  | n77 |  |  | 3390 |  |  | 10 |  |  | 50 |  |  | 3390 |  |  | N/A |  |  |  | N/A |  |
| DC_41A_n28A-n77A | n28 |  |  | 743 |  |  | 5 |  |  | 25 |  |  | 798 |  |  | 36.8 |  |  |  | IMD21,11 |  |
|  | 41 |  |  | 2642 |  |  | 5 |  |  | 25 |  |  | 2642 |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  |  | 3440 |  |  | 10 |  |  | 50 |  |  | 3440 |  |  | N/A |  |  |  | N/A |  |
| DC_66A_n2A-n77A DC_66A-66A_n2A-n77A DC_66A_n2A-n77C | n2 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1960 |  |  | 37.6 |  |  | IMD2 |  |  |
|  | 66 |  |  | 1760 |  |  | 5 |  |  | 25 |  |  | 2160 |  |  | N/A |  |  | N/A |  |  |
|  | n77 |  |  | 3720 |  |  | 10 |  |  | 50 |  |  | 3720 |  |  | N/A |  |  | N/A |  |  |
|  | n2 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 1960 |  |  | 21.1 |  |  | IMD41,2 |  |  |
|  | 66 |  |  | 1770 |  |  | 5 |  |  | 25 |  |  | 2170 |  |  | N/A |  |  | N/A |  |  |
|  | n77 |  |  | 3350 |  |  | 10 |  |  | 50 |  |  | 3350 |  |  | N/A |  |  | N/A |  |  |
| DC_66A_n5A-n77A DC_66A-66A_n5A-n77A DC_66A_n5A-n77C DC_66A-66A_n5A-n77C | 66 |  |  | 1770 |  |  | 5 |  |  | 25 |  |  | 2170 |  |  | N/A |  |  | N/A |  |  |
|  | n5 |  |  | 845 |  |  | 5 |  |  | 25 |  |  | 890 |  |  | N/A |  |  | N/A |  |  |
|  | n77 |  |  | N/A |  |  | 10 |  |  | N/A |  |  | 3460 |  |  | 24.9 |  |  | IMD3 |  |  |
|  | 66 |  |  | 1714 |  |  | 5 |  |  | 25 |  |  | 2114 |  |  | N/A |  |  | N/A |  |  |
|  | n5 |  |  | 827 |  |  | 5 |  |  | 25 |  |  | 872 |  |  | N/A |  |  | N/A |  |  |
|  | n77 |  |  | N/A |  |  | 10 |  |  | N/A |  |  | 4195 |  |  | 24.1 |  |  | IMD41,2 |  |  |
| DC_66A_n66A-n77A | 66 |  |  | 1750 |  |  | 5 |  |  | 25 |  |  | 2150 |  |  | N/A |  |  | N/A |  |  |
|  | n66 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2150 |  |  | 37 |  |  | IMD2 |  |  |
|  | n77 |  |  | 3900 |  |  | 10 |  |  | 50 |  |  | 3900 |  |  | N/A |  |  | N/A |  |  |
|  | 66 |  |  | 1750 |  |  | 5 |  |  | 25 |  |  | 2150 |  |  | N/A |  |  | N/A |  |  |
|  | n66 |  |  | N/A |  |  | 5 |  |  | N/A |  |  | 2170 |  |  | 20 |  |  | IMD5 |  |  |
|  | n77 |  |  | 3710 |  |  | 10 |  |  | 50 |  |  | 3710 |  |  | N/A |  |  | N/A |  |  |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 3: This UE channel bandwidth is optional in this release of the specification NOTE 4: Void NOTE 5: This band is subject to IMD4 also which MSD is not specified. NOTE 6: E-UTRA carrier shall be set to min(+23 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+23 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 7: The frequency range in band n79 is restricted for this band combination to 4400 - 4900 MHz for both the UL and the DL. NOTE 8: The frequency range in band 1 is restricted for this band combination to 1940 - 1960 MHz for the UL and 2130 - 2150 MHz for the DL. NOTE 9: The frequency range in band n79 is restricted for this band combination to 4500 - 5000 MHz for both the UL and the DL NOTE 10: The frequency range in band n79 is restricted for this band combination to 4500 - 4600 MHz for both the UL and the DL NOTE 11: This band is subject to IMD3 also which MSD is not specified |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

---End of changes---