3GPP RAN WG4 Meeting #110bis		R4-2406676

Changsha, China, April 15th – 19th, 2024

Title:	WF on MSD requirements with intra-band contiguous UL CA

Agenda item:	5.1.1.1

WI/SI:	NR_CA_R18_intra-Core

Release:	Rel-18

Source:	Apple

Document for:	Approval

# 1	Background

- Intra-band contiguous UL CA was first introduced in Rel-16 which covers both single TDD/FDD band and inter-band DL CA. In Rel-18, inter-band UL CA with intra-band contiguous UL CA in one band was further introduced (maximum of 3 CCs in UL configuration).

- Unlike single carrier UL, non-contiguous resource allocation may potentially be scheduled between the two contiguous UL carriers where the clustered inter-modulation products may induce more severe REFSENS degradation than contiguous UL resource allocation for FDD band self-interference and cross-band DL if simultaneous Rx/Tx is supported for the band combination.

- Owing to this concern, RAN4 had specifically introduced MSD requirements based on non-contiguous UL resource allocation in contiguous UL CA, as have been captured in TR 38.862 [1]. For inter-band UL CA with intra-band contiguous UL CA in one band, a new type of MSD requirement due to UL triple-beat inter-modulation interference was further introduced in Rel-18 [2].

- There are currently three types of MSD requirements which are associated with intra-band contiguous UL CA:

  - Single FDD band self-interference, such as for CA_n5B (Figure 1-1)

  - Inter-band CA with cross-band DL interference, such as for CA_n40A-n41C (Figure 1-2)

  - Inter-band CA with triple beat issue, such as for CA_n3A-n41C (Figure 1-3)

Figure 1-1 CA_n5B MSD test configuration

Figure 1-2 CA_n40A-n41C MSD test configuration

Figure 1-3 CA_n3A-n41C triple-beat MSD test configuration

- The UL configurations of the above MSD requirements have been specified with rather small RB allocations in the contiguous UL CA. The cross-band DL interference and triple-beat interference are even specified with only 1 RB in each of the contiguous UL carriers.

- As UL CA would be configured primarily to increase the UL throughout, it does not seem to be very practical to schedule small RB allocations non-contiguously between two contiguous UL carriers where the total resource allocation is even less than a single carrier can already provide.

- On the other hand, non-contiguous UL allocations may be subject to higher MPR/A-MPR in order to fulfill the emission requirements and the IMD power to the impacted DL carrier could potentially be very high based on the current specified MSD values.

- Based on the above concerns, in [3] it is proposed RAN4 to reconsider whether the MSD requirements resulting from intra-band contiguous UL CA are necessitated with good technical justifications.

- This way forward intends to initiate RAN4 to reconsider whether the MSD requirements resulting from intra-band contiguous UL CA configured with non-contiguous allocations are necessitated.

# Way forward

Proposal 1: Companies are encouraged to consider the following options for handling the MSD requirements resulting from intra-band contiguous UL CA configured with non-contiguous allocations:

Option 1: No change from TR 38.862 guidelines

Option 2: Do not consider all MSD requirements resulting from intra-band contiguous UL CA configured with non-contiguous allocations.

Option 3: Do not consider only the MSD requirements resulting from intra-band contiguous UL CA configured with 1RB+1RB allocations.

Proposal 2: If Option 2 or Option 3 in Proposal 1 would be considered, which release to start taking effect?

Option 1: From earliest release where such MSD requirements have been specified.

Option 2: Rel-18

Option 3: Rel-19

Proposal 3: Is there a need to introduce cross-band MSD requirements resulting from intra-band contiguous UL CA configured with fully allocated maximum aggregated BW?

Option 1: Yes

Option 2: No

# References

3GPP TR 38.862 V17.1.0 (2022-06)

R4-2220556 "WF on triple beat rules and MSD for inter-band with 2UL with intra-band ULCA", Murata Manufacturing Co Ltd. Skyworks., 3GPP TSG RAN WG4 Meeting #105, Toulouse, France, November 14th - 18th, 2022

R4-2404180 “Reconsideration on MSD requirements with intra-band contiguous UL CA”, Apple, 3GPP TSG RAN WG4 Meeting #110bis, Changsha, China, April 15th – 19th, 2024