3GPP TSG-RAN WG4 Meeting #110bis	R4-2405327

Changsha, China, April 15 – April 19, 2024

Source:	Huawei, HiSilicon, ZTE

Title:	TP for TR 38.718-02-01 to introduce CA_n28A-n79C with BCS4&5

Agenda item:	5.10.2

Document for:	Approval

1 Background

This contribution provides the technical text proposal on the NR CA band combination CA_n28A-n79C with BCS4&5, which has been captured in the latest basket WI [1].

2 Text Proposal

---Start of changes TR 38.718-02-01 ---

5.x	CA_n28-n79

5.x.1	Common for 1 band UL and 2 bands UL CA

5.x.1.1	Operating bands for CA

Table 5.x.1.1-1:  CA band combination CA_n78-n104

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n28 | 703 | – | 748 | 758 | – | 803 | FDD |
| n79 | 4400 | – | 5000 | 4400 | – | 5000 | TDD |

5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination CA_n78-n104

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n79C | CA_n79C CA_n28A-n79A CA_n28A-n79C | n28 | n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |

5.x.1.3	UE co-existence studies

The harmonic / harmonic mixing issue have been analysed in previous release.

IMD order for contiguous UL CA_n79C is calculated as below.

WGap = lowest band edge of n79 - highest DL band edge of n28 = 4400 – 803 = 3597 MHz.

Min(maxUL aggregated BW, UL band bandwidth) = Min(200, 200) = 200 MHz.

IMD order = 2*ceil(WGap /Min(maxUL aggregated BW, UL band bandwidth) + 1 = 37.

There is no need to consider IMD37.

5.x.1.4	∆TIB,c and ∆RIB,c values

The TIB,c and RIB have been specified in current spec.

5.x.1.5	REFSENS requirements

For UL CA_n79C configuration, there is no additional REFSENS exception.

5.x.1.6	OOB blocking exception requirements

It has been specified in current spec.

5.x.2	Specific for 2 bands UL CA

5.x.2.1	Maximum output power for inter-band CA

Table 5.x.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n28A-n79C | 23 | +2/-3 |

5.x.2.2	UE co-existence studies

The UE co-existence for 2UL CA n28A-n79A have already been analysed.

The bands n28 and n79 are not of same or adjacent band groups, so no requirements to be captured on MSD for TB (Triple beat UL CA_n28A-n79C).

5.x.2.3	REFSENS requirements

For UL CA_n28A-n79C configuration, there is no additional REFSENS exception.

---End of changes---

Reference

[1]	RP-240761, “Revised WID Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity  for 2 bands DL with x bands UL (x=1,2)”, ZTE Corporation