3GPP TSG-RAN4 Meeting #110-bis	R4-2404464

# Changsha, China, 15th Apr 2024 – 19th Apr 2024

Title:	TP to TR 36.718-02-01 Addition of CA_12-71

Source:	Samsung

Agenda item:	7.1.2.1

Document for:	Approval

# 1	Introduction

This TP is to add CA_12A-71A with single UL.

************************************* Start of TP*****************************************

### 5.2.x	CA_12-71

#### 5.2.x.1	Channel bandwidths per operating band for CA

Table 5.2.x.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
| 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

Table 5.2.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_12A-71A | - | 12 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 71 |  |  | Yes | Yes |  |  |  |  |

#### 5.2.x.2	Co-existence studies

Table 5.2.x.2-1 and Table 5.2.x.2-2 summarize frequency ranges where harmonics and/or harmonics mixing occur for CA_12-71.

Table 5.2.x.2-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 12 | 699 | 716 | 729 | 746 | 1398 | 1432 | 2097 | 2148 | 2796 | 2864 | 3495 | 3580 |
| 71 | 663 | 698 | 617 | 652 | 1326 | 1396 | 1989 | 2094 | 2652 | 2792 | 3315 | 3490 |

Table 5.2.x.2-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 12 | 699 | 716 | 729 | 746 | 1458 | 1492 | 2187 | 2238 | 2916 | 2984 | 3645 | 3730 |
| 71 | 663 | 698 | 617 | 652 | 1234 | 1304 | 1851 | 1956 | 2468 | 2608 | 3085 | 3260 |

As can be seen in Table 5.2.x.2-1 and Table 5.2.x.2-2 there are no harmonic or harmonic mixing issues identified.

However, there are cross band isolation interferences from UL 71 to DL 12.

#### 5.2.x.3	∆TIB and ∆RIB values

For CA_12-71, the TIB,c and RIB,c values are reused from NR CA_n12-n71 and are given in the tables below.

Table 5.2.x.3-1: TIB,c

| CA_12-71 | 12 | 1 |
| --- | --- | --- |
|  | 71 | 1 |

Table 5.2.x.3-2: RIB,c

| CA_12-71 | 12 | 0.8 |
| --- | --- | --- |
|  | 71 | 0.8 |

#### 5.2.x.4	REFSENS Requirements

Based on the co-existence studies there is a need to define additional REFSENS requirements. MSD value and test points are shown as below.

Table 5.2.x.4-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_12A-71A | 12 |  |  | -93.2 | -90.2 |  |  | FDD |
| CA_12A-71A | 71 |  |  | -96.4 | -93.4 |  |  | FDD |

Table 5.2.x.4-2: Uplink configuration for the uplink band

| E-UTRA Band / Channel bandwidth of the affected DL band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_12A-71A | 71 |  |  | 20 | 20 |  |  | FDD |
| CA_12A-71A | 12 |  |  | 20 | 20 |  |  | FDD |

************************************* End of TP*****************************************

# 2	References

[1] RP-240414_Revised WID: Rel-18 LTE-Advanced Carrier Aggregation for x bands (2<=x<= 6) DL with y bands (y=1, 2) UL