3GPP TSG-RAN WG4 Meeting # 110bis											R4-2404836

Changsha, China, 15 – 19 April, 2024

Agenda item:			6.21.4

Source:	Moderator (Ericsson)

Title:	Topic summary for [110][229] NR_redcap_enh

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

# Topic #1: Core part

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2404561 | Xiaomi | Proposal 1: The bracket in higher priority carrier measurement requirements specified in section 5.1B.2.7 of TS 38.133 could be removed. |
| R4-2405217 | ZTE Corporation | RAN4 should align the requirements on higher priority carriers between NR and LTE: UE shall search every layer of higher priority at least every Thigher_priority_search = max(60 * Nlayers , 1 * eDRX_IDLE cycle length). Proposal 2: For INACTIVE mode, the same search rate as IDLE mode applies with eDRX_INACTIVE cycle length instead of eDRX_IDLE cycle length. |
| R4-2405818 | Nokia | - When configured with eDRX_IDLE, UE shall search every layer of higher priority at least every Thigher_priority_search = max(60 * Nlayers, one eDRX_IDLE cycle) seconds. When configured with eDRX_INACTIVE, UE shall search every layer of higher priority at least every Thigher_priority_search = max(60 * Nlayers , 1 * eDRX_INACTIVE cycle length) if eDRX_INACTIVE cycle ≤ 10.24 sec or eDRX_INACTIVE cycle > 163.84 sec. When configured with eDRX_INACTIVE, UE shall search every layer of higher priority at least every Thigher_priority_search = max(60, 1 * eDRX_IDLE cycle length) * Nlayers if 10.24 sec < eDRX_INACTIVE cycle ≤ 163.84 sec. If Proposals 2 and 3 are not agreeable, RAN4 should have the same NR and LTE measurements on higher priority carriers by reusing LTE requirements: When configured with eDRX_IDLE, UE shall search every layer of higher priority at least every Thigher_priority_search = max(60 * Nlayers, one eDRX_INACTIVE cycle) seconds. |
| R4-2405940 | Qualcomm Incorporated | Proposal 1: RAN4 to follow the current principle to ensure that the high priority search rate with eDRX should be slower than the search rate for normal mode. Proposal 2: When eDRX is configured without PTW (i.e., for eDRX cycle lengths of <=10.24s), specify the high priority search rate as 24*eDRX_IDLE cycle length to ensure that the high priority search rate is not faster than the normal search rate. Proposal 3: When eDRX is configured with PTW (i.e., for eDRX cycle lengths of >10.24s), specify the high priority search rate as 12*eDRX_IDLE cycle length to ensure that the high priority search rate is not faster than the normal search rate. Proposal 4: For INACTIVE mode, the same search rate as IDLE mode applies with eDRX_INACTIVE cycle length instead of eDRX_IDLE cycle length. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1 Measurements on higher priority carriers in IDLE and INACTIVE mode with eDRX

Sub-topic description:

Open issues and candidate options before meeting:

Issue 1-3: Measurements on higher priority carriers in IDLE and INACTIVE mode when configured with eDRX

- Proposals

  - Option 1 (ZTE, Nokia, E///):

    - RAN4 should align the requirements on higher priority carriers between NR and LTE: UE shall search every layer of higher priority at least every Thigher_priority_search = max(60 * Nlayers , 1 *  eDRX_IDLE cycle length).

  - Option 2 (Xiaomi, E///): RAN4 to confirm the previous agreement on higher priority carrier measurement requirements by removing the [ ].

  - Option 3 (Nokia):

    - When configured with eDRX_IDLE:

      - UE shall search every layer of higher priority at least every Thigher_priority_search = max(60 * Nlayers, one eDRX_IDLE cycle) seconds.

    - When configured with eDRX_INACTIVE and if eDRX_INACTIVE cycle ≤ 10.24 sec or eDRX_INACTIVE cycle > 163.84 sec.

      - UE shall search every layer of higher priority at least every Thigher_priority_search = max(60 * Nlayers , 1 *  eDRX_INACTIVE cycle length)

    - When configured with eDRX_INACTIVE and if 10.24 sec < eDRX_INACTIVE cycle ≤ 163.84 sec.

      - UE shall search every layer of higher priority at least every Thigher_priority_search = max(60, 1 * eDRX_IDLE cycle length) * Nlayers

  - Option 4 (QC):

    - When eDRX is configured without PTW (i.e., for eDRX cycle lengths of <=10.24s), specify the high priority search rate as 24*eDRX_IDLE cycle length to ensure that the high priority search rate is not faster than the normal search rate.

    - When eDRX is configured with PTW (i.e., for eDRX cycle lengths of >10.24s), specify the high priority search rate as 12*eDRX_IDLE cycle length to ensure that the high priority search rate is not faster than the normal search rate.

    - For INACTIVE mode, the same search rate as IDLE mode applies with eDRX_INACTIVE cycle length instead of eDRX_IDLE cycle length.

- Recommended WF

  - Discussions needed.