# 3GPP TSG-RAN WG4 Meeting # 110-bis	R4-2406522

# Changsha, China, 15th – 19th April, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | draft | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft Big CR to TS 36.133 on performance requirements for IoT NTN enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_enh-Perf |  |  |  |  |  | Date: |  |  | 2024-4-5 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN4 endorsed the following CRs in WG4 Meeting # 110b for WI IoT_NTN_enh-Perf Perf part and test cases TDoc number; Title; Source; Summary. R4-2406315; DraftCR to 36.133 on Configuration of Cells in Neighbor Satellite; Nokia; A.3.28.5 Inter satellite configuration. R4-2405432; Draft CR on Test cases of cell re-selection and RRC reestablishment for NB IoT NTN enhancements; MediaTek inc.; NB, new test case. R4-2404995; Draft CR on TC for NB-IoT NTN CONN; Huawei, HiSilicon; NB, new test case. R4-2406316; (IoT_NTN_enh-Perf) draftCR to TS 36.133 Introduce of cell re-selection test cases for Cat-M1 UE; CMCC; M1, new test case. R4-2406319; DraftCR to 36.133 on Intra-Frequency HO for Cat-M1 UEs in CEModeA in NTN; Nokia; M1, new test case. R4-2406318; DraftCR to 36.133 on Inter-Frequency HO for Cat-M1 UEs in CEModeA in NTN; Nokia; M1, new test case. R4-2406320; DraftCR to 36.133 on Inter-Frequency Conditional HO for Cat-M1 UEs in NTN; Nokia; M1, new test case. R4-2406317; Draft CR for 36.133: E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA; Ericsson; M1, new test case. R4-2406321; Draft CR on Test cases of RSRP for Cat M1 NTN enhancements.; MediaTek inc.; M1, new test case. |
| Summary of change: |  | Changes of R4-2406315 (A.3.28.5) - Introduce Neighbor Satellite Configuration - Fix formatting on the title of subclause A.3.28.5.1 Changes of R4-2405432 (new A.13.1.1.4/5/6, A.13.3.1.3) - Introduce RRM Test cases of cell re-selection enhancement and inter-frequency RRC re-establishement for NB-IoT over NTN. Changes of R4-2404995 (new A.13.5): - Introduce following TC for R18 IoT NTN enh for NB-IoT: - HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage - HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage - HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage, location-based triggering Changes of R4-2406316 (new A.14.1.1.5/6/7/8/9/10): - Introduce following cell re-selection test cases for eMTC over NTN - EUTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage - E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage - E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage, time-based triggering - E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in [enhanced] coverage, time-based triggering - E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage, location-based triggering Changes of R4-2406319 (new A.14.2.1.5/6): - Introduce test cases for Intra-frequency HO for Cat-M1 UEs in NTN Changes of R4-2406318 (new A.14.2.1.7/8/9/10): - Introduce test cases for Inter-frequency HO for Cat-M1 UEs in NTN Changes of R4-2406320 (new A.14.2.1.11/12): - Introduce test cases for Inter-frequency Conditional HO for Cat-M1 UEs in NTN Changes of R4-2406317 (new A.14.3.1.3/4): - Test case for “E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA” is introduced. - Test case for “E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA” is introduced. Changes of R4-2406321 (new A.14.6.1.3/4/5/6): - Introduce RRM Test cases of RSRP inter frequency measurement accuracy for Cat-M1 over NTN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete test cases for IoT NTN enhancements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (existing) A.3.28.5 (new under A.13.1.1) A.13.1.1.4, A.13.1.1.5, A.13.1.1.6, A.13.3.1.3, A.13.5 (new under A.14.1.1) A.14.1.1.5, A.14.1.1.6, A.14.1.1.7, A.14.1.1.8, A.14.1.1.9, A.14.1.1.10 (new under A.14.2.1) A.14.2.1.5, A.14.2.1.6, A.14.121.7, A.14.2.1.8, A.14.2.1.9, A.14.2.1.10, A.14.2.1.11, A.14.2.1.12 (new under A.14.3.1) A.14.3.1.3, A.14.3.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

#### A.3.28.5.1	Satellite specific configuration for serving cell

The general parameters for SIB31 setup for serving satellite are specified in Table A.3.28.5.1-1.

Table A.3.28.5.1-1: SIB31/SIB31-NB parameters setup for Serving satellite

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference configuration for serving satellite |  | SSC.1 | SSC.2 |
| Scenario |  | GSO or GEO (as per test definition) | NGSO |
| Interval between adjacent epoch time | s | 10.24 | 2.56 |
| ul-SyncValidityDuration-r17 | s | 900 | 5 |
| k-Offset-r17 | slot | 258 | 14 |
| k-Mac-r17 | slot | Not configured | Not configured |
| nta-Common-r17 |  | 0 | 0 |
| nta-CommonDrift-r17 |  | 0 | 0 |
| nta-CommonDriftVariation-r17 |  | 0 | 0 |
| ephemerisInfo |  | According to Annex B.8 |  |

#### A.3.28.5.2	Satellite specific configuration for neighbour cell

The general parameters for SIB33 setup for neighbor cells in both serving and neighbour satellites are specified in Table A.3.28.5.2-1.

Table A.3.28.5.2-1: SIB33/SIB33-NB parameters setup for neighbour cell

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference configuration for satellite assistance information for neighbor cell |  | NSC.1 | NSC.2 | NSC.3 | NSC.4 |
| Scenario |  | GSO or GEO (as per test definition) | NGSO | GSO or GEO (as per test definition) | NGSO |
| Interval between adjacent epoch time | s | 10.24 | 2.56 | 10.24 | 2.56 |
| neighValidityDuration-r18 | s | 900 | 5 | 900 | 5 |
| k-Mac-r18 | slot | Not configured | Not configured | Not configured | Not configured |
| nta-Common-r18 |  | 0 | 0 | 0 | 0 |
| nta-CommonDrift-r18 |  | 0 | 0 | 0 | 0 |
| nta-CommonDriftVariation-r18 |  | 0 | 0 | 0 | 0 |
| ephemerisInfo |  | Same as used for serving satellite |  | According to Annex B.8 NOTE1 |  |
| t-ServiceStartNeigh-r18 |  | NOTE 2 |  |  |  |
| Note 1: The ephemeris information for those neighbor cells shall be different than that used for the serving cell, i.e., these cells are associated to a different satellite. Note 2: Unless otherwise stated in the test case, t-serviceStartNeigh-r18 is not configured for the neighbor satellite. |  |  |  |  |  |

## <<< NEXT CHANGE >>>

#### A.13.1.1.4	HD – FDD Inter frequency case for UE Category NB1 Standalone mode in normal coverage

##### A.13.1.1.4.1	Test Purpose and Environment

This test is to verify the requirement for the HD-FDD inter frequency cell reselection requirements for Cat-NB1 UE specified in clause 4.6A.2.5.

The test scenario comprises of 2 cells as given in tables A.13.1.1.4.1-1, A.13.1.1.4.1-2 and A.13.1.1.4.1-3. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. Only nCell1 is already identified by the UE prior to the start of the test, i.e. nCell 2 is not identified. nCell 1 and nCell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing nCell 2.

Table A.13.1.1.4.1-1: Supported test configurations

| Configuration | Description |  |
| --- | --- | --- |
| 1 | GSO, HD-FDD duplex mode |  |
| 2 | NGSO, HD-FDD duplex mode |  |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

Table A.13.1.1.4.1-2: General test parameters for HD-FDD inter frequency cell reselection test case for Cat-NB1 UE in normal coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IOT operational mode |  |  | Standalone |  |
| Satellite information | Config 1 |  | SSC.1 for nCell1 NSC.1 for nCell2 | GSO |
|  | Config 2 |  | SSC.2 for nCell1 NSC.2 for nCell2 | NGSO |
| Initial condition | Active cell |  | nCell1 |  |
|  | Neighbour cells |  | nCell2 |  |
| T2 end condition | Active cell |  | nCell2 |  |
|  | Neighbour cells |  | nCell1 |  |
| Final condition | Visited cell |  | nCell1 |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| T0 |  | s | 5 | During T0, UE decodes SIB3-NB and SIB5-NB to acquire the inter-frequency carrier information. |
| T1 |  | s | >7 | During T1, nCell2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that nCell2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 60 | T2 is defined so that cell re-selection time is taken into account. |
| T3 |  | s | 15 | T3 is defined so that cell re-selection time is taken into account. |

Table A.13.1.1.4.1-3: nCell 1, nCell 2 specific test parameters for HD-FDD inter frequency cell reselection test case for Cat-NB1 UE in normal coverage

|  |  | nCell 1 |  |  | nCell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | kHz | 180 |  |  | 180 |  |  |
| NPBCH_RA | dB | 0 |  |  | 0 |  |  |
| NPBCH_RB | dB |  |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |  |  |
| NPDSCH_RA | dB |  |  |  |  |  |  |
| NPDSCH_RB | dB |  |  |  |  |  |  |
| NOCNG_RANote 1 | dB |  |  |  |  |  |  |
| NOCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Qrxlevmin | dBm | -140 |  |  | -140 |  |  |
| Pcompensation | dB | 0 |  |  | 0 |  |  |
| Qhysts | dB | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | NRSRP |  |  | NRSRP |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |  |
|  | dB | 17 | 13 | 17 | -infinity | 17 | 13 |
| Note2 | dB | 17 | 13 | 17 | -infinity | 17 | 13 |
| NRSRP Note2 | dBm/15 kHz | -81 | -85 | -81 | -infinity | -81 | -85 |
| Treselection | s | 0 |  |  | 0 |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x1 |  |  | 1x1 |  |  |
| Timing offset to nCell 1 | ms | 0 |  |  |  |  |  |
| Note 1: NOCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Es/Iot and NRSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

##### A.13.1.1.4.2	Test Requirements

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on nCell 2, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on nCell 2.

The cell re-selection delay to a newly detectable cell shall be less than 59.32 s.

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on nCell 1, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on nCell 1.

The cell re-selection delay to an already detected cell shall be less than 14.82 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a newly detectable cell can be expressed as:  Tdetect,NB_Inter_EC + TSI, and to an already detected cell can be expressed as: Tevaluate, NB_Inter_EC + TSI,

Where:

Tdetect,NB_Inter_EC	See Table 4.6A.2.5-1 in clause 4.6A.2.5

Tevaluate, NB_Inter_EC	See Table 4.6A.2.5-1 in clause 4.6A.2.5

TSI	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 8.32 s is assumed in this test case.

This gives a total of 59.32 s, allow 60 s for the cell re-selection delay to a newly detectable cell and 14.82 s, allow 15 s for the cell re-selection delay to an already detected cell in the test case.

#### A.13.1.1.5	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in enhanced coverage, location-based cell reselection for NGSO

##### A.13.1.1.5.1	Test Purpose and Environment

This test is to verify the requirement for the HD-FDD intra frequency cell reselection requirements for Cat-NB1 UE specified in clause 4.6A.2.4.

The test scenario comprises of one NB-IoT carrier with 2 nCells of different physical cell ID, as given in tables A.13.1.1.5.1-1, A.13.1.1.5.1-2 and A.13.1.1.5.1-3. The test consists of 3 successive time periods, with time duration of T0, T1, and T2 respectively. Only nCell1 is already identified by the UE prior to the start of the test, i.e. nCell 2 is not identified. nCell 1 and nCell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing nCell 2.

The UE shall be provided with the valid information about the SAN serving cells before the test.

At 4s after the start of T2, the UE location is changed such that the distance to the reference location broadcasted in SIB31 of Cell 1 is exceeded by the configured value in distanceThresh plus 50m.

Table A.13.1.1.5.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | NGSO, HD-FDD duplex mode |

Table A.13.1.1.5.1-2: General test parameters for HD-FDD intra frequency cell reselection test case for Cat-NB1 UE in enhanced coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IOT operational mode |  |  | Standalone |  |
| Satellite information | Config 1 |  | SSC.2 for nCell1 NSC.2 for nCell2 | NGSO |
| Initial condition | Active cell |  | nCell1 |  |
|  | Neighbour cells |  | nCell2 |  |
| T2 end condition | Active cell |  | nCell2 |  |
|  | Neighbour cells |  | nCell1 |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| T1 |  | s | >7 | During T1, nCell2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that nCell2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 71 | T2 is defined so that cell re-selection time is taken into account. |

Table A.13.1.1.5.1-3: nCell 1, nCell 2 specific test parameters for HD-FDD intra frequency cell reselection test case for Cat-NB1 UE in enhanced coverage

| Parameter | Unit | nCell 1 |  |  | nCell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T0 | T1 | T2 | T0 | T1 | T2 |
| BWchannel | kHz | 200 |  |  | 200 |  |  |
| OCNG Pattern as defined in A.3 | - | NOP.3 FDD |  |  | NOP.3 FDD |  |  |
| NPBCH_RA | dB | 0 |  |  | 0 |  |  |
| NPBCH_RB | dB |  |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |  |  |
| NPDSCH_RA | dB |  |  |  |  |  |  |
| NPDSCH_RB | dB |  |  |  |  |  |  |
| NOCNG_RANote 1 | dB |  |  |  |  |  |  |
| NOCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Qrxlevmin | dBm | -156 |  |  | -156 |  |  |
| Pcompensation | dB | 0 |  |  | 0 |  |  |
| Qhysts | dB | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | NRSRP |  |  | NRSRP |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |  |
|  | dB | 10 | -9 | -9 | -infinity | -infinity | -0.7 |
| Note2 | dB | 10 | -9 | -11.67 | -infinity | -infinity | -1.21 |
| NRSRP Note2 | dBm/15 kHz | -88 | -107 | -107 | -infinity | -infinity | -98.7 |
| Treselection | s | 0 | 0 | 0 | 0 | 0 | 0 |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x1 |  |  | 1x1 |  |  |
| Timing offset to nCell 1 | ms | - |  |  | 3 |  |  |
| Note 1: NOCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Es/Iot and NRSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

##### A.13.1.1.5.2	Test Requirements

The cell reselection delay to a newly detectable cell is defined as the time from 4s after the beginning of time period T2, to the moment when the UE camps on nCell 2, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on nCell 2.

The cell re-selection delay to a newly detectable cell shall be less than 66.32 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a newly detectable cell can be expressed as: Tdetect,NB_Intra_NB-IoT-NC + TSI, and to an already detected cell can be expressed as: Tevaluate, NB_intra_NB-IoT-NC + TSI,

Where:

Tdetect,NB_Intra_NB-IoT-NC		See Table 4.6A.2.4-1 in clause 4.6A.2.4

Tevaluate, NB_intra_NB-IoT-NC	See Table 4.6A.2.4-1 in clause 4.6A.2.4

TSI	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 8.32 s is assumed in this test case.

This gives a total of (66.32+4) s, allow 69 s after T2, for the cell re-selection delay to a newly detectable cell.

#### A.13.1.1.6	HD – FDD Inter frequency case for UE Category NB1 Standalone mode in enhanced coverage, time-based cell reselection for NGSO

##### A.13.1.1.6.1	Test Purpose and Environment

This test is to verify the requirement for the HD-FDD inter frequency cell reselection requirements for Cat-NB1 UE specified in clause 4.6A.2.6.

The test scenario comprises of 2 cells as given in tables A.13.1.1.6.1-1, A.13.1.1.6.1-2 and A.13.1.1.6.1-3. The test consists of 3 successive time periods, with time duration of T0, T1, and T2 respectively. Only nCell1 is already identified by the UE prior to the start of the test, i.e. nCell 2 is not identified. nCell 1 and nCell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing nCell 2.

t-Service broadcasted in SystemInformationBlockType3-NB of Cell 1 is set to the time point that is 67s after start of T2.

Table A.13.1.1.6.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | NGSO, HD-FDD duplex mode |

Table A.13.1.1.6.1-2: General test parameters for HD-FDD inter frequency cell reselection test case for Cat-NB1 UE in enhanced coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IOT operational mode |  |  | Standalone |  |
| Satellite information | Config 1 |  | SSC.2 for nCell1 NSC.2 for nCell2 | NGSO |
| Initial condition | Active cell |  | nCell1 |  |
|  | Neighbour cells |  | nCell2 |  |
| T2 end condition | Active cell |  | nCell2 |  |
|  | Neighbour cells |  | nCell1 |  |
| E-UTRA RF Channel Number |  |  | 1 | One carrier frequency is used for eCell. |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| T0 |  | s | 5 | During T0, UE decodes SIB3-NB and SIB5-NB to acquire the inter-frequency carrier information. |
| T1 |  | s | >7 | During T1, nCell2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that nCell2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 70 | T2 is defined so that cell re-selection time is taken into account. |

Table A.13.1.1.6.1-3: nCell 1, nCell 2 specific test parameters for HD-FDD inter frequency cell reselection test case for Cat-NB1 UE in enhanced coverage

|  |  | nCell 1 |  |  | nCell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T0 | T1 | T2 | T0 | T1 | T2 |
| BWchannel | kHz | 180 |  |  | 180 |  |  |
| NPBCH_RA | dB | 0 |  |  | 0 |  |  |
| NPBCH_RB | dB |  |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |  |  |
| NPDSCH_RA | dB |  |  |  |  |  |  |
| NPDSCH_RB | dB |  |  |  |  |  |  |
| NOCNG_RANote 1 | dB |  |  |  |  |  |  |
| NOCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Qrxlevmin | dBm | -156 |  |  | -156 |  |  |
| Pcompensation | dB | 0 |  |  | 0 |  |  |
| Qhysts | dB | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | NRSRP |  |  | NRSRP |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |  |
|  | dB | 10 | -12 | -12 | -infinity | -infinity | -2.7 |
| Note2 | dB | 10 | -12 | -12 | -infinity | -infinity | -2.7 |
| NRSRP Note2 | dBm/15 kHz | -88 | -110 | -110 | -infinity | -infinity | -100.7 |
| Treselection | s | 0 |  |  | 0 |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x1 |  |  | 1x1 |  |  |
| Timing offset to nCell 1 | ms | 0 |  |  |  |  |  |
| Note 1: NOCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Es/Iot and NRSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

##### A.13.1.1.6.2	Test Requirements

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on nCell 2, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on nCell 2.

The cell re-selection delay to a newly detectable cell shall be less than 66.32 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a newly detectable cell can be expressed as:  Tdetect,NB_Inter_EC + TSI, and to an already detected cell can be expressed as: Tevaluate, NB_Inter_EC + TSI,

Where:

Tdetect,NB_Inter_EC	See Table 4.6A.2.6-1 in clause 4.6A.2.6

Tevaluate, NB_Inter_EC	See Table 4.6A.2.6-1 in clause 4.6A.2.6

TSI	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 8.32 s is assumed in this test case.

This gives a total of 66.32 s, allow 67 s for the cell re-selection delay to a newly detectable cell.

## <<< NEXT CHANGE >>>

#### A.13.3.1.3	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage

##### A.13.3.1.3.1	Test Purpose and Environment

The purpose is to verify that the NB-IoT HD-FDD inter-frequency RRC re-establishment delay is within the specified limits. These tests will verify the requirements for Cat-NB1 UE in clause 6.5A.

The test parameters are given in table A.13.3.1.3.1-1, table A.13.3.1.3.1-2 and table A.13.3.1.3.1-3 below. nCell1 and nCell2 are NB-IoT cells with different physical cell ID on the different frequency carrier. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, is deactivated. The time period T3 starts after the occurrence of the radio link failure. During T1, the UE shall be indicated with the carrier frequency of nCell 2 to ensure that the UE has the context of the carrier frequency of nCell 2.

Table A.13.3.1.3.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

Table A.13.3.1.3.1-2: General test parameters for HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IOT operational mode |  |  | Standalone |  |
| Satellite information | Config 1 |  | SSC.1 for nCell1 NSC.1 for nCell2 |  |
|  | Config 2 |  | SSC.2 for nCell1 NSC.2 for nCell2 |  |
| Initial condition | Active cell |  | nCell1 |  |
|  | Neighbour cells |  | nCell2 |  |
| Final condition | Active cell |  | nCell2 |  |
| E-UTRA RF Channel Number |  |  | 1 | One carrier frequency is used for eCell1 and eCell2. |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| NPDCCH repetition level |  |  | 16 | NPDCCH Rmax |
| N310 |  | - | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | Ms | 0 | Radio link failure timer; T310 is disabled |
| T311 |  | Ms | 60000 | RRC re-establishment timer |
| DRX |  |  | OFF |  |
| T1 |  | S | 5 |  |
| T2 |  | Ms | 400 |  |
| T3 |  | S | 60 |  |

Table A.13.3.1.3.1-3: nCell 1, nCell 2 specific test parameters for HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage

| Parameter | Unit | nCell 1 |  |  | nCell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | kHz | 200 |  |  | 200 |  |  |
| NPDSCH parameters |  | R.18 HD-FDD |  |  | R.18 HD-FDD |  |  |
| NPDCCH parameters |  | R.30 HD-FDD |  |  | R.30 HD-FDD |  |  |
| NPBCH_RA | dB | 0 |  |  | 0 |  |  |
| NPBCH_RB | dB |  |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |  |  |
| NPDSCH_RA | dB |  |  |  |  |  |  |
| NPDSCH_RB | dB |  |  |  |  |  |  |
| NOCNG_RANote 1 | dB |  |  |  |  |  |  |
| NOCNG_RBNote 1 | dB |  |  |  |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |  |
|  | dB | 7 | -Infinity | -Infinity | -Infinity | -12.6 | -12.6 |
| Note2 | dB | 7 | -Infinity | -Infinity | -Infinity | -12.6 | -12.6 |
| NRSRP Note2 | dBm/15 kHz | -91 | -Infinity | -Infinity | -Infinity | -110.6 | -110.6 |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x1 |  |  | 1x1 |  |  |
| Timing offset to nCell 1 | ms | - |  |  | 3 |  |  |
| Note 1: NOCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Es/Iot and NRSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

##### A.13.3.1.3.2	Test Requirements

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send NPRACH preambles to cell 2 for sending the RRCConnectionReestablishmentRequest message to cell 2.

The RRC re-establishment delay to an unknown NB-IoT HD-FDD inter frequency cell shall be less than 58 s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE-re-establish_delay_NB-IoT.

Where:

-	TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The NPRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

-	TUE-re-establish_delay_NB-IoT = 100 ms + NNB-Iot-freq*Tsearch_NB-IoT + TSI_NB-IoT + TPRACH_NB-IoT

-	NNB-Iot-freq = 1

-	Tsearch_NB-IoT = 14800 ms

-	TSI_NB-IoT = 41560 ms; it is the time required for receiving all the relevant system information as defined in TS 36.331 for the target NB-IoT FDD cell.

-	TPRACH_NB-IoT = 1280 ms; it is the additional delay caused by the random access procedure.

## <<< NEXT CHANGE >>>

## A.13.5	UE measurement procedures in RRC_CONNECTED state for UE category NB1 for satellite access

### A.13.5.1	HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access

#### A.13.5.1.1	Test Purpose and Environment

The purpose is to verify that the NB-IoT intra-frequency neighbour cell measurement requirement in clause 8.14A.6.3 is met.

The test parameters are given in table A.13.5.1.1-1, table A.13.5.1.1-2 and table A.13.5.1.1-3 below. nCell1 and nCell2 are NB-IoT cells with different physical cell ID on the same frequency carrier. The test consists of 5 successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively.

Table A.13.5.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

Table A.13.5.1.1-2: General test parameters for HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IOT operational mode |  |  | standalone |  |
| Initial condition | Active cell |  | nCell1 |  |
|  | Neighbour cells |  | nCell2 |  |
| Final condition | Active cell |  | nCell2 |  |
| Satellite information | Config 1 |  | SSC.1 |  |
|  | Config 2 |  | SSC.2 |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| NPRACH Configuration |  |  | NPRACH.R-1 | Refer to A.3.18 |
| NPDCCH repetition level |  |  | 8 | NPDCCH Rmax |
| N310 |  | - | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 0 | Radio link failure timer; T310 is disabled |
| T311-v13xy |  | ms | 15000 | RRC re-establishment timer |
| DRX |  |  | OFF |  |
| T1 |  | ms | 5000 |  |
| T2 |  | ms | 900 |  |
| T3 |  | ms | 3100 |  |
| T4 |  | ms | 500 |  |
| T5 |  | ms | 8520 |  |
| s-MeasureIntra |  | dBm | -95 |  |
| s-MeasureDeltaP |  | dB | 6 |  |
| t-MeasureDeltaP |  | s | 60 |  |

Table A.13.5.1.1-3: General test parameters for HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage

| Parameter | Unit | nCell 1 |  |  |  |  | nCell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| BWchannel | kHz | 200 |  |  |  |  | 200 |  |  |  |  |
| NPDSCH parameters |  | R.18 HD-FDD |  |  |  |  | R.18 HD-FDD |  |  |  |  |
| NPDCCH parameters |  | R.30 HD-FDD |  |  |  |  | R.30 HD-FDD |  |  |  |  |
| NOCNG Patterns |  | NOP.3 FDD |  |  |  |  | NOP.3 FDD |  |  |  |  |
| NPBCH_RA | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| NPBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| NPDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| NPDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| NOCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| NOCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |  |  |  |  |  |
|  | dB | 9 | -3 | -8.5 | -Infinity | -Infinity | -Infinity | -Infinity | 2.2 | 4 | 4 |
| Note2 | dB | 9 | -3 | -8.5 | -Infinity | -Infinity | -Infinity | -Infinity | 2.2 | 4 | 4 |
| NRSRP Note2 | dBm/15 kHz | -89 | -101 | -101 | -Infinity | -Infinity | -Infinity | -Infinity | -94 | -94 | -94 |
| Propagation Condition |  | AWGN |  |  |  |  | AWGN |  |  |  |  |
| Antenna Configuration |  | 1x1 |  |  |  |  | 1x1 |  |  |  |  |
| Timing offset to nCell 1 | ms | - |  |  |  |  | 3 |  |  |  |  |
| Note 1: NOCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Es/Iot and NRSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |

#### A.13.5.1.2	Test Requirements

UE shall trigger RLF during T4 and complete neighbour cell measurement before end of T4. UE shall start to send NPRACH preambles to cell 2 for sending the RRCConnectionReestablishmentRequest message to cell 2 before the end of T5 to fulfil the RRC re-establishment delay to a known NB-IoT FDD intra frequency cell.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE-re-establish_delay_NB-IoT.

Where:

-	TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The NPRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

-	TUE-re-establish_delay_NB-IoT = 100 ms + NNB-Iot-freq*Tsearch_NB-IoT + TSI_NB-IoT + TPRACH_NB-IoT

-	NNB-Iot-freq = 1

-	Tsearch_NB-IoT = 0 ms

-	TSI_NB-IoT = 8320 ms; it is the time required for receiving all the relevant system information as defined in TS 36.331 for the target NB-IoT FDD cell.

-	TPRACH_NB-IoT = 80 ms; it is the additional delay caused by the random access procedure.

### A.13.5.2	HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access

#### A.13.5.2.1	Test Purpose and Environment

The purpose is to verify that the NB-IoT inter-frequency neighbour cell measurement requirement in clause 8.14A.6.3 is met.

The test parameters are given in table A.13.5.2.1-1, table A.13.5.2.1-2, table A.13.5.2.1-3 and table A.13.5.2.1-4 below. nCell1 and nCell2 are NB-IoT cells with different physical cell ID on the different frequency carriers. The test consists of 5 successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively.

Table A.13.5.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

Table A.13.5.2.1-2: General test parameters for HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IOT operational mode |  |  | standalone |  |
| Initial condition | Active cell |  | nCell1 |  |
|  | Neighbour cells |  | nCell2 |  |
| Final condition | Active cell |  | nCell2 |  |
| Satellite information | Config 1 |  | SSC.1 |  |
|  | Config 2 |  | SSC.2 |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| NPRACH Configuration |  |  | NPRACH.R-1 | Refer to A.3.18 |
| NPDCCH repetition level |  |  | 8 | NPDCCH Rmax |
| N310 |  | - | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 0 | Radio link failure timer; T310 is disabled |
| T311-v13xy |  | ms | 15000 | RRC re-establishment timer |
| DRX |  |  | OFF |  |
| T1 |  | ms | 5000 |  |
| T2 |  | ms | 1300 |  |
| T3 |  | ms | 8500 |  |
| T4 |  | ms | 5200 |  |
| T5 |  | ms | 8520 |  |
| s-MeasureInter |  | dBm | -95 |  |
| s-MeasureDeltaP |  | dB | 6 |  |
| t-MeasureDeltaP |  | s | 60 |  |

Table A.13.5.2.1-3: General test parameters for HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access

| Parameter | Unit | nCell 1 |  |  |  |  | nCell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| BWchannel | kHz | 200 |  |  |  |  | 200 |  |  |  |  |
| NPDSCH parameters |  | R.18 HD-FDD |  |  |  |  | R.18 HD-FDD |  |  |  |  |
| NPDCCH parameters |  | R.30 HD-FDD |  |  |  |  | R.30 HD-FDD |  |  |  |  |
| NOCNG Patterns |  | NOP.3 FDD |  |  |  |  | NOP.3 FDD |  |  |  |  |
| NPBCH_RA | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| NPBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| NPDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| NPDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| NOCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| NOCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |  |  |  |  |  |
|  | dB | 9 | -3 | -3 | -Infinity | -Infinity | -Infinity | -Infinity | 4 | 4 | 4 |
| Note2 | dB | 9 | -3 | -3 | -Infinity | -Infinity | -Infinity | -Infinity | 4 | 4 | 4 |
| NRSRP Note2 | dBm/15 kHz | -89 | -101 | -101 | -Infinity | -Infinity | -Infinity | -Infinity | -94 | -94 | -94 |
| Propagation Condition |  | AWGN |  |  |  |  | AWGN |  |  |  |  |
| Antenna Configuration |  | 1x1 |  |  |  |  | 1x1 |  |  |  |  |
| Timing offset to nCell 1 | ms | - |  |  |  |  | 3 |  |  |  |  |
| Note 1: NOCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Es/Iot and NRSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |

Table A.13.5.2.1-4: DRX-Configuration for HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | pp1 | As specified in clause 6.7.3 in TS 36.331 |
| drx-InactivityTimer | pp0 |  |
| drx-RetransmissionTimer | pp0 |  |
| drx-StartOffset | 0 |  |

#### A.13.5.2.2	 Test Requirements

UE shall trigger RLF during T4 and complete neighbour cell measurement before end of T4. UE shall start to send NPRACH preambles to cell 2 for sending the RRCConnectionReestablishmentRequest message to cell 2 before the end of T5 to fulfil the RRC re-establishment delay to a known NB-IoT FDD inter frequency cell.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE-re-establish_delay_NB-IoT.

Where:

-	TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The NPRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

-	TUE-re-establish_delay_NB-IoT = 100 ms + NNB-Iot-freq*Tsearch_NB-IoT + TSI_NB-IoT + TPRACH_NB-IoT

-	NNB-Iot-freq = 1

-	Tsearch_NB-IoT = 0 ms

-	TSI_NB-IoT = 8320 ms; it is the time required for receiving all the relevant system information as defined in TS 36.331 for the target NB-IoT FDD cell.

-	TPRACH_NB-IoT = 80 ms; it is the additional delay caused by the random access procedure.

### A.13.5.3	HD-FDD Intra-frequency location-based neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access

#### A.13.5.3.1	Test Purpose and Environment

The purpose is to verify that the NB-IoT intra-frequency neighbour cell measurement requirement in clause 8.14A.6.3 is met.

The test parameters are given in table A.13.5.3.1-1, table A.13.5.3.1-2 and table A.13.5.3.1-3 below. nCell1 and nCell2 are NB-IoT cells with different physical cell ID on the same frequency carrier. The test consists of 4 successive time periods, with time duration of T1, T2, T3, T4 respectively.

At the start of T2, the UE location is changed such that the distance to the reference location broadcasted in SystemInformationBlockType31-NB of nCell 1 is exceeded by the configured value in distanceThresh plus 50m.

Table A.13.5.3.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

Table A.13.5.3.1-2: General test parameters for HD-FDD Intra-frequency location-based neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IOT operational mode |  |  | standalone |  |
| Initial condition | Active cell |  | nCell1 |  |
|  | Neighbour cells |  | nCell2 |  |
| Final condition | Active cell |  | nCell2 |  |
| Satellite information | Config 1 |  | SSC.1 |  |
|  | Config 2 |  | SSC.2 |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| NPRACH Configuration |  |  | NPRACH.R-1 | Refer to A.3.18 |
| NPDCCH repetition level |  |  | 8 | NPDCCH Rmax |
| N310 |  | - | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 0 | Radio link failure timer; T310 is disabled |
| T311-v13xy |  | ms | 15000 | RRC re-establishment timer |
| DRX |  |  | OFF |  |
| T1 |  | ms | 5000 |  |
| T2 |  | ms | 3100 |  |
| T3 |  | ms | 500 |  |
| T4 |  | ms | 8520 |  |
| s-MeasureIntra |  | dBm | -95 |  |
| s-MeasureDeltaP |  | dB | 6 |  |
| t-MeasureDeltaP |  | s | 60 |  |

Table A.13.5.3.1-3: General test parameters for HD-FDD location-based Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage

| Parameter | Unit | nCell 1 |  |  |  | nCell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| BWchannel | kHz | 200 |  |  |  | 200 |  |  |  |
| NPDSCH parameters |  | R.18 HD-FDD |  |  |  | R.18 HD-FDD |  |  |  |
| NPDCCH parameters |  | R.30 HD-FDD |  |  |  | R.30 HD-FDD |  |  |  |
| NOCNG Patterns |  | NOP.3 FDD |  |  |  | NOP.3 FDD |  |  |  |
| NPBCH_RA | dB | 0 |  |  |  | 0 |  |  |  |
| NPBCH_RB | dB |  |  |  |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |  |  |  |  |
| NPDSCH_RA | dB |  |  |  |  |  |  |  |  |
| NPDSCH_RB | dB |  |  |  |  |  |  |  |  |
| NOCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |
| NOCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |  |  |  |
|  | dB | 9 | 3.5 | -Infinity | -Infinity | -Infinity | -5.5 | 4 | 4 |
| Note2 | dB | 9 | 3.5 | -Infinity | -Infinity | -Infinity | -5.5 | 4 | 4 |
| NRSRP Note2 | dBm/15 kHz | -89 | -89 | -Infinity | -Infinity | -Infinity | -94 | -94 | -94 |
| Propagation Condition |  | AWGN |  |  |  | AWGN |  |  |  |
| Antenna Configuration |  | 1x1 |  |  |  | 1x1 |  |  |  |
| Timing offset to nCell 1 | ms | - |  |  |  | 3 |  |  |  |
| Note 1: NOCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Es/Iot and NRSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |

#### A.13.5.3.2	Test Requirements

UE shall trigger RLF during T3 and complete neighbour cell measurement before end of T3. UE shall start to send NPRACH preambles to cell 2 for sending the RRCConnectionReestablishmentRequest message to cell 2 before the end of T4 to fulfil the RRC re-establishment delay to a known NB-IoT FDD intra frequency cell.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE-re-establish_delay_NB-IoT.

Where:

-	TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The NPRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

-	TUE-re-establish_delay_NB-IoT = 100 ms + NNB-Iot-freq*Tsearch_NB-IoT + TSI_NB-IoT + TPRACH_NB-IoT

-	NNB-Iot-freq = 1

-	Tsearch_NB-IoT = 0 ms

-	TSI_NB-IoT = 8320 ms; it is the time required for receiving all the relevant system information as defined in TS 36.331 for the target NB-IoT FDD cell.

-	TPRACH_NB-IoT = 80 ms; it is the additional delay caused by the random access procedure.

## <<< NEXT CHANGE >>>

#### A.14.1.1.5	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage

##### A.14.1.1.5.1	Test Purpose and Environment

This test is to verify the requirement for the FDD-FDD inter frequency cell reselection requirements for category M1 UE in normal coverage for satellite access specified in clause 4.7A.2.1.3.

The supported test configurations are provided in Table A.14.1.1.5.1-1.

Table A.14.1.1.5.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, FDD-FDD duplex mode |
| 2 | NGSO, FDD-FDD duplex mode |

The test scenario comprises of 2 E-UTRA FDD cells on 2 different carriers as given in tables A.14.1.1.5.1-2 and A.14.1.1.5.1-3. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas and cell 2 is of higher priority than cell 1. Furthermore, UE has not registered with network for the tracking area containing cell 2.

Table A.14.1.1.5.1-2: General test parameters for FDD-FDD inter frequency cell reselection test case for Cat-M1 UE in normal coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell2 | UE shall be forced to cell 2 in the initialisation phase, so that reselection to cell 1 occurs during the first T1 phase |
| T1 end condition | Active cell |  | Cell1 | UE shall perform reselection to cell 1 during T1 |
|  | Neighbour cell |  | Cell2 |  |
| Final condition | Active cell |  | Cell2 | UE shall perform reselection to cell 2 during T3 |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two FDD carrier frequencies are used. |
| PRACH configuration |  |  | PRACH_2CE | See table in A.3.16 |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| T1 |  | s | 15 | T1 need to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | >7 | During T2, cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T3. |
| T3 |  | s | 75 | T3 need to be defined so that cell re-selection reaction time is taken into account. |

Table A.14.1.1.5.1-3: Cell specific test parameters for FDD-FDD inter frequency cell reselection test case for Cat-M1 UE in normal coverage

| Parameter | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |  | T1 | T2 | T3 |
| Satellite information |  | 1 | NSC.1 | SSC.1 | SSC.1 |  | SSC.1 | NSC.1 | NSC.1 |
|  |  | 2 | NSC.2 | SSC.2 | SSC.2 |  | SSC.2 | NSC.2 | NSC.2 |
| E-UTRA RF Channel number |  | 1, 2 | 1 |  |  |  | 2 |  |  |
| BWchannel | MHz | 1, 2 | 1.4 |  |  |  | 1.4 |  |  |
| OCNG Patterns defined in A.3.2.1.6 |  | 1, 2 | OP.7 FDD |  |  |  | OP.7 FDD |  |  |
| PBCH_RA | dB | 1, 2 | -3 |  |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |
| MPDCCH_RA | dB | 1, 2 |  |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |  |  |  |
| dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |
| Qrxlevmin | dBm | 1, 2 | -140 |  |  |  | -140 |  |  |
| Note 2 | dBm/15 kHz | 1, 2 | -98 |  |  |  |  |  |  |
|  | dB | 1, 2 | 14 | 14 | 14 |  | -4 | -infinity | 12 |
|  | dB | 1, 2 | 14 | 14 | 14 |  | -4 | -infinity | 12 |
| RSRP Note 3 | dBm/15 KHz | 1, 2 | -84 | -84 | -84 |  | -102 | -infinity | -86 |
| TreselectionEUTRAN | s | 1, 2 | 0 |  |  |  | 0 |  |  |
| Snonintrasearch | dB | 1, 2 | 50 |  |  |  | Not sent |  |  |
| Threshx, high | dB | 1, 2 | 48 |  |  |  | 48 |  |  |
| Threshserving, low | dB | 1, 2 | 44 |  |  |  | 44 |  |  |
| Threshx, low | dB | 1, 2 | 50 |  |  |  | 50 |  |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |  |  |  |
| Antenna Configuration |  | 1, 2 | 1x1 |  |  | 1x1 |  |  |  |
| Timing offset to Cell 1 | ms | 1, 2 | - |  |  | 3 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |

##### A.14.1.1.5.2	Test Requirements

The cell reselection delay to higher priority is defined as the time from the beginning of time period T3, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to higher priority shall be less than 68 s.

The cell reselection delay to lower priority is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on cell 1.

The cell re-selection delay to lower priority shall be less than 8 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to higher priority cell can be expressed as: Thigher_priority_search + Tevaluate, E-UTRAN_Inter_NC + TSI-EUTRA-M1-NC , and to lower priority cell can be expressed as: Tevaluate, E-UTRAN_Inter_NC + TSI-EUTRA-M1-NC,

Where:

Thigher_priority_search	See clause 4.7A.2.1.3

Tevaluate, E-UTRAN_Inter_NC	See Table 4.7A.2.1.3-1 in clause 4.7A.2.1.3

TSI-EUTRA-M1-NC	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case provided that SIB31 and SIB33 are scheduled with 80 ms period.

This gives a total of 67.68 s for higher priority cell search and 7.68 s for lower priority cell search, allow 68 s for higher priority cell and 8 s for lower priority cell in the test case.

#### A.14.1.1.6	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage

##### A.14.1.1.6.1	Test Purpose and Environment

This test is to verify the requirement for the HD-FDD inter frequency cell reselection requirements for category M1 UE in normal coverage for satellite access specified in clause 4.7A.2.1.3.

The supported test configurations are provided in Table A.14.1.1.6.1-1.

Table A.14.1.1.6.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |

The test scenario comprises of 2 E-UTRA carriers and 2 cells as given in tables A.14.1.1.6.1-2 and A.14.1.1.6.1-3. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas and cell 2 is of higher priority than cell 1. Furthermore, UE has not registered with network for the tracking area containing cell 2.

Table A.14.1.1.6.1-2: General test parameters for HD-FDD inter frequency cell reselection test case for Cat-M1 UE in normal coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell2 | UE shall be forced to cell 2 in the initialisation phase, so that reselection to cell 1 occurs during the first T1 phase |
| T1 end condition | Active cell |  | Cell1 | UE shall perform reselection to cell 1 during T1 |
|  | Neighbour cell |  | Cell2 |  |
| Final condition | Active cell |  | Cell2 | UE shall perform reselection to cell 2 during T3 |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two FDD carrier frequencies are used. |
| PRACH configuration |  |  | PRACH_2CE | See table in A.3.16 |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| T1 |  | s | 15 | T1 need to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | >7 | During T2, cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T3. |
| T3 |  | s | 75 | T3 need to be defined so that cell re-selection reaction time is taken into account. |

Table A.14.1.1.6.1-3: Cell specific test parameters for HD-FDD inter frequency cell reselection test case for Cat-M1 UE in normal coverage

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Satellite information |  | 1 | NSC.1 | SSC.1 | SSC.1 | SSC.1 | NSC.1 | NSC.1 |
|  |  | 2 | NSC.2 | SSC.2 | SSC.2 | SSC.2 | NSC.2 | NSC.2 |
| E-UTRA RF Channel number |  | 1, 2 | 1 |  |  | 2 |  |  |
| BWchannel | MHz | 1, 2 | 1.4 |  |  | 1.4 |  |  |
| OCNG Patterns defined in A.3.2.1.6 |  | 1, 2 | OP.7 FDD |  |  | OP.7 FDD |  |  |
| PBCH_RA | dB | 1, 2 | -3 |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |  |  |  |
| PDSCH_RA | dB | 1, 2 |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| dB |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |
| Qrxlevmin | dBm | 1, 2 | -140 |  |  | -140 |  |  |
| Note 2 | dBm/15 kHz | 1, 2 | -98 |  |  |  |  |  |
|  | dB | 1, 2 | 14 | 14 | 14 | -4 | -infinity | 12 |
|  | dB | 1, 2 | 14 | 14 | 14 | -4 | -infinity | 12 |
| RSRP Note 3 | dBm/15 KHz | 1, 2 | -84 | -84 | -84 | -102 | -infinity | -86 |
| TreselectionEUTRAN | s | 1, 2 | 0 |  |  | 0 |  |  |
| Snonintrasearch | dB | 1, 2 | 50 |  |  | Not sent |  |  |
| Threshx, high | dB | 1, 2 | 48 |  |  | 48 |  |  |
| Threshserving, low | dB | 1, 2 | 44 |  |  | 44 |  |  |
| Threshx, low | dB | 1, 2 | 50 |  |  | 50 |  |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |  |  |
| Antenna Configuration |  | 1, 2 | 1x1 |  |  | 1x1 |  |  |
| Timing offset to Cell 1 | ms | 1, 2 | - |  |  | 3 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

##### A.14.1.1.6.2	Test Requirements

The cell reselection delay to higher priority is defined as the time from the beginning of time period T3, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to higher priority shall be less than 68 s.

The cell reselection delay to lower priority is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on cell 1.

The cell re-selection delay to lower priority shall be less than 8 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to higher priority cell can be expressed as: Thigher_priority_search + Tevaluate, E-UTRAN_Inter_NC + TSI-EUTRA-M1-NC , and to lower priority cell can be expressed as: Tevaluate, E-UTRAN_Inter_NC + TSI-EUTRA-M1-NC,

Where:

Thigher_priority_search	See clause 4.7A.2.1.3

Tevaluate, E-UTRAN_Inter_NC	See Table 4.7A.2.1.3-1 in clause 4.7A.2.1.3

TSI-EUTRA-M1-NC	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case provided that SIB31 and SIB33 are scheduled with 80 ms period.

This gives a total of 67.68 s for higher priority cell search and 7.68 s for lower priority cell search, allow 68 s for higher priority cell and 8 s for lower priority cell in the test case.

#### A.14.1.1.7	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage, time-based triggering

##### A.14.1.1.7.1	Test Purpose and Environment

This test is to verify the requirement for the FDD-FDD intra frequency cell reselection requirements for category M1 UE in normal coverage for satellite access specified in clause 4.7A.2.1.2.

The supported test configurations are provided in Table A.14.1.1.7.1-1.

Table A.14.1.1.7.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, FDD-FDD duplex mode |
| 2 | NGSO, FDD-FDD duplex mode |

The test scenario comprises of 1 E-UTRA FDD carrier and 2 cells as given in tables A.14.1.1.7.1-2 and A.14.1.1.7.1-3. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Only Cell 1 is already identified by the UE prior to the start of the test, i.e. Cell 2 is not identified by the UE prior to the start of the test. Cell 1 and Cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing Cell 2.

T-Service broadcasted in SIB3 of Cell 1 is set to the time point that is 36s after start of T2.

Table A.14.1.1.7.1-2: General test parameters for FDD intra frequency cell reselection test case for Cat-M1 UE in normal coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell1 |  |
|  | Neighbour cells |  | Cell2 |  |
| T2 end condition | Active cell |  | Cell2 |  |
|  | Neighbour cells |  | Cell1 |  |
| E-UTRA RF Channel Number |  |  | 1 | Only one FDD carrier frequency is used. |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| PRACH Configuration |  |  | PRACH_2CE | Refer to A.3.16 |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| T1 |  | s | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 40 | T2 need to be defined so that cell re-selection reaction time is taken into account. |

Table A.14.1.1.7.1-3: Cell specific test parameters for FDD intra frequency cell reselection test case for Cat-M1 UE in normal coverage

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | T1 | T2 | T1 | T2 |
| Satellite information |  |  | 1 |  | SSC.1 |  | SSC.1 | NSC.1 | NSC.1 |  |
|  |  |  | 2 |  | SSC.2 |  | SSC.2 | NSC.1 | NSC.2 |  |
| E-UTRA RF Channel Number |  | 1, 2 | 1 |  |  |  |
| BWchannel | MHz | 1, 2 | 1.4 |  |  |  |
| OCNG Patterns |  | 1, 2 | OP.7 FDD |  | OP.7 FDD |  |
| PBCH_RA | dB | 1, 2 | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |
| Qrxlevmin |  |  |  |  |  |  |  |  |  |  |  |  | dBm | 1, 2 | -140 |  |  |  |  | -140 | -140 | -140 |
| Pcompensation |  |  |  |  |  |  |  |  |  |  |  |  | dB | 1, 2 | 0 |  |  |  |  | 0 | 0 | 0 |
| Qhysts |  |  |  |  |  |  |  |  |  |  |  |  | dB | 1, 2 | 0 |  |  |  |  | 0 | 0 | 0 |
| Qoffsets, n |  |  |  |  |  |  |  |  |  |  |  |  | dB | 1, 2 | 0 |  |  |  |  | 0 | 0 | 0 |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2 | RSRP |  | RSRP |  |
| Note2 | dBm/15 kHz | 1, 2 | -98 |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | dB | 1, 2 | 16 |  |  |  |  | 12 | -infinity | 16 |
|  |  |  |  |  |  |  |  |  |  |  |  |  | dB | 1, 2 | 16 |  |  |  |  | -4.11 | -infinity | 3.73 |
| RSRP Note3 |  |  |  |  |  |  |  |  |  |  |  |  | dBm/15 kHz | 1, 2 | -82 |  |  |  |  | -86 | -infinity | -82 |
| Treselection |  |  |  |  |  |  |  |  |  |  |  |  | s | 1, 2 | 0 |  |  |  |  | 0 | 0 | 0 |
| Sintrasearch | dB | 1, 2 | Not sent |  | Not sent |  |
| Propagation Condition |  | 1, 2 | AWGN |  | AWGN |  |
| Antenna Configuration |  | 1, 2 | 1x1 |  | 1x1 |  |
| Timing offset to Cell 1 | ms | 1, 2 | - |  | 3 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.14.1.1.7.2	Test Requirements

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to a newly detectable cell shall be less than 34 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a newly detectable cell can be expressed as: Tdetect,EUTRAN_Intra_NC + TSI-EUTRA-M1-NC.

Where:

Tdetect,EUTRAN_Intra_NC	See Table 4.7A.2.1.2-1 in clause 4.7A.2.1.2

TSI-EUTRA-M1-NC	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case provided that SIB3 is scheduled with 20ms period, SIB31 and SIB33 are scheduled with 80 ms period.

This gives a total of 33.28 s, allow 34 s for the cell re-selection delay to a newly detectable cell in the test case.

#### A.14.1.1.8 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage, time-based triggering

##### A.14.1.1.8.1	Test Purpose and Environment

This test is to verify the requirement for the HD-FDD intra frequency cell reselection requirements for Cat-M1 UE in enhanced coverage for satellite access specified in clause 4.7A.2.2.2.

The supported test configurations are provided in Table A.14.1.1.8.1-1.

Table A.14.1.1.8.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |

The test scenario comprises of 1 E-UTRA carrier and 2 cells as given in tables A.14.1.1.8.1-2 and A.14.1.1.8.1-3. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Only Cell 1 is already identified by the UE prior to the start of the test, i.e. Cell 2 is not identified by the UE prior to the start of the test. Cell 1 and Cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing Cell 2.

T-Service broadcasted in SIB3 of Cell 1 is set to the time point that is 36s after start of T2.

TableA.14.1.1.8.1-2: General test parameters for HD-FDD intra frequency cell reselection test case for Cat-M1 UE in enhanced coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell1 |  |
|  | Neighbour cells |  | Cell2 |  |
| T2 end condition | Active cell |  | Cell2 |  |
|  | Neighbour cells |  | Cell1 |  |
| E-UTRA RF Channel Number |  |  | 1 | Only one carrier frequency is used. |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| PRACH Parameters |  |  | PRACH_4CE | Refer to A.3.16 |
| DRX cycle length |  | s | 0.64 | The value shall be used for all cells in the test. |
| T1 |  | s | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | ≤340 | T2 need to be defined so that cell re-selection reaction time is taken into account. |

Table A.14.1.1.8.1-3: Cell specific test parameters for HD-FDD intra frequency cell reselection test case for Cat-M1 UE in enhanced coverage

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Satellite information |  | 1 | SSC.1 | SSC.1 | NSC.1 | NSC.1 |
|  |  | 2 | SSC.2 | SSC.2 | NSC.1 | NSC.2 |
| E-UTRA RF Channel Number |  | 1, 2 | 1 |  |  |  |
| BWchannel | MHz | 1, 2 | 1.4 |  |  |  |
| OCNG Patterns |  | 1, 2 | OP.7 FDD |  | OP.7 FDD |  |
| PBCH_RA | dB | 1, 2 | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |
| Qrxlevmin | dBm | 1, 2 | -140 | -140 | -140 | -140 |
| Pcompensation | dB | 1, 2 | 0 | 0 | 0 | 0 |
| Qhysts | dB | 1, 2 | 0 | 0 | 0 | 0 |
| Qoffsets, n | dB | 1, 2 | 0 | 0 | 0 | 0 |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2 | RSRP |  | RSRP |  |
| Note2 | dBm/15 kHz | 1, 2 | -98 |  |  |  |
|  | dB | 1, 2 | -7 | -12 | -infinity | -7 |
|  | dB | 1, 2 | -7 | -12.79 | -infinity | -7.27 |
| RSRP Note3 | dBm/15 kHz | 1, 2 | -105 | -110 | -infinity | -105 |
| Treselection | s | 1, 2 | 0 | 0 | 0 | 0 |
| Sintrasearch | dB | 1, 2 | Not sent |  | Not sent |  |
| Propagation Condition |  | 1, 2 | AWGN |  | AWGN |  |
| Antenna Configuration |  | 1, 2 | 1x1 |  | 1x1 |  |
| Timing offset to Cell 1 | ms | 1, 2 | - |  | 3 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.14.1.1.8.2	Test Requirements

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to a newly detectable cell shall be less than 338 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a newly detectable cell can be expressed as: Tdetect,EUTRAN_Intra_EC + TSI-EUTRA-M1-EC.

Where:

Tdetect,EUTRAN_Intra_EC	See Table 4.7A.2.2.2-1 in clause 4.7A.2.2.2.

TSI-EUTRA-M1-EC	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 6400 ms is assumed in this test case provided that SIB3 is scheduled with 20ms period, SIB31 and SIB33 are scheduled with 80 ms period.

This gives a total of 337.36 s, allow 338 s for the cell re-selection delay to a newly detectable cell in the test case.

#### A.14.1.1.9	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage, location-based triggering

##### A.14.1.1.9.1	Test Purpose and Environment

This test is to verify the requirement for the FDD-FDD inter frequency cell reselection requirements for category M1 UE in enhanced coverage for satellite access specified in clause 4.7A.2.2.3.

The supported test configurations are provided in Table A.14.1.1.9.1-1.

Table A.14.1.1.9.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, FDD-FDD duplex mode |
| 2 | NGSO, FDD-FDD duplex mode |

The test scenario comprises of 2 E-UTRA FDD cells on 2 different carriers as given in tables A.14.1.1.9.1-2 and A.14.1.1.9.1-3. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Only Cell 1 is already identified by the UE prior to the start of the test, i.e. Cell 2 is not identified by the UE prior to the start of the test. Cell 1 and Cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing Cell 2.

At 4s after the start of T2, the UE location is changed such that the distance to the referencelocation broadcasted in SIB31 of Cell 1 is exceeded by the configured value in distanceThresh plus 50m.

Table A.14.1.1.9.1-2: General test parameters for FD-FDD inter frequency cell reselection test case for Cat-M1 UE in enhanced coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell1 |  |
|  | Neighbour cells |  | Cell2 |  |
| T2 end condition | Active cell |  | Cell2 |  |
|  | Neighbour cells |  | Cell1 |  |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two FDD carrier frequencies are used. |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| PRACH Parameters |  |  | PRACH_4CE | Refer to A.3.16 |
| DRX cycle length |  | s | 0.64 | The value shall be used for all cells in the test. |
| T1 |  | s | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | ≤340 | T2 need to be defined so that cell re-selection reaction time is taken into account. |

Table A.14.1.1.9.1-3: Cell specific test parameters for FD-FDD inter frequency cell reselection test case for Cat-M1 UE in enhanced coverage

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Satellite information |  | 1 | SSC.1 | SSC.1 | NSC.1 | NSC.1 |
|  |  | 2 | SSC.2 | SSC.2 | NSC.1 | NSC.2 |
| E-UTRA RF Channel Number |  | 1, 2 | 1 |  | 2 |  |
| BWchannel | MHz | 1, 2 | 1.4 |  |  |  |
| OCNG Patterns |  | 1, 2 | OP.7 FDD |  | OP.7 FDD |  |
| PBCH_RA | dB | 1, 2 | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |
| Qrxlevmin | dBm | 1, 2 | -140 | -140 | -140 | -140 |
| Note2 | dBm/15 kHz | 1, 2 | -98 |  |  |  |
|  | dB | 1, 2 | -5 | -12 | -infinity | -5 |
|  | dB | 1, 2 | -5 | -12 | -infinity | -5 |
| RSRP Note3 | dBm/15 kHz | 1, 2 | -103 | -110 | -infinity | -103 |
| TreselectionEUTRAN | s | 1, 2 | 0 | 0 | 0 | 0 |
| Snonintrasearch | dB | 1, 2 | 50 |  | Not sent |  |
| Propagation Condition |  | 1, 2 | AWGN |  | AWGN |  |
| Antenna Configuration |  | 1, 2 | 1x1 |  | 1x1 |  |
| Timing offset to Cell 1 | ms | 1, 2 | - |  | 3 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.14.1.1.9.2	Test Requirements

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to a newly detectable cell shall be less than 337 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a newly detectable cell can be expressed as: Tdetect,EUTRAN_Inter_EC + TSI-EUTRA-M1-EC.

Where:

Tdetect,EUTRAN_Inter_EC	See Table 4.7A.2.2.3-1 in clause 4.7A.2.2.3

TSI-EUTRA-M1-EC	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 6400 ms is assumed in this test case provided that SIB31 and SIB33 are scheduled with 80 ms period.

This gives a total of 336.64 s, allow 337 s for the cell re-selection delay to a newly detectable cell in the test case.

#### A.14.1.1.10	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage, location-based triggering

##### A.14.1.1.10.1	Test Purpose and Environment

This test is to verify the requirement for the HD-FDD inter frequency cell reselection requirements for category M1 UE in normal coverage for satellite access specified in clause 4.7A.2.1.3.

The supported test configurations are provided in Table A.14.1.1.10.1-1.

Table A.14.1.1.10.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, FDD-FDD duplex mode |
| 2 | NGSO, FDD-FDD duplex mode |

The test scenario comprises of 2 E-UTRA carriers and 2 cells as given in tables Table A.14.1.1.10.1-2 and A.14.1.1.10.1-3. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas and cell 2 is of lower priority than cell 1. Furthermore, UE has not registered with network for the tracking area containing cell 2.

At 4s after the start of T2, the UE location is changed such that the distance to the referencelocation broadcasted in SIB31 of Cell 2 is exceeded by the configured value in distanceThresh plus 50m.

Table A.14.1.1.10.1-2: General test parameters for HD-FDD inter frequency cell reselection test case for Cat-M1 UE in normal coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell1 |  |
| T2 end condition | Active cell |  | Cell2 | UE shall perform reselection to cell 2 during T2 |
|  | Neighbour cell |  | Cell1 |  |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two FDD carrier frequencies are used. |
| PRACH configuration |  |  | PRACH_2CE | See table in A.3.16 |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| T1 |  | s | >7 |  |
| T2 |  | s | 15 | T2 need to be defined so that cell re-selection reaction time is taken into account. |

Table A.14.1.1.10.1-3: Cell specific test parameters for HD-FDD inter frequency cell reselection test case for Cat-M1 UE in normal coverage

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Satellite information |  | 1 | SSC.1 | SSC.1 | NSC.1 | NSC.1 |
|  |  | 2 | SSC.2 | SSC.2 | NSC.1 | NSC.2 |
| E-UTRA RF Channel Number |  | 1, 2 | 1 |  | 2 |  |
| BWchannel | MHz | 1, 2 | 1.4 |  |  |  |
| OCNG Patterns |  | 1, 2 | OP.7 FDD |  | OP.7 FDD |  |
| PBCH_RA | dB | 1, 2 | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |
| Qrxlevmin | dBm | 1, 2 | -140 | -140 | -140 | -140 |
| Note2 | dBm/15 kHz | 1, 2 | -98 |  |  |  |
|  | dB | 1, 2 | 14 | -4 | -4 | 14 |
|  | dB | 1, 2 | 14 | -4 | -4 | 14 |
| RSRP Note3 | dBm/15 kHz | 1, 2 | -84 | -102 | -102 | -84 |
| TreselectionEUTRAN | s | 1, 2 | 0 | 0 | 0 | 0 |
| Snonintrasearch | dB | 1, 2 | 50 |  | Not sent |  |
| Threshx, high | dB | 1, 2 | 48 |  | 48 |  |
| Threshserving, low | dB | 1, 2 | 44 |  | 44 |  |
| Threshx, low | dB |  | 50 |  | 50 |  |
| Propagation Condition |  | 1, 2 | AWGN |  | AWGN |  |
| Antenna Configuration |  | 1, 2 | 1x1 |  | 1x1 |  |
| Timing offset to Cell 1 | ms | 1, 2 | - |  | 3 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.14.1.1.10.2	Test Requirements

The cell reselection delay to lower priority is defined as the time from the beginning of time period T2, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to lower priority shall be less than 8 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to lower priority cell can be expressed as: Tevaluate, E-UTRAN_Inter_NC + TSI-EUTRA-M1-NC.

Where:

Tevaluate, E-UTRAN_Inter_NC	See Table 4.7A.2.1.3-1 in clause 4.7A.2.1.3

TSI-EUTRA-M1-NC	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case provided that SIB31 and SIB33 are scheduled with 80 ms period.

This gives a total of 7.68 s for lower priority cell search, allow 8 s for lower priority cell in the test case.

## <<< NEXT CHANGE >>>

#### A.14.2.1.5	E-UTRAN FDD Intra frequency handover for Cat-M1 UEs in CEModeA

##### A.14.2.1.5.1	Test Purpose and Environment

This test is to verify the requirement for the FDD intra frequency handover requirements.

The test configurations are given in Table A.14.2.1.5.1-1. The test scenario comprises one E-UTRA FDD carrier and two cells as given in tables A.14.2.1.5.1-2 and A.14.2.1.5.1-3. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE shall have had the opportunity to acquire satellite assistance information for Cell 2, provided by Cell 1 in SystemInformationBlockType33.

Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report. E-UTRAN shall send a RRC message implying handover to Cell 2 during period T2, after the UE has reported Event A3. The field sameSFN-Indication is not included in the handover command. T3 is defined as the end of the last TTI containing the RRC message implying handover.

During the test, UE is configured with measurement gap for cell search.

Table A.14.2.1.5.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, FD-FDD duplex mode |
| 2 | NGSO, FD-FDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

Table A.14.2.1.5.1-2: General test parameters for E-UTRAN FDD intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
|  | Neighbouring cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  |  | OFF |
| CP length |  |  | Normal |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| PRACH configuration |  |  | PRACH_4CE | As specified in A.3.16 |
| PRACH initial CE level |  |  | 0 | Specified in the handover message |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |
| Gap pattern ID |  |  | 1 |  |

Table A.14.2.1.5.1-3: Cell specific test parameters for E-UTRAN FDD intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition test case

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 1 |  |  |
| Satellite Information (Configuration 1)Note 1 |  | SSC.1 |  |  | NSC.1 |  |  |
| Satellite Information (Configuration 2)Note 1 |  | SSC.2 |  |  | NSC.2 |  |  |
| BWchannel | MHz | 1.4 |  |  |  |  |  |
| PDSCH Reference Channel in clause A.3.1.4.1 |  | R.48 FDD | R.48 FDD | - | - | - | R.48 FDD |
| MPDCCH Reference Channel in clause A.3.1.3.1 |  | R.46 FDD |  |  | R.46 FDD |  |  |
| OCNG Patterns in clause A.3.2.1 |  | OP.7 FDD | OP.7 FDD | OP.7 FDD | OP.7 FDD | OP.7 FDD | OP.7 FDD |
| PBCH_RA | dB | -3 |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 2 | dB |  |  |  |  |  |  |
| OCNG_RBNote 2 | dB |  |  |  |  |  |  |
| Note 3 | dBm/15 KHz | -98 |  |  |  |  |  |
|  | dB | 8 | 8 | 8 | -Infinity | 12 | 12 |
| Note 4 | dB | 8 | -4.27 | -4.27 | -Infinity | 3.36 | 3.36 |
| RSRP Note 4 | dBm/15 KHz | -90 | -90 | -90 | -Infinity | -86 | -86 |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x1 |  |  | 1x1 |  |  |
| Timing offset to Cell 1 Synchronous cells | us | - |  |  | Based on Satellite Assistance information |  |  |
| Note 1: Satellite information is determined according to the testing principles for NTN determined in clause B.3.8. If satellite movement is applicable, it should be considered for the duration of the test case. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot and RSRP level has been derived from other parameters for information purpose. They are not settable parameters themselves. |  |  |  |  |  |  |  |

##### A.14.2.1.5.2	Test Requirements

The UE shall finish the transmission of all repetitions of the PRACH to Cell 2 less than 170 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 15 ms and is specified in clause 11.2 in TS 36.331 [2].

Tinterrupt = 120+35 ms in the test; Tinterrupt is defined in clause 5.5A.2.1.2.

This gives a total of 170 ms.

#### 14.2.1.6	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA

##### A.14.2.1.6.1	Test Purpose and Environment

This test is to verify the requirement for the HD-FDD intra frequency handover requirements.

The test configurations are given in Table A.14.2.1.6.1-1. The test scenario comprises one E-UTRA FDD carrier and two cells as given in tables A.14.2.1.6.1-2 and A.14.2.1.6.1-3. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE shall have had the opportunity to acquire satellite assistance information for Cell 2, provided by Cell 1 in SystemInformationBlockType33.

Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report. E-UTRAN shall send a RRC message implying handover to Cell 2 during period T2, after the UE has reported Event A3. The field sameSFN-Indication is not included in the handover command. T3 is defined as the end of the last TTI containing the RRC message implying handover.

During the test, UE is configured with measurement gap for cell search.

Table A.14.2.1.6.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

Table A.14.2.1.6.1-2: General test parameters for E-UTRAN HD-FDD intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
|  | Neighbouring cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  |  | OFF |
| CP length |  |  | Normal |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| PRACH configuration |  |  | PRACH_4CE | As specified in A.3.16 |
| PRACH initial CE level |  |  | 0 | Specified in the handover message |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |
| Gap pattern ID |  |  | 1 |  |

Table A.14.2.1.6.1-3: Cell specific test parameters for E-UTRAN HD-FDD intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition test case

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 1 |  |  |
| Satellite Information (Configuration 1)Note 1 |  | SSC.1 |  |  | NSC.1 |  |  |
| Satellite Information (Configuration 2)Note 1 |  | SSC.2 |  |  | NSC.2 |  |  |
| BWchannel | MHz | 1.4 |  |  |  |  |  |
| PDSCH Reference Channel in clause A.3.1.4.1 |  | R.49 HD-FDD | R.49 HD-FDD | - | - | - | R.49 HD-FDD |
| MPDCCH Reference Channel in clause A.3.1.3.1 |  | R.47 HD-FDD |  |  | R.47 HD-FDD |  |  |
| OCNG Patterns in clause A.3.2.1 |  | OP.7 FDD | OP.7 FDD | OP.7 FDD | OP.7 FDD | OP.7 FDD | OP.7 FDD |
| PBCH_RA | dB | -3 |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 2 | dB |  |  |  |  |  |  |
| OCNG_RBNote 2 | dB |  |  |  |  |  |  |
| Note 3 | dBm/15 KHz | -98 |  |  |  |  |  |
|  | dB | 8 | 8 | 8 | -Infinity | 12 | 12 |
| Note 4 | dB | 8 | -4.27 | -4.27 | -Infinity | 3.36 | 3.36 |
| RSRP Note 4 | dBm/15 KHz | -90 | -90 | -90 | -Infinity | -86 | -86 |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x1 |  |  | 1x1 |  |  |
| Timing offset to Cell 1 Synchronous cells | us | - |  |  | Based on Satellite Assistance information |  |  |
| Note 1: Satellite information is determined according to the testing principles for NTN determined in clause B.3.8. If satellite movement is applicable, it should be considered for the duration of the test case. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot and RSRP level has been derived from other parameters for information purpose. They are not settable parameters themselves. |  |  |  |  |  |  |  |

##### A.14.2.1.6.2	Test Requirements

The UE shall finish the transmission of all repetitions of the PRACH to Cell 2 less than 170 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 15 ms and is specified in clause 11.2 in TS 36.331 [2].

Tinterrupt = 120+35 ms in the test; Tinterrupt is defined in clause 5.5A.2.1.2.

This gives a total of 170 ms.

#### A.14.2.1.7	E-UTRAN FD-FDD Inter frequency handover for Cat-M1 UEs in CEModeA

##### A.14.2.1.7.1	Test Purpose and Environment

This test is to verify the requirement for the FDD inter frequency handover requirements.

The test configurations are given in Table A.14.2.1.7.1-1. The test scenario comprises of two E-UTRA FDD carrier and one cell in each carrier as given in tables A.14.2.1.7.1-2 and A.14.2.1.7.1-3. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE shall have had the opportunity to acquire satellite assistance information for Cell 2, provided by Cell 1 in SystemInformationBlockType33.

Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report. E-UTRAN shall send a RRC message implying handover to Cell 2 during period T2, after the UE has reported Event A3. . The field sameSFN-Indication is not included in the handover command. T3 is defined as the end of the last TTI containing the RRC message implying handover.

During the test, UE is configured with measurement gap to enable inter-frequency measurement.

Table A.14.2.1.7.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, FD-FDD duplex mode |
| 2 | NGSO, FD-FDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

Table A.14.2.1.7.1-2: General test parameters for E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
|  | Neighbouring cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  |  | OFF |
| CP length |  |  | Normal |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| PRACH configuration |  |  | PRACH_4CE | As specified in A.3.16 |
| PRACH initial CE level |  |  | 0 | Specified in the handover message |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |
| Gap pattern ID |  |  | 1 |  |

Table A.14.2.1.7.1-3: Cell specific test parameters for E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition test case

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 1 |  |  |
| Satellite Information (Configuration 1)Note 1 |  | SSC.1 |  |  | NSC.3 |  |  |
| Satellite Information (Configuration 2)Note 1 |  | SSC.2 |  |  | NSC.4 |  |  |
| BWchannel | MHz | 1.4 |  |  |  |  |  |
| PDSCH Reference Channel in clause A.3.1.4.1 |  | R.48 FDD | R.48 FDD | - | - | - | R.48 FDD |
| MPDCCH Reference Channel in clause A.3.1.3.1 |  | R.46 FDD |  |  | R.46 FDD |  |  |
| OCNG Patterns in clause A.3.2.1 |  | OP.7 FDD | OP.7 FDD | OP.7 FDD | OP.7 FDD | OP.7 FDD | OP.7 FDD |
| PBCH_RA | dB | -3 |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 2 | dB |  |  |  |  |  |  |
| OCNG_RBNote 2 | dB |  |  |  |  |  |  |
| Note 3 | dBm/15 KHz | -98 |  |  |  |  |  |
|  | dB | 8 | 8 | 8 | -Infinity | 12 | 12 |
| Note 4 | dB | 8 | -4.27 | -4.27 | -Infinity | 3.36 | 3.36 |
| RSRP Note 4 | dBm/15 KHz | -90 | -90 | -90 | -Infinity | -86 | -86 |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x1 |  |  | 1x1 |  |  |
| Timing offset to Cell 1 Synchronous cells | us | - |  |  | Based on Satellite Assistance information |  |  |
| Note 1: Satellite information is determined according to the testing principles for NTN determined in clause B.3.8. If satellite movement is applicable, it should be considered for the duration of the test case. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot and RSRP level has been derived from other parameters for information purpose. They are not settable parameters themselves. |  |  |  |  |  |  |  |

##### A.14.2.1.7.2	Test Requirements

The UE shall finish the transmission of all repetitions of the PRACH to Cell 2 less than 170 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 15 ms and is specified in clause 11.2 in TS 36.331 [2].

Tinterrupt = 120+35 ms in the test; Tinterrupt is defined in clause 5.5A.2.1.2.

This gives a total of 170 ms.

#### A.14.2.1.8	E-UTRAN HD-FDD Inter frequency handover for Cat-M1 UEs in CEModeA

##### A.14.2.1.8.1	Test Purpose and Environment

This test is to verify the requirement for the HD-FDD inter frequency handover requirements. The test configurations are given in Table A.14.2.1.8.1-1.

The test scenario comprises of two E-UTRA FDD carrier and one cell in each carrier as given in tables A.14.2.1.8.1-2 and A.14.2.1.8.1-3. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE shall have had the opportunity to acquire satellite assistance information for Cell 2, provided by Cell 1 in SystemInformationBlockType33.

Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report. E-UTRAN shall send a RRC message implying handover to Cell 2 during period T2, after the UE has reported Event A3. The field sameSFN-Indication is not included in the handover command. T3 is defined as the end of the last TTI containing the RRC message implying handover.

During the test, UE is configured with measurement gap to enable inter-frequency measurement.

Table A.14.2.1.8.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

Table A.14.2.1.8.1-2: General test parameters for E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
|  | Neighbouring cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  |  | OFF |
| CP length |  |  | Normal |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| PRACH configuration |  |  | PRACH_4CE | As specified in A.3.16 |
| PRACH initial CE level |  |  | 0 | Specified in the handover message |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |
| Gap pattern ID |  |  | 1 |  |

Table A.14.2.1.8.1-3: Cell specific test parameters for E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition test case

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 1 |  |  |
| Satellite Information (Configuration 1)Note 1 |  | SSC.1 |  |  | NSC.3 |  |  |
| Satellite Information (Configuration 2)Note 1 |  | SSC.2 |  |  | NSC.4 |  |  |
| BWchannel | MHz | 1.4 |  |  |  |  |  |
| PDSCH Reference Channel in clause A.3.1.4.1 |  | R.49 HD-FDD | R.49 HD-FDD | - | - | - | R.49 HD-FDD |
| MPDCCH Reference Channel in clause A.3.1.3.1 |  | R.47 HD-FDD |  |  | R.47 HD-FDD |  |  |
| OCNG Patterns in clause A.3.2.1 |  | OP.7 FDD | OP.7 FDD | OP.7 FDD | OP.7 FDD | OP.7 FDD | OP.7 FDD |
| PBCH_RA | dB | -3 |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 2 | dB |  |  |  |  |  |  |
| OCNG_RBNote 2 | dB |  |  |  |  |  |  |
| Note 3 | dBm/15 KHz | -98 |  |  |  |  |  |
|  | dB | 8 | 8 | 8 | -Infinity | 12 | 12 |
| Note 4 | dB | 8 | -4.27 | -4.27 | -Infinity | 3.36 | 3.36 |
| RSRP Note 4 | dBm/15 KHz | -90 | -90 | -90 | -Infinity | -86 | -86 |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x1 |  |  | 1x1 |  |  |
| Timing offset to Cell 1 Synchronous cells | us | - |  |  | Based on Satellite Assistance information |  |  |
| Note 1: Satellite information is determined according to the testing principles for NTN determined in clause B.3.8. If satellite movement is applicable, it should be considered for the duration of the test case. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot and RSRP level has been derived from other parameters for information purpose. They are not settable parameters themselves. |  |  |  |  |  |  |  |

##### A.14.2.1.8.2	Test Requirements

The UE shall finish the transmission of all repetitions of the PRACH to Cell 2 less than 170 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 15 ms and is specified in clause 11.2 in TS 36.331 [2].

Tinterrupt = 120+35 ms in the test; Tinterrupt is defined in clause 5.5A.2.1.2.

This gives a total of 170 ms.

#### A.14.2.1.9	E-UTRAN FDD Inter frequency handover for Cat-M1 UEs in CEModeB

##### A.14.2.1.9.1	Test Purpose and Environment

This test is to verify the requirement for the FDD inter frequency handover requirements.

The test configurations are given in Table A.14.2.1.9.1-1. The test scenario comprises of two E-UTRA FDD carrier and one cell in each carrier as given in tables A.14.2.1.9.1-2 and A.14.2.1.9.1-3. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE shall have had the opportunity to acquire satellite assistance information for Cell 2, provided by Cell 1 in SystemInformationBlockType33.

Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report. E-UTRAN shall send a RRC message implying handover to Cell 2 during period T2, after the UE has reported Event A3. . The field sameSFN-Indication is not included in the handover command. T3 is defined as the end of the last TTI containing the RRC message implying handover.

During the test, UE is configured with measurement gap to enable inter-frequency measurement.

Table A.14.2.1.9.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, FD-FDD duplex mode |
| 2 | NGSO, FD-FDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

Table A.14.2.1.9.1-2: General test parameters for E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
|  | Neighbouring cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  |  | OFF |
| CP length |  |  | Normal |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| PRACH configuration |  |  | PRACH_4CE | As specified in A.3.16 |
| PRACH initial CE level |  |  | 0 | Specified in the handover message |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |
| Gap pattern ID |  |  | 1 |  |

Table A.14.2.1.9.1-3: Cell specific test parameters for E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition test case

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 1 |  |  |
| Satellite Information (Configuration 1)Note 1 |  | SSC.1 |  |  | NSC.1 |  |  |
| Satellite Information (Configuration 2)Note 1 |  | SSC.2 |  |  | NSC.2 |  |  |
| BWchannel | MHz | 1.4 |  |  |  |  |  |
| PDSCH Reference Channel in clause A.3.1.4.1 |  | R.52 FDD | R.52 FDD | - | - | - | R.52 FDD |
| MPDCCH Reference Channel in clause A.3.1.3.1 |  | R.50 FDD |  |  | R.50 FDD |  |  |
| OCNG Patterns in clause A.3.2.1 |  | OP.7 FDD | OP.7 FDD | OP.7FDD | OP.7 FDD | OP7 FDD | OP.7 FDD |
| PBCH_RA | dB | -3 |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 2 | dB |  |  |  |  |  |  |
| OCNG_RBNote 2 | dB |  |  |  |  |  |  |
| Note 3 | dBm/15 KHz | -98 |  |  |  |  |  |
|  | dB | -12 | -12 | -12 | -Infinity | -4 | -4 |
| Note 4 | dB | -12 | -12 | -12 | -Infinity | -4 | -4 |
| RSRP Note 4 | dBm/15 KHz | -110 | -110 | -110 | -Infinity | -102 | -102 |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x1 |  |  | 1x1 |  |  |
| Timing offset to Cell 1 Synchronous cells | us | - |  |  | Based on Satellite Assistance information |  |  |
| Note 1: Satellite information is determined according to the testing principles for NTN determined in clause B.3.8. If satellite movement is applicable, it should be considered for the duration of the test case. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot and RSRP level has been derived from other parameters for information purpose. They are not settable parameters themselves. |  |  |  |  |  |  |  |

##### A.14.2.1.9.2	Test Requirements

The UE shall finish the transmission of all repetitions of the PRACH to Cell 2 less than 170 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 15 ms and is specified in clause 11.2 in TS 36.331 [2].

Tinterrupt = 2560+35 ms in the test; Tinterrupt is defined in clause 5.5A.2.1.2.

This gives a total of 2610 ms.

#### A.14.2.1.10	E-UTRAN HD-FDD Inter frequency handover for Cat-M1 UEs in CEModeB

##### A.14.2.1.10.1	Test Purpose and Environment

This test is to verify the requirement for the HD-FDD inter frequency handover requirements. The test configurations are given in Table A.14.2.1.10.1-1.

The test scenario comprises of two E-UTRA FDD carrier and one cell in each carrier as given in tables A.14.2.1.10.1-2 and A.14.2.1.10.1-3. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE shall have had the opportunity to acquire satellite assistance information for Cell 2, provided by Cell 1 in SystemInformationBlockType33.

Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report. E-UTRAN shall send a RRC message implying handover to Cell 2 during period T2, after the UE has reported Event A3. The field sameSFN-Indication is not included in the handover command. T3 is defined as the end of the last TTI containing the RRC message implying handover.

During the test, UE is configured with measurement gap to enable inter-frequency measurement.

Table A.14.2.1.10.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

Table A.14.2.1.10.1-2: General test parameters for E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
|  | Neighbouring cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  |  | OFF |
| CP length |  |  | Normal |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| PRACH configuration |  |  | PRACH_4CE | As specified in A.3.16 |
| PRACH initial CE level |  |  | 0 | Specified in the handover message |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |
| Gap pattern ID |  |  | 1 |  |

Table A.14.2.1.10.1-3: Cell specific test parameters for E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition test case

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 1 |  |  |
| Satellite Information (Configuration 1)Note 1 |  | SSC.1 |  |  | NSC.1 |  |  |
| Satellite Information (Configuration 2)Note 1 |  | SSC.2 |  |  | NSC.2 |  |  |
| BWchannel | MHz | 1.4 |  |  |  |  |  |
| PDSCH Reference Channel in clause A.3.1.4.1 |  | R.53 HD-FDD | R.53 HD-FDD | - | - | - | R.53 HD-FDD |
| MPDCCH Reference Channel in clause A.3.1.3.1 |  | R.51 HD-FDD |  |  | R.51 HD-FDD |  |  |
| OCNG Patterns in clause A.3.2.1 |  | OP.7 FDD | OP.7 FDD | OP.7 FDD | OP.7 FDD | OP.7 FDD | OP.7 FDD |
| PBCH_RA | dB | -3 |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 2 | dB |  |  |  |  |  |  |
| OCNG_RBNote 2 | dB |  |  |  |  |  |  |
| Note 3 | dBm/15 KHz | -98 |  |  |  |  |  |
|  | dB | -12 | -12 | -12 | -Infinity | -4 | -4 |
| Note 4 | dB | -12 | -12 | -12 | -Infinity | -4 | -4 |
| RSRP Note 4 | dBm/15 KHz | -110 | -110 | -110 | -Infinity | -102 | -102 |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x1 |  |  | 1x1 |  |  |
| Timing offset to Cell 1 Synchronous cells | us | - |  |  | Based on Satellite Assistance information |  |  |
| Note 1: Satellite information is determined according to the testing principles for NTN determined in clause B.3.8. If satellite movement is applicable, it should be considered for the duration of the test case. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot and RSRP level has been derived from other parameters for information purpose. They are not settable parameters themselves. |  |  |  |  |  |  |  |

##### A.14.2.1.10.2	Test Requirements

The UE shall finish the transmission of all repetitions of the PRACH to Cell 2 less than 170 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 15 ms and is specified in clause 11.2 in TS 36.331 [2].

Tinterrupt = 2560+35 ms in the test; Tinterrupt is defined in clause 5.5A.2.1.2.

This gives a total of 2610 ms.

#### A.14.2.1.11	E-UTRAN FDD-FDD Inter frequency conditional handover for Cat-M1 UEs in CEModeA

##### A.14.2.1.11.1	Test Purpose and Environment

This test is to verify the requirement for the FDD inter frequency conditional handover requirements.

The test configurations are given in Table A.14.2.1.11.1-1. The test scenario comprises of two E-UTRA FDD carrier and one cell in each carrier as given in tables A.14.2.1.11.1-2 and A.14.2.1.11.1-3 The test consists of two successive time periods, with time durations of T1 and T2 respectively. At the start of time duration T1, the UE shall have had the opportunity to acquire satellite assistance information for Cell 2, provided by Cell 1 in SystemInformationBlockType33.

E-UTRAN shall send a RRC message implying conditional handover to Cell 2 during period T1 at a time earlier than TRRC before the beginning of T2. The field sameSFN-Indication is not included in the handover command. At the start of T2, cell 2 becomes detectable and meets the handover condition.

During the test, UE is configured with measurement gap to enable inter-frequency measurement.

Table A.14.2.1.11.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, FD-FDD duplex mode |
| 2 | NGSO, FD-FDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

Table A.14.2.1.11.1-2: General test parameters for E-UTRAN FDD Inter frequency conditional handover for Cat-M1 UEs in CEModeA without SFN acquisition test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
|  | Neighbouring cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  |  | OFF |
| CP length |  |  | Normal |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| PRACH configuration |  |  | PRACH_4CE | As specified in A.3.16 |
| PRACH initial CE level |  |  | 0 | Specified in the handover message |
| T1 |  | s | 5 |  |
| T2 |  | s | 2 |  |
| Gap pattern ID |  |  | 1 |  |

Table A.14.2.1.11.1-3: Cell specific test parameters for E-UTRAN FDD Inter frequency conditional handover for Cat-M1 UEs in CEModeA without SFN acquisition test case

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 |  | T1 | T3 |  |
| E-UTRA RF Channel Number |  | 1 |  |  | 1 |  |  |
| Satellite Information (Configuration 1)Note 1 |  | SSC.1 |  |  | NSC.1 |  |  |
| Satellite Information (Configuration 2)Note 1 |  | SSC.2 |  |  | NSC.2 |  |  |
| BWchannel | MHz | 1.4 |  |  |  |  |  |
| PDSCH Reference Channel in clause A.3.1.4.1 |  | R.48 FDD | R.48 FDD |  | - | R.48 FDD |  |
| MPDCCH Reference Channel in clause A.3.1.3.1 |  | R.46 FDD |  |  | R.46 FDD |  |  |
| OCNG Patterns in clause A.3.2.1 |  | OP.7 FDD |  | OP.7 FDD | OP.7 FDD |  | OP.7 FDD |
| PBCH_RA | dB | -3 |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 2 | dB |  |  |  |  |  |  |
| OCNG_RBNote 2 | dB |  |  |  |  |  |  |
| Note 3 | dBm/15 KHz | -98 |  |  |  |  |  |
|  | dB | 8 |  | -3.33 | -Infinity |  | 2.36 |
| Note 4 | dB | 8 |  | 8 | -Infinity |  | 11 |
| RSRP Note 4 | dBm/15 KHz | -90 |  | -90 | -Infinity |  | -87 |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x1 |  |  | 1x1 |  |  |
| Timing offset to Cell 1 Synchronous cells | us | - |  |  | Based on Satellite Assistance information |  |  |
| Note 1: Satellite information is determined according to the testing principles for NTN determined in clause B.3.8. If satellite movement is applicable, it should be considered for the duration of the test case. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot and RSRP level has been derived from other parameters for information purpose. They are not settable parameters themselves. |  |  |  |  |  |  |  |

##### A.14.2.1.12.2	Test Requirements

TRRC + TEvent_DU occurs during T1 as the handover condition becomes satisfied at the start of T2. The test shall verify that there are no interruptions during T1.

The UE shall start to transmit the PRACH to Cell 2 less than Tmeasure + Tinterrupt + TCHO_execution = [860 ms] from the start of T2 and interruption during T2 shall not exceed 50ms.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The conditional handover delay can be expressed as: TRRC + TDelayUncertainty + Tmeasure + TCHO_execution + Tinterrupt, where:

TRRC = 15 ms and is specified in clause 11.2 in TS 36.331 [2].

Tmeasure = [800] ms in the test; Tmeasure is defined in clause 5.5A.2.3.2 without TDelayUncertainty.

TCHO_execution = 10 ms in the test; TCHO_execution is defined in clause 5.5A.2.3.3.

#### A.14.2.1.12	E-UTRAN HD-FDD Inter frequency conditional handover for Cat-M1 UEs in CEModeA

##### A.14.2.1.12.1	Test Purpose and Environment

This test is to verify the requirement for the HD-FDD inter frequency conditional handover requirements. The test configurations are given in Table A.14.2.1.12.1-1.

The test scenario comprises of two E-UTRA FDD carrier and one cell in each carrier as given in tables A.14.2.1.12.1-2 and A.14.2.1.12.1-3. The test consists of two successive time periods, with time durations of T1 and T2 respectively. At the start of time duration T1, the UE shall have had the opportunity to acquire satellite assistance information for Cell 2, provided by Cell 1 in SystemInformationBlockType33.

E-UTRAN shall send a RRC message implying conditional handover to Cell 2 during period T1 at a time earlier than TRRC before the beginning of T2. The field sameSFN-Indication is not included in the handover command. At the start of T2, cell 2 becomes detectable and meets the handover condition.

During the test, UE is configured with measurement gap to enable inter-frequency measurement.

Table A.14.2.1.12.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

Table A.14.2.1.12.1-2: General test parameters for E-UTRAN HD-FDD Inter frequency conditional handover for Cat-M1 UEs in CEModeA without SFN acquisition test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
|  | Neighbouring cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  |  | OFF |
| CP length |  |  | Normal |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| PRACH configuration |  |  | PRACH_4CE | As specified in A.3.16 |
| PRACH initial CE level |  |  | 0 | Specified in the handover message |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |
| Gap pattern ID |  |  | 1 |  |

Table A.14.2.1.12.1-3: Cell specific test parameters for E-UTRAN HD-FDD Inter frequency conditional handover for Cat-M1 UEs in CEModeA without SFN acquisition test case

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 1 |  |  |
| Satellite Information (Configuration 1)Note 1 |  | SSC.1 |  |  | NSC.1 |  |  |
| Satellite Information (Configuration 2)Note 1 |  | SSC.2 |  |  | NSC.2 |  |  |
| BWchannel | MHz | 1.4 |  |  |  |  |  |
| PDSCH Reference Channel in clause A.3.1.4.1 |  | R.49 HD-FDD | R.49 HD-FDD | - | - | - | R.49 HD-FDD |
| MPDCCH Reference Channel in clause A.3.1.3.1 |  | R.47 HD-FDD |  |  | R.47 HD-FDD |  |  |
| OCNG Patterns in clause A.3.2.1 |  | OP.7 FDD | OP.7 FDD | OP.7 FDD | OP.7 FDD | OP.7 FDD | OP.7 FDD |
| PBCH_RA | dB | -3 |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 2 | dB |  |  |  |  |  |  |
| OCNG_RBNote 2 | dB |  |  |  |  |  |  |
| Note 3 | dBm/15 KHz | -98 |  |  |  |  |  |
|  | dB | 8 | 8 | 8 | -Infinity | 12 | 12 |
| Note 4 | dB | 8 | -4.27 | -4.27 | -Infinity | 3.36 | 3.36 |
| RSRP Note 4 | dBm/15 KHz | -90 | -90 | -90 | -Infinity | -86 | -86 |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x1 |  |  | 1x1 |  |  |
| Timing offset to Cell 1 Synchronous cells | us | - |  |  | Based on Satellite Assistance information |  |  |
| Note 1: Satellite information is determined according to the testing principles for NTN determined in clause B.3.8. If satellite movement is applicable, it should be considered for the duration of the test case. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot and RSRP level has been derived from other parameters for information purpose. They are not settable parameters themselves. |  |  |  |  |  |  |  |

##### A.14.2.1.12.2	Test Requirements

TRRC + TEvent_DU occurs during T1 as the handover condition becomes satisfied at the start of T2. The test shall verify that there are no interruptions during T1.

The UE shall start to transmit the PRACH to Cell 2 less than Tmeasure + Tinterrupt + TCHO_execution = [860 ms] from the start of T2 and interruption during T2 shall not exceed 50ms.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The conditional handover delay can be expressed as: TRRC + TDelayUncertainty + Tmeasure + TCHO_execution + Tinterrupt, where:

TRRC = 15 ms and is specified in clause 11.2 in TS 36.331 [2].

Tmeasure = [800] ms in the test; Tmeasure is defined in clause 5.5A.2.3.2 without TDelayUncertainty.

TCHO_execution = 10 ms in the test; TCHO_execution is defined in clause 5.5A.2.3.3.

Tinterrupt = 50 ms in the test; Tinterrupt is defined in clause 5.1.2.6.4.

5.5A.2.3

## <<< NEXT CHANGE >>>

#### A.14.3.1.3	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA for Satellite access

##### A.14.3.1.3.1	Test Purpose and Environment

The purpose is to verify that the E-UTRA FDD inter-frequency RRC re-establishment delay is within the specified limits. These tests will verify the requirements in clause 6.7A.2.

The UE shall be provided with the valid information about the SAN serving cells before the test.

The test parameters are given in table A.14.3.1.3-1 and table A.14.3.1.3-2 below. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. During T1, the UE shall be indicated with the carrier frequency of Cell 2 to ensure that the UE has the context of the carrier frequency of Cell 2. At the start of time period T2, cell 1, which is the active cell, is deactivated. The time period T3 starts after the occurrence of radio link failure. At the start of time period T3, cell 2, which is the neighbour cell, is activated.

Table A.14.3.1.3.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, FD-FDD duplex mode |
| 2 | NGSO, FD-FDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

Table A.14.3.1.3.1-2: General test parameters for E-UTRAN FDD inter-frequency RRC Re-establishment test case

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  |  | Cell 1 |  |
|  | Neighbouring cell |  |  | Cell 2 |  |
| Final condition | Active cell |  |  | Cell 2 |  |
| E-UTRA RF Channel Number (cell 1) |  |  |  | 1 | Only one FDD carrier frequency is used. |
| E-UTRA RF Channel Number (cell 2) |  |  |  | 2 |  |
| E-UTRA FDD inter-frequency carrier list size |  |  |  | 1 | 2 E-UTRA FDD carrier frequencies in total: 1 intra-frequency and 1 inter-frequency |
| Satellite information |  | Config 1 |  | SSC.1 | GSO |
|  |  | Config 2 |  | SSC.2 | NGSO |
| Channel Bandwidth (BWchannel) |  |  | MHz | 10 |  |
| PRACH Configuration |  |  |  | PRACH_2CE | As specified in A.3.16 |
| N310 |  |  | - | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  |  | - | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  |  | ms | 0 | Radio link failure timer; T310 is disabled |
| T311 |  |  | ms | 3000 | RRC re-establishment timer |
| DRX |  |  |  | OFF |  |
| CP length |  |  |  | Normal |  |
| Access Barring Information |  |  | - | Not Sent | No additional delays in random access procedure. |
| PRACH configuration index |  |  |  | 4 | As specified in table 5.7.1-2 in TS 36.211 |
| Time offset between cells |  |  | ms | 3 | Asynchronous cells |
| T1 |  |  | s | 5 |  |
| T2 |  |  | ms | 400 |  |
| T3 |  |  | s | 3 |  |

Table A.14.3.1.3.1-3: Cell specific test parameters for E-UTRAN FDD inter-frequency RRC Re-establishment test case

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 |  |  | T3 |  |  |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |  |  |  |  |
| BWchannel | MHz | 1.4 |  |  | 1.4 |  |  |  |  |  |  |
| PDSCH parameters (As specified in clause A.3.1.4.1) |  | DL Reference Measurement Channel R.48 FDD |  |  | DL Reference Measurement Channel R.48 FDD |  |  |  |  |  |  |
| MPDCCH parameters (As specified in clause A.3.1.3.1) |  | DL Reference Measurement Channel R.46 FDD |  |  | DL Reference Measurement Channel R.46 FDD |  |  |  |  |  |  |
| OCNG Patterns defined in A.3.2.1.21 (OP.21 FDD) and in A.3.2.1.6 (OP.6 FDD) |  | OP.21 FDD | OP.21 FDD | OP.7 FDD | OP.7 FDD |  | OP.7 FDD |  |  | OP.21 FDD |  |
| PBCH_RA | dB | -3 |  |  | -3 |  |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
|  | dB | 4 | -Infinity | -Infinity | -Infinity |  |  | -Infinity |  |  | 7 |
| Note 2 | dBm/15 KHz | -98 |  |  |  |  |  |  |  |  |  |
|  | dB | 4 | -Infinity | -Infinity | -Infinity |  |  | -Infinity |  |  | 7 |
| RSRP Note 3 | dBm/15 KHz | -94 | -Infinity | -Infinity | -Infinity |  |  | -Infinity |  |  | -91 |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |  |
| Antenna Configuration |  | 1x1 |  |  | 1x1 |  |  |  |  |  |  |
| Timing offset to Cell 1 (Asynchronous cells) | Ms | - |  |  | 3 |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |

##### A.14.3.1.3.2	Test Requirements

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send PRACH preambles to cell 2 for sending the RRCConnectionReestablishmentRequest message to cell 2.

The RRC re-establishment delay to a known E-UTRA FDD inter frequency cell shall be less than 3.5 s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE_re-establish_delay.

Where:

TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The PRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

TUE_re-establish_delay=50 ms+i=1NfreqKsatellite,i*Tsearch+TSI_EUTRA-M1-CEModeA+TPRACH

Nfreq = 2

Ksatellite,i =1

Tsearch = 1000 ms

TSI-EUTRA-M1-CEModeA = 1280 ms; it is the time required for receiving all the relevant system information as defined in TS 36.331 for the target E-UTRAN FDD cell.

TPRACH = 15 ms; it is the additional delay caused by the random access procedure.

This gives a total of 3345 ms, allow 3.5 s in the test case.

#### A.14.3.1.4	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA for Satellite access

##### A.14.3.1.4.1	Test Purpose and Environment

The purpose is to verify that the E-UTRA HD-FDD inter-frequency RRC re-establishment delay is within the specified limits. These tests will verify the requirements in clause 6.7A.2.

The UE shall be provided with the valid information about the SAN serving cells before the test.

The test parameters are given in tables A.14.3.1.4-1, A.14.3.1.4-2 and A.14.3.1.4-3 below. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. During T1, the UE shall be indicated with the carrier frequency of Cell 2 to ensure that the UE has the context of the carrier frequency of Cell 2. At the start of time period T2, cell 1, which is the active cell, is deactivated. The time period T3 starts after the occurrence of radio link failure. At the start of time period T3, cell 2, which is the neighbour cell, is activated.

Table A.14.3.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

Table A.14.3.1.4.1-2: General test parameters for E-UTRAN HD-FDD inter-frequency RRC Re-establishment test case

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  |  | Cell 1 |  |
|  | Neighbouring cell |  |  | Cell 2 |  |
| Final condition | Active cell |  |  | Cell 2 |  |
| E-UTRA RF Channel Number (cell 1) |  |  |  | 1 | Only one FDD carrier frequency is used. |
| E-UTRA RF Channel Number (cell 2) |  |  |  | 2 |  |
| E-UTRA FDD inter-frequency carrier list size |  |  |  | 1 | 2 E-UTRA FDD carrier frequencies in total: 1 intra-frequency and 1 inter-frequency |
| Satellite information |  | Config 1 |  | SSC.1 | GSO |
|  |  | Config 2 |  | SSC.2 | NGSO |
| Channel Bandwidth (BWchannel) |  |  | MHz | 10 |  |
| PRACH Configuration |  |  |  | PRACH_2CE | As specified in A.3.16 |
| N310 |  |  | - | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  |  | - | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  |  | ms | 0 | Radio link failure timer; T310 is disabled |
| T311 |  |  | ms | 3000 | RRC re-establishment timer |
| DRX |  |  |  | OFF |  |
| CP length |  |  |  | Normal |  |
| Access Barring Information |  |  | - | Not Sent | No additional delays in random access procedure. |
| PRACH configuration index |  |  |  | 4 | As specified in table 5.7.1-2 in TS 36.211 |
| Time offset between cells |  |  | ms | 3 | Asynchronous cells |
| T1 |  |  | s | 5 |  |
| T2 |  |  | ms | 400 |  |
| T3 |  |  | s | 3 |  |

Table A.14.3.1.4.1-3: Cell specific test parameters for E-UTRAN HD-FDD inter-frequency RRC Re-establishment test case

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 |  |  | T3 |  |  |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |  |  |  |  |
| BWchannel | MHz | 1.4 |  |  | 1.4 |  |  |  |  |  |  |
| PDSCH parameters (As specified in clause A.3.1.4.2) |  | DL Reference Measurement Channel R.49 HD-FDD |  |  | DL Reference Measurement Channel R.49 HD-FDD |  |  |  |  |  |  |
| MPDCCH parameters (As specified in clause A.3.1.3.2) |  | DL Reference Measurement Channel R.47 HD-FDD |  |  | DL Reference Measurement Channel R.47 HD-FDD |  |  |  |  |  |  |
| OCNG Patterns defined in A.3.2.1.21 (OP.21 FDD) and in A.3.2.1.6 (OP.6 FDD) |  | OP.21 FDD | OP.21 FDD | OP.7 FDD | OP.7 FDD |  | OP.7 FDD |  |  | OP.21 FDD |  |
| PBCH_RA | dB | -3 |  |  | -3 |  |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
|  | dB | 4 | -Infinity | -Infinity | -Infinity |  |  | -Infinity |  |  | 7 |
| Note 2 | dBm/15 KHz | -98 |  |  |  |  |  |  |  |  |  |
|  | dB | 4 | -Infinity | -Infinity | -Infinity |  |  | -Infinity |  |  | 7 |
| RSRP Note 3 | dBm/15 KHz | -94 | -Infinity | -Infinity | -Infinity |  |  | -Infinity |  |  | -91 |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |  |
| Antenna Configuration |  | 1x1 |  |  | 1x1 |  |  |  |  |  |  |
| Timing offset to Cell 1 (Asynchronous cells) | Ms | - |  |  | 3 |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |

##### A.14.3.1.4.2	 Test Requirements

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send PRACH preambles to cell 2 for sending the RRCConnectionReestablishmentRequest message to cell 2.

The RRC re-establishment delay to a known E-UTRA FDD inter frequency cell shall be less than 3.5 s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE_re-establish_delay.

Where:

TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The PRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

TUE_re-establish_delay=50 ms+i=1NfreqKsatellite,i*Tsearch+TSI_EUTRA-M1-CEModeA+TPRACH

Nfreq = 2

Ksatellite,i =1

Tsearch = 1000 ms

TSI-EUTRA-M1-CEModeA = 1280 ms; it is the time required for receiving all the relevant system information as defined in TS 36.331 for the target E-UTRAN FDD cell.

TPRACH = 15 ms; it is the additional delay caused by the random access procedure.

This gives a total of 3345 ms, allow 3.5 s in the test case.

## <<< NEXT CHANGE >>>

#### A.14.6.1.3	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA

##### A.14.6.1.3.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.21A.9 and 9.1.21A.10 for FD-FDD inter frequency RSRP measurements for Cat-M1 UE in CEModeA.

##### A.14.6.1.3.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Both absolute and relative accuracy of RSRP inter frequency measurements are tested by using the parameters in Table A.14.6.1.3.2-1 and Table A.14.6.1.3.2-2. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

Table A.14.6.1.3.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, FD-FDD duplex mode |
| 2 | NGSO, FD-FDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

Table A.14.6.1.3.2-2: FD-FDD RSRP Inter frequency test parameters for Cat-M1 UE in CEModeA

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Satellite information |  | Config 1 |  | SSC.1 | NSC.1 | SSC.1 | NSC.1 |
|  |  | Config 2 |  | SSC.2 | NSC.2 | SSC.2 | NSC.2 |
| E-UTRA RF Channel Number |  |  |  | 1 | 2 | 1 | 2 |
| BWchannel |  |  | MHz | 1.4 | 1.4 | 1.4 | 1.4 |
| Gap Pattern Id |  |  |  | 0 | - | 0 | - |
| PDSCH Reference measurement channel |  |  |  | R.48 FDD | - | R.48 FDD | - |
| PDSCH allocation |  |  |  | Follows R.48 FDD | - | Follows R.48 FDD | - |
| MPDCCH Reference measurement channel |  |  |  | R.46 FDD | R.46 FDD | R.46 FDD | R.46 FDD |
| OCNG Patterns |  |  |  | OP.7 FDD | OP.7 FDD | OP.7 FDD | OP.7 FDD |
| PBCH_RA |  |  | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |
| Note2 | FDD-M1_SAB_A |  | dBm/15 kHz | -88.65 | -88.65 | ( for Channel 2 +8dB) | -116 |
|  | FDD-M1_SAB_B |  |  |  |  |  | -115.5 |
|  |  |  | dB | 10 | 10 | 13 | -4 |
| RSRPNote3 | FDD-M1_SAB_A |  | dBm/15 kHz | -78.65 | -78.65 | (RSRP for Cell 2 +25dB) | -120 |
|  | FDD-M1_SAB_B |  |  |  |  |  | -119.5 |
| IoNote3 | FDD-M1_SAB_A |  | dBm/9 MHz | -50.45 | -50.45 | (Io for Channel 2 +19.75dB) | -86.76 |
|  | FDD-M1_SAB_B |  |  |  |  |  | -86.26 |
|  |  |  | dB | 10 | 10 | 13 | -4 |
| Propagation condition |  |  | - | AWGN | AWGN | AWGN | AWGN |
| Antenna Configuration |  |  |  | 1x1 | 1x1 | 1x1 | 1x1 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |

##### A.14.6.1.3.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.21A.9 and 9.1.21A.10.

#### A.14.6.1.4	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA

##### A.14.6.1.4.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in section 9.1.21A.9 and 9.1.21A.10 for HD-FDD inter frequency RSRP measurements for Cat-M1 UE in CEModeA.

##### A.14.6.1.4.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Both absolute and relative accuracy of RSRP inter frequency measurements are tested by using the parameters in Table A.14.6.1.4.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

Table A.14.6.1.4.2-1: HD-FDD RSRP Inter frequency test parameters for Cat-M1 UE in CEModeA

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Satellite information | Config 1 |  | SSC.1 | NSC.1 | SSC.1 | NSC.1 |
|  | Config 2 |  | SSC.2 | NSC.2 | SSC.2 | NSC.2 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 1 | 2 |
| BWchannel |  | MHz | 1.4 | 1.4 | 1.4 | 1.4 |
| Gap Pattern Id |  |  | 0 | - | 0 | - |
| PDSCH Reference measurement channel |  |  | R.49 HD-FDD | - | R.49 HD-FDD | - |
| PDSCH allocation |  |  | Follows R.49 HD-FDD | - | Follows R.49 HD-FDD | - |
| MPDCCH Reference measurement channel |  |  | R.47 HD-FDD | R.47 HD-FDD | R.47 HD-FDD | R.47 HD-FDD |
| OCNG Patterns |  |  | OP.7 FDD | OP.7 FDD | OP.7 FDD | OP.7 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |
| Note2 | FDD-M1_SAB_A | dBm/15 kHz | -88.65 | -88.65 | ( for Channel 2 +8dB) | -116 |
|  | FDD-M1_SAB_B |  |  |  |  | -115.5 |
|  |  | dB | 10 | 10 | 13 | -4 |
| RSRPNote3 | FDD-M1_SAB_A | dBm/15 kHz | -78.65 | -78.65 | (RSRP for Cell 2 +25dB) | -120 |
|  | FDD-M1_SAB_B |  |  |  |  | -119.5 |
| IoNote3 | FDD-M1_SAB_A | dBm/9 MHz | -50.45 | -50.45 | (Io for Channel 2 +19.75dB) | -86.76 |
|  | FDD-M1_SAB_B |  |  |  |  | -86.26 |
|  |  | dB | 10 | 10 | 13 | -4 |
| Propagation condition |  | - | AWGN | AWGN | AWGN | AWGN |
| Antenna Configuration |  |  | 1x1 | 1x1 | 1x1 | 1x1 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

##### A.14.6.1.4.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.21A.9 and 9.1.21A.10.

#### A.14.6.1.5	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB

##### A.14.6.1.5.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.21A.11 and 9.1.21A.12 for FD-FDD intra frequency RSRP measurements for Cat-M1 UE in CEModeB.

##### A.14.6.1.5.2	Test parameters

Both absolute and relative accuracy of RSRP inter frequency measurements are tested by using the parameters in Table A.14.6.1.5.2-1 and A.14.6.1.5.2-2. In all test cases, Cell 1 is the PCell and Cell 2 the target cell. All the tests contain MPDCCH for UL grant for reporting RSRP.

Table A.14.6.1.5.2-1: FD-FDD RSRP Inter frequency test parameters for Cat-M1 UE in CEModeB for 1.4 MHz cell BW

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Satellite information | Config 1 |  | SSC.1 | NSC.1 | SSC.1 | NSC.1 |
|  | Config 2 |  | SSC.2 | NSC.2 | SSC.2 | NSC.2 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 1 | 2 |
| BWchannel |  | MHz | 1.4 | 1.4 | 1.4 | 1.4 |
| Gap Pattern Id |  |  | 0 | - | 0 | - |
| PDSCH Reference measurement channel |  |  | R.52 FDD | - | R.52 FDD | - |
| PDSCH allocation |  |  | Follows R.52 FDD | - | Follows R.52 FDD | - |
| MPDCCH Reference measurement channel |  |  | R.50 FDD | R.50 FDD | R.50 FDD | R.50 FDD |
| OCNG Patterns |  |  | OP.7 FDD | OP.7 FDD | OP.7 FDD | OP.7 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |
| Note2 | FDD-M1_SAB_A | dBm/15 kHz | -78.5 | -78.5 | -98.5 | -108 |
|  | FDD-M1_SAB_B |  |  |  |  | -107.5 |
|  |  | dB | -12 | -12 | -12 | -12 |
| RSRPNote3 | FDD-M1_SAB_A | dBm/15 kHz | -90.5 | -90.5 | -110.5 | -120 |
|  | FDD-M1_SAB_B |  |  |  |  | -119.5 |
| IoNote3 | FDD-M1_SAB_A | dBm/9 MHz | -50.45 | -50.45 | -70.45 | -86.76 |
|  | FDD-M1_SAB_B |  |  |  |  | -86.26 |
|  |  | dB | -12 | -12 | -12 | -12 |
| Propagation condition |  | - | AWGN | AWGN | AWGN | AWGN |
| Antenna Configuration |  |  | 1x1 | 1x1 | 1x1 | 1x1 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

##### A.14.6.1.5.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.21A.11 and 9.1.21A.12.

#### A.14.6.1.6	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB

##### A.14.6.1.6.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.21A.11 and 9.1.21A.12 for HD-FDD inter frequency RSRP measurements for Cat-M1 UE in CEModeB.

##### A.14.6.1.6.2	Test parameters

Both absolute and relative accuracy of RSRP inter frequency measurements are tested by using the parameters in Table A.14.6.1.6.2-1 and A.14.6.1.6.2-2. In all test cases, Cell 1 is the PCell and Cell 2 the target cell. All the tests contain MPDCCH for UL grant for reporting RSRP.

Table A.14.6.1.6.2-1: HD-FDD RSRP Inter frequency test parameters for Cat-M1 UE in CEModeB for 1.4 Mhz Cell BW

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Satellite information | Config 1 |  | SSC.1 | NSC.1 | SSC.1 | NSC.1 |
|  | Config 2 |  | SSC.2 | NSC.2 | SSC.2 | NSC.2 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 1 | 2 |
| BWchannel |  | MHz | 1.4 | 1.4 | 1.4 | 1.4 |
| Gap Pattern Id |  |  | 0 | - | 0 | - |
| PDSCH Reference measurement channel |  |  | R.53 HD-FDD | - | R.53 HD-FDD | - |
| PDSCH allocation |  |  | Follows R.53 HD-FDD | - | Follows R.53 HD-FDD | - |
| MPDCCH Reference measurement channel |  |  | R.51 HD-FDD | R.51 HD-FDD | R.51 HD-FDD | R.51 HD-FDD |
| OCNG Patterns |  |  | OP.7 FDD | OP.7 FDD | OP.7 FDD | OP.7 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |
| Note2 | FDD-M1_SAB_A | dBm/15 kHz | -78.5 | -78.5 | -98.5 | -108 |
|  | FDD-M1_SAB_B |  |  |  |  | -107.5 |
|  |  | dB | -12 | -12 | -12 | -12 |
| RSRPNote3 | FDD-M1_SAB_A | dBm/15 kHz | -90.5 | -90.5 | -110.5 | -120 |
|  | FDD-M1_SAB_B |  |  |  |  | -119.5 |
| IoNote3 | FDD-M1_SAB_A | dBm/9 MHz | -50.45 | -50.45 | -70.45 | -86.76 |
|  | FDD-M1_SAB_B |  |  |  |  | -86.26 |
|  |  | dB | -12 | -12 | -12 | -12 |
| Propagation condition |  | - | AWGN | AWGN | AWGN | AWGN |
| Antenna Configuration |  |  | 1x1 | 1x1 | 1x1 | 1x1 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

##### A.14.6.1.6.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.21A.11 and 9.1.21A.12.

## <<< END OF CHANGES >>>