3GPP TSG-RAN WG4 Meeting # 110bis                                                                     R4-2404279

Changsha, China, 15 April – 19 April 2024

Title: 	on FR1 dynamic MIMO OTA test methodology

Source: 	Huawei, HiSilicon

Agenda item:	9.4.2.3

Document for:	discussion

# Introduction

RAN#103 approves a new work item on OTA test with the following scope for MIMO OTA.

- Study and develop FR1 dynamic MIMO OTA test methodology with the following aspects

  - Reusing Rel-17 16-probes FR1 MPAC system layout

  - Study and define dynamic channel model parameters. CDL channel models defined in TR 38.901 and TR 38.827 should be considered as the starting point

  - Specify channel model validation procedures and pass/fail limits

  - Decide environmental conditions

    - Noise-limited environmental condition is the baseline

  - Study and define appropriate performance metric under dynamic channel model

  - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system (RAN5)

This contribution discusses some aspects of OTA tests for MIMO OTA.

# Discussion

Some work has been done on MIMO OTA tests using dynamic channels [2, 3] and can be used as a reference or starting point. Although it is possible to derive dynamic channels based on channel sounding data or propagation environmental simulation using ray tracing as shown in Table I in [4], however, such approach would be time consuming and outside the scope of this work item. It would be convenient to use existing 3GPP channels for various segments of a dynamic channel.

The composition of a dynamic channel needs to reflect realistic network scenarios and therefore should be based on drive data. Such data can be simplified into various segments for ease of implementation by test equipment.

Proposal 1: dynamic channel model should consist of number of segments with fixed channel models available in 3GPP.

The number of segments need to be manageable from an implementation point of view while reflecting real network conditions.

Proposal 2: the number of segments in a dynamic channel should be in the range of [8] to [12].

Proposal 3: parameters would need to be interpolated between segments to ensure continuity of dynamic channels.

Pathloss variation determines the dynamic range of the chamber. Feedback from test equipment vendors should be sought before determining the pathloss variation.

Proposal 4: Pathloss variation in a dynamic model should account for achievable chamber dynamic range.

A dynamic channel should balance between high and low throughput segments. Such a balance is reflected in the lengths or duration of segments.

Proposal 5: A dynamic channel should balance between high and low throughput segments by using lengths of corresponding segments.

Channels for various segments can be further discussed once some drive data become available.

Throughput CDF could be a good candidate for performance metrics because it can reflect performances of all segments. A fixed percentile or a combination of a number of percentiles could be used.

Proposal 6: Throughput CDF can be a good candidate for performance metric with either a fixed percentile or a combination of a number of percentiles.

# Conclusions

This contribution makes the following proposals.

Proposal 1: dynamic channel model should consist of number of segments with fixed channel models available in 3GPP.

Proposal 2: the number of segments in a dynamic channel should be in the range of [8] to [12].

Proposal 3: parameters would need to be interpolated between segments to ensure continuity of dynamic channels.

Proposal 4: Pathloss variation in a dynamic model should account for achievable chamber dynamic range.

Proposal 5: A dynamic channel should balance between high and low throughput segments by using lengths of corresponding segments.

Proposal 6: Throughput CDF can be a good candidate for performance metric with either a fixed percentile or a combination of a number of percentiles.

# References

[1] RP-240841 WI on TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3

[2] Developing a MIMO Test Methodology using Dynamic Channel Models and Link Adaptation, M. Foegel et al, 16th European Conference on Antenna and Propagation, 2022

[3] MIMO OTA Testing for 5G User Equipment Using Dynamic Channel Model, X. Chen, et al, 2023 International Applied Computational Electromagnetics Society Symposium

[4] Time-Domain Nonstationary Channel Emulation in Multiprobe Anechoic Chamber Setups for Over-the-Air Testing, Z. Jiang, et al, IEEE Antennas and Wireless Propagation Letters, Vol. 20, December 2021, pp.2511-2515