3GPP TSG-RAN WG4 Meeting #110b	R4-2405136

# Changsha, China, 15th - 19th Apr, 2024

Title: 	Discussion on demodulation performance requirements for mobile IAB

Source: 	Huawei, HiSilicon

Agenda item:	6.23.5

Document for:	Discussion

Background

During RAN4#110 meeting, WF Error! Reference source not found. for NR mobile IAB demod was approved. In this contribution, we share our views about demodulation performance requirements for mobile IAB.

Discussion

mIAB-MT

FR1 PDSCH demodulation requirements selection from 38.101-4

Modulation order and Channel model

Way forward: Both option 1 and option 2 are introduced, but option 2 replaces a corresponding existing test case. - Proposals - Option 1: QPSK with TDLB100-400 only - Option 2: QPSK with TDLB100-400 and 16QAM with TDLC300-100.

For FR1 PDSCH, based on the way forward, the QPSK with TDLB100-400 case should be defined and the 16QAM with TDLC300-100 case should replaces a corresponding existing test case. Based on the existing IAB specification, the test 1-2 in Table 8.2.2.1.2-1 of TS38.174 and the second case in Table 11.2.2.1.1.2-1 of TS38.174 should be replaced, as following.

Table 8.2.2.1.2-1: Minimum requirements for PDSCH Type A with Rank 1 Test number; FRC (Annex A); Bandwidth (MHz) / Subcarrier spacing (kHz); Propagation conditions (Annex I); Antenna configuration; Fraction of maximum throughput (%); SNR (dB). 1-1; M-FR1-A.3.3-1; 40/30; TDLA30-10; 2x4, ULA Low; 70; 21.6. 1-2; M-FR1-A.3.1-1; 40/30; TDLA30-10; 2x4, ULA Low; 30; -1.1. Table 11.2.2.1.1.2-1: Minimum requirements for PDSCH with Rank 1, 40 MHz Channel Bandwidth, 30 kHz SCS Number of TX antennas; Number of demodulation branches; Propagation conditions and correlation matrix (Annex I); FRC (Annex A); Fraction of maximum throughput (%); SNR (dB). 2; 2; TDLA30-10, ULA Low; M-FR1-A.3.3-1; 70; 25.3. TDLA30-10, ULA Low; M-FR1-A.3.1-1; 30; 2.2.

For FR1 PDSCH, the test 1-2 in Table 8.2.2.1.2-1 of TS38.174 and the second case in Table 11.2.2.1.1.2-1 of TS38.174 should be replaced.

TDD pattern

Way forward: - Option 1: FR1 30-1, 7D1S2U, S=6D4G4U. - Option 2: FR1 30-5, DSUU, S=12D2G. - Other options are not precluded.

For FR1 PDSCH, for the TDD pattern, the same principle as Rel-16 IAB can be reused, i.e. select 7D1S2U, S=6D:4G:4U as default TDD UL-DL pattern and the same requirements are applicable to TDD with different UL-DL patterns.

For FR1 PDSCH, select 7D1S2U, S=6D:4G:4U as default TDD UL-DL pattern and the same requirements are applicable to TDD with different UL-DL patterns.

Test case

Way forward: - Conducted: - Option 1: 1-8 and 1-4 in 5.2.3.2.1. - Option 2: test num. 1-2, 1-4 in table 5.2.3.2.1-3. - Option 3: Test 1-1 in Table 5.2.3.2.1-3. - Other options are not precluded. - Radiated: - Option 1: Test 1-1 in Table 5.2.2.2.1-3. - Other options are not precluded.

For FR1 PDSCH conducted test, from our understanding, the test 1-1 and test 1-8 are corresponding to the same case since the only difference between the two cases is TDD pattern, also considering that IAB-MT use BS test method, i.e. define FRC based on single slot and same requirements are applicable to TDD with different UL-DL patterns. Also considering the discussion on clause 2.1.1.1, the test 1-2 (QPSK with TDLC300-100) is not in the list of modulation order and channel model combination. Overall, we propose to select test 1-8(1-1) and 1-4 in table 5.2.3.2.1-3 for FR1 PDSCH conducted test, as following.

Table 5.2.3.2.1-3: Minimum performance for Rank 1 Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; TDD UL-DL pattern; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 1-1; R.PDSCH.2-1.1 TDD; 40 / 30; QPSK, 0.30; FR1.30-1A; TDLB100-400; 2x4, ULA Low; 70; -4.1. 1-2; R.PDSCH.2-1.2 TDD; 40 / 30; QPSK, 0.30; FR1.30-1; TDLC300-100; 2x4, ULA Low; 70; -2.7. 1-3; R.PDSCH.2-4.1 TDD; 40 / 30; 256QAM, 0.82; FR1.30-1; TDLA30-10; 2x4, ULA Low; 70; 21.6. 1-4; R.PDSCH.2-2.1 TDD; 40 / 30; 16QAM, 0.48; FR1.30-1; TDLC300-100; 2x4, ULA Low; 30; -1.2. 1-5; R.PDSCH.2-5.1 TDD; 40 / 30; QPSK, 0.30; FR1.30-2; TDLA30-10; 2x4, ULA Low; 70; -3.8. 1-6; R.PDSCH.2-6.1 TDD; 40 / 30; QPSK, 0.30; FR1.30-3; TDLA30-10; 2x4, ULA Low; 70; -3.6. 1-7; R.PDSCH.2-10.1 TDD; 40 / 30; 16QAM, 0.48; FR1.30-1; HST-1000; 1x4; 70; 3.4. 1-8; R.PDSCH.2-11.1 TDD; 40 / 30; QPSK, 0.30; FR1.30-5; TDLB100-400; 2x4, ULA Low; 70; -4.0. 1-9; R.PDSCH.2-12.1 TDD; 40 / 30; QPSK, 0.30; FR1.30-6; TDLB100-400; 2x4, ULA Low; 70; -4.0. 1-10; R.PDSCH.2-10.2 TDD; 40 / 30; 16QAM, 0.48; FR1.30-1; TDLC300-1200; 2x4; 70; 5.8. 1-11; R.PDSCH.2-10.3 TDD; 40 / 30; 64QAM, 0.43; FR1.30-1; HST-1667; 1x4; 70; 6.8. 1-12; R.PDSCH.2-25.1 TDD; 40 / 30; 1024QAM, 0.79; FR1.30-1; TDLD30-5; 2x4, ULA Low; 70; 26.3.

For FR1 PDSCH radiated test, we propose to select the same test case as conducted test, i.e. test 1-8(1-1) and 1-4 in table 5.2.2.2.1-3, as following.

Table 5.2.2.2.1-3: Minimum performance for Rank 1 Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; TDD UL-DL pattern; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 1-1; R.PDSCH.2-1.1 TDD; 40 / 30; QPSK, 0.30; FR1.30-1A; TDLB100-400; 2x2, ULA Low; 70; -1.1. 1-2; R.PDSCH.2-1.2 TDD; 40 / 30; QPSK, 0.30; FR1.30-1; TDLC300-100; 2x2, ULA Low; 70; 0.2. 1-3; R.PDSCH.2-4.1 TDD; 40 / 30; 256QAM, 0.82; FR1.30-1; TDLA30-10; 2x2, ULA Low; 70; 25.3. 1-4; R.PDSCH.2-2.1 TDD; 40 / 30; 16QAM, 0.48; FR1.30-1; TDLC300-100; 2x2, ULA Low; 30; 1.6. 1-5; R.PDSCH.2-5.1 TDD; 40 / 30; QPSK, 0.30; FR1.30-2; TDLA30-10; 2x2, ULA Low; 70; -0.9. 1-6; R.PDSCH.2-6.1 TDD; 40 / 30; QPSK, 0.30; FR1.30-3; TDLA30-10; 2x2, ULA Low; 70; -0.8. 1-7; R.PDSCH.2-10.1 TDD; 40 / 30; 16QAM, 0.48; FR1.30-1; HST-1000; 1x2; 70; 6.4. 1-8; R.PDSCH.2-11.1 TDD; 40 / 30; QPSK, 0.30; FR1.30-5; TDLB100-400; 2x2, ULA Low; 70; -1.0. 1-9; R.PDSCH.2-12.1 TDD; 40 / 30; QPSK, 0.30; FR1.30-6; TDLB100-400; 2x2, ULA Low; 70; -1.1. 1-10; R.PDSCH.2-10.2 TDD; 40 / 30; 16QAM, 0.48; FR1.30-1; TDLC300-1200; 2x2; 70; 9.5. 1-11; R.PDSCH.2-10.3 TDD; 40 / 30; 64QAM, 0.43; FR1.30-1; HST-1667; 1x2; 70; 9.6. 1-12; R.PDSCH.2-25.1 TDD; 40 / 30; 1024QAM, 0.79; FR1.30-1; TDLD30-5; 2x2, ULA Low; 70; 29.4.

For FR1 PDSCH conducted test, the test 1-1 and test 1-8 are corresponding to the same case.

For FR1 PDSCH conducted test, select test 1-8(1-1) and 1-4 in table 5.2.3.2.1-3.

For FR1 PDSCH radiated test, select test 1-8(1-1) and 1-4 in table 5.2.2.2.1-3.

FR2-1 PDSCH demodulation requirements selection from 38.101-4

Rank

Way forward: Consider rank 1 and FFS on rank 2.

Considering that there is already rank 2 cases for FR2-1 PDSCH in the existing IAB specification, we propose to not consider rank 2 for FR2-1 PDSCH radiated test to reduce the number of the test cases.

For FR2-1 PDSCH, do not consider rank 2.

Modulation order and Channel model

Way forward: Consider 64QAM with TDLA30-300 and FFS on other modulations.

For FR2-1 PDSCH, to reduce the number of the test case, we propose to only consider 64QAM with TDLA30-300 and do not consider other modulations.

For FR2-1 PDSCH, only consider 64QAM with TDLA30-300 and do not consider other modulations.

TDD pattern

Way forward: - Option 1: FR2 120-1, DDDSU, S=10D2G2U. - Option 2: FR2.120-2, DDSU, S=11D3G0U. - Other options are not precluded.

For FR2-1 PDSCH, for the TDD pattern, the same principle as Rel-16 IAB can be reused, i.e. select 3D1S1U, S=10D:2G:2U as default TDD UL-DL pattern and the same requirements are applicable to TDD with different UL-DL patterns.

For FR2-1 PDSCH, select 3D1S1U, S=10D:2G:2U as default TDD UL-DL pattern and the same requirements are applicable to TDD with different UL-DL patterns.

Test case

Way forward: - Option 1: test 1-1, 1-3 and 2-3 in 7.2.2.2.1 of TS 38.101-4. - Option 2: test 1-1, 1-2, 1-3 in 7.2.2.2.1 of TS 38.101-4. - Option 3: test 1-1 in 7.2.2.2.1 of TS 38.101-4. - Other options are not precluded.

For FR2-1 PDSCH, to reduce the number of the test case, we propose to only consider test 1-3 in Table 7.2.2.2.1-3 of TS 38.101-4, as following.

Table 7.2.2.2.1-3: Minimum performance for Rank 1 (FRC) for FR2-1 Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation and code rate; TDD UL-DL pattern; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNRBB (dB). 1-1; R.PDSCH.5-1.1 TDD; 100 / 120; QPSK, 0.30; FR2.120-1A; TDLC60-300; 2x2 ULA Low; 70; -0.4. 1-2; R.PDSCH.5-2.1 TDD; 100 / 120; 16QAM, 0.48; FR2.120-1; TDLA30-300; 2x2 ULA Low; 30; 1.7. 1-3; R.PDSCH.5-3.1 TDD; 100 / 120; 64QAM, 0.46; FR2.120-1; TDLA30-300; 2x2 XPL Medium; 70; 12.4. 1-4; R.PDSCH.5-10.1 TDD; 50 / 120; 256QAM 0.67; FR2.120-1; TDLD30-75; 2x2 ULA Low; 70; 20.2.

For FR2-1 PDSCH, only consider test 1-3 in Table 7.2.2.2.1-3 of TS 38.101-4.

FR1 PDCCH demodulation requirements selection from 38.101-4

Antenna configuration

Way forward: - Conducted - 1Tx4Rx - FFS on 2Tx4Rx. - Radiated - 1Tx2Rx - FFS on 2Tx2Rx.

Based on the discussion on clause 2.1.3.4, we propose to consider only 1Tx4Rx for FR1 PDCCH conduct test and only 1Tx2Rx for FR1 PDCCH radiated test.

For FR1 PDCCH conduct test, consider only 1Tx4Rx.

For FR1 PDCCH radiated test, consider only 1Tx2Rx.

CORESET duration

Way forward: - Option 1: Only 1. - Option 2: 1 and 2.

Based on the discussion on clause 2.1.3.4, we propose to consider only CORESET duration 1 for FR1 PDCCH.

For FR1 PDCCH, consider only CORESET duration 1.

Aggregation level

Way forwards: - Option 1: 4 for 1Tx and 8 for 2Tx. - Option 2: 2 and 4 for 1Tx. - Other options are not precluded.

Based on the discussion on clause 2.1.3.4, we propose to consider only aggregation level 4 for 1Tx for FR1 PDCCH.

For FR1 PDCCH, consider only aggregation level 4 for 1Tx.

Test case

Way forward: - Conducted - 1Tx4Rx - Option 1: test number 2 in table 5.3.3.2.1-1. - 2Tx4Rx - Option 1: test number 1 in table 5.3.3.2.2-1 - Radiated - 1Tx2Rx - Option 1: Test 2 in Table 5.3.2.2.1-1 of TS 38.101-4.

For FR1 PDCCH conducted test, there are two candidate cases with higher Doppler can be reused from all the legacy UE PDCCH requirements, i.e. Test 2 in Table 5.3.3.2.1-1 of TS 38.101-4 and Test 1 in Table 5.3.3.2.2-1 of TS 38.101-4. To reduce the test effort, we propose to select only Test 2 in Table 5.3.3.2.1-1 of TS 38.101-4, as following.

Table 5.3.3.2.1-1: Minimum performance for PDCCH with 30 kHz SCS Test number; Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Reference Channel; Propagation Condition; Antenna configuration and correlation Matrix; Reference value. Pm-dsg (%); SNR (dB). 1; 40; 102; 1; 2; R.PDCCH. 2-1.1 TDD; TDLA30-10; 1x4 Low; 1; 2.1. 2; 40; 102; 1; 4; R.PDCCH. 2-1.2 TDD; TDLC300-100; 1x4 Low; 1; -0.9. 3; 40; 48; 2; 16; R.PDCCH. 2-2.1 TDD; TDLA30-10; 1x4 Medium A; 1; -3.6. Table 5.3.3.2.2-1: Minimum performance for PDCCH with 30 kHz SCS Test number; Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Reference Channel; Propagation Condition; Antenna configuration and correlation Matrix; Reference value. Pm-dsg (%); SNR (dB). 1; 40; 90; 1; 8; R.PDCCH. 2-1.3 TDD; TDLC300-100; 2x4 Low; 1; -4.3.

For FR1 PDCCH radiated test, there are two candidate cases with higher Doppler can be reused from all the legacy UE PDCCH requirements, i.e. Test 2 in Table 5.3.2.2.1-1 of TS 38.101-4 and Test 1 in Table 5.3.2.2.2-1 of TS 38.101-4. To reduce the test effort, we propose to select only Test 2 in Table 5.3.2.2.1-1 of TS 38.101-4, as following.

Table 5.3.2.2.1-1: Minimum performance for PDCCH with 30 kHz SCS Test number; Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Reference Channel; Propagation Condition; Antenna configuration and correlation Matrix; Reference value. Pm-dsg (%); SNR (dB). 1; 40; 102; 1; 2; R.PDCCH. 2-1.1 TDD; TDLA30-10; 1x2 Low; 1; 7.0. 2; 40; 102; 1; 4; R.PDCCH. 2-1.2 TDD; TDLC300- 100; 1x2 Low; 1; 3.0. 3; 40; 48; 2; 16; R.PDCCH. 2-2.1 TDD; TDLC300- 100; 1x2 Low; 1; -3.8. Table 5.3.2.2.2-1: Minimum performance for PDCCH with 30 kHz SCS Test number; Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Reference Channel; Propagation Condition; Antenna configuration and correlation Matrix; Reference value. Pm-dsg (%); SNR (dB). 1; 40; 90; 1; 8; R.PDCCH. 2-1.3 TDD; TDLC300-100; 2x2 Low; 1; -1.2.

For FR1 PDCCH conducted test, select only Test 2 in Table 5.3.3.2.1-1 of TS 38.101-4.

For FR1 PDCCH radiated test, select only Test 2 in Table 5.3.2.2.1-1 of TS 38.101-4.

FR2-1 PDCCH demodulation requirements selection from 38.101-4

Antenna configuration

Way forward: - 1Tx2Rx - FFS on 2Tx2Rx.

Based on the discussion on clause 2.1.4.4, we propose to only consider 1Tx2Rx for FR2-1 PDCCH.

For FR2-1 PDCCH, only consider 1Tx2Rx.

CORESET duration

Way forward: - Option 1: Only 1. - Option 2: 1 and 2.

Based on the discussion on clause 2.1.4.4, we propose to only consider CORESET duration 1 for FR2-1 PDCCH.

For FR2-1 PDCCH, only consider CORESET duration 1.

Aggregation level

Way forward: - Option 1: 4. - Option 2: - 2 and 4 for 1Tx - 8 and 16 for 2Tx

Based on the discussion on clause 2.1.4.4, we propose to only consider aggregation level 4 for FR2-1 PDCCH.

For FR2-1 PDCCH, only consider aggregation level 4.

Test case

Way forward: - 1Tx2Rx - Option 1: All tests in Table 7.3.2.2.1-1. - Option 2: Test 1- 2 in Table 7.3.2.2.1-1. - Other options are not precluded. - 2Tx2Rx - Option 1: All tests in Table 7.3.2.2.2-1. - Other options are not precluded

For FR2-1 PDCCH, there is only one case with higher Doppler can be reused from all the legacy UE PDCCH requirements, i.e. Test 1-2 in Table 7.3.2.2.1-1 of TS 38.101-4, as following.

Table 7.3.2.2.1-1: Minimum performance requirements with 120 kHz SCS for FR2-1 Test number; Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Reference Channel; Propagation Condition; Antenna configuration and correlation Matrix; Reference value. Pm-dsg (%); SNRBB (dB). 1-1; 100; 60; 1; 2; R.PDCCH. 5-1.1 TDD; TDLA30-75; 1x2 Low; 1; 6.4. 1-2; 100; 60; 1; 4; R.PDCCH. 5-1.2 TDD; TDLA30-300; 1x2 Low; 1; 3.0. Table 7.3.2.2.2-1: Minimum performance requirements with 120 kHz SCS for FR2-1 Test number; Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Reference Channel; Propagation Condition; Antenna configuration and correlation Matrix; Reference value. Pm-dsg (%); SNRBB (dB). 2-1; 100; 60; 1; 8; R.PDCCH. 5-1.3 TDD; TDLA30-75; 2x2 Low; 1; 0.1. 2-2; 100; 60; 2; 16; R.PDCCH. 5-2.1 TDD; TDLA30-75; 2x2 Low; 1; -3.0..

For FR2-1 PDCCH, select Test 1-2 in Table 7.3.2.2.1-1 of TS 38.101-4.

Test case selection for CQI reporting from 38.101-4

Way forward: - Wideband CQI reporting - Conducted: - Option 1: Section 6.2.3.2.2.1. - Option 2: Section 6.2.3.2.2. - Other options are not precluded. - Radiated: - Option 1: Section 8.2.2.2.2 for FR2-1. - Other options are not precluded. - Sub-band CQI reporting - Conducted: - Option 1: Section 6.2.3.2.2.2. - Option 2: Section 6.2.3.2.2. - Other options are not precluded.

For CQI reporting, we confirm to use the same configuration as legacy UE cases, i.e. section 6.2.3.2.2.1 for FR1 wideband conducted test, section 6.2.3.2.2.2 for FR1 sub-band conducted test, section 6.2.2.2.2.1 for FR1 wideband radiated test, section 6.2.2.2.2.2 for FR1 sub-band radiated test and section 8.2.2.2.2.1 for FR2-1 wideband radiated test.

For CQI reporting, select section 6.2.3.2.2.1 for FR1 wideband conducted test, section 6.2.3.2.2.2 for FR1 sub-band conducted test, section 6.2.2.2.2.1 for FR1 wideband radiated test, section 6.2.2.2.2.2 for FR1 sub-band radiated test and section 8.2.2.2.2.1 for FR2-1 wideband radiated test.

PBCH demodulation requirements selection from 38.101-4

SSB block index configuration

Way forward: - Option 1: both known and unknown. - Option 2: Other options are not precluded.

For SSB block index configuration, we confirm to use the same configuration as legacy UE cases, i.e. both known and unknown.

For PBCH, select both known and unknown for SSB block index configuration.

Antenna configuration

Way forward: - Conducted: - Option 1: Only 4Rx. - Option 2: Other options are not precluded. - Radiated - Option 1: 2Rx.

For antenna configuration for PBCH, the same configuration as other channels can be selected, i.e. only 4Rx for conducted test and 2Rx for radiated test.

For PBCH, select only 4Rx for conducted test and 2Rx for radiated test.

Impacted specifications

Way forward: - Option 1: Specify new mIAB performance requirements in TS 38.174 and conducted/radiated conformance testing in TS 38.176-1/2.

For the impacted specifications, we confirm to specify new mIAB performance requirements in TS 38.174 and conducted/radiated conformance testing in TS 38.176-1/2. However, for PBCH, conducted/radiated conformance testing in TS 38.176-1/2 should not be specified since PBCH is not testable.

Specify new mIAB performance requirements in TS 38.174 and conducted/radiated conformance testing in TS 38.176-1/2.

For PBCH, conducted/radiated conformance testing in TS 38.176-1/2 should not be specified since PBCH is not testable.

How to add mIAB-MT requirement to current specification

Way forward: - Option 1: Add a new section to state that demodulation performance requirements for mIAB-MT - Option 1a: New section index with suffix (e.g., Suffix A). - Option 1b: New section name includes “mobile IAB”.

Based on clause 4.12 of existing TS 38.174, clause suffix B should be used for the new selected cases for Mobile IAB-node.

- 4.12 Specification suffix information Unless stated otherwise, the suffix shown in Table 4.12-1 is used for indicating the clause for mobile-IAB node. Table 4.12-1: Definition of suffixes Clause suffix; Variant. B; Mobile IAB-node. An IAB-node which supports the mobile feature needs to meet both the general requirements of local area IAB-MT and the additional requirement applicable to the additional clause (suffixes B) in clauses 5, 6, 7,8,9,10,11 and 12. Where there is a difference in requirement between the general requirements and the additional clause requirements (suffixes B) in clauses 5, 6, 7,8,9,10,11 and 12, the tighter requirements are applicable unless stated otherwise in the additional clause.

Add a new section with suffix B for the new selected cases for Mobile IAB-node.

Applicability rule for mIAB-MT requirement

Way forward: - Option 1: Specify applicability rules for mIAB-MT performance requirements to define clearly which IAB-MT test are applicable since some of legacy IAB_MT test shall be reused.

As discussed in clause 2.1.8, in current IAB specification, there is a general applicability rule that UE should pass both legacy IAB cases and the new selected cases for mobile IAB. From our understanding, there is only one extra applicability rule for mIAB-MT need to be defined, to capture which legacy cases should be replaced by new selected cases as discussed in clause 2.1.1.1.

In current IAB specification, there is a general applicability rule that UE should pass both legacy IAB cases and the new selected cases for mobile IAB.

Define only one extra applicability rule for mIAB-MT, to capture which legacy cases should be replaced by new selected cases.

Proposals

In this contribution, we discuss on demodulation performance requirements for mobile IAB. Our observations and proposals are:

- For FR1 PDSCH, the test 1-2 in Table 8.2.2.1.2-1 of TS38.174 and the second case in Table 11.2.2.1.1.2-1 of TS38.174 should be replaced.

For FR1 PDSCH, select 7D1S2U, S=6D:4G:4U as default TDD UL-DL pattern and the same requirements are applicable to TDD with different UL-DL patterns.

- For FR1 PDSCH conducted test, the test 1-1 and test 1-8 are corresponding to the same case.

For FR1 PDSCH conducted test, select test 1-8(1-1) and 1-4 in table 5.2.3.2.1-3.

For FR1 PDSCH radiated test, select test 1-8(1-1) and 1-4 in table 5.2.2.2.1-3.

For FR2-1 PDSCH, do not consider rank 2.

For FR2-1 PDSCH, only consider 64QAM with TDLA30-300 and do not consider other modulations.

For FR2-1 PDSCH, select 3D1S1U, S=10D:2G:2U as default TDD UL-DL pattern and the same requirements are applicable to TDD with different UL-DL patterns.

For FR2-1 PDSCH, only consider test 1-3 in Table 7.2.2.2.1-3 of TS 38.101-4.

For FR1 PDCCH conduct test, consider only 1Tx4Rx.

For FR1 PDCCH radiated test, consider only 1Tx2Rx.

For FR1 PDCCH, consider only CORESET duration 1.

For FR1 PDCCH, consider only aggregation level 4 for 1Tx.

For FR1 PDCCH conducted test, select only Test 2 in Table 5.3.3.2.1-1 of TS 38.101-4.

For FR1 PDCCH radiated test, select only Test 2 in Table 5.3.2.2.1-1 of TS 38.101-4.

For FR2-1 PDCCH, only consider 1Tx2Rx.

For FR2-1 PDCCH, only consider CORESET duration 1.

For FR2-1 PDCCH, only consider aggregation level 4.

For FR2-1 PDCCH, select Test 1-2 in Table 7.3.2.2.1-1 of TS 38.101-4.

For CQI reporting, select section 6.2.3.2.2.1 for FR1 wideband conducted test, section 6.2.3.2.2.2 for FR1 sub-band conducted test, section 6.2.2.2.2.1 for FR1 wideband radiated test, section 6.2.2.2.2.2 for FR1 sub-band radiated test and section 8.2.2.2.2.1 for FR2-1 wideband radiated test.

For PBCH, select both known and unknown for SSB block index configuration.

For PBCH, select only 4Rx for conducted test and 2Rx for radiated test.

Specify new mIAB performance requirements in TS 38.174 and conducted/radiated conformance testing in TS 38.176-1/2.

For PBCH, conducted/radiated conformance testing in TS 38.176-1/2 should not be specified since PBCH is not testable.

Add a new section with suffix B for the new selected cases for Mobile IAB-node.

In current IAB specification, there is a general applicability rule that UE should pass both legacy IAB cases and the new selected cases for mobile IAB.

Define only one extra applicability rule for mIAB-MT, to capture which legacy cases should be replaced by new selected cases.

Reference

R4-2402871, WF for [110][326] NR_mobile_IAB_demod, RAN4#110, Ericsson