# 3GPP TSG-RAN WG4 Meeting #110-bis                                 R4-2405728

# Changsha, China, 15 – 19 April, 2024

Agenda item:	9.3.2

Source: 	CableLabs, Charter Communications, Cox Communications

Title: 	Technical parameters for the 4400 to 4800 MHz IMT band for sharing studies

Document for:	Discussion

# 1. Introduction

The 2023 World Radio Communication Conference (WRC-23, 20 November to 15 December 2023) approved agenda item 1.7 for the 2027 World Radio Communication Conference (WRC-27, Administrative Circular CA/270). ITU-R Working Party (WP) 5D is invited to conduct and compare the sharing and compatibility studies to ensure the protection of services to which the frequency band is allocated on a primary basis for the frequencies:

–	4 400 – 4 800 MHz;

–	7 125 – 8 400 MHz; and

–	14.8 – 15.35 GHz.

ITU-R WP5D sent an LS [1] to RAN4 asking for technical parameters. RAN #103 approved a SI [2] to prepare the requested technical parameters for ITU. In this contribution, we discuss our considerations in the 4400 – 4800 MHz band.

- Discussion

All the 3GPP-defined bands above 2.69 GHz are TDD bands. The existing band (e.g., band n79) near/overlaps the 4400 – 4800 MHz band is a TDD band. The duplex method for the 4400 – 4800 MHz should be TDD. The sharing studies should focus on synchronized TDD, which means all the base stations (BSs) intra-network and inter-network use the same TDD configuration and align the frame starting point with a less than 3 microseconds error that meets the requirement in TS 38.133. Therefore, neither downlink-to-uplink (DL-to-UL, or BS-to-BS) nor UL-to-DL (UE-to-UE) co-channel interference (CCI) or adjacent-channel interference (ACI) is present.

Sub-band full duplex (SBFD) WI is still being worked on in Rel-19, the technical parameters and performance are still unknown. SBFD should be excluded from the LS to ITU WP5D.

Proposal 1: The duplex method for the 4400 – 4800 band should focus on synchronized TDD. FDD and SBFD should be excluded.

Many operators are considering and deploying dual-layer networks: high-power tall macro cells for coverage and low-power microcells for enhancing capacity. The 4400 – 4800 MHz band could be considered for both macro and micro cells, and more likely be used as micro cells. Outdoor pole-mount or strand-mount micro small-cell BSs and indoor BSs typically use non-AAS or a small-size array due to the cost and physical dimension limitations. Non-AAS parameters are of interest for the 4400 – 4800 MHz band, especially for microcells.

Proposal 2: Non-AAS and AAS with small array sizes (e.g., 2×2 or 2×4) parameters are of interest for the 4400 – 4800 MHz band, especially for medium-range and local-area microcells.

The key technical parameters in the 4400 – 4800 MHz band are proposed in Tables 1 and 2.

Table 1. IMT-2023 specification related parameters in 4400 – 4800 MHz.

| No. | Parameter | Base station (non-AAS) | Base station (AAS) | Mobile station |
| --- | --- | --- | --- | --- |
| 1 | Duplex Method | Synchronized TDD | Synchronized TDD | Synchronized TDD |
| 2 | Channel bandwidth (MHz) | 100 MHz (typical) | 100 MHz (typical) | 100 MHz (typical) |
| 3 | Signal bandwidth (MHz) | Refer to 38.104 section 5.3.2 | Refer to 38.104 section 5.3.2 | Refer to 38.101-1 section 5.3.2 |
| 4 | Transmitter characteristics |  |  |  |
| 4.1 | Power dynamic range (dB) | 0 dB (no DL power control considered) | 0 dB (no DL power control considered) | 50 dB (-33 to 23 dBm based on UL power control) |
| 4.2 | Spectral mask (dB) | Refer to 38.104 section 6.6.4.2 | Refer to 38.104 section 6.6.4.2 | Refer to 38.101-1 section 6.5.2.2 |
| 4.3 | ACLR (dB) | 45 | 45 | 30 |
| 4.4 | Spurious emissions /out of band emissions | Refer to 38.104 section 6.6.5 | Refer to 38.104 section 6.6.5 | Refer to 38.101-1 section 6.5.3 |
| 4.5 | Maximum output power (dBm) |  |  | 23 dBm |
| 5 | Receiver characteristics |  |  |  |
| 5.1 | Noise figure (dB) | See note 1, F=5 dB for LA, 10 dB for MR, and 13 dB for WA BS | See note 1, F=5 dB for LA, 10 dB for MR, and 13 dB for WA BS | 9 dB |
| 5.2 | Sensitivity (dBm) | Refer to 38.104 section 7.2.2 | Refer to 38.104 section 7.2.2 | Refer to 38.101-1 section 7.3.2 |
| 5.3 | Blocking response | Note 1 | Note 1 | N/A |
| 5.4 | ACS | 46 dBc | 46 dBc | 33 dBc |
| 5.5 | SINR operating range (dB) | -10 to 30 | -10 to 30 | -10 to 30 |

Note 1: the BS noise figure follows the receiver blocking model described in TR 38.858 Annex E.

Table 2. Beamforming antenna characteristics for IMt-2030 in 4400 – 4800 MHz.

|  |  | Rural macro (If it’s available) | Suburban macro | Urban macro | Urban small cell (outdoor)/Micro cell | Indoor (small cell) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Base station antenna characteristics |  |  |  |  |  |
| 1.1 | Antenna pattern |  |  |  |  |  |
| 1.2 | Element gain (dBi) |  |  |  | 5 | 5 |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) |  |  |  |  |  |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) |  |  |  |  |  |
| 1.5 | Antenna polarization |  |  |  |  |  |
| 1.6 | Antenna array configuration (Row × Column) |  |  |  | 2×2 2×4 | 2×2 2×4 |
| 1.7 | Horizontal/Vertical radiating element/sub-array spacing, dh /dv |  |  |  |  |  |
| 1.7a | Number of element rows in sub-array, Msub |  |  |  |  |  |
| 1.7b | Vertical radiating element spacing in sub-array, dv,sub |  |  |  |  |  |
| 1.7c | Pre-set sub-array down-tilt, θsubtilt (degrees) |  |  |  |  |  |
| 1.8 | Array Ohmic loss (dB) |  |  |  |  |  |
| 1.9 | Conducted power (before Ohmic loss) per antenna element/sub-array (dBm) |  |  |  |  |  |
| 1.10 | Base station horizontal coverage range (degrees) |  |  |  |  |  |
| 1.11 | Base station vertical coverage range (degrees) |  |  |  |  |  |
| 1.12 | Mechanical downtilt (degrees) |  |  |  |  |  |
| 1.13 | Maximum base station output power/sector (e.i.r.p.) (dBm) |  |  |  |  |  |

# 3. Conclusions

In this contribution, terrestrial IMT parameters for new candidate IMT operation bands 4400 – 4800 MHz are discussed with the following proposals.

Proposal 1: The duplex method for the 4400 – 4800 band should focus on synchronized TDD. FDD and SBFD should be excluded.

Proposal 2: Non-AAS and AAS with small array sizes (e.g., 2×2 or 2×4) parameters are of interest for the 4400 – 4800 MHz band, especially for medium-range and local-area microcells.

# 4. Reference

- R4-2400333, Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz, ITU-R Working Party 5D.

- RP-240765, New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz, Ericsson.