3GPP TSG-RAN4 Meeting #110bis	R4-240xx

Changsha, China, 15 – 19 April, 2024

Title:	Capabilities discussion

Source:	Qualcomm Incorporated

Agenda item:	6.13.1

Release:	Rel-18

WI: 	NR_MC_enh-Core

Lead WG: 	RAN1

Document for:	Approval

# 1.	Introduction

RAN4 has received an LS [1] from RAN2 regarding a parallel switching of four bands. RAN2 is asking if RAN4 is ok with their design. In this paper we indicate our view to their question.

# 2. 	Discussion

RAN4 agreement was originally conveyed to RAN2 with an LS [2] and its attachment. The design of this feature that involved an indication switching period of min {max(Tswitch_A-C, Tswitch_B-D), max(Tswitch_A-D, Tswitch_B-C)} . and also an ability for the UE to detect the order need for switching based on scheduled transmissions. It was voiced then that his may need RAN1 involvement since the scheduling is RAN1 domain and the LS was sent to RAN1. Based on LS reply [1] it seems RAN2 dis not adopt the indication of preferred band order capability. It is unclear why this is so, it was also voiced that the LS is complicated and hard to understand but more likely it is because this feature would have RAN1 impact and RAN1 WI is closed.

Our view is that the current RAN2 design is sufficient and I tis sufficient to indicate the switching period to be the one:

min {max(Tswitch_A-C, Tswitch_B-D), max(Tswitch_A-D, Tswitch_B-C)}

Proposal: RAN4 to reply to RAN2 that RAN4 has no concerns on the adopted approach.

# Conclusion

We discussed made one proposal:

Proposal: RAN4 to reply to RAN2 that RAN4 has no concerns on the adopted approach.

# Reference

[1]	R4-2404116 (R2-2401969), “Reply LS on Rel-18 UL Tx switching for parallel switching on four bands”, RAN2, 3GPP TSG-RAN WG4 Meeting #110-bis, Changsha, China, 15 – 19 April, 2024

[2]	R4-2317609, “LS on Rel-18 UL Tx switching for parallel switching on four bands”, 3GPP TSG RAN WG4 Meeting #108-bis, October 9 – October 13, 2023 in Xiamen, China