3GPP TSG-RAN WG4 Meeting #110bis	                                           R4-2406114

Changsha, China, 15th – 19th April, 2024

Agenda item:			6.11.4

Source:	China Telcom

Title:	WF on the advanced receiver for MU-MIMO scenario

Document for:	Approval

Introduction

This document provides the way forward on advanced receiver for MU-MIMO scenario of Rel-18 NR demodulation requirement evolution WI.

The previous approved WFs are listed as below:

  - R4-2302929 - WF on advanced receiver for MU-MIMO scenario, China Telecom, RAN4#106

  - R4-2305914 - WF on advanced receiver for MU-MIMO scenario, China Telecom, RAN4#106bis-e

  - R4-2309892 - WF on advanced receiver for MU-MIMO scenario, China Telecom, RAN4#107

  - R4-2313993 - WF on advanced receiver for MU-MIMO scenario, China Telecom, RAN4#108

  - R4-2316915 - WF on advanced receiver for MU-MIMO scenario, China Telecom, RAN4#108bis

  - R4-2321114 - WF on advanced receiver for MU-MIMO scenario, China Telecom, Apple, RAN4#109

  - R4-2402864 - WF on advanced receiver for MU-MIMO scenario, China Telecom, RAN4#110

<Topic #1: Test parameters for the advanced receiver for MU-MIMO>

Test setting for when UE is indicated Modulation order (DCI index 1-5 is indicated)

- Agreement:

  - For Rank 1+1 with 2T2R

    - Case2: Orthogonal precoding, TDLC300-100, ULA medium, MCS 13 (Table 1) for Target UE, QPSK for co-UE. full FDRA for the co-UE

  - For Rank 2+2 with 4T4R:

    - Case7: Orthogonal precoding, TDLA30-10, ULA Low, MCS 17 (Table 1) for Target UE, 16QAM for co-UE. full FDRA for the co-UE

  - For Rank 1+1 with 2T4R:

    - Orthogonal precoding, TDLC300-100, ULA medium, MCS 13 (Table 1) for Target UE, QPSK for co-UE. full FDRA for the co-UE

Test setting for when UE is not indicated Modulation order (DCI index 6 is indicated)

- For Rank 1+1 with 2T2R, down-select among the following cases:

  - Orthogonal precoding, TDLC300-100, ULA low, MCS 13 (Table 1) for Target UE, QPSK for co-UE, full FDRA for the co-UE

  - Case26: Orthogonal precoding, TDLC300-100, ULA medium, MCS 17 (Table 1) for Target UE, 16QAM for co-UE, full FDRA for the co-UE

  - Case 20: Orthogonal precoding, TDLC300-100, ULA medium, MCS 13 (Table 1) for Target UE, QPSK for co-UE. full FDRA for the co-UE (as priority for requirement definition)

  - Companies are encouraged to bring simulation results for all cases above

- For Rank 2+2 with 4T4R:

  - Option 1: Introduce rank 2+2 4T4R requirements with modulation order blind detection

    - Option 1A (Case 32): Orthogonal precoding, TDLA30-10, XP medium, MCS 13 (Table 1) for Target UE, QPSK for co-UE, full FDRA for the co-UE

    - Option 1B (Case 31): Orthogonal precoding, TDLA30-10, ULA Low, MCS 13 (Table 1) for Target UE, QPSK for co-UE, full FDRA for the co-UE

  - Option 2: Do not introduce rank 2+2 4T4R requirements with modulation order blind detection

- For Rank 1+1 with 2T4R, if introduced, down-select among the following test cases:

  - Orthogonal precoding, TDLC300-100, ULA low, MCS 13 (Table 1) for Target UE, QPSK for co-UE, full FDRA for the co-UE

  - Case 29: Orthogonal precoding, TDLC300-100, ULA medium, MCS 17 (Table 1) for Target UE, 16QAM for co-UE, full FDRA for the co-UE

  - Case 23: Orthogonal precoding, TDLC300-100, ULA medium, MCS 13 (Table 1) for Target UE, QPSK for co-UE. full FDRA for the co-UE

  - Companies are encouraged to bring simulation results for all cases above

Whether to tests UE not support BD-MO with R-ML with DCI index 6 is indicated

- Agreement:

  - Do not introduce such test for UE not support BD-MO with R-ML

Whether new R-ML requirements should be captured in new clauses or in the existing clauses for MMSE-IRC under intra-cell inter-user scenario

- Agreement:

  - R-ML new requirements are introduced in the existing clauses for MMSE-IRC under intra-cell inter-user scenario.

    - R-ML receiver not intended to be release independent.

  - Update 38.307 Rel-17 and Rel-18 in Table B.3.3-1 state which tests are release independent from Rel-15.

RRC assistant information configuration on the MCS table

- For UEs not supporting modulation order blind detection:

  - Option 1: No need for the network to inform such information to the UE

  - Option 2: Signalled regardless of whether the UE supports MO BD

    - Option 2A: 256QAM MCS Table

    - Option 2B: 64QAM MCS table

- For UEs supporting modulation order blind detection:

  - Agreement:

    - 256QAM MCS Table

DCI assistant signalling configuration for tests with modulation order blind detection

- Agreement:

  - For Rank 1+1:

    - DCI signalling should be index 6.

  - For Rank 2+2 (if introduced):

    - DCI signalling should be index 6.

For UE supporting MO BD, whether to introduce applicability rule to skip test(s) with modulation order indicated

- Candidate options:

  - Option 1: Introduce applicability rule to skip tests with modulation order indicated for UEs capable of BD MO

  - Option 2: Do not introduce applicable rule skip tests with modulation order indicated

<Topic #2: UE capability aspects >

Updates to UE capability definition

- Agreement:

  - For 36-1, update the note in capability granularity column as below:

  - UE supports R-ML on MU-MIMO on single carrier operation. UE optionally supports R-ML on MU-MIMO on one or more carriers in CA, NE-DC, EN-DC and NR-DC operation

  - Update the ‘Components’ column as below:

    - For 36-1:

  - R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression, for MU-MIMO up to maxNumberMIMO-LayersPDSCH layers across target and co-scheduled UEs with 2 RX and 4RX antennas, when co-scheduled UE(s)’ modulation order is explicitly signalled by DCI index 1-5 in Table 7.3.1.2.2-12 of TS38.212.

    - For 36-2a:

  - R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO [for 2 layers across target and co-scheduled UEs with 2RX and 4RX] when co-scheduled UE(s)’ modulation order is not signalled DCI index 6 or 7 in Table 7.3.1.2.2-12 of TS38.212 is signalled.

    - For 36-2b:

  - R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO [for 2 layers across target and co-scheduled UEs with 2RX and maxNumberMIMO-LayersPDSCH layers across target and co-scheduled UEs with 4RX] when co-scheduled UE(s)’ modulation order is not signalled DCI index 6 in Table 7.3.1.2.2-12 of TS38.212 is signalled.

- Proposals on 36-2a and 36-2b:

  - Proposal 1: Combine 36-2a and 36-2b and remove number of layer descriptions if RAN4 agrees to not define 2+2 test under DCI 6

  - Proposal 2: Remove FG 36-2b and keep 36-2a from UE feature list