3GPP TSG-RAN WG4 Meeting #110-bis											             R4-2406385

Changsha, China, April 15 – April 19, 2024

Agenda item:			6.12.4

Source:	Huawei, HiSilicon

Title:	WF on LPHAP

Document for:	Approval

- Sub-Topic #1: Core requirements maintenance

Issue 1-1-1: Clarification for autonomous TA adjustment

- Agreement (based on CR discussion)

  - Clarify that UL timing in the new camping cell is

TDL_new - TAadjusted, where TAadjusted = TAold + 2*(TDL_new - TDL_old)

TAadjusted is the TA after adjustment in the new camping cell,

TAold is the TA applied in previous camping cell,

TDL_new is the DL timing of the new camping cell immediately after cell reselection, and

TDL_old is the DL timing of the previous camping cell immediately before cell reselection

# Sub-Topic #2: Performance requirements

Issue 1-2-1: whether to define accuracy TCs for LPHAP

Agreements (from Wed adhoc session)

- No new accuracy TCs will be defined for RRC_INACTIVE (legacy TCs can be used).

- TCs will be defined for RRC_IDLE (new sections will be created) by referring to the existing RRC_INACTIVE TCs.

- To avoid/reduce multiple testing, test applicability is to be further discussed (applies for all positioning enhancements in Rel-18, i.e., not limited to LPHAP only).

Issue 1-2-2: combination of measurement types and RRC states

Agreements (from Wed adhoc session)

- Define delay TCs for all applicable measurement types for:

  - RRC INACTIVE (RSTD, UE Rx-Tx, RSRP, RSRPP)

  - RRC_IDLE (RSTD, RSRP, RSRPP).

Issue 1-2-3: eDRX configurations

Agreements (from Thu adhoc session)

- Define TCs for eDRX>10.24s for case 2 only, to reduce the number of test cases.

Issue 1-2-4: measurement sample number to be tested

Agreements (based on TC list for work split)

- Test cases are defined for Nsample = 4.

Issue 1-2-5: whether to define TCs for UL timing and TA adjustment

Agreements (based on TC list for work split)

- Do not define TCs for UL timing and TA adjustment

Issue 1-2-6: whether to define TCs for TA validation

Agreements (based on TC list for work split)

- Do not define TCs for TA validation

Issue 1-2-7: UE types to be tested

Agreements (from Wed adhoc session with update based on TC list for work split)

- Define test cases for both normal (non-RedCap) and RedCap UEs

  - The test cases are defined in separate sections (aligned principle with RedCap positioning discussion in thread [214])