3GPP TSG-RAN4 Meeting #111	R4-2407686

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2244 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of parameter name for dpc-Reporting-FR1 for range 1 SA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_cov_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To correct UE capability name for dpc-Reporting-FR1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | “∆PPowerClass reporting capability XXX-r18” is replaced with “deltaPowerClassReporting-r18”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE capability name for dpc-Reporting-FR1 remains incorrect or imcomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4, 6.2A.4.1.1, 6.2A.4.1.2, 6.2A.4.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

### 6.2.4	Configured transmitted power

The UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

PCMAX_H,f,c = MIN {PEMAX,c,  PPowerClass – ΔPPowerClass + ΔPPowerBoost}

where

PEMAX,c is the value given by either the p-Max IE or the field additionalPmax of the NR-NS-PmaxList IE, whichever is applicable according to TS 38.331[7];

PPowerClass is the maximum UE power specified in Table 6.2.1-1 and in Table 6.2F.1-1 for shared spectrum access operation, without taking into account the tolerance specified in the Table 6.2.1-1 and in Table 6.2F.1-1 for shared spectrum access operation;

When  the IE powerBoostPi2BPSK is set to 1, PEMAX,c is increased by +3 dB for a power class 3 UE operating in TDD bands n40, n41, n77, n78, and n79 with PI/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and 40% or less symbols in certain evaluation period are used for UL transmission when PEMAX,c ≥ 20 dBm (The exact evaluation period is no less than one radio frame).

When the IE powerBoostPi2BPSK is set to 1, ΔPPowerClass = -3 dB for a power class 3 UE operating in TDD bands n40, n41, n77, n78, and n79 with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and 40% or less slots in radio frame are used for UL transmission.

ΔPPowerClass =

-	3 dB for a power class 2 UE or 6 dB for a power class 1.5 UE when P-max of 23 dBm or lower is indicated; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50%; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or when the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and half the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame).

-	3 dB for a power class 1.5 UE when P-max of between 23 dBm and 26 dB is indicated; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is between 25% and 50%; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is between maxUplinkDutyCycle-PC2-FR1 and maxUplinkDutyCycle-PC2-FR1/2 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or when the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame).

-	3dB when the UE is configured with SUL configurations and the requirements of default power class are applied as specified in sub-clause 6.2C.1 on the band where UE indicates power class 2;

-	3dB is applied during SRS transmission occasions with usage in SRS-ResourceSet set as ‘antennaSwitching’ with configured SRS resources in each SRS resource set(s) consisting of one SRS port when PC2 UE with txDiversity-r16 or txDiversity2Tx-r18 capability or PC1.5 UE further indicates SRS-TxSwitch capability ‘t1r2’ or ‘t1r4’ or ‘t1r1-t1r2’ or ‘t1r1-t1r2-t1r4’ or further indicates srs-AntennaSwitchingBeyond4RX-r17 as ‘t1r8’;

-	0 dB otherwise;

NOTE: 	UE reports ∆PPowerClass when deltaPowerClassReporting-r18 is present, dpc-Reporting-FR1 [7] is configured and the reporting is triggered only by uplink duty cycle exceedance or by return to the ue-PowerClass after the duty cycle exceedance.

∆TIB,c is the additional tolerance for serving cell c as specified in clause 6.2A.4.2 for NR CA, clause 6.2C.2 for SUL, or TS 38.101-3 clause  6.2B.4.2 for EN-DC; ∆TIB,c = 0 dB otherwise; In case the UE supports more than one of band combinations for V2X operating bands for concurrent operation, CA, SUL or DC, and an operating band belongs to more than one band combinations then

<Unnecessary part is omitted>

<Start of next changes>

##### 6.2A.4.1.1	Configured transmitted power for Intra-band contiguous CA

For uplink carrier aggregation the UE is allowed to set its configured maximum output power PCMAX,c for serving cell c and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c  on serving cell c shall be set as specified in clause 6.2.4, but with MPRc = MPR and A-MPRc = A-MPR with MPR and A-MPR as determined by subclause 6.2A.2 and 6.2A.3, respectively. For PH reporting the following exception applies: if the UE is configured with multiple uplink serving cells, the power PCMAX,c  used for the purpose of PH reporting on first serving cell c = c1 does not consider for computation of the PH report transmissions on a second serving cell c2 as exempted  in subclause 7.7.1 in [8]. There is one power management term for the UE, denoted P-MPR, and P-MPR c = P-MPR.

The total configured maximum output power PCMAX shall be set within the following bounds:

PCMAX_L ≤ PCMAX ≤ PCMAX_H

For uplink intra-band contiguous carrier aggregation when same slot pattern is used in all aggregated serving cells,

PCMAX_L  = MIN{10 log10 ∑ pEMAX,c  - TC , PEMAX,CA,(PPowerClass,CA– ΔPPowerClass,CA) – MAX(MAX(MPR, A-MPR) + ΔTIB,c + TC + TRxSRS, P-MPRc ) }

PCMAX_H  = MIN{10 log10 ∑ pEMAX,c , PEMAX,CA ,PPowerClass,CA– ΔPPowerClass,CA }

where

-	pEMAX,c is the linear value of PEMAX,c which is given by IE P-Max for serving cell c in [7];

PPowerClass,CA is the maximum UE power specified in Table 6.2A.1.1-1 without taking into account the tolerance;

-	MPR and A-MPR are specified in clause 6.2A.2 and 6.2A.3, respectively;

-	ΔPPowerClass,CA = 3 dB for a power class 2 UE when the requirements of default power class are applied as specified in sub-clause 6.2.A.1.1; otherwise ΔPPowerClass,CA = 0 dB;

NOTE: 	UE reports ∆PPowerClass,CA when deltaPowerClassReporting-r18 is present, dpc-Reporting-FR1 [7] is configured and the reporting is triggered only by uplink duty cycle exceedance or by return to the powerClass after the duty cycle exceedance.

<Unnecessary part is omitted>

<Start of next changes>

##### 6.2A.4.1.2	Configured transmitted power for Intra-band non-contiguous CA

For uplink carrier aggregation the UE is allowed to set its configured maximum output power PCMAX,c for serving cell c and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c  on serving cell c shall be set as specified in subclause 6.2.4.

The configured maximum output power PCMAX,c  on serving cell c shall be set as specified in subclause 6.2.4, but with MPRc = MPR and A-MPRc = A-MPR with MPR and A-MPR as determined by subclause 6.2A.2 and 6.2A.3, respectively. For PH reporting the following exception applies: if the UE is configured with multiple uplink serving cells, the power PCMAX,c  used for the purpose of PH reporting on first serving cell c = c1 does not consider for computation of the PH report transmissions on a second serving cell c2 as exempted  in subclause 7.7.1 in [8]. There is one power management term for the UE, denoted P-MPR, and P-MPR c = P-MPR.

The total configured maximum output power PCMAX shall be set within the following bounds:

PCMAX_L ≤ PCMAX ≤ PCMAX_H

For uplink intra-band non-contiguous carrier aggregation when same slot pattern is used in all aggregated serving cells,

PCMAX_L  = MIN{10 log10 ∑ pEMAX,c  - TC , PEMAX,CA,(PPowerClass,CA– ΔPPowerClass,CA) – MAX(MAX(MPRc, A-MPRc) + ΔTIB,c + TC + DTRxSRS, P-MPR ) }

PCMAX_H  = MIN{10 log10 ∑ pEMAX,c , PEMAX,CA ,PPowerClass,CA– ΔPPowerClass,CA)}

where

-	pEMAX,c is the linear value of PEMAX,c which is given by IE P-Max for serving cell c in [7];

-	PPowerClass,CA is the maximum UE power specified in Table 6.2A.1.2-1 without taking into account the tolerance;

-	MPR and A-MPR are specified in subclause 6.2A.2 and subclause 6.2A.3 respectively;

-	ΔPPowerClass,CA = 3 dB for a power class 2 UE when the requirements of default power class are applied as specified in sub-clause 6.2.A.1.2; otherwise ΔPPowerClass,CA = 0 dB;

NOTE:	UE reports ∆PPowerClass,CA when deltaPowerClassReporting-r18 is present, dpc-Reporting-FR1 [7] is configured and the reporting is triggered only by uplink duty cycle exceedance or by return to the powerClass after the duty cycle exceedance.

<Unnecessary part is omitted>

<Start of next changes>

##### 6.2A.4.1.3	Configured transmitted power for Inter-band CA

For uplink carrier aggregation the UE is allowed to set its configured maximum output power PCMAX,c for serving cell c and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c  on serving cell c shall be set as specified in clause 6.2.4, except that the UE power class for serving cell c on the specific operating band shall be determined by the ue-PowerClassPerBandPerBC-r17 IE [7] as indicated for the band combination if signalled.

For uplink inter-band carrier aggregation, MPRc and A-MPRc apply per serving cell c and are specified in clause 6.2.2 and clause 6.2.3, respectively. P-MPR c accounts for power management for serving cell c. PCMAX,c  is calculated under the assumption that the transmit power is increased independently on all component carriers.

The total configured maximum output power PCMAX shall be set within the following bounds:

PCMAX_L ≤ PCMAX ≤ PCMAX_H

For uplink inter-band carrier aggregation with one serving cell c per operating band when same slot symbol pattern is used in all aggregated serving cells,

PCMAX_L = MIN {10log10∑ MIN [ pEMAX,c/ (tC,c),  pPowerClass.c/(MAX(mprc·∆mprc, a-mprc)·tC,c ·tIB,c·tRxSRS,c) , pPowerClass,c/pmprc], PEMAX,CA, PPowerClass,CA-ΔPPowerClass, CA}

PCMAX_H = MIN{10 log10 ∑ pEMAX,c , PEMAX,CA, PPowerClass,CA-ΔPPowerClass, CA}

where

-	pEMAX,c is the linear value of PEMAX, c which is given by IE P-Max for serving cell c in [7];

-	PPowerClass,CA is the maximum UE power specified in Table 6.2A.1.3-1 without taking into account the tolerance specified in the Table 6.2A.1.3-1; If the UE indicates higherPowerLimit-r17 for an eligible CA configuration as specified in Table 6.2A.1.3-1 and ΔPPowerClass, CA = 0, PPowerClass,CA is replaced by 10 log10 ∑ pPowerClass,c.

-	pPowerClass,c is the linear value of the maximum UE power for serving cell c specified in Table 6.2.1-1 according to ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass otherwise without taking into account the tolerance;

-	ΔPPowerClass,CA = 3 dB for a power class 2 UE when the requirements of default power class are applied as specified in sub-clause 6.2.A.1.3; otherwise ΔPPowerClass, CA = 0 dB;

NOTE:	UE reports ∆PPowerClass,CA when deltaPowerClassReporting-r18 is present, dpc-Reporting-FR1 [7] is configured and the reporting is triggered only by uplink duty cycle exceedance or by return to the powerClass after the duty cycle exceedance.

<Unnecessary part is omitted>

<End of the changes>