3GPP RAN TSG-RAN4 Meeting # 111		R4-2407073

Fukuoka City, Fukuoka, Japan, May 20 – 24, 2024

Agenda item:	6.1.1.1

Source:	Apple

Title:	MSD Analysis for CA_n41C-n79A

WI/SI:	NR_CADC_R18_2BDL_xBUL-Core

Release:	Rel-18

Document for:	Discussion

# 1	Introduction and Scope

In the RAN4 Meeting #110, the CA band combination CA_n41C-n79A was introduced through a TP [1]. During the following meeting (RAN4 Meeting#110Bis, Changsha, China, April 15th – 19th, 2024) discussions were held on a proposed MSD value for that combination from one company [2]. As summarized in the WF shown below, more companies were invited to provide contributions for that MSD value in the upcoming meeting.

Sub-topic 1-4 CA_n41A-n79C and CA_n41C-n79A TRs Issue 1-4a: CA_n41C-n79A Agreement: - Note discussion papers and come back at next meeting. Companies are invited to bring MSD analysis.

As captured in the TP, it can be seen that n79DL will be subjected to intereferences caused by the 4th order and 6th  orther intermodulation products (IMDs) generated by the two contiguous carriers from UL CA_41C. In this contribution, we will present the results of our MSD analysis for n79DL due to IMD4 from UL CA_41C. The analysis is based on PC3 power level and the test point will be as shown in the Table below, taken from the TP.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n79 | n41 | 2545 | 60 | 1 (RBSTART= 0) | 2545 | N/A | TDD | N/A |
|  |  | 2625 | 100 | 1 (RBSTART= 272) | 2625 |  |  |  |
|  | n79 | N/A | 40 | N/A | 4872.5 | 3.115 | TDD | IMD4 |
| NOTE 15: This band is subject to IMD6 also which MSD is not specified |  |  |  |  |  |  |  |  |

# Discussion

As depicted in the diagram below, with UL CA_n41C, due to the non-linearities in the n41C UL, two CC’s can create an IIMD4 that could land in the n79 DL.

Figure 2.1: IIMD4 created by two CCs in n41C UL could land in n79A DL

For CA_n41C-n79A, the bandwitch class is as shown in Table 2-1 below:

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41C-n79A | CA_n41C-n79A | n41 | CA_n41C_BCS4 and 5 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |

Figure 2.1: CA_n41C-n79A BCS

## MSD analysis for CA_n41C-n79A

For the MSD analysis, we will consider the architecture shown in Figure 2.1-1 below for the case of UL CA_41C PC3 with 2 CCs. The  worst-case assumptions for the RF front-end component isolation and linearity performance are shown in Table 2.1-1.

Figure 2.1-1: Front-end architecture used for CA_n41C-n79A MSD Analysis

| Parameter | Value | Unit |
| --- | --- | --- |
| Antenna isolation | 10 | dB |
| Front-end loss | 4 | dB |
| Triplexer isolation | 20 | dB |
| n41filter rejection on n40 DL | 35 | dB |
| n40 filter TX to antenna rejection on to COM port on n41 DL | 30 | dB |
| sPnT switch IP3 | 70 | dBm |

Table 2.1-1: RF front-end rejection and linearity assumptions for CA_n41C-n79A

Given the assumptions highlighted above, the MSD analysis is summarized in Table 2.1-2.

| Parameter | Unit | Main Path | Div. Path |
| --- | --- | --- | --- |
| Total n41 PA output noise | dBm/CBW | -37 |  |
| Total n41 PA output noise at antenna port | dBm/CBW | -76 | -86 |
| n79 ideal REFSENS for 40MHz CBW | dBm/CBW | -95.8 | -95.8 |
| Thermal Noise | dBm/CBW | -91.8 | -91.8 |
| Composite noise | dBm/CBW | -75.9 | -84.9 |
| Composite noise after MRC | dBm | -83.4 |  |
| MSD | dB | 8.4 |  |

Table 2.1-2: MSD Analysis for CA_n41C_n79A

# 3	Conclusion

In this contribution, we shared the result of our CA_n41C-n79A MSD analysis due to IMD4.

Proposal: Adopt CA_n25A-n77A MSD Levels proposed in Table 3-1 shown below.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n79 | n41 | 2545 | 60 | 1 (RBSTART= 0) | 2545 | N/A | TDD | N/A |
|  |  | 2625 | 100 | 1 (RBSTART= 272) | 2625 |  |  |  |
|  | n79 | N/A | 40 | N/A | 4872.5 | 8.415 | TDD | IMD4 |
| NOTE 15: This band is subject to IMD6 also which MSD is not specified |  |  |  |  |  |  |  |  |

Table 3.1: IIMD4 MSD proposal for CA_n41C_n79A

# 4	References

R4-2401274, “TP for TR38.718-02-01_CA_n41A-n79C and CA_n41C-n79A”, ZTE Corporation, MediaTek Inc.

R4-2405445 “Considerations on CA_n41C-n79A”, Qualcomm France, 3GPP TSG-RAN WG4 Meeting #110Bis, Changsha, China, April 14th – 19th, 2024