3GPP TSG-RAN WG4 Meeting # 111		R4-2407187

# Fukuoka , JP, 20 May – 24 May 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4328 | rev | - | Current version: | 16.19.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_newRAT-Perf) CR on test for SCell activation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2024-05-20 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the SCell activation test, the UE shall start reporting CSI in PCell after at least one CSI-RS transmission occasion for channel measurement. However, the CSI report processing time (as defined in 5.2.2.5 in TS 38.214) was missed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add CSI report processing time, nCSI_ref slots, as defined in 5.2.2.5 in TS 38.214. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect test requirement, as the CSI report processing time is allowed for the UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.5.3.1, A.4.5.3.3, A.6.5.3.1, A.6.5.3.3, A.7.5.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of Change>

#### A.4.5.3.1	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle

##### A.4.5.3.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in clause 8.3, when the SCell in FR1 is known by the UE at the time of activation.

The supported test configurations for LTE PCell and NR PSCell are shown in table A.4.5.3.1.1-1 below. Supported test configurations for NR SCell are shown in table A.4.5.3.1.1-1A below. Test configuration for LTE PCell and NR PSCell and test configuration for NR SCell are chosen independently. The test parameters are given in Tables A.4.5.3.1.1-2 and cell-specific parameters in A.4.5.3.1.1-3 and A.4.5.3.1.1-4 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are three carriers, E-UTRA has one cell, NR has two cells. All cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on E-UTRA and Cell 2 (PSCell) on NR, but is not aware of Cell 3 (SCell) on NR. The UE is monitoring the PCell and PSCell. The UE shall be continuously scheduled in the PCell and PSCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 3) becomes configured on NR. The UE now starts monitoring the SCell. The test equipment sends a MAC message for activation of the SCell.

the start of time period T2. The UE shall be able to report valid CSI in PSCell for the activated SCell at latest in slot m+THARQ+Tactivation_time+TCSI_ReportingNR slot length, as defined in clause 8.3. The UE shall start reporting CSI in PSCell after at least one CSI-RS transmission occasion for channel measurement plus nCSI_ref  slots, as defined in 5.2.2.5 in [26], and reporting after slot (m+k) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PSCell interruption due to activation of SCell shall occur in the slot m+1+THARQNR slot length to slot m+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3, where Ninterruption is the interruption length given in clause 8.2. Any E-UTRA PCell interruption due to activation of SCell shall occur in the subframe m1+1+THARQEUTRA slot length to subframe m2+1+THARQ+3ms+TXEUTRA slot length+Ninterruption, where m1 and m2 are the index of the first and last subframe of E-UTRA PCell which overlaps with slot m, and Ninterruption is the interruption length given in TS 36.133 [14] clause 7.32.

Time period T3 starts when a MAC message for deactivation of SCell, sent from the test equipment to the UE in a slot # denoted n, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell in a slot n+THARQ+3msNR slot length, as defined in clause 8.3. The starting point of any PSCell interruption due to the deactivation shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+3msNR slot length, as defined in clause 8.3. The starting point of any E-UTRA PCell interruption due to the deactivation shall occur in the subframe n1+1+THARQEUTRA subframe length to subframe n2+1+THARQ+3msEUTRA subframe length, where n1 and n2 are the index of the first and last subframe of E-UTRA PCell which overlaps with slot n.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PSCell during activation and deactivation of SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the slots from the time when the SCell deactivation command is sent until CSI reporting for SCell is discontinued.

Table A.4.5.3.1.1-1: known FR1 SCell activation in non-DRX for 160ms SCell measurement cycle supported test configurations for LTE PCell and NR PSCell

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Table A.4.5.3.1.1-1A: known FR1 SCell activation in non-DRX for 160ms SCell measurement cycle supported test configurations for NR SCell

| ConfigSCell | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Table A.4.5.3.1.1-2: General test parameters for known FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2,3 | One E-UTRAN radio channel (1) and two NR radio channel (2,3) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on E-UTRAN RF channel number 1. As specified in clause A.3.7.2.1 |
| Active PSCell |  | Cell 2 | Primary secondary cell on NR RF channel number 2. |
| Configured deactivated SCell |  | Cell 3 | Configured deactivated secondary cell on NR RF channel number 3 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Cell-individual offset for cells on E-UTRA RF channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on secondary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell3 timing offset to cell2 | s | 0 |  |
| Time alignment error between cell3 and cell2 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 7 | During this time the PSCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| A3-offset | dB | -15 |  |
| THARQ | ms | k1NR slot length | k1 is a number of slots indicated by the PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ feedback timing field is not present in the DCI format, the value is defined in 38.213 [3] |
| TCSI_Reporting | ms | 15 | The delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing timefor CSI reporting (clause 5.2.2.5 in TS 38.214) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |
| k | slot |  | As specified in clause 4.3 of TS 38.213 [3] |

Table A.4.5.3.1.1-3: Cell specific test parameters for NR PSCell for known FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter |  | Unit | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| SSB ARFCN |  |  | freq1 |  |  |
| Duplex mode | Config 1,4 |  | FDD |  |  |
|  | Config 2,3,5,6 |  | TDD |  |  |
| TDD configuration | Config 1,4 |  | Not Applicable |  |  |
|  | Config 2,5 |  | TDDConf.1.1 |  |  |
|  | Config 3,6 |  | TDDConf.2.1 |  |  |
| BWchannel | Config 1,4 | MHz | Note 7 |  |  |
|  | Config 2,5 |  | Note 7 |  |  |
|  | Config 3,6 |  | Note 7 |  |  |
| BWoccupied | Config 1,4 | RB | 52 Note 5 |  |  |
|  | Config 2,5 |  | 52 Note 5 |  |  |
|  | Config 3,6 |  | 106 Note 6 |  |  |
| DL initial BWP configuration | Config 1, 2, 3, 4, 5, 6 |  | DLBWP.0.1 |  |  |
| DL dedicated BWP configuration | Config 1, 2, 3, 4, 5, 6 |  | DLBWP.1.1 |  |  |
| UL initial BWP configuration | Config 1, 2, 3, 4, 5, 6 |  | ULBWP.0.1 |  |  |
| UL dedicated BWP configuration | Config 1, 2, 3, 4, 5, 6 |  | ULBWP.1.1 |  |  |
| DRX Cycle |  | ms | Not Applicable |  |  |
| PDSCH Reference measurement channel | Config 1,4 |  | SR.1.1 FDD |  |  |
|  | Config 2,5 |  | SR.1.1 TDD |  |  |
|  | Config 3,6 |  | SR.2.1 TDD |  |  |
| RMSI CORESET Reference Channel | Config 1,4 |  | CR.1.1 FDD |  |  |
|  | Config 2,5 |  | CR.1.1 TDD |  |  |
|  | Config 3,6 |  | CR.2.1 TDD |  |  |
| RMC CORESET Reference Channel | Config 1,4 |  | CCR.1.1 FDD |  |  |
|  | Config 2,5 |  | CCR.1.1 TDD |  |  |
|  | Config 3,6 |  | CCR.2.1 TDD |  |  |
| TRS configuration | Config 1,4 |  | TRS.1.1 FDD |  |  |
|  | Config 2,5 |  | TRS.1.1 TDD |  |  |
|  | Config 3,6 |  | TRS.1.2 TDD |  |  |
| OCNG Patterns | Config 1,2,4,5 |  | OP.1 Note 5 |  |  |
|  | Config 3,6 |  | OP.1 Note 6 |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |
| SSB configuration | Config 1,2,4,5 |  | SSB.1 FR1 |  |  |
|  | Config 3,6 |  | SSB.2 FR1 |  |  |
| CSI-RS configuration for CSI reporting | Config 1,4 |  | CSI-RS.1.1 FDD |  |  |
|  | Config 2,5 |  | CSI-RS.1.1 TDD |  |  |
|  | Config 3,6 |  | CSI-RS.2.1 TDD |  |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2,4,5 | kHz | 15 |  |  |
|  | Config 3,6 |  | 30 |  |  |
| reportConfigType | Config 1-6 |  | periodic |  |  |
| reportQuantity | Config 1-6 |  | cri-RI-PMI-CQI |  |  |
| CSI reporting periodicity | Config 1,2,4,5 | slot | 5 |  |  |
|  | Config 3,6 |  | 10 |  |  |
| CSI reporting offset | Config 1,2,4,5 | slot | 2 |  |  |
|  | Config 3,6 |  | 4 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 |  | dBm/15kHz | -104 |  |  |
| Note2 | Config 1,2,4,5 | dBm/SCS | -104 |  |  |
|  | Config 3,6 |  | -101 |  |  |
|  |  | dB | 17 |  |  |
|  |  | dB | 17 |  |  |
| SS-RSRP Note3 | Config 1,2,4,5 | dBm/SCS | -87 |  |  |
|  | Config 3,6 |  | -84 |  |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -87 |  |  |
| IoNote3 | Config 1,2,4,5 | dBm/9.36MHz | -58.96 |  |  |
|  | Config 3,6 | dBm/38.16MHz | -52.87 |  |  |
| Propagation condition |  | - | AWGN |  |  |
| Correlation Matrix and Antenna Configuration |  | - | 2x2 Low |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. Note 3: SS-RSRP, Io and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2.] Note 5: All UL/DL transmission shall be confined within BWchannel_actual-occupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 6: All UL/DL transmission shall be confined within BWchannel_actual-occupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 7: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. |  |  |  |  |  |

Table A.4.5.3.1.1-4: Cell specific test parameters for NR SCell for known FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter |  | Unit | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| SSB ARFCN |  |  | freq2 |  |  |
| Duplex mode | ConfigSCell 1 |  | FDD |  |  |
|  | ConfigSCell 2,3 |  | TDD |  |  |
| TDD configuration | ConfigSCell 1 |  | Not Applicable |  |  |
|  | ConfigSCell 2 |  | TDDConf.1.1 |  |  |
|  | ConfigSCell 3 |  | TDDConf.2.1 |  |  |
| BWchannel | ConfigSCell 1 | MHz | Note 7 |  |  |
|  | ConfigSCell 2 |  | Note 7 |  |  |
|  | ConfigSCell 3 |  | Note 7 |  |  |
| BWoccupied | ConfigSCell 1 | RB | 52 Note 5 |  |  |
|  | ConfigSCell 2 |  | 52 Note 5 |  |  |
|  | ConfigSCell 3 |  | 106 Note 6 |  |  |
| DL initial BWP configuration | ConfigSCell 1-3 |  | DLBWP.0.1 |  |  |
| DL dedicated BWP configuration | ConfigSCell 1-3 |  | DLBWP.1.1 |  |  |
| UL initial BWP configuration | ConfigSCell 1-3 |  | ULBWP.0.1 |  |  |
| UL dedicated BWP configuration | ConfigSCell 1-3 |  | ULBWP.1.1 |  |  |
| DRX Cycle |  | ms | Not Applicable |  |  |
| PDSCH Reference measurement channel | ConfigSCell 1 |  | SR.1.1 FDD |  |  |
|  | ConfigSCell 2 |  | SR.1.1 TDD |  |  |
|  | ConfigSCell 3 |  | SR.2.1 TDD |  |  |
| RMSI CORESET Reference Channel | ConfigSCell 1 |  | CR.1.1 FDD |  |  |
|  | ConfigSCell 2 |  | CR.1.1 TDD |  |  |
|  | ConfigSCell 3 |  | CR.2.1 TDD |  |  |
| RMC CORESET Reference Channel | ConfigSCell 1 |  | CCR.1.1 FDD |  |  |
|  | ConfigSCell 2 |  | CCR.1.1 TDD |  |  |
|  | ConfigSCell 3 |  | CCR.2.1 TDD |  |  |
| TRS configuration | ConfigSCell 1 |  | TRS.1.1 FDD |  |  |
|  | ConfigSCell 2 |  | TRS.1.1 TDD |  |  |
|  | ConfigSCell 3 |  | TRS.1.2 TDD |  |  |
| OCNG Patterns | ConfigSCell 1,2 |  | OP.1 Note 5 |  |  |
|  | ConfigSCell 3 |  | OP.1 Note 6 |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |
| SSB configuration | ConfigSCell 1,2 |  | SSB.1 FR1 |  |  |
|  | ConfigSCell 3 |  | SSB.2 FR1 |  |  |
| CSI-RS configuration for CSI reporting | ConfigSCell 1 |  | CSI-RS.1.1 FDD |  |  |
|  | ConfigSCell 2 |  | CSI-RS.1.1 TDD |  |  |
|  | ConfigSCell 3 |  | CSI-RS.2.1 TDD |  |  |
| PDSCH/PDCCH subcarrier spacing | ConfigSCell 1,2 | kHz | 15 |  |  |
|  | ConfigSCell 3 |  | 30 |  |  |
| reportConfigType | ConfigSCell 1-3 |  | periodic |  |  |
| reportQuantity | ConfigSCell 1-3 |  | cri-RI-PMI-CQI |  |  |
| CSI reporting periodicity | ConfigSCell 1,2 | slot | 5 |  |  |
|  | ConfigSCell 3 |  | 10 |  |  |
| CSI reporting offset | ConfigSCell 1,2 | slot | 2 |  |  |
|  | ConfigSCell 3 |  | 4 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 |  | dBm/15kHz | -104 |  |  |
| Note2 | ConfigSCell 1,2 | dBm/SCS | -104 |  |  |
|  | ConfigSCell 3 |  | -101 |  |  |
|  |  | dB | 17 |  |  |
|  |  | dB | 17 |  |  |
| SS-RSRPNote3 | ConfigSCell 1,2 | dBm/SCS | -87 |  |  |
|  | ConfigSCell 3 |  | -84 |  |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -87 |  |  |
| IoNote3 | ConfigSCell 1,2 | dBm/9.36MHz | -58.96 |  |  |
|  | ConfigSCell 3 | dBm/38.16MHz | -52.87 |  |  |
| Propagation condition |  | - | AWGN |  |  |
| Correlation Matrix and Antenna Configuration |  | - | 2x2 Low |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. Note 3: SS-RSRP, Io and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2.] Note 5: All UL/DL transmission shall be confined within BWchannel_actual-occupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 6: All UL/DL transmission shall be confined within BWchannel_actual-occupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 7: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. |  |  |  |  |  |

##### A.4.5.3.1.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell in the first available uplink resource after slot (m+k). UE is allowed to postpone CSI report to next available uplink resource if an available uplink resource is subject to interruption.  Whether CSI report in slot (m+k) was interrupted is checked by monitoring ACK/NACK sent in PCell in slot (m+k).

During T2 the UE shall start sending CSI reports for SCell with non-zero CQI index at latest in a slot m+THARQ+Tactivtion_time+TCSI_ReportingNR slot length, Tactivation_time = TFirstSSB+ 5ms, as defined in clause 8.3.

During T3 the UE shall stop sending CSI reports for SCell at latest in a slot n+THARQ+3msNR slot length, as defined in clause 8.3.

During T2 interruption of PSCell during SCell activation shall not happen outside the slot m+1+THARQNR slot length to  m+1+THARQ+3ms+TXNR slot length+Ninterruption, and interruption of E-UTRA PCell during SCell activation shall not happen outside the subframe m1+1+THARQEUTRA slot length to subframe m2+1+THARQ+3ms+TXEUTRA slot length+Ninterruption, as defined in clause 8.3.

During T3 the starting point of interruption of PSCell during SCell deactivation shall not happen outside the slot n+1+THARQNR slot length to n+1+THARQ+3msNR slot length, as defined in clause 8.3 and the starting point of interruption of E-UTRA PCell during SCell deactivation shall not happen outside the subframe n1+1+THARQEUTRA subframe length to subframe n2+1+THARQ+3msEUTRA subframe length.

The interruption of PSCell shall not be more than the values specified for EN-DC in Clause 8.2.1.2.4.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation delay and SCell deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a slot m+THARQ+Tactivtion_time+TCSI_ReportingNR slot length as defined in clause 8.3 then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### <Next Change>

#### A.4.5.3.3	SCell Activation and deactivation of unknown SCell in FR1

##### A.4.5.3.3.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in clause 8.3, when the SCell in FR1 is unknown by the UE at the time of activation.

The supported test configurations are defined in clause A.4.5.3.1.1. The test parameters are the same except those described in the following clause. The listed parameter values in Tables A.4.5.3.3.1-1 will replace the values of corresponding parameters in Tables A.4.5.3.1.1-2. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are three carriers, E-UTRA has one cell, NR has two cells. Cell 1 and Cell 2 have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on E-UTRAN and Cell 2 (PSCell) on NR, but is not aware of Cell 3 (SCell) on NR. The UE is monitoring the PCell and PSCell. The UE shall be continuously scheduled in the PCell and PSCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 3) becomes configured on NR. During T1 the SCell is powered off and UE is not aware of SCell.

A MAC message for activation of SCell is sent by the test equipment 100ms after the RRC message, in a slot # denoted m. The point in time at which the MAC message for activation of SCell is received at the UE antenna connector defines the start of time period T2. The UE shall be able to report valid CSI for the activated SCell at latest in slot m+THARQ+Tactivtion_time+TCSI_ReportingNR slot length  as defined in clause 8.3 provided the SCell can be successfully detected on the first attempt. The UE shall start reporting CSI after at least one CSI-RS transmission occasion for channel measurement plus nCSI_ref  slots, as defined in 5.2.2.5 in [26], and reporting after slot (m+k) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PSCell interruption due to activation of SCell shall occur in the slot m+1+THARQNR slot length to slot m+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3, where Ninterruption is the interruption length given in clause 8.2. Any E-UTRA PCell interruption due to activation of SCell shall occur in the subframe m1+1+THARQEUTRA slot length to subframe m2+1+THARQ+3ms+TXEUTRA slot length+Ninterruption, where m1 and m2 are the index of the first and last subframe of E-UTRA PCell which overlaps with slot m, and Ninterruption is the interruption length given in TS 36.133 [14] clause 7.32.

Time period T3 starts when a MAC message for deactivation of the SCell, sent from the test equipment to the UE in a slot # denoted n, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell at latest in slot n+THARQ+3msNR slot length as defined in clause 8.3. The starting point of any PSCell interruption due to the deactivation shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+3msNR slot length, as defined in clause 8.3. The starting point of any E-UTRA PCell interruption due to the deactivation shall occur in the subframe n1+1+THARQEUTRA subframe length to subframe n2+1+THARQ+3msEUTRA subframe length, where n1 and n2 are the index of the first and last subframe of E-UTRA PCell which overlaps with slot n.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the slots from the time when the SCell1 deactivation command is sent until CSI reporting for SCell1 is discontinued.

Table A.4.5.3.3.1-1: General test parameters for unknown FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| T1 | ms | 100 | During this time the PSCell shall be known and the SCell configured, but not detected. |

##### A.4.5.3.3.2	Test Requirements

The test requirements defined in clause A.4.5.3.1.2 shall apply to this test case, except Tactivation_time will be replaced with the value TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms as defined in clause 8.3.

### <Next Change>

#### A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle

##### A.6.5.3.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in clause 8.3, when the SCell in FR1 is known by the UE at the time of activation.

The supported test configurations for NR PCell are shown in table A.6.5.3.1.1-1 below. Supported test configurations for NR SCell are shown in table A.6.5.3.1.1-1A. Test configuration for NR PCell and test configuration for NR SCell are chosen independently. The test parameters are given in Tables A.6.5.3.1.1-2 and cell-specific parameters in A.6.5.3.1.1-3 and A.6.5.3.1.1-4 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two NR carriers, each with one cell. Both cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1, but is not aware of Cell2. The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2. The UE now starts monitoring the SCC. The test equipment sends a MAC message for activation of the SCell.

The point in time at which the MAC message is received at the UE antenna connector, in slot # denoted n, defines the start of time period T2. The UE shall be able to report valid CSI in PCell for the activated SCell at latest in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length, as defined in clause 8.3. The UE shall start reporting CSI in PCell after at least one CSI-RS transmission occasion for channel measurement plus nCSI_ref  slots, as defined in 5.2.2.5 in [26], and reporting after slot n+THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCell shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3, where Ninterruption is the interruption length given in clause 8.2.

Time period T3 starts when a MAC message for deactivation of SCell, sent from the test equipment to the UE in a slot # denoted m, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell in a slot m+THARQ+3msNR slot length, as defined in clause 8.3, and The starting point of any PCell interruption due to the deactivation shall occur in the slot m+1+THARQNR slot length to m+1+THARQ+3msNR slot length, as defined in clause 8.3.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the slots from the time when the SCell deactivation command is sent until CQI reporting for SCell is discontinued.

Table A.6.5.3.1.1-1: known FR1 SCell activation in non-DRX for 160ms SCell measurement cycle supported test configurations for NR PCell

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Table A.6.5.3.1.1-1A: known FR1 SCell activation in non-DRX for 160ms SCell measurement cycle supported test configurations for NR SCell

| ConfigSCell | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Table A.6.5.3.1.1-2: General test parameters for known FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two NR radio channel (1, 2) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell2 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 7 | During this time the PCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| A3-offset | dB | -15 |  |
| THARQ | ms | Config 1: 2 Config 2: 3 Config 3: 2.5 | k1×NR slot length k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [3] that will meet the timing constraints of this test case. |
| TCSI_Reporting | ms | 15 | The delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |

Table A.6.5.3.1.1-3: Cell specific test parameters for NR PCell for known FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter |  | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| Duplex mode | Config 1 |  | FDD |  |  |
|  | Config 2,3 |  | TDD |  |  |
| TDD configuration | Config 1 |  | Not applicable |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |
| BWchannel | Config 1,2 | MHz | Note 7 |  |  |
|  | Config 3 |  | Note 7 |  |  |
| BWoccupied | Config 1,2 | RB | 52 Note 5 |  |  |
|  | Config 3 |  | 106 Note 6 |  |  |
| Initial BWP configuration |  |  | DLBWP.0.1 |  |  |
| TCI state |  |  | TCI.State.0 |  |  |
| TRS Configuration | Config 1 |  | TRS.1.1 FDD |  |  |
|  | Config 2 |  | TRS.1.1 TDD |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  |  |
|  | Config 2 |  | SR.1.1 TDD |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD |  |  |
|  | Config 2 |  | CCR.1.1 TDD |  |  |
|  | Config 3 |  | CCR.2.1 TDD |  |  |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  |
| OCNG Patterns | Config 1,2 |  | OP.1Note 5 |  |  |
|  | Config 3, |  | OP.1 Note 6 |  |  |
| SSB Configuration | Config 1,2 |  | SSB.1 FR1 |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |
| CSI-RS configuration for CSI reporting (Note 8) | Config 1 |  | CSI-RS.1.1 FDD |  |  |
|  | Config 2 |  | CSI-RS.1.1 TDD |  |  |
|  | Config 3 |  | CSI-RS.2.1 TDD |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |
| reportConfigType |  |  | periodic |  |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |  |
| CSI reporting periodicity | Config 1,2 | slot | 5 |  |  |
|  | Config 3 |  | 10 |  |  |
| CSI reporting offset | Config 1,2 | slot | 3 |  |  |
|  | Config 3 |  | 5 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 | Config 1,2 | dBm/SCS | -104 |  |  |
|  | Config 3 |  | -101 |  |  |
|  |  | dB | 17 |  |  |
|  |  | dB | 17 |  |  |
| SS-RSRPNote3 | Config 1,2 | dBm/SCS | -87 |  |  |
|  | Config 3 |  | -84 |  |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -87 |  |  |
| Io Note3 | Config 1,2 | dBm/ 9.36MHz | -58.96 |  |  |
|  | Config 3 | dBm/ 38.16MHz | -52.87 |  |  |
| Propagation condition |  | - | AWGN |  |  |
| Correlation Matrix and Antenna Configuration |  | - | 2x2 Low |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. Note 3: SS-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 7: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. Note 8: On top of the reference configurations, CSI-RS offset should be set to meet the CSI reference resource timing definition in TS 38.214 cl. 5.2.2.5. |  |  |  |  |  |

Table A.6.5.3.1.1-4: Cell specific test parameters for NR SCell for known FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter |  | Unit | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| Duplex mode | ConfigSCell 1 |  | FDD |  |  |
|  | ConfigSCell 2,3 |  | TDD |  |  |
| TDD configuration | ConfigSCell 1 |  | Not applicable |  |  |
|  | ConfigSCell 2 |  | TDDConf.1.1 |  |  |
|  | ConfigSCell 3 |  | TDDConf.2.1 |  |  |
| BWchannel | ConfigSCell 1,2 | MHz | Note 7 |  |  |
|  | ConfigSCell 3 |  | Note 7 |  |  |
| BWoccupied | ConfigSCell 1,2 | RB | 52 Note 5 |  |  |
|  | ConfigSCell 3 |  | 106 Note 6 |  |  |
| Initial BWP configuration |  |  | DLBWP.0.1 |  |  |
| TCI state |  |  | TCI.State.0 |  |  |
| TRS Configuration | ConfigSCell 1 |  | TRS.1.1 FDD |  |  |
|  | ConfigSCell 2 |  | TRS.1.1 TDD |  |  |
|  | ConfigSCell 3 |  | TRS.1.2 TDD |  |  |
| PDSCH Reference measurement channel | ConfigSCell 1 |  | N/A |  |  |
|  | ConfigSCell 2 |  | N/A |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |
| Dedicated CORESET parameters | ConfigSCell 1 |  | N/A |  |  |
|  | ConfigSCell 2 |  | N/A |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |
| RMSI CORESET parameters | ConfigSCell 1 |  | N/A |  |  |
|  | ConfigSCell 2 |  | N/A |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |
| OCNG Patterns | ConfigSCell 1,2 |  | OP.1Note 5 |  |  |
|  | ConfigSCell 3, |  | OP.1 Note 6 |  |  |
| SSB Configuration | ConfigSCell 1,2 |  | SSB.1 FR1 |  |  |
|  | ConfigSCell 3 |  | SSB.2 FR1 |  |  |
| CSI-RS configuration for CSI reporting Note 8 | ConfigSCell 1 |  | CSI-RS.1.1 FDD |  |  |
|  | ConfigSCell 2 |  | CSI-RS.1.1 TDD |  |  |
|  | ConfigSCell 3 |  | CSI-RS.2.1 TDD |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |
| reportConfigType |  |  | N/A |  |  |
| reportQuantity |  |  | N/A |  |  |
| CSI reporting periodicity | ConfigSCell 1,2 | slot | N/A |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |
| CSI reporting offset | ConfigSCell 1,2 | slot | N/A |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 | ConfigSCell 1,2 | dBm/SCS | -104 |  |  |
|  | ConfigSCell 3 |  | -101 |  |  |
|  |  | dB | 17 |  |  |
|  |  | dB | 17 |  |  |
| SS-RSRPNote3 | ConfigSCell 1,2 | dBm/SCS | -87 |  |  |
|  | ConfigSCell 3 |  | -84 |  |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -87 |  |  |
| Io Note3 | ConfigSCell 1,2 | dBm/ 9.36MHz | -58.96 |  |  |
|  | ConfigSCell 3 | dBm/ 38.16MHz | -52.87 |  |  |
| Propagation condition |  | - | AWGN |  |  |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. Note 3: SS-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 7: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. Note 8: On top of the reference configurations, CSI-RS offset should be set to meet the CSI reference resource timing definition in TS 38.214 cl. 5.2.2.5. |  |  |  |  |  |

##### A.6.5.3.1.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell in the first available uplink resource after at least one CSI-RS transmission occasion for channel measurement plus nCSI_ref  slots, as defined in 5.2.2.5 in [26], and reporting after slot (n+1+THARQ+3msNR slot length). UE is allowed to postpone CSI report to next available uplink resource if an available uplink resource is subject to interruption.

During T2 the UE shall start sending CSI reports for SCell with non-zero CQI index at latest in a slot n+THARQ+Tactivtion_time+TCSI_ReportingNR slot length, Tactivation_time = TFirstSSB+ 5ms, as defined in clause 8.3.

During T3 the UE shall stop sending CSI reports for SCell at latest in a slot m+THARQ+3msNR slot length, as defined in clause 8.3.

During T2 interruption of PCell  during SCell activation shall not happen outside the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3.

During T3 the starting point of interruption of PCell during SCell deactivation shall not happen outside the slot m+1+THARQNR slot length to m+1+THARQ+3msNR slot length, as defined in clause 8.3.

The interruption on any activated serving cell shall not be more than the values specified for SA in clause 8.2.2.2.2.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation delay and SCell deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a slot THARQ+Tactivtion_time+TCSI_ReportingNR slot length as defined in clause 8.3 then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### <Next Change>

#### A.6.5.3.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX

##### A.6.5.3.3.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in clause 8.3, when the SCell in FR1 is known by the UE at the time of activation.

The supported test configurations are shown in table A.6.5.3.1.1-1 below. The test parameters are given in Tables A.6.5.3.1.1-2 and cell-specific parameters in A.6.5.3.1.1-3 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two NR carriers, each with one cell. Both cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1, but is not aware of Cell2. The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2. The UE now starts monitoring the SCC. The test equipment sends a MAC message for activation of the SCell.

The point in time at which the MAC message is received at the UE antenna connector, in slot # denoted n, defines the start of time period T2. The UE shall be able to report valid CSI in PCell for the activated SCell at latest in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length, as defined in clause 8.3. The UE shall start reporting CSI in PCell after at least one CSI-RS transmission occasion for channel measurement plus nCSI_ref  slots, as defined in 5.2.2.5 in [26], and reporting after slot n+THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCell shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3, where Ninterruption is the interruption length given in clause 8.2.

Time period T3 starts when a MAC message for deactivation of SCell, sent from the test equipment to the UE in a slot # denoted m, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell in a slot m+THARQ+3msNR slot length, as defined in clause 8.3, and The starting point of any PCell interruption due to the deactivation shall occur in the slot m+1+THARQNR slot length to m+1+THARQ+3msNR slot length, as defined in clause 8.3.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the slots from the time when the SCell deactivation command is sent until CQI reporting for SCell is discontinued.

Table A.6.5.3.3.1-1: General test parameters for unknown FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| T1 | ms | 100 | During this time the PCell shall be known and the SCell configured, but not detected. |

##### A.6.5.3.3.2	Test Requirements

The test requirements defined in clause A.6.5.3.1.2 shall apply to this test case, except Tactivation_time will be replaced with the value TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms as defined in clause 8.3.

### <Next Change>

#### A.7.5.3.2	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2

##### A.7.5.3.2.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in clause A.7.5.3.1.1 except the PCell is in FR1 and SCell is in FR2.

The supported test configurations are defined in Table A.7.5.3.2.1-1. The general test parameters are the same as defined in Table A.6.5.3.1.1-2 except that the length of T2 is 2s. And cell specific test parameters are described in Tables A.7.5.3.2.1-2. OTA related test parameters are defined in Table A.7.5.3.2.1-3.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on NR. During T1 the SCell is powered off and UE is not aware of SCell.

A MAC message for activation of SCell is sent by the test equipment 100ms after the RRC message, in a slot # denoted m. The point in time at which the MAC message for activation of SCell is received at the UE antenna connector defines the start of time period T2.

During T2, the test equipment monitors the L1-RSRP measurement reporting for the SCell. The time when test equipment receives a valid L1-RSRP report is denoted as slot m+TL1-RSRP. In the next DL slot after slot m+TL1-RSRP, the test equipment sends a MAC message for the activation of the TCI state of the RMC CORESET of the SCell. In the same slot, the test equipment also sends an RRC message to configure the CSI-RS resources for SCell.

Time period T3 starts when a MAC message for deactivation of the SCell, sent from the test equipment to the UE in a slot # denoted n, is received at the UE antenna connector.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell and PCell during activation of SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the slots from the time when the SCell1 deactivation command is sent until CSI reporting for SCell1 is discontinued.

Table A.7.5.3.2.1-1: Supported test configurations for FR2 SCell activation case

| Configuration | Description |
| --- | --- |
| 1 | PCell: 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode Target SCell: 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 2 | PCell: 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode Target SCell: 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 3 | PCell: 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode Target SCell: 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations |  |

Table A.7.5.3.2.1-2: Cell specific test parameters for FR2 SCell activation case

| ParameterNote 5 |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |  | T1 | T2 | T3 |
| SSB ARFCN |  |  | Freq1 |  |  |  | Freq2 |  |  |
| Duplex mode | Config 1 |  | FDD |  |  |  | TDD |  |  |
|  | Config 2,3 |  | TDD |  |  |  |  |  |  |
| TDD configuration | Config 1 |  | Not Applicable |  |  |  | TDDConf.3.1 |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |
| Downlink initial BWP Configuration | Config 1,2,3 |  | DLBWP.0.1 |  |  |  |  |  |  |
| Downlink dedicated BWP Configuration | Config 1,2,3 |  | DLBWP.1.1 |  |  |  |  |  |  |
| Uplink initial BWP configuration | Config 1,2,3 |  | ULBWP.0.1 |  |  |  |  |  |  |
| Uplink dedicated BWP configuration | Config 1,2,3 |  | ULBWP.1.1 |  |  |  |  |  |  |
| TRS configuration | Config 1,2,3 |  | N/A |  |  |  | TRS.2.1 TDD |  |  |
| TCI state | Config 1,2,3 |  | TCI.State.0 |  |  |  |  |  |  |
| BWchannel | Config 1,2 | MHz | 10: NRB,c = 52 |  |  |  | 100: NRB,c = 66 |  |  |
|  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| Data RBs allocated | Config 1,2 |  | 52 |  |  |  | 66 |  |  |
|  | Config 3 |  | 106 |  |  |  |  |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  |  |  | - |  |  |
|  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  |  |  |  |  |
| RMSI CORESET Parameters | Config 1 |  | CR.1.1 FDD |  |  |  | - |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  |  |  |  |  |
| Dedicated CORESET Parameters | Config 1 |  | CCR.1.1 FDD |  |  |  | - |  |  |
|  | Config 2 |  | CCR.1.1 TDD |  |  |  |  |  |  |
|  | Config 3 |  | CCR.2.1 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |  |  |  |
| SSB configuration | Config 1,2 |  | SSB.1 FR1 |  |  |  | SSB.3 FR2 |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |
| CSI-RS configuration for CSI reporting | Config 1~3 |  | N/A |  |  |  | N/A | CSI-RS.3.1 TDD Note 6 | CSI-RS.3.1 TDD |
| reportConfigType for CSI reporting |  |  | periodic |  |  |  | N/A |  |  |
| reportConfigType for L1-RSRP |  |  | periodic |  |  |  | N/A |  |  |
| reportQuantity for CSI reporting |  |  | cri-RI-PMI-CQI |  |  |  | N/A |  |  |
| reportQuantity for L1-RSRP |  |  | ssb-Index-RSRP |  |  |  | N/A |  |  |
| CSI reporting periodicity | Config 1,2 | slot | 5 |  |  |  | N/A |  |  |
|  | Config 3 |  | 10 |  |  |  |  |  |  |
| L1-RSRP reporting periodicity Note 7 | Config 1,2 | slot | 5 |  |  |  | N/A |  |  |
|  | Config 3 |  | 10 |  |  |  |  |  |  |
| CSI reporting offset | Config 1,2 | slot | 2 |  |  |  | N/A |  |  |
|  | Config 3 |  | 4 |  |  |  |  |  |  |
| L1-RSRP reporting offset | Config 1,2 | slot | 2 |  |  |  | N/A |  |  |
|  | Config 3 |  | 4 |  |  |  |  |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |
| Propagation conditions |  |  | N/A Link only, see clause A.3.7A |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Void Note 4: Void Note 5: All parameters apply for configuration 1 and 2 Note 6: CSI-RS for CSI measurement is (re)configured in the next DL slot after slot m+TL1-RSRP during T2. Note 7: L1-RSRP measurement and reporting are configured to the the UE prior to the start of time period T1. |  |  |  |  |  |  |  |  |  |

Table A.7.5.3.2.1-3: OTA related test parameters for FR1 PCell activation case with FR2 SCell

| Parameter |  | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Angle of arrival configuration |  |  | N/A |  |  | According to clause A.3.15.1 |  |  |
| Assumption for UE beams Note 7 |  |  | N/A |  |  | Rough |  |  |
| Note 1 | Config 1,2,3 | dBm/15kHz | Link only, see clause A.3.7A |  |  | -104.7 |  |  |
| Note 1 | Config 1,2,3 | dBm/SCS |  |  |  | -95.7 |  |  |
|  | Config 1,2,3 | dB |  |  |  | -∞ | 7 | 7 |
|  | Config 1,2,3 | dB |  |  |  | -∞ | 7 | 7 |
| SSB_RPNote 2, Note 4 | Config 1,2,3 | dBm/SCS |  |  |  | -∞ | -88.7 | -88.7 |
| IoNote 2, Note 4 | Config 1,2,3 | dBm/95.04 MHz |  |  |  | -66.68 | -58.92 | -58.92 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: Void Note 6: Void Note 7: Information about types of UE beam is given in B.2.1.3 and does not imit UE implementation or test system implementation. |  |  |  |  |  |  |  |  |

##### A.7.5.3.2.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell in the first available uplink resource after at least one CSI-RS transmission occasion for channel measurement plus nCSI_ref  slots, as defined in 5.2.2.5 in [26], and reporting after slot (m+k). UE is allowed to postpone CSI report to next available UL resource if an available uplink resource is subject to interruption.  Whether CSI report in a slot was interrupted is checked by monitoring ACK/NACK sent in PCell in the slot.

During T2 the UE shall start sending valid L1-RSRP report for the SCell in the configured slots for CSI reporting after slot (m+TL1-RSRP), where TL1-RSRP is no larger than

3ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report

as defined in clause 8.3.2. For this test case, TFirstSSB_MAX=TSMTC_MAX=Trs=20ms; TL1-RSRP, measure=160ms and TL1-RSRP, report=5ms, which allows TL1-RSRP 680 ms.

During T2 the UE shall start sending CSI reports for the SCell with non-zero CQI index in the configured slots for CSI reporting no later than slot m+THARQ+Tactivtion_time+TCSI_ReportingNR slot length, where

- THARQ is defined in Table A.5.5.3.1.1-2

- Tactivation_time = 3ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}, which allows 710 ms

- TCSI_Reporting = 10ms

- NR slot length is 0.125ms for this test case.

During T3 the UE shall stop sending CSI reports for both SCells no later than slot n+THARQ+3msNR slot length, as defined in clause 8.3.

During T2 interruption of PCell during SCell activation shall not happen outside the slot m+1+THARQNR slot length  to m+1+THARQ+3ms+TXNR slot length, as defined in clause 8.3, where TX =20ms.

During T3 the starting point of interruption of PCell during SCell deactivation shall not happen outside the slot n+1+THARQNR slot length to n+1+THARQ+3msNR slot length, as defined in clause 8.3.

The interruption of PCell due to activation of SCell shall not be more than the values specified for SA in Clause 8.2.2.2.7.

### <End of Change>