3GPP TSG-RAN WG4 Meeting #111															R4-2407227

Fukuoka, Japan, May 20th – 24th, 2024

Agenda item:	10.1.1.3.2

Source:	Apple

Title:	On MPR reduction for NR UL CA

WI/SI:	NR_ENDC_RF_Ph4

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

Power domain enhancement is a re-occurring topic and enters the 4th phase when starting the Rel-19 work. The objectives are covered in the WID [1]. This contribution focuses on the high-level aspects of MPR reduction. The WID lists those goals as:

# 2  Discussion

Over the past releases RAN4 discussed various aspects allowing to increase the output power and consequentially improve UL coverage. In Rel-19 a new angle is approached by evaluating potential power domain improvements from relaxing emission requirements. Such requirements might be ACLR, SEM and spurious emissions. Since ACLR is typically associated with being the limiting factor for almost fully allocated channels and SEM often for narrow allocations, high-throughput and cell-edge scenarios might see performance gains.

The WF [2] captures the following objectives:

Last meeting proposals were made to consider an activation-based scheme for requirements such as MPR and A-MPR. The idea is that in case of carrier aggregation with two configured CCs there is not always the need to use CA MPR. If the configured RB allocation is fully contained in of the CCs then there might be potential to reduce power back-off. Probably even down to single CC MPR and A-MPR. The current assumption is that the device always places the LO frequency in the middle of the aggregated bandwidth. This placement determines the location of IQ-Image and IMD products with respect to the configured RB allocation. Since the device keeps the LO frequency at a fixed position independent on the configured RB allocation there is need for significant power back-off even if the allocation is fully contained inside one of the CCs. When considering an activation-based scheme which implies changes of LO frequency then RAN4 should discuss a basic RF architecture assumption to achieve a common understanding for further analysis.

Observation: RAN4 uses the assumption of configured CCs for UL CA when specifying MPR and A-MPR. When changing this assumption to a different scheme such as activation-based then then RAN4 should discuss a basic RF architecture assumption to achieve a common understanding for further analysis.

Proposal: RAN4 should find a common understanding on how activation-based UL CA affects the architecture assumptions.

# 4	Conclusions

This contribution provides certain considerations to the discussion on the MPR reduction which his part of the power domain enhancement WID. The following observation and proposal are made:

Observation: RAN4 uses the assumption of configured CCs for UL CA when specifying MPR and A-MPR. When changing this assumption to a different scheme such as activation-based then then RAN4 should discuss a basic RF architecture assumption to achieve a common understanding for further analysis.

Proposal: RAN4 should find a common understanding on how activation-based UL CA affects the architecture assumptions.

# 4	References

- RP-240828, “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, 3GPP TSG RAN Meeting #103

- R4-2406584, “WF on power domain enhancements”, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #110bis