# 3GPP TSG-RAN WG4 Meeting #111	R4-2407262

# Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Agenda item:	7.25.1

Source:	Apple

Title:	UE demodulation performance requirements for NR dynamic spectrum sharing

Release:	Rel-18

Document for:	Discussion

# 1. Introduction

In RAN4#110bis requirements for enhanced DSS were discussed and way forward [1] were agreed.  In this contribution we present our simulation results for PDCCH demodulation requirements for eDSS.

# 2. Discussion

In RAN4#110bis the simulation assumptions were agreed for PDCCH demodulation requirements for eDSS in [1].

| Parameters | Values |
| --- | --- |
| Duplex mode | FDD/TDD |
| SCS | 15 kHz |
| Bandwidth | 10 MHz |
| Channel model | TDLA30-10 and TDLC300-100 |
| Antenna configuration | 2x2, 2x4, 4x2 |
| CRS | 4 port CRS |
| DCI payload (excluding CRC) | 39 bits |
| Interleaving | Non-interleaved |
| Precoding | Precoder cycling per REG bundle |
| REG bundle size | 6 |
| Shift index | 0 |
| CORSET duration | 2 (2nd symbol and 3rd symbol) |
| CORSET RB | 48 |
| AL | 4, 8 |
| - Note 1: The bandwidth of NR CORESET is within the bandwidth of LTE. - Note 2: gNB punctures the PDCCH data and DMRS REs overlapped with LTE CRS. Note 3: PDCCH channel estimation is assumed to use only the clean PDCCH symbol. |  |

Our simulation results are presented below.

Table 1: PDCCH performance results in eDSS

| Test number | Duplex | BW (MHz) | CORESET RB | CORESET duration | AL | Propagation Condition | Antenna configuration and correlation Matrix | SNR @ 1% Pm-dsg |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Alignment | Impairment |
| 1 | FDD | 10 | 48 | 2 | 4 | TDLA30-10 | 2x4 Low | -1.6 | 0.4 |
| 2 | FDD | 10 | 48 | 2 | 8 | TDLC300-100 | 4x2 Low | -3.0 | -1.0 |
| 3 | TDD | 10 | 48 | 2 | 4 | TDLA30-10 | 4x4 Low | -2.4 | -0.4 |
| 4 | TDD | 10 | 48 | 2 | 8 | TDLC300-100 | 2x2 Low | -2.4 | -0.4 |

# 3. Conclusion

In this paper, we provide simulation requirements for UE demodulation requirements for DSS enhancements.

# Reference

[1].  R4-2406019,	 “Way Forward for [110bis][333] NR_DSS_enh”, Ericsson.