3GPP TSG-RAN WG4 Meeting #111 	R4-2407265

# Fukuoka, Japan, May 20 – May 24, 2024

Agenda item:	7.8.7.1

Source:	Apple Inc.

Title:	Summary of Simulation Results for NR support for dedicated spectrum less than 5MHz for FR1

Release:	Rel-18

Document for:	Discussion

# 1. PBCH Results

Ideal Results

| Duplex | Bandwidth (MHz) / Subcarrier spacing (kHz) | SSB/PBCH index | Propagation condition | Antenna configuration and correlation matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |
| FDD | 3 / 15 | Unknown | TDLC300-100 | 1 x 2 Low, | 1 | -6.3 |
|  |  |  |  | 1 x 4 Low | 1 | -8.6 |

Impairment Results

| Duplex | Bandwidth (MHz) / Subcarrier spacing (kHz) | SSB/PBCH index | Propagation condition | Antenna configuration and correlation matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |
| FDD | 3 / 15 | Unknown | TDLC300-100 | 1 x 2 Low, | 1 | -3.8 |
|  |  |  |  | 1 x 4 Low | 1 | -6.1 |

# 2. PDCCH Requirements

Ideal Results

| Number of Tx | BW (MHz) | CORESET RB | CORESET duration | Aggr. Level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 3 | 15 | 3 | 4 | [R.PDCCH. 1-2.x FDD] | TDLA30-10 | 1x2 Low | 1 | 5.0 |
| 2 | 3 | 15 | 3 | 4 | [R.PDCCH. 1-2.x FDD] | TDLC300-100 | 2x4 Low | 1 | -2.7 |

Impairment Results

| Number of Tx | BW (MHz) | CORESET RB | CORESET duration | Aggr. Level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 3 | 15 | 3 | 4 | [R.PDCCH. 1-2.x FDD] | TDLA30-10 | 1x2 Low | 1 | 7.0 |
| 2 | 3 | 15 | 3 | 4 | [R.PDCCH. 1-2.x FDD] | TDLC300-100 | 2x4 Low | 1 | -0.2 |