3GPP TSG-RAN WG4 Meeting # 111	R4-2407392

# Fukuoka, Japan,  20th – 24st May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 7319 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (LTE_NBIoT_eMTC_NTN_req-Perf) CR to TS 36.133: Corrections to IE configurations (Rel 18) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIoT_eMTC_NTN_req-Perf |  |  |  |  |  | Date: |  |  | 2024-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Incorrect implementation of R4-2400665 for A.13.1.1.3 - IE s-IntraSearchP revised wrongly to s-IntraSearchPTable A.13.1.1.3.1-2: G instead of s-IntraSearchP-v1360s. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In TCs A.13.1.1.3 corrected IE s-IntraSearchP-v1360s |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Implementaion of the TC not possible due to wrong value. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.13.1.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.521-3 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

#### A.13.1.1.3	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX

##### A.13.1.1.3.1	Test Purpose and Environment

This test is to verify the requirement for the HD-FDD intra frequency cell reselection requirements for Cat-NB1 UE specified in clause 4.6A.2.2.

The test scenario comprises of one NB-IoT carrier with 2 nCells of different physical cell ID, as given in tables A.13.1.1.3.1-1, A.13.1.1.3.1-2 and A.13.1.1.3.1-3. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Only nCell1 is already identified by the UE prior to the start of the test, i.e. nCell 2 is not identified. nCell 1 and nCell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing nCell 2. In Test 1, UE supports the UE specific DRX cycle of 0.32 s and the UE shall be configured with DRX cycle of 0.32 s prior to the start of the test. In Test 2, UE supports the UE specific DRX cycle of 0.64 s and the UE shall be configured with DRX cycle of 0.64 s prior to the start of the test.

The UE shall be provided with the valid information about the SAN serving cells before the test.

Table A.13.1.1.3.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GEO, HD-FDD duplex mode |

Table A.13.1.1.3.1-2: General test parameters for HD-FDD intra frequency cell reselection test case for Cat-NB1 UE in normal coverage

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NB-IOT operational mode |  |  | Standalone |  |  |
| Satellite information | Config 1 |  | GEO |  |  |
| Initial condition | Active cell |  | nCell1 |  |  |
|  | Neighbour cells |  | nCell2 |  |  |
| T2 end condition | Active cell |  | nCell2 |  |  |
|  | Neighbour cells |  | nCell1 |  |  |
| Final condition | Visited cell |  | nCell1 |  |  |
| Access Barring Information |  | - | Not Sent |  | No additional delays in random access procedure. |
| s-IntraSearchP- v1360s |  |  | 63 (126 dB) |  | to trigger intra-frequency measurement in this test |
| DRX cycle length |  | s | 0.32 | 0.64 | The value shall be used for all cells in the test. |
| T1 |  | s | >7 |  | During T1, nCell2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that nCell2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 35 |  | T2 is defined so that cell re-selection time is taken into account. Once the UE has reselected to nCell2 (within T2) T3 starts |
| T3 |  | s | 14 |  | T3 is defined so that cell re-selection time is taken into account. |

Table A.13.1.1.3.1-3: nCell 1, nCell 2 specific test parameters for HD-FDD intra frequency cell reselection test case for Cat-NB1 UE in normal coverage

| Parameter | Unit | nCell 1 |  |  | nCell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | kHz | 180 |  |  | 180 |  |  |
| OCNG Pattern as defined in A.3 | - | NOP.3 FDD |  |  | NOP.3 FDD |  |  |
| NPBCH_RA | dB | 0 |  |  | 0 |  |  |
| NPBCH_RB | dB |  |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |  |  |
| NPDSCH_RA | dB |  |  |  |  |  |  |
| NPDSCH_RB | dB |  |  |  |  |  |  |
| NOCNG_RANote 1 | dB |  |  |  |  |  |  |
| NOCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Qrxlevmin | dBm | -140 | -140 | -140 | -140 | -140 | -140 |
| Pcompensation | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| Qhysts | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| Qoffsets, n | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| Cell_selection_and_ reselection_quality_measurement |  | NRSRP |  |  | NRSRP |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |  |
|  | dB | 17 | 13 | 17 | -infinity | 17 | 13 |
| Note2 | dB | 17 | -4.09 | 3.79 | -infinity | 3.79 | -4.09 |
| NRSRP Note2 | dBm/15 kHz | -81 | -85 | -81 | -infinity | -81 | -85 |
| Treselection | s | 0 | 0 | 0 | 0 | 0 | 0 |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x1 |  |  | 1x1 |  |  |
| Timing offset to nCell 1 | ms | - |  |  | 3 |  |  |
| Note 1: NOCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Es/Iot and NRSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

##### A.13.1.1.3.2	Test Requirements

In each test, the cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on nCell 2, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on nCell 2.

The cell re-selection delay to a newly detectable cell shall be less than 34.32 s in test 1 and test 2.

In each test, the cell reselection delay to an already detected cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on nCell 1, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on nCell 1.

The cell re-selection delay to an already detected cell shall be less than 13.44 s in test 1 and test 2.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a newly detectable cell can be expressed as: Tdetect,NB_Intra_NC + TSI, and to an already detected cell can be expressed as: Tevaluate, NB_intra_NC + TSI,

Where:

Tdetect,NB_Intra_NC		See Table 4.6A.2.2-1 in clause 4.6A.2.2

Tevaluate, NB_intra_NC	See Table 4.6A.2.2-1 in clause 4.6A.2.2

TSI	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 8.32 s is assumed in this test case.

This gives a total of 34.32 s, allow 35 s for the cell re-selection delay to a newly detectable cell and 13.44 s, allow 14s for the cell re-selection delay to an already detected cell in the test case.

< End of changes >