3GPP TSG-RAN WG4 Meeting # 111		R4-2407431

Fukuoka, Japan, 20 – 24 April 2024

Agenda Item:	6.14.2

Source: 	Nokia

Title: 	TP to TR 37.829: Editorial corrections

Document for:	Approval

# 1.	Introduction

The Rel-18 WI on High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands should be completed and TR 37.829 should be submitted for approval in RAN#104. This contribution provides a text proposal to correct the identified editorial errors in the latest version of TR 37.829 [1].

# 2.	Text proposal

<Start of change>

# 5 	Band specific requirements for a UE

Editor note: This section relates to the Band Specific objectives of the WI.

Editor note: All sub-clauses are not necessary for all bands.

## 5.1	Bands n71 and n85

Editor note: All sub-clauses are not necessary for all bands.

### 5.1.1	REFSENS exception

Not needed.

<Next change>

### 5.3.1	REFSENS exception

Not needed.

<Next change>

### 5.5.1	REFSENS exception

Not needed.

### 5.5.2	A-MPR and MPR

#### 5.5.2.1 	A-MPR simulation assumption

- Operating band n41 (2496 MHz – 2690 MHz)

- Power class PC1

- WOLA processing

- I/Q image -28 dBc

- Carrier leakage -28 dBc

- CIM3 -60 dBc, CIM5 -70 dBc

- Only additional SEM and additional spurious emission mask are simulated. Other gating factors (ACLR, etc.) are excluded.

- Additional SEM and additional spurious emission mask for NS_04:

In TS 38.101-1, NS_04 defines both an additional spectrum emission mask and an additional spurious emission mask:

Table 6.5.2.3.2-3: n41 and n90 SEM with "NS_04"

| ΔfOOB MHz | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 35 |  | 40 | 45 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
| ± 0 - 1 | -10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 % channel bandwidth |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -10 |  |  |  |  | 1 MHz |
| ± 1 - 5 | -10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 MHz |
| ± 5 - X | -13 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± X - (BWChannel + 5 MHz) | -25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: X is defined in Table 6.5.2.3.2-1 for CP-OFDM and 6.5.2.3.2-2 for DFT-S-OFDM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.5.3.3.1-1: Additional requirements for "NS_04"

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 MHz |  |
| 2495 ≤ f < 2496 | -13 | 1 % of Channel BW |
| 2490.5 ≤ f < 2495 | -13 | 1 MHz |
| 0.009 < f < 2490.5 | -25 | 1 MHz |

#### 5.5.2.2 	A-MPR simulation results

##### 5.5.2.2.1 	General

The SEM for NS_04 does not require A-MPR in PC1. Thus, the A-MPR will be entirely based on the additional spurious mask. 256-QAM does not need A-MPR due to its high MPR.

Fig. 5.5.2.2-1 and 5.5.2.2-2 show examples of A-MPR needed due to the additional spurious emission mask when the channel is at the lower edge of n41. The triangle plots show only allocations that need A-MPR. We can distinguish the following allocation regions:

- spectral regrowth (the largest area on the left)

- IMD3 of RB allocation and its image (narrow allocations close to lower channel edge)

- CIM3 (narrow allocations close to upper channel edge)

- edge allocations at the lower channel edge (typically too few to be visible in the figures)

These regions are depicted in Fig. 5.5.2.2-3.

Figure 5.5.2.2-1: Simulated back-off for QPSK 
in 20 MHz channel at lower edge of n41

Figure 5.5.2.2-2: Simulated back-off for QPSK 
in 100 MHz channel at lower edge of n41

Figure 5.5.2.2-3: Simplified sketch of the allocation regions; the edge region is 
too small to be visible; “MPR” indicates the region where A-MPR is not applied

In PC3, NS_04 A-MPR was applied in a binary manner: if the channel is close to the lower band edge, fixed A-MPR is applied; if the channel is far enough away from the lower band edge, A-MPR is not applied. This results in excess A-MPR that grows with increasing distance from the lower band edge.

For PC1, NS_04 A-MPR is specified as
A-MPR = max( MPR, A-MPRregrowth, A-MPRIMD3, A-MPRCIM3, A-MPRedge ),
The A-MPR components of this formula are explained in the following sections.

This formula allows to define the A-MPR component values separately without explicitly defining their allocation regions. There will be implicit allocation regions determined by which of the component A-MPR values is the greatest. Such implicit regions depend on the position of the channel in the operating band. Determining such region boundaries may help understand the proposed scheme but is not needed in implementation.

##### 5.5.2.2.2 	A-MPR based on spectral regrowth

The A-MPR needed due to spectral regrowth, A-MPRregrowth,  depends on how deep the regrowth reaches into the protected frequency region. This depends on the frequency distance between the protected region and RB allocation. We refer to this distance as the total guard bandwith (TGBW). TGBW is the sum of the following distances / bandwidths as depicted in Fig. 5.5.2.2-4:

- distance between the protected frequency region and closest operating band edge

- distance between operating band edge and channel edge (frequency offset in NS_04 PC3 A-MPR spec)

- guard band within the channel

- unallocated RBs between the guard and RB allocation

Figure 5.5.2.2-4: Total guard bandwidth (TGBW) for protected region below the UL operating band

Among regrowth-limited allocations, for given allocation bandwidth and TGBW, the channel bandwidth and SCS have a negligible effect on required power back-off. Thus, the simulation results of all channel bandwidths and SCSs can be combined, and that causes barely any excess A-MPR because the TGBW approach takes into account the different guard bandwidths of CBWs and SCSs.

Fig. 5.5.2.2-5 to 5.5.2.2-11 present the TGBW required to avoid violating the NS_04 additional spurious emission mask, as a function of allocation bandwidth. Each blue dot represents the maximum TGBW among all allocations with the same allocation bandwidth and back-off. Notice that the TGBW is placed on the horizontal axis, while the allocation bandwidth is on the vertical axis. This corresponds to allocation triangle plots like Fig. 5.5.2.2-1 and 5.5.2.2-2: the vertical axis represents the allocation size (Hz or RB) whereas the horizontal axis represents the (partial) distance from the emission mask (also Hz or RB). Correspondingly, the shapes in Fig. 5.5.2.2-5 to 5.5.2.2-11 resemble those of the regrowth region in Fig. 5.5.2.2-1 and 5.5.2.2-2. The difference is that each RB-based triangle is valid only for a single combination of CBW, SCS, and frequency offset, whereas a TGBW plot conveniently represents all CBWs, SCSs, and frequency offsets.

Figure 5.5.2.2-5: DFT-S-OFDM, pi/2-BPSK

Figure 5.5.2.2-6: DFT-S-OFDM, QPSK

Figure 5.5.2.2-7: DFT-S-OFDM, 16-QAM

Figure 5.5.2.2-8: DFT-S-OFDM, 64-QAM

Figure 5.5.2.2-9: OFDM, QPSK

Figure 5.5.2.2-10: OFDM, 16-QAM

Figure 5.5.2.2-11: OFDM, 64-QAM

In Fig. 5.5.2.2-5 to 5.5.2.2-11, each green line displays a second-order polynomial fitted to the corresponding blue data points as a nearest upper bound for the TGBW. These are TGBW thresholds, defined as

GA(BWalloc)=max⁡(0, C2BWalloc100MHz2+C1BWalloc100MHz+C0)

where the coefficients C2, C1, and C0 are defined separately for each waveform, modulation, and tentative A-MPR value A. The polynomials have been fitted in such a way that their proper ordering is guaranteed, i.e., always GA1BWalloc  GA2(BWalloc) for A1<A2, i.e., a lower back-off requires a larger TGBW. In the above polynomial, division by 100 MHz ensures convenient scaling and units for the polynomial coefficients.

The curved green TGBW threshold lines correspond to boundaries of quantized back-off values in Fig. 5.5.2.2-1 and 5.5.2.2-2. The curved shape is caused by two phenomena:

- The spectral regrowth of a very narrow allocation cannot reach the additional spurious emission mask, particularly its strictest segment. The wider the allocation, the deeper its regrowth reaches into the protected frequency region.

- Beyond a certain allocation bandwidth, the allocation PSD becomes so low that the spectral regrowth becomes weaker, eventually rendering it unable to violate the additional spurious emission mask.

The magenta lines show the approximation error for each data point. The fit is not perfect, but the resulting excess A-MPR is still much lower than in earlier methods of specifying A-MPR.

The regrowth-based A-MPR value, A-MPRregrowth, is determined through the inequalities given in Table 5.5.2.3-1. The coefficients for the TGBW threshold polynomials are given in Table 5.5.2.3-2.

Fig. 5.5.2.2-12 depicts the regrowth-based A-MPR in pseudocolor in a similar way as allocation triangles, e.g., Fig. 5.5.2.2-1. However, instead of a single channel bandwidth at given center frequency, this figure covers the entire n41 and applies to all channel bandwidths and positions as well as SCSs. For that end, it defines the RB allocation size and position in Hz rather than RBs. This allows to present the entire regrowth-based results in a single figure for each combination of waveform and modulation. The green curves in Fig. 5.5.2.2-5 to 5.5.2.2-11 are the boundaries between colors (A-MPR values) in Fig. 5.5.2.2-12. The black triangles in the figure represent allocation triangles of example channel bandwidths and positions.

Figure 5.5.2.2-12: Example of regrowth-based A-MPR across entire n41; 
black triangles are example channel bandwidths and positions

##### 5.5.2.2.3 	A-MPR based on IMD3

CIM3 may need A-MPR only when it reaches the strictest segment of the additional spurious emission mask, i.e.,

4 FC-3 falloc,high   2490.5 MHz

where

falloc,high=falloc,low+BWalloc

is the upper edge frequency of the RB allocation and BWalloc=LCRB∙12 SCS is the allocation bandwidth.

For OFDM, A-MPRCIM3 = max0,  min11,  12-2 BWalloc1 MHz dB  if the above condition holds; otherwise, 
A-MPRCIM3 = 0 dB.

For DFT-S-OFDM, A-MPRCIM3 = 3 dB if BWalloc 1.08 MHz if the above inequality holds; otherwise, 
A-MPRCIM3 = 0 dB.

Hence, OFDM is clearly more sensitive to CIM3.

##### 5.5.2.2.3 	A-MPR for edge allocations

In the case of edge allocations, the linear spectral regrowth (due to the windowing effect) of a very narrow RB allocation violates the additional spurious emission mask. Even a small shift of the allocation or frequency offset of the channel edge from the operating band edge will suffice to prevent the need for A-MPR.

For both OFDM and DFT-S-OFDM, if RBstart=0, LCRB=1,  and FC-BWChannel2-2496 MHz<360 kHz, 
A-MPRedge is defined in Table 5.5.2.3-3. Otherwise, A-MPRedge = 0 dB.

The dependence on channel bandwidth is caused by the different minimum guard bandwidths (TS 38.101-1, Table 5.3.3-1) and the fact that the measurement bandwidth in the closest additional spurious mask segment depends on the channel bandwidth.

#### 5.5.2.3 	A-MPR

It is agreed to specify the A-MPR for n41 NS_04 as follows:

For Power Class 1, NS_04 A-MPR is defined as 
A-MPR = max(MPR, A-MPRregrowth, A-MPRIMD3, A-MPRCIM3, A-MPRedge).

A-MPRregrowth is obtained from Table 5.5.2.3-1 in terms of total guard bandwidth (TGBW). The TGBW is defined as the frequency distance between the RB allocation and the additional spurious emission limit defined in Table 6.5.3.3.1-1 of TS 38.101-1, i.e.,

TGBW=falloc,low-2496 MHz,

where

falloc,low=FC-BWChannel2+BWGB+RBstart∙12 SCS

is the lower edge frequency of the RB allocation, FC is the channel centre frequency, BWChannel is the channel bandwidth, and BWGB is the minimum guard bandwidth defined in Table 5.3.3-1 of TS 38.101-1.

Table 5.5.2.3-1: A-MPRregrowth for NS_04 (Power Class 1)

| TGBW range | A-MPRregrowth (dB) |
| --- | --- |
| G0dB(BWalloc) TGBW | 0 |
| G1dB(BWalloc) TGBW < G0dB(BWalloc) | 1 |
| G2dB(BWalloc) TGBW < G1dB(BWalloc) | 2 |
| G3dB(BWalloc) TGBW < G2dB(BWalloc) | 3 |
| G4dB(BWalloc) TGBW < G3dB(BWalloc) | 4 |
| G5dB(BWalloc) TGBW < G4dB(BWalloc) | 5 |
| TGBW < G5dB(BWalloc) | 6 |

Each function GA(BWalloc) defines the required minimum total guard bandwidth for A-MPR value A and is defined as

GA(BWalloc)=max⁡(0,  C2BWalloc100MHz2+C1BWalloc100MHz+C0),

where BWalloc=LCRB∙12 SCS is the allocation bandwidth, and C2, C1, and C0 are obtained from Table 5.5.2.3-2 for each combination of waveform, modulation, and back-off value A.

Table 5.5.2.3-2: Polynomial coefficients for determining the required 
total guard bandwidth for each value of A-MPRregrowth (Power Class 1)

| Waveform | Modulation | Back-off value A [dB] / Polynomial coefficients C2, C1, C0 [MHz] |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| DFT-s-OFDM | Pi/2-BPSK | -3, 40, 10 | -91, 98, 0 | -248, 143, -9 | -2148, 604, -40 | 0,0,0 | N/A | N/A |
|  | QPSK | -27, 71, 1 | -49, 73, 0 | -76, 76, -3 | -108, 62, -3 | 0,0,0 | N/A | N/A |
|  | 16-QAM | -34, 81, -1 (NOTE 1) | -57, 92, -5 | -69, 82, -5 | -119, 79, -5 | 0,0,0 | N/A | N/A |
|  | 64-QAM | -23, 74, -1 (NOTE 1) | -61, 91, -4 (NOTE 1) | -84, 92, -5 | -116, 80, -5 | 0,0,0 | N/A | N/A |
|  | 256-QAM | 0, 0, 0 (NOTE 2) | N/A | N/A | N/A | N/A | N/A | N/A |
| CP-OFDM | QPSK | -32, 81, -2 (NOTE 1) | -36, 77, -3 | -40, 70, -3 | -52, 67, -3 | -67, 60, -3 | -101, 56, -4 | 0,0,0 |
|  | 16-QAM | -31, 80, -2 (NOTE 1) | -32, 72, -2 (NOTE 1) | -36, 66, -2 | -48, 62, -2 | -66, 58, -2 | -108, 60, -4 | 0,0,0 |
|  | 64-QAM | -28, 78, -2 (NOTE 1) | -33, 73, -2 (NOTE 1) | -36, 66, -2 (NOTE 1) | -48, 62, -2 | -65, 58, -2 | -124, 67, -5 | 0,0,0 |
|  | 256-QAM | 0, 0, 0 (NOTE 2) | N/A | N/A | N/A | N/A | N/A | N/A |
| NOTE 1: Back-off value A is less than floor(MPR), thus this polynomial is irrelevant. NOTE 2: For 256-QAM, always A-MPRregrowth = 0. |  |  |  |  |  |  |  |  |

For both OFDM and DFT-S-OFDM, A-MPRIMD3 = max0,  min3,  4-2.77 BWalloc1 MHz dB 
if  3falloc,low-2FC  2490.5 MHz; otherwise, A-MPRIMD3 = 0 dB.

For OFDM, A-MPRCIM3 = max0,  min11,  12-2 BWalloc1 MHz dB if  4 FC-3 falloc,high   2490.5 MHz; 
otherwise, A-MPRCIM3 = 0.

For DFT-S-OFDM, A-MPRCIM3 = 3 dB if BWalloc 1.08 MHz and 4 FC-3 falloc,high   2490.5 MHz; 
otherwise, A-MPRCIM3 = 0.

Here, falloc,high=falloc,low+BWalloc is the upper edge frequency of the RB allocation.

For both OFDM and DFT-S-OFDM, if RBstart=0, LCRB=1,  and FC-BWChannel2-2496 MHz<360 kHz, A-MPRedge is defined in Table 5.5.2.3-3. Otherwise, A-MPRedge = 0 dB.

Table 5.5.2.3-3: A-MPRedge for NS_04  (Power Class 1)

| Waveform | BWChannel [MHz] | BWalloc [MHz] | A-MPR [dB] |
| --- | --- | --- | --- |
| DFT-s-OFDM | < 40 | N/A | 0 |
|  | 40…90 | -9.8CBW100MHz2+24.1CBW100MHz-6.3 | 3.5 |
|  | 100 |  | 11 |
| OFDM | < 90 | N/A | 0 |
|  | 90 | 6.5 | 3.5 |
|  | 100 | 8 | 11 |

<Next change>

Annex B:
Coexistence studies for 31 dBm UE Power Class for NR Band n77

<Next change>

Annex C:
Coexistence studies for 31 dBm UE Power Class for NR Band n25

<Next change>

Annex D:
Coexistence studies for 31 dBm UE Power Class for NR Band n66

<End of change>

# References

[1]	R4-2405401, “TR 37.829 v0.9.0”, Nokia.