3GPP TSG-RAN WG4 Meeting # 111	R4-2407439

Fukuoka, Japan, 20 – 24 May 2024

Agenda Item:	10.10.2

Source: 	Nokia

Title: 	UE RF requirements for inter-band NR CA/DC with 3MHz CBW

Document for:	Approval

# 1.	Introduction

A new WI for NR channel BW less than 5MHz for FR1 Phase 2 was approved at TSG RAN#103 [1]. One of the objectives of this WI is to:

- Define common co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band

  - Example band combination: CA/DC of 3MHz in band n100 and 5MHz or 10MHz in band n101

This topic was discussed at TSG RAN4#110bis, and the Way Forward was agreed [2].

This contribution provides proposals to progress the issues that were FFS in the agreed WF.

# 2.	Discussion

Two issues were FFS in the agreed WF as follows:

## Issue 2-10: Configured transmitted power for Inter-band CA: ΔTIB,c Agreement - ΔTIB,c is FFS - Option 1: ΔTIB,c = [0.3] dB - Other option are not precluded ## Issue 2-12: Reference sensitivity: MSD requirements Agreement - FFS on MSD requirements

## Issue 2-10: Configured transmitted power for Inter-band CA: ΔTIB,c

For the UE which supports inter-band NR CA configuration, ΔTIB,c due to NR CA (two bands) are specified in table 6.2A.4.2.3-1 of TS 38.101-1 [3]. It can be seen that ΔTIB,c for CA_n8-n39 are specified as 0.3 dB for both bands.

It is also specified in clause 6.2B.4.2 of TS 38.101-1 that:

For inter-band NR-DC with one uplink carrier assigned per NR band, the ΔTIB,c for the corresponding inter-band CA configuration as specified in clause 6.2A.4.2 applies.

Since band n100 is just below band n8 and band n101 is a sub-band of band n39, there is no technical reason why ΔTIB,c for CA_n100-n101 should be larger than 0.3 dB in anyone of the two bands. Therefore, it is propose to specify ΔTIB,c for CA_n100-n101 and NR DC_n100-n101 as 0.3 dB for both bands.

## Issue 2-12: Reference sensitivity: MSD requirements

For the UE which supports inter-band NR CA configuration, the reference sensitivity exceptions due to harmonic, harmonic mixing, cross band isolation are specified in clauses 7.3A.4, 7.3A.5 and 7.3A.6 of TS 38.101-1.

The UL and DL harmonic and harmonic mixing for CA_ n100-n101 are provided in tables 1 and 2 below, respectively.

Table 1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n100 | 874.4 | 880 | 919.4 | 925 | 1748.8 | 1760 | 2623.2 | 2640 | 3497.6 | 3520 | 4372 | 4400 |
| n101 | 1900 | 1910 | 1900 | 1910 | 3800 | 3820 | 5700 | 5730 | 7600 | 7640 | 9500 | 9550 |

Table 2: Impact of UL/DL Harmonic Mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n100 | 874.4 | 880 | 919.4 | 925 | 1838.8 | 1850 | 2758.2 | 2775 | 3677.6 | 3700 | 4597 | 4625 |
| n101 | 1900 | 1910 | 1900 | 1910 | 3800 | 3820 | 5700 | 5730 | 7600 | 7640 | 9500 | 9550 |

The IMD interference analysis for CA_ n100-n101 with uplink CA is provided in table 3 below.

Table 3: CA_ n100-n101 IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 874.4 | 880 | 1900 | 1910 |
| DL Frequency [MHz] | 919.4 | 925 | 1900 | 1910 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1020 | 1035.6 | 2774.4 | 2790 |
| 3rd order IMD products | ¦fy_low – 2*fx_high¦ | ¦fy_high – 2*fx_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 140 | 161.2 | 2920 | 2945.6 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3648.8 | 3670 | 4674.4 | 4700 |
| Two-tone 4th order IMD products | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2040 | 2071.2 | 5548.8 | 5580 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 713.2 | 740 | 4820 | 4855.6 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4523.2 | 4550 | 6574.4 | 6610 |
| Two-tone 5th order IMD products | ¦fx_high – 4*fy_low¦ | ¦fx_low – 4*fy_high¦ | ¦fy_high – 4*fx_low¦ | ¦fy_low – 4*fx_high¦ |
| IMD frequency limits (MHz) | 6720 | 6765.6 | 1587.6 | 1620 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8474,4 | 8520 | 5397.6 | 5430 |
| Two-tone 5th order IMD products | ¦2*fx_high – 3*fy_low¦ | ¦2*fx_low – 3*fy_high¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3940 | 3981.2 | 1160 | 1196.8 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7448.8 | 7490 | 6423.2 | 6460 |

It can be seen that there is no harmonic, harmonic mixing or IMD interference issue for CA_ n100-n101. Therefore, it is proposed not to specify MSD for CA_n100-n101 and NR DC_n100-n101.

# 3.	Conclusion

This contribution has provided proposals to prgoress the issues that were FFS in the agreed WF in last RAN4 meeting.

Proposal 1: To specify ΔTIB,c for CA_n100-n101 and NR DC_n100-n101 as 0.3 dB for both bands.

Proposal 2: Not to specify MSD for CA_n100-n101 and NR DC_n100-n101.

# References

[1]	RP-240832, “New WID: WI resulting from discussion on NR channel BW less than 5MHz for TN”, Moderator (vivo).

[2]	R4-2406623, “WF on RF requirements for less than 5MHz band combinations”, Intel.

[3]	3GPP TS 38.101-1 v18.5.0: “NR; User Equipment (UE) radio transmission and reception”.