3GPP TSG-RAN WG4 Meeting #111	R4-2407548

Fukuoka, Japan, May 20th ‒ 24th, 2024

Title: 	Further discussion on UE RF requirements for inter-band NR CA/DC with 3MHz CBW

Agenda Item:	10.10.2

Source: 	CATT

Document for:	Approval

# 1 Introduction

For UE RF requirements for inter-band NR CA/DC with 3MHz channel bandwidth, RAN4#110bis has commenced the works and agreed a WF where several open issues require further discussions [1]:

## Issue 2-4: Power class for UL inter-band CA Agreement (Wed online discussion): - Define PC3 inter-band CA/DC combinations - FFS on whether to define PC1 inter-band CA/DC combinations ## Issue 2-6: Signalling aspects Agreement - FFS whether to introduce new UE capability and/or other RRC signalling to support NR CA/DC with less than 5MHz CBW and support of 12 or 20 RB transmission bandwidth ## Issue 2-10: Configured transmitted power for Inter-band CA: ΔTIB,c Agreement - ΔTIB,c is FFS - Option 1: ΔTIB,c = [0.3] dB - Other option are not precluded ## Issue 2-12: Reference sensitivity: MSD requirements Agreement - FFS on MSD requirements

In this contribution, we offer our opinions and viewpoints concerning these open issues.

# 2 Discussion

Power class

According to the latest UE specs [2], n100 has already supported PC1 for non-smartphone form factors, though it seems not a good practice with lack of PC1.5 or PC2 requirements as some other bands like n14, n31, n71, n72, n85, n101 etc. However, for a band combination, all fallback with lower capabilities should be defined at first before a band combination with higher capabilities is specified, hence it is not ready to define PC1 inter-band CA/DC for the band combination CA_n100A-n101A.

Moreover, in the current WID [3], the power class is not clearly indicated, therefore, RAN plenary intervention is required if introducing PC1.

Proposal 1: RAN4 not to define PC1 inter-band CA/DC for the band combination CA_n100A-n101A until receiving approval from RAN plenary.

Signaling aspects

For the open issue on whether to introduce new UE capability and/or other RRC signalling to support NR CA/DC with less than 5MHz CBW and support of 12 or 20 RB transmission bandwidth, current RAN2 signaling design has already had the full freedom to report the support of NR/DC with less than 5MHz CBW and 12 or 20 RB transmission bandwidth, therefore, there is no need to introduce a new UE capability for this purpose.

Proposal 2: RAN4 not to introduce a new UE capability to support NR CA/DC with less than 5MHz CBW and support of 12/20 RB transmission bandwidth.

ΔTIB,c

The determination of ΔTIB,c heavily relies on the evaluation and inputs provided by vendors. RAN4 may specify the requirement accordingly, given sufficient inputs from vendors.

Proposal 3: RAN4 to specify the requirement ΔTIB,c according to sufficient inputs and evaluation from vendors.

MSD requirements

For 2UL configuration of the band combination CA_n100A-n101A, the following table indicates that there is no need to define MSD for this band combination.

| UL frequency | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
|  | 874.4 | 880 | 1900 | 1910 |
| 2nd order harmonics frequency range (MHz) | 1748.8 | 1760 | 3800 | 3820 |
| 3rd order harmonics frequency range (MHz) | 2623.2 | 2640 | 5700 | 5730 |
| 2nd order IMD products | ¦fy_low-fx_high¦ | ¦fy_high-fx_low¦ | ¦fy_low+fx_low¦ | ¦fy_high+fx_high¦ |
| IMD frequency limits (MHz) | 1020 | 1035.6 | 2774.4 | 2790 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 161.2 | 140 | 2920 | 2945.6 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3648.8 | 3670 | 4674.4 | 4700 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2071.2 | 2040 | 5548.8 | 5580 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 713.2 | 740 | 4820 | 4855.6 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4523.2 | 4550 | 6574.4 | 6610 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6765.6 | 6720 | 1620 | 1587.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8474.4 | 8520 | 5397.6 | 5430 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3981.2 | 3940 | 1160 | 1196.8 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7448.8 | 7490 | 6423.2 | 6460 |

Proposal 4: RAN4 not to specify the requirement MSD for the band combination CA_n100A-n101A.

# 3 Conclusion

In this contribution, we have the following proposals for UE RF requirements for inter-band NR CA/DC with 3MHz CBW:

Proposal 1: RAN4 not to define PC1 inter-band CA/DC for the band combination CA_n100A-n101A until receiving approval from RAN plenary.

Proposal 2: RAN4 not to introduce a new UE capability to support NR CA/DC with less than 5MHz CBW and support of 12/20 RB transmission bandwidth.

Proposal 3: RAN4 to specify the requirement ΔTIB,c according to sufficient inputs and evaluation from vendors.

Proposal 4: RAN4 not to specify the requirement MSD for the band combination CA_n100A-n101A.

# Reference

- R4-2406623, WF on UE RF requirements for NR channel BW less than 5MHz for FR1 Phase 2, Intel.

- 3GPP TS 38.101-1 18.5.0

- RP-240832, New WID: WI resulting from discussion on NR channel BW less than 5MHz for TN, Moderator(vivo)