3GPP TSG-RAN WG4 Meeting #111                                           R4-2307612

Fukuoka, Japan, May 20 – 24, 2024

Source: 	Huawei, HiSilicon

Title: 	On Tx requirements for sidelink intra-band contiguous CA in PC2 in Rel-19

Agenda Item:	10.9.3.2

Document for:	Approval

Introduction

The New WID revision [1] was approved in RAN#103 with the scope for sidelink contiguous and non-contiguous CA operation in ITS band.

- Specify the RF requirements for NR sidelink CA in n47: - Intra-band non-contiguous CA with power class 2 and power class 3, with Component Carrier (CC) combinations 10MHz + 10MHz and 10MHz + 20MHz - Intra-band contiguous CA with power class 2 - Introduce necessary changes for release independence specification, if identified Note: Work for power class 3 starts first in RAN4. Whether there is any RRM core requirement to be specified will be confirmed in Q3 2024.

In this contribution, we provide our consideration and initial analysis on RAN4 work for NR sidelink intra-band contiguous CA with power class 2. The work in Rel-19 would include at least intra-band contiguous CA band combination(s) and configurations in ITS band, Tx requirements mainly on maximum output power and MPR for PC2.

Discussion

In the WID for sidelink CA, intra-band contiguous CA with PC2 is to be considered. For most of the requirements, there is no different between PC3 and PC2 of intra-band contiguous CA. In general, there would be at least two aspects that need to be updated for PC2 intra-band contiguous SL CA, based on PC3 SL CA.

- Minimum requirements for Power Class 2 should be applicable for SL intra-band contiguous CA combination configuration, and update the maximum output power for sidelink CA

- MPR/A-MPR requirements for intra-band contiguous SL CA with PC2, including, PSSCH/PSCCH, PSFCH and S-SSB.

Proposal 1: For PC2 intra-band contiguous SL CA, the following two aspects should be specified in Rel-19

- Minimum requirements for Power Class 2 should be applicable for SL intra-band contiguous CA combination configuration, and update the maximum output power for sidelink CA

- MPR/A-MPR requirements for intra-band contiguous SL CA with PC2, including, PSSCH/PSCCH, PSFCH and S-SSB.

In Rel-18 intra-band contiguous SL CA with PC3, the simulation assumptions were agreed to reuse the simulation assumptions in TR38.785 (Rel-17 enhanced NR sidelink). Other constraints for PSCCH/PSSCH/PSFCH/S-SSB can be assumed roughly based on the agreement in Rel-18.

Table 1 General simulation assumptions for intra-band contiguous SL CA with PC2

| Center frequency | 5.9GHz |
| --- | --- |
| Bandwidth | per CC: 10/20/30/40MHz Aggregated CBW: Table 5.2.3-1 (up to 70MHz CBW) |
| Maximum output power for aggregated CBW | 23dBm ->26dBm |
| Numerology | 15 kHz/30kHz/60kHz |
| Modulation per CC | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM |
| Carrier leakage | 34dBc -> 25dBc |
| IQ image | 25dBc |
| CIM3 | 60dBc |
| PA calibration | PA calibrated to deliver 30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case. |

Table 2 Intra-band contiguous SLCA PSSCH/PSCCH with PC2 MPR simulation scenarios

| Scenario | Aggregated CBW | Subcarrier spacing | Waveform | Modulation |
| --- | --- | --- | --- | --- |
| 1 | 10MHz + 10MHz | 15kHz+15kHz | CP-OFDM | QPSK/16QAM/64QAM/256QAM |
| 2 | 10MHz + 10MHz | 30kHz+30kHz | CP-OFDM | QPSK/16QAM/64QAM/256QAM |
| 3 | 30MHz + 40MHz | 30kHz+30kHz | CP-OFDM | QPSK/16QAM/64QAM/256QAM |
| 4 | 20MHz + 20MHz | 30kHz+30kHz | CP-OFDM | QPSK/16QAM/64QAM/256QAM |

Table 3 Detailed Evaluation assumptions of PSFCH for intra-band contiguous SL CA with PC2

| Frequency band | n47 |
| --- | --- |
| Bandwidth | 10MHz+10MHz/ 20MHz+20MHz |
| Maximum output power | Power class 2: 26dBm |
| numerology | 30kHz |
| Modulation | QPSK |
| PSFCH | ZC sequence |
| CIM3 | 60dBc |
| PA calibration | PA calibrated to deliver 30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case. |

Based on the simulation assumptions above, intra-band contiguous SL CA with PC2 should be specified in Rel-19 referring to the methodology of that with PC3 in Rel-18.

- For PSSCH/PSCCH: specify MPR for power class 2 contiguous CA with contiguous RB allocation, including inner and outer.

- For PSFCH: specify MPR for PC2 contiguous CA based on different range of R, R is the ratio of the gap bandwidth between the two PSFCH transmitted on the two intra-band carrier by the total bandwidth of the two carrier

- For S-SSB: specify MPR for PC2 contiguous CA by single S-SSB and two S-SSB, separately.

Proposal 2: For the MPR evaluation of intra-band contiguous CA with PC2, the simulation assumptions as follows should be agreed

| Center frequency | 5.9GHz |
| --- | --- |
| Bandwidth | per CC: 10/20/30/40MHz Aggregated CBW: Table 5.2.3-1 (up to 70MHz CBW) |
| Maximum output power for aggregated CBW | 26dBm |
| Numerology | 15 kHz/30kHz/60kHz |
| Modulation per CC | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM |
| Carrier leakage | 25dBc |
| IQ image | 25dBc |
| CIM3 | 60dBc |
| PA calibration | PA calibrated to deliver 30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case. |

Proposal 3: Intra-band contiguous SL CA with PC2 should be specified in Rel-19 referring to the methodology of that with PC3 in Rel-18.

- For PSSCH/PSCCH: specify MPR for power class 2 contiguous CA with contiguous RB allocation, including inner and outer.

- For PSFCH: specify MPR for PC2 contiguous CA based on different range of R, R is the ratio of the gap bandwidth between the two PSFCH transmitted on the two intra-band carrier by the total bandwidth of the two carrier

- For S-SSB: specify MPR for PC2 contiguous CA by single S-SSB and two S-SSB, separately.

Conclusion

This contribution provides our consideration and initial analysis on RAN4 work for Rel-19 SL intra-band contiguous CA with PC2 with the following proposals:

Proposal 1: For PC2 intra-band contiguous SL CA, the following two aspects should be specified in Rel-19.

- Minimum requirements for Power Class 2 should be applicable for SL intra-band contiguous CA combination configuration, and update the maximum output power for sidelink CA

- MPR/A-MPR requirements for intra-band contiguous SL CA with PC2, including, PSSCH/PSCCH, PSFCH and S-SSB.

Proposal 2: For the MPR evaluation of intra-band contiguous CA with PC2, the simulation assumptions as follows should be agreed

| Center frequency | 5.9GHz |
| --- | --- |
| Bandwidth | per CC: 10/20/30/40MHz Aggregated CBW: Table 5.2.3-1 (up to 70MHz CBW) |
| Maximum output power for aggregated CBW | 26dBm |
| Numerology | 15 kHz/30kHz/60kHz |
| Modulation per CC | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM |
| Carrier leakage | 25dBc |
| IQ image | 25dBc |
| CIM3 | 60dBc |
| PA calibration | PA calibrated to deliver 30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case. |

Proposal 3: Intra-band contiguous SL CA with PC2 should be specified in Rel-19 referring to the methodology of that with PC3 in Rel-18.

- For PSSCH/PSCCH: specify MPR for power class 2 contiguous CA with contiguous RB allocation, including inner and outer.

- For PSFCH: specify MPR for PC2 contiguous CA based on different range of R, R is the ratio of the gap bandwidth between the two PSFCH transmitted on the two intra-band carrier by the total bandwidth of the two carrier

- For S-SSB: specify MPR for PC2 contiguous CA by single S-SSB and two S-SSB, separately.

- References

[1] RP-240840 New WID on NR Sidelink: Intra-band Carrier Aggregation in ITS band