3GPP TSG-RAN WG4 Meeting #111	R4-2407742

Fukuoka, Japan, May 20-24, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM performance aspects on SSB-less operation

Agenda item:	7.24.2.1

Document for:	Discussion

Introduction

At RAN4#110bis meeting, the principle for test cases design were further discussed and the draft BigCR was agreed in [1]. In this paper, we are discussing some other aspects on A-TRS based SSB-less SCell activation test cases.

Discussion

The following has been agreed on SSB-less SCell activation from last meeting. It was agreed single P-TRS is configured in SSB-less SCell in the P-TRS based SCell activation test cases. A-TRS based SCell activation was not discussed.

Issue 2-1-1: Test configurations for SSB-less

- Agreement on Test configurations for SSB-less

  - Single P-TRS is configured in SSB-less SCell in the SSB-less SCell activation TCs

  - Don’t configure the parameter SCell measurement cycle (measCycleSCell), SSB configuration and SMTC configuration for SSB-less SCell

  - Set EPRE difference as [9 dB] + [ΔPL] in the test cases.

    - the [9] can be updated to the same number as in core requirement if any updates on the core requirements.

    - Pcell has [higher or lower] EPRE than Scell.

    - Further discuss whether and how [ΔPL] can be reflected in the TC.

      - ΔPL is the power difference caused by BW difference and/or frequency difference between two Cells.

      - Encourage concrete proposals on how ΔPL due to BW difference and PL difference can be reflected in the test.

  - The TCI.State.0 could be used for the TRS of the SSB-less SCell with an additional note that the reference signal is SSB0 from spCell.

For A-TRS based SCell activation, it has been designed following R17 fast SCell activation. That is, the A-TRS index for SCell activation shall be indicated in the SCell activation command, which is named as enhanced SCell activation command according to TS38.321.

Observation#1: The A-TRS index is indicated in SCell activation command based on R17 fast SCell activation.

In the endorsed TCs for A-TRS based SCell activation, A-TRS is configured with TCI.state.0 which indicates QCL-typeC with single SSB0 as cited below. Then the QCL source cell containing the SSB0 shall be taken as the default reference cell. Hence the explicit indication of reference cell is not needed for A-TRS based SCell activation TCs.

Observation #2: The A-TRS in TCs is configured with QCL-typeC with one source cell and this QCL source cell shall be taken as the default reference cell.

Proposal 1: For A-TRS based SCell activation TCs, explicit indication of reference cell is not needed.

### A.3.16.2	TCI states

Table A.3.16.2-1: TCI States

| Parameter | TCI.State.0 | TCI.State.1 | TCI.State.2 | TCI.State.3 |
| --- | --- | --- | --- | --- |
| tci-StateId | Id0 | Id1 | Id2 | Id3 |
| qcl-Type1 | typeC | typeC | typeA | typeA |
| qcl-Type2Note1 | typeD | typeD | typeD | typeD |
| referenceSignal | SSB0 | SSB1 | Resource #4 in TRS resource set 1 Note3 | Resource #4 in TRS resource set 2 Note3 |
| Note 1: qcl-Type2 of typeD only where applicable. For RRM test cases, this will be only in FR2 Note 2: referenceSignal configurations towards which the TCI states are configured are defined in a test-specific manner. Note 3: Reference TRS resource sets are defined in A.3.17, and the applicable TRS resource set(s) are specified in each test case. When a single TRS resource set is configured in a test case, it is considered as resource set 1. |  |  |  |  |

In order to verify the explicit signaling of reference cell, we propose defining a test case with multiple QCL source cells. This needs to be based on the P-TRS for activating a SSB-less SCell.

Proposal 2: To define a P-TRS based SCell activation TC with multiple QCL source cells to verify the explicit signalling of reference cell.

Conclusion

In this paper we have made the following proposals and observations related to the RRM performance aspects:

Observation#1: The A-TRS index is indicated in SCell activation command based on R17 fast SCell activation.

Observation #2: The A-TRS in TCs is configured with QCL-typeC with one source cell and this QCL source cell shall be taken as the default reference cell.

Proposal 1: For A-TRS based SCell activation TCs, explicit indication of reference cell is not needed.

Proposal 2: To define a P-TRS based SCell activation TC with multiple QCL source cells to verify the explicit signalling of reference cell.

References

[1] R4-2406520 Draft Big CR to TS 38.133 on performance requirements for NR Network energy saving, Huawei