# 3GPP TSG-RAN WG4 Meeting #111	R4-2407886

# Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Agenda item:	10.14.4

Source:	OPPO

Title:	Discussion on core requirements for LP-WUS/WUR

Document for:	Approval

- Introduction

RANP #103 meeting revised the WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR) [1] and necessary RRM impacts are expected to be discussed in RAN4. Based on the conclusions reached during previous meeting [2], this contribution will provide our considerations on this topic.

- Discussion

- Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

Issue 2-2-2: Core requirements to be specified for MR RRM relaxation Proposals - P1: RAN4 specifies MR relaxation requirements at idle/inactive mode for UE supporting LP-WUR (vivo LG CMCC Nokia Huawei ZTE). - P2: RAN4 needs further discussion on MR RRM requirements - Discusses MR RRM relaxation for both serving cell and neighbour cell measurements. (CATT) - RAN4 needs further evaluate serving cell measurement relaxation by MR when LP-WUR is enabled, neighbour cell evaluation can be discussed after further input from RAN2. (Samsung) - RAN4 to discuss the impact on the serving cell evaluation requirements when using the following signals: LP-SS, legacy SSS, hybrid LP-SS and SSS and identify corresponding requirements, whether/when/how to relax the UE MR requirement and whether/when/how to cease/relax/initiate neighbour cells’ measurement (Ericsson) - The criteria design for RRM relaxation of UE MR for both serving and neighbor cell measurements can be left to RAN2 to decide, but RAN4 can investigate the mobility performance to quantify the relaxation, e.g., scaling factor for measurement period. (Apple)

Based on the WID [1], further RRM relaxation of MR measurement for both serving cell and neighbor cells should be specified. The above P1 and first bullet of P2 are aligned with this objective and can be supported.

Proposal 1: Specify MR relaxation requirements for both serving cell and neighbor cell measurements in RRC idle/inactive mode.

Issue 2-2-3: Criteria (entry/exit conditions) for LP-WUR measurement Proposals - P1: RAN2 is be the main group for criteria (entry/exit conditions) design (vivo Qualcomm) - P1-1: RAN4 shall work on defining the RRM measurement relaxations and/or RRM measurement offloading mechanisms considering the criteria defined by RAN2 (Qualcomm) - P2-1: Study other conditions for the entry/exit condition for LP-WUS monitoring other than the measurement based condition from the RAN4 pespective (LG) - P2-2: RAN4 to confirm that same entry/exit condition for LP-WUS monitoring applies for serving cell RRM measurement offloading (from MR to LP-WUR) (Huawei) - P3: Considerations on criteria and entry/exit conditions - RAN4 to consider the necessary conditions for RSRP/RSRQ measurement after offloading from MR to LP-WUR, such as the NF difference between LP-WUR and MR and applicable coverage conditions of LP-WUR (CATT) - The existing RRM relaxed criteria for intra-frequency and inter-frequency measurement in RRC IDLE could be considered as the starting point to specify the conditions to offload serving cell measurement from MR to LP-WUR (oppo) - RAN4 to discuss the conditions to enter/exit the LP-WUS evaluation of serving cell based on the RSRP/RSRQ simulation results. (Ericsson) - RAN4 to define requirements for LP-WUR Entry and Exit criteria based on RAN1 and RAN2 design. FFS if RRM simulations for entry and exit criteria are needed. (Nokia)

Issue 2-2-4: Criteria (entry/exit conditions) for MR RRM measurement relaxation Proposals - P1: RAN2 is be the main group for criteria (entry/exit conditions) design (Apple vivo Qualcomm) - P2: For the Measurement based on both MR with RRM relaxation (X time relaxation) and LP-WUR, both MR measurement result and LP-WUR measurement result can be used to determine the link condition, how to define the relaxation criteria should be further studied (CMCC) - P3: RAN4 to discuss scenarios for MR relaxation upon LP-WUR entry and exit (Nokia) - P4: Discuss if triggering conditions for intra/inter-frequency and inter-RAT measurement based on LP-WUR are possible or if other conditions should be defined (LG)

Companies’ views on the criteria (entry/exit conditions) for LP-WUR measurement or MR RRM relaxation are provided and summarized in the last meeting. Basically, the existing approach for RRM relaxation by comparing measured RSRP/RSRQ with a configured threshold is considered. But the details should be up to RAN2/RAN1 and further progress may be needed. For example, new measurement metrics LP-RSRP and LP-RSRQ are introduced for OOK-based LP-WUR and they are considered for exit conditions for LP-WUR measurement. Another approach is to use the entry/exit conditions for LP-WUS monitoring, which are still under discussion in RAN1.

Proposal 2: The criteria (entry/exit conditions) for LP-WUR measurement or MR RRM relaxation should be determined by RAN2/RAN1.

Issue 2-2-2: Noise figure and others impact(RF impairment) on SNR target X Proposals - P1: More discussion is needed to decide if SNR of -3dB(mainly for serving cell) or -6dB(mainly for neighbor cell) can be used to represent the equivalent coverage as PUSCH for msg3. (Apple) - P2: The SNR target X input from RAN1 should be adjusted by considering the practical NF of a LP-WUR receiver. Assuming the NF gap ∆ is ∆ = NF (LP-WUR) – NF (MR), then the new SNR target X adjusted after considering the NF of LP-WUR is X = X - ∆ (vivo)

Conclusion (in RAN1 #116bis): For calibration purposes, companies are encouraged to report the SNR to achieve the coverage of PUSCH for message3, at least with the following assumptions: - Carrier frequency: 2.6 GHz - The number of Tx chains: 1 - MIL of MSG 3: use the average one in R17 coverage, i.e.,153.51 dB for non-redcap UE - Transmit antenna gain correction factors for WUS: up to company report - Noise Figure: All three values +2dB, +5dB, +8dB on top of NF of MR (7dB) are to be reported, SNR for different assumptions on NF are determined separately

It is proposed to consider the impacts on target SNR due to noise figure, for example to adjust the target SNR by the noise figure gap between MR and LP-WUR. According to the assumptions shown above, three different noise figure values are considered for different types of LP-WUR. And, we think this impact has already been considered in RAN1 when deriving the SNR. RAN4 does not need to consider it again.

Proposal 3: Not consider adjustment on the target SNR from RAN1 due to the noise figure of LP-WUR since it has already been considered in RAN1 assumption.

Issue 2-2-4: Measurement metrics Proposals - P1: RAN4 to discuss the metrics for defining entry/exit condition for LP-WUS monitoring. At least RSRP is considered, and other metrics are FFS. (Huawei)

Agreement (in RAN1 #116bis) From RAN1 perspective, at least the following metrics can be supported for RRM serving cell measurement performed by OOK-based receiver based on LP-SS: - LP-RSRP - LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols. - FFS: How to determine the received power of LP-SS in OOK ON symbols - LP-RSRQ - LP-RSRQ = LP-RSRP/LP-RSSI - For the definition of LP-RSSI for determination of LP-RSRQ, further consider the following options: - Option 1: LP-RSSI is the linear average of total received power in all LP-SS OOK symbols. - Option 2: LP-RSSI is the linear average of total received power in LP-SS OOK OFF symbols. - Option 3: LP-RSSI is the linear average of total received power in LP-SS OOK ON symbols. - FFS: LP-SINR, Power ratio of OOK-ON symbol and OOK-OFF symbol Note: RAN1 will send an LS to RAN2 and RAN4 on the measurement metrics that can be supported from RAN1 perspective, to facilitate RAN2/RAN4 discussions. The exact metrics for OOK-based receiver to be used and defined in the specifications depend on the outcome of [RAN1]/RAN2/RAN4 discussions.

As mentioned before, the measurement metric like RSRP/RSRQ is considered to define the entry/exit conditions of LP-WUR measurement. Considering the newly introduced OOK waveform, RAN1 agreed to support at least LP-RSRP/LP-RSRQ based on OOK symbols as measurement metrics. If no technology issue is identified, the agreed LP-RSRP/LP-RSRQ should be considered in RAN4.

Proposal 4: Follow RAN1 agreement that at least LP-RSRP/LP-RSRQ for OOK-based LP-WUR should be considered.

Agreement (in RAN1#116) - Multi-beam operations are supported for LP-WUS and LP-SS for idle mode

As per RAN1 agreement, multi-beam operations are supported for LP-WUS and LP-SS. When defining measurement delay with multi-beam scenario, a Rx beam sweeping factor should be considered. However, whether 8 Rx beams or a smaller number of Rx beams can be assumed for LP-WUR is not clear at this stage and further discussion is needed.

Proposal 5: Discuss the Rx beam sweeping factor to define delay requirements in case of multiple-beam.

Issue 2-5-1: LP-WUR at CONNECTED mode Proposals - P1: No RRM objectives is for connected mode in this WI. (Apple vivo Xiaomi LG Nokia, ZTE) - P1a: No impact on existing RRM requirements at connected mode (Samsung) - P2: Consider this issue later - FFS whether need to introduce LP-WUS monitoring activation and deactivation delay requirements pending on RAN2/RAN1 progress (Samsung) - RAN4 to wait for RAN1 and RAN2 progress before defining any requirements for CONNECTED mode. (Nokia) - RAN4 should address issues, if any, related to CONNECTED mode at a later stage (Qualcomm)

WID objective: - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

Based on the WID objective [1], MR ultra-deep sleep is not considered and the RRM/RLM/BFD/CSI measurements are supposed to be performed by MR in RRC connected mode. With this guidance, at least we can agree on that no impact is needed for the existing measurement delay and accuracy requirements, including RRM/RLM/BFD/L1-RSRP. Whether/how to define requirement for LP-WUS monitoring activation and deactivation delay or other procedure can be further discussed or trigged by RAN1/RAN2 conclusions at a later stage.

Proposal 6: No impact on the existing RRM/RLM/BFD/L1-RSRP measurement requirements in RRC connected mode.

- Conclusion

This contribution gave the following proposals on the core requirements for LP-WUR/WUS.

Proposal 1: Specify MR relaxation requirements for both serving cell and neighbor cell measurements in RRC idle/inactive mode.

Proposal 2: The criteria (entry/exit conditions) for LP-WUR measurement or MR RRM relaxation should be determined by RAN2/RAN1.

Proposal 3: Not consider adjustment on the target SNR from RAN1 due to the noise figure of LP-WUR since it has already been considered in RAN1 assumption.

Proposal 4: Follow RAN1 agreement that at least LP-RSRP/LP-RSRQ for OOK-based LP-WUR should be considered.

Proposal 5: Discuss the Rx beam sweeping factor to define delay requirements in case of multiple-beam.

Proposal 6: No impact on the existing RRM/RLM/BFD/L1-RSRP measurement requirements in RRC connected mode.

References

[1] RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO, Ericsson, MediaTek, Samsung, Sony, RANP #103

[2] R4-2406367, WF for RRM requirements for LP-WUSWUR, vivo, RAN4 #110bis

[3] Chairman Notes, 3GPP RAN1#116.

[4] Chairman Notes, 3GPP RAN1#116bis.