3GPP TSG-RAN WG4 Meeting # 111	R4-2408069

# Fukuoka City, Fukuoka, Japan, May 20 – 24, 2024

Source: 	China Telecom

Title: 	On SRS IL imbalance

Agenda Item:	10.1.1.4.4 SRS IL imbalance

Document for:	Approval

Introduction

In the way forward at last RAN4 meeting [1], the future discussion focus and candidate solutions for the SRS IL imbalance have been clarified. The candidate solutions for the SRS IL imbalance is concentrating on the report issue.

Way forward: Further discuss the topic of SRS IL imbalance issues with the following proposals used as a starting point. Whether a solution is agreeable, depends on further discussion of the following candidate solutions and/or identified issues. - Proposal 1: Study the issues of insertion loss imbalance across SRS ports. - Proposal 2: Further discuss SRS IL for 6RX type of devices taking into consideration: - SRS insertion loss requirements for 6RX UEs - SRS IL impact on performance - SRS insertion loss compensation - UE assistance on SRS insertion loss (power imbalance) - Others - Proposal 3: Companies should, independent of their view, provide constructive technical inputs on SRS TX power imbalance topic.

So in this contribution, we will elucidate our points about it.

Discussion

- 2.1 General aspect

Based on the previous discussion among RAN1/4, we can regard SRS IL compensation behaviour as influenced by UE implementation and maximum output power configured by NW. The actually power needed for SRS transmission can be assume as:

PSRS,total = PSRS,b,f,c(I,qs,l) + PIL

Where PSRS,b,f,c(I,qs,l) can be calculated according to TS 38.213.

If PSRS,total ≤ PCMAX_H,f,c, which means the power headroom is enough for UE to compensate the SRS IL (i.e. if PSRS,total ≤ PCMAX,f,c, UE can simply increase PSRS,total to compensate IL and do not have to change PCMAX,f,c; if PSRS,total ≥ PCMAX,f,c, UE also have chance to compensate IL through increasing PCMAX,f,c since the upper bound will not be exceeded.);

If PSRS,total ≥ PCMAX_H,f,c, UE is impossible to compensate the whole SRS IL, but can still compensate partial of it by extending output power to PCMAX_H,f,c.

No matter what strategy UE takes to compensate the imbalance, a situation will arise that UE cannot compensate all the SRS IL due to limit power headroom, so NW will misunderstand the actual SRS power and cannot perform compensation at it is side. Therefore, an SRS IL imbalance reporting mechanism should exist to handle the mismatch between UE and NW when UE cannot compensate the total IL.

Obsveration1: UE may not compensate all the SRS IL due to limit power headroom regardless of the UE implementation.

Proposal1: An SRS IL imbalance reporting mechanism should be adopted to handle the mismatch between UE and NW when UE cannot compensate the total IL.

- 2.2 reporting issue

SRS IL is based on scenarios and it changes when SRS antenna switching mode changes. The upper bound of IL is ∆TRxSRS, which is a fix value based on SRS antenna switching scenarios, but the actual IL may be different from UEs since the UE implement.

Static reporting could report ∆TRxSRS (or another fix value) at different SRS antenna switching mode to NW. But the actual IL is influenced by power headroom and it changes when PCMAX,f,c changes configured by NW (i.e. TPC command). UE can compensate partial of it. Static reporting is a compromise between complexity and accuracy.

Dynamic reporting can reflect the actual information in real time, therefore it is more helpful for gNB to take steps. Just like DPC reporting, dynamic reporting based on trigger- execute mechanism will let NW have options to acquire more information timely。

Continuing follow the RAN4 tradition that define the requirement based on the worst case, the static reporting is more sensible. But if we want more explicit report and complexity is tolerable, dynamic method should be considered.

Proposal2: IL imbalance reporting is necessary, and further discuss whether static or dynamic way should be adopted.

Motivated by UE strategy and network demand, both static and dynamic way need to consider reporting granularity, i.e. per band or per BC or per UE or per SRS antenna switching mode or per SRS resource, and analyse the corresponding impact. Besides, assistance information like MOP or power of SRS port also should be considered.

Proposal3: Both static and dynamic SRS IL report need to consider reporting granularity and assistance information.

For dynamic way, clear trigger condition should be specified in RAN4. Setting a power or IL threshold may be acceptable.

Proposal4: For dynamic way, clear trigger condition should be specified in RAN4.

Conclusion

Obsveration1: UE may not compensate all the SRS IL due to limit power headroom regardless of the UE implementation.

Proposal1: An SRS IL imbalance reporting mechanism should be adopted to handle the mismatch between UE and NW when UE cannot compensate the total IL.

Proposal2: IL imbalance reporting is necessary, and further discuss whether static or dynamic way should be adopted.

Proposal3: Both static and dynamic SRS IL report need to consider reporting granularity and assistance information.

Proposal4: For dynamic way, clear trigger condition should be specified in RAN4.

References

- R4-2406585, WF on requirements for 6Rx, RAN4#110-bis