3GPP TSG-WG4 Meeting #111	R4-2408197

# Fukuoka, Japan, 20 - 24 May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.108 | CR | 0063 | rev |  | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.108: Minimum requirements for unwanted emissions for SAN (rel-17) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2024-5-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current specifications for SAN RF requirements, munimum requirements for unwanted emissions are missing. In the corresponding sub-clauses, basic limits are specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For SAN type 1-H, rename the subclause titles for basic limits and add new subclauses for minimum requirements. For SAN type 1-O, add text to specify the minimum requirements. Typo correction in the OOBE basic limits table. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Minimum requirements are not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.4.2, 6.6.4.3, 6.6.5.2, 6.6.5.3, 9.7.5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------Start of change-------------

6.6.4	Out-of-band emissions

6.6.4.1	General

Unless otherwise stated, the out-of-band emission (OOBE) limits for SAN in FR1-NTN are defined from BWSAN channel edge up to frequencies separated from the BWSAN channel edge by 200% of the necessary bandwidth, where the necessary bandwidth is BWSAN.

The requirements shall apply whatever the type of transmitter considered and for all transmission modes foreseen by the manufacturer’s specification.

Basic limits are specified in the tables below, where:

-	f is the separation between the BWSAN channel edge frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the channel edge frequency and the centre of the measuring filter.

-	ΔSat_ClassdB is the SAN class parameter in dB identified to characterize different SAN classes.

6.6.4.2	Basic limits

For SAN operating in Bands n256, n255, n254, the requirements are specified in table 6.6.4.2-1 for GEO and LEO class respectively, in line with Annex 5 of ITU recommendation SM.1541-6 [9].

The SAN out-of-band emissions(OOBE) requirements for GEO and LEO classes are therefore defined as described in Table 6.6.4.2-1 below.

Table 6.6.4.2-1: SAN LEO and GEO Classes OOBE basic limits

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (dBm) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 2× BWSAN | 0.002 MHz f_offset < 2× BWSAN + 0.002 MHz | maxSE limit, Prated,t,sys – 10log10(BWSAN) – 24 – ΔSat_ClassdB-40×log10 f_offset-0.002BWSANBWSAM×2+1dBm | 4 kHz |
| NOTE 1: BWSAN is in the unit of MHz. NOTE 2: SE limit is spurious emission limit specified in spurious emission clause 6.6.5. NOTE 3: PSD attenuation as in ITU-R SM.1541-6 [9], Annex 5 OoB domain emission limits for space services. NOTE 4: ΔSat_ClassdB=0 dB for GEO class and ΔSat_ClassdB=3 dB for LEO class. |  |  |  |

6.6.4.3	Minimum requirements for SAN type 1-H

The out-of-band emissions minimum requirements for SAN type 1-H are that for each TAB connector TX min cell group and each applicable basic limit in clause 6.6.4.2, the power summation emissions at the TAB connectors of the TAB connector TX min cell group shall not exceed a SAN limit specified as the basic limit + X, where X = 10log10(NTXU,countedpercell).

NOTE:	Conformance to the SAN type 1-H spurious emission requirement can be demonstrated by meeting at least one of the following criteria as determined by the manufacturer:

1)	The sum of the emissions power measured on each TAB connector in the TAB connector TX min cell group shall be less than or equal to the limit as defined in this clause for the respective frequency span.

Or

2)	The unwanted emissions power at each TAB connector shall be less than or equal to the SAN type 1-H limit as defined in this clause for the respective frequency span, scaled by -10log10(n), where n is the number of TAB connectors in the TAB connector TX min cell group.

6.6.5	Transmitter spurious emissions

6.6.5.1	General

The transmitter spurious emission limits shall apply from 30 MHz to the fifth harmonic of the upper frequency edge of the DL operating band, excluding the SAN transponder bandwidth BWSAN and the frequency range where the out-of-band emissions apply. For some operating bands, the upper limit is higher than 12.75 GHz in order to comply with the 5th harmonic limit of the downlink operating band, as specified in ITU-R recommendation SM.329 [2].

The requirements shall apply whatever the type of transmitter considered (single carrier or multi-carrier). It applies for all transmission modes foreseen by the manufacturer's specification.

Unless otherwise stated, all requirements are measured as mean power (RMS).

6.6.5.2	Basic limits

6.6.5.2.1	General transmitter spurious emissions requirements

The requirements in table 6.6.5.2.1-1 shall apply. The application of those limits shall be the same as for out-of-band emissions in clause 6.6.4.

Table 6.6.5.2.1-1: General SAN transmitter spurious emission limits in FR1-NTN

| Spurious frequency range | Prated,t,sys (dBm) | Basic limit (dBm) | Measurement bandwidth (kHz) | Notes |
| --- | --- | --- | --- | --- |
| 30 MHz – 5th harmonic of the upper frequency edge of the DL operating band | ≤ 47 | -13 | 4 | NOTE 1, NOTE 2, NOTE 3 |
|  | > 47 | Prated,t,sys – 60dB |  |  |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [2], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [2], s2.5 table 1. NOTE 3: The lower frequency limit is replaced by 0.7 times the waveguide cut-off frequency, according to ITU-R SM.329 [2], for systems having an integral antenna incorporating a waveguide section, or with an antenna connection in such form, and of unperturbed length equal to at least twice the cut-off. |  |  |  |  |

6.6.5.2.2	Protection of the own Satellite Access Node receiver

This requirement shall be applied for NR FDD operation in order to prevent the receivers of the SAN being de-sensitized by emissions from its own SAN transmitter. It is measured at the TAB connector for SAN type 1-H for any type of SAN which has common or separate Tx/Rx TAB connectors.

The spurious emission basic limits are provided in table 6.6.5.2.2-1.

Table 6.6.5.2.2-1: SAN spurious emissions basic limits for protection of the SAN receiver

| Frequency range | Basic limits | Measurement bandwidth |
| --- | --- | --- |
| FUL,low – FUL,high | -96 dBm | 100 kHz |

6.6.5.2.3	Additional spurious emissions requirements

The additional spurious emissions requirement is not applicable for SAN.

6.6.5.2.4	Co-location with other Satellite Access Nodes

The co-location requirement is not applicable for SAN.

6.6.5.3	Minimum requirements for SAN type 1-H

The transmitter spurious emissions requirements for SAN type 1-H are that for each TAB connector TX min cell group and each applicable basic limit in clause 6.6.5.2, the power summation emissions at the TAB connectors of the TAB connectors of the TAB connector TX min cell group shall not exceed a limit specified as the basic limit + X, where X = 10log10(NTXU,countedpercell), unless stated differently in regional regulation.

NOTE:	Conformance to the SAN type 1-H spurious emission requirement can be demonstrated by meeting at least one of the following criteria as determined by the manufacturer:

1)   The sum of the emissions power measured on each TAB connector in the TAB connector TX min cell group shall be less than or equal to the limit as defined in this clause for the respective frequency span.

Or

2)   The unwanted emissions power at each TAB connector shall be less than or equal to the SAN type 1-H limit as defined in this clause for the respective frequency span, scaled by -10log10(n), where n is the number of TAB connectors in the TAB connector TX min cell group.

--------------Next change-------------

9.7.5.2	Minimum requirement for SAN type 1-O

9.7.5.2.1	General

The OTA transmitter spurious emission limits for FR1 shall apply from 30 MHz to the 5th harmonic of the upper frequency edge of the DL operating band, excluding the SAN transponder bandwidth BWSAN and the frequency range where the out-of-band emissions apply.

The requirements shall apply whatever the type of transmitter considered (single carrier or multi-carrier). It applies for all transmission modes foreseen by the manufacturer's specification.

9.7.5.2.2	General OTA transmitter spurious emissions requirements

The basic limits of table 9.7.5.2.2-1 shall apply. The application of those limits shall be the same as for out-of-band emissionsin clause 6.6.4.

Table 9.7.5.2.2-1: General SAN transmitter spurious emission limits in FR1

| Spurious frequency range | Prated,t,TRP (dBm) | Basic limit (dBm) | Measurement bandwidth (kHz) | Notes |
| --- | --- | --- | --- | --- |
| 30 MHz – 5th harmonic of the upper frequency edge of the DL operating band | ≤ 47 | -13 | 4 | NOTE 1, NOTE 2, NOTE 3 |
|  | > 47 | Prated,t,TRP – 60dB |  |  |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [2], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [2], s2.5 table 1. NOTE 3: The lower frequency limit is replaced by 0.7 times the waveguide cut-off frequency, according to ITU-R SM.329 [2], for systems having an integral antenna incorporating a waveguide section, or with an antenna connection in such form, and of unperturbed length equal to at least twice the cut-off. |  |  |  |  |

The transmitter spurious emissions minimum requirements for SAN type 1-O are that for each applicable basic limit above 30 MHz in table 9.7.5.2.2-1, the TRP of any spurious emission shall not exceed an OTA limit specified as the basic limit + X, where X = 9 dB, unless stated differently in regional regulation.

--------------End of change-------------