3GPP TSG-RAN WG4 Meeting #111 	R4-2408346

# Fukuoka City, Fukuoka, Japan, May 20th – 24th 2024

Source:	Ericsson

Title:	Simulation results for Rel-18 NR PRACH repetitions demodulation requirements

Agenda item:	7.17.2

Document for:	Information

# Introduction

In RAN4#110bis meeting, companies agreed to check the simulation assumptions including TDD pattern and PRACH repetition interval for both FR1 and FR2. Some other parameters were also agreed. Therefore, we introduced slot interval between two PRACH repetitions, as discussed in [2]. Based on the WF [1].

# Simulation result

## 2.1	Simulation assumptions

Table 2-1: Test parameters for PRACH repetition demodulation requirement for FR1

| PRACH | PRACH SCS | Time error tolerance |  |
| --- | --- | --- | --- |
| preamble | (kHz) | AWGN | TDLC300-100 |
| B4, A2, C2 | 15 | 0.52 us | 2.03 us |
|  | 30 | 0.26 us | 1.77 us |

Table 2-1: Test parameters for PRACH repetition demodulation requirement for FR2-1

| PRACH | PRACH SCS | Time error tolerance |  |
| --- | --- | --- | --- |
| preamble | (kHz) | AWGN | TDLA30-300 |
| B4, A2, C2 | 120 | 0.07 us | 0.22 us |

## 2.2	Simulation results

Table 2-1: PRACH repetition missed detection requirements for Normal Mode, 15 kHz SCS in FR1

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix (Annex G) | Frequency offset | Interval (slots) | SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Burst format A2 | Burst format B4 | Burst format C2 |
| 1 | 2 | TDLC300-100 Low | 400 Hz | 1 | -10.69 | -14.95 | -10.63 |
|  |  |  |  | 5 | -12.18 | -15.43 | -11.98 |
|  |  |  |  | 10 | -12.64 | -15.72 | -12.26 |
|  |  |  |  | 15 | -12.42 | -15.69 | -11.79 |

Table 2-2: PRACH repetition missed detection requirements for Normal Mode, 30 kHz SCS in FR1

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix (Annex G) | Frequency offset | Interval (slots) | SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Burst format A2 | Burst format B4 | Burst format C2 |
| 1 | 2 | TDLC300-100 Low | 400 Hz | 1 | -11.24 | -15.66 | -10.84 |
|  |  |  |  | 5 | -11.58 | -17.05 | -11.61 |
|  |  |  |  | 10 | -12.31 | -17.05 | -12.41 |
|  |  |  |  | 20 | -12.83 | -17.08 | -12.96 |

Table 2-3: PRACH repetition missed detection requirements for Normal Mode, 120 kHz SCS in FR2-1

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix (Annex G) | Frequency offset | Interval (slots) | SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Burst format A2 | Burst format B4 | Burst format C2 |
| 1 | 2 | TDLA30-300 Low | 4000 Hz | 1 | -11.02 | -13.13 | -10.59 |
|  |  |  |  | 10 | -11.64 | -14.97 | -11.86 |
|  |  |  |  | 20 | -12.5 | -14.94 | -12.73 |
|  |  |  |  | 40 | -12.52 | -15.01 | -12.55 |

# Conclusions

Simulation results for Rel-18 NR PRACH enhancement demodulation requirements are delivered.

# References

[1] R4-2406137, “WF on NR_cov_enh2_demod”, China Telecom, RAN4#110bis

[2] R4-2408345, Discussion on NR further coverage enhancement demodulation requirements, Ericsson