# 3GPP TSG-RAN WG4 Meeting # 111                                                        R4-2408382

# Fukuoka, Japan, May. 20th – 24th, 2024

Source:	ZTE Corporation, Sanechips

Title:	TP for TR38.718-03-01_3DL_xUL CA_n8A-n39A-n40A

Agenda Item:	6.11.2

Document for:	Approval

- Introduction

3DL/2UL CA_n8A-n39A-n40A_BCS 4 and 5 have already been included in the revised basket WID, the fallbacks were already completed.

This contribution provides a text proposal to introduce 3DL/2UL CA_n8A-n39A-n40A_BCS 4 and 5 in TR38.718-03-01,

- Reference

- [1] TR38.718-03-01,Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for3 bands DL with x bands UL (x=1,2),v0.8.0

- Text Proposal

----- Start of TP -----

  - 5.x			n8A-n39A-n40A

    - 5.x.1	Common for 1 band UL and 2 bands UL CA

      - 5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: Inter-band CA operating bands involving FR1 (three bands)

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| n8A-n39A-n40A | n8, n39, n40 |

      - 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band nX+nY+nZ

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n8A-n39A-n40A | CA_n8A-n39A CA_n8A-n40A CA_n39A-n40A | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |

      - 5.x.1.3	∆TIB,c and ∆RIB,c values

For CA_n8-n39-n40, the TIB,c and RIB,c values are proposed to be the same with DC_8_n39-n40, which are defined in the following tables:

Table 5.x.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n8-n39-n40 | 0.3 | 0.3 | 0.3 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.x.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n8-n39-n40 | - | - | - |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

    - 5.x.2	Specific for 2 bands UL CA

      - 5.x.2.1	UE co-existence studies

The co-existence for the fallback 2DL/2UL of CA_n8A-n39A, CA_n8A-n40A and CA_n39A-n40A have already been analyzed. In terms of the co-existence studies of corresponding fallbacks, it can be observed:

No IMD issue caused by n8+n39 fall into the its own band n40 Rx;

No IMD issue caused by n8+n40 fall into the its own band n39 Rx;

IMD4 and IMD5 issue caused by n39+n40 fall into the its own band n8 Rx.

      - 5.x.2.2	REFSENS requirements

Based on co-existence studies additional MSD is needed to be defined, shown in table 5.x.2.2-1., where MSD4 requirements are reused form DC_n28-n39-n40.

Table 5.x.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n8-n39-n40 | n8 | N/A | 5 | N/A | 940 | 8.6 | IMD4 |
|  | n39 | 1920 | 5 | 25 | 1920 | N/A | N/A |
|  | n40 | 2370 | 2 | 25 | 2370 | N/A | N/A |
|  | n8 | N/A | 5 | N/A | 950 | 4.3 | IMD5 |
|  | n39 | 1910 | 5 | 25 | 1910 | N/A | N/A |
|  | n40 | 2390 | 5 | 25 | 2390 | N/A | N/A |

----- End of TP -----