# 3GPP TSG-RAN WG4 Meeting # 111                                                      R4-2408390

# Fukuoka, Japan, May. 20th – 24th, 2024

# Source:	ZTE Corporation

# Title:	New WID: Rel-19 Dual Connectivity (DC) of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and y bands (3<=y<=5 and x+y<=6) NR inter-band CA (yDL/1UL)

# Document for:	Approval

Agenda Item:	13

# 3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Rel-19 Dual Connectivity (DC) of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and y bands (3<=y<=5 and x+y<=6) NR inter-band CA (yDL/1UL)

Acronym:  [DC_R19_xBLTE_yBNR_zDL2UL]

Unique identifier: 	{A number to be provided by MCC at the plenary}

NOTE:	For new WIs/SIs leave the Unique identifier empty or you can make a proposal for an Acronym.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part |  |

# Potential target Release: Rel-19

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan. NOTE: In case of contradiction with the target dates of clause 5, clause 5 determines the target release.

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  |  |  |
| No | X |  | X | X | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
| X | Building Block |
|  | Work Task |
|  | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, we define them as work tasks. If you are in doubt, please contact MCC.

2.2	Parent and child Work Items

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one WID. Therefore the table above should just include the feature WI Unique ID and title and Nature of relationship is "parent WID".

2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one but are not part of the hierarchical structure, such as preceding SI or a preceding WI (e.g. if you further enhance a topic).}

| Other related Work Items (if any) |  |  |  |
| --- | --- | --- | --- |
| Acronym | Unique ID | Title | Nature of relationship |
| DC_R17_xBLTE_3BNR_yDL2UL | 881104 | Rel-17 Dual Connectivity (DC) of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and 3 bands NR inter-band CA (3DL/1UL) | {optional free text} |
| DC_R17_xBLTE_3BNR_yDL2UL | 881204 | Rel-17 Dual Connectivity (DC) of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and 3 bands NR inter-band CA (3DL/1UL) | {optional free text} |
| DC_R18_xBLTE_yBNR_zDL2UL | 961106 | Core part: Rel-18 Dual Connectivity (DC) of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and y bands (3<=y<=5 and x+y<=6) NR inter-band CA (yDL/1UL) | {optional free text} |
| DC_R18_xBLTE_yBNR_zDL2UL | 961206 | Perf. part: Rel-18 Dual Connectivity (DC) of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and y bands (3<=y<=5 and x+y<=6) NR inter-band CA (yDL/1UL) | {optional free text} |

NOTE:	Also related or dependent WIs in other TSGs should be indicated.

# 3	Justification

All new DC configurations including x LTE bands(x=1,2,3) and y bands(3<=y<=5 and x+y<=6) NR inter band CA for both EN-DC and NE-DC will be defined under this WI. New configurations still emerge from exiting bands and whenever new band is specified, it will create a potential for several new DC of LTE CA for up to 3 different bands DL with 1 band UL + NR CA for y different bands (3<=y<=5) DL with 1 band UL, i.e. xBLTE_yBNR_zDL2UL, where x=1, 2, 3, 3<=y<=5 , z≤6

Request for additions of band combinations to this WI shall be provided using an agreed template and sent to the 3GPP_TSG_RAN_WG4_NR_BANDS email reflector before a RAN4 Tdoc submission deadline and no new band combinations are allowed to be requested after the deadline except to correct the missing fallback and add more supporting companies for the proposed band combinations..

When a proponent requests a new band combination, all the next level fallback configurations shall be listed and recorded in the request template and the status (“New”, “Ongoing”, “Completed”) of all the fallback configurations shall be declared accurately and clearly. For “New” fallback configurations, the proponent shall ensure these fallback configurations are also requested together with the higher order band combination in the same meeting.

A band combination configuration can only be considered as completed when all of the fallback configurations are completed and specified in advance or at the same meeting. It is the responsibility of the proponent to ensure the status of all of the fallback mode configurations. Rapporteurs and other companies are encouraged to check the status of all of the fallback configurations once the higher order band combinations are declared as completed.

- The preconditions to propose DC of LTE CA for up to 3 different bands DL with 1 band UL + NR CA for y different bands (3<=y<=5) with 1 band UL in Rel-19 are as follows.

- Constituent NR inter band CA including intra band CA for y different bands (3<=y<=5) with 1 band UL shall be completed and specified in advance.

- Constituent LTE inter band CA including intra band CA for up to 3 different bands DL with 1 band UL shall be completed and specified in advance.

- Each of the paired DC configurations of 3 different LTE band including intra band CA + 2 NR band used in a certain DC of LTE CA for up to 3 different bands DL with 1 band UL + NR CA for 3 bands DL with 1 band UL shall be completed and specified in advance.

Example 1: If the following configuration is proposed,

| DC Downlink configuration | DC Uplink configuration |
| --- | --- |
| DC_1A-2A_n3A-n4A-n5A | DC_1A_n3A, DC_1A_n4A, DC_1A_n5A, DC_2A_n3A, DC_2A_n4A, DC_2A_n5A |

- NR CA of 3 different bands DL with 1 band UL of CA_n3A-n4A-n5A shall be completed and specified in advance.

- LTE CA of 2 different bands DL with 1 band UL of CA_1A-2A shall be completed and specified in advance

- DC of DC_1A-2A_n3A-n4A, DC_1A-2A_n3A-n5A and DC_1A-2A_n4A-n5A shall be completed and specified in advance.

Example 2: If the following configuration is proposed,

| DC Downlink configuration | DC Uplink configuration |
| --- | --- |
| DC_1C-2A_n3C-n4A-n5A | DC_1C_n3C |

- NR CA of 3 different bands DL with 1 band UL of CA_n3C-n4A-n5A shall be completed and specified in advance.

- LTE CA of 2 different bands DL with 1 band UL of CA_1C-2A shall be completed and specified in advance

- DC of DC_1C-2A_n3C-n4A, DC_1C-2A_n3C-n5A and DC_1C-2A_n3A-n4A-n5A shall be completed and specified in advance.

Example 3: If the following configuration is proposed,

| DC Downlink configuration | DC Uplink configuration |
| --- | --- |
| DC_1A-2A_n3A-n4A-n5A-n6A | DC_1A_n3A, DC_1A_n4A, DC_1A_n5A, DC_1A_n6A, DC_2A_n3A, DC_2A_n4A, DC_2A_n5A, DC_2A_n6A |

- NR CA of 4 different bands DL with 1 band UL of CA_n3A-n4A-n5A-n6A shall be completed and specified in advance.

- LTE CA of 2 different bands DL with 1 band UL of CA_1A-2A shall be completed and specified in advance

- DC of DC_1A-2A_n3A-n4A-n5A, DC_1A-2A_n3A-n4A-n6A, DC_1A-2A_n3A-n5A-n6A and DC_1A-2A_n4A-n5A-n6A shall be completed and specified in advance.

Example 4: If the following configuration is proposed,

| DC Downlink configuration | DC Uplink configuration |
| --- | --- |
| DC_1C-2A_n3C-n4A-n5A-n6A | DC_1C_n3C |

- NR CA of 4 different bands DL with 1 band UL of CA_n3C-n4A-n5A-n6A shall be completed and specified in advance.

- LTE CA of 2 different bands DL with 1 band UL of CA_1C-2A shall be completed and specified in advance

- DC of DC_1C-2A_n3C-n4A-n5A, DC_1C-2A_n3C-n4A-n6A, DC_1C-2A_n3C-n5A-n6A and DC_1C-2A_n3A-n4A-n5A-n6A shall be completed and specified in advance.

# 4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

- Specify the DC configurations listed in the tables below,  including at least

  - Applicable frequencies if necessary

  - Applicable bandwidths and bandwidth sets if necessary

- Specify the band-combination specific RF requirements for these DC configurations, i.e.

  - Analyse combinations that have self-desensitization, specify at least needed

    - ∆TIB, c and ∆RIB, c

    - Reference sensitivity exceptions including MSD test cases

of all Rel-19 DC including EN-DC and NE-DC of LTE CA for up to 3 different bands DL with 1 band UL + NR CA for y different bands (3<=y<=5 and x+y<=6) with 1 band UL that fall into the category is defined by the WI title.

An overview table of these EN-DC and NE-DC configurations are provided in the EXCEL file zipped together with WORD file WID.

Unless stated otherwise, DC combinations of this WI are introduced in a REL-independent way starting from release 15 according to Table 8.1.2.3-1 and Table 8.2.1.2-1 of TS 38.307.

For the DC combination, between FR1 and FR2 where the supported bandwidth classes for the FR2 band are R2, R3, R4, R5, R6, R7, R8, R9, R10,  the DC combinations are introduced in a release independent way starting with Rel-17.

However no changes to TS 38.307 are needed.

4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

# 5	Expected Output and Time scale

NOTE:	If this is a RAN WID including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.101-3 | Add new DC band combinations and related RF core requirements | TSG#xxx | Core part |

NOTE:	If this is a RAN WID including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

6	Work item Rapporteur(s)

xxx

# 7	Work item leadership

R4

# 8	Aspects that involve other WGs

None

NOTE:	For RAN WIDs: Section 8 applies only toWGs outside of TSG RAN because RAN WG aspects have to be covered in section 4.

# 9	Supporting Individual Members

Supporting IM name

[CMCC]

[CATT]

ZTE

Sanechips

[T-Mobile]

[Xiaomi]

[CHTTL]

[SoftBank]

[China Unicom]