3GPP TSG-RAN WG4 Meeting # 111		R4-2408424

Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Agenda item:		7.19.2.2

Source:		Samsung

Title:	Discussion on other RRM tests for MIMO evolution for downlink and uplink

Document for:		Discussion

- Introduction

In last RAN4#110-bis meeting, RAN4 discussed performance part of Rel-18 MIMO and WF has been approved in [1]. In this contribution, we continue to discuss the RRM tests for Rel-18 MIMO such as timing and mTRP TCI state switching.

- Discussion

In last RAN4#110-bis meeting, the test case list has been agreed in [1] as below:

TC: FR1 UE transmit timing from two TRPs.

TC: FR2 UE transmit timing from two TRPs.

TC: sDCI mTRP, FR2 separate DL TCI state switching

TC: sDCI mTRP, FR2 separate UL TCI state switching

TC: sDCI mTRP, FR2 joint TCI state switching

TC: mDCI FR1 two TA, RTD>CP, joint TCI state switching

For the test cases of “FR1 UE transmit timing from two TRPs” and “FR2 UE transmit timing from two TRPs”:

In legacy UE transmit timing test, there are Test1 and Test2 for non-DRX and with DRX separately.

In two TA test, according to the core requirements, MRTD can be different depends on UE capability “rxTimingDiff-r18”.

Proposal 1: For UE transmit timing from two TRPs TCs, define different configurations of Test 1/2/3/4 in the same TC.

Test 1: non DRX; RTD<CP

Test 2: non DRX; RTD>CP

Test 3: DRX; RTD<CP

Test 4: DRX; RTD>CP

For UE not support the capability of “rxTimingDiff-r18”, the UE is only required to be tested in Test1 and Test3.

For UE supports the capability of “rxTimingDiff-r18”, the UE is only required to be tested in Test2 and Test4.

Proposal 2: The DL timing is changed for one TRP while another TRP keeps the DL timing unchanged. Or two DL timing are changed for two TRPs with different values.

Issue 3-3-2: Test configuration of for s-DCI mTRP cases: - Proposals - Proposal 1: (Apple) - Dual TCI state switching - Proposal 2: (Ericsson, Nokia) - Single to dual TCI state switching

For the test configuration of TCI state switching in mTRP cases, it is different from that in multi-RX WI. In multi-RX, the real dual TCI switching as option 3 in 38.871 cannot be supported because it is hard to find proper AoA pairs from the multi-Rx spherical coverage requirements defined in UE RF core requirements.

However, in Rel-18 MIMO, the dual TCI states is not transmit/received simultaneously. It is TDMed. For the same moment, the AoA is only to be met for legacy RF spherical coverage requirements. Setup 3 of AoA can be reused as baseline. Before MAC-CE, two probes of two AoAs with “30°” are configured in TDM. After MAC-CE, two probes of two AoAs with a different relative angular offset between the directions are configured in TDM.

Proposal 3: For the test configuration of TCI state switching in mTRP, dual TCI state switching can be used. Setup 3 of AoA can be reused as baseline. Two probes of two AoAs are configured in TDM.

- Conclusion

In this contribution, we provide our proposals for RRM tests for Rel-18 MIMO such as timing and mTRP TCI state switching:

Proposal 1: For UE transmit timing from two TRPs TCs, define different configurations of Test 1/2/3/4 in the same TC.

Test 1: non DRX; RTD<CP

Test 2: non DRX; RTD>CP

Test 3: DRX; RTD<CP

Test 4: DRX; RTD>CP

For UE not support the capability of “rxTimingDiff-r18”, the UE is only required to be tested in Test1 and Test3.

For UE supports the capability of “rxTimingDiff-r18”, the UE is only required to be tested in Test2 and Test4.

Proposal 2: The DL timing is changed for one TRP while another TRP keeps the DL timing unchanged. Or two DL timing are changed for two TRPs with different values.

Proposal 3: For the test configuration of TCI state switching in mTRP, dual TCI state switching can be used. Setup 3 of AoA can be reused as baseline. Two probes of two AoAs are configured in TDM.

- Reference

[1] R4-2406503, WF on RRM requirements for NR_MIMO_evo_DL_UL, Samsung