3GPP TSG-RAN4 Meeting #111	R4-2408433

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Source: 	Qualcomm Incorporated

Title: 	MG-Enh2-Perf remaining issues for part 2 perf requirements

Agenda item:	7.5.4

Document for:	Discussion

# Introduction

In this paper, we share our view for remaining RRM performance requirements issue about R18 FeMG WI.

# Discussion

Intra-frequency measurement TC

In TS38.533, the spec clearly define the applicability rule for the legacy test case about intra-frequency measurement with gap and SSB is outside of active BWP.

If RAN4 agreed to define the test case for R18 NFG then same applicability rule should be applied or UE support FG 53-4 CSI-RS based L1 for Rel-18 WI of BWP without restriction.

Proposal : If RAN4 agreed to define the intra-frequency without gap but SSB outside of BWP test case, the following applicability rule should be applied :

1) capable of both 'CSI-RS based RLM' and 'BWP without restriction,' or

2) capable of FG 53-4 (CSI-RS based L1) for Rel-18 WI of BWP without Restriction

TC with DRX

Besides of DRX with interruption discussion in core requirement, it is not testable for no-gap interruption. Interruption can be only testable with continuous data transmission on PCell and check missing ACK/NCK ratio at TE side. If DRX is configured, first UE cannot receive continuous data transmission; second if UE decode PDCCH the UE will wake up. 
Therefore, interruption ratio cannot properly calculate unlike to no-DRX test case.

Observation : If DRX is configured, first UE cannot receive continuous data transmission; second if UE decode PDCCH the UE will wake up. Therefore, interruption ratio cannot properly calculate unlike to no-DRX test case.

Proposal : RAN4 do not define test with DRX configuration.

# Conclusions

Proposal : If RAN4 agreed to define the intra-frequency without gap but SSB outside of BWP test case, the following applicability rule should be applied :

1) capable of both 'CSI-RS based RLM' and 'BWP without restriction,' or

2) capable of FG 53-4 (CSI-RS based L1) for Rel-18 WI of BWP without Restriction

Observation : If DRX is configured, first UE cannot receive continuous data transmission; second if UE decode PDCCH the UE will wake up. Therefore, interruption ratio cannot properly calculate unlike to no-DRX test case.

Proposal : RAN4 do not define test with DRX configuration.

# References

[1] R4-2403543, WF on NR_MG_enh2_part2, Intel, RAN #110

[2] R4-2406442, WF on RRM requirements for NR_MG_enh2_part2, Intel, RAN #110bis