3GPP TSG-RAN WG4 Meeting #111	R4-2408511

Fukuoka, Japan, May 20 – May 24, 2024

Source:	Nokia

Title:	Dicussion on UE RX-TX Time measurements

Agenda item:	7.16.6.2

Document for:	Discussion

Introduction

The NTN WI, as presented in [1], includes objectives for the NTN enhancements in Rel-18, including the introduction of a feature of network verified UE location.:

4.1.3 Network verified UE location Based on RAN1 conclusions of the study phase, RAN to prioritize the specification of necessary enhancements to multi-RTT to support the network verified UE location in NTN assuming a single satellite in view [RAN1, 2, 3, 4]. DL-TDoA methods for verification may be considered as lower priority and if time permits and condition in Note is satisfied. Note 1: Enhancements assume reuse of the RAT dependent positioning framework Note 2: The specification of DL-TDOA enhancements will be subject to the study of the impact of realistic UE clock drift onto DL-TDOA performance Note 3: The target accuracy for position verification purposes is as documented in clause « recommendations » of the 3GPP TR 38.882 (i.e. 10 km granularity) Note 4 : Multiple satellite in view by the UE may be considered if time allows Note 5 : The enhancements may be subject to relevant SA WGs (e.g. SA3/SA3-LI) feedbacks on the reliability of UE reports involved Note 6 : The enhancements should take into account the mirror-image ambiguity Note 7 : Network verified UE location is an optional UE feature

The present document will discuss maintenance aspects related to the agreements made for this topic and current open issues that can be found in [2], such as:

- The impact of satellite switching with re-sync

- The impact of UE autonomous updates on Timing Advance, caused by UE specific TA or by Common TA variation.

Discussion

Impact of Satellite switching with re-synchronization

In previous meeting RAN4 has discussed whether the change of the synchronization point would require the UE to restart the RX-TX measurement as to avoid ambiguity in the reporting sent towards the network:

### Issue 3-5: Other impact on RRM Agreement (online): - When UE switches to a new satellite switch with same PCI (through HO, CHO or satellite switch with re-sync for both hard and soft satellite switch), select one option from: - Option 1a: UE stops the PRS measurement after t_service and restart the UE Rx-Tx time difference measurement after switch is complete. - The PRS configuration of the two satellites with the same PCI follows RAN1/2 specficiaton, and no further restriction for gNB/LMF to be defined in RAN4. - The UE measurement accuracy requiremet does not apply if the PRS transmission from two satelliates overlap in time/frequency domain. Further clarify the definition of “overlapping” offline. - Option 1b: UE stops the PRS measurement after t_servicestart and restart the UE Rx-Tx time difference measurement after switch is complete. - The PRS configuration of the two satellites with the same PCI follows RAN1/2 specficiaton, and no further restriction for gNB/LMF to be defined in RAN4. - Option 2: UE stops the PRS measurement after t_service and until LMF triggers new measurement. FFS (online): - When UE switches to a new satellite switch with different PCI (through HO or CHO), - UE stops the PRS measurement for the source cell and re-start PRS measurement for the target cell after SRS (re)configuration on the target cell is complete.

The objective of the discussion is to avoid ambiguity at the network side. For achieving that it is important that the network unequivocally can distinguish whether the current report was collected under the serving or target satellite.

RAN4 has to guarantee that the network understands unequivocally whether the UE measurements were collected in the source or target satellites.

The measurements collected in source and target satellies are not contiguous, and therefore, it is impacted whether the measurement is collected in the source or in the target satellite: after a satellite switching, even though the configuration of SRS parameters might be considered the same, there will be a significant modification in the SRS signal, as there will be an offset in time between the SRS transmitted from the source and target satellites (this might be interpreted similarly as a SRS reconfiguration).

We also see that the objective of satellite switching is a smooth transition of UE context, enabling a faster reconfiguration. We also believe, based on RAN 2 current agreements and definition is that, except for satellite assistance information and offset on SSB transmission, the configuration of the cell in both source and target satellites are the same. Therefore, our understanding is that the measurements might be re-started once the UE synchronizes to the same cell in the target satellite.

Therefore, for issue 3-5 we prefer to select either option 1a or 1b.

For satellite switching with same PCI, select option 1a:

- UE stops the PRS measurement after t_service and restart the UE Rx-Tx time difference measurement after switch is complete.

    - The PRS configuration of the two satellites with the same PCI follows RAN1/2 specficiaton, and no further restriction for gNB/LMF to be defined in RAN4.

    - The UE measurement accuracy requiremet does not apply if the PRS transmission from two satelliates overlap in time/frequency domain. Further clarify the definition of “overlapping” offline.

For the FFS part, we do not see this as a problem, as this has already addressed in legacy specifications.

When a serving cell change occurs during the measurement period, the UE shall continue and complete the UE Rx-Tx time difference measurement provided that the serving cell change does not impact SRS configuration for the UE Rx-Tx time difference measurement.

In other words, serving cell changes to the same satellite might not require the measurements to be interrupted, whereas serving cell changes towards target satellites will necessarily require the measurements to be stopped.

No further agreements are needed for serving cell changes with different PCI. But a clarification note can be added to the specification for the case of different satellites.

Impact of UE autonomous pre compensation of timing advance (Issue 3.4)

In our previous contribution[3], we have extensively discussed the importance for the network to be aware of changes in UE timing advance when the measurements were collected, the following tentative agreement was reached during the meeting:

### Issue 3-4: Measurement accuracy requirements on UL timing drift FFS (ad-hoc): - No new applicability condition for UE Rx-Tx measurement requirements related to amount of variation in the applied TA during measurement period. - Including DL timing drift reporting as condition

Although we do not oppose with this clarification, it is important to understand what is included in DL timing drift reporting. As, according to 38.215, the DL timing drift is given by:

Definition; DL timing drift is defined as the variation rate of the downlink delay in ppm due to the service link Doppler over the UE Rx-Tx time difference measurement period. For frequency range 1, the reference point for the DL timing drift measurement shall be the Rx antenna connector of the UE. For frequency range 2, the reference point for the DL timing drift measurement shall be the Rx antenna of the UE.. Applicable for; RRC_CONNECTED.

Hence the report of DL timing drift would include the variation of the DL timing reference during the measurements. Note that, changes in satellite position, common delay and UE position will affect the total DL reference timing. But changes in UE position will not be directly translated into timing advance changes, as they shall follow the Tp and Tq requirements.

Another problem is that the DL timing drift report has still not being implemented by RAN2, as the signalling message for the reporting has not been specified. Therefore, it is important to wait for the implementation of the DL timing drift before concluding whether it is sufficient to guarantee an harmonious measurement even when the UE is updating its timing information.

Wait until RAN2 implements the DL Timing Drift Report in order to decide whether the report is sufficient to guarantee that UE RX-TX measurement requirements are applicable even UE applies significant changes on its timing advance within the measurement period.

Conclusion

The paper has discussed the need of side conditions for the applicability of mobility requirements for enhanced mobility features in NTN. The following observations and proposals were made:

Observation 1: RAN4 has to guarantee that the network understands unequivocally whether the UE measurements were collected in the source or target satellites.

Proposal 1: For satellite switching with same PCI, select option 1a:

UE stops the PRS measurement after t_service and restart the UE Rx-Tx time difference measurement after switch is complete.

i.	The PRS configuration of the two satellites with the same PCI follows RAN1/2 specficiaton, and no further restriction for gNB/LMF to be defined in RAN4.

ii.	The UE measurement accuracy requiremet does not apply if the PRS transmission from two satelliates overlap in time/frequency domain. Further clarify the definition of “overlapping” offline.

Proposal 2: No further agreements are needed for serving cell changes with different PCI. But a clarification note can be added to the specification for the case of different satellites.

Proposal 3: Wait until RAN2 implements the DL Timing Drift Report in order to decide whether the report is sufficient to guarantee that UE RX-TX measurement requirements are applicable even UE applies significant changes on its timing advance within the measurement period.

- References

- RP-223534, “NR NTN (Non-Terrestrial Networks) enhancements”, Thales

- R4-2406430, “WF on RRM requirements for NR_NTN_enh”, Qualcomm (Moderator), Changsha, China, 15th – 19th April, 2024

- R4-2405750, “Maintenance aspects of network verified UE location”, Nokia, Changsha, China, 15th – 19th April, 2024